| Project | Specification of Sensor Interface for Cyber and Physical World <https://sagroups.ieee.org/2888/ >> |
| --- | --- |
| Title | IEEESTD-2888.1D1.2 |
| DCN | 2888-20-0041-00-0001 |
| Date Submitted | July 23, 2020 |
| Source(s) | Kyoungro Yoon, yoonk@konkuk.ac.kr (Konkuk University) Sang-Kyun Kim, goldmunt@gmail.com (Myongji University) Sangkwon Jeong, ceo@joyfun.kr (Joyfun) HyeonWoo Nam, hwnam@dongduk.ac.kr (Dongduk Women’s University) Dong Soo Choi, soochoi@dau.ac.kr (Dong-A University) Jeonghwoan Choi jordhanchoi@skonec.com (Skonec Entertainment) |
| Re: |  |
| Abstract | Integrated version of P2888.1 D1.2 with input contributions from the Session 2 |
| Purpose | To improve P2888.1 D1 |
| Notice | This document has been prepared to assist the IEEE 2888 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that IEEE 2888 may make this contribution public. |
| Patent Policy | The contributor is familiar with IEEE patent policy, as stated in Section 6 of the IEEE-SA Standards Board bylaws <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and in Understanding Patent Issues During IEEE Standards Development http://standards.ieee.org/board/pat/faq.pdf |

PvarDesignation™/D1 
Draft Standard for Specification of Sensor Interface for Cyber and Physical World

Developed by the

Standard Activities Board (SAB)

of the

IEEE Computer Society

Approved varApprovedDate

IEEE SA Standards Board

Copyright © varCRYear by The Institute of Electrical and Electronics Engineers, Inc.

Three Park Avenue

New York, New York 10016-5997, USA

All rights reserved.

This document is an unapproved draft of a proposed IEEE Standard. As such, this document is subject to change. USE AT YOUR OWN RISK! IEEE copyright statements SHALL NOT BE REMOVED from draft or approved IEEE standards, or modified in any way. Because this is an unapproved draft, this document must not be utilized for any conformance/compliance purposes. Permission is hereby granted for officers from each IEEE Standards Working Group or Committee to reproduce the draft document developed by that Working Group for purposes of international standardization consideration.  IEEE Standards Department must be informed of the submission for consideration prior to any reproduction for international standardization consideration (stds.ipr@ieee.org). Prior to adoption of this document, in whole or in part, by another standards development organization, permission must first be obtained from the IEEE Standards Department (stds.ipr@ieee.org). When requesting permission, IEEE Standards Department will require a copy of the standard development organization's document highlighting the use of IEEE content. Other entities seeking permission to reproduce this document, in whole or in part, must also obtain permission from the IEEE Standards Department.

IEEE Standards Department

445 Hoes Lane

Piscataway, NJ 08854, USA

Abstract: <Select this text and type or paste Abstract—contents of the Scope may be used>

Keywords: <Select this text and type or paste keywords>

Important Notices and Disclaimers Concerning IEEE Standards Documents

IEEE documents are made available for use subject to important notices and legal disclaimers. These notices and disclaimers, or a reference to this page, appear in all standards and may be found under the heading “Important Notices and Disclaimers Concerning IEEE Standards Documents.” They can also be obtained on request from IEEE or viewed at http://standards.ieee.org/ipr/disclaimers.html.

Notice and Disclaimer of Liability Concerning the Use of IEEE Standards Documents

IEEE Standards documents (standards, recommended practices, and guides), both full-use and trial-use, are developed within IEEE Societies and the Standards Coordinating Committees of the IEEE Standards Association (“IEEE SA”) Standards Board. IEEE (“the Institute”) develops its standards through a consensus development process, approved by the American National Standards Institute (“ANSI”), which brings together volunteers representing varied viewpoints and interests to achieve the final product. IEEE Standards are documents developed through scientific, academic, and industry-based technical working groups. Volunteers in IEEE working groups are not necessarily members of the Institute and participate without compensation from IEEE. While IEEE administers the process and establishes rules to promote fairness in the consensus development process, IEEE does not independently evaluate, test, or verify the accuracy of any of the information or the soundness of any judgments contained in its standards.

IEEE Standards do not guarantee or ensure safety, security, health, or environmental protection, or ensure against interference with or from other devices or networks. Implementers and users of IEEE Standards documents are responsible for determining and complying with all appropriate safety, security, environmental, health, and interference protection practices and all applicable laws and regulations.

IEEE does not warrant or represent the accuracy or content of the material contained in its standards, and expressly disclaims all warranties (express, implied and statutory) not included in this or any other document relating to the standard, including, but not limited to, the warranties of: merchantability; fitness for a particular purpose; non-infringement; and quality, accuracy, effectiveness, currency, or completeness of material. In addition, IEEE disclaims any and all conditions relating to: results; and workmanlike effort. IEEE standards documents are supplied “AS IS” and “WITH ALL FAULTS.”

Use of an IEEE standard is wholly voluntary. The existence of an IEEE standard does not imply that there are no other ways to produce, test, measure, purchase, market, or provide other goods and services related to the scope of the IEEE standard. Furthermore, the viewpoint expressed at the time a standard is approved and issued is subject to change brought about through developments in the state of the art and comments received from users of the standard.

In publishing and making its standards available, IEEE is not suggesting or rendering professional or other services for, or on behalf of, any person or entity nor is IEEE undertaking to perform any duty owed by any other person or entity to another. Any person utilizing any IEEE Standards document, should rely upon his or her own independent judgment in the exercise of reasonable care in any given circumstances or, as appropriate, seek the advice of a competent professional in determining the appropriateness of a given IEEE standard.

IN NO EVENT SHALL IEEE BE LIABLE FOR ANY DIRECT, INDIRECT, INCIDENTAL, SPECIAL, EXEMPLARY, OR CONSEQUENTIAL DAMAGES (INCLUDING, BUT NOT LIMITED TO: PROCUREMENT OF SUBSTITUTE GOODS OR SERVICES; LOSS OF USE, DATA, OR PROFITS; OR BUSINESS INTERRUPTION) HOWEVER CAUSED AND ON ANY THEORY OF LIABILITY, WHETHER IN CONTRACT, STRICT LIABILITY, OR TORT (INCLUDING NEGLIGENCE OR OTHERWISE) ARISING IN ANY WAY OUT OF THE PUBLICATION, USE OF, OR RELIANCE UPON ANY STANDARD, EVEN IF ADVISED OF THE POSSIBILITY OF SUCH DAMAGE AND REGARDLESS OF WHETHER SUCH DAMAGE WAS FORESEEABLE.

Translations

The IEEE consensus development process involves the review of documents in English only. In the event that an IEEE standard is translated, only the English version published by IEEE should be considered the approved IEEE standard.

Official statements

A statement, written or oral, that is not processed in accordance with the IEEE SA Standards Board Operations Manual shall not be considered or inferred to be the official position of IEEE or any of its committees and shall not be considered to be, or be relied upon as, a formal position of IEEE. At lectures, symposia, seminars, or educational courses, an individual presenting information on IEEE standards shall make it clear that his or her views should be considered the personal views of that individual rather than the formal position of IEEE.

Comments on standards

Comments for revision of IEEE Standards documents are welcome from any interested party, regardless of membership affiliation with IEEE. However, IEEE does not provide consulting information or advice pertaining to IEEE Standards documents. Suggestions for changes in documents should be in the form of a proposed change of text, together with appropriate supporting comments. Since IEEE standards represent a consensus of concerned interests, it is important that any responses to comments and questions also receive the concurrence of a balance of interests. For this reason, IEEE and the members of its societies and Standards Coordinating Committees are not able to provide an instant response to comments or questions except in those cases where the matter has previously been addressed. For the same reason, IEEE does not respond to interpretation requests. Any person who would like to participate in revisions to an IEEE standard is welcome to join the relevant IEEE working group.

Comments on standards should be submitted to the following address:

Secretary, IEEE SA Standards Board

445 Hoes Lane

Piscataway, NJ 08854 USA

Laws and regulations

Users of IEEE Standards documents should consult all applicable laws and regulations. Compliance with the provisions of any IEEE Standards document does not imply compliance to any applicable regulatory requirements. Implementers of the standard are responsible for observing or referring to the applicable regulatory requirements. IEEE does not, by the publication of its standards, intend to urge action that is not in compliance with applicable laws, and these documents may not be construed as doing so.

Copyrights

IEEE draft and approved standards are copyrighted by IEEE under US and international copyright laws. They are made available by IEEE and are adopted for a wide variety of both public and private uses. These include both use, by reference, in laws and regulations, and use in private self-regulation, standardization, and the promotion of engineering practices and methods. By making these documents available for use and adoption by public authorities and private users, IEEE does not waive any rights in copyright to the documents.

Photocopies

Subject to payment of the appropriate fee, IEEE will grant users a limited, non-exclusive license to photocopy portions of any individual standard for company or organizational internal use or individual, non-commercial use only. To arrange for payment of licensing fees, please contact Copyright Clearance Center, Customer Service, 222 Rosewood Drive, Danvers, MA 01923 USA; +1 978 750 8400. Permission to photocopy portions of any individual standard for educational classroom use can also be obtained through the Copyright Clearance Center.

Updating of IEEE Standards documents

Users of IEEE Standards documents should be aware that these documents may be superseded at any time by the issuance of new editions or may be amended from time to time through the issuance of amendments, corrigenda, or errata. An official IEEE document at any point in time consists of the current edition of the document together with any amendments, corrigenda, or errata then in effect.

Every IEEE standard is subjected to review at least every 10 years. When a document is more than 10 years old and has not undergone a revision process, it is reasonable to conclude that its contents, although still of some value, do not wholly reflect the present state of the art. Users are cautioned to check to determine that they have the latest edition of any IEEE standard.

In order to determine whether a given document is the current edition and whether it has been amended through the issuance of amendments, corrigenda, or errata, visit IEEE Xplore at http://ieeexplore.ieee.org/ or contact IEEE at the address listed previously. For more information about the IEEE SA or IEEE’s standards development process, visit the IEEE SA Website at http://standards.ieee.org.

Errata

Errata, if any, for IEEE standards can be accessed via https://standards.ieee.org/standard/index.html. Search for standard number and year of approval to access the web page of the published standard. Errata links are located under the Additional Resources Details section. Errata are also available in IEEE Xplore: https://ieeexplore.ieee.org/browse/standards/collection/ieee/. Users are encouraged to periodically check for errata.

Patents

Attention is called to the possibility that implementation of this standard may require use of subject matter covered by patent rights. By publication of this standard, no position is taken by the IEEE with respect to the existence or validity of any patent rights in connection therewith. If a patent holder or patent applicant has filed a statement of assurance via an Accepted Letter of Assurance, then the statement is listed on the IEEE SA Website at https://standards.ieee.org/about/sasb/patcom/patents.html. Letters of Assurance may indicate whether the Submitter is willing or unwilling to grant licenses under patent rights without compensation or under reasonable rates, with reasonable terms and conditions that are demonstrably free of any unfair discrimination to applicants desiring to obtain such licenses.

Essential Patent Claims may exist for which a Letter of Assurance has not been received. The IEEE is not responsible for identifying Essential Patent Claims for which a license may be required, for conducting inquiries into the legal validity or scope of Patents Claims, or determining whether any licensing terms or conditions provided in connection with submission of a Letter of Assurance, if any, or in any licensing agreements are reasonable or non-discriminatory. Users of this standard are expressly advised that determination of the validity of any patent rights, and the risk of infringement of such rights, is entirely their own responsibility. Further information may be obtained from the IEEE Standards Association.

Participants

At the time this draft Standard was completed, the Interfacing Cyber and Physical World Working Group had the following membership:

Kyoungro Yoon, Chair

Sang-Kyun Kim, Vice Chair

SangKwon Jeong

Participant2

Participant3

Participant4

Participant5

Participant6

Participant7

Participant8

Participant9

The following members of the <individual/entity> Standards Association balloting group voted on thistxtTrialUsetxtGorRPorSTD. Balloters may have voted for approval, disapproval, or abstention.

[To be supplied by IEEE]

Balloter1

Balloter2

Balloter3

Balloter4

Balloter5

Balloter6

Balloter7

Balloter8

Balloter9

When the IEEE SA Standards Board approved thistxtTrialUsetxtGorRPorSTD on varApprovedDate, it had the following membership:

[To be supplied by IEEE]

<Name>, Chair

<Name>, Vice Chair

<Name>, Past Chair

Konstantinos Karachalios, Secretary

SBMember1

SBMember2

SBMember3

SBMember4

SBMember5

SBMember6

SBMember7

SBMember8

SBMember9

*Member Emeritus

Introduction

This introduction is not part of PvarDesignation/DvarDraftNumber, DrafttxtTrialUsetxtGorRPorSTD for varTitlePAR.

<Select this text and type or paste introduction text>

Contents

1. Overview	11

1.1 Scope	11

1.2 Purpose	11

1.3 Word usage	11

2. Normative references	12

3. Definitions, acronyms, and abbreviations	12

3.1 Definitions	12

3.2 Acronyms and abbreviations	12

4. Data formats for interfacing sensors	12

4.1 Root schema structure	12

4.2 Common class	15

4.3 Data format for individual sensors	43

4.4 Data format for recognizing sensor characteristics(capabilities?)	78

4.5 Data format to sensors for sensor setting	78

5. Application programming interface for smart sensors	79

5.1 General	79

5.2 API for individual sensors	79

Draft Standard for Specification of Sensor Interface for Cyber and Physical World

- Overview

  - Scope

  - Purpose

  - Word usage

<This subclause is mandatory and shall appear after the Scope and Purpose (if included).>

The word shall indicates mandatory requirements strictly to be followed in order to conform to the standard and from which no deviation is permitted (shall equals is required to).,

The word should indicates that among several possibilities one is recommended as particularly suitable, without mentioning or excluding others; or that a certain course of action is preferred but not necessarily required (should equals is recommended that).

The word may is used to indicate a course of action permissible within the limits of the standard (may equals is permitted to).

The word can is used for statements of possibility and capability, whether material, physical, or causal (can equals is able to).

- Normative references

The following referenced documents are indispensable for the application of this document (i.e., they must be understood and used, so each referenced document is cited in text and its relationship to this document is explained). For dated references, only the edition cited applies. For undated references, the latest edition of the referenced document (including any amendments or corrigenda) applies.

- Definitions, acronyms, and abbreviations

  - Definitions

For the purposes of this document, the following terms and definitions apply. The IEEE Standards Dictionary Online should be consulted for terms not defined in this clause.

  - Acronyms and abbreviations

- Data formats for interfacing sensors

  - Root schema structure

    - General

For the purposes of this document, the following terms and definitions apply. The IEEE Standards

    - Syntax

{ "$schema": "http://json-schema.org/draft-07/schema#", "title": "Sensor data", "description": "Schema for sensor data", "type": "object", "properties": { "timeStamp": {"type": "datetime"}, "sensedInfoBaseAttributes": { "$ref": "#/definitions/sensedInfoBaseAttributes" }, // This is where the properties of each type of sensor data is declared by reference. // For Example, // "microphoneSensorType": { // "$ref": "#/definitions/microphoneSensorType" // }, // "cameraSensorType": { // "$ref": "#/definitions/cameraSensorType " // }, // … }, "additionalProperties": false, "required": [ "sensedInfoBaseAttributes" ], "minProperties": 2, "maxProperties": 3, "definitions": { "sensedInfoBaseAttributes": { "additionalProperties": false, "type": "object", "properties": { "id": {"type": "string"}, "sensorIdRef": {"type": "string"}, "linkedList": {"type": "string"}, "groupID": {"type": "string"}, "activate": {"type": "boolean"}, "priority": {"type": "integer", "minimum": 1} } }, // This is where the properties of each type of sensor data is actually defined. // For Example, // "microphoneSensorType": { // … // }, // "cameraSensorType": { // … // }, // … } }

    - Semantics

Semantics of the root element

| Name | Definition |
| --- | --- |
| sensedInfoBaseAttributes | Describes a group of attributes for the sensed information. |
| timeStamp | Provides the time information at which the sensed information is acquired. |
| id | Unique identifier for identifying individual sensed information |
| sensorIdRef | References a sensor that has generated the information included in this specific sensed information. |
| linkedlist | Describes the multi-sensor structure that consists of a group of sensors in a way that each record contains a reference to the ID of the next sensor. |
| groupID | Identifier for a group multi-sensor structure to which this specific sensor belongs. |
| activate | Describes whether the sensor shall be activated. A value of ”true” means the sensor shall be activated and ”false” means the sensor shall be deactivated. In the binary representation, a value of ”1” means the sensor shall be activated and ”0” means the sensor shall be deactivated. |
| priority | Describes a priority for sensed information with respect to other sensed information sharing the same point in time when the sensed information becomes adapted. A value of one indicates the highest priority and larger values indicate lower priorities. If there is more than one sensed information with the same priority, the order of process can be determined by the Adaptation engine itself. EXAMPLE The adaptation RV processes the individual sensed information of a group of sensors according to their priority in descending order due to its limited capabilities. That is, the sensed information with the lower priority might get lost. |

    - Examples

In this example, a heart rate sensor with its sensor ID “HR-001” measured 98 BPM of heart rate at 18:25:43 23/04/2020. The metadata ID of this example is “HR000000023” and the priority is 1.

{ “timeStamp”: “2020-04-23T18:25:43.511Z”, “sensedInfoBaseAttributes”: { “id”: “HR000000023” “sensorIdRef”: “HR-001” “activate”: true “priority”: 1 }, “heartRateSensorType”: { “value”: 98, } }

  - Common class

    - General

For the purposes of this document, the following terms and definitions apply. The IEEE Standards

    - integer3DVectorType

      - General

Should be added.

      - Syntax

"integer3DVectorType" : { "type": "array", "items": { "type": "integer" }, "minItems": 3, "maxItems": 3 },

      - Semantics

Should be added.

      - Examples

Should be added.

    - float3DVector

      - General

Should be added.

      - Syntax

"float3DVector": { "type": "array", "items": { "type": "number" }, "minItems": 3, "maxItems": 3 },

      - Semantics

Should be added.

      - Examples

Should be added.

    - float4DVector

      - General

Should be added.

      - Syntax

"float4DVector": { "type": "array", "items": { "type": "number" }, "minItems": 4, "maxItems": 4 },

      - Semantics

Should be added.

      - Examples

Should be added.

    - unitType

      - General

This sub-clause specifies a data type, which describes units

      - Syntax

"unitType": { "type": "string", "enum": [ "micrometer", "mm", "cm", "meter", "km", "inch", "yard", "mile”, “mg”, "gram", “kilogram”, “ton”, "meterpersec", “kmperhour”, “hPa”, “newton", “hz", “khz”, “mhz”, “ghz”, "volt", "watt", “ampere”, "lux", "celsius”, "fahrenheit”, "radian”, "degree", "ppm" ] },

      - Semantics

Semantics of the unitType:

| Name | Definition |
| --- | --- |
| micrometer | A unit of length or distance equal to one millionth of a meter. |
| mm | A unit of length or distance equal to one thousandth of a meter. |
| cm | A unit of length or distance equal to one hundredth of a meter. |
| meter | The basic unit of length in International System of Units. |
| km | A unit of length or distance equal to one thousand meter. |
| inch | A unit of length or distance equal to 0.0254 meter. |
| yard | A unit of length or distance equal to 0.9144 meter. |
| mile | A unit of length or distance equal to 1609.344 meter. |
| mg | A unit of mass or weight equal to one millionth of a kilogram. |
| gram | A unit of mass or weight equal to one thousandth of a kilogram. |
| kg | The base unit of mass in International System of Units. |
| ton | A unit of mass or weight equal to a thousand kilogram |
| meterpersec | The SI coherent derived unit of velocity in International System of Units. |
| kmperhour | A unit of velocity equal to a thousand meter per hour. |
| hPa | A unit for pressure, which is equal to 100 pascal. |
| newton | The SI coherent derived unit of force in International System of Units, which is equal to one kilogram-meter per second squared. |
| hz | The derived unit of frequency in International System of Units, which is equal to one over second. |
| khz | A unit of frequency equal to a thousand Hertz. |
| mhz | A unit of frequency equal to a million Hertz. |
| ghz | A unit of frequency equal to a billion Hertz. |
| volt | The SI coherent derived unit of electric potential difference or electromotive force in International System of Units, which is equal to one kilogram-square meter per second cubed-ampere. |
| watt | The SI coherent derived unit of power or radiant flux in International System of Units, which is equal to one kilogram-square meter per second cubed. |
| ampere | The basic unit of electric current in International System of Units. |
| lux | The SI coherent derived unit of illuminance in International System of Units, which is equal to one candela per square meter. |
| celsius | The SI coherent derived unit of Celsius temperature in International System of Units. |
| fahrenheit | A unit of temperature which is equal to a Celius temperature times nine fifths plus thirty-two. |
| radian | The SI coherent derived unit of plane angle in International System of Units. |
| degree | A unit of plane angle which is equal to pi over hundred-eighty radian (pi is a mathematical constant whose value is the ratio of any circle’s circumference to its diameter in Euclidean space) |
| ppm | A unit of a way of quantifying small concentrations, equal to part(s) per million. |

    - colorType

      - General

This sub-clause specifies a data type, which describes the type of colors.

      - Syntax

"colorType": { "type": "string", "enum": [ "alice_blue", "alizarin", "amaranth", "amaranth_pink", "amber", "amethyst", "apricot", "aqua", "aquamarine" "army_green", "asparagus", "atomic_tangerine", "auburn", "azure_color_wheel", "azure_web", "baby_blue", "beige", "bistre", "black", "blue", "blue_pigment", "blue_ryb", "blue-green", "blue-violet", "bondi_blue", "brass", "bright_green" "bright_pink", "bright_turquoise", "brilliant_rose", "brink_pink", "bronze", "brown", "buff", "burgundy", "burnt_orange", "burnt_sienna", "burnt_umber", "camouflage_green", "caput_mortuum", "cardinal", "carmine", "carmine_pink", "carnation_pink", "carolina_blue" "carrot_orange", "celadon", "cerise", "cerise_pink", "cerulean", "cerulean_blue", "champagne", "charcoal", "chartreuse_traditional", "chartreuse_web", "cherry_blossom_pink", "chestnut", "chocolate", "cinnabar", "cinnamon", "cobalt", "columbia_blue", "copper" "copper_rose", "coral", "coral_pink", "coral_red", "corn", "cornflower_blue", "cosmic_latte", "cream", "crimson", "cyan", "cyan_process", "dark_blue", "dark_brown", "dark_cerulean", "dark_chestnut", "dark_coral", "dark_goldenrod", "dark_green" "dark_kahki", "dark_magenta", "dark_pastel_green", "dark_pink", "dark_scarlet", "dark_salmon", "dark_slate_gray", "dark_spring_green", "dark_tan", "dark_turquoise", "dark_violet", "deep_carmine_pink", "deep_cerise", "deep_chestnut", "deep_fuchsia", "deep_lilac", "deep_magenta", "deep_peach" "deep_pink", "denim", "dodger_blue", "ecru", "Egyptian_blue", "electric_blue", "electric_green_x11_green", "electric_indigo", "electric_lime", "electric_purple", "emerald", "eggplant", "falu_red", "fern_green", "firebrick", "flax", "forest_green", "french_rose" "fuchsia", "fuchsia_pink", "gamboge", "gold_metallic", "gold_web_golden", "golden_brown", "golden_yellow", "goldenrod", "grey-asparagus", "green_color_wheel_x11_green", "green_pigment", "green_ryb", "green_yellow", "grey", "han_purple", "harlequin", "heliotrope", "hollywood_cerise" "hot_magenta", "hot_pink", "indigo_dye", "indigo_web", "international_klein_blue", "international_orange", "islamic_green", "ivory", "jade", "kelly_green", "khaki", "khaki_x11_light_khaki", "lavender_floral", "lavender_web", "lavender_blue", "lavender_blush", "lavender_grey", "lavender_magenta" "lavender_pink", "lavender_purple", "lavender_rose", "lawn_green", "lemon", "lemon_chiffon", "light_blue", "light_pink", "lilac", "lime_color_wheel", "lime_web_x11_green", "lime_green", "linen", "magenta", "magenta_dye", "magenta_process", "magic_mint", "magnolia" "malachite", "maroon_html/css", "marron_x11", "maya_blue", "mauve", "mauve_taupe", "medium_blue", "medium_carmine", "medium_lavender_magenta", "medium_purple", "medium_spring_green", "midnight_blue", "midnight_green_eagle_green", "mint_green", "misty_rose", "moss_green", "mountbatten_pink", "mustard" "myrtle", "navajo_white", "navy_blue", "ochre", "office_green", "old_gold", "old_lace", "old_lavender", "old_rose", "olive", "olive_drab", "olivine", "orange_color_wheel", "orange_ryb", "orange_web", "orange-red", "orchid", "pale_blue" "pale_brown", "pale_carmine", "pale_chestnut", "pale_cornflower_blue", "pale_magenta", "pale_pink", "pale_red-violet", "papaya_whip", "pastel_green", "pastel_pink", "peach", "peach-orange", "peach-yellow", "pear", "periwinkle", "persian_blue", "persian_green", "persian_indigo" "persian_orange", "persian_red", "persian_pink", "persian_rose", "persimmon", "pine_green", "pink", "pink-orange", "platinum", "plum_web", "powder_blue_web", "puce", "prussian_blue", "psychedelic_purple", "pumpkin", "purple_html/css", "purple_x11", "purple_taupe" "raw_umber", "razzmatazz", "red", "red_pigment", "red_ryb", "red-violet", "rich_carmine", "robin_egg_blue", "rose", "rose_madder", "rose_taupe", "royal_blue", "royal_purple", "ruby", "russet", "rust", "safety_orange_blaze_orange", "saffron" "salmon", "sandy_brown", "sangria", "sapphire", "scarlet", "school_bus_yellow", "sea_green", "seashell", "selective_yellow", "sepia", "shamrock_green", "shocking_pink", "silver", "sky_blue", "slate_grey", "smalt_dark_powder_blue", "spring_bud", "spring_green" "steel_blue", "tan", "tangerine", "tangerine_yellow", "taupe", "tea_green", "tea_rose_orange", "tea_rose_rose", "teal", "tenné_tawny", "terra_cotta", "thistle", "tomato", "turquoise", "tyrian_purple", "ultramarine", "ultra_pink", "united_nation_blue" "vegas_gold", "vermillion", "violet", "violet_web", "violet_ryb", "viridian", "wheat", "white", "wisteria", "yellow", "yellow_process", "yellow_ryb", "yellow_green", ] }

      - Semantics

Semantics of the colorType:

| Name | Definition |
| --- | --- |
| alice_blue | Describes the color Alice blue. Hex: #F0F8FF; RGB: 240, 248, 255; HSV: 208°, 6%, 100%. |
| alizarin | Describes the color Alizarin. Hex: #E32636; RGB: 227, 38, 54; HSV: 355°, 83%, 89%. |
| amaranth | Describes the color Amaranth. Hex: #E52B50; RGB: 229, 43, 80; HSV: 345°, 78%, 64%. |
| amaranth_pink | Describes the color Amaranth. Hex: #E52B50; RGB: 229, 43, 80; HSV: 345°, 78%, 64%. |
| amber | Describes the color Amber. Hex: #FFBF00; RGB: 255, 191, 0; HSV: 45°, 100%, 100%. |
| amethyst | Describes the color Amethyst. Hex: #9966CC; RGB: 153, 102, 204; HSV: 270°, 50%, 80%. |
| apricot | Describes the color Apricot. Hex: #FBCEB1; RGB: 251, 206, 177; HSV: 30°, 25%, 87%. |
| aqua | Describes the color Aqua. Hex: #00FFFF; RGB: 0, 255, 255; HSV: 180°, 100%, 100%. |
| aquamarine | Describes the color Aquamarine. Hex: #7FFFD4; RGB: 127, 255, 212; HSV: 160°, 50%, 100%. |
| army_green | Describes the color Army green. Hex: #4B5320; RGB: 75, 83, 32; HSV: 46°, 106%, 54%. |
| asparagus | Describes the color Asparagus. Hex: #7BA05B; RGB: 123, 160, 91; HSV: 92°, 43%, 63%. |
| atomic_tangerine | Describes the color Atomic tangerine. Hex: #FF9966; RGB: 255, 153, 102; HSV: 20°, 100%, 75%. |
| auburn | Describes the color Auburn. Hex: #6D351A; RGB: 111, 53, 26; HSV: 7°, 67%, 45%. |
| azure_color_wheel | Describes the color Azure (color wheel). Hex: #007FFF; RGB: 0, 127, 255; HSV: 210°, 100%, 50%. |
| azure_web | Describes the color Azure (web). Hex: #F0FFFF; RGB: 240, 255, 255; HSV: 210°, 100%, 98%. |
| baby_blue | Describes the color Baby blue. Hex: #E0FFFF; RGB: 224, 255, 255; HSV: 180°, 12%, 100%. |
| beige | Describes the color Beige. Hex: #F5F5DC; RGB: 245, 245, 220; HSV: 60°, 10%, 96%. |
| bistre | Describes the color Bistre. Hex: #3D2B1F; RGB: 61, 43, 31; HSV: 24°, 49%, 24%. |
| black | Describes the color Black. Hex: #000000; RGB: 0, 0, 0; HSV: 0°, 0%, 0%. |
| blue | Describes the color Blue. Hex: #0000FF; RGB: 0, 0, 255; HSV: 240°, 100%, 100%. |
| blue_pigment | Describes the color Blue (pigment). Hex: #333399; RGB: 51, 51, 153; HSV: 240°, 50%, 35%. |
| blue_ryb | Describes the color Blue (RYB). Hex: #0247FE; RGB: 2, 71, 254; HSV: 240°, 100%, 82%. |
| blue-green | Describes the color Blue-green. Hex: #00DDDD; RGB: 0, 223, 223; HSV: 180°, 100%, 50%. |
| blue-violet | Describes the color Blue-violet. Hex: #8A2BE2; RGB: 138, 43, 226; HSV: 271°, 81%, 42%. |
| bondi_blue | Describes the color Bondi blue. Hex: #0095B6; RGB: 0, 149, 182; HSV: 191°, 100%, 71%. |
| brass | Describes the color Brass. Hex: #B5A642; RGB: 181, 166, 66; HSV: 37°, 119%, 124%. |
| bright_green | Describes the color Bright green. Hex: #66FF00; RGB: 102, 255, 0; HSV: 96°, 100%, 100%. |
| bright_pink | Describes the color Bright pink. Hex: #FF007F; RGB: 255, 0, 127; HSV: 330°, 100%, 100%. |
| bright_turquoise | Describes the color Bright turquoise. Hex: #08E8DE; RGB: 8, 232, 222; HSV: 177°, 97%, 91%. |
| brilliant_rose | Describes the color Brilliant rose. Hex: #FF55A3; RGB: 255, 85, 163; HSV: 330°, 75%, 84%. |
| brink_pink | Describes the color Brink Pink. Hex: #FB607F; RGB: 251, 96, 127; HSV: 333°, 88%, 80%. |
| bronze | Describes the color Bronze. Hex: #CD7F32; RGB: 205, 127, 50; HSV: 21°, 155%, 128%. |
| brown | Describes the color Brown. Hex: #964B00; RGB: 150, 75, 0; HSV: 30°, 100%, 59%. |
| buff | Describes the color Buff. Hex: #F0DC82; RGB: 240, 220, 130; HSV: 49°, 46%, 94%. |
| burgundy | Describes the color Burgundy. Hex: #900020; RGB: 128, 0, 32; HSV: 345°, 50%, 50%. |
| burnt_orange | Describes the color Burnt orange. Hex: #CC5500; RGB: 204, 85, 0; HSV: 25°, 100%, 80%. |
| burnt_sienna | Describes the color Burnt sienna. Hex: #E97451; RGB: 233, 116, 81; HSV: 14°, 65%, 91%. |
| burnt_umber | Describes the color Burnt umber. Hex: #8A3324; RGB: 138, 51, 36; HSV: 9°, 74%, 54%. |
| camouflage_green | Describes the color Camouflage green. Hex: #78866B; RGB: 120, 134, 107; HSV: 91°, 20%, 53%. |
| caput_mortuum | Describes the color Caput Mortuum. Hex: #592720; RGB: 89, 39, 32; HSV: 7°, 64%, 35%. |
| cardinal | Describes the color Cardinal. Hex: #C41E3A; RGB: 196, 30, 58; HSV: 350°, 85%, 77%. |
| carmine | Describes the color Carmine. Hex: #960018; RGB: 150, 0, 24; HSV: 350°, 100%, 59%. |
| carmine_pink | Describes the color Carmine Pink. Hex: #EB4C42; RGB: 235, 76, 66; HSV: 0°, 75%, 80%. |
| carnation_pink | Describes the color Carnation pink. Hex: #FFA6C9; RGB: 255, 166, 201; HSV: 330°, 100%, 80%. |
| carolina_blue | Describes the color Carolina blue. Hex: #99BADD; RGB: 156, 186, 227; HSV: 210.9°, 30.8%, 86.7%. |
| carrot_orange | Describes the color Carrot orange. Hex: #ED9121; RGB: 237, 145, 33; HSV: 33°, 86%, 93%. |
| celadon | Describes the color Celadon. Hex: #ACE1AF; RGB: 172, 225, 175; HSV: 123°, 24%, 88%. |
| cerise | Describes the color Cerise. Hex: #DE3163; RGB: 222, 49, 99; HSV: 343°, 78%, 87%. |
| cerise_pink | Describes the color Cerise Pink. Hex: #EC3B83; RGB: 236, 59, 131; HSV: 343°, 70%, 89%. |
| cerulean | Describes the color Cerulean. Hex: #007BA7; RGB: 0, 123, 167; HSV: 196°, 100%, 65%. |
| cerulean_blue | Describes the color Cerulean blue. Hex: #2A52BE; RGB: 42, 82, 190; HSV: 224°, 78%, 75%. |
| champagne | Describes the color Champagne. Hex: #F7E7CE; RGB: 247, 231, 206; HSV: 37°, 17%, 97%. |
| charcoal | Describes the color Charcoal. Hex: #464646; RGB: 70, 70, 70; HSV: 170°, 0%, 70%. |
| chartreuse_traditional | Describes the color Chartreuse (traditional). Hex: #DFFF00; RGB: 223, 255, 0; HSV: 67.5°, 100%, 100%. |
| chartreuse_web | Describes the color Chartreuse (web). Hex: #7FFF00; RGB: 127, 255, 0; HSV: 90°, 100%, 100%. |
| cherry_blossom_pink | Describes the color Cherry blossom pink. Hex: #FFB7C5; RGB: 255, 183, 197; HSV: 350°, 100%, 84%. |
| chestnut | Describes the color Chestnut. Hex: #CD5C5C; RGB: 205, 92, 92; HSV: 0°, 55%, 80%. |
| chocolate | Describes the color Chocolate. Hex: #7B3F00; RGB: 123, 63, 0; HSV: 31°, 100%, 48%. |
| cinnabar | Describes the color Cinnabar. Hex: #E34234; RGB: 227, 66, 52; HSV: 5°, 77%, 89%. |
| cinnamon | Describes the color Cinnamon. Hex: #D2691E; RGB: 210, 105, 30; HSV: 25°, 86%, 82%. |
| cobalt | Describes the color Cobalt. Hex: #0047AB; RGB: 0, 71, 171; HSV: 215°, 100%, 67%. |
| columbia_blue | Describes the color Columbia blue. Hex: #9BDDFF; RGB: 155, 221, 255; HSV: 200°, 39%, 100%. |
| copper | Describes the color Copper. Hex: #B87333; RGB: 184, 115, 51; HSV: 29°, 72%, 72%. |
| copper_rose | Describes the color Copper rose. Hex: #996666; RGB: 153, 102, 102; HSV: 344°, 35%, 57%. |
| coral | Describes the color Coral. Hex: #FF7F50; RGB: 255, 127, 80; HSV: 16°, 69%, 100%. |
| coral_pink | Describes the color Coral pink. Hex: #F88379; RGB: 248, 131, 121; HSV: 16°, 69%, 100%. |
| coral_red | Describes the color Coral red. Hex: #FF4040; RGB: 255, 64, 64; HSV: 3°, 82%, 100%. |
| corn | Describes the color Corn. Hex: #FBEC5D; RGB: 251, 236, 93; HSV: 54°, 63%, 98%. |
| cornflower_blue | Describes the color Cornflower blue. Hex: #6495ED; RGB: 100, 149, 237; HSV: 219°, 58%, 93%. |
| cosmic_latte | Describes the color Cosmic latte. Hex: #FFF8E7; RGB: 255, 248, 231; HSV: 40°, 94%, 90%. |
| cream | Describes the color Cream. Hex: #FFFDD0; RGB: 255, 253, 208; HSV: 57°, 18%, 100%. |
| crimson | Describes the color Crimson. Hex: #DC143C; RGB: 220, 20, 60; HSV: 348°, 91%, 86%. |
| cyan | Describes the color Cyan. Hex: #00FFFF; RGB: 0, 255, 255; HSV: 180°, 100%, 100%. |
| cyan_process | Describes the color Cyan (process). Hex: #00B7EB; RGB: 0, 180, 247; HSV: 180°, 100%, 97%. |
| dark_blue | Describes the color Dark blue. Hex: #00008B; RGB: 0, 0, 139; HSV: 240°, 100%, 25%. |
| dark_brown | Describes the color Dark brown. Hex: #654321; RGB: 101, 67, 33; HSV: 30°, 67%, 40%. |
| dark_cerulean | Describes the color Dark cerulean. Hex: #08457E; RGB: 8, 69, 126; HSV: 209°, 94%, 49%. |
| dark_chestnut | Describes the color Dark chestnut. Hex: #986960; RGB: 152, 105, 96; HSV: 10°, 37%, 60%. |
| dark_coral | Describes the color Dark coral. Hex: #CD5B45; RGB: 205, 91, 69; HSV: 10°, 66%, 80%. |
| dark_goldenrod | Describes the color Dark goldenrod. Hex: #B8860B; RGB: 184, 134, 11; HSV: 43°, 94%, 72%. |
| dark_green | Describes the color Dark green. Hex: #013220; RGB: 1, 50, 32; HSV: 158°, 98%, 20%. |
| dark_khaki | Describes the color Dark khaki. Hex: #BDB76B; RGB: 189, 183, 107; HSV: 56°, 43%, 74%. |
| dark_magenta | Describes the color Dark magenta. Hex: #8B008B; RGB: 139, 0, 139; HSV: 300°, 33%, 25%. |
| dark_pastel_green | Describes the color Dark pastel green. Hex: #03C03C; RGB: 3, 192, 60; HSV: 138°, 98%, 75%. |
| dark_pink | Describes the color Dark pink. Hex: #E75480; RGB: 231, 84, 128; HSV: 342°, 64%, 91%. |
| dark_scarlet | Describes the color Dark scarlet. Hex: #560319; RGB: 86, 3, 25; HSV: 8°, 100%, 13%. |
| dark_salmon | Describes the color Dark salmon. Hex: #E9967A; RGB: 233, 150, 122; HSV: 15°, 48%, 91%. |
| dark_slate_gray | Describes the color Dark slate gray. Hex: #2F4F4F; RGB: 47, 79, 79; HSV: 180°, 41%, 31%. |
| dark_spring_green | Describes the color Dark spring green. Hex: #177245; RGB: 23, 114, 69; HSV: 150°, 80%, 45%. |
| dark_tan | Describes the color Dark tan. Hex: #918151; RGB: 145, 129, 81; HSV: 45°, 44%, 57%. |
| dark_turquoise | Describes the color Dark turquoise. Hex: #00CED1; RGB: 0, 206, 209; HSV: 175°, 40%, 94%. |
| dark_violet | Describes the color Dark violet. Hex: #9400D3; RGB: 148, 0, 211; HSV: 282°, 40%, 40%. |
| deep_carmine_pink | Describes the color Deep Carmine Pink. Hex: #EF3038; RGB: 239, 48, 56; HSV: 10°, 80%, 80%. |
| deep_cerise | Describes the color Deep cerise. Hex: #DA3287; RGB: 218, 50, 135; HSV: 317°, 57%, 62%. |
| deep_chestnut | Describes the color Deep chestnut. Hex: #B94E48; RGB: 185, 78, 72; HSV: 0°, 50%, 75%. |
| deep_fuchsia | Describes the color Deep fuchsia. Hex: #C154C1; RGB: 193, 84, 193; HSV: 300°, 67%, 72%. |
| deep_lilac | Describes the color Deep lilac. Hex: #9955BB; RGB: 153, 85, 187; HSV: 270°, 68%, 67%. |
| deep_magenta | Describes the color Deep magenta. Hex: #CD00CC; RGB: 204, 0, 204; HSV: 300°, 80%, 37%. |
| deep_peach | Describes the color Deep peach. Hex: #FFCBA4; RGB: 255, 203, 164; HSV: 40°, 34%, 100%. |
| deep_pink | Describes the color Deep pink. Hex: #FF1493; RGB: 255, 20, 147; HSV: 328°, 92%, 100%. |
| denim | Describes the color Denim. Hex: #1560BD; RGB: 21, 96, 189; HSV: 213°, 89%, 74%. |
| dodger_blue | Describes the color Dodger blue. Hex: #1E90FF; RGB: 30, 144, 255; HSV: 210°, 88%, 100%. |
| ecru | Describes the color Ecru. Hex: #C2B280; RGB: 194, 178, 128; HSV: 39°, 27%, 77%. |
| egyptian_blue | Describes the color Egyptian blue. Hex: #1034A6; RGB: 16, 52, 166; HSV: 244°, 77%, 42%. |
| electric_blue | Describes the color Electric blue. Hex: #7DF9FF; RGB: 125, 249, 255; HSV: 180°, 40%, 90%. |
| electric_green_x11_green | Describes the color Electric green (X11 green). Hex: #00FF00; RGB: 0, 255, 0; HSV: 120°, 100%, 100%. |
| electric_indigo | Describes the color Electric indigo. Hex: #6600FF; RGB: 102, 0, 255; HSV: 264°, 100%, 50%. |
| electric_lime | Describes the color Electric lime. Hex: #CCFF00; RGB: 204, 255, 0; HSV: 75°, 100%, 63%. |
| electric_purple | Describes the color Electric purple. Hex: #BF00FF; RGB: 191, 0, 255; HSV: 285°, 100%, 80%. |
| emerald | Describes the color Emerald. Hex: #50C878; RGB: 80, 200, 120; HSV: 140°, 60%, 78%. |
| eggplant | Describes the color Eggplant. Hex: #614051; RGB: 97, 64, 81; HSV: 320°, 100%, 50%. |
| falu_red | Describes the color Falu red. Hex: #801818; RGB: 128, 24, 24; HSV: 0°, 81%, 50%. |
| fern_green | Describes the color Fern green. Hex: #4F7942; RGB: 79, 121, 66; HSV: 106°, 45%, 47%. |
| firebrick | Describes the color Firebrick. Hex: #B22222; RGB: 178, 34, 34; HSV: 0°, 81%, 70%. |
| flax | Describes the color Flax. Hex: #EEDC82; RGB: 238, 220, 130; HSV: 50°, 45%, 93%. |
| forest_green | Describes the color Forest green. Hex: #228B22; RGB: 34, 139, 34; HSV: 120°, 76%, 55%. |
| french_rose | Describes the color French Rose. Hex: #F64A8A; RGB: 246, 74, 138; HSV: 330°, 76%, 55%. |
| fuchsia | Describes the color Fuchsia. Hex: #FF00FF; RGB: 255, 0, 255; HSV: 300°, 100%, 100%. |
| fuchsia_pink | Describes the color Fuchsia Pink. Hex: #FF77FF; RGB: 255, 119, 255; HSV: 300°, 47%, 84%. |
| gamboge | Describes the color Gamboge. Hex: #E49B0F; RGB: 228, 155, 15; HSV: 38°, 94%, 94%. |
| gold_metallic | Describes the color Gold (metallic). Hex: #D4AF37; RGB: 212, 175, 55; HSV: 51°, 67%, 72%. |
| gold_web_golden | Describes the color Gold (web) (Golden). Hex: #FFD700; RGB: 255, 215, 0; HSV: 51°, 100%, 100%. |
| golden_brown | Describes the color Golden brown. Hex: #996515; RGB: 153, 101, 21; HSV: 51°, 37%, 47%. |
| golden_yellow | Describes the color Golden yellow. Hex: #FFDF00; RGB: 255, 223, 0; HSV: 52.5°, 100%, 100%. |
| goldenrod | Describes the color Goldenrod. Hex: #DAA520; RGB: 218, 165, 32; HSV: 43°, 85%, 85%. |
| grey-asparagus | Describes the color Grey-asparagus. Hex: #465945; RGB: 70, 89, 69; HSV: 117°, 22%, 35%. |
| green_color_wheel_x11_green | Describes the color Green (color wheel) (X11 green). Hex: #00FF00; RGB: 0, 255, 0; HSV: 120°, 100%, 100%. |
| green_html/css_green | Describes the color Green (HTML/CSS green). Hex: #008000; RGB: 0, 128, 0; HSV: 120°, 80%, 50%. |
| green_pigment | Describes the color Green (pigment). Hex: #00A550; RGB: 0, 165, 80; HSV: 125°, 100%, 65%.</ |
| green_ryb | Describes the color Green (RYB). Hex: #66B032; RGB: 102, 176, 50; HSV: 120°, 100%, 57%. |
| green-yellow | Describes the color Green-yellow. Hex: #ADFF2F; RGB: 173, 255, 47; HSV: 84°, 100%, 67%. |
| grey | Describes the color Grey. Hex: #808080; RGB: 128, 128, 128; HSV: 0°, 0%, 50%. |
| han_purple | Describes the color Han Purple. Hex: #5218FA; RGB: 82, 24, 250; HSV: 260°, 97%, 47%. |
| harlequin | Describes the color Harlequin. Hex: #3FFF00; RGB: 63, 255, 0; HSV: 105°, 100%, 100%. |
| heliotrope | Describes the color Heliotrope. Hex: #DF73FF; RGB: 223, 115, 255; HSV: 286°, 55%, 100%. |
| hollywood_cerise | Describes the color Hollywood Cerise. Hex: #F400A1; RGB: 244, 0, 161; HSV: 320°, 100%, 96%. |
| hot_magenta | Describes the color Hot Magenta. Hex: #FF00CC; RGB: 255, 0, 204; HSV: 310°, 57%, 74%. |
| hot_pink | Describes the color Hot Pink. Hex: #FF69B4; RGB: 255, 105, 180; HSV: 330°, 59%, 100%. |
| indigo_dye | Describes the color Indigo (dye). Hex: #00416A; RGB: 0, 65, 106; HSV: 275°, 40%, 40%. |
| indigo_web | Describes the color Indigo (web). Hex: #4B0082; RGB: 75, 0, 130; HSV: 275°, 100%, 27%. |
| international_klein_blue | Describes the color International Klein Blue. Hex: #002FA7; RGB: 0, 47, 167; HSV: 223°, 100%, 65%. |
| international_orange | Describes the color International orange. Hex: #FF4F00; RGB: 255, 79, 0; HSV: 19°, 100%, 100%. |
| islamic_green | Describes the color Islamic green. Hex: #009000; RGB: 0, 153, 0; HSV: 120°, 90%, 60%. |
| ivory | Describes the color Ivory. Hex: #FFFFF0; RGB: 255, 255, 240; HSV: 60°, 5%, 100%. |
| jade | Describes the color Jade. Hex: #00A86B; RGB: 0, 168, 107; HSV: 158°, 100%, 66%. |
| kelly_green | Describes the color Kelly green. Hex: #4CBB17; RGB: 76, 187, 23; HSV: 120°, 48%, 48%. |
| khaki | Describes the color Khaki. Hex: #C3B091; RGB: 195, 176, 145; HSV: 37°, 26%, 76%. |
| khaki_x11_light_khaki | Describes the color Khaki (X11) (Light khaki). Hex: #F0E68C; RGB: 240, 230, 140; HSV: 54°, 41%, 94%. |
| lavender_floral | Describes the color Lavender (floral). Hex: #B57EDC; RGB: 181, 126, 220; HSV: 275°, 43%, 86%. |
| lavender_web | Describes the color Lavender (web). Hex: #E6E6FA; RGB: 230, 230, 250; HSV: 245°, 8%, 98%. |
| lavender_blue | Describes the color Lavender blue. Hex: #CCCCFF; RGB: 204, 204, 255; HSV: 240°, 20%, 100%. |
| lavender_blush | Describes the color Lavender blush. Hex: #FFF0F5; RGB: 255, 240, 245; HSV: 340°, 6%, 100%. |
| lavender_grey | Describes the color Lavender grey. Hex: #C4C3D0; RGB: 196, 195, 221; HSV: 245°, 6%, 82%. |
| lavender_magenta | Describes the color Lavender magenta. Hex: #EE82EE; RGB: 238, 130, 238; HSV: 300°, 45%, 93%. |
| lavender_pink | Describes the color Lavender pink. Hex: #FBAED2; RGB: 251, 174, 210; HSV: 332°, 31%, 98%. |
| lavender_purple | Describes the color Lavender purple. Hex: #967BB6; RGB: 150, 120, 182; HSV: 270°, 60%, 65%. |
| lavender_rose | Describes the color Lavender rose. Hex: #FBA0E3; RGB: 251, 160, 227; HSV: 310°, 57%, 90%. |
| lawn_green | Describes the color Lawn green. Hex: #7CFC00; RGB: 124, 252, 0; HSV: 90°, 98%, 48%. |
| lemon | Describes the color Lemon. Hex: #FDE910; RGB: 253, 233, 16; HSV: 55°, 94%, 99%. |
| lemon_chiffon | Describes the color Lemon chiffon. Hex: #FFFACD; RGB: 255, 250, 205; HSV: 54°, 20%, 100%. |
| light_blue | Describes the color Light blue. Hex: #ADD8E6; RGB: 173, 216, 230; HSV: 240°, 90%, 80%. |
| light_pink | Describes the color Light pink. Hex: #FFB6C1; RGB: 255, 182, 193; HSV: 351°, 100%, 86%. |
| lilac | Describes the color Lilac. Hex: #C8A2C8; RGB: 200, 162, 200; HSV: 300°, 19%, 78%. |
| lime_color_wheel | Describes the color Lime (color wheel). Hex: #BFFF00; RGB: 191, 255, 0; HSV: 75°, 100%, 100%. |
| lime_web_x11_green | Describes the color Lime (web) (X11 green). Hex: #00FF00; RGB: 0, 255, 0; HSV: 120°, 100%, 100%. |
| lime_green | Describes the color Lime green. Hex: #32CD32; RGB: 50, 205, 50; HSV: 120°, 67%, 40%. |
| linen | Describes the color Linen. Hex: #FAF0E6; RGB: 250, 240, 230; HSV: 30°, 8%, 98%. |
| magenta | Describes the color Magenta. Hex: #FF00FF; RGB: 255, 0, 255; HSV: 300°, 100%, 100%. |
| magenta_dye | Describes the color Magenta (dye). Hex: #CA1F7B; RGB: 202, 31, 23; HSV: 327°, 96%, 34%. |
| magenta_process | Describes the color Magenta (process). Hex: #FF0090; RGB: 255, 0, 144; HSV: 320°, 100%, 100%. |
| magic_mint | Describes the color Magic mint. Hex: #AAF0D1; RGB: 170, 240, 209; HSV: 150°, 84%, 80%. |
| magnolia | Describes the color Magnolia. Hex: #F8F4FF; RGB: 248, 244, 255; HSV: 247°, 94%, 92%. |
| malachite | Describes the color Malachite. Hex: #0BDA51; RGB: 11, 218, 81; HSV: 140°, 95%, 85%. |
| maroon_html/css | Describes the color Maroon (HTML/CSS). Hex: #800000; RGB: 128, 0, 0; HSV: 0°, 100%, 50%. |
| maroon_x11 | Describes the color Maroon (X11). Hex: #B03060; RGB: 176, 48, 96; HSV: 333°, 65%, 42%. |
| maya_blue | Describes the color Maya blue. Hex: #73C2FB; RGB: 115, 194, 251; HSV: 210°, 96%, 87%. |
| mauve | Describes the color Mauve. Hex: #E0B0FF; RGB: 224, 176, 255; HSV: 276°, 31%, 100%. |
| mauve_taupe | Describes the color Mauve Taupe. Hex: #915F6D; RGB: 145, 95, 109; HSV: 285°, 37%, 54%. |
| medium_blue | Describes the color Medium blue. Hex: #0000CD; RGB: 0, 0, 205; HSV: 240°, 100%, 40%. |
| medium_carmine | Describes the color Medium carmine. Hex: #AF4035; RGB: 175, 64, 53; HSV: 5°, 69%, 68%. |
| medium_lavender_magenta | Describes the color Medium lavender magenta. Hex: #CC99CC; RGB: 204, 153, 204; HSV: 300°, 80%, 25%. |
| medium_purple | Describes the color Medium purple. Hex: #9370DB; RGB: 147, 112, 219; HSV: 270°, 68%, 72%. |
| medium_spring_green | Describes the color Medium spring green. Hex: #00FA9A; RGB: 0, 250, 154; HSV: 150°, 97%, 97%. |
| midnight_blue | Describes the color Midnight Blue. Hex: #191970; RGB: 25, 25, 112; HSV: 240°, 78%, 44%. |
| midnight_green_eagle_green | Describes the color Midnight Green (Eagle Green). Hex: #004953; RGB: 0, 73, 83; HSV: 187°, 100%, 33%. |
| mint_green | Describes the color Mint green. Hex: #98FF98; RGB: 152, 255, 152; HSV: 140°, 40%, 100%. |
| misty_rose | Describes the color Misty rose. Hex: #FFE4E1; RGB: 255, 228, 225; HSV: 337°, 37%, 94%. |
| moss_green | Describes the color Moss green. Hex: #ADDFAD; RGB: 173, 223, 173; HSV: 120°, 22%, 87%. |
| mountbatten_pink | Describes the color Mountbatten pink. Hex: #997A8D; RGB: 153, 122, 141; HSV: 323°, 20%, 60%. |
| mustard | Describes the color Mustard. Hex: #FFDB58; RGB: 255, 219, 88; HSV: 47°, 65%, 100%. |
| myrtle | Describes the color Myrtle. Hex: #21421E; RGB: 33, 66, 30; HSV: 115°, 54%, 26%. |
| navajo_white | Describes the color Navajo white. Hex: #FFDEAD; RGB: 255, 222, 173; HSV: 32°, 27%, 100%. |
| navy_blue | Describes the color Navy Blue. Hex: #000080; RGB: 0, 0, 128; HSV: 240°, 100%, 50%. |
| ochre | Describes the color Ochre. Hex: #CC7722; RGB: 204, 119, 34; HSV: 30°, 83%, 80%. |
| office_green | Describes the color Office green. Hex: #008000; RGB: 0, 128, 0; HSV: 120°, 80%, 50%. |
| old_gold | Describes the color Old Gold. Hex: #CFB53B; RGB: 207, 181, 59; HSV: 49°, 71%, 81%. |
| old_lace | Describes the color Old Lace. Hex: #FDF5E6; RGB: 253, 245, 230; HSV: 40°, 6%, 100%. |
| old_lavender | Describes the color Old Lavender. Hex: #796878; RGB: 121, 104, 120; HSV: 270°, 3%, 22%. |
| old_rose | Describes the color Old Rose. Hex: #C08081; RGB: 192, 128, 129; HSV: 330°, 59%, 57%. |
| olive | Describes the color Olive. Hex: #808000; RGB: 128, 128, 0; HSV: 60°, 100%, 50%. |
| olive_drab | Describes the color Olive Drab. Hex: #6B8E23; RGB: 107, 142, 35; HSV: 80°, 75%, 56%. |
| olivine | Describes the color Olivine. Hex: #9AB973; RGB: 154, 185, 115; HSV: 58°, 80%, 141%. |
| orange_color_wheel | Describes the color Orange (color wheel). Hex: #FF7F00; RGB: 255, 127, 0; HSV: 30°, 100%, 100%. |
| orange_ryb | Describes the color Orange (RYB). Hex: #FB9902; RGB: 251, 153, 2; HSV: 60°, 100%, 73%. |
| orange_web | Describes the color Orange (web). Hex: #FFA500; RGB: 255, 165, 0; HSV: 39°, 100%, 100%. |
| orange_peel | Describes the color Orange Peel. Hex: #FFA000; RGB: 255, 160, 0; HSV: 38°, 100%, 100%. |
| orange-red | Describes the color Orange-Red. Hex: #FF4500; RGB: 255, 69, 0; HSV: 5°, 100%, 52%. |
| orchid | Describes the color Orchid. Hex: #DA70D6; RGB: 218, 112, 214; HSV: 302°, 49%, 85%. |
| pale_blue | Describes the color Pale blue. Hex: #AFEEEE; RGB: 175, 238, 238; HSV: 180°, 26%, 93%. |
| pale_brown | Describes the color Pale brown. Hex: #987654; RGB: 152, 118, 84; HSV: 30°, 45%, 60%. |
| pale_carmine | Describes the color Pale carmine. Hex: #AF4035; RGB: 175, 64, 53; HSV: 5°, 69%, 68%. |
| pale_chestnut | Describes the color Pale chestnut. Hex: #DDADAF; RGB: 221, 173, 175; HSV: 358°, 22%, 87%. |
| pale_cornflower_blue | Describes the color Pale cornflower blue. Hex: #ABCDEF; RGB: 171, 205, 239; HSV: 210°, 28%, 94%. |
| pale_magenta | Describes the color Pale magenta. Hex: #F984E5; RGB: 249, 132, 229; HSV: 310°, 47%, 98%. |
| pale_pink | Describes the color Pale pink. Hex: #FADADD; RGB: 250, 218, 221; HSV: 354°, 13%, 98%. |
| pale_red-violet | Describes the color Pale red-violet. Hex: #DB7093; RGB: 219, 112, 147; HSV: 340°, 49%, 86%. |
| papaya_whip | Describes the color Papaya whip. Hex: #FFEFD5; RGB: 255, 239, 213; HSV: 37°, 16%, 100%. |
| pastel_green | Describes the color Pastel green. Hex: #77DD77; RGB: 119, 221, 119; HSV: 120°, 46%, 87%. |
| pastel_pink | Describes the color Pastel pink. Hex: #FFD1DC; RGB: 255, 209, 220; HSV: 346°, 18%, 100%. |
| peach | Describes the color Peach. Hex: #FFE5B4; RGB: 255, 229, 180; HSV: 39°, 29%, 100%. |
| peach-orange | Describes the color Peach-orange. Hex: #FFCC99; RGB: 255, 204, 153; HSV: 30°, 40%, 100%. |
| peach-yellow | Describes the color Peach-yellow. Hex: #FADFAD; RGB: 250, 223, 173; HSV: 39°, 31%, 98%. |
| pear | Describes the color Pear. Hex: #D1E231; RGB: 209, 226, 49; HSV: 66°, 78%, 89%. |
| periwinkle | Describes the color Periwinkle. Hex: #CCCCFF; RGB: 204, 204, 255; HSV: 240°, 20%, 100%. |
| persian_blue | Describes the color Persian blue. Hex: #1C39BB; RGB: 28, 57, 187; HSV: 248°, 75%, 50%. |
| persian_green | Describes the color Persian green. Hex: #00A693; RGB: 0, 166, 147; HSV: 135°, 75%, 60%. |
| persian_indigo | Describes the color Persian indigo. Hex: #32127A; RGB: 50, 18, 122; HSV: 249°, 85%, 49%. |
| persian_orange | Describes the color Persian orange. Hex: #D99058; RGB: 217, 144, 88; HSV: 26°, 59%, 85%. |
| persian_red | Describes the color Persian red. Hex: #CC3333; RGB: 204, 51, 51; HSV: 5°, 50%, 50%. |
| persian_pink | Describes the color Persian pink. Hex: #F77FBE; RGB: 247, 127, 190; HSV: 330°, 72%, 77%. |
| persian_rose | Describes the color Persian rose. Hex: #FE28A2; RGB: 254, 40, 162; HSV: 318°, 96%, 88%. |
| persimmon | Describes the color Persimmon. Hex: #EC5800; RGB: 236, 88, 0; HSV: 10°, 85%, 94%. |
| pine_green | Describes the color Pine Green. Hex: #01796F; RGB: 1, 121, 111; HSV: 175°, 99%, 47%. |
| pink | Describes the color Pink. Hex: #FFC0CB; RGB: 255, 192, 203; HSV: 350°, 25%, 100%. |
| pink-orange | Describes the color Pink-orange. Hex: #FF9966; RGB: 255, 153, 102; HSV: 20°, 60%, 100%. |
| platinum | Describes the color Platinum. Hex: #E5E4E2; RGB: 229, 228, 226; HSV: 40°, 1%, 90%. |
| plum_web | Describes the color Plum (web). Hex: #CC99CC; RGB: 204, 153, 204; HSV: 300°, 80%, 25%. |
| powder_blue_web | Describes the color Powder blue (web). Hex: #B0E0E6; RGB: 176, 224, 230; HSV: 220°, 70%, 90%. |
| puce | Describes the color Puce. Hex: #CC8899; RGB: 204, 136, 153; HSV: 345°, 33%, 80%. |
| prussian_blue | Describes the color Prussian blue. Hex: #003153; RGB: 0, 49, 83; HSV: 205°, 100%, 33%. |
| psychedelic_purple | Describes the color Psychedelic purple. Hex: #DD00FF; RGB: 221, 0, 255; HSV: 290°, 100%, 92%. |
| pumpkin | Describes the color Pumpkin. Hex: #FF7518; RGB: 255, 117, 24; HSV: 24°, 90%, 100%. |
| purple_html/css | Describes the color Purple (HTML/CSS). Hex: #800080; RGB: 128, 0, 128; HSV: 300°, 67%, 44%. |
| purple_x11 | Describes the color Purple (X11). Hex: #A020F0; RGB: 160, 92, 240; HSV: 285°, 97%, 77%. |
| purple_taupe | Describes the color Purple Taupe. Hex: #50404D; RGB: 80, 64, 77; HSV: 285°, 19%, 33%. |
| raw_umber | Describes the color Raw umber. Hex: #734A12; RGB: 115, 74, 18; HSV: 34°, 84%, 45%. |
| razzmatazz | Describes the color Razzmatazz. Hex: #E30B5C; RGB: 227, 11, 92; HSV: 339°, 100%, 70%. |
| red | Describes the color Red. Hex: #FF0000; RGB: 255, 0, 0; HSV: 0°, 100%, 100%. |
| red_pigment | Describes the color Red (pigment). Hex: #ED1C24; RGB: 237, 28, 36; HSV: 0°, 100%, 65%. |
| red_ryb | Describes the color Red (RYB). Hex: #FE2712; RGB: 254, 39, 18; HSV: 0°, 100%, 87%. |
| red-violet | Describes the color Red-violet. Hex: #C71585; RGB: 199, 21, 133; HSV: 322°, 89%, 78%. |
| rich_carmine | Describes the color Rich carmine. Hex: #D70040; RGB: 215, 0, 64; HSV: 356°, 94%, 44%. |
| robin_egg_blue | Describes the color Robin egg blue. Hex: #00CCCC; RGB: 0, 204, 204; HSV: 180°, 100%, 80%. |
| rose | Describes the color Rose. Hex: #FF007F; RGB: 255, 0, 127; HSV: 330°, 100%, 100%. |
| rose_madder | Describes the color Rose Madder. Hex: #E32636; RGB: 227, 38, 54; HSV: 355°, 83%, 89%. |
| rose_taupe | Describes the color Rose Taupe. Hex: #905D5D; RGB: 144, 93, 93; HSV: 330°, 42%, 46%. |
| royal_blue | Describes the color Royal blue. Hex: #4169E1; RGB: 65, 105, 225; HSV: 225°, 71%, 88%. |
| royal_purple | Describes the color Royal purple. Hex: #6B3FA0; RGB: 107, 63, 160; HSV: 273°, 62%, 54%. |
| ruby | Describes the color Ruby. Hex: #E0115F; RGB: 224, 17, 95; HSV: 338°, 100%, 40%. |
| russet | Describes the color Russet. Hex: #80461B; RGB: 128, 70, 27; HSV: 25°, 78%, 50%. |
| rust | Describes the color Rust. Hex: #B7410E; RGB: 183, 65, 14; HSV: 18°, 92%, 72%. |
| safety_orange_blaze_orange | Describes the color Safety orange (blaze orange). Hex: #FF6600; RGB: 255, 102, 0; HSV: 24°, 100%, 100%. |
| saffron | Describes the color Saffron. Hex: #F4C430; RGB: 244, 196, 48; HSV: 45°, 80%, 96%. |
| salmon | Describes the color Salmon. Hex: #FF8C69; RGB: 255, 140, 105; HSV: 14°, 59%, 100%. |
| sandy_brown | Describes the color Sandy brown. Hex: #F4A460; RGB: 244, 164, 96; HSV: 28°, 61%, 96%. |
| sangria | Describes the color Sangria. Hex: #92000A; RGB: 146, 0, 10; HSV: 356°, 100%, 57%. |
| sapphire | Describes the color Sapphire. Hex: #082567; RGB: 8, 37, 103; HSV: 222°, 92%, 40%. |
| scarlet | Describes the color Scarlet. Hex: #FF2400; RGB: 255, 36, 0; HSV: 8°, 100%, 100%. |
| school_bus_yellow | Describes the color School bus yellow. Hex: #FFD800; RGB: 255, 216, 0; HSV: 36°, 100%, 50%. |
| sea_green | Describes the color Sea Green. Hex: #2E8B57; RGB: 46, 139, 87; HSV: 146°, 77%, 55%. |
| seashell | Describes the color Seashell. Hex: #FFF5EE; RGB: 255, 245, 238; HSV: 25°, 7%, 100%. |
| selective_yellow | Describes the color Selective yellow. Hex: #FFBA00; RGB: 255, 186, 0; HSV: 44°, 100%, 100%. |
| sepia | Describes the color Sepia. Hex: #704214; RGB: 112, 66, 20; HSV: 30°, 82%, 44%. |
| shamrock_green | Describes the color Shamrock green. Hex: #009E60; RGB: 0, 158, 96; HSV: 120°, 91%, 75%. |
| shocking_pink | Describes the color Shocking Pink. Hex: #FC0FC0; RGB: 252, 15, 192; HSV: 315°, 94%, 99%. |
| silver | Describes the color Silver. Hex: #C0C0C0; RGB: 192, 192, 192; HSV: 0°, 0%, 75%. |
| sky_blue | Describes the color Sky Blue. Hex: #87CEEB; RGB: 135, 206, 235; HSV: 210°, 67%, 96%. |
| slate_grey | Describes the color Slate grey. Hex: #708090; RGB: 112, 128, 144; HSV: 210°, 33%, 56%. |
| smalt_dark_powder_blue | Describes the color Smalt (Dark powder blue). Hex: #003399; RGB: 0, 51, 153; HSV: 200°, 70%, 60%. |
| spring_bud | Describes the color Spring bud. Hex: #A7FC00; RGB: 167, 252, 0; HSV: 88°, 90%, 63%. |
| spring_green | Describes the color Spring green. Hex: #00FF7F; RGB: 0, 255, 127; HSV: 150°, 100%, 100%. |
| steel_blue | Describes the color Steel blue. Hex: #4682B4; RGB: 70, 130, 180; HSV: 207°, 61%, 71%. |
| tan | Describes the color Tan. Hex: #D2B48C; RGB: 210, 180, 140; HSV: 34°, 33%, 82%. |
| tangerine | Describes the color Tangerine. Hex: #F28500; RGB: 242, 133, 0; HSV: 33°, 100%, 95%. |
| tangerine_yellow | Describes the color Tangerine yellow. Hex: #FFCC00; RGB: 255, 204, 0; HSV: 33°, 100%, 50%. |
| taupe | Describes the color Taupe. Hex: #483C32; RGB: 72, 60, 50; HSV: 30°, 17%, 34%. |
| tea_green | Describes the color Tea Green. Hex: #D0F0C0; RGB: 208, 240, 192; HSV: 100°, 20%, 94%. |
| tea_rose_orange | Describes the color Tea rose (orange). Hex: #F88379; RGB: 248, 131, 194; HSV: 16°, 70%, 70%. |
| tea_rose_rose | Describes the color Tea rose (rose). Hex: #F4C2C2; RGB: 244, 194, 194; HSV: 337°, 47%, 93%. |
| teal | Describes the color Teal. Hex: #008080; RGB: 0, 128, 128; HSV: 180°, 100%, 50%. |
| tenné_tawny | Describes the color Tenné (Tawny). Hex: #CD5700; RGB: 205, 87, 0; HSV: 25°, 100%, 80%. |
| terra_cotta | Describes the color Terra cotta. Hex: #E2725B; RGB: 226, 114, 91; HSV: 10°, 70%, 62%. |
| thistle | Describes the color Thistle. Hex: #D8BFD8; RGB: 216, 191, 216; HSV: 300°, 12%, 85%. |
| tomato | Describes the color Tomato. Hex: #FF6347; RGB: 255, 99, 71; HSV: 15°, 75%, 50%. |
| turquoise | Describes the color Turquoise. Hex: #30D5C8; RGB: 48, 213, 200; HSV: 175°, 77%, 84%. |
| tyrian_purple | Describes the color Tyrian purple. Hex: #66023C; RGB: 102, 2, 60; HSV: 277°, 67%, 44%. |
| ultramarine | Describes the color Ultramarine. Hex: #120A8F; RGB: 18 , 10 , 143; HSV: 244°, 93%, 56%. |
| ultra_pink | Describes the color Ultra pink. Hex: #FF6FFF; RGB: 255, 111, 255; HSV: 300°, 48%, 83%. |
| united_nations_blue | Describes the color United Nations blue. Hex: #5B92E5; RGB: 91, 146, 229; HSV: 210°, 60%, 90%. |
| vegas_gold | Describes the color Vegas Gold. Hex: #C5B358; RGB: 197, 179, 88; HSV: 50°, 55%, 77%. |
| vermilion | Describes the color Vermilion. Hex: #E34234; RGB: 227, 66, 51; HSV: 5°, 77%, 89%. |
| violet | Describes the color Violet. Hex: #8B00FF; RGB: 139, 0, 255; HSV: 273°, 100%, 100%. |
| violet_web | Describes the color Violet (web). Hex: #EE82EE; RGB: 238, 130, 238; HSV: 300°, 45%, 93%. |
| violet_ryb | Describes the color Violet (RYB). Hex: #8601AF; RGB: 2, 71, 54; HSV: 270°, 100%, 71%. |
| viridian | Describes the color Viridian. Hex: #40826D; RGB: 64, 130, 109; HSV: 161°, 51%, 51%. |
| wheat | Describes the color Wheat. Hex: #F5DEB3; RGB: 245, 222, 179; HSV: 39°, 26%, 96%. |
| white | Describes the color White. Hex: #FFFFFF; RGB: 255, 255, 255; HSV: –––°, 0%, 100%. |
| wisteria | Describes the color Wisteria. Hex: #C9A0DC; RGB: 201, 160, 220; HSV: 281°, 27%, 86%. |
| yellow | Describes the color Yellow. Hex: #FFFF00; RGB: 255, 255, 0; HSV: 60°, 100%, 100%. |
| yellow_process | Describes the color Yellow (process). Hex: #FFEF00; RGB: 255, 239, 0; HSV: 56°, 100%, 100%. |
| yellow_ryb | Describes the color Yellow (RYB). Hex: #FEFE33; RGB: 254, 254, 51; HSV: 60°, 99%, 60%. |
| yellow-green | Describes the color Yellow-green. Hex: #9ACD32; RGB: 154, 205, 50; HSV: 60°, 60%, 54%. |

    - gasType

      - General

This sub-clause specifies a data type, which describes the type of gases.

      - Syntax

"gasType": { "type": "string", "enum": [ "CO", "CO2", "H2SO4", "NO", "O2", "O3", "H2", "VOC", "ETH", "Propane", "METH", "butane", "Form", "Rn" ] }

      - Semantics

Semantics of the gasType:

| Name | Definition |
| --- | --- |
| CO | Carbon monoxide |
| CO2 | Carbon dioxide |
| H2SO4 | Sulfurous acid |
| NO | Nitrogen Oxide |
| O2 | Oxygen |
| O3 | Ozone |
| H2 | Hydrogen |
| VOC | Volatile organic compunds |
| ETH | Ethanol |
| Propane | Propane |
| METH | Methane |
| Butane | Butane |
| Form | Formaldehyde |
| Rn | Radon-222 |

  - Data format for individual sensors

    - General

For the purposes of this document, the following terms and definitions apply. The IEEE Standards

    - Audio-Visual Sensors

      - General

For the purposes of this document, the following terms and definitions apply. The IEEE Standards

      - Microphone Sensor

        - General

This sub-clause specifies a sensor data type, which describes an audio information captured from a microphone sensor.

        - Syntax

"microphoneSensorType": { "type": "object", "properties": { "microphoneOrientation": { "$ref": "#/definitions/orientationSensorType" }, "microphoneLocation": { "$ref": "#/definitions/globalPositionSensorType " }, "microphoneAltitude": { "$ref": "#/definitions/altitudeSensorType" }, "audioData": { "$ref": "#/definitions/rawAudioType" } }, "additionalProperties": false }, "rawAudioType": { "type": "object", "properties": { "sample_rate": { "$ref": "#/definitions/sampleRateType" }, "byte_order": { "$ref": "#/definitions/byteOrderType" }, "sign": { "$ref": "#/definitions/signType" }, "resolution": { "$ref": "#/definitions/resolutionType" } }, "additionalProperties": false }, "sampleRateType": { "type": "number", "enum": [ 8.0, 11.025, 16.0, 22.05, 32.0, 44.056, 44.1, 47.25, 48.0, 50.0, 50.4, 88.2, 96.0, 176.4, 192.0, 352.8, 2822.4, 5644.8 ] }, "byteOrderType": { "type": "string", "enum": [ "littleEndian", "bigEndian" ] }, "signType": { "type": "string", "enum": [ "signed", "unsigned" ] }, "resolutionType": { "type": "integer", "enum": [ 4, 8, 12, 16, 20, 24, 32, 48, 64 ] },

        - Semantics

Semantics of the microphoneSensorType:

| Name | Definition |
| --- | --- |
| microphoneSensorType | Tool for describing sensed information with respect to a microphone sensor. |
| microphoneLocation | Describes the location of a microphone using the structure defined by globalPositionSensorType. |
| microphoneOrientation | Describes the orientation of a microphone using the structure defined by orientationSensorType. |
| microphoneAltitude | Describes the altitude of a microphone using the structure defined by altitudeSensorType. |
| audioData | Describes audio data refer to rawAudioType. |
| sample_rate | Sample rate is the number of samples of audio carried per second, measured in Hz or kHz (one kHz being 1 000 Hz). For example, 44 100 samples per second can be expressed as either 44 100 Hz, or 44.1 kHz. Bandwidth is the difference between the highest and lowest frequencies carried in an audio stream. |
| byte_order | It tells how the data is stored with the most significant byte on one end or the other. When more than one byte is used to represent a PCM sample, the byte order (big endian vs. little endian) must be known. Due to the widespread use of little-endian Intel CPUs, little-endian PCM tends to be the most common byte orientation. |
| sign | It is not enough to know that a PCM sample is, for example, 8 bits wide. Whether the sample is signed or unsigned is needed to understand the range. If the sample is unsigned, the sample range is 0...255 with a center point of 128. If the sample is signed, the sample range is -128...127 with a center point of 0. If a PCM type is signed, the sign encoding is almost always 2's complement. In very rare cases, signed PCM audio is represented as a series of sign/magnitude coded numbers |
| resolution | This parameter specifies the amount of data used to represent each discrete amplitude sample. The most common values are 8 bits (1 byte), which gives a range of 256 amplitude steps, or 16 bits (2 bytes), which gives a range of 65536 amplitude steps. Other sizes, such as 12, 20, and 24 bits, are occasionally seen. Some king-sized formats even opt for 32 and 64 bits per sample |
| Signed | Specifies that the raw audio data coming from the microphone sensor is stored as signed numbers. |
| Unsigned | Specifies that the raw audio data coming from the microphone sensor is stored as unsigned numbers. |
| bigEndian | It specifies that the audio data is stored in the Big Endian format: the most significant byte of a word in the smallest address and the least significant byte is stored in the largest address |
| littleEndian | It specifies that the audio data is stored in the Little Endian format: the least significant byte in the smallest address. |

        - Examples

In this example, the orientation has (60, 30, 120), the location is (23.215, 33.971), and altitude is 122.2 meter. The sampling rate is 44.1kHz and the byte order follows the little-endian method. It is signed data with a resolution 8 bits.

{ “sensedInfoBaseAttributes”: {}, “microphoneSensorType”: { “microphoneOrientation”: { “orientation”: [60, 30, 120] }, “microphoneLocation”: { “longitude”: 23.215, “latitude”: 33.971 }, “microphoneAltitude”: { “altitude”: 122.2, “unit”: “meter” } “audioData”: { “sample_rate”: 44.1, “byte_order”: “littleEndian”, “sign”: “signed”, “resolution”: 8 } } }

      - Color Camera Sensor

        - General

This sub-clause specifies a sensor data type, which describes the color of the captured objects in the environment, encoded as a sequence of pixel values.

        - Syntax

"colorCameraSensorType": { "type": "object", "properties": { "cameraSensor": { "$ref": "#/definitions/cameraSensorType" }, "rawVideo": { "$ref": "#/definitions/rawVideoType" } } }, "rawVideoType": { "type": "object", "properties": { "videoData16": { "type": "string" }, "videoData64": { "type": "string" }, "width": { "type": "integer" }, "height": { "type": "integer" }, "bit_depth": { "type": "integer" }, "stride": { "type": "integer" }, "coding4CC": { "type": "integer" }, "fps": { "type": "integer" }, "use_frame_packing": { "type": "boolean" }, "frame_packing": { "type": "integer" } } }, "cameraSensorType": { "type": "object", "properties": { "cameraOrientation": { "$ref": "#/definitions/orientationSensorType" }, "cameraLocation": { "$ref": "#/definitions/globalPositionSensorType" }, "cameraAltitude": { "$ref": "#/definitions/altitudeSensorType" }, "focalLength": { "type": "number" }, "aperture": { "type": "number" }, "shutterSpeed": { "type": "number" }, "filter": { "type": "string", "enum": [ "UV", "Polarizing", "NB", "Diffusion", "Star" ] } }, "additionalProperties": false },

        - Semantics

Semantics of the colorCameraSensorType:

| Name | Definition |
| --- | --- |
| colorCameraSensorType | Tool for describing sensed information with respect to a color camera sensor. |
| rawVideo | Describes the sensed raw video data by the color camera sensor. |
| rawVideoType | Tool for describing raw video data for color camera sensor. |
| width | Width of the video in number of pixels. |
| height | Height of the video in number of pixels. |
| bit_depth | Number of bits for each channel sample from the set of permitted values as defined by coding4CC. |
| stride | Size in bytes of one horizontal line. |
| coding4CC | A 4 character code representing the parameters of the raw data as specified by MP4RA. |
| fps | Frames per second of the video stream; if 0 then the frame rate is not known or variable. |
| use_frame_packing | This indicates if a frame contains two or more views. |
| frame_packing | Frame Packing as Coding Independent Code Points. |
| videoData16 | Holds binary video data encoded as a textual string in base-16 format. |
| videoData64 | Holds binary video data encoded as a textual string in base-64 format. |
| rawVideoDataSize | Describes the size of the rawVideoData in bytes. This field is only present in binary representation. |
| rawVideoData | Actual data holder for binary raw video data, only in binary representation. The size of this field is given in rawVideoDataSize field. |
| cameraSensorType | Tool for describing sensed information with respect to a camera sensor. |
| cameraLocation | Describes the location of a camera using the structure defined by globalPositionSensorType. |
| cameraOrientation | Describes the orientation of a camera using the structure defined by orientationSensorType. |
| cameraAltitude | Describes the altitude of a camera using the structure defined by altitudeSensorType. |
| focalLength | Describes the distance between the lens and the image sensor when the subject is in focus, in terms of millimeters (mm). |
| aperture | Describes the diameter of the lens opening. It is expressed as F-stop, e.g. F2.8. It may also be expressed as f-number notation such as f/2.8. |
| shutterSpeed | Describes the time that the shutter remains open when taking a photograph in terms of seconds (sec). |
| filter | Describes kinds of camera filters. |

        - Examples

In this example, the camera orientation values are yaw, pitch, and roll values of 60, 30, and 120, respectively. The camera altitude is 123.21 meter and the camera location has a longitude of 23.215 and a latitude of 33.971. The flocal length is 55mm and the aperture is f/2.8. The shutter speed is 0.008sec and the filter indicates that a UV filter is used.

The videoData16 is base-16 encoded data starting with "0314BA3827CFF2938...". The width and height of the image are 640 and 480 pixels, respectively. The bit_depth is 8 and the stride is 10. The coding4CC is 2 and the fps of the image is 30 frames/sec. The use_frame_packing is “true” and the frame_packing is 10.

{ “sensedInfoBaseAttributes”: {}, “colorCameraSensorType”: { “cameraSensor”: { “cameraOrientation”: { “orientation”: [60, 30, 120] }, “cameraLocation”: { “longitude”: 23.215, “latitude”: 33.971 }, “microphoneAltitude”: { “altitude”: 123.1, “unit”: “meter” } “focalLength”: 55, “aperture”: 2.8, “shutterSpeed”: 0.008, “filter”: “UV” }, “rawVideo”: { “videoData16”: “0314BA3827CFF2938...”, “width”: 640, “height”: 480, “bit_depth”: 8, “stride”: 10, “coding4CC”: 2, “fps”: 30, “use_frame_packing”: true, “frame_packing”: 10 } } }

    - Biosensors

      - General

For the purposes of this document, the following terms and definitions apply. The IEEE Standards

      - Blood pressure Sensor

        - General

This sub-clause specifies a sensor data type, which describes a blood pressure.

        - Syntax

"bloodPressureSensorType": { "type": "object", "properties": { "systolicBP": { "type": "number" }, "diastolicBP": { "type": "number" }, "MAP": { "type": "number" }, "unit": { "$ref": "#/definitions/unitType" } } },

        - Semantics

Semantics of the bloodPressureSensorType:

| Name | Definition |
| --- | --- |
| bloodPressureSensorType | Tool for describing sensed information with respect to a blood pressure sensor. |
| systolicBP | Describes the sensed value of the systolic blood pressure with respect to the millimeters of mercury (mmHg). |
| diastolicBP | Describes the sensed value of the diastolic blood pressure with respect to the millimeters of mercury (mmHg). |
| MAP | Describes the sensed value of the mean arterial pressure with respect to the millimeters of mercury (mmHg). |

        - Examples

In this example, the systolic blood pressure measured by the blood pressure sensor is 121, the diastolic blood pressure is 83, and the mean arterial pressure is 100.

{ “sensedInfoBaseAttributes”: {}, “bloodPressureSensorType”: { “systolicBP”: 121, “diastolicBP”: 83, “MAP”: 100, } }

      - Heart rate Sensor

        - General

This sub-clause specifies a sensor data type, which describes a heart rate.

        - Syntax

"heartRateSensorType": { "type": "object", "properties": { "value": { "type": "number" }, "unit": { "$ref": "#/definitions/unitType" } }, "required": { "value" } },

        - Semantics

Semantics of the heartRateSensorType:

| Name | Definition |
| --- | --- |
| heartRateSensorType | Tool for describing sensed information with respect to a heart rate sensor. |
| value | Describes the sensed value of the heart rate with respect to the beats per minute (BPM). |

        - Examples

In this example, the heartbeat value measured by the heart rate sensor is 98 BPM.

{ “sensedInfoBaseAttributes”: {}, “heartRateSensorType”: { “value”: 98, } }

    - Environment Related Sensors

      - General

For the purposes of this document, the following terms and definitions apply. The IEEE Standards

      - Ambient Light Sensor

        - General

This sub-clause specifies a sensor data type, which describes a light intensity and its color.

        - Syntax

"lightSensorType": { "type": "object", "properties": { "value": { "$ref:": "#/definitions/colorWType" }, "unit": { "$ref": "#/definitions/unitType" }, "color": { "%ref": "#/definitions/colorType" }, "colorValue": { "%ref": "#/definitions/colorValueType" }, "model": { "$ref": "#/definitions/colorSpaceType" } } }, "colorWType": { "type": "array", "items": [ { "type": "number" }, { "type": "string", "pattern": "#[0-9A-Fa-f]{2}" } ] }, "colorValueType": { "type": "array", "items": [ { "type": "number", } ], "minItems": 3, "maxItems": 3 }, "colorSpaceType": { "type": "string", "enum": [ "XYZ", "Yxy", "Lab", "Lch", "LUV", "HunterLab" ] }, }

        - Semantics

Semantics of the lightSensorType:

| Name | Definition |
| --- | --- |
| lightSensorType | Tool for describing sensed information with respect to a light sensor. |
| value | Describes the sensed intensity of the light with respect to the unit defined in the unit attribute. This attribute can be used to represent “White” when the light sensor senses “RGBW”. |
| unit | Specifies the unit of the sensed value as a reference to a term that shall be using the valueType. |
| color | Describes the list of colors which the light sensor can sense either as a reference to a term that shall be using the colorType. |
| colorWType | Tool for describing a color in 1 byte (256 level) value of White. |
| colorValue | Describes the sensed values of a color sensor with respect to color space models. |
| model | Describes the color model of the sensed values from a color sensor using colorSpaceType. |
| colorValueType | Describes three values from a color sensor. The meaning of the three values is determined by the color space model. EXAMPLE The color model CIEXYZ would have three values of X, Y, and Z in order. |
| colorSpaceType | The color space models utilized by a color sensor are Yxy, CIEXYZ, CIELAB, CIELCH, CIELUV, and Hunter Lab. |

        - Examples

The light sensor detected 200 lux of light. Colors represent blue with 0.14, 0.8, and 0.78 in the CIE XYZ color space

{ “sensedInfoBaseAttributes”:{}, “lightSensorType”: { “value”: [200], “unit”: “lux”, “colorValue”: [0.14, 0.8, 0.78], “model”: “XYZ” } }

      - Ambient Noise Sensor

        - General

This sub-clause specifies a sensor data type, which describes an ambient noise and its duration.

        - Syntax

"ambientNoiseSensorType": { "type": "object", "properties": { "lifespan": { "type": "number" }, "value": { "type": "number" } } }

        - Semantics

Semantics of the ambientNoiseSensorType:

| Name | Definition |
| --- | --- |
| ambientNoiseSensorType | Tool for describing sensed information using an ambient noise sensor. |
| lifespan | Describes the duration taken to measure the information based on the timestamp. The unit of lifespan is the internal clock count. |
| value | Describes the sensed value of the ambient noise with respect to decibel (dB). |

        - Examples

The ambient noise intensity is measured every 100 ticks of the clock in the ambient noise sensor. The ambient noise measured is 10 decibels.

{ “sensedInfoBaseAttributes”:{}, “ambientNoiseSensorType”: { “lifespan”: 100, “value”: 10, } }

      - Temperature Sensor

        - General

This sub-clause specifies a sensor data type, which describe a temperature.

        - Syntax

"temperatureSensorType": { "type": "object", "properties": { "value": { "type": "number" }, "unit": { "$ref": "#/definitions/unitType" } } },

        - Semantics

Semantics of the temeratureSensorType:

| Name | Definition |
| --- | --- |
| temperatureSensorType | Tool for describing sensed information with respect to a temperature sensor. |
| value | Describes the sensed value of the temperature with respect to the unit defined in the unit attribute. |
| unit | Specifies the unit of the sensed value as a reference to a term that shall be using the unitType. |

        - Examples

The measured temperature is 36.5 C˚.

{ “sensedInfoBaseAttributes”:{}, “temperatureSensorType”: { “value”: 36.5, “unit”: “celsius” } }

      - Humidity Sensor

        - General

This sub-clause specifies a sensor data type, which describe a humidity.

        - Syntax

"humiditySensorType": { "type": "object", "properties": { "value": { "type": "number" }, } },

        - Semantics

Semantics of the humiditySensorType:

| Name | Definition |
| --- | --- |
| humiditySensorType | Tool for describing sensed information with respect to a humidity sensor. |
| value | Describes the value sensed by the humidity sensor with respect to percentage (%). |

        - Examples

The humidity measured was 60%.

{ “sensedInfoBaseAttributes”: {}, “humiditySensorType”: { “value”: 60, } }

      - Wind Sensor

        - General

This sub-clause specifies a sensor data type also known as “anemometer”, which measures a velocity of wind at a certain position.

        - Syntax

"windSensorType": { "$ref": "#/definitions/velocitySensorType" }, "velocitySensorType": { "type": "object", "properties": { "velocity": { "$ref": "#/definitions/float3DVectorType" }, "unit": { "$ref": "#/definitions/unitType" } } },

        - Semantics

Semantics of the windSensorType:

| Name | Definition |
| --- | --- |
| windSensorType | Tool for describing sensed information captured wind sensor. |
| velocity | Describes the speed and direction of a wind flow with respect to the unit defined in the unit attribute. Values represent the wind speeds from the x, y, and z directions, respectively. |
| unit | Specifies the unit of the sensed value as a reference to a term that shall be using the unitType. |

        - Examples

In this example, the velocities of wind measured were 1m/s from x direction, 0.7m/s from y direction, and 3.5m/s from z direction.

{ “sensedInfoBaseAttributes”: {}, “windSensorType”: { “velocity”: [1, 0.7, 3.5] “unit”: “meterpersec” } }

      - Gas Sensor

        - General

This sub-clause specifies a sensor data type, which describes a gas type and its gas concentration degree.

        - Syntax

"gasSensorType": { "type": "object", "properties": { "gasType": { "$ref": "#/definitions/gasType" }, "value": { "type": "number" }, "unit": { "$ref": "#/definitions/unitType" } }

        - Semantics

Semantics of the gasSensorType:

| Name | Definition |
| --- | --- |
| gasSensorType | Tool for describing sensed information with respect to a gas sensor. |
| gasType | Specifies the type of gas as a reference to a term provided by gasType. |
| value | Describes the sensed gas concentration value by the gas sensor with respect to the unit defined in the unit attribute. |
| unit | Specifies the unit of the sensed value as a reference to a term provided by UnitType. |

        - Examples

In this example, gas sensor measured 100ppm of methane.

{ “sensedInfoBaseAttributes”: {}, “gasSensorType”: { “gasType”: “methane”, “value”: 100, “unit”: “ppm” } }

      - Dust Sensor

        - General

This sub-clause specifies a sensor data type, which describes a dust concentration value without identifying the type of dust.

        - Syntax

"dustSensorType": { "type": "object", "properties": { "value": { "type": "number" }, "unit": { "$ref": "#/definitions/unitType" } } },

        - Semantics

Semantics of the dustSensorType:

| Name | Definition |
| --- | --- |
| dustSensorType | Tool for describing sensed information with respect to a dust sensor. |
| value | Describes the sensed dust concentration value by the dust check with respect to the unit defined in the unit attribute. |
| unit | Specifies the unit of the sensed value as a reference to a term provided by UnitType. |

        - Examples

The dust concentration value measured was 100 micro gram per cubic meters.

{ “sensedInfoBaseAttributes”: {}, “dustSensorType”: { “value”: 100, “unit”: “microgpcm” } }

Editor’s Note: There are couple of types in the Dust depending on the size? May need to check.

      - Weather

        - General

For the purposes of this document, the following terms and definitions apply. The IEEE Standards

        - Syntax

"weather" : { "title" : "Weather Information Scheme", "type" : "object", "properties" : { "Region" : {"type" : "string"}, "Date" : {"type" : "string"}, "Atmospheric-Condition" : {"type" : "string"}, "Temperature" : { "Value" : {"type" : "number"}, "Unit" : {"type" : "string"} }, "Wind" : { "Direction" : {"type" : "string"}, "Speed" : { "Value" : {"type" : "number"}, "Unit" : {"type" : "string"} } }, "Precipitation" : { "Value" : {"type" : "number"}, "Unit" : {"type" : "string"} }, "Pressure" : { "Value" : {"type" : "number"}, "Unit" : {"type" : "string"} }, "Sunrise" : {"type" : "string"}, "Sunset" : {"type" : "string"} } }

        - Semantics

For the purposes of this document, the following terms and definitions apply. The IEEE Standards

        - Examples

For the purposes of this document, the following terms and definitions apply. The IEEE Standards

    - Location and Position Related Sensors

      - General

For the purposes of this document, the following terms and definitions apply. The IEEE Standards

      - Global position sensor

        - General

This sub-clause specifies a sensor data type, which describes a global position.

        - Syntax

"globalPositionSensorType": { "type": "object", "properties": { "longitude": { "type": "number", "minimum": -180.0, "maximum": 180.0 }, "latitude": { "type": "number", "minimum": -90.0, "maximum": 90.0 } }, "required": [ "longitude", "latitude" ], "additionalProperties": false },

        - Semantics

Semantics of the GlobalPositionSensorType:

| Name | Definition |
| --- | --- |
| globalPositionSensorType | Tool for describing sensed information through global positioning system (gps) sensor with respect to a global position. |
| longitude | Describes the position of the sensor in terms of degrees of longitude. Positive values represent eastern longitude and negative values represent western longitude. ex: -132.236 represents 132.236 degrees West. |
| latitude | Describes the position of the sensor in terms of degrees of latitude. Positive value represents northern latitude and negative value represents southern latitude. ex: 37.103 represents 37.103 degrees North. |

        - Examples

In this example, a longitude is 115.5 and a latitude is 33.971.

{ “sensedInfoBaseType”: {}, “globalPositionSensorType”: { “longitude”: 115.5”, “latitude”: 33.971 } }

      - Three-axis accelerometer

        - General

For the purposes of this document, the following terms and definitions apply. The IEEE Standards

        - Syntax

"ThreeAxisAccelerometers" : { "title" : "3-Axis-Accelerometers ", "$ref": "#/definitions/float3DVectorType" }

Editor’s Note: Basic three-dimensional vector should be in the common classes. Should refer it here.

        - Semantics

For the purposes of this document, the following terms and definitions apply. The IEEE Standards

        - Examples

For the purposes of this document, the following terms and definitions apply. The IEEE Standards

      - Three-axis Gyroscope

        - General

For the purposes of this document, the following terms and definitions apply. The IEEE Standards

        - Syntax

"ThreeAxisGyroscope" : { "title" : "3-Axis-Gyroscopes", "$ref": "#/definitions/float3DVectorType" }

Editor’s Note: Basic three-dimensional vector should be in the common classes. Should refer it here.

        - Semantics

For the purposes of this document, the following terms and definitions apply. The IEEE Standards

        - Examples

For the purposes of this document, the following terms and definitions apply. The IEEE Standards

      - Magnetic field sensor

        - General

For the purposes of this document, the following terms and definitions apply. The IEEE Standards

        - Syntax

"MagneticField" : { "title" : "Magnetic-Field", "$ref": "#/definitions/float3DVectorType" }

Editor’s Note: Basic three-dimensional vector should be in the common classes. Should refer it here.

        - Semantics

For the purposes of this document, the following terms and definitions apply. The IEEE Standards

        - Examples

For the purposes of this document, the following terms and definitions apply. The IEEE Standards

      - Barometer

        - General

For the purposes of this document, the following terms and definitions apply. The IEEE Standards

        - Syntax

"Barometer" : { "title" : "Barometer", "type" : "object", "properties" : { "Value" : {"type" : "number"}, "Unit" : {"type" : "string"} } }

        - Semantics

For the purposes of this document, the following terms and definitions apply. The IEEE Standards

        - Examples

For the purposes of this document, the following terms and definitions apply. The IEEE Standards

      - Compass Sensor

        - General

This sub-clause specifies a sensor data type, which describes an azimuth.

        - Syntax

"compassSensorType": { "type": "object", "properties": { "azimuth": { "type": "number", "minimum": 0, "maximum": 360 }, }, "additionalProperties": false },

        - Semantics

Semantics of the compassSensorType:

| Name | Definition |
| --- | --- |
| compassSensorType | Tool for describing sensed information with respect to a compass sensor. |
| azimuth | Describes the sensed value by the compass sensor in 0 to 360 degree. The value 0 means the “Magnetic North pole” direction and 90 means the “East” with clockwise. |

        - Examples

In this example, the measured azimuth is 270.

{ “sensedInfoBaseAttributes”: {}, “compassSensorType”: { “azimuth”: 270 } }

      - Orientation Sensor

        - General

This sub-clause specifies a sensor data type, which describes an orientation.

        - Syntax

"orientationSensorType": { "type": "object", "properties": { "orientation": { "$ref": "#/definitions/float3DVectorType" } } },

        - Semantics

Semantics of the orientationSensor:

| Name | Definition |
| --- | --- |
| orientationSensorType | Tool for describing sensed information with respect to an orientation sensor. |
| orientation | Describes the sensed value by the orientation sensor in a three- dimensional vector in the unit of degree. The orientation shall be measured as the inclined degree (orientation) with respect to the original pose. The original pose shall be the pose of the object sensed at the time of sensor activation. If a calibration has been performed on the orientation of the sensor after activation, the orientation after the calibration shall be considered as the original pose of the object. |

        - Examples

In this example, the measured orientation has yaw, pitch, and roll values of 36, -45, and 80, respectively.

{ “sensedInfoBaseAttributes”: {}, “orientationSensorType”: { “orientation”: [36, -45, 80] } }

      - Position Sensor

        - General

This sub-clause specifies a sensor data type, which describes a position.

        - Syntax

"positionSensorType": { "type": "object", "properties": { "position": { "$ref": "#/definitions/float3DVectorType" }, } },

        - Semantics

Semantics of the positionSensorType:

| Name | Definition |
| --- | --- |
| positionSensorType | Tool for describing sensed information with respect to a position sensor. |
| Position | Describes the 3D value of the position sensor in the unit of meter (m). The origin of the coordinate shall be the position of the object sensed at the time of sensor activation. If a calibration has been performed on the position of the sensor, the origin shall be the position after the calibration. |

        - Examples

In this example, the sensor sensed that it moved 20 meters in the x direction, 20 meters in the y direction, and 1 meter in the z direction.

{ “sensedInfoBaseAttributes”: {}, “positionSensorType”: { “position”: [20, 10, 1] } }

      - Distance Sensor

        - General

This sub-clause specifies a sensor data type, which describes a distance.

        - Syntax

"distanceSensorType": { "type": "object", "properties": { "value": { "type": "number" }, "unit": { "$ref": "#/definitions/unitType" } } },

        - Semantics

Semantics of the distanceSensorType:

| Name | Definition |
| --- | --- |
| distanceSensorType | Tool for describing sensed information with respect to a distance sensor. |
| value | Describes the sensed value from the length sensor with respect to the unit defined in the unit attribute. |
| unit | Specifies the unit of the sensed value as a reference to a term that shall be using the unitType. |

        - Examples

In this example, the distance from the distance sensor to the object is 15 meters.

{ “sensedInfoBaseAttributes”: {}, “distanceSensorType”: { “value”: 15, “unit”: “meter” } }

      - Altitude Sensor

        - General

This sub-clause specifies a sensor data type, which describes an altitude.

        - Syntax

"altitudeSensorType": { "type": "object", "properties": { "altitude": { "type": "number", }, "unit": { "$ref": "#/definitions/unitType" } }, "additionalProperties": false },

        - Semantics

Semantics of the altitudeSensorType:

| Name | Definition |
| --- | --- |
| altitudeSensorType | Tool for describing sensed information with respect to a altitude sensor. |
| altitude | Describes the sensed value by the altitude sensor in the unit defined in the unit attribute. |
| unit | Specifies the unit of the sensed value as a reference to a term that shall be using the unitType. |

        - Examples

In this example, the measured altitude is 122.1 meter.

{ “sensedInfoBaseAttributes”: {}, “compassSensorType”: { “altitude”: 122.1”, “unit”: “meter” } }

  - Data format for recognizing sensor characteristics(capabilities?)

For the purposes of this document, the following terms and definitions apply. The IEEE Standards

We need this section if we have some sensors of which settings can be changed.

  - Data format to sensors for sensor setting

For the purposes of this document, the following terms and definitions apply. The IEEE Standards

We need this section to change the setting of a sensor. (Such sensors usually have their capabilities defined.)

- Application programming interface for smart sensors

  - General

For the purposes of this document, the following terms and definitions apply. The IEEE Standards

  - API for individual sensors

    - General

    - Microphone sensor

Table 1 - Microphone API

| Nested Classes |  |
| --- | --- |
| Modifier and Type | Method and Description |
| Constructor |  |
| Constructor and Description |  |
| Microphone() |  |
| Default constructor. |  |
| Microphone(String id) |  |
| Microphone(String id, String serverIPAddress, integer serverPort) |  |
| Fields |  |
| Modifier and Type | Field and Description |
| Methods |  |
| Modifier and Type | Method and Description |
| JSONObject | getMicrophoneSensorData() |
|  | This function returns sensor data from a microphone in JSON format. |

    - Color camera sensor

Table 2 – Color Camera API

| Nested Classes |  |
| --- | --- |
| Modifier and Type | Method and Description |
| Constructor |  |
| Constructor and Description |  |
| ColorCamera() |  |
| Default constructor. |  |
| ColorCamera(String id) |  |
| ColorCamera(String id, String serverIPAddress, integer serverPort) |  |
| Fields |  |
| Modifier and Type | Field and Description |
| Methods |  |
| Modifier and Type | Method and Description |
| JSONObject | getColorCameraSensorData() |
|  | This function returns sensor data from a ColorCamera in JSON format. |

# (informative)
Bibliography

Bibliographical references are resources that provide additional or helpful material but do not need to be understood or used to implement this standard. Reference to these resources is made for informational use only.

The Institute of Electrical and Electronics Engineers, Inc.

3 Park Avenue, New York, NY 10016-5997, USA

Copyright © varCRYear by The Institute of Electrical and Electronics Engineers, Inc.

All rights reserved. Published varPublishedDate. Printed in the United States of America.

IEEE is a registered trademark in the U.S. Patent & Trademark Office, owned by The Institute of Electrical and Electronics 
Engineers, Incorporated.

PDF:	ISBN varISBNpdf	varStdIDpdf

Print:	ISBN varISBNprint	varStdIDprint

IEEE prohibits discrimination, harassment, and bullying.

For more information, visit https://www.ieee.org/about/corporate/governance/p9-26.html.

No part of this publication may be reproduced in any form, in an electronic retrieval system or otherwise, without the prior written permission of the publisher.

The use of the word must is deprecated and cannot be used when stating mandatory requirements, must is used only to describe unavoidable situations.

The use of will is deprecated and cannot be used when stating mandatory requirements, will is only used in statements of fact.

IEEE Standards Dictionary Online is available at: http://dictionary.ieee.org. An IEEE Account is required for access to the dictionary, and one can be created at no charge on the dictionary sign-in page.