| Project | IEEE 802 Executive Committee on TV White Space |  |
| --- | --- | --- |
| Title | Whitespace Cross 802 Interface Requirements for Upper Layers |  |
| Date Submitted | 2009-03-11 |  |
| Source | Richard H. Paine Self 6115 72nd Dr NE Marysville, Wa 98270 | Voice: [2068548199] Fax: [3606590324] E-mail: [richard.h.paine@gmail.com] |
| Re: | [] |  |
| Abstract | RFC 4907 Interface Requirements from Data Link (IEEE) to Network (IETF) |  |
| Purpose | To provide interface requirements for an 802 common interface to upper layers. |  |
| Notice | This document has been prepared to assist the IEEE 802. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by 802 |  |

The following document describes the data link requirements for network:

-----------------------------------------------------------

Network Working Group                                      B. Aboba, Ed.

Request for Comments: 4907                   Internet Architecture Board

Category: Informational                                              IAB

June 2007

Architectural Implications of Link Indications

Status of This Memo

This memo provides information for the Internet community.  It does

not specify an Internet standard of any kind.  Distribution of this

memo is unlimited.

Copyright Notice

Copyright (C) The IETF Trust (2007).

Abstract

A link indication represents information provided by the link layer

to higher layers regarding the state of the link.  This document

describes the role of link indications within the Internet

architecture.  While the judicious use of link indications can

provide performance benefits, inappropriate use can degrade both

robustness and performance.  This document summarizes current

proposals, describes the architectural issues, and provides examples

of appropriate and inappropriate uses of link indications.

IAB                          Informational                      [Page 1]

RFC 4907                    Link Indications                   June 2007

Table of Contents

1. Introduction ....................................................3

1.1. Requirements ...............................................3

1.2. Terminology ................................................3

1.3. Overview ...................................................5

1.4. Layered Indication Model ...................................7

2. Architectural Considerations ...................................14

2.1. Model Validation ..........................................15

2.2. Clear Definitions .........................................16

2.3. Robustness ................................................17

2.4. Congestion Control ........................................20

2.5. Effectiveness .............................................21

2.6. Interoperability ..........................................22

2.7. Race Conditions ...........................................22

2.8. Layer Compression .........................................25

2.9. Transport of Link Indications .............................26

3. Future Work ....................................................27

4. Security Considerations ........................................28

4.1. Spoofing ..................................................28

4.2. Indication Validation .....................................29

4.3. Denial of Service .........................................30

5. References .....................................................31

5.1. Normative References ......................................31

5.2. Informative References ....................................31

6. Acknowledgments ................................................40

Appendix A. Literature Review .....................................41

A.1. Link Layer .................................................41

A.2. Internet Layer .............................................53

A.3. Transport Layer ............................................55

A.4. Application Layer ..........................................60

Appendix B. IAB Members ...........................................60

IAB                          Informational                      [Page 2]

RFC 4907                    Link Indications                   June 2007

1.  Introduction

A link indication represents information provided by the link layer

to higher layers regarding the state of the link.  While the

judicious use of link indications can provide performance benefits,

inappropriate use can degrade both robustness and performance.

This document summarizes the current understanding of the role of

link indications within the Internet architecture, and provides

advice to document authors about the appropriate use of link

indications within the Internet, transport, and application layers.

Section 1 describes the history of link indication usage within the

Internet architecture and provides a model for the utilization of

link indications.  Section 2 describes the architectural

considerations and provides advice to document authors.  Section 3

describes recommendations and future work.  Appendix A summarizes the

literature on link indications, focusing largely on wireless Local

Area Networks (WLANs).

1.1.  Requirements

The key words "MUST", "MUST NOT", "REQUIRED", "SHALL", "SHALL NOT",

"SHOULD", "SHOULD NOT", "RECOMMENDED", "MAY", and "OPTIONAL" in this

document are to be interpreted as described in [RFC2119].

1.2.  Terminology

Access Point (AP)

A station that provides access to the fixed network (e.g., an

802.11 Distribution System), via the wireless medium (WM) for

associated stations.

Asymmetric

A link with transmission characteristics that are different

depending upon the relative position or design characteristics

of the transmitter and the receiver is said to be asymmetric.

For instance, the range of one transmitter may be much higher

than the range of another transmitter on the same medium.

Beacon

A control message broadcast by a station (typically an Access

Point), informing stations in the neighborhood of its continuing

presence, possibly along with additional status or configuration

information.

IAB                          Informational                      [Page 3]

RFC 4907                    Link Indications                   June 2007

Binding Update (BU)

A message indicating a mobile node's current mobility binding,

and in particular its Care-of Address.

Correspondent Node

A peer node with which a mobile node is communicating.  The

correspondent node may be either mobile or stationary.

Link

A communication facility or medium over which nodes can

communicate at the link layer, i.e., the layer immediately below

the Internet Protocol (IP).

Link Down

An event provided by the link layer that signifies a state

change associated with the interface no longer being capable of

communicating data frames; transient periods of high frame loss

are not sufficient.

Link Indication

Information provided by the link layer to higher layers

regarding the state of the link.

Link Layer

Conceptual layer of control or processing logic that is

responsible for maintaining control of the link.  The link layer

functions provide an interface between the higher-layer logic

and the link.  The link layer is the layer immediately below the

Internet Protocol (IP).

Link Up

An event provided by the link layer that signifies a state

change associated with the interface becoming capable of

communicating data frames.

Maximum Segment Size (MSS)

The maximum payload size available to the transport layer.

Maximum Transmission Unit (MTU)

The size in octets of the largest IP packet, including the IP

header and payload, that can be transmitted on a link or path.

Mobile Node

A node that can change its point of attachment from one link to

another, while still being reachable via its home address.

IAB                          Informational                      [Page 4]

RFC 4907                    Link Indications                   June 2007

Operable Address

A static or dynamically assigned address that has not been

relinquished and has not expired.

Point of Attachment

The endpoint on the link to which the host is currently

connected.

Routable Address

Any IP address for which routers will forward packets.  This

includes private addresses as specified in "Address Allocation

for Private Internets" [RFC1918].

Station (STA)

Any device that contains an IEEE 802.11 conformant medium access

control (MAC) and physical layer (PHY) interface to the wireless

medium (WM).

Strong End System Model

The Strong End System model emphasizes the host/router

distinction, tending to model a multi-homed host as a set of

logical hosts within the same physical host.  In the Strong End

System model, addresses refer to an interface, rather than to

the host to which they attach.  As a result, packets sent on an

outgoing interface have a source address configured on that

interface, and incoming packets whose destination address does

not correspond to the physical interface through which it is

received are silently discarded.

Weak End System Model

In the Weak End System model, addresses refer to a host.  As a

result, packets sent on an outgoing interface need not

necessarily have a source address configured on that interface,

and incoming packets whose destination address does not

correspond to the physical interface through which it is

received are accepted.

1.3.  Overview

The use of link indications within the Internet architecture has a

long history.  In response to an attempt to send to a host that was

off-line, the ARPANET link layer protocol provided a "Destination

Dead" indication, described in "Fault Isolation and Recovery"

[RFC816].  The ARPANET packet radio experiment [PRNET] incorporated

frame loss in the calculation of routing metrics, a precursor to more

recent link-aware routing metrics such as Expected Transmission Count

(ETX), described in "A High-Throughput Path Metric for Multi-Hop

Wireless Routing" [ETX].

IAB                          Informational                      [Page 5]

RFC 4907                    Link Indications                   June 2007

"Routing Information Protocol" [RFC1058] defined RIP, which is

descended from the Xerox Network Systems (XNS) Routing Information

Protocol.  "The OSPF Specification" [RFC1131] defined Open Shortest

Path First, which uses Link State Advertisements (LSAs) in order to

flood information relating to link status within an OSPF area.

[RFC2328] defines version 2 of OSPF.  While these and other routing

protocols can utilize "Link Up" and "Link Down" indications provided

by those links that support them, they also can detect link loss

based on loss of routing packets.  As noted in "Requirements for IP

Version 4 Routers" [RFC1812]:

It is crucial that routers have workable mechanisms for determining

that their network connections are functioning properly.  Failure to

detect link loss, or failure to take the proper actions when a

problem is detected, can lead to black holes.

Attempts have also been made to define link indications other than

"Link Up" and "Link Down".  "Dynamically Switched Link Control

Protocol" [RFC1307] defines an experimental protocol for control of

links, incorporating "Down", "Coming Up", "Up", "Going Down", "Bring

Down", and "Bring Up" states.

"A Generalized Model for Link Layer Triggers" [GenTrig] defines

"generic triggers", including "Link Up", "Link Down", "Link Going

Down", "Link Going Up", "Link Quality Crosses Threshold", "Trigger

Rollback", and "Better Signal Quality AP Available".  IEEE 802.21

[IEEE-802.21] defines a Media Independent Handover Event Service

(MIH-ES) that provides event reporting relating to link

characteristics, link status, and link quality.  Events defined

include "Link Down", "Link Up", "Link Going Down", "Link Signal

Strength", and "Link Signal/Noise Ratio".

Under ideal conditions, links in the "up" state experience low frame

loss in both directions and are immediately ready to send and receive

data frames; links in the "down" state are unsuitable for sending and

receiving data frames in either direction.

Unfortunately, links frequently exhibit non-ideal behavior.  Wired

links may fail in half-duplex mode, or exhibit partial impairment

resulting in intermediate loss rates.  Wireless links may exhibit

asymmetry, intermittent frame loss, or rapid changes in throughput

due to interference or signal fading.  In both wired and wireless

links, the link state may rapidly flap between the "up" and "down"

states.  This real-world behavior presents challenges to the

integration of link indications with the Internet, transport, and

application layers.

IAB                          Informational                      [Page 6]

RFC 4907                    Link Indications                   June 2007

1.4.  Layered Indication Model

A layered indication model is shown in Figure 1 that includes both

internally generated link indications (such as link state and rate)

and indications arising from external interactions such as path

change detection.  In this model, it is assumed that the link layer

provides indications to higher layers primarily in the form of

abstract indications that are link-technology agnostic.

IAB                          Informational                      [Page 7]

RFC 4907                    Link Indications                   June 2007

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

Application   |                                               |

Layer         |                                               |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

^     ^   ^

!     !   !

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-!-+-+-!-+-!-+-+-+-+

|                             !     !   !       |

|                             !     ^   ^       |

|     Connection Management   !     ! Teardown  |

Transport     |                             !     !           |

Layer         +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-!-+-+-!-+-+-+-+-+-+

|                             !     !           |

|                             !     !           |

|                             ^     !           |

|  Transport Parameter Estimation   !           |

|(MSS, RTT, RTO, cwnd, bw, ssthresh)!           |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-!-+-+-+-+-+-+

^   ^           ^       ^   ^     !

!   !           !       !   !     !

+-!-+-!-+-+-+-+-+-!-+-+-+-!-+-!-+-+-!-+-+-+-+-+-+

| !   ! Incoming  !MIP    !   !     !           |

| !   ! Interface !BU     !   !     !           |

| !   ! Change    !Receipt!   !     !           |

| !   ^           ^       ^   !     ^           |

Internet      | !   ! Mobility  !       !   !     !           |

Layer         +-!-+-!-+-+-+-+-+-!-+-+-+-!-+-!-+-+-!-+-+-+-+-+-+

| !   ! Outgoing  ! Path  !   !     !           |

| !   ! Interface ! Change!   !     !           |

| ^   ^ Change    ^       ^   !     ^           |

| !                       !   !     !           |

| !     Routing           !   !     !           |

+-!-+-+-+-+-+-+-+-+-+-+-+-!-+-!-+-+-!-+-+-+-+-+-+

| !                       !   v     ! IP        |

| !                       !  Path   ! Address   |

| !   IP Configuration    ^  Info   ^ Config/   |

| !                       !  Cache    Changes   |

+-!-+-+-+-+-+-+-+-+-+-+-+-!-+-+-+-+-+-+-+-+-+-+-+

!                       !

!                       !

+-!-+-+-+-+-+-+-+-+-+-+-+-!-+-+-+-+-+-+-+-+-+-+-+

| !                       !                     |

Link          | ^                       ^                     |

Layer         | Rate, FER,            Link                    |

| Delay                 Up/Down                 |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

Figure 1.  Layered Indication Model

IAB                          Informational                      [Page 8]

RFC 4907                    Link Indications                   June 2007

1.4.1.  Internet Layer

One of the functions of the Internet layer is to shield higher layers

from the specifics of link behavior.  As a result, the Internet layer

validates and filters link indications and selects outgoing and

incoming interfaces based on routing metrics.

The Internet layer composes its routing table based on information

available from local interfaces as well as potentially by taking into

account information provided by routers.  This enables the state of

the local routing table to reflect link conditions on both local and

remote links.  For example, prefixes to be added or removed from the

routing table may be determined from Dynamic Host Configuration

Protocol (DHCP) [RFC2131][RFC3315], Router Advertisements

[RFC1256][RFC2461], redirect messages, or route updates incorporating

information on the state of links multiple hops away.

As described in "Packetization Layer Path MTU Discovery" [RFC4821],

the Internet layer may maintain a path information cache, enabling

sharing of Path MTU information between concurrent or subsequent

connections.  The shared cache is accessed and updated by

packetization protocols implementing packetization layer Path MTU

Discovery.

The Internet layer also utilizes link indications in order to

optimize aspects of Internet Protocol (IP) configuration and

mobility.  After receipt of a "Link Up" indication, hosts validate

potential IP configurations by Detecting Network Attachment (DNA)

[RFC4436].  Once the IP configuration is confirmed, it may be

determined that an address change has occurred.  However, "Link Up"

indications may not necessarily result in a change to Internet layer

configuration.

In "Detecting Network Attachment in IPv4" [RFC4436], after receipt of

a "Link Up" indication, potential IP configurations are validated

using a bidirectional reachability test.  In "Detecting Network

Attachment in IPv6 Networks (DNAv6)" [DNAv6], IP configuration is

validated using reachability detection and Router

Solicitation/Advertisement.

The routing sub-layer may utilize link indications in order to enable

more rapid response to changes in link state and effective

throughput.  Link rate is often used in computing routing metrics.

However, in wired networks the transmission rate may be negotiated in

order to enhance energy efficiency [EfficientEthernet].  In wireless

networks, the negotiated rate and Frame Error Rate (FER) may change

IAB                          Informational                      [Page 9]

RFC 4907                    Link Indications                   June 2007

with link conditions so that effective throughput may vary on a

packet-by-packet basis.  In such situations, routing metrics may also

exhibit rapid variation.

Routing metrics incorporating link indications such as Link Up/Down

and effective throughput enable routers to take link conditions into

account for the purposes of route selection.  If a link experiences

decreased rate or high frame loss, the route metric will increase for

the prefixes that it serves, encouraging use of alternate paths if

available.  When the link condition improves, the route metric will

decrease, encouraging use of the link.

Within Weak End System implementations, changes in routing metrics

and link state may result in a change in the outgoing interface for

one or more transport connections.  Routes may also be added or

withdrawn, resulting in loss or gain of peer connectivity.  However,

link indications such as changes in transmission rate or frame loss

do not necessarily result in a change of outgoing interface.

The Internet layer may also become aware of path changes by other

mechanisms, such as receipt of updates from a routing protocol,

receipt of a Router Advertisement, dead gateway detection [RFC816] or

network unreachability detection [RFC2461], ICMP redirects, or a

change in the IPv4 TTL (Time to Live)/IPv6 Hop Limit of received

packets.  A change in the outgoing interface may in turn influence

the mobility sub-layer, causing a change in the incoming interface.

The mobility sub-layer may also become aware of a change in the

incoming interface of a peer (via receipt of a Mobile IP Binding

Update [RFC3775]).

1.4.2.  Transport Layer

The transport layer processes received link indications differently

for the purposes of transport parameter estimation and connection

management.

For the purposes of parameter estimation, the transport layer is

primarily interested in path properties that impact performance, and

where link indications may be determined to be relevant to path

properties they may be utilized directly.  Link indications such as

"Link Up"/"Link Down" or changes in rate, delay, and frame loss may

prove relevant.  This will not always be the case, however; where the

bandwidth of the bottleneck on the end-to-end path is already much

lower than the transmission rate, an increase in transmission rate

may not materially affect path properties.  As described in Appendix

A.3, the algorithms for utilizing link layer indications to improve

transport parameter estimates are still under development.

IAB                          Informational                     [Page 10]

RFC 4907                    Link Indications                   June 2007

Strict layering considerations do not apply in transport path

parameter estimation in order to enable the transport layer to make

use of all available information.  For example, the transport layer

may determine that a link indication came from a link forming part of

a path of one or more connections.  In this case, it may utilize the

receipt of a "Link Down" indication followed by a subsequent "Link

Up" indication to infer the possibility of non-congestive packet loss

during the period between the indications, even if the IP

configuration does not change as a result, so that no Internet layer

indication would be sent.

The transport layer may also find Internet layer indications useful

for path parameter estimation.  For example, path change indications

can be used as a signal to reset path parameter estimates.  Where

there is no default route, loss of segments sent to a destination

lacking a prefix in the local routing table may be assumed to be due

to causes other than congestion, regardless of the reason for the

removal (either because local link conditions caused it to be removed

or because the route was withdrawn by a remote router).

For the purposes of connection management, layering considerations

are important.  The transport layer may tear down a connection based

on Internet layer indications (such as a endpoint address changes),

but does not take link indications into account.  Just as a "Link Up"

event may not result in a configuration change, and a configuration

change may not result in connection teardown, the transport layer

does not tear down connections on receipt of a "Link Down"

indication, regardless of the cause.  Where the "Link Down"

indication results from frame loss rather than an explicit exchange,

the indication may be transient, to be soon followed by a "Link Up"

indication.

Even where the "Link Down" indication results from an explicit

exchange such as receipt of a Point-to-Point Protocol (PPP) Link

Control Protocol (LCP)-Terminate or an IEEE 802.11 Disassociate or

Deauthenticate frame, an alternative point of attachment may be

available, allowing connectivity to be quickly restored.  As a

result, robustness is best achieved by allowing connections to remain

up until an endpoint address changes, or the connection is torn down

due to lack of response to repeated retransmission attempts.

For the purposes of connection management, the transport layer is

cautious with the use of Internet layer indications.  Changes in the

routing table are not relevant for the purposes of connection

management, since it is desirable for connections to remain up during

transitory routing flaps.  However, the transport layer may tear down

transport connections due to invalidation of a connection endpoint IP

address.  Where the connection has been established based on a Mobile

IAB                          Informational                     [Page 11]

RFC 4907                    Link Indications                   June 2007

IP home address, a change in the Care-of Address need not result in

connection teardown, since the configuration change is masked by the

mobility functionality within the Internet layer, and is therefore

transparent to the transport layer.

"Requirements for Internet Hosts -- Communication Layers" [RFC1122],

Section 2.4, requires Destination Unreachable, Source Quench, Echo

Reply, Timestamp Reply, and Time Exceeded ICMP messages to be passed

up to the transport layer.  [RFC1122], Section 4.2.3.9, requires

Transmission Control Protocol (TCP) to react to an Internet Control

Message Protocol (ICMP) Source Quench by slowing transmission.

[RFC1122], Section 4.2.3.9, distinguishes between ICMP messages

indicating soft error conditions, which must not cause TCP to abort a

connection, and hard error conditions, which should cause an abort.

ICMP messages indicating soft error conditions include Destination

Unreachable codes 0 (Net), 1 (Host), and 5 (Source Route Failed),

which may result from routing transients; Time Exceeded; and

Parameter Problem.  ICMP messages indicating hard error conditions

include Destination Unreachable codes 2 (Protocol Unreachable), 3

(Port Unreachable), and 4 (Fragmentation Needed and Don't Fragment

Was Set).  Since hosts implementing classical ICMP-based Path MTU

Discovery [RFC1191] use Destination Unreachable code 4, they do not

treat this as a hard error condition.  Hosts implementing "Path MTU

Discovery for IP version 6" [RFC1981] utilize ICMPv6 Packet Too Big

messages.  As noted in "TCP Problems with Path MTU Discovery"

[RFC2923], classical Path MTU Discovery is vulnerable to failure if

ICMP messages are not delivered or processed.  In order to address

this problem, "Packetization Layer Path MTU Discovery" [RFC4821] does

depend on the delivery of ICMP messages.

"Fault Isolation and Recovery" [RFC816], Section 6, states:

It is not obvious, when error messages such as ICMP Destination

Unreachable arrive, whether TCP should abandon the connection.  The

reason that error messages are difficult to interpret is that, as

discussed above, after a failure of a gateway or network, there is a

transient period during which the gateways may have incorrect

information, so that irrelevant or incorrect error messages may

sometimes return.  An isolated ICMP Destination Unreachable may

arrive at a host, for example, if a packet is sent during the period

when the gateways are trying to find a new route.  To abandon a TCP

connection based on such a message arriving would be to ignore the

valuable feature of the Internet that for many internal failures it

reconstructs its function without any disruption of the end points.

IAB                          Informational                     [Page 12]

RFC 4907                    Link Indications                   June 2007

"Requirements for IP Version 4 Routers" [RFC1812], Section 4.3.3.3,

states that "Research seems to suggest that Source Quench consumes

network bandwidth but is an ineffective (and unfair) antidote to

congestion", indicating that routers should not originate them.  In

general, since the transport layer is able to determine an

appropriate (and conservative) response to congestion based on packet

loss or explicit congestion notification, ICMP Source Quench

indications are not needed, and the sending of additional Source

Quench packets during periods of congestion may be detrimental.

"ICMP attacks against TCP" [Gont] argues that accepting ICMP messages

based on a correct four-tuple without additional security checks is

ill-advised.  For example, an attacker forging an ICMP hard error

message can cause one or more transport connections to abort.  The

authors discuss a number of precautions, including mechanisms for

validating ICMP messages and ignoring or delaying response to hard

error messages under various conditions.  They also recommend that

hosts ignore ICMP Source Quench messages.

The transport layer may also provide information to the link layer.

For example, the transport layer may wish to control the maximum

number of times that a link layer frame may be retransmitted, so that

the link layer does not continue to retransmit after a transport

layer timeout.  In IEEE 802.11, this can be achieved by adjusting the

Management Information Base (MIB) [IEEE-802.11] variables

dot11ShortRetryLimit (default: 7) and dot11LongRetryLimit (default:

4), which control the maximum number of retries for frames shorter

and longer in length than dot11RTSThreshold, respectively.  However,

since these variables control link behavior as a whole they cannot be

used to separately adjust behavior on a per-transport connection

basis.  In situations where the link layer retransmission timeout is

of the same order as the path round-trip timeout, link layer control

may not be possible at all.

1.4.3.  Application Layer

The transport layer provides indications to the application layer by

propagating Internet layer indications (such as IP address

configuration and changes), as well as providing its own indications,

such as connection teardown.

Since applications can typically obtain the information they need

more reliably from the Internet and transport layers, they will

typically not need to utilize link indications.  A "Link Up"

indication implies that the link is capable of communicating IP

packets, but does not indicate that it has been configured;

applications should use an Internet layer "IP Address Configured"

event instead.  "Link Down" indications are typically not useful to

IAB                          Informational                     [Page 13]

RFC 4907                    Link Indications                   June 2007

applications, since they can be rapidly followed by a "Link Up"

indication; applications should respond to transport layer teardown

indications instead.  Similarly, changes in the transmission rate may

not be relevant to applications if the bottleneck bandwidth on the

path does not change; the transport layer is best equipped to

determine this.  As a result, Figure 1 does not show link indications

being provided directly to applications.

2.  Architectural Considerations

The complexity of real-world link behavior poses a challenge to the

integration of link indications within the Internet architecture.

While the literature provides persuasive evidence of the utility of

link indications, difficulties can arise in making effective use of

them.  To avoid these issues, the following architectural principles

are suggested and discussed in more detail in the sections that

follow:

(1)  Proposals should avoid use of simplified link models in

circumstances where they do not apply (Section 2.1).

(2)  Link indications should be clearly defined, so that it is

understood when they are generated on different link layers

(Section 2.2).

(3)  Proposals must demonstrate robustness against spurious link

indications (Section 2.3).

(4)  Upper layers should utilize a timely recovery step so as to

limit the potential damage from link indications determined to

be invalid after they have been acted on (Section 2.3.2).

(5)  Proposals must demonstrate that effective congestion control is

maintained (Section 2.4).

(6)  Proposals must demonstrate the effectiveness of proposed

optimizations (Section 2.5).

(7)  Link indications should not be required by upper layers, in

order to maintain link independence (Section 2.6).

(8)  Proposals should avoid race conditions, which can occur where

link indications are utilized directly by multiple layers of the

stack (Section 2.7).

(9)  Proposals should avoid inconsistencies between link and routing

layer metrics (Section 2.7.3).

IAB                          Informational                     [Page 14]

RFC 4907                    Link Indications                   June 2007

(10) Overhead reduction schemes must avoid compromising

interoperability and introducing link layer dependencies into

the Internet and transport layers (Section 2.8).

(11) Proposals for transport of link indications beyond the local

host need to carefully consider the layering, security, and

transport implications (Section 2.9).

2.1.  Model Validation

Proposals should avoid the use of link models in circumstances where

they do not apply.

In "The mistaken axioms of wireless-network research" [Kotz], the

authors conclude that mistaken assumptions relating to link behavior

may lead to the design of network protocols that may not work in

practice.  For example, the authors note that the three-dimensional

nature of wireless propagation can result in large signal strength

changes over short distances.  This can result in rapid changes in

link indications such as rate, frame loss, and signal strength.

In "Modeling Wireless Links for Transport Protocols" [GurtovFloyd],

the authors provide examples of modeling mistakes and examples of how

to improve modeling of link characteristics.  To accompany the paper,

the authors provide simulation scenarios in ns-2.

In order to avoid the pitfalls described in [Kotz] [GurtovFloyd],

documents that describe capabilities that are dependent on link

indications should explicitly articulate the assumptions of the link

model and describe the circumstances in which they apply.

Generic "trigger" models may include implicit assumptions that may

prove invalid in outdoor or mesh wireless LAN deployments.  For

example, two-state Markov models assume that the link is either in a

state experiencing low frame loss ("up") or in a state where few

frames are successfully delivered ("down").  In these models,

symmetry is also typically assumed, so that the link is either "up"

in both directions or "down" in both directions.  In situations where

intermediate loss rates are experienced, these assumptions may be

invalid.

As noted in "Hybrid Rate Control for IEEE 802.11" [Haratcherev],

signal strength data is noisy and sometimes inconsistent, so that it

needs to be filtered in order to avoid erratic results.  Given this,

link indications based on raw signal strength data may be unreliable.

In order to avoid problems, it is best to combine signal strength

data with other techniques.  For example, in developing a "Going

Down" indication for use with [IEEE-802.21] it would be advisable to

IAB                          Informational                     [Page 15]

RFC 4907                    Link Indications                   June 2007

validate filtered signal strength measurements with other indications

of link loss such as lack of Beacon reception.

2.2.  Clear Definitions

Link indications should be clearly defined, so that it is understood

when they are generated on different link layers.  For example,

considerable work has been required in order to come up with the

definitions of "Link Up" and "Link Down", and to define when these

indications are sent on various link layers.

Link indication definitions should heed the following advice:

(1)  Do not assume symmetric link performance or frame loss that is

either low ("up") or high ("down").

In wired networks, links in the "up" state typically experience

low frame loss in both directions and are ready to send and

receive data frames; links in the "down" state are unsuitable

for sending and receiving data frames in either direction.

Therefore, a link providing a "Link Up" indication will

typically experience low frame loss in both directions, and high

frame loss in any direction can only be experienced after a link

provides a "Link Down" indication.  However, these assumptions

may not hold true for wireless LAN networks.  Asymmetry is

typically less of a problem for cellular networks where

propagation occurs over longer distances, multi-path effects may

be less severe, and the base station can transmit at much higher

power than mobile stations while utilizing a more sensitive

antenna.

Specifications utilizing a "Link Up" indication should not

assume that receipt of this indication means that the link is

experiencing symmetric link conditions or low frame loss in

either direction.  In general, a "Link Up" event should not be

sent due to transient changes in link conditions, but only due

to a change in link layer state.  It is best to assume that a

"Link Up" event may not be sent in a timely way.  Large handoff

latencies can result in a delay in the generation of a "Link Up"

event as movement to an alternative point of attachment is

delayed.

(2)  Consider the sensitivity of link indications to transient link

conditions.  Due to common effects such as multi-path

interference, signal strength and signal to noise ratio (SNR)

may vary rapidly over a short distance, causing erratic behavior

of link indications based on unfiltered measurements.  As noted

in [Haratcherev], signal strength may prove most useful when

IAB                          Informational                     [Page 16]

RFC 4907                    Link Indications                   June 2007

utilized in combination with other measurements, such as frame

loss.

(3)  Where possible, design link indications with built-in damping.

By design, the "Link Up" and "Link Down" events relate to

changes in the state of the link layer that make it able and

unable to communicate IP packets.  These changes are generated

either by the link layer state machine based on link layer

exchanges (e.g., completion of the IEEE 802.11i four-way

handshake for "Link Up", or receipt of a PPP LCP-Terminate for

"Link Down") or by protracted frame loss, so that the link layer

concludes that the link is no longer usable.  As a result, these

link indications are typically less sensitive to changes in

transient link conditions.

(4)  Do not assume that a "Link Down" event will be sent at all, or

that, if sent, it will be received in a timely way.  A good link

layer implementation will both rapidly detect connectivity

failure (such as by tracking missing Beacons) while sending a

"Link Down" event only when it concludes the link is unusable,

not due to transient frame loss.

However, existing wireless LAN implementations often do not do a good

job of detecting link failure.  During a lengthy detection phase, a

"Link Down" event is not sent by the link layer, yet IP packets

cannot be transmitted or received on the link.  Initiation of a scan

may be delayed so that the station cannot find another point of

attachment.  This can result in inappropriate backoff of

retransmission timers within the transport layer, among other

problems.  This is not as much of a problem for cellular networks

that utilize transmit power adjustment.

2.3.  Robustness

Link indication proposals must demonstrate robustness against

misleading indications.  Elements to consider include:

Implementation variation

Recovery from invalid indications

Damping and hysteresis

2.3.1.  Implementation Variation

Variations in link layer implementations may have a substantial

impact on the behavior of link indications.  These variations need to

be taken into account in evaluating the performance of proposals.

For example, radio propagation and implementation differences can

impact the reliability of link indications.

IAB                          Informational                     [Page 17]

RFC 4907                    Link Indications                   June 2007

In "Link-level Measurements from an 802.11b Mesh Network" [Aguayo],

the authors analyze the cause of frame loss in a 38-node urban

multi-hop IEEE 802.11 ad-hoc network.  In most cases, links that are

very bad in one direction tend to be bad in both directions, and

links that are very good in one direction tend to be good in both

directions.  However, 30 percent of links exhibited loss rates

differing substantially in each direction.

As described in [Aguayo], wireless LAN links often exhibit loss rates

intermediate between "up" (low loss) and "down" (high loss) states,

as well as substantial asymmetry.  As a result, receipt of a "Link

Up" indication may not necessarily indicate bidirectional

reachability, since it could have been generated after exchange of

small frames at low rates, which might not imply bidirectional

connectivity for large frames exchanged at higher rates.

Where multi-path interference or hidden nodes are encountered, signal

strength may vary widely over a short distance.  Several techniques

may be used to reduce potential disruptions.  Multiple transmitting

and receiving antennas may be used to reduce multi-path effects;

transmission rate adaptation can be used to find a more satisfactory

transmission rate; transmit power adjustment can be used to improve

signal quality and reduce interference; Request-to-Send/Clear-to-Send

(RTS/CTS) signaling can be used to reduce hidden node problems.

These techniques may not be completely effective, so that high frame

loss may be encountered, causing the link to cycle between "up" and

"down" states.

To improve robustness against spurious link indications, it is

recommended that upper layers treat the indication as a "hint"

(advisory in nature), rather than a "trigger" dictating a particular

action.  Upper layers may then attempt to validate the hint.

In [RFC4436], "Link Up" indications are rate limited, and IP

configuration is confirmed using bidirectional reachability tests

carried out coincident with a request for configuration via DHCP.  As

a result, bidirectional reachability is confirmed prior to activation

of an IP configuration.  However, where a link exhibits an

intermediate loss rate, demonstration of bidirectional reachability

may not necessarily indicate that the link is suitable for carrying

IP data packets.

Another example of validation occurs in IPv4 Link-Local address

configuration [RFC3927].  Prior to configuration of an IPv4 Link-

Local address, it is necessary to run a claim-and-defend protocol.

Since a host needs to be present to defend its address against

another claimant, and address conflicts are relatively likely, a host

returning from sleep mode or receiving a "Link Up" indication could

IAB                          Informational                     [Page 18]

RFC 4907                    Link Indications                   June 2007

encounter an address conflict were it to utilize a formerly

configured IPv4 Link-Local address without rerunning claim and

defend.

2.3.2.  Recovery from Invalid Indications

In some situations, improper use of link indications can result in

operational malfunctions.  It is recommended that upper layers

utilize a timely recovery step so as to limit the potential damage

from link indications determined to be invalid after they have been

acted on.

In Detecting Network Attachment in IPv4 (DNAv4) [RFC4436],

reachability tests are carried out coincident with a request for

configuration via DHCP.  Therefore, if the bidirectional reachability

test times out, the host can still obtain an IP configuration via

DHCP, and if that fails, the host can still continue to use an

existing valid address if it has one.

Where a proposal involves recovery at the transport layer, the

recovered transport parameters (such as the Maximum Segment Size

(MSS), RoundTrip Time (RTT), Retransmission TimeOut (RTO), Bandwidth

(bw), congestion window (cwnd), etc.) should be demonstrated to

remain valid.  Congestion window validation is discussed in "TCP

Congestion Window Validation" [RFC2861].

Where timely recovery is not supported, unexpected consequences may

result.  As described in [RFC3927], early IPv4 Link-Local

implementations would wait five minutes before attempting to obtain a

routable address after assigning an IPv4 Link-Local address.  In one

implementation, it was observed that where mobile hosts changed their

point of attachment more frequently than every five minutes, they

would never obtain a routable address.  The problem was caused by an

invalid link indication (signaling of "Link Up" prior to completion

of link layer authentication), resulting in an initial failure to

obtain a routable address using DHCP.  As a result, [RFC3927]

recommends against modification of the maximum retransmission timeout

(64 seconds) provided in [RFC2131].

2.3.3.  Damping and Hysteresis

Damping and hysteresis can be utilized to limit damage from unstable

link indications.  This may include damping unstable indications or

placing constraints on the frequency of link indication-induced

actions within a time period.

IAB                          Informational                     [Page 19]

RFC 4907                    Link Indications                   June 2007

While [Aguayo] found that frame loss was relatively stable for

stationary stations, obstacles to radio propagation and multi-path

interference can result in rapid changes in signal strength for a

mobile station.  As a result, it is possible for mobile stations to

encounter rapid changes in link characteristics, including changes in

transmission rate, throughput, frame loss, and even "Link Up"/"Link

Down" indications.

Where link-aware routing metrics are implemented, this can result in

rapid metric changes, potentially resulting in frequent changes in

the outgoing interface for Weak End System implementations.  As a

result, it may be necessary to introduce route flap dampening.

However, the benefits of damping need to be weighed against the

additional latency that can be introduced.  For example, in order to

filter out spurious "Link Down" indications, these indications may be

delayed until it can be determined that a "Link Up" indication will

not follow shortly thereafter.  However, in situations where multiple

Beacons are missed such a delay may not be needed, since there is no

evidence of a suitable point of attachment in the vicinity.

In some cases, it is desirable to ignore link indications entirely.

Since it is possible for a host to transition from an ad-hoc network

to a network with centralized address management, a host receiving a

"Link Up" indication cannot necessarily conclude that it is

appropriate to configure an IPv4 Link-Local address prior to

determining whether a DHCP server is available [RFC3927] or an

operable configuration is valid [RFC4436].

As noted in Section 1.4, the transport layer does not utilize "Link

Up" and "Link Down" indications for the purposes of connection

management.

2.4.  Congestion Control

Link indication proposals must demonstrate that effective congestion

control is maintained [RFC2914].  One or more of the following

techniques may be utilized:

Rate limiting.  Packets generated based on receipt of link

indications can be rate limited (e.g., a limit of one packet per

end-to-end path RTO).

Utilization of upper-layer indications.  Applications should

depend on upper-layer indications such as IP address

configuration/change notification, rather than utilizing link

indications such as "Link Up".

IAB                          Informational                     [Page 20]

RFC 4907                    Link Indications                   June 2007

Keepalives.  In order to improve robustness against spurious link

indications, an application keepalive or transport layer

indication (such as connection teardown) can be used instead of

consuming "Link Down" indications.

Conservation of resources.  Proposals must demonstrate that they

are not vulnerable to congestive collapse.

As noted in "Robust Rate Adaptation for 802.11 Wireless Networks"

[Robust], decreasing transmission rate in response to frame loss

increases contention, potentially leading to congestive collapse.  To

avoid this, the link layer needs to distinguish frame loss due to

congestion from loss due to channel conditions.  Only frame loss due

to deterioration in channel conditions can be used as a basis for

decreasing transmission rate.

Consider a proposal where a "Link Up" indication is used by a host to

trigger retransmission of the last previously sent packet, in order

to enable ACK reception prior to expiration of the host's

retransmission timer.  On a rapidly moving mobile node where "Link

Up" indications follow in rapid succession, this could result in a

burst of retransmitted packets, violating the principle of

"conservation of packets".

At the application layer, link indications have been utilized by

applications such as Presence [RFC2778] in order to optimize

registration and user interface update operations.  For example,

implementations may attempt presence registration on receipt of a

"Link Up" indication, and presence de-registration by a surrogate

receiving a "Link Down" indication.  Presence implementations using

"Link Up"/"Link Down" indications this way violate the principle of

"conservation of packets" since link indications can be generated on

a time scale less than the end-to-end path RTO.  The problem is

magnified since for each presence update, notifications can be

delivered to many watchers.  In addition, use of a "Link Up"

indication in this manner is unwise since the interface may not yet

even have an operable Internet layer configuration.  Instead, an "IP

address configured" indication may be utilized.

2.5.  Effectiveness

Proposals must demonstrate the effectiveness of proposed

optimizations.  Since optimizations typically increase complexity,

substantial performance improvement is required in order to make a

compelling case.

IAB                          Informational                     [Page 21]

RFC 4907                    Link Indications                   June 2007

In the face of unreliable link indications, effectiveness may depend

on the penalty for false positives and false negatives.  In the case

of DNAv4 [RFC4436], the benefits of successful optimization are

modest, but the penalty for being unable to confirm an operable

configuration is a lengthy timeout.  As a result, the recommended

strategy is to test multiple potential configurations in parallel in

addition to attempting configuration via DHCP.  This virtually

guarantees that DNAv4 will always result in performance equal to or

better than use of DHCP alone.

2.6.  Interoperability

While link indications can be utilized where available, they should

not be required by upper layers, in order to maintain link layer

independence.  For example, if information on supported prefixes is

provided at the link layer, hosts not understanding those hints must

still be able to obtain an IP address.

Where link indications are proposed to optimize Internet layer

configuration, proposals must demonstrate that they do not compromise

robustness by interfering with address assignment or routing protocol

behavior, making address collisions more likely, or compromising

Duplicate Address Detection (DAD) [RFC4429].

To avoid compromising interoperability in the pursuit of performance

optimization, proposals must demonstrate that interoperability

remains possible (potentially with degraded performance) even if one

or more participants do not implement the proposal.

2.7.  Race Conditions

Link indication proposals should avoid race conditions, which can

occur where link indications are utilized directly by multiple layers

of the stack.

Link indications are useful for optimization of Internet Protocol

layer addressing and configuration as well as routing.  Although "The

BU-trigger method for improving TCP performance over Mobile IPv6"

[Kim] describes situations in which link indications are first

processed by the Internet Protocol layer (e.g., MIPv6) before being

utilized by the transport layer, for the purposes of parameter

estimation, it may be desirable for the transport layer to utilize

link indications directly.

In situations where the Weak End System model is implemented, a

change of outgoing interface may occur at the same time the transport

layer is modifying transport parameters based on other link

IAB                          Informational                     [Page 22]

RFC 4907                    Link Indications                   June 2007

indications.  As a result, transport behavior may differ depending on

the order in which the link indications are processed.

Where a multi-homed host experiences increasing frame loss or

decreased rate on one of its interfaces, a routing metric taking

these effects into account will increase, potentially causing a

change in the outgoing interface for one or more transport

connections.  This may trigger Mobile IP signaling so as to cause a

change in the incoming path as well.  As a result, the transport

parameters estimated for the original outgoing and incoming paths

(congestion state, Maximum Segment Size (MSS) derived from the link

maximum transmission unit (MTU) or Path MTU) may no longer be valid

for the new outgoing and incoming paths.

To avoid race conditions, the following measures are recommended:

Path change re-estimation

Layering

Metric consistency

2.7.1.  Path Change Re-estimation

When the Internet layer detects a path change, such as a major change

in transmission rate, a change in the outgoing or incoming interface

of the host or the incoming interface of a peer, or perhaps even a

substantial change in the IPv4 TTL/IPv6 Hop Limit of received

packets, it may be worth considering whether to reset transport

parameters (RTT, RTO, cwnd, bw, MSS) to their initial values so as to

allow them to be re-estimated.  This ensures that estimates based on

the former path do not persist after they have become invalid.

Appendix A.3 summarizes the research on this topic.

2.7.2.  Layering

Another technique to avoid race conditions is to rely on layering to

damp transient link indications and provide greater link layer

independence.

The Internet layer is responsible for routing as well as IP

configuration and mobility, providing higher layers with an

abstraction that is independent of link layer technologies.

In general, it is advisable for applications to utilize indications

from the Internet or transport layers rather than consuming link

indications directly.

IAB                          Informational                     [Page 23]

RFC 4907                    Link Indications                   June 2007

2.7.3.  Metric Consistency

Proposals should avoid inconsistencies between link and routing layer

metrics.  Without careful design, potential differences between link

indications used in routing and those used in roaming and/or link

enablement can result in instability, particularly in multi-homed

hosts.

Once a link is in the "up" state, its effectiveness in transmission

of data packets can be used to determine an appropriate routing

metric.  In situations where the transmission time represents a large

portion of the total transit time, minimizing total transmission time

is equivalent to maximizing effective throughput.  "A High-Throughput

Path Metric for Multi-Hop Wireless Routing" [ETX] describes a

proposed routing metric based on the Expected Transmission Count

(ETX).  The authors demonstrate that ETX, based on link layer frame

loss rates (prior to retransmission), enables the selection of routes

maximizing effective throughput.  Where the transmission rate is

constant, the expected transmission time is proportional to ETX, so

that minimizing ETX also minimizes expected transmission time.

However, where the transmission rate may vary, ETX may not represent

a good estimate of the estimated transmission time.  In "Routing in

multi-radio, multi-hop wireless mesh networks" [ETX-Rate], the

authors define a new metric called Expected Transmission Time (ETT).

This is described as a "bandwidth adjusted ETX" since ETT = ETX * S/B

where S is the size of the probe packet and B is the bandwidth of the

link as measured by a packet pair [Morgan].  However, ETT assumes

that the loss fraction of small probe frames sent at 1 Mbps data rate

is indicative of the loss fraction of larger data frames at higher

rates, which tends to underestimate the ETT at higher rates, where

frame loss typically increases.  In "A Radio Aware Routing Protocol

for Wireless Mesh Networks" [ETX-Radio], the authors refine the ETT

metric further by estimating the loss fraction as a function of

transmission rate.

However, prior to sending data packets over the link, the appropriate

routing metric may not easily be predicted.  As noted in [Shortest],

a link that can successfully transmit the short frames utilized for

control, management, or routing may not necessarily be able to

reliably transport larger data packets.

Therefore, it may be necessary to utilize alternative metrics (such

as signal strength or Access Point load) in order to assist in

attachment/handoff decisions.  However, unless the new interface is

the preferred route for one or more destination prefixes, a Weak End

System implementation will not use the new interface for outgoing

traffic.  Where "idle timeout" functionality is implemented, the

IAB                          Informational                     [Page 24]

RFC 4907                    Link Indications                   June 2007

unused interface will be brought down, only to be brought up again by

the link enablement algorithm.

Within the link layer, metrics such as signal strength and frame loss

may be used to determine the transmission rate, as well as to

determine when to select an alternative point of attachment.  In

order to enable stations to roam prior to encountering packet loss,

studies such as "An experimental study of IEEE 802.11b handover

performance and its effect on voice traffic" [Vatn] have suggested

using signal strength as a mechanism to more rapidly detect loss of

connectivity, rather than frame loss, as suggested in "Techniques to

Reduce IEEE 802.11b MAC Layer Handover Time" [Velayos].

[Aguayo] notes that signal strength and distance are not good

predictors of frame loss or throughput, due to the potential effects

of multi-path interference.  As a result, a link brought up due to

good signal strength may subsequently exhibit significant frame loss

and a low throughput.  Similarly, an Access Point (AP) demonstrating

low utilization may not necessarily be the best choice, since

utilization may be low due to hardware or software problems.  "OSPF

Optimized Multipath (OSPF-OMP)" [Villamizar] notes that link-

utilization-based routing metrics have a history of instability.

2.8.  Layer Compression

In many situations, the exchanges required for a host to complete a

handoff and reestablish connectivity are considerable, leading to

proposals to combine exchanges occurring within multiple layers in

order to reduce overhead.  While overhead reduction is a laudable

goal, proposals need to avoid compromising interoperability and

introducing link layer dependencies into the Internet and transport

layers.

Exchanges required for handoff and connectivity reestablishment may

include link layer scanning, authentication, and association

establishment; Internet layer configuration, routing, and mobility

exchanges; transport layer retransmission and recovery; security

association reestablishment; application protocol re-authentication

and re-registration exchanges, etc.

Several proposals involve combining exchanges within the link layer.

For example, in [EAPIKEv2], a link layer Extensible Authentication

Protocol (EAP) [RFC3748] exchange may be used for the purpose of IP

address assignment, potentially bypassing Internet layer

configuration.  Within [PEAP], it is proposed that a link layer EAP

exchange be used for the purpose of carrying Mobile IPv6 Binding

Updates.  [MIPEAP] proposes that EAP exchanges be used for

configuration of Mobile IPv6.  Where link, Internet, or transport

IAB                          Informational                     [Page 25]

RFC 4907                    Link Indications                   June 2007

layer mechanisms are combined, hosts need to maintain backward

compatibility to permit operation on networks where compression

schemes are not available.

Layer compression schemes may also negatively impact robustness.  For

example, in order to optimize IP address assignment, it has been

proposed that prefixes be advertised at the link layer, such as

within the 802.11 Beacon and Probe Response frames.  However,

[IEEE-802.1X] enables the Virtual LAN Identifier (VLANID) to be

assigned dynamically, so that prefix(es) advertised within the Beacon

and/or Probe Response may not correspond to the prefix(es) configured

by the Internet layer after the host completes link layer

authentication.  Were the host to handle IP configuration at the link

layer rather than within the Internet layer, the host might be unable

to communicate due to assignment of the wrong IP address.

2.9.  Transport of Link Indications

Proposals for the transport of link indications need to carefully

consider the layering, security, and transport implications.

As noted earlier, the transport layer may take the state of the local

routing table into account in improving the quality of transport

parameter estimates.  While absence of positive feedback that the

path is sending data end-to-end must be heeded, where a route that

had previously been absent is recovered, this may be used to trigger

congestion control probing.  While this enables transported link

indications that affect the local routing table to improve the

quality of transport parameter estimates, security and

interoperability considerations relating to routing protocols still

apply.

Proposals involving transport of link indications need to demonstrate

the following:

(a)  Superiority to implicit signals.  In general, implicit signals

are preferred to explicit transport of link indications since

they do not require participation in the routing mesh, add no

new packets in times of network distress, operate more reliably

in the presence of middle boxes such as NA(P)Ts, are more likely

to be backward compatible, and are less likely to result in

security vulnerabilities.  As a result, explicit signaling

proposals must prove that implicit signals are inadequate.

(b)  Mitigation of security vulnerabilities.  Transported link

indications should not introduce new security vulnerabilities.

Link indications that result in modifications to the local

routing table represent a routing protocol, so that the

IAB                          Informational                     [Page 26]

RFC 4907                    Link Indications                   June 2007

vulnerabilities associated with unsecured routing protocols

apply, including spoofing by off-link attackers.  While

mechanisms such as "SEcure Neighbor Discovery (SEND)" [RFC3971]

may enable authentication and integrity protection of router-

originated messages, protecting against forgery of transported

link indications, they are not yet widely deployed.

(c)  Validation of transported indications.  Even if a transported

link indication can be integrity protected and authenticated, if

the indication is sent by a host off the local link, it may not

be clear that the sender is on the actual path in use, or which

transport connection(s) the indication relates to.  Proposals

need to describe how the receiving host can validate the

transported link indication.

(d)  Mapping of Identifiers.  When link indications are transported,

it is generally for the purposes of providing information about

Internet, transport, or application layer operations at a remote

element.  However, application layer sessions or transport

connections may not be visible to the remote element due to

factors such as load sharing between links, or use of IPsec,

tunneling protocols, or nested headers.  As a result, proposals

need to demonstrate how the link indication can be mapped to the

relevant higher-layer state.  For example, on receipt of a link

indication, the transport layer will need to identify the set of

transport sessions (source address, destination address, source

port, destination port, transport) that are affected.  If a

presence server is receiving remote indications of "Link

Up"/"Link Down" status for a particular Media Access Control

(MAC) address, the presence server will need to associate that

MAC address with the identity of the user

(pres:user@example.com) to whom that link status change is

relevant.

3.  Future Work

Further work is needed in order to understand how link indications

can be utilized by the Internet, transport, and application layers.

More work is needed to understand the connection between link

indications and routing metrics.  For example, the introduction of

block ACKs (supported in [IEEE-802.11e]) complicates the relationship

between effective throughput and frame loss, which may necessitate

the development of revised routing metrics for ad-hoc networks.  More

work is also needed to reconcile handoff metrics (e.g., signal

strength and link utilization) with routing metrics based on link

indications (e.g., frame error rate and negotiated rate).

IAB                          Informational                     [Page 27]

RFC 4907                    Link Indications                   June 2007

A better understanding of the use of physical and link layer metrics

in rate negotiation is required.  For example, recent work

[Robust][CARA] has suggested that frame loss due to contention (which

would be exacerbated by rate reduction) can be distinguished from

loss due to channel conditions (which may be improved via rate

reduction).

At the transport layer, more work is needed to determine the

appropriate reaction to Internet layer indications such as routing

table and path changes.  More work is also needed in utilization of

link layer indications in transport parameter estimation, including

rate changes, "Link Up"/"Link Down" indications, link layer

retransmissions, and frame loss of various types (due to contention

or channel conditions).

More work is also needed to determine how link layers may utilize

information from the transport layer.  For example, it is undesirable

for a link layer to retransmit so aggressively that the link layer

round-trip time approaches that of the end-to-end transport

connection.  Instead, it may make sense to do downward rate

adjustment so as to decrease frame loss and improve latency.  Also,

in some cases, the transport layer may not require heroic efforts to

avoid frame loss; timely delivery may be preferred instead.

4.  Security Considerations

Proposals for the utilization of link indications may introduce new

security vulnerabilities.  These include:

Spoofing

Indication validation

Denial of service

4.1.  Spoofing

Where link layer control frames are unprotected, they may be spoofed

by an attacker.  For example, PPP does not protect LCP frames such as

LCP-Terminate, and [IEEE-802.11] does not protect management frames

such as Associate/Reassociate, Disassociate, or Deauthenticate.

Spoofing of link layer control traffic may enable attackers to

exploit weaknesses in link indication proposals.  For example,

proposals that do not implement congestion avoidance can enable

attackers to mount denial-of-service attacks.

However, even where the link layer incorporates security, attacks may

still be possible if the security model is not consistent.  For

example, wireless LANs implementing [IEEE-802.11i] do not enable

IAB                          Informational                     [Page 28]

RFC 4907                    Link Indications                   June 2007

stations to send or receive IP packets on the link until completion

of an authenticated key exchange protocol known as the "4-way

handshake".  As a result, a link implementing [IEEE-802.11i] cannot

be considered usable at the Internet layer ("Link Up") until

completion of the authenticated key exchange.

However, while [IEEE-802.11i] requires sending of authenticated

frames in order to obtain a "Link Up" indication, it does not support

management frame authentication.  This weakness can be exploited by

attackers to enable denial-of-service attacks on stations attached to

distant Access Points (APs).

In [IEEE-802.11F], "Link Up" is considered to occur when an AP sends

a Reassociation Response.  At that point, the AP sends a spoofed

frame with the station's source address to a multicast address,

thereby causing switches within the Distribution System (DS) to learn

the station's MAC address.  While this enables forwarding of frames

to the station at the new point of attachment, it also permits an

attacker to disassociate a station located anywhere within the ESS,

by sending an unauthenticated Reassociation Request frame.

4.2.  Indication Validation

"Fault Isolation and Recovery" [RFC816], Section 3, describes how

hosts interact with routers for the purpose of fault recovery:

Since the gateways always attempt to have a consistent and correct

model of the internetwork topology, the host strategy for fault

recovery is very simple.  Whenever the host feels that something is

wrong, it asks the gateway for advice, and, assuming the advice is

forthcoming, it believes the advice completely.  The advice will be

wrong only during the transient period of negotiation, which

immediately follows an outage, but will otherwise be reliably

correct.

In fact, it is never necessary for a host to explicitly ask a gateway

for advice, because the gateway will provide it as appropriate.  When

a host sends a datagram to some distant net, the host should be

prepared to receive back either of two advisory messages which the

gateway may send.  The ICMP "redirect" message indicates that the

gateway to which the host sent the datagram is no longer the best

gateway to reach the net in question.  The gateway will have

forwarded the datagram, but the host should revise its routing table

to have a different immediate address for this net.  The ICMP

"destination unreachable" message indicates that as a result of an

outage, it is currently impossible to reach the addressed net or host

IAB                          Informational                     [Page 29]

RFC 4907                    Link Indications                   June 2007

in any manner.  On receipt of this message, a host can either abandon

the connection immediately without any further retransmission, or

resend slowly to see if the fault is corrected in reasonable time.

Given today's security environment, it is inadvisable for hosts to

act on indications provided by routers without careful consideration.

As noted in "ICMP attacks against TCP" [Gont], existing ICMP error

messages may be exploited by attackers in order to abort connections

in progress, prevent setup of new connections, or reduce throughput

of ongoing connections.  Similar attacks may also be launched against

the Internet layer via forging of ICMP redirects.

Proposals for transported link indications need to demonstrate that

they will not add a new set of similar vulnerabilities.  Since

transported link indications are typically unauthenticated, hosts

receiving them may not be able to determine whether they are

authentic, or even plausible.

Where link indication proposals may respond to unauthenticated link

layer frames, they should utilize upper-layer security mechanisms,

where possible.  For example, even though a host might utilize an

unauthenticated link layer control frame to conclude that a link has

become operational, it can use SEND [RFC3971] or authenticated DHCP

[RFC3118] in order to obtain secure Internet layer configuration.

4.3.  Denial of Service

Link indication proposals need to be particularly careful to avoid

enabling denial-of-service attacks that can be mounted at a distance.

While wireless links are naturally vulnerable to interference, such

attacks can only be perpetrated by an attacker capable of

establishing radio contact with the target network.  However, attacks

that can be mounted from a distance, either by an attacker on another

point of attachment within the same network or by an off-link

attacker, expand the level of vulnerability.

The transport of link indications can increase risk by enabling

vulnerabilities exploitable only by attackers on the local link to be

executed across the Internet.  Similarly, by integrating link

indications with upper layers, proposals may enable a spoofed link

layer frame to consume more resources on the host than might

otherwise be the case.  As a result, while it is important for upper

layers to validate link indications, they should not expend excessive

resources in doing so.

Congestion control is not only a transport issue, it is also a

security issue.  In order to not provide leverage to an attacker, a

single forged link layer frame should not elicit a magnified response

IAB                          Informational                     [Page 30]

RFC 4907                    Link Indications                   June 2007

from one or more hosts, by generating either multiple responses or a

single larger response.  For example, proposals should not enable

multiple hosts to respond to a frame with a multicast destination

address.

5.  References

5.1.  Normative References

[RFC2119]      Bradner, S., "Key words for use in RFCs to Indicate

Requirement Levels", BCP 14, RFC 2119, March 1997.

5.2.  Informative References

[RFC816]       Clark, D., "Fault Isolation and Recovery", RFC 816,

July 1982.

[RFC1058]      Hedrick, C., "Routing Information Protocol", RFC 1058,

June 1988.

[RFC1122]      Braden, R., "Requirements for Internet Hosts --

Communication Layers", STD 3, RFC 1122, October 1989.

[RFC1131]      Moy, J., "The OSPF Specification", RFC 1131, October

1989.

[RFC1191]      Mogul, J. and S. Deering, "Path MTU discovery", RFC

1191, November 1990.

[RFC1256]      Deering, S., "ICMP Router Discovery Messages", RFC

1256, September 1991.

[RFC1305]      Mills, D., "Network Time Protocol (Version 3)

Specification, Implementation and Analysis", RFC 1305,

March 1992.

[RFC1307]      Young, J. and A. Nicholson, "Dynamically Switched Link

Control Protocol", RFC 1307, March 1992.

[RFC1661]      Simpson, W., "The Point-to-Point Protocol (PPP)", STD

51, RFC 1661, July 1994.

[RFC1812]      Baker, F., "Requirements for IP Version 4 Routers",

RFC 1812, June 1995.

[RFC1918]      Rekhter, Y., Moskowitz, B., Karrenberg, D., de Groot,

D., and E. Lear, "Address Allocation for Private

Internets", BCP 5, RFC 1918, February 1996.

IAB                          Informational                     [Page 31]

RFC 4907                    Link Indications                   June 2007

[RFC1981]      McCann, J., Deering, S. and J. Mogul, "Path MTU

Discovery for IP version 6", RFC 1981, June 1996.

[RFC2131]      Droms, R., "Dynamic Host Configuration Protocol", RFC

2131, March 1997.

[RFC2328]      Moy, J., "OSPF Version 2", STD 54, RFC 2328, April

1998.

[RFC2461]      Narten, T., Nordmark, E., and W. Simpson, "Neighbor

Discovery for IP Version 6 (IPv6)", RFC 2461, December

1998.

[RFC2778]      Day, M., Rosenberg, J., and H. Sugano, "A Model for

Presence and Instant Messaging", RFC 2778, February

2000.

[RFC2861]      Handley, M., Padhye, J., and S. Floyd, "TCP Congestion

Window Validation", RFC 2861, June 2000.

[RFC2914]      Floyd, S., "Congestion Control Principles", RFC 2914,

BCP 41, September 2000.

[RFC2923]      Lahey, K., "TCP Problems with Path MTU Discovery", RFC

2923, September 2000.

[RFC2960]      Stewart, R., Xie, Q., Morneault, K., Sharp, C.,

Schwarzbauer, H. Taylor, T., Rytina, I., Kalla, M.,

Zhang, L., and V. Paxson, "Stream Control Transmission

Protocol" RFC 2960, October 2000.

[RFC3118]      Droms, R. and B. Arbaugh, "Authentication for DHCP

Messages", RFC 3118, June 2001.

[RFC3315]      Droms, R., Bound, J., Volz, B., Lemon, T., Perkins,

C., and M. Carney, "Dynamic Host Configuration

Protocol for IPv6 (DHCPv6)", RFC 3315, July 2003.

[RFC3366]      Fairhurst, G. and L. Wood, "Advice to link designers

on link Automatic Repeat reQuest (ARQ)", BCP 62, RFC

3366, August 2002.

[RFC3428]      Campbell, B., Rosenberg, J., Schulzrinne, H., Huitema,

C., and D. Gurle, "Session Initiation Protocol (SIP)

Extension for Instant Messaging", RFC 3428, December

2002.

IAB                          Informational                     [Page 32]

RFC 4907                    Link Indications                   June 2007

[RFC3748]      Aboba, B., Blunk, L., Vollbrecht, J., Carlson, J., and

H. Levkowetz, "Extensible Authentication Protocol

(EAP)", RFC 3748, June 2004.

[RFC3775]      Johnson, D., Perkins, C., and J. Arkko, "Mobility

Support in IPv6", RFC 3775, June 2004.

[RFC3921]      Saint-Andre, P., "Extensible Messaging and Presence

protocol (XMPP):  Instant Messaging and Presence", RFC

3921, October 2004.

[RFC3927]      Cheshire, S., Aboba, B., and E. Guttman, "Dynamic

Configuration of Link-Local IPv4 Addresses", RFC 3927,

May 2005.

[RFC3971]      Arkko, J., Kempf, J., Zill, B., and P. Nikander,

"SEcure Neighbor Discovery (SEND)", RFC 3971, March

2005.

[RFC4340]      Kohler, E., Handley, M., and S. Floyd, "Datagram

Congestion Control Protocol (DCCP)", RFC 4340, March

2006.

[RFC4423]      Moskowitz, R. and P. Nikander, "Host Identity Protocol

(HIP) Architecture", RFC 4423, May 2006.

[RFC4429]      Moore, N., "Optimistic Duplicate Address Detection

(DAD) for IPv6", RFC 4429, April 2006.

[RFC4436]      Aboba, B., Carlson, J., and S. Cheshire, "Detecting

Network Attachment in IPv4 (DNAv4)", RFC 4436, March

2006.

[RFC4821]      Mathis, M. and J. Heffner, "Packetization Layer Path

MTU Discovery", RFC 4821, March 2007.

[Alimian]      Alimian, A., "Roaming Interval Measurements",

11-04-0378-00-roaming-intervals-measurements.ppt, IEEE

802.11 submission (work in progress), March 2004.

[Aguayo]       Aguayo, D., Bicket, J., Biswas, S., Judd, G., and R.

Morris, "Link-level Measurements from an 802.11b Mesh

Network", SIGCOMM '04, September 2004, Portland,

Oregon.

IAB                          Informational                     [Page 33]

RFC 4907                    Link Indications                   June 2007

[Bakshi]       Bakshi, B., Krishna, P., Vadiya, N., and D.Pradhan,

"Improving Performance of TCP over Wireless Networks",

Proceedings of the 1997 International Conference on

Distributed Computer Systems, Baltimore, May 1997.

[BFD]          Katz, D. and D. Ward, "Bidirectional Forwarding

Detection", Work in Progress, March 2007.

[Biaz]         Biaz, S. and N. Vaidya, "Discriminating Congestion

Losses from Wireless Losses Using Interarrival Times

at the Receiver", Proceedings of the IEEE Symposium on

Application-Specific Systems and Software Engineering

and Technology, Richardson, TX, Mar 1999.

[CARA]         Kim, J., Kim, S., and S. Choi, "CARA: Collision-Aware

Rate Adaptation for IEEE 802.11 WLANs", Korean

Institute of Communication Sciences (KICS) Journal,

Feb. 2006

[Chandran]     Chandran, K., Raghunathan, S., Venkatesan, S., and R.

Prakash, "A Feedback-Based Scheme for Improving TCP

Performance in Ad-Hoc Wireless Networks", Proceedings

of the 18th International Conference on Distributed

Computing Systems (ICDCS), Amsterdam, May 1998.

[DNAv6]        Narayanan, S., "Detecting Network Attachment in IPv6

(DNAv6)", Work in Progress, March 2007.

[E2ELinkup]    Dawkins, S. and C. Williams, "End-to-end, Implicit

'Link-Up' Notification", Work in Progress, October

2003.

[EAPIKEv2]     Tschofenig, H., Kroeselberg, D., Pashalidis, A., Ohba,

Y., and F. Bersani, "EAP IKEv2 Method", Work in

Progress, March 2007.

[Eckhardt]     Eckhardt, D. and P. Steenkiste, "Measurement and

Analysis of the Error Characteristics of an In-

Building Wireless Network", SIGCOMM '96, August 1996,

Stanford, CA.

[Eddy]         Eddy, W. and Y. Swami, "Adapting End Host Congestion

Control for Mobility", Technical Report CR-2005-

213838, NASA Glenn Research Center, July 2005.

IAB                          Informational                     [Page 34]

RFC 4907                    Link Indications                   June 2007

[EfficientEthernet]

Gunaratne, C. and K. Christensen, "Ethernet Adaptive

Link Rate: System Design and Performance Evaluation",

Proceedings of the IEEE Conference on Local Computer

Networks, pp. 28-35, November 2006.

[Eggert]       Eggert, L., Schuetz, S., and S. Schmid, "TCP

Extensions for Immediate Retransmissions", Work in

Progress, June 2005.

[Eggert2]      Eggert, L. and W. Eddy, "Towards More Expressive

Transport-Layer Interfaces", MobiArch '06, San

Francisco, CA.

[ETX]          Douglas S. J. De Couto, Daniel Aguayo, John Bicket,

and Robert Morris, "A High-Throughput Path Metric for

Multi-Hop Wireless Routing", Proceedings of the 9th

ACM International Conference on Mobile Computing and

Networking (MobiCom '03), San Diego, California,

September 2003.

[ETX-Rate]     Padhye, J., Draves, R. and B. Zill, "Routing in

multi-radio, multi-hop wireless mesh networks",

Proceedings of ACM MobiCom Conference, September 2003.

[ETX-Radio]    Kulkarni, G., Nandan, A., Gerla, M., and M.

Srivastava, "A Radio Aware Routing Protocol for

Wireless Mesh Networks", UCLA Computer Science

Department, Los Angeles, CA.

[GenTrig]      Gupta, V. and D. Johnston, "A Generalized Model for

Link Layer Triggers", submission to IEEE 802.21 (work

in progress), March 2004, available at:

<http://www.ieee802.org/handoff/march04_meeting_docs/

Generalized_triggers-02.pdf>.

[Goel]         Goel, S. and D. Sanghi, "Improving TCP Performance

over Wireless Links", Proceedings of TENCON'98, pages

332-335.  IEEE, December 1998.

[Gont]         Gont, F., "ICMP attacks against TCP", Work in

Progress, October 2006.

[Gurtov]       Gurtov, A. and J. Korhonen, "Effect of Vertical

Handovers on Performance of TCP-Friendly Rate

Control", to appear in ACM MCCR, 2004.

IAB                          Informational                     [Page 35]

RFC 4907                    Link Indications                   June 2007

[GurtovFloyd]  Gurtov, A. and S. Floyd, "Modeling Wireless Links for

Transport Protocols", Computer Communications Review

(CCR) 34, 2 (2003).

[Haratcherev]  Haratcherev, I., Lagendijk, R., Langendoen, K., and H.

Sips, "Hybrid Rate Control for IEEE 802.11", MobiWac

'04, October 1, 2004, Philadelphia, Pennsylvania, USA.

[Haratcherev2] Haratcherev, I., "Application-oriented Link Adaptation

for IEEE 802.11", Ph.D. Thesis, Technical University

of Delft, Netherlands, ISBN-10:90-9020513-6, ISBN-

13:978-90-9020513-7, March 2006.

[HMP]          Lee, S., Cho, J., and A. Campbell, "Hotspot Mitigation

Protocol (HMP)", Work in Progress, October 2003.

[Holland]      Holland, G. and N. Vaidya, "Analysis of TCP

Performance over Mobile Ad Hoc Networks", Proceedings

of the Fifth International Conference on Mobile

Computing and Networking, pages 219-230.  ACM/IEEE,

Seattle, August 1999.

[Iannaccone]   Iannaccone, G., Chuah, C., Mortier, R., Bhattacharyya,

S., and C. Diot, "Analysis of link failures in an IP

backbone", Proc. of ACM Sigcomm Internet Measurement

Workshop, November, 2002.

[IEEE-802.1X]  Institute of Electrical and Electronics Engineers,

"Local and Metropolitan Area Networks: Port-Based

Network Access Control", IEEE Standard 802.1X,

December 2004.

[IEEE-802.11]  Institute of Electrical and Electronics Engineers,

"Wireless LAN Medium Access Control (MAC) and Physical

Layer (PHY) Specifications", IEEE Standard 802.11,

2003.

[IEEE-802.11e] Institute of Electrical and Electronics Engineers,

"Standard for Telecommunications and Information

Exchange Between Systems - LAN/MAN Specific

Requirements - Part 11: Wireless LAN Medium Access

Control (MAC) and Physical Layer (PHY) Specifications

- Amendment 8: Medium Access Control (MAC) Quality of

Service Enhancements", IEEE 802.11e, November 2005.

IAB                          Informational                     [Page 36]

RFC 4907                    Link Indications                   June 2007

[IEEE-802.11F] Institute of Electrical and Electronics Engineers,

"IEEE Trial-Use Recommended Practice for Multi-Vendor

Access Point Interoperability via an Inter-Access

Point Protocol Across Distribution Systems Supporting

IEEE 802.11 Operation", IEEE 802.11F, June 2003 (now

deprecated).

[IEEE-802.11i] Institute of Electrical and Electronics Engineers,

"Supplement to Standard for Telecommunications and

Information Exchange Between Systems - LAN/MAN

Specific Requirements - Part 11:  Wireless LAN Medium

Access Control (MAC) and Physical Layer (PHY)

Specifications: Specification for Enhanced Security",

IEEE 802.11i, July 2004.

[IEEE-802.11k] Institute of Electrical and Electronics Engineers,

"Draft Amendment to Telecommunications and Information

Exchange Between Systems - LAN/MAN Specific

Requirements - Part 11:  Wireless LAN Medium Access

Control (MAC) and Physical Layer (PHY) Specifications

- Amendment 7: Radio Resource Management", IEEE

802.11k/D7.0, January 2007.

[IEEE-802.21]  Institute of Electrical and Electronics Engineers,

"Draft Standard for Telecommunications and Information

Exchange Between Systems - LAN/MAN Specific

Requirements - Part 21:  Media Independent Handover",

IEEE 802.21D0, June 2005.

[Kamerman]     Kamerman, A. and L. Monteban, "WaveLAN II: A High-

Performance Wireless LAN for the Unlicensed Band",

Bell Labs Technical Journal, Summer 1997.

[Kim]          Kim, K., Park, Y., Suh, K., and Y. Park, "The BU-

trigger method for improving TCP performance over

Mobile IPv6", Work in Progress, August 2004.

[Kotz]         Kotz, D., Newport, C., and C. Elliot, "The mistaken

axioms of wireless-network research", Dartmouth

College Computer Science Technical Report TR2003-467,

July 2003.

[Krishnan]     Krishnan, R., Sterbenz, J., Eddy, W., Partridge, C.,

and M. Allman, "Explicit Transport Error Notification

(ETEN) for Error-Prone Wireless and Satellite

Networks", Computer Networks, 46 (3), October 2004.

IAB                          Informational                     [Page 37]

RFC 4907                    Link Indications                   June 2007

[Lacage]       Lacage, M., Manshaei, M., and T. Turletti, "IEEE

802.11 Rate Adaptation: A Practical Approach", MSWiM

'04, October 4-6, 2004, Venezia, Italy.

[Lee]          Park, S., Lee, M., and J. Korhonen, "Link

Characteristics Information for Mobile IP", Work in

Progress, January 2007.

[Ludwig]       Ludwig, R. and B. Rathonyi, "Link-layer Enhancements

for TCP/IP over GSM", Proceedings of IEEE Infocom '99,

March 1999.

[MIPEAP]       Giaretta, C., Guardini, I., Demaria, E., Bournelle,

J., and M. Laurent-Maknavicius, "MIPv6 Authorization

and Configuration based on EAP", Work in Progress,

October 2006.

[Mishra]       Mitra, A., Shin, M., and W. Arbaugh, "An Empirical

Analysis of the IEEE 802.11 MAC Layer Handoff

Process", CS-TR-4395, University of Maryland

Department of Computer Science, September 2002.

[Morgan]       Morgan, S. and S. Keshav, "Packet-Pair Rate Control -

Buffer Requirements and Overload Performance",

Technical Memorandum, AT&T Bell Laboratories, October

1994.

[Mun]          Mun, Y. and J. Park, "Layer 2 Handoff for Mobile-IPv4

with 802.11", Work in Progress, March 2004.

[ONOE]         Onoe Rate Control,

<http://madwifi.org/browser/trunk/ath_rate/onoe>.

[Park]         Park, S., Njedjou, E., and N. Montavont, "L2 Triggers

Optimized Mobile IPv6 Vertical Handover: The

802.11/GPRS Example", Work in Progress, July 2004.

[Pavon]        Pavon, J. and S. Choi, "Link adaptation strategy for

IEEE802.11 WLAN via received signal strength

measurement", IEEE International Conference on

Communications, 2003 (ICC '03), volume 2, pages 1108-

1113, Anchorage, Alaska, USA, May 2003.

[PEAP]         Palekar, A., Simon, D., Salowey, J., Zhou, H., Zorn,

G., and S. Josefsson, "Protected EAP Protocol (PEAP)

Version 2", Work in Progress, October 2004.

IAB                          Informational                     [Page 38]

RFC 4907                    Link Indications                   June 2007

[PRNET]        Jubin, J. and J. Tornow, "The DARPA packet radio

network protocols", Proceedings of the IEEE, 75(1),

January 1987.

[Qiao]         Qiao D., Choi, S., Jain, A., and Kang G. Shin, "MiSer:

An Optimal Low-Energy Transmission Strategy for IEEE

802.11 a/h", in Proc. ACM MobiCom'03, San Diego, CA,

September 2003.

[RBAR]         Holland, G., Vaidya, N., and P. Bahl, "A Rate-Adaptive

MAC Protocol for Multi-Hop Wireless Networks",

Proceedings ACM MOBICOM, July 2001.

[Ramani]       Ramani, I. and S. Savage, "SyncScan: Practical Fast

Handoff for 802.11 Infrastructure Networks",

Proceedings of the IEEE InfoCon 2005, March 2005.

[Robust]       Wong, S., Yang, H ., Lu, S., and V. Bharghavan,

"Robust Rate Adaptation for 802.11 Wireless Networks",

ACM MobiCom'06, Los Angeles, CA, September 2006.

[SampleRate]   Bicket, J., "Bit-rate Selection in Wireless networks",

MIT Master's Thesis, 2005.

[Scott]        Scott, J., Mapp, G., "Link Layer Based TCP

Optimisation for Disconnecting Networks", ACM SIGCOMM

Computer Communication Review, 33(5), October 2003.

[Schuetz]      Schutz, S., Eggert, L., Schmid, S., and M. Brunner,

"Protocol Enhancements for Intermittently Connected

Hosts", ACM SIGCOMM Computer Communications Review,

Volume 35, Number 2, July 2005.

[Shortest]     Douglas S. J. De Couto, Daniel Aguayo, Benjamin A.

Chambers and Robert Morris, "Performance of Multihop

Wireless Networks: Shortest Path is Not Enough",

Proceedings of the First Workshop on Hot Topics in

Networking (HotNets-I), Princeton, New Jersey, October

2002.

[TRIGTRAN]     Dawkins, S., Williams, C., and A. Yegin, "Framework

and Requirements for TRIGTRAN", Work in Progress,

August 2003.

[Vatn]         Vatn, J., "An experimental study of IEEE 802.11b

handover performance and its effect on voice traffic",

TRITA-IMIT-TSLAB R 03:01, KTH Royal Institute of

Technology, Stockholm, Sweden, July 2003.

IAB                          Informational                     [Page 39]

RFC 4907                    Link Indications                   June 2007

[Velayos]      Velayos, H. and G. Karlsson, "Techniques to Reduce

IEEE 802.11b MAC Layer Handover Time", TRITA-IMIT-LCN

R 03:02, KTH Royal Institute of Technology, Stockholm,

Sweden, April 2003.

[Vertical]     Zhang, Q., Guo, C., Guo, Z., and W. Zhu, "Efficient

Mobility Management for Vertical Handoff between WWAN

and WLAN", IEEE Communications Magazine, November

2003.

[Villamizar]   Villamizar, C., "OSPF Optimized Multipath (OSPF-OMP)",

Work in Progress, February 1999.

[Xylomenos]    Xylomenos, G., "Multi Service Link Layers: An Approach

to Enhancing Internet Performance over Wireless

Links", Ph.D. thesis, University of California at San

Diego, 1999.

[Yegin]        Yegin, A., "Link-layer Triggers Protocol", Work in

Progress, June 2002.

6.  Acknowledgments

The authors would like to acknowledge James Kempf, Phil Roberts,

Gorry Fairhurst, John Wroclawski, Aaron Falk, Sally Floyd, Pekka

Savola, Pekka Nikander, Dave Thaler, Yogesh Swami, Wesley Eddy, and

Janne Peisa for contributions to this document.

IAB                          Informational                     [Page 40]

RFC 4907                    Link Indications                   June 2007

Appendix A.  Literature Review

This appendix summarizes the literature with respect to link

indications on wireless local area networks.

A.1.  Link Layer

The characteristics of wireless links have been found to vary

considerably depending on the environment.

In "Performance of Multihop Wireless Networks: Shortest Path is Not

Enough" [Shortest], the authors studied the performance of both an

indoor and outdoor mesh network.  By measuring inter-node throughput,

the best path between nodes was computed.  The throughput of the best

path was compared with the throughput of the shortest path computed

based on a hop-count metric.  In almost all cases, the shortest path

route offered considerably lower throughput than the best path.

In examining link behavior, the authors found that rather than

exhibiting a bi-modal distribution between "up" (low loss rate) and

"down" (high loss rate), many links exhibited intermediate loss

rates.  Asymmetry was also common, with 30 percent of links

demonstrating substantial differences in the loss rates in each

direction.  As a result, on wireless networks the measured throughput

can differ substantially from the negotiated rate due to

retransmissions, and successful delivery of routing packets is not

necessarily an indication that the link is useful for delivery of

data.

In "Measurement and Analysis of the Error Characteristics of an

In-Building Wireless Network" [Eckhardt], the authors characterize

the performance of an AT&T Wavelan 2 Mbps in-building WLAN operating

in Infrastructure mode on the Carnegie Mellon campus.  In this study,

very low frame loss was experienced.  As a result, links could be

assumed to operate either very well or not at all.

In "Link-level Measurements from an 802.11b Mesh Network" [Aguayo],

the authors analyze the causes of frame loss in a 38-node urban

multi-hop 802.11 ad-hoc network.  In most cases, links that are very

bad in one direction tend to be bad in both directions, and links

that are very good in one direction tend to be good in both

directions.  However, 30 percent of links exhibited loss rates

differing substantially in each direction.

Signal to noise ratio (SNR) and distance showed little value in

predicting loss rates, and rather than exhibiting a step-function

transition between "up" (low loss) or "down" (high loss) states,

inter-node loss rates varied widely, demonstrating a nearly uniform

IAB                          Informational                     [Page 41]

RFC 4907                    Link Indications                   June 2007

distribution over the range at the lower rates.  The authors

attribute the observed effects to multi-path fading, rather than

attenuation or interference.

The findings of [Eckhardt] and [Aguayo] demonstrate the diversity of

link conditions observed in practice.  While for indoor

infrastructure networks site surveys and careful measurement can

assist in promoting ideal behavior, in ad-hoc/mesh networks node

mobility and external factors such as weather may not be easily

controlled.

Considerable diversity in behavior is also observed due to

implementation effects.  "Techniques to reduce IEEE 802.11b MAC layer

handover time" [Velayos] measured handover times for a stationary STA

after the AP was turned off.  This study divided handover times into

detection (determination of disconnection from the existing point of

attachment), search (discovery of alternative attachment points), and

execution (connection to an alternative point of attachment) phases.

These measurements indicated that the duration of the detection phase

(the largest component of handoff delay) is determined by the number

of non-acknowledged frames triggering the search phase and delays due

to precursors such as RTS/CTS and rate adaptation.

Detection behavior varied widely between implementations.  For

example, network interface cards (NICs) designed for desktops

attempted more retransmissions prior to triggering search as compared

with laptop designs, since they assumed that the AP was always in

range, regardless of whether the Beacon was received.

The study recommends that the duration of the detection phase be

reduced by initiating the search phase as soon as collisions can be

excluded as the cause of non-acknowledged transmissions; the authors

recommend three consecutive transmission failures as the cutoff.

This approach is both quicker and more immune to multi-path

interference than monitoring of the SNR.  Where the STA is not

sending or receiving frames, it is recommended that Beacon reception

be tracked in order to detect disconnection, and that Beacon spacing

be reduced to 60 ms in order to reduce detection times.  In order to

compensate for more frequent triggering of the search phase, the

authors recommend algorithms for wait time reduction, as well as

interleaving of search and data frame transmission.

"An Empirical Analysis of the IEEE 802.11 MAC Layer Handoff Process"

[Mishra] investigates handoff latencies obtained with three mobile

STA implementations communicating with two APs.  The study found that

there is a large variation in handoff latency among STA and AP

implementations and that implementations utilize different message

sequences.  For example, one STA sends a Reassociation Request prior

IAB                          Informational                     [Page 42]

RFC 4907                    Link Indications                   June 2007

to authentication, which results in receipt of a Deauthenticate

message.  The study divided handoff latency into discovery,

authentication, and reassociation exchanges, concluding that the

discovery phase was the dominant component of handoff delay.  Latency

in the detection phase was not investigated.

"SyncScan: Practical Fast Handoff for 802.11 Infrastructure Networks"

[Ramani] weighs the pros and cons of active versus passive scanning.

The authors point out the advantages of timed Beacon reception, which

had previously been incorporated into [IEEE-802.11k].  Timed Beacon

reception allows the station to continually keep up to date on the

signal to noise ratio of neighboring APs, allowing handoff to occur

earlier.  Since the station does not need to wait for initial and

subsequent responses to a broadcast Probe Response (MinChannelTime

and MaxChannelTime, respectively), performance is comparable to what

is achievable with 802.11k Neighbor Reports and unicast Probe

Requests.

The authors measured the channel switching delay, the time it takes

to switch to a new frequency and begin receiving frames.

Measurements ranged from 5 ms to 19 ms per channel; where timed

Beacon reception or interleaved active scanning is used, switching

time contributes significantly to overall handoff latency.  The

authors propose deployment of APs with Beacons synchronized via

Network Time Protocol (NTP) [RFC1305], enabling a driver implementing

SyncScan to work with legacy APs without requiring implementation of

new protocols.  The authors measured the distribution of inter-

arrival times for stations implementing SyncScan, with excellent

results.

"Roaming Interval Measurements" [Alimian] presents data on the

behavior of stationary STAs after the AP signal has been shut off.

This study highlighted implementation differences in rate adaptation

as well as detection, scanning, and handoff.  As in [Velayos],

performance varied widely between implementations, from half an order

of magnitude variation in rate adaptation to an order of magnitude

difference in detection times, two orders of magnitude in scanning,

and one and a half orders of magnitude in handoff times.

"An experimental study of IEEE 802.11b handoff performance and its

effect on voice traffic" [Vatn] describes handover behavior observed

when the signal from the AP is gradually attenuated, which is more

representative of field experience than the shutoff techniques used

in [Velayos].  Stations were configured to initiate handover when

signal strength dipped below a threshold, rather than purely based on

frame loss, so that they could begin handover while still connected

to the current AP.  It was noted that stations continued to receive

data frames during the search phase.  Station-initiated

IAB                          Informational                     [Page 43]

RFC 4907                    Link Indications                   June 2007

Disassociation and pre-authentication were not observed in this

study.

A.1.1.  Link Indications

Within a link layer, the definition of "Link Up" and "Link Down" may

vary according to the deployment scenario.  For example, within PPP

[RFC1661], either peer may send an LCP-Terminate frame in order to

terminate the PPP link layer, and a link may only be assumed to be

usable for sending network protocol packets once Network Control

Protocol (NCP) negotiation has completed for that protocol.

Unlike PPP, IEEE 802 does not include facilities for network layer

configuration, and the definition of "Link Up" and "Link Down" varies

by implementation.  Empirical evidence suggests that the definition

of "Link Up" and "Link Down" may depend on whether the station is

mobile or stationary, whether infrastructure or ad-hoc mode is in

use, and whether security and Inter-Access Point Protocol (IAPP) is

implemented.

Where a STA encounters a series of consecutive non-acknowledged

frames while having missed one or more Beacons, the most likely cause

is that the station has moved out of range of the AP.  As a result,

[Velayos] recommends that the station begin the search phase after

collisions can be ruled out; since this approach does not take rate

adaptation into account, it may be somewhat aggressive.  Only when no

alternative workable rate or point of attachment is found is a "Link

Down" indication returned.

In a stationary point-to-point installation, the most likely cause of

an outage is that the link has become impaired, and alternative

points of attachment may not be available.  As a result,

implementations configured to operate in this mode tend to be more

persistent.  For example, within 802.11 the short interframe space

(SIFS) interval may be increased and MIB variables relating to

timeouts (such as dot11AuthenticationResponseTimeout,

dot11AssociationResponseTimeout, dot11ShortRetryLimit, and

dot11LongRetryLimit) may be set to larger values.  In addition, a

"Link Down" indication may be returned later.

In IEEE 802.11 ad-hoc mode with no security, reception of data frames

is enabled in State 1 ("Unauthenticated" and "Unassociated").  As a

result, reception of data frames is enabled at any time, and no

explicit "Link Up" indication exists.

In Infrastructure mode, IEEE 802.11-2003 enables reception of data

frames only in State 3 ("Authenticated" and "Associated").  As a

result, a transition to State 3 (e.g., completion of a successful

IAB                          Informational                     [Page 44]

RFC 4907                    Link Indications                   June 2007

Association or Reassociation exchange) enables sending and receiving

of network protocol packets and a transition from State 3 to State 2

(reception of a "Disassociate" frame) or State 1 (reception of a

"Deauthenticate" frame) disables sending and receiving of network

protocol packets.  As a result, IEEE 802.11 stations typically signal

"Link Up" on receipt of a successful Association/Reassociation

Response.

As described within [IEEE-802.11F], after sending a Reassociation

Response, an Access Point will send a frame with the station's source

address to a multicast destination.  This causes switches within the

Distribution System (DS) to update their learning tables, readying

the DS to forward frames to the station at its new point of

attachment.  Were the AP to not send this "spoofed" frame, the

station's location would not be updated within the distribution

system until it sends its first frame at the new location.  Thus, the

purpose of spoofing is to equalize uplink and downlink handover

times.  This enables an attacker to deny service to authenticated and

associated stations by spoofing a Reassociation Request using the

victim's MAC address, from anywhere within the ESS.  Without

spoofing, such an attack would only be able to disassociate stations

on the AP to which the Reassociation Request was sent.

The signaling of "Link Down" is considerably more complex.  Even

though a transition to State 2 or State 1 results in the station

being unable to send or receive IP packets, this does not necessarily

imply that such a transition should be considered a "Link Down"

indication.  In an infrastructure network, a station may have a

choice of multiple Access Points offering connection to the same

network.  In such an environment, a station that is unable to reach

State 3 with one Access Point may instead choose to attach to another

Access Point.  Rather than registering a "Link Down" indication with

each move, the station may instead register a series of "Link Up"

indications.

In [IEEE-802.11i], forwarding of frames from the station to the

distribution system is only feasible after the completion of the

4-way handshake and group-key handshake, so that entering State 3 is

no longer sufficient.  This has resulted in several observed

problems.  For example, where a "Link Up" indication is triggered on

the station by receipt of an Association/Reassociation Response, DHCP

[RFC2131] or Router Solicitation/Router Advertisement (RS/RA) may be

triggered prior to when the link is usable by the Internet layer,

resulting in configuration delays or failures.  Similarly, transport

layer connections will encounter packet loss, resulting in back-off

of retransmission timers.

IAB                          Informational                     [Page 45]

RFC 4907                    Link Indications                   June 2007

A.1.2.  Smart Link Layer Proposals

In order to improve link layer performance, several studies have

investigated "smart link layer" proposals.

"Advice to link designers on link Automatic Repeat reQuest (ARQ)"

[RFC3366] provides advice to the designers of digital communication

equipment and link-layer protocols employing link-layer Automatic

Repeat reQuest (ARQ) techniques for IP.  It discusses the use of ARQ,

timers, persistency in retransmission, and the challenges that arise

from sharing links between multiple flows and from different

transport requirements.

In "Link-layer Enhancements for TCP/IP over GSM" [Ludwig], the

authors describe how the Global System for Mobile Communications

(GSM)-reliable and unreliable link layer modes can be simultaneously

utilized without higher layer control.  Where a reliable link layer

protocol is required (where reliable transports such TCP and Stream

Control Transmission Protocol (SCTP) [RFC2960] are used), the Radio

Link Protocol (RLP) can be engaged; with delay-sensitive applications

such as those based on UDP, the transparent mode (no RLP) can be

used.  The authors also describe how PPP negotiation can be optimized

over high-latency GSM links using "Quickstart-PPP".

In "Link Layer Based TCP Optimisation for Disconnecting Networks"

[Scott], the authors describe performance problems that occur with

reliable transport protocols facing periodic network disconnections,

such as those due to signal fading or handoff.  The authors define a

disconnection as a period of connectivity loss that exceeds a

retransmission timeout, but is shorter than the connection lifetime.

One issue is that link-unaware senders continue to back off during

periods of disconnection.  The authors suggest that a link-aware

reliable transport implementation halt retransmission after receiving

a "Link Down" indication.  Another issue is that on reconnection the

lengthened retransmission times cause delays in utilizing the link.

To improve performance, a "smart link layer" is proposed, which

stores the first packet that was not successfully transmitted on a

connection, then retransmits it upon receipt of a "Link Up"

indication.  Since a disconnection can result in hosts experiencing

different network conditions upon reconnection, the authors do not

advocate bypassing slow start or attempting to raise the congestion

window.  Where IPsec is used and connections cannot be differentiated

because transport headers are not visible, the first untransmitted

packet for a given sender and destination IP address can be

retransmitted.  In addition to looking at retransmission of a single

packet per connection, the authors also examined other schemes such

IAB                          Informational                     [Page 46]

RFC 4907                    Link Indications                   June 2007

as retransmission of multiple packets and simulated duplicate

reception of single or multiple packets (known as rereception).

In general, retransmission schemes were superior to rereception

schemes, since rereception cannot stimulate fast retransmit after a

timeout.  Retransmission of multiple packets did not appreciably

improve performance over retransmission of a single packet.  Since

the focus of the research was on disconnection rather than just lossy

channels, a two-state Markov model was used, with the "up" state

representing no loss, and the "down" state representing 100 percent

loss.

In "Multi Service Link Layers: An Approach to Enhancing Internet

Performance over Wireless Links" [Xylomenos], the authors use ns-2 to

simulate the performance of various link layer recovery schemes (raw

link without retransmission, go back N, XOR-based FEC, selective

repeat, Karn's RLP, out-of-sequence RLP, and Berkeley Snoop) in

stand-alone file transfer, Web browsing, and continuous media

distribution.  While selective repeat and Karn's RLP provide the

highest throughput for file transfer and Web browsing scenarios,

continuous media distribution requires a combination of low delay and

low loss and the out-of-sequence RLP performed best in this scenario.

Since the results indicate that no single link layer recovery scheme

is optimal for all applications, the authors propose that the link

layer implement multiple recovery schemes.  Simulations of the

multi-service architecture showed that the combination of a low-error

rate recovery scheme for TCP (such as Karn's RLP) and a low-delay

scheme for UDP traffic (such as out-of-sequence RLP) provides for

good performance in all scenarios.  The authors then describe how a

multi-service link layer can be integrated with Differentiated

Services.

In "WaveLAN-II: A High-Performance Wireless LAN for the Unlicensed

Band" [Kamerman], the authors propose an open-loop rate adaptation

algorithm known as Automatic Rate Fallback (ARF).  In ARF, the sender

adjusts the rate upwards after a fixed number of successful

transmissions, and adjusts the rate downwards after one or two

consecutive failures.  If after an upwards rate adjustment the

transmission fails, the rate is immediately readjusted downwards.

In "A Rate-Adaptive MAC Protocol for Multi-Hop Wireless Networks"

[RBAR], the authors propose a closed-loop rate adaptation approach

that requires incompatible changes to the IEEE 802.11 MAC.  In order

to enable the sender to better determine the transmission rate, the

receiver determines the packet length and signal to noise ratio (SNR)

of a received RTS frame and calculates the corresponding rate based

on a theoretical channel model, rather than channel usage statistics.

The recommended rate is sent back in the CTS frame.  This allows the

IAB                          Informational                     [Page 47]

RFC 4907                    Link Indications                   June 2007

rate (and potentially the transmit power) to be optimized on each

transmission, albeit at the cost of requiring RTS/CTS for every frame

transmission.

In "MiSer: An Optimal Low-Energy Transmission Strategy for IEEE

802.11 a/h" [Qiao], the authors propose a scheme for optimizing

transmit power.  The proposal mandates the use of RTS/CTS in order to

deal with hidden nodes, requiring that CTS and ACK frames be sent at

full power.  The authors utilize a theoretical channel model rather

than one based on channel usage statistics.

In "IEEE 802.11 Rate Adaptation: A Practical Approach" [Lacage], the

authors distinguish between low-latency implementations, which enable

per-packet rate decisions, and high-latency implementations, which do

not.  The former implementations typically include dedicated CPUs in

their design, enabling them to meet real-time requirements.  The

latter implementations are typically based on highly integrated

designs in which the upper MAC is implemented on the host.  As a

result, due to operating system latencies the information required to

make per-packet rate decisions may not be available in time.

The authors propose an Adaptive ARF (AARF) algorithm for use with

low-latency implementations.  This enables rapid downward rate

negotiation on failure to receive an ACK, while increasing the number

of successful transmissions required for upward rate negotiation.

The AARF algorithm is therefore highly stable in situations where

channel properties are changing slowly, but slow to adapt upwards

when channel conditions improve.  In order to test the algorithm, the

authors utilized ns-2 simulations as well as implementing a version

of AARF adapted to a high-latency implementation, the AR 5212

chipset.  The Multiband Atheros Driver for WiFi (MadWiFi) driver

enables a fixed schedule of rates and retries to be provided when a

frame is queued for transmission.  The adapted algorithm, known as

the Adaptive Multi Rate Retry (AMRR), requests only one transmission

at each of three rates, the last of which is the minimum available

rate.  This enables adaptation to short-term fluctuations in the

channel with minimal latency.  The AMRR algorithm provides

performance considerably better than the existing MadWifi driver.

In "Link Adaptation Strategy for IEEE 802.11 WLAN via Received Signal

Strength Measurement" [Pavon], the authors propose an algorithm by

which a STA adjusts the transmission rate based on a comparison of

the received signal strength (RSS) from the AP with dynamically

estimated threshold values for each transmission rate.  Upon

reception of a frame, the STA updates the average RSS, and on

transmission the STA selects a rate and adjusts the RSS threshold

values based on whether or not the transmission is successful.  In

order to validate the algorithm, the authors utilized an OPNET

IAB                          Informational                     [Page 48]

RFC 4907                    Link Indications                   June 2007

simulation without interference, and an ideal curve of bit error rate

(BER) vs. signal to noise ratio (SNR) was assumed.  Not surprisingly,

the simulation results closely matched the maximum throughput

achievable for a given signal to noise ratio, based on the ideal BER

vs. SNR curve.

In "Hybrid Rate Control for IEEE 802.11" [Haratcherev], the authors

describe a hybrid technique utilizing Signal Strength Indication

(SSI) data to constrain the potential rates selected by statistics-

based automatic rate control.  Statistics-based rate control

techniques include:

Maximum Throughput

This technique, which was chosen as the statistics-based technique in

the hybrid scheme, sends a fraction of data at adjacent rates in

order to estimate which rate provides the maximum throughput.  Since

accurate estimation of throughput requires a minimum number of frames

to be sent at each rate, and only a fraction of frames are utilized

for this purpose, this technique adapts more slowly at lower rates;

with 802.11b rates, the adaptation time scale is typically on the

order of a second.  Depending on how many rates are tested, this

technique can enable adaptation beyond adjacent rates.  However,

where maximum rate and low frame loss are already being encountered,

this technique results in lower throughput.

Frame Error Rate (FER) Control

This technique estimates the FER, attempting to keep it between a

lower limit (if FER moves below, increase rate) and upper limit (if

FER moves above, decrease rate).  Since this technique can utilize

all the transmitted data, it can respond faster than maximum

throughput techniques.  However, there is a tradeoff of reaction time

versus FER estimation accuracy; at lower rates either reaction times

slow or FER estimation accuracy will suffer.  Since this technique

only measures the FER at the current rate, it can only enable

adaptation to adjacent rates.

Retry-based

This technique modifies FER control techniques by enabling rapid

downward rate adaptation after a number (5-10) of unsuccessful

retransmissions.  Since fewer packets are required, the sensitivity

of reaction time to rate is reduced.  However, upward rate adaptation

proceeds more slowly since it is based on a collection of FER data.

This technique is limited to adaptation to adjacent rates, and it has

the disadvantage of potentially worsening frame loss due to

contention.

IAB                          Informational                     [Page 49]

RFC 4907                    Link Indications                   June 2007

While statistics-based techniques are robust against short-lived link

quality changes, they do not respond quickly to long-lived changes.

By constraining the rate selected by statistics-based techniques

based on ACK SSI versus rate data (not theoretical curves), more

rapid link adaptation was enabled.  In order to ensure rapid

adaptation during rapidly varying conditions, the rate constraints

are tightened when the SSI values are changing rapidly, encouraging

rate transitions.  The authors validated their algorithms by

implementing a driver for the Atheros AR5000 chipset, and then

testing its response to insertion and removal from a microwave oven

acting as a Faraday cage.  The hybrid algorithm dropped many fewer

packets than the maximum throughput technique by itself.

In order to estimate the SSI of data at the receiver, the ACK SSI was

used.  This approach does not require the receiver to provide the

sender with the received power, so that it can be implemented without

changing the IEEE 802.11 MAC.  Calibration of the rate versus ACK SSI

curves does not require a symmetric channel, but it does require that

channel properties in both directions vary in a proportional way and

that the ACK transmit power remains constant.  The authors checked

the proportionality assumption and found that the SSI of received

data correlated highly (74%) with the SSI of received ACKs.  Low pass

filtering and monotonicity constraints were applied to remove noise

in the rate versus SSI curves.  The resulting hybrid rate adaptation

algorithm demonstrated the ability to respond to rapid deterioration

(and improvement) in channel properties, since it is not restricted

to moving to adjacent rates.

In "CARA: Collision-Aware Rate Adaptation for IEEE 802.11 WLANs"

[CARA], the authors propose Collision-Aware Rate Adaptation (CARA).

This involves utilization of Clear Channel Assessment (CCA) along

with adaptation of the Request-to-Send/Clear-to-Send (RTS/CTS)

mechanism to differentiate losses caused by frame collisions from

losses caused by channel conditions.  Rather than decreasing rate as

the result of frame loss due to collisions, which leads to increased

contention, CARA selectively enables RTS/CTS (e.g., after a frame

loss), reducing the likelihood of frame loss due to hidden stations.

CARA can also utilize CCA to determine whether a collision has

occurred after a transmission; however, since CCA may not detect a

significant fraction of all collisions (particularly when

transmitting at low rate), its use is optional.  As compared with

ARF, in simulations the authors show large improvements in aggregate

throughput due to addition of adaptive RTS/CTS, and additional modest

improvements with the additional help of CCA.

In "Robust Rate Adaptation for 802.11 Wireless Networks" [Robust],

the authors implemented the ARF, AARF, and SampleRate [SampleRate]

algorithms on a programmable Access Point platform, and

IAB                          Informational                     [Page 50]

RFC 4907                    Link Indications                   June 2007

experimentally examined the performance of these algorithms as well

as the ONOE [ONOE] algorithm implemented in MadWiFi.  Based on their

experiments, the authors critically examine the assumptions

underlying existing rate negotiation algorithms:

Decrease transmission rate upon severe frame loss

Where severe frame loss is due to channel conditions, rate

reduction can improve throughput.  However, where frame loss is

due to contention (such as from hidden stations), reducing

transmission rate increases congestion, lowering throughput and

potentially leading to congestive collapse.  Instead, the

authors propose adaptive enabling of RTS/CTS so as to reduce

contention due to hidden stations.  Once RTS/CTS is enabled,

remaining losses are more likely to be due to channel

conditions, providing more reliable guidance on increasing or

decreasing transmission rate.

Use probe frames to assess possible new rates

Probe frames reliably estimate frame loss at a given rate unless

the sample size is sufficient and the probe frames are of

comparable length to data frames.  The authors argue that rate

adaptation schemes such as SampleRate are too sensitive to loss

of probe packets.  In order to satisfy sample size constraints,

a significant number of probe frames are required.  This can

increase frame loss if the probed rate is too high, and can

lower throughput if the probed rate is too low.  Instead, the

authors propose assessment of the channel condition by tracking

the frame loss ratio within a window of 5 to 40 frames.

Use consecutive transmission successes/losses to increase/decrease

rate

The authors argue that consecutive successes or losses are not a

reliable basis for rate increases or decreases; greater sample

size is needed.

Use PHY metrics like SNR to infer new transmission rate

The authors argue that received signal to noise ratio (SNR)

routinely varies 5 dB per packet and that variations of 10-14 dB

are common.  As a result, rate decisions based on SNR or signal

strength can cause transmission rate to vary rapidly.  The

authors question the value of such rapid variation, since

studies such as [Aguayo] show little correlation between SNR and

frame loss probability.  As a result, the authors argue that

neither received signal strength indication (RSSI) nor

background energy level can be used to distinguish losses due to

contention from those due to channel conditions.  While multi-

path interference can simultaneously result in high signal

strength and frame loss, the relationship between low signal

IAB                          Informational                     [Page 51]

RFC 4907                    Link Indications                   June 2007

strength and high frame loss is stronger.  Therefore,

transmission rate decreases due to low received signal strength

probably do reflect sudden worsening in channel conditions,

although sudden increases may not necessarily indicate that

channel conditions have improved.

Long-term smoothened operation produces best average performance

The authors present evidence that frame losses more than 150 ms

apart are uncorrelated.  Therefore, collection of statistical

data over intervals of 1 second or greater reduces

responsiveness, but does not improve the quality of transmission

rate decisions.  Rather, the authors argue that a sampling

period of 100 ms provides the best average performance.  Such

small sampling periods also argue against use of probes, since

probe packets can only represent a fraction of all data frames

and probes collected more than 150 ms apart may not provide

reliable information on channel conditions.

Based on these flaws, the authors propose the Robust Rate Adaptation

Algorithm (RRAA).  RRAA utilizes only the frame loss ratio at the

current transmission rate to determine whether to increase or

decrease the transmission rate; PHY layer information or probe

packets are not used.  Each transmission rate is associated with an

estimation window, a maximum tolerable loss threshold (MTL) and an

opportunistic rate increase threshold (ORI).  If the loss ratio is

larger than the MTL, the transmission rate is decreased, and if it is

smaller than the ORI, transmission rate is increased; otherwise

transmission rate remains the same.  The thresholds are selected in

order to maximize throughput.  Although RRAA only allows movement

between adjacent transmission rates, the algorithm does not require

collection of an entire estimation window prior to increasing or

decreasing transmission rates; if additional data collection would

not change the decision, the change is made immediately.

The authors validate the RRAA algorithm using experiments and field

trials; the results indicate that RRAA without adaptive RTS/CTS

outperforms the ARF, AARF, and Sample Rate algorithms.  This occurs

because RRAA is not as sensitive to transient frame loss and does not

use probing, enabling it to more frequently utilize higher

transmission rates.  Where there are no hidden stations, turning on

adaptive RTS/CTS reduces performance by at most a few percent.

However, where there is substantial contention from hidden stations,

adaptive RTS/CTS provides large performance gains, due to reduction

in frame loss that enables selection of a higher transmission rate.

In "Efficient Mobility Management for Vertical Handoff between WWAN

and WLAN" [Vertical], the authors propose use of signal strength and

link utilization in order to optimize vertical handoff.  WLAN to WWAN

IAB                          Informational                     [Page 52]

RFC 4907                    Link Indications                   June 2007

handoff is driven by SSI decay.  When IEEE 802.11 SSI falls below a

threshold (S1), Fast Fourier Transform (FFT)-based decay detection is

undertaken to determine if the signal is likely to continue to decay.

If so, then handoff to the WWAN is initiated when the signal falls

below the minimum acceptable level (S2).  WWAN to WLAN handoff is

driven by both PHY and MAC characteristics of the IEEE 802.11 target

network.  At the PHY layer, characteristics such as SSI are examined

to determine if the signal strength is greater than a minimum value

(S3).  At the MAC layer, the IEEE 802.11 Network Allocation Vector

(NAV) occupation is examined in order to estimate the maximum

available bandwidth and mean access delay.  Note that depending on

the value of S3, it is possible for the negotiated rate to be less

than the available bandwidth.  In order to prevent premature handoff

between WLAN and WWAN, S1 and S2 are separated by 6 dB; in order to

prevent oscillation between WLAN and WWAN media, S3 needs to be

greater than S1 by an appropriate margin.

A.2.  Internet Layer

Within the Internet layer, proposals have been made for utilizing

link indications to optimize IP configuration, to improve the

usefulness of routing metrics, and to optimize aspects of Mobile IP

handoff.

In "Analysis of link failures in an IP backbone" [Iannaccone], the

authors investigate link failures in Sprint's IP backbone.  They

identify the causes of convergence delay, including delays in

detection of whether an interface is down or up.  While it is fastest

for a router to utilize link indications if available, there are

situations in which it is necessary to depend on loss of routing

packets to determine the state of the link.  Once the link state has

been determined, a delay may occur within the routing protocol in

order to dampen link flaps.  Finally, another delay may be introduced

in propagating the link state change, in order to rate limit link

state advertisements, and guard against instability.

"Bidirectional Forwarding Detection" [BFD] notes that link layers may

provide only limited failure indications, and that relatively slow

"Hello" mechanisms are used in routing protocols to detect failures

when no link layer indications are available.  This results in

failure detection times of the order of a second, which is too long

for some applications.  The authors describe a mechanism that can be

used for liveness detection over any media, enabling rapid detection

of failures in the path between adjacent forwarding engines.  A path

is declared operational when bidirectional reachability has been

confirmed.

IAB                          Informational                     [Page 53]

RFC 4907                    Link Indications                   June 2007

In "Detecting Network Attachment (DNA) in IPv4" [RFC4436], a host

that has moved to a new point of attachment utilizes a bidirectional

reachability test in parallel with DHCP [RFC2131] to rapidly

reconfirm an operable configuration.

In "L2 Triggers Optimized Mobile IPv6 Vertical Handover: The

802.11/GPRS Example" [Park], the authors propose that the mobile node

send a router solicitation on receipt of a "Link Up" indication in

order to provide lower handoff latency than would be possible using

generic movement detection [RFC3775].  The authors also suggest

immediate invalidation of the Care-of Address (CoA) on receipt of a

"Link Down" indication.  However, this is problematic where a "Link

Down" indication can be followed by a "Link Up" indication without a

resulting change in IP configuration, as described in [RFC4436].

In "Layer 2 Handoff for Mobile-IPv4 with 802.11" [Mun], the authors

suggest that MIPv4 Registration messages be carried within

Information Elements of IEEE 802.11 Association/Reassociation frames,

in order to minimize handoff delays.  This requires modification to

the mobile node as well as 802.11 APs.  However, prior to detecting

network attachment, it is difficult for the mobile node to determine

whether or not the new point of attachment represents a change of

network.  For example, even where a station remains within the same

ESS, it is possible that the network will change.  Where no change of

network results, sending a MIPv4 Registration message with each

Association/Reassociation is unnecessary.  Where a change of network

results, it is typically not possible for the mobile node to

anticipate its new CoA at Association/Reassociation; for example, a

DHCP server may assign a CoA not previously given to the mobile node.

When dynamic VLAN assignment is used, the VLAN assignment is not even

determined until IEEE 802.1X authentication has completed, which is

after Association/Reassociation in [IEEE-802.11i].

In "Link Characteristics Information for Mobile IP" [Lee], link

characteristics are included in registration/Binding Update messages

sent by the mobile node to the home agent and correspondent node.

Where the mobile node is acting as a receiver, this allows the

correspondent node to adjust its transport parameters window more

rapidly than might otherwise be possible.  Link characteristics that

may be communicated include the link type (e.g., 802.11b, CDMA (Code

Division Multiple Access), GPRS (General Packet Radio Service), etc.)

and link bandwidth.  While the document suggests that the

correspondent node should adjust its sending rate based on the

advertised link bandwidth, this may not be wise in some

circumstances.  For example, where the mobile node link is not the

bottleneck, adjusting the sending rate based on the link bandwidth

could cause congestion.  Also, where the transmission rate changes

frequently, sending registration messages on each transmission rate

IAB                          Informational                     [Page 54]

RFC 4907                    Link Indications                   June 2007

change could by itself consume significant bandwidth.  Even where the

advertised link characteristics indicate the need for a smaller

congestion window, it may be non-trivial to adjust the sending rates

of individual connections where there are multiple connections open

between a mobile node and correspondent node.  A more conservative

approach would be to trigger parameter re-estimation and slow start

based on the receipt of a registration message or Binding Update.

In "Hotspot Mitigation Protocol (HMP)" [HMP], it is noted that Mobile

Ad-hoc NETwork (MANET) routing protocols have a tendency to

concentrate traffic since they utilize shortest-path metrics and

allow nodes to respond to route queries with cached routes.  The

authors propose that nodes participating in an ad-hoc wireless mesh

monitor local conditions such as MAC delay, buffer consumption, and

packet loss.  Where congestion is detected, this is communicated to

neighboring nodes via an IP option.  In response to moderate

congestion, nodes suppress route requests; where major congestion is

detected, nodes rate control transport connections flowing through

them.  The authors argue that for ad-hoc networks, throttling by

intermediate nodes is more effective than end-to-end congestion

control mechanisms.

A.3.  Transport Layer

Within the transport layer, proposals have focused on countering the

effects of handoff-induced packet loss and non-congestive loss caused

by lossy wireless links.

Where a mobile host moves to a new network, the transport parameters

(including the RTT, RTO, and congestion window) may no longer be

valid.  Where the path change occurs on the sender (e.g., change in

outgoing or incoming interface), the sender can reset its congestion

window and parameter estimates.  However, where it occurs on the

receiver, the sender may not be aware of the path change.

In "The BU-trigger method for improving TCP performance over Mobile

IPv6" [Kim], the authors note that handoff-related packet loss is

interpreted as congestion by the transport layer.  In the case where

the correspondent node is sending to the mobile node, it is proposed

that receipt of a Binding Update by the correspondent node be used as

a signal to the transport layer to adjust cwnd and ssthresh values,

which may have been reduced due to handoff-induced packet loss.  The

authors recommend that cwnd and ssthresh be recovered to pre-timeout

values, regardless of whether the link parameters have changed.  The

paper does not discuss the behavior of a mobile node sending a

Binding Update, in the case where the mobile node is sending to the

correspondent node.

IAB                          Informational                     [Page 55]

RFC 4907                    Link Indications                   June 2007

In "Effect of Vertical Handovers on Performance of TCP-Friendly Rate

Control" [Gurtov], the authors examine the effect of explicit

handover notifications on TCP-friendly rate control (TFRC).  Where

explicit handover notification includes information on the loss rate

and throughput of the new link, this can be used to instantaneously

change the transmission rate of the sender.  The authors also found

that resetting the TFRC receiver state after handover enabled

parameter estimates to adjust more quickly.

In "Adapting End Host Congestion Control for Mobility" [Eddy], the

authors note that while MIPv6 with route optimization allows a

receiver to communicate a subnet change to the sender via a Binding

Update, this is not available within MIPv4.  To provide a

communication vehicle that can be universally employed, the authors

propose a TCP option that allows a connection endpoint to inform a

peer of a subnet change.  The document does not advocate utilization

of "Link Up" or "Link Down" events since these events are not

necessarily indicative of subnet change.  On detection of subnet

change, it is advocated that the congestion window be reset to

INIT_WINDOW and that transport parameters be re-estimated.  The

authors argue that recovery from slow start results in higher

throughput both when the subnet change results in lower bottleneck

bandwidth as well as when bottleneck bandwidth increases.

In "Efficient Mobility Management for Vertical Handoff between WWAN

and WLAN" [Vertical], the authors propose a "Virtual Connectivity

Manager", which utilizes local connection translation (LCT) and a

subscription/notification service supporting simultaneous movement in

order to enable end-to-end mobility and maintain TCP throughput

during vertical handovers.

In an early version of "Datagram Congestion Control Protocol (DCCP)"

[RFC4340], a "Reset Congestion State" option was proposed in Section

11.  This option was removed in part because the use conditions were

not fully understood:

An HC-Receiver sends the Reset Congestion State option to its

sender to force the sender to reset its congestion state -- that

is, to "slow start", as if the connection were beginning again.

...

The Reset Congestion State option is reserved for the very few

cases when an endpoint knows that the congestion properties of a

path have changed.  Currently, this reduces to mobility: a DCCP

endpoint on a mobile host MUST send Reset Congestion State to its

peer after the mobile host changes address or path.

IAB                          Informational                     [Page 56]

RFC 4907                    Link Indications                   June 2007

"Framework and Requirements for TRIGTRAN" [TRIGTRAN] discusses

optimizations to recover earlier from a retransmission timeout

incurred during a period in which an interface or intervening link

was down.  "End-to-end, Implicit 'Link-Up' Notification" [E2ELinkup]

describes methods by which a TCP implementation that has backed off

its retransmission timer due to frame loss on a remote link can learn

that the link has once again become operational.  This enables

retransmission to be attempted prior to expiration of the backed-off

retransmission timer.

"Link-layer Triggers Protocol" [Yegin] describes transport issues

arising from lack of host awareness of link conditions on downstream

Access Points and routers.  Transport of link layer triggers is

proposed to address the issue.

"TCP Extensions for Immediate Retransmissions" [Eggert] describes how

a transport layer implementation may utilize existing "end-to-end

connectivity restored" indications.  It is proposed that in addition

to regularly scheduled retransmissions that retransmission be

attempted by the transport layer on receipt of an indication that

connectivity to a peer node may have been restored.  End-to-end

connectivity restoration indications include "Link Up", confirmation

of first-hop router reachability, confirmation of Internet layer

configuration, and receipt of other traffic from the peer.

In "Discriminating Congestion Losses from Wireless Losses Using

Interarrival Times at the Receiver" [Biaz], the authors propose a

scheme for differentiating congestive losses from wireless

transmission losses based on inter-arrival times.  Where the loss is

due to wireless transmission rather than congestion, congestive

backoff and cwnd adjustment is omitted.  However, the scheme appears

to assume equal spacing between packets, which is not realistic in an

environment exhibiting link layer frame loss.  The scheme is shown to

function well only when the wireless link is the bottleneck, which is

often the case with cellular networks, but not with IEEE 802.11

deployment scenarios such as home or hotspot use.

In "Improving Performance of TCP over Wireless Networks" [Bakshi],

the authors focus on the performance of TCP over wireless networks

with burst losses.  The authors simulate performance of TCP Tahoe

within ns-2, utilizing a two-state Markov model, representing "good"

and "bad" states.  Where the receiver is connected over a wireless

link, the authors simulate the effect of an Explicit Bad State

Notification (EBSN) sent by an Access Point unable to reach the

receiver.  In response to an EBSN, it is advocated that the existing

retransmission timer be canceled and replaced by a new dynamically

IAB                          Informational                     [Page 57]

RFC 4907                    Link Indications                   June 2007

estimated timeout, rather than being backed off.  In the simulations,

EBSN prevents unnecessary timeouts, decreasing RTT variance and

improving throughput.

In "A Feedback-Based Scheme for Improving TCP Performance in Ad-Hoc

Wireless Networks" [Chandran], the authors proposed an explicit Route

Failure Notification (RFN), allowing the sender to stop its

retransmission timers when the receiver becomes unreachable.  On

route reestablishment, a Route Reestablishment Notification (RRN) is

sent, unfreezing the timer.  Simulations indicate that the scheme

significantly improves throughput and reduces unnecessary

retransmissions.

In "Analysis of TCP Performance over Mobile Ad Hoc Networks"

[Holland], the authors explore how explicit link failure notification

(ELFN) can improve the performance of TCP in mobile ad hoc networks.

ELFN informs the TCP sender about link and route failures so that it

need not treat the ensuing packet loss as due to congestion.  Using

an ns-2 simulation of TCP Reno over 802.11 with routing provided by

the Dynamic Source Routing (DSR) protocol, it is demonstrated that

TCP performance falls considerably short of expected throughput based

on the percentage of the time that the network is partitioned.  A

portion of the problem was attributed to the inability of the routing

protocol to quickly recognize and purge stale routes, leading to

excessive link failures; performance improved dramatically when route

caching was turned off.  Interactions between the route request and

transport retransmission timers were also noted.  Where the route

request timer is too large, new routes cannot be supplied in time to

prevent the transport timer from expiring, and where the route

request timer is too small, network congestion may result.

For their implementation of ELFN, the authors piggybacked additional

information (sender and receiver addresses and ports, the TCP

sequence number) on an existing "route failure" notice to enable the

sender to identify the affected connection.  Where a TCP receives an

ELFN, it disables the retransmission timer and enters "stand-by"

mode, where packets are sent at periodic intervals to determine if

the route has been reestablished.  If an acknowledgment is received,

then the retransmission timers are restored.  Simulations show that

performance is sensitive to the probe interval, with intervals of 30

seconds or greater giving worse performance than TCP Reno.  The

effect of resetting the congestion window and RTO values was also

investigated.  In the study, resetting the congestion window to one

did not have much of an effect on throughput, since the

bandwidth/delay of the network was only a few packets.  However,

resetting the RTO to a high initial value (6 seconds) did have a

substantial detrimental effect, particularly at high speed.  In terms

of the probe packet sent, the simulations showed little difference

IAB                          Informational                     [Page 58]

RFC 4907                    Link Indications                   June 2007

between sending the first packet in the congestion window, or

retransmitting the packet with the lowest sequence number among those

signaled as lost via the ELFNs.

In "Improving TCP Performance over Wireless Links" [Goel], the

authors propose use of an ICMP-DEFER message, sent by a wireless

Access Point on failure of a transmission attempt.  After exhaustion

of retransmission attempts, an ICMP-RETRANSMIT message is sent.  On

receipt of an ICMP-DEFER message, the expiry of the retransmission

timer is postponed by the current RTO estimate.  On receipt of an

ICMP-RETRANSMIT message, the segment is retransmitted.  On

retransmission, the congestion window is not reduced; when coming out

of fast recovery, the congestion window is reset to its value prior

to fast retransmission and fast recovery.  Using a two-state Markov

model, simulated using ns-2, the authors show that the scheme

improves throughput.

In "Explicit Transport Error Notification (ETEN) for Error-Prone

Wireless and Satellite Networks" [Krishnan], the authors examine the

use of explicit transport error notification (ETEN) to aid TCP in

distinguishing congestive losses from those due to corruption.  Both

per-packet and cumulative ETEN mechanisms were simulated in ns-2,

using both TCP Reno and TCP SACK over a wide range of bit error rates

and traffic conditions.  While per-packet ETEN mechanisms provided

substantial gains in TCP goodput without congestion, where congestion

was also present, the gains were not significant.  Cumulative ETEN

mechanisms did not perform as well in the study.  The authors point

out that ETEN faces significant deployment barriers since it can

create new security vulnerabilities and requires implementations to

obtain reliable information from the headers of corrupt packets.

In "Towards More Expressive Transport-Layer Interfaces" [Eggert2],

the authors propose extensions to existing network/transport and

transport/application interfaces to improve the performance of the

transport layer in the face of changes in path characteristics

varying more quickly than the round-trip time.

In "Protocol Enhancements for Intermittently Connected Hosts"

[Schuetz], the authors note that intermittent connectivity can lead

to poor performance and connectivity failures.  To address these

problems, the authors combine the use of the Host Identity Protocol

(HIP) [RFC4423] with a TCP User Timeout Option and TCP Retransmission

trigger, demonstrating significant improvement.

IAB                          Informational                     [Page 59]

RFC 4907                    Link Indications                   June 2007

A.4.  Application Layer

In "Application-oriented Link Adaptation for IEEE 802.11"

[Haratcherev2], rate information generated by a link layer utilizing

improved rate adaptation algorithms is provided to a video

application, and used for codec adaptation.  Coupling the link and

application layers results in major improvements in the Peak Signal

to Noise Ratio (PSNR).  Since this approach assumes that the link

represents the path bottleneck bandwidth, it is not universally

applicable to use over the Internet.

At the application layer, the usage of "Link Down" indications has

been proposed to augment presence systems.  In such systems, client

devices periodically refresh their presence state using application

layer protocols such as SIP for Instant Messaging and Presence

Leveraging Extensions (SIMPLE) [RFC3428] or Extensible Messaging and

Presence Protocol (XMPP) [RFC3921].  If the client should become

disconnected, their unavailability will not be detected until the

presence status times out, which can take many minutes.  However, if

a link goes down, and a disconnect indication can be sent to the

presence server (presumably by the Access Point, which remains

connected), the status of the user's communication application can be

updated nearly instantaneously.

Appendix B.  IAB Members at the Time of This Writing

Bernard Aboba

Loa Andersson

Brian Carpenter

Leslie Daigle

Elwyn Davies

Kevin Fall

Olaf Kolkman

Kurtis Lindqvist

David Meyer

David Oran

Eric Rescorla

Dave Thaler

Lixia Zhang

IAB                          Informational                     [Page 60]

RFC 4907                    Link Indications                   June 2007

Author's Address

Bernard Aboba, Ed.

Microsoft Corporation

One Microsoft Way

Redmond, WA 98052

EMail: bernarda@microsoft.com

Phone: +1 425 706 6605

Fax:   +1 425 936 7329

IAB

EMail: iab@iab.org

URI:   http://www.iab.org/

IAB                          Informational                     [Page 61]

RFC 4907                    Link Indications                   June 2007

Full Copyright Statement

Copyright (C) The IETF Trust (2007).

This document is subject to the rights, licenses and restrictions

contained in BCP 78, and except as set forth therein, the authors

retain all their rights.

This document and the information contained herein are provided on an

"AS IS" basis and THE CONTRIBUTOR, THE ORGANIZATION HE/SHE REPRESENTS

OR IS SPONSORED BY (IF ANY), THE INTERNET SOCIETY, THE IETF TRUST AND

THE INTERNET ENGINEERING TASK FORCE DISCLAIM ALL WARRANTIES, EXPRESS

OR IMPLIED, INCLUDING BUT NOT LIMITED TO ANY WARRANTY THAT THE USE OF

THE INFORMATION HEREIN WILL NOT INFRINGE ANY RIGHTS OR ANY IMPLIED

WARRANTIES OF MERCHANTABILITY OR FITNESS FOR A PARTICULAR PURPOSE.

Intellectual Property

The IETF takes no position regarding the validity or scope of any

Intellectual Property Rights or other rights that might be claimed to

pertain to the implementation or use of the technology described in

this document or the extent to which any license under such rights

might or might not be available; nor does it represent that it has

made any independent effort to identify any such rights.  Information

on the procedures with respect to rights in RFC documents can be

found in BCP 78 and BCP 79.

Copies of IPR disclosures made to the IETF Secretariat and any

assurances of licenses to be made available, or the result of an

attempt made to obtain a general license or permission for the use of

such proprietary rights by implementers or users of this

specification can be obtained from the IETF on-line IPR repository at

http://www.ietf.org/ipr.

The IETF invites any interested party to bring to its attention any

copyrights, patents or patent applications, or other proprietary

rights that may cover technology that may be required to implement

this standard.  Please address the information to the IETF at

ietf-ipr@ietf.org.

Acknowledgement

Funding for the RFC Editor function is currently provided by the

Internet Society.