IEEE P802.11
Wireless LANs

Draft TGg Functional Requirements

July 26, 2000

Author:	Matthew B. Shoemake, Ph.D.
	HRb SG Chairperson
	shoemake@alantro.com

Abstract

The Proposal Functional Requirements defined herein as drafted by the HRb SG is intended to be used as a draft for the IEEE 802.11g Task Group pending the approval of the associated PAR.

# Functional Requirements

The task group reserves the right to change and modify the functional requirements as needed.

### General Requirements

- The proposal must be an extension of the IEEE 802.11b standard.

- The proposal shall specify a PHY that implements all mandatory portions of the IEEE 802.11b PHY standard

- Must comply with IEEE 802 patent policy

- Backward compatibility with 802.11b

- All proposals must not render existing 802.11b compliant products non-conformant to the resulting, supplemented IEEE 802.11 2.4GHz standard.

##### MAC Interface Requirements

- The proposal must be compatible with the IEEE 802.11 MAC standard.  Clarification note: Compatibility with the IEEE 802.11 MAC may be achieved by changes to MIB variables.

##### Performance Requirements

- The maximum PHY data rate of the proposal must be at least 20Mbps

#### RF Requirements

- Same spectral mask as 802.11b

- Channelization same as 802.11b

- Spectrally compatible with 802.11b

- Maximum transmit power requirement