IEEE P802.11
Wireless LANs

802.11e Draft Standard D1.0 Letter Ballot 27 Comments, Clause 6

Date:	May 14th, 2001

Author:	John Fakatselis, Chair, Et Al

|  | 6. | Greg Parks | T | YES | Comment reserved – no text | TBD |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 6.1.1 | Greg Parks | T | YES | Line 16, QoS facility is required not optional, should include reference to traffic specification as well as priority | IBID |  |
|  | 6.2.1.1.2 | J. Ho | t | Y | Eight additional priority parameter values is not adequate in the identification of parameterized and prioritized QoS. | Expand the priority parameter values to sixteen or the service class parameter values to four. |  |
|  | 6.2.1.1.2 | APS | T | Yes | Having a variable MSDU MTU depending on features seems to leak unnecessary MAC-layer knowledge into layers above the MAC. | Define a constant MSDU MTU. |  |
|  | 6.2.1.1.2 | Greg Parks | T | YES | QoS is not an optional facility | Delete “optional” |  |
|  | 6.2.1.1.2 | APS | T | Yes | The semantics of the “Contention” and “Contention-Free” service parameters are not adequately defined. Do they express a preference or an absolute constraint? This parameter would also appear to be able to affect MSDU re-ordering over and above that implied in 6.1.3. Also these two cases and the priority classes are different. The priority class parameter can be transported exactly, but the Contention/Contention-free cannot – it has to be inferred inexactly by the receiver. | Remove the Contention & Contention-free priority classes. |  |
|  | 6.2.1.1.2 | Bob O’Hara | T | Y | Why has the MTU been reduced when an option is used? Other standards, notably 802.3, have expanded their MTU when required to carry additional information, such as VLAN tags. | Extend the allowable size of the MSDU rather than reducing it, when using the “optional facilities”. |  |
|  | 6.2.1.3.2 | Matthew Sherman | T | YES | It is not clear why the dropping of a packet (due to failed delivery) is not reportable to the upper layers. | Clarify. |  |
|  | 6.2.1.3.2 | Mathilde Benveniste | T | Yes | P 14, L 10 Missing information. Change is needed for completeness. | Insert text: Undeliverable (MSDUTimer reached aMaxMSDULifetime[TC] before successful delivery of an MSDU of traffic category TC) |  |
|  | 6.2.1.3.2 | Matthew Sherman | T | YES | See rationale for 9.2.5.3. | P 14, L 10, Insert text: Undeliverable (MSDUTimer reached aMaxMSDULifetime[TC] before successful delivery of an MSDU of traffic category TC) |  |
|  | 6.3.1.3.2 | Greg Parks | T | YES | Should we include a response for “uncorrectable error” and possibly “correctable error” | We should include as suggested. |  |
|  | 6.2.1.3.2 | Bob O’Hara | T | Y | The text in brackets is entirely incorrect. 802.11E will be a supplement to 802.11. IT IS NOT A SEPARATE STANDARD. Therefore any statement that say “never” as this one and NOTE 2, is incorrect, because current implementations can do what is said to never happen. “Not likely” and “never” are not identical. | Delete the bracketed text (in two places) and NOTE 2. |  |
|  | 6.1.3 | Greg Parks | T | YES | Does MSDU reordering make sense in the QoS environment? Is this on a per Queue or per traffic class basis? Strictly Ordered Service does not make sense, should not be on a packet-by-packet basis, but on a session wide basis. | Explain how this will work Make strictly ordered service definition on a session wide basis not on a packet by packet basis. |  |
|  | 6.1.3 | APS | T | Yes | There is an “information gap” between the receiving station that knows it wants to receive strictly-ordered MSDUs or would like to save power and the transmitting station that specifies this parameter. | Remove this section and the strictly ordered traffic category elsewhere. |  |
|  | 6.2.1.1.4 | Greg Parks | T | YES | We should consider passing back what is available if a request cannot be fulfilled. That way there can be negotiation rather than a game of 20 questions | IBID |  |