IEEE P802.11
Wireless LANs

802.11e Draft Standard D1.0 Letter Ballot 27 Comments, Clause 6

Date:	June 11, 2001

Revision 0: Comments from ballots received through morning of May 14, 2001.
	Revision 2: Incorporate comments from ballots received between May 14 and close of LB27.

Author:	John Fakatselis, Chair, Et Al

|  | 6. | Amar Ghori | T | YES | Comment reserved – no text | TBD |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 6. | Greg Parks | T | YES | Comment reserved – no text | TBD |  |
|  | 6.1.1 | Amar Ghori | T | YES | Line 16, QoS facility is required not optional, should include reference to traffic specification as well as priority | IBID |  |
|  | 6.1.1 | Greg Parks | T | YES | Line 16, QoS facility is required not optional, should include reference to traffic specification as well as priority | IBID |  |
|  | 6.1.1 | Ken Kimura | T | YES | Line 16, QoS facility is required not optional, should include reference to traffic specification as well as priority | IBID |  |
|  | 6.1.1 | Matthew Fischer | T | Y | Draft language: In a QBSS with some ESTAs, which support the optional QoS facility,and some STAs, which do not, the STA MSDU delivery corresponds to ESTA delivery of MSDUs belonging to a traffic category with a priority of best effort. Well, this all sort of depends upon what the AIFS[TC] values are set to, doesn’t it? Some settings of AIFS[TC] can cause STA traffic to receive lower priority than IEEE priority 2. | If we really want to insure that legacy STA traffic receives best effort service, then we must legislate that the settings for the QOS delivery of traffic classes of best effort and less than best effort are appropriately set so as not to receive better service than the legacy STA traffic. |  |
|  | 6.1.1 | RAJU GUBBI | T | Y | Draft language: In a QBSS with some ESTAs, which support the optional QoS facility,and some STAs, which do not, the STA MSDU delivery corresponds to ESTA delivery of MSDUs belonging to a traffic category with a priority of best effort. Well, this all sort of depends upon what the AIFS[TC] values are set to, doesn’t it? Some settings of AIFS[TC] can cause STA traffic to receive lower priority than IEEE priority 2. | If we really want to insure that legacy STA traffic receives best effort service, then we must legislate that the settings for the QOS delivery of traffic classes of best effort and less than best effort are appropriately set so as not to receive better service than the legacy STA traffic. |  |
|  | 6.1.1 | Ryoji Kido | T | YES | Line 16, QoS facility is required not optional, should include reference to traffic specification as well as priority | IBID |  |
|  | 6.1.3 |  | t | YES | Given current state of 802.1D, “StrictlyOrdered” should apply to re-ordering of multicast vs. unicast within a class of service (including priority), and not to maintaining ordering of MSDUs belonging to different classes of service or priority. | Modify text to indicate that “StrictlyOrdered” applies to maintaining ordering of MSDUs within a class of service; modify editing instructions to preserve the name “ReorderableMulticast”. |  |
|  | 6.1.3 |  | t | YES | reordering of MSDUs from different traffic classes applies to multicast as well as unicast frames. | Delete word “unicast” from phrase “reordering of unicast MSDUs belonging to different traffic categories …” |  |
|  | 6.1.3 | Amar Ghori | T | YES | Does MSDU reordering make sense in the QoS environment? Is this on a per Queue or per traffic class basis? Strictly Ordered Service does not make sense, should not be on a packet-by-packet basis, but on a session wide basis. | Explain how this will work Make strictly ordered service definition on a session wide basis not on a packet by packet basis. |  |
|  | 6.1.3 | APS | T | Yes | There is an “information gap” between the receiving station that knows it wants to receive strictly-ordered MSDUs or would like to save power and the transmitting station that specifies this parameter. | Remove this section and the strictly ordered traffic category elsewhere. |  |
|  | 6.1.3 | Diepstraten | T |  | There is no reordering of unicast MSDU’s with the same TC, but that only applies when all MSDU’s are destined for the same station (RA) | Expand the sentence 12:3 with “when destined to the same RA” |  |
|  | 6.1.3 | Greg Parks | T | YES | Does MSDU reordering make sense in the QoS environment? Is this on a per Queue or per traffic class basis? Strictly Ordered Service does not make sense, should not be on a packet-by-packet basis, but on a session wide basis. | Explain how this will work Make strictly ordered service definition on a session wide basis not on a packet by packet basis. |  |
|  | 6.1.3 | Keith Amann | T | Yes | Page 12, Line 3: The statement is made that “There is no reordering of unicast MSDUs belonging to the same traffic category”. If this occurs a packet with a transmission failure could stall a queue and have a direct impact on the QoS of other ESTAs utilizing the same queue. | Remove this restriction and redefine the retry mechanism to allow for transmission of traffic to other ESTAs while this retry is pending. |  |
|  | 6.1.3 | Keith Amann | T | Yes | Page 12, Line 15: The note states that “the optional QoS facility requires unicast ordering to be maintained only within individual traffic categories”. If this occurs it is possible to “starve” traffic to ESTAs which utilize the same traffic category when a transmission failure occurs within a traffic class and a retry must be attempted. This will directly impact the QoS in the EDCF case. | Remove this restriction and allow traffic destined for other ESTAs to precede the retry. |  |
|  | 6.2.1.1.2 | Amar Ghori | T | YES | QoS is not an optional facility | Delete “optional” |  |
|  | 6.2.1.1.2 | APS | T | Yes | Having a variable MSDU MTU depending on features seems to leak unnecessary MAC-layer knowledge into layers above the MAC. | Define a constant MSDU MTU. |  |
|  | 6.2.1.1.2 | APS | T | Yes | The semantics of the “Contention” and “Contention-Free” service parameters are not adequately defined. Do they express a preference or an absolute constraint? This parameter would also appear to be able to affect MSDU re-ordering over and above that implied in 6.1.3. Also these two cases and the priority classes are different. The priority class parameter can be transported exactly, but the Contention/Contention-free cannot – it has to be inferred inexactly by the receiver. | Remove the Contention & Contention-free priority classes. |  |
|  | 6.2.1.1.2 | Bob O’Hara | T | Y | Why has the MTU been reduced when an option is used? Other standards, notably 802.3, have expanded their MTU when required to carry additional information, such as VLAN tags. | Extend the allowable size of the MSDU rather than reducing it, when using the “optional facilities”. |  |
|  | 6.2.1.1.2 | Greg Parks | T | YES | QoS is not an optional facility | Delete “optional” |  |
|  | 6.2.1.1.2 | J. Ho | t | Y | Eight additional priority parameter values is not adequate in the identification of parameterized and prioritized QoS. | Expand the priority parameter values to sixteen or the service class parameter values to four. |  |
|  | 6.2.1.1.2 | Simon Black | T |  | I think it is possible that AES might push the max MSDU length beyond 2304 octets | Check with TGi |  |
|  | 6.2.1.1.2 | Spiess | T | N | “Contention” should be represented by a traffic category of zero. Is there a reason Contention should not be identical to zero? What is the integral value of ContentionFree? | Since the traffic category values are explicitly defined, the old Contention and ContentionFree values should also be mapped to integers. |  |
|  | 6.2.1.1.2 | Spiess | T | N | The restriction on StrictlyOrdered should be mentioned here. | Append “StrictlyOrdered is not available when Traffic Categories 1-7 are in use.” |  |
|  | 6.2.1.1.2 | Steve Gray | T | Y | I think it is possible that AES might push the max MSDU length beyond 2304 octets | Check with TGi |  |
|  | 6.2.1.1.4 | Amar Ghori | T | YES | We should consider passing back what is available if a request cannot be fulfilled. That way there can be negotiation rather than a game of 20 questions | IBID |  |
|  | 6.2.1.1.4 | Greg Parks | T | YES | We should consider passing back what is available if a request cannot be fulfilled. That way there can be negotiation rather than a game of 20 questions | IBID |  |
|  | 6.2.1.3.2 | Bob O’Hara | T | Y | The text in brackets is entirely incorrect. 802.11E will be a supplement to 802.11. IT IS NOT A SEPARATE STANDARD. Therefore any statement that say “never” as this one and NOTE 2, is incorrect, because current implementations can do what is said to never happen. “Not likely” and “never” are not identical. | Delete the bracketed text (in two places) and NOTE 2. |  |
|  | 6.2.1.3.2 | Fischer,Michael | T | yes | The wording of the text in the insertions to transmission status (b) and (i) and to Note 2 are not appropriate for a draft generated pursuant to a supplement PAR. | Change both instances of the added text for (b) and for (i) to: should not be used by new implementations, see NOTE 2 Change note 2 text to read: It is not necessary, and not recommended, ever to return transmission status (b) or (i), especially for implementations which support the QoS facility. It is not possible to determine the occurrence of either of these two cases of an MSDU delivery attempt being abandoned (due to excessive retries for status (b) or due to exceeding the limit on transmit lifetime for status (i)) from local state available within the MAC sublayer entity at the time the MA-UNITDATA.request is received from LLC. Because the MAC data service provided by IEEE Std 802.11 is connectionless, return of transmission status (b) or (i) is incompatible with the semantics of the MA-UNITDATA-STATUS.indication primitive. Implementers are advised that the inclusion of transmission status (b) and (i) in this clause should not be interpreted as a requirement that conformant implementations be able to return these status indications, and are directed to the absence of any reference to this clause in Annex A.4, of this standard as well as in IEEE Std 802.11-1999 and IEEE Std 802.11-1997. |  |
|  | 6.2.1.3.2 | Harry Worstell | T | YES | It is not clear why the dropping of a packet (due to failed delivery) is not reportable to the upper layers. | Clarify. |  |
|  | 6.2.1.3.2 | Harry Worstell | T | YES | See rationale for 9.2.5.3. | P 14, L 10, Insert text: Undeliverable (MSDUTimer reached aMaxMSDULifetime[TC] before successful delivery of an MSDU of traffic category TC) |  |
|  | 6.2.1.3.2 | Mathilde Benveniste | T | Yes | P 14, L 10 Missing information. Change is needed for completeness. | Insert text: Undeliverable (MSDUTimer reached aMaxMSDULifetime[TC] before successful delivery of an MSDU of traffic category TC) |  |
|  | 6.2.1.3.2 | Matthew Sherman | T | YES | It is not clear why the dropping of a packet (due to failed delivery) is not reportable to the upper layers. | Clarify. |  |
|  | 6.2.1.3.2 | Matthew Sherman | T | YES | See rationale for 9.2.5.3. | P 14, L 10, Insert text: Undeliverable (MSDUTimer reached aMaxMSDULifetime[TC] before successful delivery of an MSDU of traffic category TC) |  |
|  | 6.3.1.3.2 | Amar Ghori | T | YES | Should we include a response for “uncorrectable error” and possibly “correctable error” | We should include as suggested. |  |
|  | 6.3.1.3.2 | Greg Parks | T | YES | Should we include a response for “uncorrectable error” and possibly “correctable error” | We should include as suggested. |  |