IEEE P802.11
Wireless LANs

Normative Text for Timestamp field

Date:	July 9, 2001

Author:	Yoshihiro Ohtani
	Sharp Corporation
	2613-1 Ichinomoto-cho Tenri Nara Japan
	Phone: +81 (0743) 65-4529
	Fax: +81 (0743) 65-1841
	e-Mail: ohtani@slab.tnr.sharp.co.jp

Abstract

This document contains the material proposed to Tge for inclusion in the draft in the form of insertions into and replacements for material in of IEEE std 802.11-1999, as updated by IEEE STd 802.11e/D1, March 2001.

Editorial notes appear in bold italic Times New Roman font, informative notes appear in normal Arial font, and normative text appears in normal Times New Roman font.  Open issues are highlighted using red text in normal Arial font, and begin with "OPEN ISSUE:".  Changes to existing text in current standard are shown underlined and in red for additions and red with strikethrough for deletions.

Modify Table 22B in clause 7.2.2 as follows:

| octets: 2 | 2 | 6 | 6 | 6 | 2 | 6 or 0 | 2 | 2 or 0 | 0-2310 | 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frame Control | Dur/ID | Address 1 | Address 2 | Address 3 | Seq Ctrl | Address 4 | QoS Control | Timestamp | Frame Body | FCS |

Add the following paragraph to Clause 7.2.2 at a place where the editor deems appropriate:

The timestamp field is optional and is allowed only for data frames associated with a parameterized QoS. The TSInfo of the respective TSPEC of the parameterized QoS TS should have bit 1 set to 1. The field indicates the time value in reference to the TSF timer value in units of 16 s.  The sending ESTA computes this field, by adding the Delay Bound value of the stream to the TSF timer value at the time the MSDU is received by the MAC from LLC and discarding the four least significant bits and the twelve most significant bits.

Change the contents of Figure 42.8 as follows:

| bits: 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Traffic Type | Time Stamp | Ack Policy |  | FEC | Delivery Priority (0-7) |  |  | reserved (0) |  |  |  |  |  |  |  |

Figure 42.8 – TS Info field

Add the following paragraph to Clause 7.3.2.15:

The timestamp field indicates whether the time stamp would be inserted in the MPDU headers belonging to this TC. The field is set to 1 if such a time stamp will be inserted and will be set to 0, otherwise.