IEEE P802.11
Wireless LANs

Proposed 802.11h base document 2.0

Date	December 2001

Author	Andrew Myles
	Cisco Systems
	Level 2, 3 Innovation Road,
	North Ryde, 2113, Australia
	Phone: +61 2 84461010
	Fax: +61 2 84461001
	e-Mail: andrew.myles@cisco.com

Abstract

The purpose of this submission is to provide a self-consistent, relatively complete base document that the 802.11h group can use to move discussions forward quickly. The submission reflects most of the 800+ editorial and technical comments in LB29 The author has also generated a spreadsheet containing responses to all 800+ comments, which can be submitted  if required. This document is not an official 802.11h draft.

The elements of this document related to TPC are generally as approved by the group at the Austin meeting. In a number of cases, the author has either added text that was missing or slightly modified text that was ambiguous or required improvement in expression. All (most) of these variations are highlighted in editor’s notes in green.

The elements of this document related to DFS were mostly not discussed at the Austin meeting and so not agreed. The author has combined various bits of available text and LB29 comments with D1.0 in an attempt to produce a document that is at least reasonably consistent and ready for discussion for the group in teleconferences and at the Dallas meeting. Although the text is substantially different from D1.0 the technical content is roughly the same. All DFS related elements of this document are highlighted in blue.

Draft Supplement to STANDARD FOR
Telecommunications and Information Exchange Between Systems -
LAN/MAN Specific Requirements -

Part 11: Wireless Medium Access Control (MAC) and physical layer (PHY) specifications:

Spectrum and Transmit Power Management
extensions in the 5GHz band in Europe

Sponsored by the
IEEE 802 Committee
of the
IEEE Computer Society

Copyright (c) 2001 by the Institute of Electrical and Electronics Engineers, Inc.
345 East 47th Street
New York, NY 10017, USA
All rights reserved.

This is an unapproved draft of a proposed IEEE Standard, subject to change. Permission is hereby granted for IEEE Standards Committee participants to reproduce this document for purposes of IEEE standardization activities. If this document is to be submitted to ISO or IEC, notification shall be given to the IEEE Copyright Administrator. Permission is also granted for member bodies and technical committees of ISO and IEC to reproduce this document for purposes of developing a national position. Other entities seeking permission to reproduce this document for standardization or other activities, or to reproduce portions of this document for these or other uses, must contact the IEEE Standards Department for the appropriate license. Use of information contained in this unapproved draft is at your own risk.

IEEE Standards Department
Copyright and Permissions
445 Hoes Lane, P.O. Box 1331
Piscataway, NJ 08855-1331, USA

# Introduction

(This introduction is not part of IEEE P802.11h, Draft Supplement to STANDARD FOR Telecommunications and Information Exchange Between Systems - LAN/MAN Specific Requirements - Part 11: Wireless Medium Access Control (MAC) and physical layer (PHY) specifications: DFS and TPC extensions in the 5GHz band in Europe)

Example:

At the time this standard was completed, the working group had the following membership:

Chair of working group (to be provided by IEEE editor at time of publication)

List of members of working group (to be provided by IEEE editor at time of publication)

The following persons were on the balloting committee:

List of members of balloting committee (to be provided by IEEE editor at time of publication)

# Contents

Introduction	ii

Contents	iii

2. Normative References	2

3. Definitions	2

4. Abbreviations and acronyms	3

5.5 Relationships between services	3

5.7.2 Association	3

5.7.3 Reassociation	3

7.1.3.1.2 Type and Subtype fields	4

7.2.3.1 Beacon frame format	5

7.2.3.4 Association Request frame format	5

7.2.3.5 Reassociation Request frame format	5

7.2.3.9 Probe Response frame format	6

7.2.3.12 Action frame body format	7

7.3.1.4 Capability Information field	8

7.3.1.7 Reason Code field	9

7.3.1.9 Status Code field	9

7.3.2 Information elements	10

7.3.2.8 Power Constraint element	11

7.3.2.9 Power Capability element	13

7.3.2.10 TPC Request element	14

7.3.2.11 TPC Report element	14

7.3.2.12 Supported Channels element	16

7.3.2.13 Channel Switch Announcement element	17

7.3.2.14 Channel Measurement Request element	18

7.3.2.15 Channel Measurement Report element	19

7.4 Spectrum Management Actions	20

7.4.1 Channel Measurement Request frame format	21

7.4.2 Channel Measurement Report frame format	22

7.4.3 TPC Request frame format	23

7.4.2 TPC Report frame format	24

9.11 TPC procedures	25

9.11.1 Association based on transmit power capability	26

9.11.2 Specification of a regulatory and local maximum transmit power levels	27

9.11.3 Selection of a transmit power	28

9.11.4 Optimization of the transmit power	29

9.12 DFS procedures	30

9.12.1 Measuring of channels	31

9.12.2 Reporting channel measurements	32

9.12.3 Operating in a channel	33

9.12.4 Selecting and advertising a new channel	34

10.3 MLME SAP interface	35

17.2.2 TXVECTOR parameters	35

17.2.3 RXVECTOR parameters	35

Annex A – PICS	36

A.4.3 IUT configuration	36

A.4.11 TPC extensions	36

A.4.12 DFS extensions	36

Annex C – Formal description of MAC operation	37

Annex D – ASN.1 encoding of the MAC and PHY MIB	38

Annex E – Bibliography	42

Annex F – Measurement Requests and Reports in DFS	43

F.1 Basic Request	44

F.2 Basic Report	44

F.3 RSSRI Histogram Request	45

F.4 RSSRI Histogram Report	45

F.5 Extended CCA Request	46

F.6 Extended CCA Report	46

F.7 Unknown RSSRI Histogram Request	47

F.8 Unknown RSSRI Histogram Report	47

F.9 BSS Request	48

F.10 BSS Report	48

F.11 STA Request	49

F.12 STA Report	49

Annex G – Using TPC and DFS facilities for other purposes	51

Draft Supplement to STANDARD FOR
Telecommunications and Information Exchange Between Systems -
LAN/MAN Specific Requirements -

Part 11: Wireless Medium Access Control (MAC) and physical layer (PHY) specifications:

Spectrum and Transmit Power Management
extensions in the 5GHz band in Europe

Editors note: the official name of the standard from the PAR is “Spectrum and Transmit Power Management extensions in the 5GHz band in Europe”. The editor proposes that the name be simplified to “Spectrum management in the 5GHz band in Europe” so that the language is more consistent with the terminology used in the rest of the text. Any comments?

[ The editing instructions contained in this supplement define how to merge the material contained herein into the current edition of IEEE Std 802.11 1999 Edition and the 802.11a, 802.11b, 802.11b-cor1 and 802.11d supplements

Editors note: D1.0 only referred to 802.11a and 802.11d. Shouldn’t it also refer to 802.11b and 802.11b-cor1?

The editing instructions are shown in bold italic. Three editing instructions are used:

Change is used to make small corrections in existing text or tables. The editing instruction specifies the location of the change and describes what is being changed either by using strikethrough (to remove old material) or underscore (to add new material).

Delete removes existing material.

Insert adds new material without disturbing the existing material. Insertions may require renumbering. If so, renumbering instructions are given in the editing instructions.

Editorial notes will not be carried over into future editions.]

Editors note: many of the editorial changes in this document reflect the comments in LB29

# 2. Normative References

Insert the following citation at the appropriate location in clause 2:

ERC/DEC/(99)23, ERC Decision of 29 November 1999 on the harmonized frequency bands to be designated for the introduction of High Performance Radio Local Area Networks (HIPERLANs) .

Editors note: the normative reference now contains a footnote that indicates where the document may be obtained. This change is consistent with a number of LB29 comments

Editors note: it was suggested in the conference call on 2 January 2002 that the 802.11d document should be included as a normative reference. However, on reflection, it is the editor’s view that this is probably not necessary because this document will be merged with 802.11d in the future. Comments?

# 3. Definitions

Insert the following new definitions at the appropriate locations in clause 3, renumbering as necessary:

3.51 dynamic frequency selection (DFS): Facilities mandated to satisfy requirements in some regulatory domains that specify precedence for licensed users and uniform channel spreading in the 5GHz band. DFS facilities may also be used for other purposes, such as radio resource management.

Editors note: the language used to define DFS has been changed to make it consistent with the language approved for the TPC definition at the Austin meeting. The intent of the text has not changed. Comments?

3.52 transmit power control (TPC): Facilities mandated to satisfy requirements in some regulatory domains for maximum transmit power and average transmit power mitigation in the 5GHz band. TPC facilities may also be used for other purposes, such as interference reduction, range control and power optimization.

3.53 transmit power: transmit power is the power measured at the antenna.

3.54 receive power: receive power is the power measured at the antenna.

Editors note: the editor has added definitions for “transmit power”  and “receive power” to satisfy various LB29 comments. During the conference call on 2 January 2002, Peter Ecclesine (Cisco) offered to provide better definitions. They will be incorporated after they are provided.

# 4. Abbreviations and acronyms

Insert the following text in the appropriate location in clause 4:

DFS	dynamic frequency selection

TPC	transmit power control

## 5.5 Relationships between services

Add to the definition of class 1, management frames as follows:

vi)

Action

### 5.7.2 Association

Insert the following text under "Association request" just after entry for "ESS ID":

- Power Capability

- Supported channels

### 5.7.3 Reassociation

Insert the following text under "Reassociation request" just after entry for "ESS ID":

- Power Capability

- Supported channels

##### 7.1.3.1.2 Type and Subtype fields

Change the contents of Table 1 as follows:

Table 1 - Valid type and subtype combinations

| Type Value b3 b2 | Type description | Subtype value b7 b6 b5 b4 | Subtype description |
| --- | --- | --- | --- |
| 00 | Management | 1101 | Action |
| 00 | Management | 1110-1111 | Reserved |

#### 7.2.3.1 Beacon frame format

Change order 11 information field and add order 14 and order 15 information fields as follows:

Table 5 - Beacon frame body

| Order | Information | Notes |
| --- | --- | --- |
| 11 | Country | Country element shall be present if dot11MultiDomainCapabilityEnabled is true or dot11Spectrum ManagementCapabilityEnabled is true |
| 14 | Power Constraint | Power Constraint element shall be presentif dot11SpectrumManagementCapabilityEnabled is true |
| 15 | Channel Switch Announcement | Channel Switch Announcement element may be present if dot11SpectrumManagementCapabilityEnabled is true |

#### 7.2.3.4 Association Request frame format

Add order 5 and order 6 information fields as follows:

Table 7 - Association Request frame body

| Order | Information | Notes |
| --- | --- | --- |
| 5 | Power Capability | Power Capability element shall be present if dot11SpectrumManagementCapabilityEnabled is true |
| 6 | Supported Channels | Supported Channels element shall be present if dot11SpectrumManagementCapabilityEnabled is true |

#### 7.2.3.5 Reassociation Request frame format

Add order 6 and order 7 information fields as follows:

Table 9 - Reassociation Request frame body

| Order | Information | Notes |
| --- | --- | --- |
| 6 | Power Capability | Power Capability element shall be present if dot11SpectrumManagementCapabilityEnabled is true |
| 7 | Supported Channels | Supported Channels element shall be present if dot11SpectrumManagementCapabilityEnabled is true |

#### 7.2.3.9 Probe Response frame format

Change order 10 information field and add order 14 and 15 information fields as follows:

Table 12 Probe Response frame body

| Order | Information | Notes |
| --- | --- | --- |
| 10 | Country | Country element shall be present if dot11MultiDomainCapabilityEnabled or dot11SpectrumManagementCapabilityEnabled is true |
| 13 | Power Constraint | Power Constraint element shall be present if dot11SpectrumManagementCapabilityEnabled is true |
| 14 | Channel Switch Announcement | Channel Switch Announcement element may be present if dot11SpectrumManagementCapabilityEnabled is true |

#### Table 1 - Hidden Station frame body

Insert the following clause and renumber as necessary:

#### 7.2.3.12 Action frame body format

Editors note: this section contains possible text for an Action frame:

- It uses the basic format for the Action frame suggested by 802.11e. However, it is far less prescriptive about the use of the fields than 802.11e, thus providing far more flexibility in its use.

- During the conference on 2 January 2002 it was decided to use this text and that 802.11h should discuss the issues with 802.11e over the coming months.

- An even simpler version would remove the Action Specific and Dialog Token fields, making the Action frame body format a proxy for an escape code.

The Action frame indicates a management action from a particular category. The Action frame body is shown in Figure 1.

|  | Category | Action | Action Specific | Dialog Token |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 |
|  | Actions |  |  |  |
| Octets: | variable |  |  |  |

Figure 1 – Action frame body

The Category field shall be set to one of the values shown in Table 1. The details of the actions allowed in each category are described in the clause for the category referenced in Table 1.

The Action field shall be set to a value that is allowed in the Category. The most significant bit shall normally be set equal to 0. However, if a STA receives a unicast Action frame with an unrecognized Category field, Action field, Action Specific field, Dialog Token field or some other syntactic error and the most significant bit of the Action field set to 0 then the STA shall return the entire frame to the source with the most significant bit of the Action field set equal to 1.

The Action Specific field shall be set to a value that is specific to the particular Category and Action. The Action Specific field is often used to carry a status value or an activation delay.

The Dialog Token field shall be set to a value that is specific to the particular Category and Action. The Dialog Token field is normally used to associate a series of frames in a multi-management frame transaction.

The Actions field shall contain fixed fields and elements that are specific to the Category and Action.

Table 1 - Category values

| Name | Value | See clause |
| --- | --- | --- |
| Spectrum management | 0 | 7.4 |
| Reserved | 1-255 | - |

#### 7.3.1.4 Capability Information field

Change the supporting text as follows:

The length of the Capability Information field is two octets. The Capability Information field consists of the following subfields: ESS, IBSS, CF-Pollable, CF-Poll Request, Privacy, Short Preamble, PBCC, Channel Agility and Spectrum Management. The format of the Capability Information field is as illustrated in Figure 27.

Editors note: this change is required to fit in with the existing text

Change the contents of Figure 27 as shown:

| B0 | B1 | B2 | B3 | B4 | B5 | B6 | B7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ESS | IBSS | CF Pollable | CF Poll Request | Privacy | Short Preamble | PBCC | Channel Agility |
| B8 | B9 | B10 | B11 | B12 | B13 | B14 | B15 |
| Spectrum Management | Reserved (0) | Reserved (0) | Reserved (0) | Reserved (0) | Reserved (0) | Reserved (0) | Reserved (0) |

Figure 27 - Capability Information fixed field

Insert the following text at the end of clause 7.3.1.4:

A STA shall set the Spectrum Management subfield in the Capability Information field equal to 1 if the boolean MIB attribute dot11SpectrumManagementCapabilityEnabled is true, otherwise it shall be set equal to 0.

Editors note: the primary purpose of the Spectrum Management bit is to allow an AP to disallow association of STAs based on their spectrum management capability or for a STA to not attempt association with an AP that does not have spectrum management capability.  It was decided during the conference call on 2 January 2002 that text is required to describe this functionality. Appropriate text has been inserted into 9.11 and 9.12.

#### 7.3.1.7 Reason Code field

Change table 18 as follows:

Table 18 - Reason codes

| Reason Code | Meaning |
| --- | --- |
| 10 | Disassociated because the Power Capability element is missing or the Maximum Transmit Power Capability field in the Power Capability element is unacceptable |
| 11 | Disassociated because the Supported Channels element is missing or the Supported Channels field in the Supported Channels element is unacceptable |

Editors note: These reason codes were discussed during the conference call on 2 January 2002:

- It was decided that a reason code to inform a STA that it was “Dissassociated because Spectrum Management capability is now required” is not needed because we do not expect or want to encourage an AP to suddenly decide that Spectrum Management capability is required.

- It was decided that a reason code to inform a STA that it was “Disassociated because the Power Capability element is missing or the Maximum Transmit Power Capability field in the Power Capability element is unacceptable” is probably reasonable.

- There was some discussion about the reason code relating to Supported Channels. The editor suggests we should wait for the DFS related discussions before changing it.

#### 7.3.1.9 Status Code field

Change table 19 as follows:

Table 19 - Status codes

| Status Code | Meaning |
| --- | --- |
| 19 | Association request rejected because Spectrum Management capability is required |
| 20 | Association request rejected because the Power Capability element is missing or the Maximum Transmit Power Capability field in the Power Capability element is unacceptable |
| 21 | Association request rejected because the Supported Channels element is missing or the Supported Channels field in the Supported Channels element is unacceptable |

Editors note: These status codes were discussed during the conference call on 2 January 2002. It was decided that these status codes were probably reasonable.

### 7.3.2 Information elements

Insert the following information elements into Table 20 of clause 7.3.2 in place of the reserved IDs:

Table 20 - Element IDs

| Information element | Element ID |
| --- | --- |
| Power Constraint | 32 |
| Power Capability | 33 |
| TPC Request | 34 |
| TPC Report | 35 |
| Supported Channels | 36 |
| Channel Switch Announcement | 37 |
| Channel Measurement Request | 38 |
| Channel Measurement Report | 39 |
| Reserved | 11-15, 40-255 |

Editors note: The Channel Measurement Request and Report elements have not yet been discussed or agreed by the group.

Insert the following sub-clause. Renumber figures and tables as appropriate.

Editors note: deleted all hidden station text

#### 7.3.2.8 Power Constraint element

The Power Constraint element contains the information necessary to allow a STA to determine the local maximum transmit power level that shall not be exceeded for transmissions in various channels. The format of the Power Constraint element is shown in Figure 2.

|  | Element ID (32) | Length (variable) |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 |  |  |  |
|  | First Channel Number | Number of Channels | Local Power Constraint | The element contains one or more triplets |  |
| Octets: | 1 | 1 | 1 |  |  |

Figure 2 - Power Constraint element format

The Element ID field shall be set equal to 32. The Length field is variable and depends on the number of triplets in the element. The minimum value of the Length field is 3.

The First Channel Number field shall indicate the first channel number in the sub-band described by the triplet. The field is coded as an unsigned integer. The First Channel Number field shall monotonically increase in successive triplets.

The Number of Channels field shall indicate the number of channels in the sub-band described by the triplet. The field is coded as a positive integer. The Number of Channels field shall not contain a value that causes the sub-bands described by successive triplets to overlap.

The Local Power Constraint field shall be set equal to a value greater than or equal to the minimum transmit power mitigation required to operate in the sub-band described by the triplet to satisfy regulatory requirements. The field is coded as an unsigned integer in units of dBm. The local maximum transmit power for a channel is thus defined as the Maximum Transmit Power Level specified for the channel in the Country element minus the Local Power Constraint specified for the channel in the Power Constraint element.

Table 2 - Relative power constraints coding

Editors note: the editor has changed the format of the Power Constraint element from that in D1.0 to make it consistent with the definition of the Country element in 802.11d. In addition, the new format allows mitigation to be specified in 1dB decrements.

Editors note: during conference call on 2 January 2002 it was decided that the Pad field used by 802.11d is not necessary and that it should not be included in the 802.11h draft.

Editors note: While it is possible to use the Power Constraint element to specify mitigation of up to 255dB, it is unlikely that such a limit would ever be used. Any AP that did specify such a large limit would not find any STAs to associate with it. Thus there is no need to specify a lower limit in the standard explicitly. Is this acceptable? Are there any objections?

Insert the following sub-clause. Renumber figures and tables as appropriate.

#### 7.3.2.9 Power Capability element

The Power Capability element specifies the maximum power with which a STA is capable of transmitting in the various channels. The format of the Power Capability element is shown in Figure 3.

|  | Element ID (33) | Length (variable) |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 |  |  |  |
|  | First Channel Number | Number of Channels | Maximum Transmit Power Capability | The element contains one or more triplets |  |
| Octets: | 1 | 1 | 1 |  |  |

Figure 3 - Power Capability element format

The Element ID field shall be set equal to 33. The Length field is variable and depends on the number of triplets in the element. The minimum value of the Length field is 3.

The First Channel Number field shall indicate the first channel number in the sub-band described by the triplet. The field is coded as an unsigned integer. The First Channel Number field shall monotonically increase in successive triplets.

The Number of Channels field shall indicate the number of channels in the sub-band described by the triplet. The field is coded as a positive integer. The Number of Channels field shall not contain a value that causes the sub-bands described by successive triplets to overlap.

The Maximum Transmit Power Capability field shall be set equal to the maximum transmit power with which a STA is capable of transmitting in the sub-band described by the triplet. The field is coded as a signed integer in units of dBm.

Editors note: The inclusion of the Power Capability element is the result of a motion passed in Austin.

Editors note: During the conference call on 2 January 2002 it was decided to change the format of the Power Capability element so that it defined capabilities in multiple channels using a format similar to that used by 802.11d for the Country element, with the exception of the Pad field

Editors note: Do we need to include a tolerance for the Maximum Transmit Power Capability field?

Insert the following sub-clause. Renumber figures and tables as appropriate.

#### 7.3.2.10 TPC Request element

The TPC Request element contains a request for a STA to report current transmit power and link status information using a TPC Report element. The format of the TPC Request element is shown in Figure 4.

|  | Element ID (34) | Length (0) |
| --- | --- | --- |
| Octets: | 1 | 1 |

Figure 4 - TPC Report element format

The Element ID field shall be set equal to 34. The Length field shall be set equal to 0.

Editors note: The inclusion of the TPC Request element is the result of a motion passed in Austin.

#### 7.3.2.11 TPC Report element

The TPC Report element contains current transmit power and link status information sent in response to a TPC Request element. The format of the TPC Report element is shown in Figure 5.

|  | Element ID (35) | Length (2) | Transmit Power | Link Margin | Link Margin field is optional |
| --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 |  |

Figure 5 - TPC Report element format

The Element ID field shall be set equal to 35. The Length field shall be set equal to 2 when the Link Margin field is included and 1 otherwise.

The Transmit Power field shall be set equal to the transmit power used to transmit the frame containing the TPC Report element. The field is coded as a signed integer in units of dBm. The assumed tolerances of the Transmit Power values are shown in Table 2.

Table 2 – Tolerances for Transmit Power field in TPC Report element

| Transmit Power (dBm) | Tolerance (dBm) |
| --- | --- |

The Link Margin field, which is optional, contains the estimated link margin at the time and for the rate at which the frame containing the TPC Request element was received. The link margin is defined as the ratio of the received signal power to that required for effective communication. The field is coded as a signed integer in units of dB. The tolerances of the Link Margin field values are shown in Table 3.

Table 3 – Tolerances for Link Margin field in TPC Report element

| Link Margin (dB) | Tolerance (dB) |
| --- | --- |

Editors note: the inclusion of the TPC Report element is the result of a motion passed in Austin.

Editors note:  the editor made a number of changes from the Austin proposal, for which comment is invited:

- The name was changed to TPC Report for stylistic consistency with the rest of the text.

- The text proposed in Austin was unclear as to whether the Transmit Power field was supposed to be relative to the regulatory or local transmit power constraint. It is also not clear why this field needs to be expressed as a relative value, given that most other transmit powers in the draft are expressed as absolute values. Therefore, for consistency and clarity, the editor changed the syntax so that the Transmit Power is expressed as an absolute value.

- The proposal accepted in Austin constrained the link margin to a range of –3dB to +45dB. It was decided during the conference call on 2 January 2002 that such a restriction, if required, should probably in the PICS rather than in the main text.

- During the conference call on 2 January 2002 it was decided that a reference to the rate at which the TPC Report was received should be included in the text.

Editors note: In Austin it was decided tolerances were required for many parameters. What are the suggested tolerances for the:

- Transmit Power field

- Link Margin field?

Insert the following sub-clause. Renumber figures and tables as appropriate.

#### 7.3.2.12 Supported Channels element

The Supported Channels element contains a list of channels in which a STA is capable of operating. The format of the Supported Channels element is shown in Figure 6.

|  | Element ID (36) | Length (28) |
| --- | --- | --- |
| Octets: | 1 | 1 |
|  | Supported Channels |  |
| Octets: | 28 |  |

Figure 6 - Supported Channels element format

The Element ID field shall be set equal to 36. The Length field shall be set equal to 28.

The Supported Channels field is a 28-octet bit field, where bit 0 corresponds to channel 1 and bit 199 corresponds to channel 200. A bit in the field shall be set equal to 1 when the STA supports operation in the corresponding channel, otherwise it shall be set equal to 0. Bits 200-223 shall be set equal to 0.

Insert the following sub-clause. Renumber figures and tables as appropriate.

#### 7.3.2.13 Channel Switch Announcement element

The Channel Switch Announcement element allows an AP in a BSS to advertise when it is changing to a new channel and the channel number of the new channel. The format of the Channel Switch Announcement element is shown in Figure 7.

|  | Element ID (37) | Length (2) |
| --- | --- | --- |
| Octets: | 1 | 1 |
|  | New Channel Number | Channel Switch Count |
| Octets: | 1 | 1 |

Figure 7 - Channel Switch Announcement element format

The Element ID field shall be set equal to 37. The Length field shall be set equal to 2.

The New Channel Number field shall be set equal to the number of the channel to which the AP is moving.

The Channel Switch Count field shall be set equal to the number of TBTTs until the AP sending the Channel Switch Announcement element will switch to the new channel. A value of 1 indicates that the switch will occur immediately after the next TBTT. A value of 0 indicates that the switch will occur immediately.

Editors note: No way is specified for a channel switch to be advertised in an IBSS because it is not yet clear DFS can be achieved reasonably simply in an IBSS.

Insert the following sub-clause. Renumber figures and tables as appropriate.

#### 7.3.2.14 Channel Measurement Request element

The Channel Measurement Request element contains a request that the receiving STA undertake a measurement for a specified duration in a particular channel. The format of the Channel Measurement Request element is shown in Figure 8.

|  | Element ID (38) | Length (variable) |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Octets: | 1 |  |  |  |  |
|  | Channel Number | Measurement Token | Measurement Duration | Measurement ID | Measurement Request |
| Octets: | 1 | 1 | 2 | 2 | variable |

Figure 8 - Channel Measurement Request element format

The Element ID field shall be set equal to 38. The Length field is variable and depends on the length of the Measurement Request field. The minimum value of the Length field is 6.

The Channel Number field shall be set equal to the channel number for which a measurement report is requested.

The Measurement Token shall be set to a non-zero number that is unique among the Channel Measurement Request elements in a particular Channel Measurement Request frame.

The Measurement Duration field shall be set equal to the number of TUs for which the channel specified in the Channel Number field shall be measured. A value of zero shall mean that the measurement shall use the same duration and starting time as the immediately previous Channel Measurement Request element in the same Channel Measurement Request frame. A value of zero shall not be used when the Channel Measurement Request element is the first element in the Channel Measurement Request frame

The Measurement ID field shall be set to a number that identifies the format and contents of the Measurement Request field. The IEEE shall be responsible for allocating the Measurement ID number space. Annex F describes those Measurement ID’s that have been allocated and the format of the corresponding Measurement Request field.

The Measurement Request field shall contain the specification of a requested channel measurement.

Insert the following sub-clause. Renumber figures and tables as appropriate.

#### 7.3.2.15 Channel Measurement Report element

The Channel Measurement Report element contains a measurement report for a specified channel. The format of the Channel Measurement Report element is shown in Figure 9.

|  | Element ID (39) | Length (variable) |  |  |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 |  |  |
|  | Channel Number | Measurement Token | Measurement ID | Measurement Report |
| Octets: | 1 | 1 | 2 | variable |

Figure 9 - Channel Measurement Report element format

The Element ID field shall be set equal to 39. The Length field is variable and depends on the length of the Measurement Report field. The minimum value of the Length field is 4.

The Channel Number field shall be set equal to the channel to which the Measurement Report element refers.

The Measurement Token shall be set equal to the Measurement Token in the corresponding Channel Measurement Request element. If the Channel Measurement Report is being provided autonomously then the Measurement Token shall be set equal to zero.

The Measurement ID field shall be set to a number that identifies the format and contents of the Measurement Report field. The IEEE shall be responsible for allocating the Measurement ID number space. Annex F describes those Measurement ID’s that have been allocated and the format of the corresponding Measurement Report field.

The Measurement Report field shall contain the specification of the channel measurement report.

Insert clause 7.4 as follows. Renumber figures and tables as appropriate:

## 7.4 Spectrum Management Actions

The Action codes within the Spectrum Management category of an Action frame are defined in Table 4. The details of the frame bodies associated with each Action code are described in the following sub-clauses.

Table 4 – Spectrum Management Action codes

| Action code | Meaning |
| --- | --- |
| 0 | Channel Measurement Request |
| 1 | Channel Measurement Report |
| 2 | TPC Request |
| 3 | TPC Report |
| 4-255 | Reserved |

Insert clause 7.4.1 as follows. Renumber figures and tables as appropriate:

### 7.4.1 Channel Measurement Request frame format

The Channel Measurement Request frame uses the Action frame body format and is transmitted by a STA requesting another STA to measure one or more channels. The format of the Channel Measurement Request frame body is shown in Figure 10.

|  | Category (0) | Action (0) | Action Specific | Dialog Token |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 |
|  | Channel Measurement Request elements |  | Shall contain one or more Channel Measurement Request elements |  |
| Octets: | variable |  |  |  |

Figure 10 - Channel Measurement Request frame body format

The Category field shall be set to 0 (representing Spectrum Management).

The Action field shall be set to 0 (representing a Channel Measurement Request frame).

The Action Specific field shall be set equal to an activation delay, which represents the number of TBTTs until the measurement specified by the first Channel Measurement Request element should start. A value of 0 indicates that processing shall commence immediately. A value of 1 indicates that processing shall commence after the next TBBT. Processing of subsequent Channel Measurement Request elements shall occur sequentially.

The Dialog Token field shall be set to a non-zero value chosen by the STA sending the request to identify the request/report transaction. A pending request is superseded by a subsequent request from the same STA with the same Dialog Token value.

The Channel Measurement Request elements shall be set as described in 7.3.2.14.

Insert clause 7.4.2 as follows. Renumber figures and tables as appropriate:

### 7.4.2 Channel Measurement Report frame format

The Channel Measurement Report frame uses the Action frame body format and shall be transmitted by a STA in response to a Channel Measurement Request frame or by a STA autonomously providing channel measurement information. The format of the Channel Measurement Report frame body is show in Figure 11.

|  | Category (0) | Action (1) | Action Specific | Dialog Token |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 |
|  | Channel measurement Report elements |  | Shall contain one or more Channel Measurement Report elements |  |
| Octets: | variable |  |  |  |

Figure 11 - Channel Measurement Report frame body format

The Category field shall be set to 0 (representing Spectrum Management).

The Action field shall be set equal to 1 (representing a Channel Measurement Report frame).

The Action Specific field shall contain the status of the response as shown in Table 5.

The Dialog Token field shall be set equal to the value in the corresponding Channel Measurement Request frame. If the Channel Measurement Report frame is not being transmitted in response to a Channel Measurement Request frame then the Dialog token shall be set equal to zero.

The Channel Measurement Report elements shall be set as described in 7.3.2.15.

Table 5 - Channel Measurement Report Action Specific codes

| Value | Status |
| --- | --- |
| 0 | The frame contains one or more Channel Measurement Report elements in response to one or more Channel Measurement Request elements in a Channel Measurement Request. |
| 1 | The frame contains one or more Channel Measurement Report elements provided autonomously. |
| 2 | The status indicates that a STA received one or more Channel Measurement Request elements in a Channel Measurement Request frame for which it is unable to provide a response. The frame contains the corresponding Channel Measurement Request elements. This status is not available for use with those Channel Measurement Request elements that are described as mandatory in Annex F. |
| 3-255 | Reserved. |

Insert clause 7.4.3 as follows. Renumber figures and tables as appropriate:

### 7.4.3 TPC Request frame format

The TPC Request frame uses the Action frame body format and is transmitted by a STA requesting another STA, with which it can communicate directly, for transmit power and link margin information. The format of the TPC Request frame body is shown in Figure 12.

|  | Category (0) | Action (2) | Action Specific (0) | Dialog Token |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 |
|  | TPC Request element |  |  |  |
| Octets: | 2 |  |  |  |

Figure 12 - TPC Request frame body format

The Category field shall be set to 0 (representing Spectrum Management).

The Action field shall be set to 2 (representing a TPC Request frame).

The Action Specific field shall be set equal to 0.

The Dialog Token field shall be set to a non-zero value chosen by the STA sending the request to identify the transaction. A pending request is superseded by a subsequent request from the same STA with the same Dialog Token.

The TPC Request element shall be set as described in 7.3.2.10.

Insert clause 7.4.2 as follows. Renumber figures and tables as appropriate:

### 7.4.2 TPC Report frame format

The TPC Report frame uses the Action frame body format and shall be transmitted by a STA in response to a TPC Request frame. The format of the TPC Report frame body is show in Figure 13.

|  | Category (0) | Action Code (1) | Action Specific (0) | Dialog Token |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 |
|  | TPC Report element |  |  |  |
| Octets: | variable |  |  |  |

Figure 13 - TPC Report frame body format

The Category field shall be set to 0 (representing Spectrum Management).

The Action field shall be set equal to 3 (representing a TPC Report frame).

The Action Specific field shall be set to 0.

The Dialog Token field shall be set equal to the value in the corresponding TPC Request frame.

The TPC Report element shall be set as described 7.3.2.11.

Insert sub-clause 9.11 as follows. Renumber figures and tables, as appropriate:

Editors note: We need to decide if 9.11 is part of clause 9 or clause 11, which was suggested by some in the LB29

## 9.11 TPC procedures

The CEPT Member Administrations in Europe require the use of transmit power control (TPC) by wireless LANs operating in the 5GHz band to reduce interference with licensed users. The current CEPT regulations specify a regulatory maximum transmit power and an average mitigation requirement for each allowed channel (ERC/DEC/(99)23).

This clause describes TPC procedures that may be used to satisfy these and similar future regulatory requirements in Europe. The procedures may also satisfy comparable needs in other regulatory domains and may be useful for other purposes, such as interference reduction, range control and power optimization.

STAs shall use the TPC procedures defined in this clause if and only if the boolean MIB attribute dot11SpectrumManagementEnabled is true. dot11SpectrumManagementEnabled shall be set true when required by regulatory authorities. It may also be set true in other circumstances. The TPC procedures provide for the:

- Association of STAs with an AP in a BSS based on the STA’s power capability using Power Capability elements in Association Requests and Reassociation Requests (9.11.1).

- Specification of regulatory and local maximum transmit power levels for multiple channels using Country and Power Constraint elements in Beacons and Probe Responses (9.11.2).

- Selection of a transmit power for each MPDU transmitted in a channel within the constraints imposed by the rules (9.11.3).

- Optimization of transmit power based on a range of information, including path loss and receive margin estimates (9.11.4).

For the purposes of TPC, a STA with the boolean MIB attribute dot11SpectrumManagementEnabled set equal to true shall not operate in a BSS or IBSS unless the Spectrum Management bit is set equal to 1 in the Capability Information field in Beacons and Probe Responses received from the BSS or IBSS. Similarly, an AP with the boolean MIB attribute dot11SpectrumManagementEnabled set equal to true shall not associate or reassociate a STA unless the Spectrum Management bit is set equal to 1 in the Capability Information field in Association and Reassociation Requests received from the STA.

Editors note: the Austin meeting decided to change the introductory text in 9.11 to align with 01-543

### 9.11.1 Association based on transmit power capability

A STA shall provide an AP with its maximum transmit power capability when associating or reassociating using a Power Capability element in Association Request or Reassociation Request frames.

An AP may use the maximum transmit power capability of associated STAs to help determine the local transmit power constraint for any BSS it maintains.

An AP may reject an association or reassociation request from a STA if it considers the STA’s maximum transmit power capability is unacceptable or a transmit power capability is not provided.

Editors note: the text in 9.11.1 is proposed to align with the Austin decision to support TPC Request and Report frames

### 9.11.2 Specification of a regulatory and local maximum transmit power levels

Regulatory authorities are responsible for defining a regulatory maximum transmit power level and a mitigation requirement for each allowed channel in a regulatory domain.

An AP operating a BSS, a STA starting an IBSS or a STA actively scanning shall not transmit in a channel unless they know, by whatever means, the TPC related regulatory requirements for the channel.

An AP in a BSS or a STA starting an IBSS shall determine a local maximum transmit power level for other STAs in the BSS or IBSS so that the average mitigation requirements are satisfied. A conservative approach is to set the local maximum transmit power level equal to the regulatory maximum transmit power level minus the average mitigation requirement. However, it may be possible to satisfy the average mitigation requirement using a higher local maximum transmit power level. A lower local maximum transmit power level may be used for other purposes, such as range control or interference reduction.

The regulatory maximum transmit power level for at least the current channel shall be communicated to other STAs in a BSS or an IBSS using a Country element in Beacons and Probe Responses. The local maximum transmit power level for at least the current channel shall be communicated to other STAs in the a BSS or an IBSS using a combination of the Country and Power Constraint elements in Beacons and Probe Responses.

In a BSS, the AP shall specify the local maximum transmit power in all Beacons and Probe Responses it transmits. The AP may vary the local maximum transmit power level during the life of a BSS. However, it should consider network stability in deciding how often or by how much to change the local maximum transmit power level.

In an IBSS, the STA starting the IBSS shall specify the local maximum transmit power level in all Beacons and Probe Responses it transmits. Other STAs in the IBSS shall adopt the same local maximum transmit power level for all Beacons and Probe Responses they transmit. The STAs in an IBSS shall not change the local maximum transmit power level during the life of an IBSS. A change to another channel shall require a new IBSS to be established.

9.11.3 Selection of a transmit power

A STA may select any transmit power for each MPDU transmitted in a channel within the constraints imposed by the following rules:

- An AP operating in a BSS shall use a transmit power less than or equal to the regulatory maximum transmit power level for the channel. However, the AP must also ensure the average regulatory mitigation requirement is respected.

- Non-AP STAs operating in a BSS or an IBSS shall use a transmit power less than or equal to the local maximum transmit power level for the channel.

- STAs that are actively scanning for a BSS or an IBSS shall use a transmit power less than or equal to the regulatory maximum transmit power level for the channel. However, the STA shall also ensure the average regulatory mitigation requirement is respected.

### 9.11.4 Optimization of the transmit power

A STA may use any information, and in particular any path loss and receive margin estimates, to dynamically optimize the transmit power for transmissions of an MPDU to another STA. The optimization methods or criteria are not standardized.

A STA may use a TPC Request frame to request another STA with which it directly communicates to respond with a TPC Report frame containing information that allows the path loss between the STAs and the receive margin at the other STA to be estimated.

A STA receiving a TPC Request Frame shall respond with a TPC Report frame containing the power used to transmit the response in the Transmit Power field. However, the STA may choose whether or not it provides an estimated link margin in a Link Margin field.

Editors note: this text was added to support the Austin decision to include TPC Request and TPC Report frames

Insert clause 9.12 as follows. Renumber as appropriate.

Editors note: We need to decide if 9.12 is part of clause 9 or clause 11, which was suggested by some in the LB29

## 9.12 DFS procedures

The CEPT Member Administrations in Europe require dynamic frequency selection (DFS) for wireless LANs operating in the 5GHz band to reduce interference with licensed users. The current CEPT regulations specify that systems should detect and avoid licensed users. Further, they require the use of channel selection algorithms that result in uniform channel spreading across a minimum number of channels (ERC/DEC/(99)23). Regulatory authorities may, in the future, specify a maximum time to detect a licensed user and a maximum time that a STA may continue transmitting in a channel after detecting a licensed user.

This clause describes DFS procedures that may be used to satisfy these and similar future regulatory requirements in Europe. The procedures may also satisfy comparable needs in other regulatory domains and may be useful for other purposes, such as radio resource management.

STAs shall use the DFS procedures defined in this clause if and only if the boolean MIB attribute dot11SpectrumManagementEnabled is true. dot11SpectrumManagementEnabled shall be set true when required by regulatory authorities. It may also be set true in other circumstances. The DFS procedures provide for:

- Measurement of channels to detect licensed users directly by a STA or by a STA on behalf of an AP (9.12.1).

- Reporting of channel measurements by a STA as the result of a request by an AP or autonomously (9.12.2).

- Operating in a channel by STAs that take responsibility for detecting a licensed user and other STAs (9.12.3)

- Selection of a new channel and advertising of a channel switch (9.12.4)

A STA shall not scan for, start or join an IBSS when dot11SpectrumManagementEnabled is set equal to true.

For the purposes of DFS, a STA with the boolean MIB attribute dot11SpectrumManagementEnabled set equal to true shall not operate in a BSS unless the Spectrum Management bit is set equal to 1 in the Capability Information field in Beacons and Probe Responses received from the BSS. Similarly, an AP with the boolean MIB attribute dot11SpectrumManagementEnabled set equal to true shall not associate or reassociate a STA unless the Spectrum Management bit is set equal to 1 in the Capability Information field in Association and Reassociation Requests received from the STA.

Editor's note: we need to be careful not to put the detailed DFS regulations in the standard - they might change in the future.

Editor’s note: this text assumes DFS is not allowed in an IBSS; if this assumption is false the group needs to adopt a method of creating a quiet measurement period in an IBSS; the Nokia proposal is one solution.

### 9.12.1 Measuring of channels

The main purpose of channel measurement is to detect licensed users. A STA may measure one or more channels itself or an AP may request other STAs in the same BSS to measure one or more channels on its behalf. Channel measurements shall be undertaken at least as often as required to satisfy regulatory requirements.

A STA may quiet any channel it measures or requests another STA to measure, using various non-standardized methods, to avoid confusion between energy from other STAs and energy from licensed users. Methods of quieting a channel may include an AP arranging for part of a CFP to contain no traffic or a STA sending CTS to itself.

When requesting other STAs to measure one or more channels, an AP shall use a Channel Measurement Request frame containing one or more Channel Measurement Request elements. The measurement request may be sent to a unicast, multicast or broadcast destination address.

An AP that requests another STA to perform a channel measurement shall not transmit MPDUs to that STA or retransmit the measurement request until a reasonable time has passed for any required channel switches to occur, the measurement to be undertaken and the results reported. In determining this period, a STA shall assume that any required channel switches take less than 2 ms per switch.

Editors note: Sending a CTS to itself is not currently an allowed sequence. It also has the disadvantage that it is not acknowledged.

Editors note: Another mechanism for quieting a channel is to use a Measurement Interval Control Frame (as suggested by Philips), although it too suffers from a lack of an ACK.

Editors note: How frequently should channel measurements occur? Alternatively, what is the maximum time allowed by the European regulations to detect a licensed user?

Editors note: Is 2 ms the right value? Should it really be a MIB variable?

### 9.12.2 Reporting channel measurements

A STA that receives a Channel Measurement Request frame from the AP in its BSS shall process the frame’s Channel Measurement Request elements sequentially, starting at the time specified in the Action Specific field. A STA shall carry out at least those channel measurements that are specified as mandatory.

The results of all requested and completed channel measurements shall be returned to the requesting AP in one or more Channel Measurement Report elements, in one or more Channel Measurement Report frames with an Action Specific field equal to 0.

When a STA declines to carry out channel measurements specified in optional Channel Measurement Request elements, it shall return the elements containing the refused measurement requests to the AP in one or more Channel Measurement Report frames with an Action Specific field equal to 2.

A STA may autonomously report channel measurements to the AP in its BSS using a Channel Measurement Report frame with an Action Specific field equal to 1 containing one or more Channel Measurement Report elements. The Channel Measurement Report elements shall contain sufficient information for the STA receiving the frame to understand the channel measurement results.

#### 9.12.3 Operating in a channel

A STA shall not actively scan for a BSS and an AP shall not transmit Beacons or Probe Responses in a channel unless it knows, by whatever means, the DFS related regulatory requirements for the channel and it takes responsibility for respecting the regulations.

A STA that does not take responsibility for respecting the DFS regulations for a channel shall not transmit any MPDUs in a channel unless it has received an MPDU from its AP within dot11MaxDfsQuietTime. This restriction means that a STA waking up after a period of sleep may have to wait until it hears a transmission from its AP before transmitting an MPDU.

A STA that takes responsibility for respecting the DFS regulations shall ensure all its transmissions in a channel have stopped within dot11MaxDfsContinueTime of detecting a licensed user in the channel.

Editors note: the idea is that STAs may not propagate a BSS using Beacons or Probe Responses unless it respects the DFS regulatory requirements by arranging for appropriate channel measurements and leaving the channel if a licensed user is found.

Editors note: the idea is that other STAs can only continue to transmit in a channel while STAs taking responsibility for respecting the DFS regulations continue to transmit. Any transmission from an AP infers that it is still OK to transmit.

Editors note: these mechanisms should ensure that no one is left transmitting in a channel dot11MaxDfsContinueTime + dot11MaxDfsQuietTime after a licensed user is detected. These parameters have not been discussed or agreed by the group.

Editors note: an underlying assumption is that we have at least a few TBTTs to leave a channel after discovering a licensed user. Is this true? If not then we have a fundamental problem.

#### 9.12.4 Selecting and advertising a new channel

A STA may decide to move to a new channel at any time. The algorithm used to decide to move to a new channel is not standardized.

A STA may use Supported Channel elements and other information to select the new channel. The new channel selection algorithm is also not standardized but shall satisfy regulatory uniform spreading rules.

An AP may attempt to move the STAs associated with its BSS to a new channel by advertising the imminent move using the Channel Switch Announcement element in Beacons and Probe Responses. The channel switch should be scheduled, if possible, so that all STAs in the BSS, including those in sleep mode, have the opportunity to receive at least one Channel Switch Announcement element before the switch.

Editors note: it is not always possible to notify a channel switch to others if the Beacons are lost or STAs are sleeping. Regulations may also limit the time a STA can transmit in a channel once a licensed user has been detected.

## 10.3 MLME SAP interface

Editors note: Need appropriate changes here – but probably not worth doing until the basic features in the draft are decided

### 17.2.2 TXVECTOR parameters

Editors note: Need appropriate changes here to allow specification of tx power for TPC and measurement requests for DFS

### 17.2.3 RXVECTOR parameters

Editors note: Need appropriate changes here to allow specification of measurement reports for DFS. This could be a little tricky given that we do not know what many of the measurements are likely to be.

# Annex A – PICS

Editors note: probably not worth completing until the basic details of the draft agreed

### A.4.3 IUT configuration

| Item | IUT configuration | References | Status | Support |
| --- | --- | --- | --- | --- |
| *CF9 | Is Spectrum Management operation supported? | ? | O.4 | Yes, No |

### A.4.11 TPC extensions

| Item | Protocol Capability | References | Status | Support |
| --- | --- | --- | --- | --- |
| TP1 |  |  |  |  |
| TP2 |  |  |  |  |
| TP3 |  |  |  |  |
| … |  |  |  |  |
| TPn |  |  |  |  |

### A.4.12 DFS extensions

| Item | Protocol Capability | References | Status | Support |
| --- | --- | --- | --- | --- |
| DF1 |  |  |  |  |
| DF2 |  |  |  |  |
| DF3 |  |  |  |  |
| … |  |  |  |  |
| DFn |  |  |  |  |

# Annex C – Formal description of MAC operation

Editors note: the formal description in the 802.11 MAC is filled with syntactic and semantic errors. 802.11h are not proposing to add anything to this very flawed description unless it is verified first.

# Annex D – ASN.1 encoding of the MAC and PHY MIB

Editors note: initial attempt at ASN.1 coding based on 802.11d coding

In “Major Sections” of Annex D, add the following text to the end of the Station Management attributes

-- dot11SpectrumManagementCapabilityTable ::= {dot11smt 8}

In “SMT Station Config Table of Annex D, add the following text to the end of the dot11StationConfigEntry sequence list

dot11SpectrumManagementCapabilityImplemented TruthValue,

dot11SpectrumManagementCapabilityEnabled TruthValue

Add the following elements to the end of the dot11StationConfigEntry element definitions behind dot11AuthenticateFailStation

dot11SpectrumManagementCapabilityImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

“This attribute, when TRUE, indicates that the station implementation is capable of supporting spectrum management. The capability is disabled otherwise. The default value of this attribute is FALSE.”

::= { dot11StationConfigEntry 24 }

dot11SpectrumManagementCapabilityEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

“A STA will use the defined TPC and DFS procedures if and only if this attribute is TRUE. The default value of this attribute is FALSE.”

::= { dot11StationConfigEntry 25 }

-- ********************************************************************

-- * dot11SpectrumManagementCapability TABLE

-- ********************************************************************

dot11SpectrumManagementCapabilityTable OBJECT-TYPE

SYNTAX SEQUENCE OF dot11SpectrumManagementCapabilityEntry

MAX-ACCESS not-accessible

STATUS current

DESCIPTION

“This (conceptual) table of attributes for spectrum management”

::= {dot11smt 8}

dot11SpectrumManagementCapabilityEntry OBJECT-TYPE

SYNTAX Dot11SpectrumManagementCapabilityEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

“An entry (conceptual row) in the Spectrum Management Capability Table.

IfIndex – Each 802.11 interface is represented by an ifEntry. Interface tables in this MIB are indexed by ifIndex.”

INDEX {ifIndex, dot11SpectrumManagementCapabilityIndex}

::= {dot11SpectrumManagementCapabilityTable 1}

dot11SpectrumManagementCapabilityEntry ::=

SEQUENCE {

dot11SpectrumManagementCapabilityIndex Integer32,

dot11FirstChannelNumber Integer32,

dot11NumberofChannels Integer32,

dot11MitigationRequirement Integer32,

dot11MaxDfsContinueTime Integer32,

dot11MaxDfsQuietTime Integer32}

dot11SpectrumManagementCapabilityIndex OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"The auxiliary variable used to identify instances of the columnar objects in the Spectrum Management Capability Table."

::= { dot11SpectrumManagementCapabilityEntry 1}

dot11FirstChannelNumber OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall indicate the value of the lowest channel number in the sub-band for the associated domain country string. The default value of this attribute shall be zero."

::= { dot11SpectrumManagementCapabilityEntry 2}

dot11NumberofChannels OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall indicate the value of the total number of channels allowed in the sub-band for the associated domain country string. The default value of this attribute shall be zero."

::= { dot11SpectrumManagementCapabilityEntry 3}

dot11MitigationRequirement OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall indicate the average mitigation requirement in dB required in the sub-band. The default value of this attribute shall be 3dB."

::= { dot11SpectrumManagementCapabilityEntry 4}

dot11MaxDfsContinueTime OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

“This attribute shall indicate the maximum time (in us) that a STA that takes responsibility for detecting a licensed user may continue operating in a channel after detecting a licensed user”

::= { dot11SpectrumManagementCapabilityEntry 5}

dot11MaxDfsQuietTime OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

“This attribute shall indicate the maximum time (in us) that a STA that does not take responsibility for detecting a licensed user may continue operating in a channel after last receiving an MPDU from its AP”

::= { dot11SpectrumManagementCapabilityEntry 6}

-- ******************************************************************

-- * End of dot11SpectrumManagementCapability TABLE

-- ******************************************************************

# Annex E – Bibliography

Editors note: none so far

# Annex F – Measurement Requests and Reports in DFS

Clause 7.3.2.14 defines a Channel Measurement Request element by which a STA can request another STA to make a channel measurement. Clause 7.3.2.15 defines a Channel Measurement Report element by which a STA can report the results of a channel measurement, either as the result of a measurement request or autonomously.

A Measurement ID identifies the format of a measurement request within a Channel Measurement Request element and the format of a measurement report within a Channel Measurement Report element. The IEEE shall allocate the Measurement ID code space. This annex defines the semantics of the currently defined Measurement ID’s shown in Table 6.

Editors note: suggestions for other definitions invited

Table 6 – Measurement ID definitions

| Name | Measurement ID |
| --- | --- |
| Basic Request | 0 |
| Basic Report | 1 |
| RSSRI Histogram Request | 2 |
| RSSRI Histogram Report | 3 |
| Extended CCA Request | 4 |
| Extended CCA Report | 5 |
| Unknown RSSRI Histogram Request | 6 |
| Unknown RSSRI Histogram Report | 7 |
| BSS Request | 8 |
| BSS Report | 9 |
| STA Request | 10 |
| STA Report | 11 |

## F.1 Basic Request

A Measurement ID equal to 0 in the Channel Measurement Request element indicates a Basic Request. The Measurement Request field corresponding to a Basic Request is null. The response to a Basic Request is a Basic Report. It is mandatory for a STA to respond to this request.

## F.2 Basic Report

A Measurement ID equal to 1 in the Channel Measurement Report element indicates a Basic Report. The format of the Measurement Report field corresponding to a Basic Report is shown in Figure 14 and contains a measurement summary, with a focus on the detection of licensed users.

| Bits | 0 | 1 | 2 | 3 | 4 | 5-15 |
| --- | --- | --- | --- | --- | --- | --- |
| Name | BSS | Foreign PLCP Header | Possible Licensed User | Licensed User | Clean Channel | Reserved (0) |

Figure 14 - Measurement Report field format for a Basic Report

The Measurement Report field is coded as a bit field and shall contain a:

- BSS field, which indicates (when it is set equal to 1) at least one valid MPDU was decoded in the channel during the measurement period.

- Foreign PLCP Header field, which indicates (when it is set equal to 1) at least one PLCP preamble was detected in the channel during the measurement period without a subsequent valid Signal field period. This may indicate the presence of a foreign PLCP header such as Hiperlan2.

- Possible Licensed User field, which indicates (when it is set equal to 1) transmissions were detected in the channel during the measurement period that cannot be characterized as either from a licensed user or an unlicensed user.

- Licensed User field, which indicates (when it is set equal to 1) a licensed user was detected operating in the channel during the measurement period.

- Clean Channel field, which indicates (when it is set equal to 1) there were no users (licensed or unlicensed) operating in the channel during the measurement period. The algorithm used to draw this conclusion is not standardized.

Editors note: Need to define PHY/MAC interface signaling to enable Foreign PLCP Header bit

Editors note: There is contention as to the exact bits that should be included; some want exactly the same as in D1.0, while another suggestion is to reduce the signaling to one bit, with a name like “interference detected”. What do people think?

## F.3 RSSRI Histogram Request

A Measurement ID equal to 0 in the Channel Measurement Request element indicates an RSSRI Histogram Request. The Measurement Request field corresponding to an RSSRI Histogram Request is null. A response to an RSSRI Histogram Request is an RSSRI Histogram Report. It is optional for a STA to carry out a RSSRI Histogram Request.

## F.4 RSSRI Histogram Report

A Measurement ID equal to 1 in the Channel Measurement Report element indicates an RSSRI Histogram Report. The format of the Measurement Report field corresponding to an RSSRI Histogram Report is shown in Figure 15.

|  | RSSRI 0 density | RSSRI 1 density | RSSRI 2 density | RSSRI 3 density | RSSRI 4 density | RSSRI 5 density | RSSRI 6 density | RSSRI 7 density |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |

Figure 15 - Measurement Report field format for an RSSRI Histogram Report

The RSSRI Histogram Report shall contain the RSSRI densities in the channel over the measurement period for the eight RSSRIs defined in Table 7. The RSSRI density is defined as Ceiling (255 * [Period receiving at RSSRI / Measurement Period]).

Table 7 - RSSRI definitions for an RSSRI Histogram Report

| RSSRI | Power Observed at the Antenna (dBm) | Tolerance (dBm) |
| --- | --- | --- |
| 0 | Power  -87 | +5 |
| 1 | -87 < Power  -82 | ±5 |
| 2 | 82 < Power  -77 | ±5 |
| 3 | -77 < Power  -72 | ±5 |
| 4 | -72 < Power  -67 | ±5 |
| 5 | -67 < Power  -62 | ±5 |
| 6 | -62 < Power  -57 | ±5 |
| 7 | -57 < Power | -5 |

## F.5 Extended CCA Request

A Measurement ID equal to 4 in the Channel Measurement Report element indicates an Extended CCA Request. The Measurement Request field corresponding to an Extended CCA Request is null. A response to an Extended CCA Request is an Extended CCA Report. It is optional for a STA to carry out an Extended CCA Request.

## F.6 Extended CCA Report

A Measurement ID equal to 5 in the Channel Measurement Report element indicates an Extended CCA Report. The format of the Measurement Report field corresponding to an Extended CCA Report is shown in Figure 16.

|  | CCA Busy Fraction | CCA Busy Duration | CCA Busy Interval |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 |

Figure 16 - Measurement Report field format for an Extended CCA Report

CCA Busy Fraction field shall contain the fractional period over which CCA indicated the channel was busy during the measurement period. It is defined as Ceiling (255 * [Period CCA indicated channel was busy] / [Measurement duration]).

CCA is determined to be periodic in a channel during the measurement period, if CCA indicates at least two consecutive cycles where the period (in slots) of the cycle and the period (in slots) when CCA is asserted in the cycle do not change (plus or minus one slot).

CCA Busy Duration field shall contain the number of slots for which CCA indicated the channel was busy during the last cycle in which CCA was determined to be periodic during the measurement period. If the channel was not determined to be periodic then CCA Busy Duration shall be set equal to zero.

CCA Busy Interval field shall contain the number of slots during the last cycle for which CCA was determined to be periodic in the channel during the measurement period. If the channel was not determined to be periodic then CCA Busy Interval shall be set equal to zero.

Editors note: Is a slot the 9us 802.11a slot?

Editors note: If CCA indicates the channel is busy at any time during a slot does this mean the slot is considered to be busy for the purposes of CCA Busy Duration?

Editors note: CCA Busy Fraction is defined to have infinite resolution, whereas CCA Busy Duration and CCA Busy Internal have resolution of a slot. Is this consistent?

Editors note: What are we trying to detect using CCA Busy Duration, CCA Busy Internal and CCA Busy Fraction?

## F.7 Unknown RSSRI Histogram Request

A Measurement ID equal to 7 in the Channel Measurement Report element indicates an Unknown RSSRI Histogram Request. The Measurement Request field corresponding to an RSSRI Histogram Request is null. A response to an Unknown RSSRI Histogram Request is an Unknown RSSRI Histogram Report. It is optional for a STA carry out an Unknown RSSRI Histogram Request.

## F.8 Unknown RSSRI Histogram Report

A Measurement ID equal to 8 in the Channel Measurement Report element indicates an Unknown RSSRI Histogram Report. The format of the Measurement Report field corresponding to an Unknown RSSRI Histogram Report is shown in Figure 17.

|  | RSSRI 0 density | RSSRI 1 density | RSSRI 2 density | RSSRI 3 density | RSSRI 4 density | RSSRI 5 density | RSSRI 6 density | RSSRI 7 density |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |

Figure 17 - Measurement Report field format for an Unknown RSSRI Histogram Report

The Unknown RSSRI Histogram Report shall contain the RSSRI densities during the reception of unknown PPDUs (i.e. those with a valid preamble, a valid PLCP header and an invalid CRC) in the channel over the measurement period for eight RSSRIs, as defined in Table 8. The RSSRI density is defined as Ceiling (255 * [Period receiving at RSSRI during reception of unknown MPDUs / Period receiving unknown MPDUs]).

Table 8 –RSSRI definitions for an Unknown RSSRI Histogram Report

| RSSRI | Power Observed at the Antenna (dBm) | Tolerance (dBm) |
| --- | --- | --- |
| 0 | Power  -87 | +5 |
| 1 | -87 < Power  -82 | ±5 |
| 2 | 82 < Power  -77 | ±5 |
| 3 | -77 < Power  -72 | ±5 |
| 4 | -72 < Power  -67 | ±5 |
| 5 | -67 < Power  -62 | ±5 |
| 6 | -62 < Power  -57 | ±5 |
| 7 | -57 < Power | -5 |

## F.9 BSS Request

A Measurement ID equal to 8 in the Channel Measurement Request element indicates a BSS Request. The Measurement Request field corresponding to a BSS Request is null. A response to a BSS Request is one or more BSS Reports in one or more Channel Measurement Report frames. It is optional for a STA to carry out a BSS request.

## F.10 BSS Report

A Measurement ID equal to 9 in the Channel Measurement Report element indicates a BSS Report. The format of the Measurement Report field corresponding to a BSS Report when a BSS is detected is shown in Figure 18Figure 16. If no BSS was detected then the Measurement Report field is null.

|  | BSSID | Beacon Interval | Capability Information | Supported Rates |
| --- | --- | --- | --- | --- |
| Octets: | 6 | 2 | 2 | 3-10 |
|  | CF Parameter Set | Only included if detected Beacon or Probe Response contains a CF Parameter Set |  |  |
| Octets: | 6 |  |  |  |
|  | IBSS Parameter Set | Only included if detected Beacon or Probe Response contains an IBSS Parameter Set |  |  |
| Octets: | 4 |  |  |  |

Figure 18 - Measurement Report field format for a BSS Report

BSSID field shall contain the BSSID of the BSS found in a Beacon or Probe Response received from the BSS in the channel during the measurement period.

Beacon Interval field shall contain the Beacon Interval from the last Beacon or Probe Response received from the BSS in the channel during the measurement period.

Capability Information field shall contain the Capability Information from the last Beacon or Probe Response received from the BSS in the channel during the measurement period.

Supported Rates field shall contain the Supported Rates from the last Beacon or Probe Response received from the BSS or IBSS in the channel during the measurement period.

CF Parameter Set field shall contain the CF Parameter Set from the last Beacon or Probe Response received from the BSS in the channel during the measurement period. If the last Beacon or Probe Response does not contain a CF Parameter Set then this field shall be null.

IBSS Parameter Set field shall contain the IBSS Parameter Set from the last Beacon or Probe Response received from the IBSS during the measurement period. If the last Beacon or Probe Response does not contain a IBSS Parameter Set then this field shall be null.

## F.11 STA Request

A Measurement ID equal to 9 in the Channel Measurement Request element indicates a STA Request. The Measurement Request field corresponding to a BSS Request is shown in Figure 19 and shall contain the maximum number of STAs that shall be reported. It is optional for a STA to carry out a STA Request.

|  | Number of STAs |
| --- | --- |
| Octets: | 1 |

Figure 19 - Measurement Report field format for a STA Request

Number of STAs field shall contain the maximum (non-zero) number of STAs that may be reported in response to this STA Request.

## F.12 STA Report

A Measurement ID equal to 10 in the Channel Measurement Report element indicates a STA Report. The format of the Measurement Report field corresponding to a STA Report is shown in Figure 20 and shall contain information about zero or more STAs that were detected operating in the specified channel during the measurement period.

|  | BSSID | STA Address | RSSRI Histogram | Zero or more tuples |
| --- | --- | --- | --- | --- |
| Octets: | 6 | 6 | 8 |  |

Figure 20 - Measurement Report field format for a STA Report

BSSID field shall contain the BSSID of the BSS in which the detected STA was operating during the measurement period.

STA Address field shall contain the 48-bit MAC address of the STA detected during the measurement period.

RSSRI Histogram field shall contain the rounded up period, measured in TUs, that a STA was detected operating at each of the eight RSSRI levels shown in Table 9 during the measurement period. The format of the RSSI Histogram field is shown in Figure 21.

|  | RSSRI 0 time | RSSRI 1 time | RSSRI 2 time | RSSRI 3 time | RSSRI 4 time | RSSRI 5 time | RSSRI 6 time | RSSRI 7 time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |

Figure 21 – RSSRI Histogram format for a STA Report

Table 9 - RSSRI definitions for a STA Report

| RSSRI | Power Observed at the Antenna (dBm) | Tolerance (dBm) |
| --- | --- | --- |
| 0 | Power  -87 | +5 |
| 1 | -87 < Power  -82 | ±5 |
| 2 | 82 < Power  -77 | ±5 |
| 3 | -77 < Power  -72 | ±5 |
| 4 | -72 < Power  -67 | ±5 |
| 5 | -67 < Power  -62 | ±5 |
| 6 | -62 < Power  -57 | ±5 |
| 7 | -57 < Power | -5 |

# Annex G – Using TPC and DFS facilities for other purposes

Editors note: it has been suggested that an informative annex be added which describes how TPC and DFS could be used by other PHYs or outside Europe.

ERC documents are available from European Radiocommunications Office, Midtermolen 1, DK-2100, Copenhagen, Denmark (http://www.ero.dk).