IEEE P802.11
Wireless LANs

AES Encryption & Authentication Using CTR Mode & CBC-MAC

Date:	January 15, 2002

Authors:		Doug Whiting
	Hifn
	5973 Avenida Encinas, #110
	Carlsbad, CA  92009
	Phone: +1 760-827-4502
	E-mail: DWhiting@hifn.com

Russ Housley
	RSA Laboratories
	918 Spring Knoll Drive
	Herndon, Virginia  20170
	Phone: +1 703-435-1775
	E-mail: RHousley@rsasecurity.com

and

Niels Ferguson 
	MacFergus BV
	Bart de Ligtstraat 64 
	1097 JE Amsterdam, Netherlands
	Phone: +31 20 643 0977
	E-mail: Niels@ferguson.net

Abstract

Use of the AES is desired as part of the 802.11i, the Specification for Enhanced Security. A combination of counter mode encryption and CBC-MAC authentication is proposed here. These modes have been used and studied for a long time, with well-understood cryptographic properties, and no known patent encumbrances. They provide for good security and performance, whether implemented in hardware or software. Sample source code and test vectors are provided.

# Motivation

IEEE Std 802.11-1999 is the wireless LAN standard. The new AES has many properties are desirable for securing traffic on wireless networks. However, any encryption algorithm can be used in many different ways, each of which will have different performance and security characteristics. Important criteria for comparison of cryptographic algorithms and protocols include:

| - Patent Status | - Cleartext Integrity Coverage |
| --- | --- |
| - Size of Implementation | - Simplicity of Key Management |
| - Power Consumption | - Packet Overhead |
| - Speed | - Crypto Confidence |

These criteria are discussed in a separate presentation (IEEE 802.11-01/634r1). In that presentation, the use of OCB mode is compared to this proposal, called CTR+CBC-MAC. The purpose of this document is to completely specify CTR+CBC-MAC, allowing the transforms to be evaluated and implemented with confidence.

# Definitions

The following variables are used in the description of this cryptographic mode, as discussed below.

| Name | Description | Field Size | Comments |
| --- | --- | --- | --- |
| C | Block number “counter” within a packet | 16 bits |  |
| H | Number of cleartext header octets covered by MIC | 8 bits | Up to 255 header octets allowed |
| L | Length of the input packet | 16 bits | Packet size up to 64K octets |
| M | Size (in octets) of MIC “tag” | 5 bits | Recommended value: M = 8 |
| N | Unique packet number | 32 bits | Similar to the WEP IV |
| P | Input packet | L octets | Octets p[0], p[1], … , p[L-1] |
| A | Authentication (MIC) value, computed over P | M octets | Octets a[0], a[1], … , a[M-1] |
| Q | Output packet | L+M octets | Octets q[0], q[1], … , q[L+M–1] |
| RA | Receiver address | 48 bits | Octets RA[0], … , RA[5] |
| TA | Transmitter address | 48 bits | Octets TA[0], … , TA[5] |
| K | AES Encryption Key | 128 bits | Octets k[0], k[1], … , k[15] |

All key material for a given connection will be derived from mechanisms outside the scope of this document. In general, each pair of stations in the network that desire to communicate will have a unique pairwise key. A new key will also need to be negotiated periodically. This proposal uses AES with a 128-bit key and 128-bit block size.

Figure 1. Encryption Process

Before encryption, a packet P has a length of L octets. Of these octets, the first H  L octets (i.e., the “header”) are to be covered by authentication but not encrypted. In other words, all the octets in the input packet are covered by CBC-MAC authentication, while only the input data octets p[H], p[H+1] … p[L–1] are encrypted. If any octets in the physical packet are not to be covered by either authentication or encryption, for purposes of this discussion, they are not included in the packet P.  Any mutable fields (e.g., the packet “re-transmit” bit) must not be covered by the CBC-MAC or the encryption, in order to avoid having re-encryption.

The length of the MIC authentication value, A, is M octets, and it is generally fixed for each connection. We propose that M=8 octets (64 bits) be used for 802.11i.

To encode a packet, the MIC value is computed over all the octets of the input packet P, and the M octets of MIC value are appended to P. Then the L–H+M octets p[H], p[H+1], … , p[L–1], a[0], a[1], … , a[M–1] are encrypted to produce the output packet Q. In other words, q[j] = p[j] for j=0 to H–1.

The notation <a,b,c,d> = W, where W is a 32-bit integer value, means that the octets a,b,c,d are assigned to the octets of the integer W, in big-endian order. In other words, using C syntax:

a = (W >> 24) & 0xFF;

b = (W >> 16) & 0xFF;

c = (W >>  8) & 0xFF;

d =  W	  & 0xFF;

# CBC-MAC Computation (MIC)

For purposes of computing the MIC value using CBC-MAC, the packet data is logically padded with zeroes, if needed, up to a multiple of the AES block size (i.e., 16 octets). In other words, the values p[k] are assumed to be zero for all k  L. The first AES block of the MAC computation is used to generate the CBC IV. It does not cover any of the packet data, but instead includes the values shown below in Table 1.

| Field | Description | Size |
| --- | --- | --- |
| RA | “Low” bits of receiver address (octets RA[4] and RA[5]) | 16 bits |
| TA | Transmitter address (octets TA[0] to TA[5]) | 48 bits |
| N | Unique packet number | 32 bits |
| H | Number of cleartext header octets covered by MIC | 8 bits |
| 000 | Indicates CBC-MAC, not encryption | 3 bits |
| M | Size (in octets) of MIC “tag” | 5 bits |
| L | Length (in octets) of the input packet | 16 bits |

Table 1.  Bit Fields of CBC-MAC IV Input Block

These values are used in network order (big-endian) in the AES block. In other words, if we define the plaintext of this first encryption as the octets t[j] for j=0 to 15, this plaintext block is generated as follows, using a C-like pseudo-code syntax:

<t[ 0],t[ 1],t[ 2],t[ 3]> = <RA[4],RA[5],TA[0],TA[1]>;

<t[ 4],t[ 5],t[ 6],t[ 7]> = <TA[2],TA[3],TA[4],TA[5]>;

<t[ 8],t[ 9],t[10],t[11]> = N;      /* packet sequence number */

<t[12],t[13],t[14],t[15]> = (H << 24) + (M << 16) + L;

This plaintext block is then encrypted, using AES with key K, to produce the CBC IV for the remainder of the (padded) packet data:

<t[0],t[1], … ,t[15] > = AES_Encrypt(K,<t[0],t[1], … ,t[15]>);

Now, CBC-MAC proceeds over the plaintext data as follows:

for (i=0;i<L;i+=16)

{

for (j=0;j<16;j++) t[j]^= p[i+j];	/* zero pad p[] if necessary */

<t[0],t[1], … ,t[15]>   = AES_Encrypt(K,<t[0],t[1], … ,t[15]>);

}

Using the final AES encryption result, the MAC value, A, is then defined by a[j] = t[j], for j=0 to M–1. These M octets of MIC are then appended to the packet data by p[L+j] = a[j], for j=0 to M–1.

# Counter Mode Encryption

The first step in counter mode encryption consists of setting the counter plaintext “template” similar to the first step of the CBC-MAC, as follows:

| Field | Description | Size |
| --- | --- | --- |
| RA | “Low” bits of receiver address (octets RA[4] to RA[5]) | 16 bits |
| TA | Transmitter address (octets TA[0] to TA[5]) | 48 bits |
| N | Unique packet number | 32 bits |
| H | Number of cleartext header octets covered by MIC | 8 bits |
| 100 | Indicates encryption, not CBC-MAC | 3 bits |
| M | Size (in octets) of MIC “tag” | 5 bits |
| C | Block number “counter” within the packet | 16 bits |

Table 2.  Bit Fields of Counter Mode Encryption Counter

In other words,

<t[ 0],t[ 1],t[ 2],t[ 3]> = <RA[4],RA[5],TA[0],TA[1]>;

<t[ 4],t[ 5],t[ 6],t[ 7]> = <TA[2],TA[3],TA[4],TA[5]>;

<t[ 8],t[ 9],t[10],t[11]> = N;      /* packet sequence number */

<t[12],t[13],t[14],t[15]> = (H << 24) + ((M+0x80) << 16);

Then the packet data, including the MIC, is encrypted, using the following steps:

for (i=0;i<L+M-H;i+=16)

{

C = i/16;       /* block counter C */

t[14] = C/256;

t[15] = C%256;

<u[0],u[1], … ,u[15]>   = AES_Encrypt(K,<t[0],t[1], … ,t[15]>);

for (j= 0;j<16;j++)

if (i+j < L+M-H)

p[H+i+j] ^= u[j];

}

# Implementation Notes and Options

Note that the upper 64 bits of the AES input blocks do not change during the lifetime of the connection. The purpose of these “salt” bits is to minimize the impact of certain classes of attacks (e.g., Hellman80, Biham96). For 802.11i, we have chosen them as addresses. Another possible approach is to choose these salt bits at random when the connection is established, possibly by hashing connection attributes, such as the receiver/transmitter addresses. These salt bits do not have to remain secret.

# Security Considerations

Jakob Jonsson from RSA Laboratories is working on a security proof of CTR+CBC-MAC.  While the proof is not finished yet, initial results indicate that CTR+CBC-MAC has roughly the same security bound as OCB mode.

# Appendix A. Test Vectors

Borland C compiler [Jan 15 2002 14:21:05].

Random seed = 1011133266

AES KAT Vectors:

Key:       00 00 00 00  00 00 00 00  00 00 00 00  00 00 00 00

PT:        80 00 00 00  00 00 00 00  00 00 00 00  00 00 00 00

CT:        3A D7 8E 72  6C 1E C0 2B  7E BF E9 2B  23 D9 EC 34

Key:       80 00 00 00  00 00 00 00  00 00 00 00  00 00 00 00

PT:        00 00 00 00  00 00 00 00  00 00 00 00  00 00 00 00

CT:        0E DD 33 D3  C6 21 E5 46  45 5B D8 BA  14 18 BE C8

NIST AES Vectors: OK

=============== Packet Vector #1 ==================

AES Key:   00 01 02 03  04 05 06 07  08 09 0A 0B  0C 0D 0E 0F

TA = 00 01 02 03  04 05      seqNum = 00000000

RA = 40 41 42 43  44 45

Packet length =   31. [Input (8 cleartext header octets)]

00 01 02 03  04 05 06 07  08 09 0A 0B  0C 0D 0E 0F

10 11 12 13  14 15 16 17  18 19 1A 1B  1C 1D 1E

CBC IV in: 44 45 00 01  02 03 04 05  00 00 00 00  08 08 00 1F

CBC IV out:92 53 8F B6  05 8A 6B 64  70 33 6D 29  78 45 29 B9

CBC[0000]: 31 44 F3 5A  7D 6F 98 9E  38 01 ED C7  BB F3 9D 35

CBC[0010]: 97 5C 94 C1  88 5B 0B 3B  36 9A 21 09  DE D0 76 2C

Packet length =   39. [MIC appended (8 octets)]

00 01 02 03  04 05 06 07  08 09 0A 0B  0C 0D 0E 0F

10 11 12 13  14 15 16 17  18 19 1A 1B  1C 1D 1E 97

5C 94 C1 88  5B 0B 3B

CTR Start: 44 45 00 01  02 03 04 05  00 00 00 00  08 88 00 00

CTR[0001]: 38 5F 10 73  E4 B7 C5 13  0C B6 93 EB  60 B6 12 AB

CTR[0002]: EE FF 8C 88  01 DC DE E1  4D 83 A1 00  30 46 2C 02

Packet length =   39. [Encrypted]

00 01 02 03  04 05 06 07  30 56 1A 78  E8 BA CB 1C

1C A7 81 F8  74 A3 04 BC  F6 E6 96 93  1D C1 C0 76

11 17 60 88  6B 4D 17

=============== Packet Vector #2 ==================

AES Key:   00 01 02 03  04 05 06 07  08 09 0A 0B  0C 0D 0E 0F

TA = 00 01 02 03  04 05      seqNum = 00000001

RA = 40 41 42 43  44 45

Packet length =   32. [Input (8 cleartext header octets)]

00 01 02 03  04 05 06 07  08 09 0A 0B  0C 0D 0E 0F

10 11 12 13  14 15 16 17  18 19 1A 1B  1C 1D 1E 1F

CBC IV in: 44 45 00 01  02 03 04 05  00 00 00 01  08 08 00 20

CBC IV out:B8 0E 48 EB  93 39 61 8B  A7 D9 38 62  31 7D E8 67

CBC[0000]: E9 C0 09 91  34 E1 71 5F  3B BC A5 38  73 E4 77 0B

CBC[0010]: 17 1D D1 9F  60 2D 86 7B  B9 B0 A4 36  D1 FD 6D E0

Packet length =   40. [MIC appended (8 octets)]

00 01 02 03  04 05 06 07  08 09 0A 0B  0C 0D 0E 0F

10 11 12 13  14 15 16 17  18 19 1A 1B  1C 1D 1E 1F

17 1D D1 9F  60 2D 86 7B

CTR Start: 44 45 00 01  02 03 04 05  00 00 00 01  08 88 00 00

CTR[0001]: F3 30 03 80  D3 20 A6 3B  2D C0 00 26  7C 8E 47 5C

CTR[0002]: F3 D4 6F C3  BB 42 8C FA  C6 08 88 F7  6C A9 55 F6

Packet length =   40. [Encrypted]

00 01 02 03  04 05 06 07  FB 39 09 8B  DF 2D A8 34

3D D1 12 35  68 9B 51 4B  EB CD 75 D8  A7 5F 92 E5

D1 15 59 68  0C 84 D3 8D

=============== Packet Vector #3 ==================

AES Key:   00 01 02 03  04 05 06 07  08 09 0A 0B  0C 0D 0E 0F

TA = 00 01 02 03  04 05      seqNum = 00000002

RA = 40 41 42 43  44 45

Packet length =   33. [Input (8 cleartext header octets)]

00 01 02 03  04 05 06 07  08 09 0A 0B  0C 0D 0E 0F

10 11 12 13  14 15 16 17  18 19 1A 1B  1C 1D 1E 1F

20

CBC IV in: 44 45 00 01  02 03 04 05  00 00 00 02  08 08 00 21

CBC IV out:D8 E1 CA 18  B1 42 7D D2  AA 8D CF 4F  ED DE 91 FE

CBC[0000]: A8 9B 62 44  92 B3 0A 20  40 CF 44 47  68 28 AB DF

CBC[0010]: 35 88 5F D9  67 F4 E2 BC  0D 7E DC 1C  1E B7 22 1A

CBC[0020]: 58 97 6F 9E  BD DB C5 6E  E6 57 E3 4F  3C 9F D0 F5

Packet length =   41. [MIC appended (8 octets)]

00 01 02 03  04 05 06 07  08 09 0A 0B  0C 0D 0E 0F

10 11 12 13  14 15 16 17  18 19 1A 1B  1C 1D 1E 1F

20 58 97 6F  9E BD DB C5  6E

CTR Start: 44 45 00 01  02 03 04 05  00 00 00 02  08 88 00 00

CTR[0001]: 66 D3 2A BB  26 21 AD 48  31 AD 32 45  7C 7E CD C0

CTR[0002]: 2C 0C B1 5E  90 DB 51 66  8E CF 4D D2  8A 22 66 8F

CTR[0003]: 44 2D 2D 59  8F CE 04 99  BF B1 70 CA  76 BE A1 8A

Packet length =   41. [Encrypted]

00 01 02 03  04 05 06 07  6E DA 20 B0  2A 2C A3 47

21 BC 20 56  68 6B DB D7  34 15 AB 45  8C C6 4F 79

AE 97 DA BD  14 9F BD 4A  2A

=============== Packet Vector #4 ==================

AES Key:   00 01 02 03  04 05 06 07  08 09 0A 0B  0C 0D 0E 0F

TA = 00 01 02 03  04 05      seqNum = 00000003

RA = 40 41 42 43  44 45

Packet length =   31. [Input (12 cleartext header octets)]

00 01 02 03  04 05 06 07  08 09 0A 0B  0C 0D 0E 0F

10 11 12 13  14 15 16 17  18 19 1A 1B  1C 1D 1E

CBC IV in: 44 45 00 01  02 03 04 05  00 00 00 03  0C 08 00 1F

CBC IV out:B6 27 27 8B  F7 B8 61 88  A9 73 85 81  DC DB E1 CB

CBC[0000]: D4 C8 65 D9  0A A7 CF 80  2D 02 92 44  DC C0 8E 5A

CBC[0010]: CB 8F 38 A7  EC D7 85 56  79 1F 4D 96  69 6C F5 80

Packet length =   39. [MIC appended (8 octets)]

00 01 02 03  04 05 06 07  08 09 0A 0B  0C 0D 0E 0F

10 11 12 13  14 15 16 17  18 19 1A 1B  1C 1D 1E CB

8F 38 A7 EC  D7 85 56

CTR Start: 44 45 00 01  02 03 04 05  00 00 00 03  0C 88 00 00

CTR[0001]: 38 4D E7 0C  AF 1A 93 A5  BD F8 6F 57  C1 C4 8A A0

CTR[0002]: AB 24 96 99  25 F4 3E 68  75 AC 5B AF  9C AE 0C F2

Packet length =   39. [Encrypted]

00 01 02 03  04 05 06 07  08 09 0A 0B  34 40 E9 03

BF 0B 81 B6  A9 ED 79 40  D9 DD 90 BB  B7 39 88 52

AA CC 99 84  A2 29 0D

=============== Packet Vector #5 ==================

AES Key:   00 01 02 03  04 05 06 07  08 09 0A 0B  0C 0D 0E 0F

TA = 00 01 02 03  04 05      seqNum = 00000004

RA = 40 41 42 43  44 45

Packet length =   32. [Input (12 cleartext header octets)]

00 01 02 03  04 05 06 07  08 09 0A 0B  0C 0D 0E 0F

10 11 12 13  14 15 16 17  18 19 1A 1B  1C 1D 1E 1F

CBC IV in: 44 45 00 01  02 03 04 05  00 00 00 04  0C 08 00 20

CBC IV out:07 F6 59 95  05 3D EA DA  6D E3 35 16  59 41 C0 37

CBC[0000]: 32 39 EB BD  DB D8 9D A2  9B 26 AD 0E  28 1C D9 2F

CBC[0010]: 51 86 16 AB  91 49 F4 87  88 76 4F 82  A4 CE DB 16

Packet length =   40. [MIC appended (8 octets)]

00 01 02 03  04 05 06 07  08 09 0A 0B  0C 0D 0E 0F

10 11 12 13  14 15 16 17  18 19 1A 1B  1C 1D 1E 1F

51 86 16 AB  91 49 F4 87

CTR Start: 44 45 00 01  02 03 04 05  00 00 00 04  0C 88 00 00

CTR[0001]: 0F 5C 40 9F  7A 29 AF 74  0F 67 F7 EE  F0 EB 3A 9C

CTR[0002]: 13 F6 DB 3F  9A AE 53 58  9B 4A 27 05  70 A2 01 C4

Packet length =   40. [Encrypted]

00 01 02 03  04 05 06 07  08 09 0A 0B  03 51 4E 90

6A 38 BD 67  1B 72 E1 F9  E8 F2 20 87  0F EB C5 20

CB 28 45 F3  0A 03 D3 82

=============== Packet Vector #6 ==================

AES Key:   00 01 02 03  04 05 06 07  08 09 0A 0B  0C 0D 0E 0F

TA = 00 01 02 03  04 05      seqNum = 00000005

RA = 40 41 42 43  44 45

Packet length =   33. [Input (12 cleartext header octets)]

00 01 02 03  04 05 06 07  08 09 0A 0B  0C 0D 0E 0F

10 11 12 13  14 15 16 17  18 19 1A 1B  1C 1D 1E 1F

20

CBC IV in: 44 45 00 01  02 03 04 05  00 00 00 05  0C 08 00 21

CBC IV out:88 E5 92 F7  CD BA C6 FF  71 1B 8A 16  F0 B8 80 52

CBC[0000]: D7 D4 66 C8  A7 2B 25 0E  19 25 88 39  0F 29 6B 60

CBC[0010]: 62 F1 0B A4  FB 16 D6 BD  37 2C BF B7  7A 41 47 5E

CBC[0020]: 22 DD 11 82  F3 F3 71 6A  B3 B3 D0 57  CE A3 AB 94

Packet length =   41. [MIC appended (8 octets)]

00 01 02 03  04 05 06 07  08 09 0A 0B  0C 0D 0E 0F

10 11 12 13  14 15 16 17  18 19 1A 1B  1C 1D 1E 1F

20 22 DD 11  82 F3 F3 71  6A

CTR Start: 44 45 00 01  02 03 04 05  00 00 00 05  0C 88 00 00

CTR[0001]: A0 FC 14 C2  CF 2A 49 19  B7 34 9D BB  67 D9 E0 5D

CTR[0002]: 9C 04 BC FB  B6 65 D8 C5  34 57 23 2D  B3 93 D9 DA

Packet length =   41. [Encrypted]

00 01 02 03  04 05 06 07  08 09 0A 0B  AC F1 1A CD

DF 3B 5B 0A  A3 21 8B AC  7F C0 FA 46  80 19 A2 E4

96 47 05 D4  B6 A4 D0 5C  D9

=============== Packet Vector #7 ==================

AES Key:   00 01 02 03  04 05 06 07  08 09 0A 0B  0C 0D 0E 0F

TA = 00 01 02 03  04 05      seqNum = 00000006

RA = 40 41 42 43  44 45

Packet length =   31. [Input (8 cleartext header octets)]

00 01 02 03  04 05 06 07  08 09 0A 0B  0C 0D 0E 0F

10 11 12 13  14 15 16 17  18 19 1A 1B  1C 1D 1E

CBC IV in: 44 45 00 01  02 03 04 05  00 00 00 06  08 0A 00 1F

CBC IV out:6A 4D BD 4A  C7 AB C8 15  1E 52 1F 05  89 85 15 0D

CBC[0000]: 9B DC 54 AC  81 BC FE F2  35 71 97 A1  25 DF BF 6E

CBC[0010]: 2B 1F 2C D1  81 BF BC F3  82 92 00 3A  36 A3 F3 35

Packet length =   41. [MIC appended (10 octets)]

00 01 02 03  04 05 06 07  08 09 0A 0B  0C 0D 0E 0F

10 11 12 13  14 15 16 17  18 19 1A 1B  1C 1D 1E 2B

1F 2C D1 81  BF BC F3 82  92

CTR Start: 44 45 00 01  02 03 04 05  00 00 00 06  08 8A 00 00

CTR[0001]: 19 28 23 4E  C2 4C F4 AD  BA 20 97 0D  5E 49 AC 12

CTR[0002]: AC 2D 02 A6  18 93 BD 0B  FC F6 56 95  12 46 4B 91

CTR[0003]: FE 86 CB C9  55 C6 C2 9F  CB 4F F5 A2  BD B1 B5 85

Packet length =   41. [Encrypted]

00 01 02 03  04 05 06 07  11 21 29 45  CE 41 FA A2

AA 31 85 1E  4A 5C BA 05  B4 34 18 BD  04 8E A3 20

E3 DA 87 14  AD FA B8 13  6C

=============== Packet Vector #8 ==================

AES Key:   00 01 02 03  04 05 06 07  08 09 0A 0B  0C 0D 0E 0F

TA = 00 01 02 03  04 05      seqNum = 00000007

RA = 40 41 42 43  44 45

Packet length =   32. [Input (8 cleartext header octets)]

00 01 02 03  04 05 06 07  08 09 0A 0B  0C 0D 0E 0F

10 11 12 13  14 15 16 17  18 19 1A 1B  1C 1D 1E 1F

CBC IV in: 44 45 00 01  02 03 04 05  00 00 00 07  08 0A 00 20

CBC IV out:BA 3A 15 3B  D5 C4 BC A6  B5 D5 80 1F  1E F2 21 6E

CBC[0000]: FE CC A4 4E  DB 0E E7 E6  F1 5E 4C 96  21 A1 40 92

CBC[0010]: 7C F6 38 9B  D8 15 CF 70  BC 3F 95 02  68 76 51 40

Packet length =   42. [MIC appended (10 octets)]

00 01 02 03  04 05 06 07  08 09 0A 0B  0C 0D 0E 0F

10 11 12 13  14 15 16 17  18 19 1A 1B  1C 1D 1E 1F

7C F6 38 9B  D8 15 CF 70  BC 3F

CTR Start: 44 45 00 01  02 03 04 05  00 00 00 07  08 8A 00 00

CTR[0001]: 29 F2 B0 65  0B 6C CB 4D  DB 72 F8 E4  DD 25 AF AC

CTR[0002]: 17 90 34 43  EC EB C1 36  CC F2 3E EE  C9 9D 45 7B

CTR[0003]: B8 CB 13 A6  25 6C 6F 67  4B D8 40 01  3F 6E 06 24

Packet length =   42. [Encrypted]

00 01 02 03  04 05 06 07  21 FB BA 6E  07 61 C5 42

CB 63 EA F7  C9 30 B9 BB  0F 89 2E 58  F0 F6 DF 29

B0 04 06 75  11 88 8A 0B  04 F4

=============== Packet Vector #9 ==================

AES Key:   00 01 02 03  04 05 06 07  08 09 0A 0B  0C 0D 0E 0F

TA = 00 01 02 03  04 05      seqNum = 00000008

RA = 40 41 42 43  44 45

Packet length =   33. [Input (8 cleartext header octets)]

00 01 02 03  04 05 06 07  08 09 0A 0B  0C 0D 0E 0F

10 11 12 13  14 15 16 17  18 19 1A 1B  1C 1D 1E 1F

20

CBC IV in: 44 45 00 01  02 03 04 05  00 00 00 08  08 0A 00 21

CBC IV out:72 86 4A EC  B1 7C DA 26  F9 86 9A FB  37 AD D7 8A

CBC[0000]: 51 20 24 77  8B E3 18 C1  BE 77 BA 21  0A 1B 89 F8

CBC[0010]: 68 2D A0 5A  0D D5 02 43  6B 73 C7 1C  38 BA A9 20

CBC[0020]: B6 C5 45 DB  A2 20 8C D7  32 7D 61 D0  11 02 ED 6B

Packet length =   43. [MIC appended (10 octets)]

00 01 02 03  04 05 06 07  08 09 0A 0B  0C 0D 0E 0F

10 11 12 13  14 15 16 17  18 19 1A 1B  1C 1D 1E 1F

20 B6 C5 45  DB A2 20 8C  D7 32 7D

CTR Start: 44 45 00 01  02 03 04 05  00 00 00 08  08 8A 00 00

CTR[0001]: 08 A8 1A FD  2E 91 80 F5  79 D1 DF 85  B7 FC 1F C9

CTR[0002]: 71 73 06 F5  97 36 28 BB  89 A3 BC 97  B0 83 DE DE

CTR[0003]: 95 04 6C 31  CD FB F1 AC  94 90 A2 C4  AF 42 DE 93

Packet length =   43. [Encrypted]

00 01 02 03  04 05 06 07  00 A1 10 F6  22 9C 8E FA

69 C0 CD 96  A3 E9 09 DE  69 6A 1C EE  8B 2B 36 A4

A9 15 79 D2  6B 21 FE 52  42 36 11

=============== Packet Vector #10 ==================

AES Key:   00 01 02 03  04 05 06 07  08 09 0A 0B  0C 0D 0E 0F

TA = 00 01 02 03  04 05      seqNum = 00000009

RA = 40 41 42 43  44 45

Packet length =   31. [Input (12 cleartext header octets)]

00 01 02 03  04 05 06 07  08 09 0A 0B  0C 0D 0E 0F

10 11 12 13  14 15 16 17  18 19 1A 1B  1C 1D 1E

CBC IV in: 44 45 00 01  02 03 04 05  00 00 00 09  0C 0A 00 1F

CBC IV out:3D F8 F0 B1  3D 9B 26 E1  62 98 04 72  1C 89 44 8A

CBC[0000]: E0 AB 2E C6  77 73 AE EB  2F 3D 56 B4  24 E5 B5 0E

CBC[0010]: 66 7A 8B EE  43 3F 86 53  37 6B AE 00  23 2C 7E ED

Packet length =   41. [MIC appended (10 octets)]

00 01 02 03  04 05 06 07  08 09 0A 0B  0C 0D 0E 0F

10 11 12 13  14 15 16 17  18 19 1A 1B  1C 1D 1E 66

7A 8B EE 43  3F 86 53 37  6B

CTR Start: 44 45 00 01  02 03 04 05  00 00 00 09  0C 8A 00 00

CTR[0001]: DF 69 65 EF  26 C0 BF 52  5F 60 E0 BC  05 46 74 12

CTR[0002]: A4 07 B7 F2  0B CE D1 54  8E 4A 1E A0  60 2E B0 7F

Packet length =   41. [Encrypted]

00 01 02 03  04 05 06 07  08 09 0A 0B  D3 64 6B E0

36 D1 AD 41  4B 75 F6 AB  1D 5F 6E 09  B8 1A A9 94

71 45 3F 17  B1 CC 4D 97  0B

=============== Packet Vector #11 ==================

AES Key:   00 01 02 03  04 05 06 07  08 09 0A 0B  0C 0D 0E 0F

TA = 00 01 02 03  04 05      seqNum = 0000000A

RA = 40 41 42 43  44 45

Packet length =   32. [Input (12 cleartext header octets)]

00 01 02 03  04 05 06 07  08 09 0A 0B  0C 0D 0E 0F

10 11 12 13  14 15 16 17  18 19 1A 1B  1C 1D 1E 1F

CBC IV in: 44 45 00 01  02 03 04 05  00 00 00 0A  0C 0A 00 20

CBC IV out:A7 6C 6B E2  D5 FB 5F 8D  0C 25 06 07  52 00 5E 5A

CBC[0000]: 18 6F 0F 6B  E2 DC A7 F3  45 7D FF 27  9A 1B A8 3D

CBC[0010]: 4D 62 28 50  7D 61 E0 E4  1F 9F FE 07  53 DB D1 57

Packet length =   42. [MIC appended (10 octets)]

00 01 02 03  04 05 06 07  08 09 0A 0B  0C 0D 0E 0F

10 11 12 13  14 15 16 17  18 19 1A 1B  1C 1D 1E 1F

4D 62 28 50  7D 61 E0 E4  1F 9F

CTR Start: 44 45 00 01  02 03 04 05  00 00 00 0A  0C 8A 00 00

CTR[0001]: 18 CC 6D 52  47 25 C6 B2  80 89 26 CE  44 1D 52 A2

CTR[0002]: 02 F6 F1 F2  A5 F4 7F 99  F8 D6 A1 9F  21 29 DE 19

Packet length =   42. [Encrypted]

00 01 02 03  04 05 06 07  08 09 0A 0B  14 C1 63 5D

57 34 D4 A1  94 9C 30 D9  5C 04 48 B9  1E EB EF ED

E8 96 57 C9  85 B7 41 7B  3E B6

=============== Packet Vector #12 ==================

AES Key:   00 01 02 03  04 05 06 07  08 09 0A 0B  0C 0D 0E 0F

TA = 00 01 02 03  04 05      seqNum = 0000000B

RA = 40 41 42 43  44 45

Packet length =   33. [Input (12 cleartext header octets)]

00 01 02 03  04 05 06 07  08 09 0A 0B  0C 0D 0E 0F

10 11 12 13  14 15 16 17  18 19 1A 1B  1C 1D 1E 1F

20

CBC IV in: 44 45 00 01  02 03 04 05  00 00 00 0B  0C 0A 00 21

CBC IV out:09 3F 87 48  63 03 5C 18  63 4F BE A7  99 C7 AA E0

CBC[0000]: A9 DB B4 1B  9E 18 26 F9  38 0E 02 F0  0C D9 CD 34

CBC[0010]: 16 D5 D4 AD  8C 6E A8 EF  54 39 20 23  0A C2 64 95

CBC[0020]: 15 1D 21 97  FC 66 DA CD  2E 75 2A B3  3E E5 8D 82

Packet length =   43. [MIC appended (10 octets)]

00 01 02 03  04 05 06 07  08 09 0A 0B  0C 0D 0E 0F

10 11 12 13  14 15 16 17  18 19 1A 1B  1C 1D 1E 1F

20 15 1D 21  97 FC 66 DA  CD 2E 75

CTR Start: 44 45 00 01  02 03 04 05  00 00 00 0B  0C 8A 00 00

CTR[0001]: 0B A6 14 2A  D4 EC 93 6D  74 64 CD F9  96 29 0E 2A

CTR[0002]: A3 71 49 C0  6C C3 A9 C6  38 ED E4 AE  9D 57 D1 7A

Packet length =   43. [Encrypted]

00 01 02 03  04 05 06 07  08 09 0A 0B  07 AB 1A 25

C4 FD 81 7E  60 71 DB EE  8E 30 14 31  BF 6C 57 DF

4C D6 B4 E7  AF 11 82 74  50 79 A4

=============== Packet Vector #13 ==================

AES Key:   98 07 44 7D  42 1F BD B0  97 FE 6C E2  87 DE 1E 7C

TA = 51 98 33 6D  97 4A      seqNum = B8105D9F

RA = 17 6C 4D 48  7A 4D

Packet length =   31. [Input (8 cleartext header octets)]

FC C5 33 32  18 18 84 CA  48 DD 6D 54  90 17 3E 1C

26 D1 97 D2  56 43 B9 60  28 72 AC A0  12 B0 4C

CBC IV in: 7A 4D 51 98  33 6D 97 4A  B8 10 5D 9F  08 08 00 1F

CBC IV out:E3 78 6F 0D  1D D1 3A 0F  2F 79 C3 8E  C8 3A 8A F7

CBC[0000]: 20 B4 C0 79  98 DC 27 1B  80 D4 6B DC  17 DF 4F 89

CBC[0010]: DD DB F7 60  3A 42 B3 EF  0F F5 41 B2  B9 BB 7C 82

Packet length =   39. [MIC appended (8 octets)]

FC C5 33 32  18 18 84 CA  48 DD 6D 54  90 17 3E 1C

26 D1 97 D2  56 43 B9 60  28 72 AC A0  12 B0 4C DD

DB F7 60 3A  42 B3 EF

CTR Start: 7A 4D 51 98  33 6D 97 4A  B8 10 5D 9F  08 88 00 00

CTR[0001]: 90 04 73 9B  36 71 F0 45  23 4F FE 20  3E 08 B3 40

CTR[0002]: 54 F6 BF DB  E9 A4 56 D6  8C 01 75 74  0A CE 55 EC

Packet length =   39. [Encrypted]

FC C5 33 32  18 18 84 CA  D8 D9 1E CF  A6 66 CE 59

05 9E 69 F2  68 4B 0A 20  7C 84 13 7B  FB 14 1A 0B

57 F6 15 4E  48 7D BA

=============== Packet Vector #14 ==================

AES Key:   98 07 44 7D  42 1F BD B0  97 FE 6C E2  87 DE 1E 7C

TA = 51 98 33 6D  97 4A      seqNum = 30FF8C53

RA = 17 6C 4D 48  7A 4D

Packet length =   32. [Input (8 cleartext header octets)]

94 85 6A 4C  DE A6 14 ED  72 7F 4D 33  63 FA 64 98

DE A7 38 52  A7 22 F5 D3  22 54 E7 5D  68 97 68 86

CBC IV in: 7A 4D 51 98  33 6D 97 4A  30 FF 8C 53  08 08 00 20

CBC IV out:01 48 0E 95  BD 38 58 86  D7 5E 5F 05  CC 2F AB 94

CBC[0000]: 68 12 6B D0  A4 8D 06 01  55 D0 39 14  CC 30 B3 06

CBC[0010]: 72 A8 65 DF  35 91 1D 70  4B 74 82 A9  7A A9 B0 3C

Packet length =   40. [MIC appended (8 octets)]

94 85 6A 4C  DE A6 14 ED  72 7F 4D 33  63 FA 64 98

DE A7 38 52  A7 22 F5 D3  22 54 E7 5D  68 97 68 86

72 A8 65 DF  35 91 1D 70

CTR Start: 7A 4D 51 98  33 6D 97 4A  30 FF 8C 53  08 88 00 00

CTR[0001]: 08 88 9D E8  29 3A DB D1  72 F3 C3 2A  5E 18 F2 2E

CTR[0002]: CE 45 A8 D3  82 D4 9C 53  66 B7 8A 43  DD 1A 84 EE

Packet length =   40. [Encrypted]

94 85 6A 4C  DE A6 14 ED  7A F7 D0 DB  4A C0 BF 49

AC 54 FB 78  F9 3A 07 FD  EC 11 4F 8E  EA 43 F4 D5

14 1F EF 9C  E8 8B 99 9E

=============== Packet Vector #15 ==================

AES Key:   98 07 44 7D  42 1F BD B0  97 FE 6C E2  87 DE 1E 7C

TA = 51 98 33 6D  97 4A      seqNum = C3EAAF67

RA = 17 6C 4D 48  7A 4D

Packet length =   33. [Input (8 cleartext header octets)]

B0 80 4C 3C  EF DC D0 49  52 C8 7E 31  BE CE 12 5F

CD 6A 44 62  BC 65 40 71  B0 24 97 DA  B8 87 41 E1

27

CBC IV in: 7A 4D 51 98  33 6D 97 4A  C3 EA AF 67  08 08 00 21

CBC IV out:C5 D0 C1 48  66 EA D0 E9  24 51 14 F8  4C A9 AA 33

CBC[0000]: D1 C8 22 9B  23 1D B4 77  83 7F 01 62  35 8A B1 4B

CBC[0010]: F7 5F 29 F0  93 E1 58 09  CC 22 FB B0  B1 9F 42 72

CBC[0020]: 88 AB BE B2  A8 1E B2 F8  E4 9F 3F 91  CE 97 30 C2

Packet length =   41. [MIC appended (8 octets)]

B0 80 4C 3C  EF DC D0 49  52 C8 7E 31  BE CE 12 5F

CD 6A 44 62  BC 65 40 71  B0 24 97 DA  B8 87 41 E1

27 88 AB BE  B2 A8 1E B2  F8

CTR Start: 7A 4D 51 98  33 6D 97 4A  C3 EA AF 67  08 88 00 00

CTR[0001]: 53 33 53 D6  B5 E3 D6 88  B5 CA D3 28  F6 73 77 5C

CTR[0002]: 12 58 F6 5D  44 5A 1A 25  4F 2A F7 A4  F1 E4 A9 20

CTR[0003]: 89 B2 3E 96  D8 49 EE 8F  AC 17 5E F5  BB 66 2A F8

Packet length =   41. [Encrypted]

B0 80 4C 3C  EF DC D0 49  01 FB 2D E7  0B 2D C4 D7

78 A0 97 4A  4A 16 37 2D  A2 7C 61 87  FC DD 5B C4

68 A2 5C 1A  43 4C B7 92  71

=============== Packet Vector #16 ==================

AES Key:   98 07 44 7D  42 1F BD B0  97 FE 6C E2  87 DE 1E 7C

TA = 51 98 33 6D  97 4A      seqNum = 2DF690A8

RA = 17 6C 4D 48  7A 4D

Packet length =   31. [Input (12 cleartext header octets)]

95 8F 9B 6B  69 0B 86 3A  9E 6D 66 D4  59 D7 E9 D1

0E E3 A1 2E  9B F6 A6 7A  16 96 D3 1E  DA 5A DB

CBC IV in: 7A 4D 51 98  33 6D 97 4A  2D F6 90 A8  0C 08 00 1F

CBC IV out:F2 14 0D 47  1A BD 45 5D  EF DC 2D 46  1D 92 43 B6

CBC[0000]: EB 2C 23 FD  77 C7 50 9D  86 02 69 C7  78 C8 C4 5E

CBC[0010]: 64 8D A3 4D  25 D0 80 74  D8 6C 8A A6  B2 D9 27 FC

Packet length =   39. [MIC appended (8 octets)]

95 8F 9B 6B  69 0B 86 3A  9E 6D 66 D4  59 D7 E9 D1

0E E3 A1 2E  9B F6 A6 7A  16 96 D3 1E  DA 5A DB 64

8D A3 4D 25  D0 80 74

CTR Start: 7A 4D 51 98  33 6D 97 4A  2D F6 90 A8  0C 88 00 00

CTR[0001]: 61 D3 AD 80  41 6B B5 D5  34 CD 51 BF  02 51 ED 0C

CTR[0002]: 7D D8 25 A9  D2 C6 13 5A  88 52 9B 4C  E0 BF A9 D9

Packet length =   39. [Encrypted]

95 8F 9B 6B  69 0B 86 3A  9E 6D 66 D4  38 04 44 51

4F 88 14 FB  AF 3B F7 C5  14 C7 3E 12  A7 82 FE CD

5F 65 5E 7F  58 D2 EF

=============== Packet Vector #17 ==================

AES Key:   98 07 44 7D  42 1F BD B0  97 FE 6C E2  87 DE 1E 7C

TA = 51 98 33 6D  97 4A      seqNum = 686613C7

RA = 17 6C 4D 48  7A 4D

Packet length =   32. [Input (12 cleartext header octets)]

EE 35 B1 CE  8B 99 34 8E  84 4A 1E 5D  E0 32 AD 31

9A AB 07 57  5F 60 69 B8  B0 F4 42 A6  4C D6 A3 76

CBC IV in: 7A 4D 51 98  33 6D 97 4A  68 66 13 C7  0C 08 00 20

CBC IV out:C5 5A F3 F6  8F C6 C5 2A  C5 F0 33 9B  F3 68 BB 3A

CBC[0000]: D8 F8 B9 A4  38 B5 65 FF  BA 44 81 16  6A 52 24 2C

CBC[0010]: BA 9A A6 B9  41 9F E2 94  7B 1C CE C3  3A BD 10 47

Packet length =   40. [MIC appended (8 octets)]

EE 35 B1 CE  8B 99 34 8E  84 4A 1E 5D  E0 32 AD 31

9A AB 07 57  5F 60 69 B8  B0 F4 42 A6  4C D6 A3 76

BA 9A A6 B9  41 9F E2 94

CTR Start: 7A 4D 51 98  33 6D 97 4A  68 66 13 C7  0C 88 00 00

CTR[0001]: 2B FF 08 02  D6 A5 11 4A  D8 91 43 8F  63 6D DF 4A

CTR[0002]: D2 FF F9 79  A8 D5 65 5A  28 94 26 EB  9B FA 30 D4

Packet length =   40. [Encrypted]

EE 35 B1 CE  8B 99 34 8E  84 4A 1E 5D  CB CD A5 33

4C 0E 16 1D  87 F1 2A 37  D3 99 9D EC  9E 29 5A 0F

12 4F C3 E3  69 0B C4 7F

=============== Packet Vector #18 ==================

AES Key:   98 07 44 7D  42 1F BD B0  97 FE 6C E2  87 DE 1E 7C

TA = 51 98 33 6D  97 4A      seqNum = 75912FEC

RA = 17 6C 4D 48  7A 4D

Packet length =   33. [Input (12 cleartext header octets)]

1B E5 C8 F8  D7 4C D9 CA  F5 21 35 E5  5C 0D 04 58

93 7C 1A 98  BA F5 13 8C  BB F8 E5 6D  A9 6A 4D 94

C6

CBC IV in: 7A 4D 51 98  33 6D 97 4A  75 91 2F EC  0C 08 00 21

CBC IV out:C9 87 FD 36  BA 3B 59 9D  CE AD 4A 43  F7 A1 E6 30

CBC[0000]: CC 64 13 9D  5C 1E 6D 4D  70 80 FA EA  20 3C 34 A4

CBC[0010]: C1 76 E1 5A  7C FF 41 03  F8 0D 41 10  0A E8 40 BA

CBC[0020]: 9F 05 72 AF  53 BF 6F 32  EB CD 9E BF  2C 83 D8 EA

Packet length =   41. [MIC appended (8 octets)]

1B E5 C8 F8  D7 4C D9 CA  F5 21 35 E5  5C 0D 04 58

93 7C 1A 98  BA F5 13 8C  BB F8 E5 6D  A9 6A 4D 94

C6 9F 05 72  AF 53 BF 6F  32

CTR Start: 7A 4D 51 98  33 6D 97 4A  75 91 2F EC  0C 88 00 00

CTR[0001]: EE 54 AF 28  33 25 28 42  19 E1 E9 DA  3A 2A 45 51

CTR[0002]: 9B F1 DA 7C  38 A2 87 0A  6D 47 C0 FF  98 61 45 0F

Packet length =   41. [Encrypted]

1B E5 C8 F8  D7 4C D9 CA  F5 21 35 E5  B2 59 AB 70

A0 59 32 DA  A3 14 FA 56  81 D2 A0 3C  32 9B 97 E8

FE 3D 82 78  C2 14 7F 90  AA

=============== Packet Vector #19 ==================

AES Key:   98 07 44 7D  42 1F BD B0  97 FE 6C E2  87 DE 1E 7C

TA = 51 98 33 6D  97 4A      seqNum = 111900AF

RA = 17 6C 4D 48  7A 4D

Packet length =   31. [Input (8 cleartext header octets)]

F8 E4 4B 74  59 2C 12 2F  58 79 07 B9  C4 78 F0 24

D7 50 6C B4  23 66 4C 58  14 C1 D8 79  8D CD 41

CBC IV in: 7A 4D 51 98  33 6D 97 4A  11 19 00 AF  08 0A 00 1F

CBC IV out:11 2E 18 D6  8E 74 16 92  32 F1 39 CA  7B 5B B7 1E

CBC[0000]: 30 20 56 E4  5C D9 AC CE  A8 E0 1D 3E  F9 09 4C 70

CBC[0010]: 34 84 69 E5  9F 41 FC A4  90 2C 5B 09  11 2A 88 FF

Packet length =   41. [MIC appended (10 octets)]

F8 E4 4B 74  59 2C 12 2F  58 79 07 B9  C4 78 F0 24

D7 50 6C B4  23 66 4C 58  14 C1 D8 79  8D CD 41 34

84 69 E5 9F  41 FC A4 90  2C

CTR Start: 7A 4D 51 98  33 6D 97 4A  11 19 00 AF  08 8A 00 00

CTR[0001]: 74 0E AE DB  4E E9 45 01  0B 17 EF 39  2D 38 3F A6

CTR[0002]: E1 4F 3A F4  2A 28 44 4F  53 B7 AA 22  05 0C 44 AF

CTR[0003]: 0C EA BF 2D  2F CF 21 72  E2 09 5B 53  F8 11 D2 F5

Packet length =   41. [Encrypted]

F8 E4 4B 74  59 2C 12 2F  2C 77 A9 62  8A 91 B5 25

DC 47 83 8D  0E 5E 73 FE  F5 8E E2 8D  A7 E5 05 7B

D7 DE 4F BD  44 F0 E0 3F  20

=============== Packet Vector #20 ==================

AES Key:   98 07 44 7D  42 1F BD B0  97 FE 6C E2  87 DE 1E 7C

TA = 51 98 33 6D  97 4A      seqNum = 75FBD7E1

RA = 17 6C 4D 48  7A 4D

Packet length =   32. [Input (8 cleartext header octets)]

23 E9 14 CE  1A 8D 18 5D  6A 81 8E 4B  97 62 D4 BD

25 61 16 0A  29 13 E7 BA  1F 91 AD 0D  51 C3 7B D6

CBC IV in: 7A 4D 51 98  33 6D 97 4A  75 FB D7 E1  08 0A 00 20

CBC IV out:B0 72 3B BB  26 10 03 D7  E6 75 10 95  78 1E E0 AC

CBC[0000]: C3 E9 50 46  29 E9 48 B3  1B 9B 00 E2  40 2C 2D 2F

CBC[0010]: DE FB 46 CA  8A 93 70 C6  0C E1 E6 B0  15 9C 75 AE

Packet length =   42. [MIC appended (10 octets)]

23 E9 14 CE  1A 8D 18 5D  6A 81 8E 4B  97 62 D4 BD

25 61 16 0A  29 13 E7 BA  1F 91 AD 0D  51 C3 7B D6

DE FB 46 CA  8A 93 70 C6  0C E1

CTR Start: 7A 4D 51 98  33 6D 97 4A  75 FB D7 E1  08 8A 00 00

CTR[0001]: 14 DC D9 BF  A9 76 73 2E  81 4B DC 9C  41 A4 6E 54

CTR[0002]: 3A 40 A2 60  EB EE 06 51  9A 57 D9 D5  DB DC 5E E9

CTR[0003]: 25 23 20 B7  61 34 34 BD  31 2F E2 2A  3B A8 94 2E

Packet length =   42. [Encrypted]

23 E9 14 CE  1A 8D 18 5D  7E 5D 57 F4  3E 14 A7 93

A4 2A CA 96  68 B7 89 EE  25 D1 0F 6D  BA 2D 7D 87

44 AC 9F 1F  51 4F 2E 2F  29 C2

=============== Packet Vector #21 ==================

AES Key:   98 07 44 7D  42 1F BD B0  97 FE 6C E2  87 DE 1E 7C

TA = 51 98 33 6D  97 4A      seqNum = 305A175C

RA = 17 6C 4D 48  7A 4D

Packet length =   33. [Input (8 cleartext header octets)]

A4 EB 63 B1  F6 A8 07 CC  30 A8 64 AB  2F 60 7C 2D

44 11 B6 AB  01 62 3D 1E  8B E4 D0 B0  8C 72 08 D5

A6

CBC IV in: 7A 4D 51 98  33 6D 97 4A  30 5A 17 5C  08 0A 00 21

CBC IV out:37 DE E3 9C  4B EF 1A 85  DB E9 7C C9  58 4F 8B 23

CBC[0000]: AD 28 08 B7  B1 B4 86 49  E2 64 89 42  9E E2 CA CC

CBC[0010]: 29 66 42 1F  AA 60 A5 AF  83 E2 EC 56  37 8A 6D 27

CBC[0020]: 72 8F 12 61  10 0F 0D B8  8F 72 72 1D  04 DC 8E 2E

Packet length =   43. [MIC appended (10 octets)]

A4 EB 63 B1  F6 A8 07 CC  30 A8 64 AB  2F 60 7C 2D

44 11 B6 AB  01 62 3D 1E  8B E4 D0 B0  8C 72 08 D5

A6 72 8F 12  61 10 0F 0D  B8 8F 72

CTR Start: 7A 4D 51 98  33 6D 97 4A  30 5A 17 5C  08 8A 00 00

CTR[0001]: 0E F6 9B 57  E1 72 41 88  BF E3 7B 36  54 2B 0E C1

CTR[0002]: 84 0B E6 04  99 09 57 0A  E1 62 DF 59  90 F6 50 35

CTR[0003]: 98 58 77 C2  1C 72 AA 28  E8 A6 F1 AC  8D E1 E5 01

Packet length =   43. [Encrypted]

A4 EB 63 B1  F6 A8 07 CC  3E 5E FF FC  CE 12 3D A5

FB F2 CD 9D  55 49 33 DF  0F EF 36 B4  15 7B 5F DF

47 10 50 4B  F1 E6 5F 38  20 D7 05

=============== Packet Vector #22 ==================

AES Key:   98 07 44 7D  42 1F BD B0  97 FE 6C E2  87 DE 1E 7C

TA = 51 98 33 6D  97 4A      seqNum = 673AF43C

RA = 17 6C 4D 48  7A 4D

Packet length =   31. [Input (12 cleartext header octets)]

6E 45 1C EB  FF FF D7 6E  1E FF A8 E6  6F AA 23 3A

08 2D 7B 33  8E 7B 3D 85  3D 40 D3 E3  90 30 38

CBC IV in: 7A 4D 51 98  33 6D 97 4A  67 3A F4 3C  0C 0A 00 1F

CBC IV out:94 30 BB D2  5E D7 F3 8C  B1 C1 2A 10  D6 64 F1 B9

CBC[0000]: DF 29 8D D5  EF E4 43 E7  6A 1C 25 7F  90 D6 49 3B

CBC[0010]: D9 59 B6 3F  C7 BF 84 D4  53 FC 41 1C  37 70 BE 16

Packet length =   41. [MIC appended (10 octets)]

6E 45 1C EB  FF FF D7 6E  1E FF A8 E6  6F AA 23 3A

08 2D 7B 33  8E 7B 3D 85  3D 40 D3 E3  90 30 38 D9

59 B6 3F C7  BF 84 D4 53  FC

CTR Start: 7A 4D 51 98  33 6D 97 4A  67 3A F4 3C  0C 8A 00 00

CTR[0001]: 95 C9 E1 02  1B 72 EB 6F  0A 71 02 2E  04 A0 01 62

CTR[0002]: 7C 99 24 51  08 2A 12 EB  1F F2 95 B5  87 FB C1 F8

Packet length =   41. [Encrypted]

6E 45 1C EB  FF FF D7 6E  1E FF A8 E6  FA 63 C2 38

13 5F 90 5C  84 0A 3F AB  39 E0 D2 81  EC A9 1C 88

51 9C 2D 2C  A0 76 41 E6  7B

=============== Packet Vector #23 ==================

AES Key:   98 07 44 7D  42 1F BD B0  97 FE 6C E2  87 DE 1E 7C

TA = 51 98 33 6D  97 4A      seqNum = D929DDA2

RA = 17 6C 4D 48  7A 4D

Packet length =   32. [Input (12 cleartext header octets)]

17 99 E4 7D  8B 8A A0 07  E4 17 23 C4  F4 8C 5E DC

C6 8D B9 F2  F0 7A 01 6A  49 5E 25 66  40 31 80 FD

CBC IV in: 7A 4D 51 98  33 6D 97 4A  D9 29 DD A2  0C 0A 00 20

CBC IV out:88 E5 C8 A8  9E 66 A7 0F  39 99 58 E2  3B 48 DF 46

CBC[0000]: 27 9D AC B7  56 F9 CE 40  18 A0 66 4A  50 65 E1 0E

CBC[0010]: 65 5B 1B 0F  EB 70 CD 97  02 1F 10 15  2C 77 47 8C

Packet length =   42. [MIC appended (10 octets)]

17 99 E4 7D  8B 8A A0 07  E4 17 23 C4  F4 8C 5E DC

C6 8D B9 F2  F0 7A 01 6A  49 5E 25 66  40 31 80 FD

65 5B 1B 0F  EB 70 CD 97  02 1F

CTR Start: 7A 4D 51 98  33 6D 97 4A  D9 29 DD A2  0C 8A 00 00

CTR[0001]: 48 B5 87 39  E7 BE CD A0  FC 7A FF EA  D3 3C 4C 16

CTR[0002]: DC BB 4C E3  11 6A DE 9A  70 36 11 91  39 F7 27 1F

Packet length =   42. [Encrypted]

17 99 E4 7D  8B 8A A0 07  E4 17 23 C4  BC 39 D9 E5

21 33 74 52  0C 00 FE 80  9A 62 69 70  9C 8A CC 1E

74 31 C5 95  9B 46 DC 06  3B E8

=============== Packet Vector #24 ==================

AES Key:   98 07 44 7D  42 1F BD B0  97 FE 6C E2  87 DE 1E 7C

TA = 51 98 33 6D  97 4A      seqNum = 52E1B7EB

RA = 17 6C 4D 48  7A 4D

Packet length =   33. [Input (12 cleartext header octets)]

FB B8 3D B7  DC E1 0B 13  D8 88 1B 60  60 04 6F 4A

9D E9 4C AB  B5 82 87 08  34 85 F6 F8  3D 0F 16 23

E6

CBC IV in: 7A 4D 51 98  33 6D 97 4A  52 E1 B7 EB  0C 0A 00 21

CBC IV out:9A 95 CF 11  2C 33 7B BB  1E 36 C0 2A  44 B4 78 88

CBC[0000]: D7 35 4B 0E  5E C8 7E BB  71 0A 7F 3B  AB AD 9F 43

CBC[0010]: B9 A4 FA CD  C0 50 D0 2A  65 8B BA 1B  32 4A 21 EC

CBC[0020]: F1 AA 34 E8  57 17 7A B2  F0 E4 E0 E9  FF B5 DF F7

Packet length =   43. [MIC appended (10 octets)]

FB B8 3D B7  DC E1 0B 13  D8 88 1B 60  60 04 6F 4A

9D E9 4C AB  B5 82 87 08  34 85 F6 F8  3D 0F 16 23

E6 F1 AA 34  E8 57 17 7A  B2 F0 E4

CTR Start: 7A 4D 51 98  33 6D 97 4A  52 E1 B7 EB  0C 8A 00 00

CTR[0001]: A8 33 A2 04  8F 3C F9 D8  35 B6 46 55  4F B3 DE 21

CTR[0002]: E2 B5 15 15  BB 88 07 5E  48 21 B8 E7  73 E5 DC B4

Packet length =   43. [Encrypted]

FB B8 3D B7  DC E1 0B 13  D8 88 1B 60  C8 37 CD 4E

12 D5 B5 73  80 34 C1 5D  7B 36 28 D9  DF BA 03 36

# 5D 79 AD 6A  A0 76 AF 9D  C1 15 38

# Appendix B. Test Vector Generation Source Code

//

//================================================================

// Proposed AES CTR/CBC-MAC mode test vector generation

//

// Author:  Doug Whiting, Hifn  (dwhiting@hifn.com)

//

// This code is released to the public domain, on an as-is basis.

//================================================================

//

#include <stdio.h>

#include <stdlib.h>

#include <string.h>

#include <time.h>

#include <assert.h>

#include "aes_defs.h"       // AES calling interface

#include "aes_vect.h"       // NIST AES test vectors

typedef int BOOL;           // boolean

enum

{

BLK_SIZE    =   16,     // # octets in an AES block

MAX_PACKET  =   3*512   // largest packet size

};

union block                 // AES cipher block

{

u32b  x[BLK_SIZE/4];    // access as 8-bit octets or 32-bit words

u08b  b[BLK_SIZE];

};

struct packet

{

BOOL    encrypted;      // TRUE if encrypted

u08b    TA[6],RA[6];    // xmit/rcv address

int     micLength;      // # octets of MIC appended to plaintext (M)

int     clrCount;       // # cleartext octets covered by MIC (H)

block   key;            // the encryption key (K)

u32b    seqNum;         // unique packet sequence number (e.g., WEP IV)

int     length;         // # octets in data[]

u08b    data[MAX_PACKET+2*BLK_SIZE];    // packet contents

};

struct

{

int     cnt;            // how many words left in ct

block   ptCntr;         // the counter input

block   ct;             // the ciphertext (prng output)

} prng;

// return the 32-bit value read to be stored as a big-endian word

u32b   BigEndian(u32b   x)

{

static block b={0,0,0,0};

if (b.x[0]==0)          // first time, figure out endianness

b.x[0]=0xFF000001;

if (b.b[0]==0xFF)

return x;

if (b.b[0]==1)

return (x >> 24) + (x << 24) +

((x >>  8) & 0x00FF00) + ((x <<  8) & 0xFF0000);

assert(0);              // should never happen

return 0;

}

void InitRand(u32b   seed)

{

memset(prng.ptCntr.b,0,BLK_SIZE);

prng.ptCntr.x[(BLK_SIZE/4)-1]=seed*17;

prng.cnt=0;             // the pump is dry

}

// prng: does not use C rand(), so this should be repeatable across platforms

u32b   Random32(void)

{

if (prng.cnt == 0)

{   // use whatever key is currently defined

prng.cnt=BLK_SIZE/4;

prng.ptCntr.x[0]++;

if (prng.ptCntr.x[0]==0)    // ripple carry?

prng.ptCntr.x[1]++;     // (stop at 64 bits)

AES_Encrypt(prng.ptCntr.x,prng.ct.x);

}

--prng.cnt;

return BigEndian(prng.ct.x[prng.cnt]);

}

// display a block

void ShowBlock(const block *blk,const char *prefix,const char *suffix,int a)

{

int i;

printf(prefix,a);

for (i=0;i<BLK_SIZE;i++)

printf("%02X%s",blk->b[i],((i&3)==3)?"  ":" ");

printf(suffix);

}

void ShowAddr(const packet *p)

{

int i;

printf("      TA = ");

for (i=0;i<6;i++) printf("%02X%s",p->TA[i],(i==3)?"  ":" ");

printf("     seqNum = %08X\n",p->seqNum);

printf("      RA = ");

for (i=0;i<6;i++) printf("%02X%s",p->RA[i],(i==3)?"  ":" ");

printf("\n");

}

// display a packet

void ShowPacket(const packet *p,const char *pComment,int a)

{

int i;

printf("Packet length = %4d. ",p->length);

printf(pComment,a);

if (p->encrypted) printf("[Encrypted]");

for (i=0;i<p->length;i++)

{

if ((i & 15) == 0) printf("\n%11s","");

printf("%02X%s",p->data[i],((i&3)==3)?"  ":" ");

}

printf("\n");

}

// make sure that encrypt/decrypt work according to NIST vectors

void Validate_NIST_AES_Vectors(int verbose)

{

int     i;

block   key,pt,ct,rt;

printf("AES KAT Vectors:\n");   // known-answer tests

// variable text (fixed-key) tests

memcpy(key.b,VT_key,BLK_SIZE);

AES_SetKey(key.x,BLK_SIZE*8);

for (i=0;i<sizeof(VT_pt_ct_pairs);i+=2*BLK_SIZE)

{

memcpy(pt.b,VT_pt_ct_pairs+i,BLK_SIZE);

AES_Encrypt(pt.x,ct.x);

if (memcmp(ct.x,VT_pt_ct_pairs+i+BLK_SIZE,BLK_SIZE))

{

printf("Vector miscompare at VT test #%d",i);

exit(1);

}

AES_Decrypt(ct.x,rt.x);    // sanity check on decrypt

if (memcmp(pt.b,rt.b,BLK_SIZE))

{

printf("Decrypt miscompare at VT test #%d",i);

exit(1);

}

if (verbose)   // only do a little if we're "debugging"

{  printf("\n");    break; }

else if (i==0)

{          //  display the first vector

ShowBlock(&key,"Key:       ","\n",0);

ShowBlock(&pt ,"PT:        ","\n",0);

ShowBlock(&ct ,"CT:        ","\n\n",0);

}

}

// variable key (fixed-text) tests

memcpy(pt.b,VK_pt,BLK_SIZE);

for (i=0;i<sizeof(VK_key_ct_pairs);i+=2*BLK_SIZE)

{

memcpy(key.b,VK_key_ct_pairs+i,BLK_SIZE);

AES_SetKey(key.x,BLK_SIZE*8);

AES_Encrypt(pt.x,ct.x);

if (memcmp(ct.x,VK_key_ct_pairs+i+BLK_SIZE,BLK_SIZE))

{

printf("Vector miscompare at VK test #%d",i);

exit(1);

}

AES_Decrypt(ct.x,rt.x);    // sanity check on decrypt

if (memcmp(pt.b,rt.b,BLK_SIZE))

{

printf("Decrypt miscompare at VK test #%d",i);

exit(1);

}

if (verbose)   // only do a little if we're "debugging"

{  printf("\n");    break; }

else if (i==0)

{          //  display the first vector

ShowBlock(&key,"Key:       ","\n",0);

ShowBlock(&pt ,"PT:        ","\n",0);

ShowBlock(&ct ,"CT:        ","\n\n",0);

}

}

printf("NIST AES Vectors: OK\n");   // if we got here, it's all cool

}

// assumes AES_SetKey is called elsewhere

void Generate_CTR_CBC_Vector(packet *p,int verbose)

{

int     i,j,C;

block   m,x;

assert(p->length    >= p->clrCount && p->length    <= MAX_PACKET);

assert(p->micLength >  0           && p->micLength <= BLK_SIZE);

ShowPacket(p,"[Input (%d cleartext header octets)]",p->clrCount);

// compute & append the CBC-MAC

m.b[0]  = p->RA[4];

m.b[1]  = p->RA[5];

m.b[2]  = p->TA[0];

m.b[3]  = p->TA[1];

m.b[4]  = p->TA[2];

m.b[5]  = p->TA[3];

m.b[6]  = p->TA[4];

m.b[7]  = p->TA[5];

m.x[2]  = BigEndian(p->seqNum);

m.x[3]  = BigEndian(p->length + (p->micLength << 16) + (p->clrCount<<24));

AES_Encrypt(m.x,x.x);               // produce the CBC IV

ShowBlock(&m,"CBC IV in: ","\n",0);

if (verbose) ShowBlock(&x,"CBC IV out:","\n",0);

memset(p->data+p->length,0,BLK_SIZE); // zero pad for purposes of CBC-MAC

for (i=0;i<p->length;i+=BLK_SIZE)

{

for (j=0;j<BLK_SIZE;j++)        // do the CBC xor

x.b[j] = x.b[j] ^ p->data[i+j];

AES_Encrypt(x.x,x.x);           // and encrypt the block in place

if (verbose) ShowBlock(&x,"CBC[%04X]: ","\n",i);

}

for (i=0;i<p->micLength;i++)        // truncate & append MAC to packet

p->data[p->length+i]=x.b[i];

p->length+=p->micLength;

if (verbose) ShowPacket(p,"[MIC appended (%d octets)]",p->micLength);

// now encrypt the packet+CBC-MAC, using CTR mode

m.b[13] |= 0x80;                   // set the encrypt bit

for (i=0;i+p->clrCount < p->length;i++)

{

if ((i % BLK_SIZE) == 0)

{                           // generate new keystream block

C = i/16;

m.b[14] = C/256;

m.b[15] = C%256;

AES_Encrypt(m.x,x.x);       // then encrypt the counter

if (verbose && i==0) ShowBlock(&m,"CTR Start: ","\n",0);

if (verbose) ShowBlock(&x,"CTR[%04X]: " ,"\n",1+(i/BLK_SIZE));

}

p->data[i+p->clrCount] ^= x.b[i % BLK_SIZE];    // merge in the keystream

}

p->encrypted = 1;

ShowPacket(p,"",0);                 // show the final encrypted packet

}

int main(int argc,char *argv[])

{

int     i,j,k,len,pktNum,seed;

packet  p;

seed = (argc > 1) ? atoi(argv[1]) : (int) time(NULL);

InitRand(seed);

printf("%s C compiler [%s %s].\nRandom seed = %d\n",

COMPILER_ID,__DATE__,__TIME__,seed);

// first, make sure that our AES code matches NIST KAT vectors

Validate_NIST_AES_Vectors(_VERBOSE_);

// generate CTR-CBC vectors for various parameter settings

for (k=pktNum=0;k<2;k++)

{   // k==1 --> random vectors. k==0 --> "visually simple" vectors

for (i=0;i<BLK_SIZE  ;i++)

p.key.b [i]=(k) ? (u08b) Random32() & 0xFF : i;

for (i=0;i<6;i++)

{

p.TA[i]=(k) ? (u08b) Random32() & 0xFF : i;

p.RA[i]=(k) ? (u08b) Random32() & 0xFF : i + 0x40;

}

AES_SetKey(p.key.x,BLK_SIZE*8);     // run the key schedule

// now generate the vectors

for (p.micLength  = 8;p.micLength  <12;p.micLength+=2)

for (p.clrCount   = 8;p.clrCount   <16;p.clrCount+=4)

for (len          =32;len          <64;len*=2)

for (i            =-1;i            < 2;i++)

{

p.seqNum = (k) ? Random32() : pktNum;

p.length = len+i;               // len+i is packet length

p.encrypted = 0;

assert(p.length <= MAX_PACKET);

for (j=0;j<p.length;j++)        // generate random packet contents

p.data[j]=(k) ? (u08b  ) Random32() & 0xFF : j;

pktNum++;

printf("=============== Packet Vector #%d ==================\n",pktNum);

ShowBlock(&p.key ,"AES Key:   ","\n",0);

ShowAddr (&p);

Generate_CTR_CBC_Vector(&p,1);

}

}

return 0;

}