IEEE P802.11
Wireless LANs

Compilation of TKIP Meeting Notes

Date:	February 6, 2002

Authors:		Dorothy Stanley
	Agere Systems
	2000 North Naperville Rd
	Naperville, IL, 60566
	Phone: +1 630-979-1572
	E-mail: dstanley@agere.com

Abstract

During the period from September 2001 through January 2002, several ad-hoc conference calls were held to discuss and define the TKIP solution, including re-keying methods. For the record, this document contains the notes from those meetings.

Tuesday January 8th

Monday December 17th, 2001

Friday, November 30th, 2001

Wednesday October 31, 2001

Wednesday October 24, 2001

Thursday, October 18, 2001

Wednesday, September 26th, 2001

Thursday, September 20, 2001

Wednesday, September 12th, 2001

Wednesday, September 5th, 2001

# Tuesday, January 8th

Below are brief notes from our conference call on Tuesday, January 8th.

Attendees January 7th Conference call:

Jesse Walker, Donald Eastlake, Niels Furgeson, Marty Lefkowitz,

Onno Letanche, Bob Moskowitz, Dorothy Stanley, Clint Chaplin,

Garrett S, Glen Zorn, Henry PT., Ron Brockman, Arnoud Zwemmer,

Doug Smith, Carlos Rios, Frank Ciotti, Nancy Can-Winget

Murli Rao, Dave Richkaf, Jon Edney, Russ Housley, TK Tan,

Keith Smith, Mike Sabin, Albert Young, Duncan Kitchin,

Simon Blake-Wilson, Kelly Mclelland, Dave Halasz.

Again apologies for misspellings and omissions.

Notes from January 8th Conference call:

1. MIC selection for TKIP - We have consensus on a MIC for TKIP and the MIC is Michael.

Michael is acceptable to the cryptographers, and has performance

acceptable on x486-type processors & other processors without a

multiply instruction. Michael has no patent encumberances. Niels/Intersil

will make it public and freely available.

Michael applies ONLY to TKIP. The proposed AES based solutions use

OCB or CBC-MAC as the MIC.

Criteria: Rank ordering of alternatives

(a) Acceptance in crypto community: MMH, MPH, Michael

MMH has the most "field experience", while MPH is new but

provably good, and Michael is the newest.

All are considered by our cryptographer consultants -Russ, Niels, Doug (post

call),

and Jesse to be "above the bar" required for this application. Russ, Simon

have asked RSA Labs, Certicom folks to look at Michael further.

(b) Performance on x486/no-multiply processors: Michael, MMH & MPH

Michael is only one which meets the acceptable performance on these

processors. Many of these processors in the deployed base. Michael also

can be implemented on some MAC processors.

(c) Performance on processors with multiply instruction: MMH & MPH, Michael

(d) Memory Usage: Michael, MMH & MPH (implementation trade-offs)

In these 2 categories, MMH and MPH are roughly equivalent, with the specific

implementation choice allowing trade-offs between speed and memory. One level

versus 2-level tree hashing in MMH - 15% performance penalty to gain order of

magnitude memory savings. Michael uses less memory.

(e) Patent claims: Michael, MMH and MPH are all to our best knowledge

unencumbered.

Action items relating to Michael

- Document active countermeasures for inclusion into the .11i draft. These

have been discussed on previous calls. (Dorothy)

- Craft motions for adopting Michael (Ron Brockman)

- Will Niels attend in Dallas? Niels and Ron to resolve

- Verify buy-in from Doug Whiting (Done), Tim Moore-MSFT, Bob O'Hara

2. 802.1X based Re-keying

(a) The current RC4 based EAPOL-Key mechanism for transferring the keys

is insecure. Half of the key material is exposed in the IV, and the same

algorithm as WEP is used. Thus a new mechanism is needed.

AES based Key Descriptor. Clint proposed a solution, and there has been

ongoing discussion on the timestamp mechanism and key

wrap algorithm. One proposed solution works, but requires 12 AES operations.

Clint will document the proposal, several people offered to make the necessary

motions for adoption in Dallas. Discussion is continuing on the reflector.

3. Jesse has put draft 1.7 (working version) up on the 802.11 members area.

Everyone needs to review this.

Are there still issues on the MAC ciphersuite negotiation algorithm? TBD, need

To review the current draft.

4. Per-packet hash - Done & documented (Russ)

5. IV sequencing - Done, needs to be documented (Dorothy)

# Monday December 17th  2001

Notes from December 17th conference call

Attendees:

Albert Young, Nancy Cam-Winget, Russ Housley, Jim Lansford, Frank Ciotti,

Jon Edney, Harschal Chaaya, Carlos Rios, Tim Moore, Marty Lefkowitz,

Mike Sabin, Onno Letanche, Bob Moskowitz, Anil Sinwalha, Tom Tsouloginnis,

Murli Rao, Dorothy Stanley, Donald Eastlake, Doug Whiting, Clint Chaplin,

TK Tan, Garrett, Henry........Apologies for mis-spellings and omissions.

1. Temporal Key Hash Updates - Russ, Doug

Russ has sent out an updated version of the proposal (550r2), with an expected

final version of the IV generation. Unless there are problems

uncovered with Re-keying, this hash should be stable. Scott

Fluhrer's initial review was positive, and he will complete his review before

Christmas.

RSA Labs's review is complete.

2. MIC Definition

Will a mode without the MIC be valid? NO. Support for the solution as a whole

from the cryptographers would evaporate without a MIC.

Concern for extremely low power - e.g. voice terminals. If WECA

wants to adopt special rules for application specific devices, they

are free to do that. However the TKIP solution will include the MIC.

Discussion regarding performance/memory tradeoffs between the MIC proposals.

Currently there are 3 proposals on the table: Michael, MPH, MMH.

(a) Michael - Being developed by Niels; goal is to have a MIC which could be

impemented on the MAC processor rather than host processor. Unclear this is

achievable.

Expect a final version from Niels shortly.

(b) MPH - Requires calculation of primes; was the subteam's recommendation

prior to the focus on Michael; lower memory requirements; requires protocol

message to convey info from client to AP; calculation of primes can be done

via table look-up, Doug Whiting had volunteered to generate sample code for

this.

(c) MMH - Requires a large MIC key, 100-200 bytes per station; concern for

large number of stations - e.g. 1024 stations. 1024 isn't realistic-

in most cases, much less than 50 stations per AP is the norm. Can make the

tradeoff in

number of stations supported; small key set-up time.

A subteam had looked at MMH versus MPH in September/October, making a

Recommendation to use MPH.  I've attached two e-mail

summaries from those meetings at the end of this e-mail, for reference/information on

past discussions.

3. 802.1X Re-key Mechanisms - All

Not everyone had Tim's documents; sent to them.

Comments to Tim on the current drafts:

- Current terminology is confusing - too many "session" keys; Consider

use of master/base/temporal keys

- What changes to the key-mapping keys are required? Require 2 key mapping keys

per MAC address.

- Is explicit key enablement verification provided? No. If the station receives the

key but can't set it, there's nothing the AP can do.

- Is a new key descriptor which uses AES instead of RC4 needed?

Consensus: Yes. The current mechanism exposes the first half of the key (IV) - which

is bad. Clint will draft a proposal for an AES based descriptor/wrap.

- If .11g happens, will potentially need re-keying in less than 1 minute.

Comment - Yes, but when g arrives, will have AES too.

- How is the broadcast key updated? Each station is sent an EAPOL Key message with the new key.

- Tim will update the documents - in January.

4. TKIP/WEP/AES coexistence.

Provide a mechanism for the transition case, when all

clients have not migrated from WEP.

Discussion: Should we go with a "quick" solution - using one of the spare

key-id bits to indicate TKIP encryption? Or user MAC level Ciphersuite negotiation

as per the current draft? Consensus: use the negotiation as per the current draft.

We will need to snap a version of the required fields/values to proceed with, prior

to standardization. Suggestion - take 1.6 draft & any Dallas changes.

Is there a problem with the current proposal re: negotiate to lowest level?

Possibly, need to look at this. If there is, need to fix it.

Don't really need negotiation - need an indication

of which algorithm was used to encrypt: WEP/TKIP/AES.

5. Next Meeting

January 8th. Discussed combining with Jan 11 .11i conference call;

decided not to - 11th agenda is full - 1.6 draft review

6. Sequencing of IVs. This still needs to be documented.

Dorothy will document current view, including result from tge/tgi meeting.

----------------------------------------------------------------------------------------

MIC Subteam e-mail

Subject:

Minutes from 802.11 MIC meeting

Date:

Fri, 21 Sep 2001 13:28:41 -0700

From:

"Walker, Jesse" <jesse.walker@intel.com>

To:

"'Martin Lefkowitz'" <lefko@ti.com>, Dorothy

Stanley<dstanley@agere.com>,

Doug Whiting <DWHITING@hifn.com>, Nancy Cam-Winget<nance@atheros.com>,

mandrade@cisco.com, batchar@cisco.com, bobb@sj.symbol.com,

barweg@ti.com, hchhaya@ti.com,

deaton@intersil.com, melkins@nai.com, dhala@cisco.com, Glenn Haley

<GHaley@hifn.com>,

thardjono@verisign.com, dan_nesset@3com.com,

Today Doug Smith (Cisco), Doug Whiting (HiFn), Russ Housley (RSA), and Jesse

Walker (Intel) met to discuss an 802.11 MIC. They discussed the following

issues: the MIC to recommend, what should the size of the MIC be, when to

change the MIC key, the RSA solution, and the interaction of the MIC

algorithm with fragmentation.

1. Which MIC

For a short term solution it is infeasible to use anything but something

like MMH or MPH; we immediately ruled out an HMAC or AES-based scheme for

the short term. We did not adequately discuss what to use for the long term,

but everyone though an HMAC-based algorithm would work.

MMH and MPH have similar costs once they are set up. MPH has more setup

complexity, but requires much less state.

MPH requires generation and agreement of a 64 bit prime number. This can be

done in background between rekeys, and the expected number of primality

tests is 2. This is going to require some authenticated protocol exchange,

or the same value has to be generated by peers. We need to definitively

specify this, so folks can determine whether they can do it on the existing

AP hardware.

Treeless MMH requires the same number of bytes of key as the maximum packet

size (2308 bytes, + 12 bytes for source and destination address). A

tree-based MMH implementation can be built using 192 bytes of key. MMH can't

have a zero byte anywhere in the key.

We are leaning toward recommend MPH, but we need to document the costs of

both and share them with the rest of the group.

2. MIC Size

We decided a 32-bit MIC is OK if we rekey after at most 2^16 packets, but a

64-bit MIC is essential if we rekey less frequently than that.

We believe we can make a rationale that sticks based on the rekey frequency.

Roughly, the argument goes: an attacker can forge a packet in fewer attempts

if we use a smaller MIC. Say we specify we can tolerate at most 2^16 forgery

attempts when we use a 32-bit MIC, and 2^32 forgery attempts when we use a

64-bit MIC. Intuitively it seemre likely that it is more likely we will hit

one of these thresholds when we wait longer between rekeys. We still have to

ellaborate the rationale and document it.

We can hardcode the MIC size for the short-term fix to WEP, but if we do

this, then we have to negotiate it later for backward compatibility, because

rekeying after every 2^16 packets seems frivolous when we migrate to AES.

3. When to change the MIC key

We recommend changing this whenever the temporal encryption key changes as

well. At 802.11b rates, the maximum data packet rate is about 3000

packet/sec. If we change the temporal key every 2^15 packets for WEP, this

means we only need to rekey about once every 10 seconds.

We do not want to generate a per-packet MIC key.

4. An RSA Solution?

In the 80's RSA Labs apparently invented a MIC based on RC4. By the time of

the teleconference next week we will know whether this can be used in 802.11

and then compare it with the recommended solution. This algorithm is table

based and uses no multiplies. It is probably slower than MMH or MPH, because

a per-byte table look-up will cost more than a multiply on reasonable CPUs.

5. Interaction with fragmentation

In most of the discussion we have been assuming that the MIC is in every

packet. However, we are assuming it is computed and checked on the host, not

on the card. This implies that if jumbo datagrams are used, the MIC will not

be each packet. We convinced ourselves this probably did not matter as long

as the receiver check the MIC over the whole packet before releasing it up

he protocol stack, but we haven't gone through a detailed analysis.

6. Other issues

We need to revisit where the MIC is computed for a long term solution.

7. Actions:

Doug Whiting: will quantify the costs.

Jesse Walker: work on protocol issues, merge eveything into one document.

Doug Whiting: send costs to  Russ to run past RSA folks.

-----Original Message-----

From: Walker, Jesse [mailto:jesse.walker@intel.com]

Sent: Friday, October 05, 2001 9:15 PM

To: Brockmann, Ronald; Nancy Cam-Winget; Dorothy Stanley; Doug Whiting;

mandrade@cisco.com; bobb@sj.symbol.com; barweg@ti.com; hchhaya@ti.com;

Eaton, Dennis; melkins@nai.com; dhala@cisco.com; GHaley@hifn.com;

thardjono@verisign.com; rhaven@cisco.com; heegard@ti.com;

rhousley@rsasecurity.com; denis.kuwahara@boeing.com; lefko@ti.com;

DMakishima@hifn.com; kpm@valencesemi.com; timmoore@microsoft.com;

rgm@trusecure.com; apotter@icsa.net; proshan@cisco.com; dsmit@cisco.com;

albert_young@3com.com; FZarghami@hifn.com; gwz@cisco.com;

waa@cs.umd.edu; Zwemmer, Arnoud; sblakewilson@certicom.com;

jon.edney@nokia.com; Monteban, Leo (Leo); Diepstraten, Wim (Wim);

Letanche, Onno (Onno); HAndersson@rsasecurity.com; rfriend@hifn.com;

cchaplin@sj.symbol.com; greg chesson; bob@bstormnetworks.com;

phil@wayport.net

Subject: Minutes from MIC ad hoc meeting

Doug Smith (Cisco), Doug Whiting (HiFn), Russ Housley (RSA) and Jesse

Walker (Intel) attended this phone meeting. The goal of the meeting was to

determine whether our legacy systems MIC design is adequate in light of the

newly understood fact that the sequence number (IV) is not necessarily

available to the driver software, so unavailable to the driver for

replay protection.

Observation: we have already agreed to require that WEP IVs must be received

in order. The IV is still used to construct the per-packet encryption key.

It appears the MIC is indeed protected from attack since the receiver will

have to enforce IV monotonicity and since the per-packet key constructed

from it is used to encrypt the MIC. The group thought this arguement is

valid, but Doug Whiting will talk with David Wagner about the scheme to look

for problems. If there is a problem, we will instead recommend including

a new 16-bit counter in the payload to use for replay protection, and the

MIC will protect this counter.

The group revisited how often to rekey. It seems as though we should

Rekey the MIC key at the same time as the encryption key, to minimize the

complexity of the overall design. Clearly, one can get away with rekeying

less frequently with per-link keys than with a group key. A 16-bit counter

in the payload and simultaneous rekeying for both the MIC and the encryption

keys would mean we have to rekey at least every 2^16 packets.

The group discussed alternative MIC algorithms and reaffirmed its

recommendation of MPH as its recommended MIC, as no one had a strong

preference for a different algorithm. This choice the amount of state

that has to be kept for each MIC key, but adds some set up cost. If we rekey

every 2^15 packets, and there are 2^12 packets a second, then we need a new

prime every 2^3 = 8 seconds. The prime computation overhead then works out

to about 4750 cycles/second.

What does MIC cover? DA, SA, and data. What order? DA || SA || data, with DA

and SA represented as in clause 7.1.1 of the 802.1 standard. Obviously, if

we have to include a sequence number, this would be covered, too. We talked

about appending such a sequence number to the data, and appending the MIC to

this (or to the data if we don't have to include a sequence number).

There was a side discussion on the mixing function. Doug W. needs to hear

back from Ron Rivest whether the two phase key mixing is sufficiently sound

cryptographically. If it isn't, then there is really no choice but to find

some analog of association ids for the IBSS case.

Actions:

Doug Smith to generate packets for Doug Whiting to construct MPH test vectors.

Doug Whiting to generate test vectors, and to provide sample prime generation code for MPH.

- No specific IV windowing is needed.

- Friday, November 30th, 2001

Atttendees: Bob Moskowitz, Clint Chaplin, Jesse Walker,

Donald Eastlake, Doug Whitinig, Keith Smith, Jon Edney,

Harschal C., Mike Sabin, Onno Letanche, Niels Furgeson,

Bob Beach, Carlos Rios, Russ Housley, Albert Young

Kelly McClellan, Dorothy Stanley

November 30 Discussion:

1. Temporal Key Hash Updates - Russ, Doug, Niels

Specification of first octet of IV - Scott Fluhrer has publicized a weakness

that the first 2 bytes predict the key schedule. In addition, there are unpublished

results showing that the sum of the first 2 bytes should be even. A modified algorithm

for specification of the first octet of the IV is being evaluated, within

RSA labs, and with Scott Fluhrer. Russ is working this.

Alternative key hash algorithm - There is one alternative key-mixing algorithm,

which would use 24 bits of IV

together with per-packet keying. It is sound cryptographically, but requires

additional memory to cache intermediate results. Another 128 bits per

connection must be stored. We agreed to table this approach at

present, pending additional clarification on the re-key algorithm to determine the

need for this. Focus energies on the MIC & re-keying at the moment.

2. MIC Definition - Niels

There will be modifications to "Michael" as it stands, based on feedback

from Scott Fluhrer.  Niels is making small changes

to strengthen the algorithm, with modest additional performace costs - 5% or so

additional. Implementers should tell Niels which parts of the algorithm are

costly on their processors, so  that he can minimize additional performance cost

in the changes. Niels is also running differential analyses, and these should be

finished before the end of the year. (His Christmas present to us all!) Niels

will re-send the current definition of "Michael", per request.

Countermeasures:

The MIC is weak as MICs go, requiring active countermeasures

to be paired with it. The approach we discussed for countermeasures is:

AP detects MIC failure on received packet

AP stops transmission and reception of all packets for a provisioned interval

AP Sets SNMP trap to notify administrators

Station (in ESS) detects MIC failure

Station stops transmission and reception

Station re-associates & re-authenticates, with a (new) indication in the

associate request that it is re-associating due to MIC failure. This

alerts the AP to the attack experienced by the station.

The AP must not allow a re-association due

to MIC failures more than once per minute. It will refuse new associations

with a reason code.

Ad-hoc case:IBSS

Station detects the MIC failure

Re-associates with a new temporal key

Can keep a count of the number of MIC failures detected

Questions on 802.1X re-keying in an IBSS - need to see text.

3. 802.1X Re-key Mechanisms - All

The text describing use of 802.1X for re-keying is due to Jesse from

Tim today. Jesse will forward the text when he receives it.

It's an issue that the text was not available on the 802.11

server prior to the vote.

Potential weakness due to use of RC4 as the encryption algorithm in

the EAPOL-key message. New types can be defined for this - e.g. AES, or

RC4, discarding the first 256 bytes. We could also consider an approach where

the nonce is sent in EAPOL-Key, rather than the encrypted value.

4. AES-OCB - All

Any Progress on Patent/Licensing from Gligor, IBM/Jutla? No updates

Specification of MIC to cover additional fields. This is viewed as necessary but

not sufficient for proceeding with OCB mode. Developing the text for this is

work, and people are unwilling to undertake this work if it isn't going to be

adopted.

5. AES-CTR/CBC-MAC -Russ, Doug, Niels

A block diagram of the algorithm is available. Discussion on MIC-then-encrypt

versus encrypt-then-MIC. Sometimes there is an advantage to encrypt-then-MIC,

since

on the receive side,

the receiver determines whether or not the frame is a good one before expending

the energy to de-crypt. In our case, the valuable resource is the wireless link

- which

has been used just to deliver the frame.

# Wednesday October 31, 2001

No notes available.

# Wednesday October 24th, 2001

Notes from October 24th:

1. How to Proceed with this work?

(a) Contributions to 802.11i in November?

(b) Goal to include in .11i?

(c) Goal to document recommended practice to WECA?

We had quite a long discussion around how to proceed with this work

in the upcoming November IEEE meeting. Motions are required to be composed,

voted upon and passed before the editor (Jesse) can include the changed

in the draft.

Should motions be crafted

to include portions of the solutions we have identified into the standard?

Conclusion - YES.

Will all parts of the solution be ready to be included in motions?

Conclusion - NO. The miximg function and a definition for sequencing IVs

should be ready. Re-keying is a maybe, and the MIC(Including countermeasures)

probably won't be ready.

What the objectives/criteria for proceeding with the next letter ballot?

Objective is to get feedback from the IEEE voting members on the significant

progress which has been made in the proposed standard specification.

The spec need not be complete. The .11i Chair's (Dave Halasz) goal is to

be ready for another letter ballot at the end of the November meeting.

Criteris is that significant progress has been made since March, Chair

(and ohers) believe this is true.

Should an "interim fix for WEP" be identified and presented to

WECA? View 1 - The market critically needs a solution to the problems of

WEP. We need to show progress, one way is to provide a "recommended

practice" which could be advanced via WECA. View 2 - Yes the market needs

a solution. Better to work it in one forum first- IEEE.

Conclusion - Potential time advantage of recommended practice

outweighed by potential duplication of specification and testing work.

Continue to use model of "IEEE defines, WECA tests". Monitor IEEE spec

progress.

2. Updates on per-packet "mixing" function.

Russ and Doug W have documented the current version of the mixing function.

Doug W will be adding test vectors. Some small changes still need

to be included, dealing with Endian-ness, and cache clarification.

The current algorithm uses a 16 bit IV, rather than 24 bit as imput.

Can the algorithm be modified to increase above 16 (Doug S)?

Current view no, discussion ongoing.

3. Re-keying approaches for shared key & session key - update

from Nancy, Albert, Greg, Jesse et al.

Still not sure there is complete understanding on differences

between Albert/Bob proposal and Nancy et al proposal.

Discussions continuing.

4. Protocol Proposals for a MIC - Update from Subteam

No update. Niels and subgroup working on this.

5. "Time to Market" versus "Complete solution"

Phased approach

No discussion.

6. Attendees:

Clint Chaplin,  Albert Young, Doug Smith, Dave Halasz

Henry Ptasinski, Carlos Rios, Jesse Walker,

Nancy Cam-Winget, Russ Housley, Greg Chesson, Dorothy Stanley, Bob Moskowitz,

Marty Lefkowitz, Doug Whiting, Kelly McClellan.

Apologies if I missed you.

# Thursday October 18, 2001

Attendees:

Niels Furgeson, Clint Chaplin, Donald Eastlake, Albert Young,

Onno Letanche, Henry Ptasinski, Carlos Rios, Jesse Walker,

Nancy Cam-Winget, Russ Housley, Dorothy Stanley, Bob Moskowitz,

Marty Lefkowitz, Doug Whiting, Kelly McClellan, Ron Brockman.

Apologies if I missed you.

Notes from October 18 Conference Call

1. Request for agenda item to discuss on how to proceed with this work.

Discussion included:

(a) Contribution to 802.11i in November?

(b) Goal to include in .11i? (Slow track)

(c) Goal to document recommended practice to WECA? (Fast track)

(d) All or some of above

Comment: This solution will be fielded for 5-10 years, need to make sure

it can, verify interoperability.

Discussion: What form will the documentation take? One document?

Three documents, one each for the mixing function, MIC and re-key

protocol, with a intro section?

Comment: First generate content, then edit.

Discussion: Since we are proposing changes to the beacon, is a standard .11i solution needed? Yes.

1. Per Packet Mixing Function

The purpose of the mixing function is to generate good per-packet

encryption.

Comments on the modification proposed by Russ Housley: reduces the calculations

needed "per-temporal key", but (unfortunately) doesn't reduce the per-packet calculations.

One data point - Implementation of new function - reduced number of cycles by 40

percent.

At this point, we're done with the per-packet hash. Doug Whiting and

Russ Housley will document the per-packet key hash mixing function.

2. Re-keying

Nancy is updating her document.

Albert and Bob O'Hara are updating their document. Still alot of discussion

and confusion over terminology, contents of the proposals. Jesse has

sent out an initial performance analysis, which generated discussion.

Comment: Number of stations affecting beacons affects performance. Need to

get a message back from each of the stations. A: But there is no exchange in

the count-down method. Comment: the analysis assumes there is only one key

updated. All n elements have to be in every beacon.

Comment: thinking separate sequence number would be a good idea. Could use

this to figure out which key you are using. A: There is a sequence number in

each information element. Comment: yes, but didn't think this was

incremental by 1, but that it depended on key. Why do you need all four

keys? If you are changing two keys but at different times.

Q: In message-based proposal, when receive on keyid(3)/key(Key-map-key). If

you receive on either key, always receive on key-mapping key. A: No, this

has been updated to tell you whether to  use ping v. pong value. "3" not

intended to be hardwired, provided only by example. Now that we are

rekeying, you need a second location for transition.

Q: But by Albert's reading of standard is that if you have a key-mapping

key, you never look at the key id. A: But the proposal is to change this

when you are using the rekey protocol. You always have to look at these in

this case. Legacy devices can continue to use existing rules, because they

don't implement the protocol.

3. MIC proposals

Niels and Doug will work together to find a new MIC that can run on

either the MAC or on the host CPU. and will be optimzed for 32-bit processors.

Niels' current proposal combines a relatively weak MIC calculation

with a policy on failure reporting:

If the MIC calculation is incorrect, then positive action/countermeasures, such as

sending an SNMP trap must be taken.

There was agreement on this approach, but no consensus on what the

appropriate countermeasures should be.

SNMP trap each MIC failure? Set a threshold for these failures prior to

notification? Stop transmitting? Jam the band?

Is there data on CRC failures in the field? Send any data to Niels.

Send data on host processor types also to Niels, so that the new MIC  design takes this into account.

4. "Time to Market" versus "Complete solution"

No additional discussion.

# Wednesday, September 26, 2001

Attendees:

Bob M, Doug S, Onno, Nancy, Greg, Albert, Russ, Harschal, Jesse, Dave H., Bill A., Marty, Dorothy.

Notes from September 26th Conference Call

1. Doug Whiting and Doug Smith have created and edited a C implemetation

of the per-packet mixing function. This has been sent out for implementers

to test against. Thank you to Doug and Doug for their work on this.

Issue Raised- The current algorithm uses the AID. What should be

used for the AID value in the IBSS case? The discussion identified several

options:

- XOR 3 chunks of MAC addresses (very minimum)

- Add associations to the IBSS case

- Create a centralized source of "control" for an AID-like value

- Include the transmitter address in the mixing function instead of the

AID. This removes the need for the AID.

Doug S and Russ took an action item to discuss the options with Doug W, and come

back with

a recommendation. The reference C implementation will have to be updated to

include the recommendation.

Issue: Is changing from + to xor in the "final sweep" ok with Scott F?

Don't have the answer yet. Doug S will follow-up with Scott F. The C reference

implementation has xor in the final sweep.

Issue: Performance impact of per-packet mixing function. One implementer

has a result of 520 cycles on a RISC machine. This is still alot of processing.

Challenge to mixing function developers: Is any further

simplification/streamlining

possible?

2. Rekey approaches for shared key and session key

Nancy has prepared 2 charts, showing the transitions and messages for

both shared and session cases. Changes - check use of keymapping keys.

Issue: Must the solution be designed for a "encrypt before QOS Queue" scenario,

or is a design for "encrypt after queue" sufficient. The current proposal

assumes that

the former, "encrypt before queue" is needed. This adds complexity,

and is unlikely to be the case for most implementations. Most implementations

have hardware or tightly coupled encryption-to-transmitter links.

What assumptions if any are being made by .11e? Solutions with software

encryption

may encrypt first and queue later.

Proposal: Albert has proposed a variation which integrates aspects of the time

based

approach. People need to read and understand this proposal. Please follow-up

with Albert for clarification.

Discussion: "Key mapping" - What is specified in .11? .11 specifies

a single key mapping key per station, and the existence of

key mapping keys is checked prior to using the default key mappings.

Observation: Key mapping is not needed in the stations. Each station can

use the 4 key IDs for key transitions. The AP however would potentially

have to manage two keys for more than one station at a time.

For the per-session case this means supporting multiple keys for

multiple stations.

Observation: Traffic analysis for 54Mbps worst case, 60 byte

packets - get 10K packets in one second. But traffic is distributed

across stations, and can measure bytes per user to determine

key-change-over time per user.

Issue: The underlying need is that there be 2 keys available for a period of time

to be used on transition/change-over of keys. This need exists for both the

station-AP pairwise key and the shared key cases.

Observation: Bulk of deployed .11b systems in shared mode can support this now.

Initial 802.11a systems can't double key for all stations at once

(per-station key case). Trying to find a way to support 2 keys per

station, one station at a time.

Issue: If only one key per station is supported, then is

the same key is used for transmit/receive to/from a given station?

Doug W's initial proposal had a way to provide 2 keys. Need to go back and

look at this.

Issue: Philosophy for adding messages:

(a) Introduce new management messages

(b) Piggyback on existing messages - e.g. association, re-association, beacon

Discussion on pros and cons of each approach.

3. Protocol Proposals for a MIC - Update from Subteam

Oct 26 - No specific discussion Russ has sent out link for MMH.

4. "Time to Market" versus "Complete solution"  Phased approach

Oct 26 - No specific discussion.

Related discussion: While the most secure solutions require

all elements of the solution (per-packet mixing, temporal keys,

MIC), implementations may (a) not be able to implement one element

in a given system, and/or (b) choose to make one or more element

user-configurable. We haven't yet had the discussion on what the standard will say.

5. Is a new MAC level Authentication type needed? Oct 26 - No discussion

6. My apologies again to Nancy and Greg for omitting them from the

prior notes distribution. Russ Housley (RSA) volunteered to set up an alias for us. Expect to see details soon from Russ.

End of notes.

# Thursday, September 20, 2001

Notes from September 20 Conference call

1. Per-packet Mixing Function.

The feedback from Scott Fluhrer is that the mixing function looks good.

The current crypto view is that it gets us to 2(91), which is

very good. Doug will code a reference implementation in C, together

with test vectors for implementers to test against.

Chris has a Matlab implementation which he is willing to share.

Contact him if you are interested.

We discussed the performance impacts of the current mixing function.

Doug S. suggested pre-computation of "mixed key" on receive side.

Could apply to shared key and session key scenarios.

This assumes sequentially incrementing IVs (would need to be specified).

The current CRC-32 is sufficient for protecting against re-play attacks.

2. Re-keying Approaches.

Nancy and Jesse are working on updating the proposal with comments.

The algorithm to expand the key can be found in RFC2409, Appendix B (Bob M.)

Marty and Greg will follow-up on issues on the protocol handshake

on transition.

IBSS - Is it sufficient to support shared key for IBSS? Probably.

The beacon generation does shift around among the stations in the IBSS.

3. MIC Proposals.

We discussed why a MIC is required, and that it is needed over the

payload, source and destination addresses. The MIC protectes against

bit-flipping attacks and re-direct attacks.

Now, which algorithm to use?

MMH - A possibility. Large memory reqt.

HMAC-SHA-1 - Too much computation required.

HMAC-MD5 - Too much computation required.

MPH - A possibility. Less memory, more cycles. 64bit? 32bit?

AES_CBC_MAC - Probably too computation intensive.

A MIC subteam was formed to further analyze the alternatives.

4. Phasing/Scope of Solutions

The solution we are working on is one which in totality applies to WEP rather

than AES based solutions. However the AES solution also needs a MIC and a

re-keying

mechanism. The additional per-packet mixing function is not needed for AES.

All elements of the solution (temporal key generation, per-packet mixing

function and MIC) are needed to secure 802.11b deployed systems

to the level that we can get the crypto community's support of the solution.

Specific implementations may choose to implement one or more of the solution

elements in a phased manner. Implementations may make selection of capabilities

provisionable by the end customer. However only the complete solution will have

approval of the crypto community.

5. We agreed to have another conference call on Wednesday September 26th.

Attendees: I counted 15 people -  Dennis Eaton, Onno, Dorothy, Doug W,

Bob M, Russ, Albert, Greg, Nancy, Harschal, Chris, Marty, Doug S., Jesse

and Bob B. Apologies if I missed you.

# Wednesday, September 12th, 2001

Notes from September 12 Conference call

1. Action items from Sept 5th - Per-packet Mixing Function.

Doug Whiting has developed a mixing function, which he forwarded to

everyone. Bob Beach has done initial coding, and it's predicted to

be ~150 cycles. Both Ron Rivest, and another contact at RSA Labs have

said that it looks strong enough, and Scott Fluhrer is thinking about

it now.

Next steps: Others to implement, await Scott's opinion.

2. Rapid re-keying to accomodate both session and shared key cases.

Nancy walked through the presentation which she, Greg Chesson and

Jesse prepared. Several comments were received, and Nancy (& others) will

incorporate them in the next version. Comments include:

a) Specify use of key ID fields so that a single system can support both

shared and session keys. Need one key field for the global or master secret key.

b) Consider use of the TIM/DTIM concept to specify the re-key interval -

use a rekey interval and a re-key count.

c) Add a mechanism to expand a user provided shared key to 256 bits

d) Add the base key and temporal key calculations for the shared key case

using the XWEP definitions.

e) Clarify how IBSS case is covered. Suggest using the shared key model.

3. We agreed to have another conference call on Tuesday September 20, if

the IEEE meeting was cancelled. Since it's cancelled, the conference call

is on, with the info above.

Attendees: I counted 16 people.

l

# Wednesday ,September 5th, 2001

Attendees:

Glen Zorn, Farshad Zarghami, Albert Young, Doug Whiting, Jesse Walker,

Dorothy Stanley, Doug Smith, Dan Nemits,

Robert Moskowitz, Onno Letanche, Marty Lefkowitz, Russ Housley,

Chris Heegard, Thomas Hardjono, Mike Elkins,

Jon Edney, Dennis Eaton, Harshal Chhaya,

Greg Chesson, Bill Carney, Nancy Cam-Winget, Ron Brockman

Notes from September 5th Call

(A) ACTION ITEMS

1. Define a "mixing" function for per-packet operation. Doug Whiting has

made one proposal, getting feedback from implementers that it was too costly.

Continue working to get a function which can both meet the timing constraints

and

provide enough mixing. Implementers to provide input to Doug on operations,

cycle budgets. Russ Housley(RSA) volunteered RSA Labs' assistance. Doug to work

interface

with RSA through Russ.

2. Temporal Key generation for the Session Key Case.

A subgroup of interested parties was formed to come back on the 12th

with one or two proposals. Nance to coordinate this. Ideally

have a solution which builds on top of the shared key solution base.

3. Authentication of portions of the beacon to prevent denial

of service/forced key rollover. Advocates of this need to define

the message, attributes, in a specific proposal. Possible

feature/option in phase 2 of a solution. Did anyone volunteer

to do this? Doug W, Jesse?

4. Set-up arrangements for Sept 12 conference call, notes for 5th - Dorothy.

(B) DISCUSSION

1. General Synchronization Approach for accomplishing Re-keying

The proposals presented in Akron use either Time Based

(Agere/Cisco/Intersil -TSF/TTBT)or Message Based (WSP - M0-M3, TI - SNAP

or MAC management, Atheros/Intel - 3 message) methods for accomplishing

re-keying.

Are there any others?

Ron Brockman - Use a change in the key ID bits to indicate

"change key" and imply, time to derive a new one. May need a new message

or way to acknowledge the change. Key Rollover may cover this. Ron to follow-up

as he wishes.

Time based - Simpler, no additional management messages

Message based - Can incorporate additional negotiation (e,g WSP ciphersuite)

2. Key-Rollover Indication

The current "XWEP" proposal uses the TBTT to indicate which key to use.

The key-rollover is handled via having both even/odd temporal keys available.

Transparent to current uses of the key-id bits.

Message based approaches - Message exchange (3 or 4 messages)

Proposal from Nance - Use keyID bits to synchronize key switch.

Use key id fields 0-3 for specific roles in the re-synchronization

process (per her Sept 3 e-mail)

Discussion : Using key ID bits doesn't use the reserved bits, doesn't

change logic in the .11b spec.

Discussion: Using odd/even doesn't impact how customers may be

using the keys today. But, hardware-only implementations can't support this.

Discussion: Might have issues with lease time concept, which

require exact timing. Need to couple with dual-key rollover interval.

Discussion: Agreement here on what needs to be accomplished - mechanism

for keeping the "older" key around until the "younger" key is in force.

3. Temporal Key Generation

The current "XWEP" proposal is to xor the 104-bit secret key with a

F(BSSID,44bits of TSF timer,4-0bits, and 4-0bytes). F is either

AES-CBC mode or RC4.

Discussion - Is this adequate mixing? Yes, strong enough.

Discussion - Since the beacon is used as catalyst for moving to the

"next" key, do we need to protect against denial of service attacks,

i.e authenticate portions of information in the beacon?

No agreement that this threat is severe enough to warrant the

proposed remedy. If a "rogue" beacon is sent, stations are "off"

for a second (or the next interval), and then will re-synch

with the "real" beacon. To move forward: Consider the "authenticated

timestamp" as a capability/option in a phase 2 of the

solution. Action: Advocates of this approach need to define a solution

in more detail. Several mechanisms were discussed: nonce

at association, nonce in beacon with authenticated timestamp,

additional management nmessages.

Discussion - There were 2 algorithms included in the initial

proposal, has one been selected? Work underway to measue impact.

Agere preference is AES-CBC.

Discussion: How is indication given to go launch the

key generation process? This can be time based, or based on number of packets.

Time based is simpler, and works well for the shared key case.

Packet count appears attractive for the session based, to deal

with re-keying many stations keys at the AP. Maybe there is a way

to use the time-indication with varying frequency? See action item 2.

4. Per Packet Operations

The current XWEP proposal is to xor the temporal key with the low order

bytes of the MAC address, and then apend the IV.

The current WSP proposal is to use RC4, discarding the first 32 bytes of output.

Discussion: Feedback from Scott Fluhrer is that discarding the first

n bytes is not secure. (not easy from an implementation viewpoint

either...).

Discussion: What is the benefit of adding more "mixing" in the

per-packet operation? Haven't we solved FMS by reducing the number

of packets per (now) temporal key? Temporal keys are a big improvement.

But the attacks will keep getting better, and if we can define an

additional per-packet operation, it will make the attacks much harder.

See action item # 1. Find the "low-water mark" of the implementations.

Drawbacks - isn't a per-packet operation introducing additional latency in

line-speed operations? Yes. It's part of the price of privacy.

5. General

5a. Shared Key environments and Session Key environments.

Need to support both shared key and per-station key environments.

No reason why the same basic approach can't work for both (Doug W).

Feedback - some of the first-out 802.11a implementations will not be

able to implement an additional per-packet operation. But keys per

station can be supported. The solution here will have to be to re-key

frequently.

End of Notes September 5th

--