IEEE P802.11
Wireless LANs

Normative Text for direct WSTA-WSTA communication

Date:	December 30, 2001

Author:        	 Srinivas Kandala	
 	Sharp Labs of America	
 	Camas WA 98683	
 	Ph: (360) 817-7512	
 	Fax: (360) 834-8696	
 	e-Mail: srini@sharplabs.com

Abstract

This document contains the material proposed to Tge for inclusion in the draft in the form of insertions into and replacements for material in of IEEE std 802.11-1999, as updated by IEEE STd 802.11e/2.0a, December 2001.

Editorial notes appear in bold italic Times New Roman font, informative notes appear in normal Arial font, and normative text appears in normal Times New Roman font.  Open issues are highlighted using red text in normal Arial font, and begin with "OPEN ISSUE:".  Changes to existing text in current standard are shown underlined and in red for additions and red with strikethrough for deletions.

Add a new subclause in clause 9 with the following text:

### Direct WSTA-WSTA communication

IEEE802.11e allows direct transfer between two WSTAs in an infrastructure BSS without using the distribution service provided by the AP. The MAC shall maintain a Connectivity Table, an illustration shown in Table X, from which it determines if the data to be sent directly to the destination or should be sent to the AP.

| MAC Address | State | Capability Information | PHY Rates |
| --- | --- | --- | --- |
| MAC Address 1 | Direct | Capability Information of WSTA 1 | PHY rates that can be sustained to WSTA 1 |
| MAC Address 2 | Distribution | NULL | NULL |
| MAC Address 3 | Distribution | NULL | NULL |
| MAC Address 4 | Direct | Capability Information of WSTA 4 | PHY rates that can be sustained to WSTA 4 |

Table X

When a data frame arrives to the head of the queue, the MAC determines if the destination address has an entry in the Connectivity table. If there is an entry, it will transmit the frame based on the entry in the table. If there is no entry in the table, it shall send a directed probe request frame at its own highest PHY rate to the destination. If the destination is within the range, it shall update its own Connectivity Table and send a directed probe response frame to the sender. Upon the receipt of the directed probe response frame, the sender shall update its Connectivity table and transmit the data frame to the destination. If there is no response, the sending MAC shall repeat the process, but using the next highest PHY rate. This procedure shall be repeated until there is a response or if the rate at which the probe request can be sent falls below half of the highest rate in the BSSBasicRateSet. If there is no response, then the MAC shall make an entry of “distribution” and will transmit the data frame to the AP.

The MAC shall use a similar procedure for determining the connectivity if the SME initiates a MLME-CONNECTIVITY.request which is described in 10.3.12.

Add a new subclause below 10.3.11 with the following text:

### 10.3.12 Direct WSTA-WSTA Communication

This mechanism supports the determination of direct WSTA to WSTA link in order for data transfers without involving the distribution services provided by the AP.

The primitives used for this mechanism are called Direct WSTA-WSTA communication primitives, which include MLME-CONNECTIVITY.request and MLME-CONNECTIVITY.response.

#### 10.3.12.1 	MLME-CONNECTIVITY.request

##### 10.3.12.1.1	Function

This primitive requests that the MAC entity determine the connectivity with another WSTA.

##### 10.3.12.1.2 	Semantics of the service primitive

The primitive parameters are as follows:

MLME-CONNECTIVITY.request 		{

WSTA MAC Address

}

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| WSTA MAC Address | MAC Address | N/A | The MAC address of the WSTA for which the connectivity results are generated. |

##### When generated

This primitive is generated by the SME at a WSTA to determine the connectivity to another WSTA in order for the initiating WSTA determine how the data frames generated will be handled by the MAC.

##### Effect of receipt

This primitive initiates a connectivity determination procedure at the MAC. The MAC will examine its own Connectivity table if there is an entry corresponding to the target WSTA MAC address. If there is no entry, it will initiate a direct probe request/response frame exchange to determine the connectivity status and update its own table.

#### 10.3.12.1 	MLME-CONNECTIVITY.response

##### 10.3.12.1.1	Function

This primitive returns the connectivity with a WSTA for which a MLME-CONNECTIVITY.request has been made by the SME.

##### 10.3.12.1.2 	Semantics of the service primitive

The primitive parameters are as follows:

MLME-CONNECTIVITY.response		{

WSTA MAC Address

Connection Type

Capability Information

PHY Rates

}

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| WSTA MAC Address | MAC Address | N/A | The MAC address of the WSTA for which the connectivity results are generated. |
| Connection Type | Enumeration | DIRECT, DISTRIBUTION | Indicates the result of the MLME-CONNECTIVITY.response |
| Capability Information | As defined in frame format | As defined in frame format | This field is used only when the Connection Type is “DIRECT” and shall have a NULL otherwise. The advertised capabilities of the WSTA with the WSTA MAC address. |
| PHY Rates | Set of Integers | 2-127 inclusive (for each integer in the set | This field is used only when the Connection Type is “DIRECT” and shall have a NULL otherwise. The set of data rates (in units of 500 kb/s) that can be used for a direct transfer of data to the WSTA with the WSTA MAC address. |

##### When generated

This primitive is generated by the MLME as a result of an MLME-CONNECTIVITY.request to ascertain the connectivity with a specific WSTA.

##### Effect of receipt

The SME is notified of the results of the connectivity determination procedure.