IEEE P802.11
Wireless LANs

Proposed additions to the AP Mobility Mechanism

Date:	March 4th, 2002

Author:	Mike Moreton
	Synad Technologies Ltd
	Cutbush Court						Lower Earley					Reading						RG6 4UW						UK
	Phone: +44 118 9131 500
	e-Mail:  Mike.Moreton@synad.com

Abstract

The aim of this document is to propose text for as many as possible of the open issues described in Adrian Stephens’ document “AP Mobility Mechanism” (IEEE 802.11-02/066r5), and to add text to reflect previously agreed decisions.

# Editing Instructions

Editing instructions are shown double underlined.

# 5. General Description

Add the following section:

## 5.5 Relationships Between Services

Add the following to the end of the list of Class 1 management frames:-

(vi) QAPCS assertion action frames.

# 5.9 AP Mobility

Remove the following text from the last bullet point:

“In order to protect against the possibility of hidden nodes, QAPCS may use one of a plurality of SSIDs, identifying different wireless networks.”

Change the text:

“Following a transfer, the BSS identity is not preserved, but the SSID is.”

to:

Following a transfer, the BSS identity is not preserved.  The choice of the configuration parameters for the new BSS is the responsibility of the newly active QAPCS, and they may not retain any of the previous settings, with the exception of the SSID which must be maintained.

# 5.9.1 Effect of AP Mobility On Higher Layers

Delete first paragraph (starting “A transfer of AP responsibility…”).

Replace the second paragraph (starting “When an inactive QAPCS…”) with:

A QAPCS shall  inform the higher layers whether it is the active QAPCS by means of the MLME-QAPCS-STATUS.indication primitive.  The higher layers may make use of this information as they wish, for example to connect or disconnect from the DS.

Add a new service primitive as follows:

# 10.3.10 Start

Add the following new service primitive:

10.3.10.x MLME-QAPCS-STATUS.indication

10.3.10.x.1 Function

This primitive is used when the QAPCS function either starts or stops being the active AP in the BSS.

10.3.10.x.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-QAPCS-STATUS.indication 	(

QAPCS active

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| QAPCS active | Enumeration | “Active” and “Inactive” | Whether the QAPCS function is the active AP in the BSS or not. |

10.3.10.x.3 When generated

This service primitive is generated by the MLME to inform the SME of a change in state of the QAPCS function.  It will be generated with “Active” each time the QAPCS function becomes the active AP in the BSS, and with “Inactive” each time it stops being the active AP.

The initial state of the QAPCS function is “inactive” so this primitive will not be generated until the QAPCS function first becomes the active AP in a BSS.

10.3.10.x.4 Effect of Receipt

The SME may take whatever action it deems appropriate as a result of the starting or stopping of the AP function.

# 11.4 Description of QAPCS behaviour

Add the following paragraph at the end of the section:

After receipt of the MLME-START.request primitive, the QAPCS function will become inactive, and follow the procedures described in section 11.4.2.

# 11.4.2 Behavior supported by the inactive QAPCS

Replace the third paragraph (ending “…defined in 11.4.2.2”) with:

An inactive QAPCS shall determine that the active AP for the BSS has failed if it receives no valid beacon frames from it in a period equal to the product of dot11MissedBeaconThreshold and the Beacon Interval from the last valid beacon frame.  If the inactive QAPCS has never received a valid beacon frame (for example after initialisation) it shall use the value of the beacon period from the MLME-START.request primitive instead.  An inactive QAPCS that determines failure of the active AP shall perform the passive takeover procedure defined in 11.4.2.2.

Replace the fourth paragraph (ending “…or a different channel”) with:

An inactive QAPCS that becomes active shall operate using its own BSSID and an unchanged SSID.  It shall use the BSS configuration parameters specified to it in the MLME-START.request primitive, and may, for example, start operation on the same or a different channel.

# 11.4.1.2 Scanning by the active QAPCS

Add the following paragraph to the end of the section:

If the QAPCS has set the Spectrum Management subfield of the Capability Information Field in its beacons and probe responses, then it may make use of the Channel Measurement Request facility as an alternative to performing the scan itself.

# 11.4.2.2 Passive Takeover

Remove the following sentence “The delay starts a beacon interval after the last missed beacon’s TBTT” as the method of initiating passive takeover has been slightly modified to handle QAPCS that have just been initialised.

# 11.4.3 Behaviour at the QSTA

Add new first paragraph:

This section describes procedures that a QSTA may carry out to minimise the disruption caused to it by changes of the active QAPCS.  It is for information only, as they are not required for the QAPCS function to work.

Change the remainder of 11.4.3 and 11.4.3.1 to make them informative.

# 7.3.2.xxx QAPCS parameter set element

Replace the existing section with:

This element is present in Beacon, Probe Response, Association Response and Reassociation Response MMPDUs transmitted by a QAPCS.

| Element ID (xxxx) | Length (5) | QAPCS Control Field (2 Octets) |
| --- | --- | --- |

Figure xxx.1 – QAPCS parameter set element format

The QAPCS Control Field is defined in figure xxx.2

| bits: 0-2 | 3 | 4-6 | 7 | 8-11 | 2-3 |
| --- | --- | --- | --- | --- | --- |
| User Priority 1 | Inhibit QAP Mobility | User Priority 2 | Line Power | Highest Supported PHY Rate | Infrastructure Bandwidth |

Figure xxx.2 – QAPCS Control Field

The User Priority 1 field is set to the value specified in the MLME-START.request primitive.

The Inhibit QAP Mobility field is 1 when the active QAPCS is not willing to become inactive.  Otherwise it contains 0.  See 11.4.1.3.

The User Priority 2 field is set to the value specified in the MLME-START.request primitive.

The Line Power field is set to the value specified in the MLME-START.request primitive.  It is 1 when the QAPCS is operating off line power.  Otherwise it is 0.

The Highest Supported PHY rate field is set according to the encodings described in table xxx.3 for the highest rate supported by the QAPCS on the PHY used to transmit the element.

The Infrastructure Bandwidth field is set to the value specified in the MLME-START.request primitive.  It is set according to figure xxx.3 based on the bandwidth of the “from infrastructure” direction of the connection to an infrastructure.

| Bandwidth Value | Description |
| --- | --- |
| 0 | No connection |
| 1 | 0 to 64 kbps |
| 2 | >64 to 128 kbps |
| 3 | >128 kbps to 1 Mbps |
| 4 | >1Mbps to 10Mbps |
| 5 | >10Mbps to 100Mbps |
| 6 | >100 Mbps to 1Gbps |
| 7 | >1 Gbps |

Figure xxx.3 – Coding of Bandwidth Values