IEEE P802.11
Wireless LANs

TGe AP Mobility Ad-hoc Editorial Clarifications

Date:	March 12, 2002

Author:	Adrian P Stephens
	Mobilian Corporation
	7431 NW Evergreen Pkwy, Hillsboro, OR 97124
	Phone: +44 771 276 3448
	e-Mail: adrian.stephens@mobilian.com

Abstract

This document contains proposed editing instructions to resolve LB30 letter-ballot comments assigned to the AP Mobility Ad-hoc group, and not covered in document 11-02-066.

# Section 7.1.3.3.   Remove the text: “The value of this field, in a QBSS, is the address of the STA at which the MLME-START.request that started the QBSS was executed. The BSSID remains unchanged for the life of a QBSS.”

Section 5.6.  Replace the text: “Only limited QoS support is available in an IBSS.” with the following:

A QIBSS supports EDCF operation.  The parameters that control separation of traffic classes in EDCF are fixed.  A QIBSS does not support setting up TSPECS,  has no HC and cannot support HCF operation.

Operation as an QIBSS can be avoided if one or more of its QSTAs support the QAPCS feature described in section 5.9.

In Annex D,  provide in a suitable place a table of default values for AIFS, CWmin, CWmax and MaxCFBlength as a function of TC.

| TC | AIFS[TC] | CWmin[TC] | CWmax[TC] | MaxCFBlength[TC] Units: ms |
| --- | --- | --- | --- | --- |
| 0 (be) | 2 | aCWmin | aCWmax | 0 |
| 1 (ee) | 1 | aCWmin | aCWmax | 1.5 |
| 2 (it) | 1 | aCWmin | 2*aCWmin+1 | 0 |
| 3 |  |  |  |  |
| 4 (vi) | 1 | floor (aCWmin/2) | aCWmin | 3 |
| 5 (vo) | 1 | floor(aCWmin/4) | floor(aCWmin/2) | 1.5 |
| 6 |  |  |  |  |
| 7 |  |  |  |  |

Note – aCWmin and aCWmax are the values appropriate to the PHY being used.

Note – there are no default values specified for TC 3,6,7

There is no MSDU lifetime specified for any TC.

(Ed: the name for MaxCFBlength may need to be changed to be consistent with other TGe  adopted proposals.

The representation of this MIB variable needs to be consistent with any signalling of this value in the Management elements (none yet proposed),  or other QoS durations – i.e. units of 32us, 16-bit width resulting in a range of 0..209720us.)