IEEE P802.11
Wireless LANs

Updated QOS Control Field

Date:	April 15, 2002

Author:	Bobby Jose,

Prepared for HCF Adhoc Discussions (LB30 Comment resolution)
	
	Phone: (916) 9618977
	
	e-Mail: bobby@ieee.org

Abstract

Enclosed below is an updated qos control field incorporating changes reflecting comment resolution of ballot comments made on letter ballot from Draft 2.0, and incorporating changes made in Draft 2.0a by various adhoc groups

# QOS Control Field

Table x.x – QOS Control Field

| Bits 0-6 | Bit 7 | Bit8 | Bit9 | Bit 10 | Bit 11 | Bits 12-15 | Usage |
| --- | --- | --- | --- | --- | --- | --- | --- |
| TXOP limit, Bit 8 = 0: units of 32 us Bit 8 = 1: units of 128 us |  | Scale | Ack Policy |  | FEC | TID | QoS data type frames that include CF-Poll sent by the HC |
| reserved (0) |  |  | Ack Policy |  | FEC | TID | QoS data type frames without CF-Poll sent by the HC |
| TXOP duration requested or Queue Size (7 bits) | Scale | Request Type | Ack Policy |  | FEC | TID | QoS data and QOS null frames sent by WSTAs |
| TXOP duration requested or Queue Size (9 bits) |  |  | Scale | Request Type | FEC | TID | RR frames |

Table x.x       Scale, Request Type Field Encoding

| Scale | Req Type | Usage |
| --- | --- | --- |
| 0 | 0 | TXOP duration requested, units of 32 us |
| 1 | 0 | TXOP duration requested, units of 128 us |
| 0 | 1 | Queue Size units of 128 octets |
| 1 | 1 | Queue Size units of 256 octets |

# Max, Min Values (Informative)

| Usage | Min Time | Max Time | Min Bytes | Max Bytes |
| --- | --- | --- | --- | --- |
| QoS data type frames that include CF-Poll sent by the HC | 32us | 32.64 ms | N.A. | N.A. |
| QoS data , and QOS Null frames sent by WSTAs | 32us | 16.256 ms | 128 Bytes | 32512 Bytes |
| RR frames | 32us | 65.408 ms | 128 Bytes | 130816 Bytes |

# Discussion

- What should the maximum supported value for Queue Size in Bytes?

- Need to consider another scale.

- FEC bit in RR is it of any use, even if RR is not aggregated, it would have to be in the same offset as data packet?

- FEC Field in RR for aggregated RR format (RR FEC depends only on 1 FEC bit, as with other management and control frames).