IEEE P802.11
Wireless LANs

Proposed edits to clause 11 for 802.11i

Date:	October 24, 2002

Author:	David Halasz
	Cisco Systems, Inc.
	320 Springside Dr. Suite 350
	Akron, Ohio 44333
	Phone: 330-664-7389
	Fax: 330-664-7301
	e-Mail: dhala@cisco.com

Abstract

The following contain proposed edits to clause 11, for 802.11i.

### 11.3.1 Stations association procedures

Change the text of Clause 11.3.1 from:

Upon receipt of an MLME-ASSOCIATE.request, a STA shall associate with an AP via the following procedure:

- The STA shall transmit an association request to an AP with which that STA is authenticated.

- If an Association Response frame is received with a status value of “successful,” the STA is now associated with the AP and the MLME shall issue an MLME-ASSOCIATE.confirm indicating the successful completion of the operation.

- If an Association Response frame is received with a status value other than “successful” or the AssociateFailureTimeout expires, the STA is not associated with the AP and the MLME shall issue an MLME-ASSOCIATE.confirm indicating the failure of the operation.

to:

Upon receipt of an MLME-ASSOCIATE.request, a STA shall associate with an AP via the following procedure:

- The STA shall transmit an association request to an AP with which that STA is authenticated. If the STA is operating in an RSN, the STA shall include the RSN IE with only one pairwaise key cipher suite and only one authenticated key suite.

- If an Association Response frame is received with a status value of “successful,” the STA is now associated with the AP and the MLME shall issue an MLME-ASSOCIATE.confirm indicating the successful completion of the operation.

- If an Association Response frame is received with a status value other than “successful” or the AssociateFailureTimeout expires, the STA is not associated with the AP and the MLME shall issue an MLME-ASSOCIATE.confirm indicating the failure of the operation.

### 11.3.2 AP association procedures

Change the text of Clause 11.3.2 from:

An AP shall operate as follows in order to support the association of STAs.

- Whenever an Association Request frame is received from a STA and the STA is authenticated, the AP shall transmit an association response with a status code as defined in 7.3.1.9. If the status value is “successful,” the Association ID assigned to the STA shall be included in the response. If the STA is not authenticated, the AP shall transmit a Deauthentication frame to the STA.

- When the association response with a status value of “successful” is acknowledged by the STA, the STA is considered to be associated with this AP.

- The AP shall inform the distribution system (DS) of the association and the MLME shall issue an MLME-ASSOCIATE.indication.

to:

An AP shall operate as follows in order to support the association of STAs.

- Whenever an Association Request frame is received from a STA and the STA is authenticated, the AP shall transmit an association response with a status code as defined in 7.3.1.9. If the AP is operating as an RSN, the AP will check the values received in the RSN IE, to see if the values received match the APs security policy. If the status value is “successful,” the Association ID assigned to the STA shall be included in the response. If the STA is not authenticated, the AP shall transmit a Deauthentication frame to the STA.

- When the association response with a status value of “successful” is acknowledged by the STA, the STA is considered to be associated with this AP.

- The AP shall inform the distribution system (DS) of the association and the MLME shall issue an MLME-ASSOCIATE.indication.

### 11.3.3 Station reassociation procedures

Change the text of Clause 11.3.3 from:

Upon receipt of an MLME-REASSOCIATE.request, a station shall reassociate with an Access Point via the following procedure:

- The station shall transmit a Reassociation Request frame to an Access Point. If the STA is operating in an RSN, the STA shall include the RSN IE with only one pairwaise key cipher suite and only one authenticated key suite.

- If a Reassociation Response frame is received with a status value of “successful”, the station is now associated with the Access Point and the MLME shall issue an MLME-REASSOCIATE.confirm indicating the successful completion of the operation.

- If a Reassociation Response frame is received with a status value other than “successful” or the ReassociateFailureTimeout expires, the station is not associated with the Access Point and the MLME shall issue an MLME-REASSOCIATE.confirm indicating the failure of the operation.

to:

Upon receipt of an MLME-REASSOCIATE.request, a station shall reassociate with an Access Point via the following procedure:

- The station shall transmit a Reassociation Request frame to an Access Point.

- If a Reassociation Response frame is received with a status value of “successful”, the station is now associated with the Access Point and the MLME shall issue an MLME-REASSOCIATE.confirm indicating the successful completion of the operation.

- If a Reassociation Response frame is received with a status value other than “successful” or the ReassociateFailureTimeout expires, the station is not associated with the Access Point and the MLME shall issue an MLME-REASSOCIATE.confirm indicating the failure of the operation.

### 11.3.4 AP reassociation procedures

Change the text of Clause 11.3.4 from:

An AP shall operate as follows in order to support the reassociation of STAs.

- Whenever a Reassociation Request frame is received from a STA and the STA is authenticated, the AP shall transmit a reassociation response with a status value as defined in 7.3.1.9. If the status value is “successful,” the Association ID assigned to the STA shall be included in the response. If the STA is not authenticated, the AP shall transmit a Deauthentication frame to the STA.

- When the reassociation response with a status value of “successful” is acknowledged by the STA, the STA is considered to be associated with this AP.

- The AP shall inform the DS of the reassociation and the MLME shall issue an MLME-REASSOCIATE. indication.

to:

An AP shall operate as follows in order to support the reassociation of STAs.

- Whenever a Reassociation Request frame is received from a STA and the STA is authenticated, the AP shall transmit a reassociation response with a status value as defined in 7.3.1.9. If the AP is operating as an RSN, the AP will check the values received in the RSN IE, to see if the values received match the APs security policy. If the status value is “successful,” the Association ID assigned to the STA shall be included in the response. If the STA is not authenticated, the AP shall transmit a Deauthentication frame to the STA.

- When the reassociation response with a status value of “successful” is acknowledged by the STA, the STA is considered to be associated with this AP.

- The AP shall inform the DS of the reassociation and the MLME shall issue an MLME-REASSOCIATE. indication.