IEEE P802.11
Wireless LANs

802.1X Reencapsulation

Date:	October 16th, 2002

Authors:		David Johnston
	Mobilian Corporation
	7431 NW Evergreen Parkway,
	Suite 220,
	Hillsboro, OR, 97006
	Phone: +1 503-681-6800 x282
	E-mail: dj@mobilian.com; david.johnston@ieee.org

This document describes modifications to IEEE Std 802.11i/D2.5, edited by Jesse Walker.

My thanks go to Jin-Meng Ho of Texas Instrument who has made substantial contributions to this document.

To distinguish between editing instruction in the draft text that relate to the standard and editing instructions in this submission that relate to the draft text, the latter will be represented with a yellow background, like this.

### 5.9.2  IEEE 802.11 usage of 802.1X

IEEE 802.11 depends upon IEEE 802.1X to control the flow of MSDUs between the DS and unauthorized stations by use of the controlled/uncontrolled port model outlined above. EAP authentication packets (contained in IEEE 802.11 MAC MMPDU frames of type management, subtype authentication) are passed to the IEEE 802.1X authenticator via the uncontrolled port. IEEE 802.11 MAC data frames are passed (or blocked) via the controlled port. Each association between a pair of stations creates a unique IEEE 802.1X “port,” and authentication takes place relative to that port alone.

IEEE 802.11 depends upon IEEE 802.1X to change its cryptographic keys. IEEE 802.1X may choose to change the keys for a variety of reasons. Some of the reasons include elapsed time or when a certain number of packets have been transmitted or received.

Change table 13 in section 7.2.3.10 to:

| Order | Information | Notes |
| --- | --- | --- |
| 1 | Authentication algorithm number |  |
| 2 | Authentication transaction sequence number |  |
| 3 | Status code | The status code information is reserved an set to 0 in certain authentication frames as defined in Table 14 |
| 4 | Challenge text | The challenge text information is only present in certain authentication frames as defined in Table 14 |
| 4 | 802.1X message | 802.1X message information is only present in authentication frames pertaining to the IEEE 802.1X Authentication algorithm |

Change table 14 in section 7.2.3.10 to:

| Authentication algorithm | Authentication transaction sequence number | Status Code | Challenge text | 802.1X Message |
| --- | --- | --- | --- | --- |
| Open System | 1 | Reserved | Not Present | Not Present |
| Open System | 2 | Status | Not Present | Not Present |
| Shared Key | 1 | Reserved | Not Present | Not Present |
| Shared Key | 2 | Status | Present | Not Present |
| Shared Key | 3 | Reserved | Present | Not Present |
| Shared Key | 4 | Status | Not Present | Not Present |
| RSN | n (n = 1, 2, …) | Status | Not Present | Present |

Change section 7.3.1.1 to:

7.3.1.1 Authentication Algorithm Number field

The Authentication Algorithm Number field indicates a single authentication algorithm. The length of the Authentication Algorithm Number field is 2 octets. The Authentication Algorithm Number field is illustrated in Figure 24. The following values are defined for authentication algorithm number:

Authentication algorithm number = 0: Open System

Authentication algorithm number = 1: Shared Key

Authentication algorithm number = 2 : RSN

All other values of authentication number are reserved.

Figure 24—Authentication Algorithm Number fixed field

#### Change section 7.3.1.4 to:

#### 7.3.1.4 Capability Information field

Change “Figure 27 – Capability Information Fixed Field” to add B11 which is “Robust Security”.

B0       B1         B2             B3             B4         B5                              B10     B11    B12                        B15

| ESS | IBSS | CF Pollable | CF Poll Request | Privacy | Reserved | RSN | Reserved |
| --- | --- | --- | --- | --- | --- | --- | --- |

Figure 1—Capability Information fixed field

Add the following paragraphs to Clause 7.3.1.4:

RSN-capable APs shall assert the Enhanced Security Subfield (B11) of the Capability Information field to 1 in Beacon and Probe Response Management frames, to indicate support for enhanced security negotiation. RSN-capable APs also assert this bit in Association and Reassociation Responses to Association and Reassociation Requests with the bit set. RSN-capable APs do not assert this bit in Probe, Association, or Reassociation Responses to Probe, Association, Reassociation Requests that do not assert the bit.

RSN-capable STAs in ESS mode set the Enhanced Security subfield to 1 in Association and Reassociation messages sent to APs that assert the bit in their own Beacons and Probe Responses; they can always assert the bit in any of these messages, to indicate support for enhanced security negotiation.

When the Robust Security Subfield is asserted, the Privacy Subfield shall also be asserted as well, meaning that privacy is always required in an RSN.

Change Table 19 in section 7.3.1.9 to:

Table 19—Status codes

| Status Code | Meaning |
| --- | --- |
| 0 | Successful |
| 1 | Unspecified failure |
| 2-9 | Reserved |
| 10 | Cannot support all requested capabilities in the Capability Information field |
| 11 | Reassociation denied due to inability to confirm that association exists |
| 12 | Association denied due to reason outside the scope of this standard |
| 13 | Responding station does not support the specified authentication algorithm |
| 14 | Received an Authentication frame with authentication transaction sequence number out of expected sequence |
| 15 | Authentication rejected because of challenge failure |
| 16 | Authentication rejected due to timeout waiting for next frame in sequence |
| 17 | Association denied because AP is unable to handle additional associated stations |
| 18 | Association denied due to requesting station not supporting all of the data rates in the BSSBasicRateSet parameter |
| 19 | RSN Authentication in progress |
| 20-65,535 | Reserved |

Add section 7.3.2.9:

7.3.2.9 802.1 Message element

The 802.1X Message element contains an IEEE 802.1X authentication message. See figure 43.

Figure 43—802.1X Message element

Change section 8.1.1 to:

### 8.1.1 Security components

Pre-RSN security consists of two basic subsystems:

- WEP privacy, to encapsulate data, and

- IEEE 802.11 authentication.

8.2.2.1 describes WEP, while 8.2.3.1 describes the 802.11 authentication procedures.

RSN security consists of two basic subsystems:

- Data privacy mechanism:

  - TKIP, to provide minimally adequate level of data privacy for pre-RSN hardware conforming to the 1999 issue of this standard;

  - WRAP, an optional AES-based protocol, to provide robust data privacy for the long term; and

  - CCMP, another AES-based protocol, to provide robust data privacy. Any implementation claiming to provide security shall implement CCMP

- Security association management:

  - RSN negotiation procedures, to establish a security context;

  - An authentication algorithm using IEEE 802.1X methods;

  - IEEE 802.1X key management, to provide cryptographic keys;

Change section 8.1.5 to:

### 8.1.5  Operation

RSN supports two models of operation. One model is based on IEEE 802.1X authentication, while the other depends on a global pre-shared key.

RSN-capable STAs use Beacons and Probe request to identify other RSN-capable peer STAs. When the peer indicates it is RSN-capable, the STA shall implement the following sequence of procedures in the IEEE 802.1X authentication model:

- First it determines the available group key cipher suites pairwise key ciphersuites and authenticated key management cipher suites from the information presented in the RSNE.

- It authenticates to the peer using the RSN authentication mechanism. Sections 8.4.6 and 8.4.7 describe the IEEE 802.11 use of IEEE 802.1X authentication.

- During the RSN authentication, it negotiates the cipher suites to be used and executes a key exchange algorithm, based on the IEEE 802.1X EAPOL Rekey protocol. Clause 8.5 describes the IEEE 802.11 use of IEEE 802.1X key management, to obtain temporal keys.

- The STA associates with the peer

- Finally, it uses the agreed upon temporal keys and cipher suites to protect the data traffic. 8.3.2, 8.3.3, and 8.3.4 describe the three defined data RSN encapsulation mechanisms.

If the peer fails to indicate it is RSN-capable, the STA may fall back to the following procedures:

- first uses 1999 IEEE 802.11 (Pre-RSN) authentication;

- followed by association;

- optionally followed by use of legacy WEP.

8.2.3.1 describes pre-RSN authentication, while 8.2.2.1 describes WEP.

If the BSS is based on a global pre-shared key, the STA instead executes the following sequence of procedures:

- It runs the Clause 8.5 the key exchange, to establish pairwise and group keys and cipher suites. It uses the global pre-shared key as the pairwise master key for each such exchange

- It uses the established keys and cipher suites to protect the link.

Change section 8.1.6 to:

### 8.1.6  RSN assumptions and constraints

RSN assumes:

- Mutual authentication of the IEEE 802.1X AS and the STA. This assumption is intrinsic to IEEE 802.11 LANs and cannot be removed without compromising security.

- In particular, the mutual authentication requirement implies an unspecified prior enrollment process, as the STA must be able to identify the ESS or IBSS as an entity that it regards as genuinely trustworthy. The non-secured IEEE 802.11 model of promiscuous roaming does not and cannot provide security in a WLAN. This assumption is intrinsic to IEEE 802.11 and cannot be removed without compromising security.

Informative Note: This assumption complicates some business models, such as those used by 802.11 hot spot providers, but this in no way eliminates the assumption. Enrollment can be indirect, e.g., an organization might use a PKI for authentication, signing the hot spot provider’s certificate with a key STAs from their organization trust. Such a signing key can only be employed for this one purpose—certifying that the bearer’s is a party trusted to enforce the signer’s security policy—or security of the WLAN is lost. In practice service level agreements and auditing will be needed to be able to verify that the provider actually enforces the security policy delegated in this manner.

- RSN assumes that either the mutual authentication is strong or is somehow shielded from unauthorized reception. This assumption is intrinsic to IEEE 802.11 LANs and cannot be removed without compromising security.

- Authentication derives a fresh—i.e., never before used—session key.

- In an ESS all APs lie entirely within the security boundary surrounding the IEEE 802.1X AS. This is a very strong configuration constraint. In practice this implies that either the IEEE 802.1X server is embedded in the AP, or else the AP is physically secure (e.g., physical access to the AP is controlled; access—both physical and by network—to the DS is controlled; the AP shielded from all unauthorized radio transmissions, etc.), and the communication channel between the AS and the AP lies entirely within the security boundary as well.

- Key generation of a 256-bit key at the Supplicant and Authentication Server for use by the Supplicant and Authenticator.

Change section 8.4 and 8.4.1 to:

## 8.4  RSN security association management

### 8.4.1  Security association life cycle

IEEE 802.11 uses the notion of a security association to describe secure operation. Secure communications are possible only within the context of a security association, as this is the context providing the state—cryptographic keys, counters, sequence spaces, etc.—needed for correct operation of the IEEE 802.11 cipher suites.

The life cycle of a security association is naturally intertwined with the other IEEE 802.11 mechanisms. A STA can operate in either an ESS or in an IBSS, and a security association has a distinct life cycle for each.

In an ESS there are two cases: initial contact between the STA and the ESS, and roaming by the STA within the ESS. A STA and AP establish an initial security association via the following steps:

- The STA selects an authorized ESS by selecting among APs that advertise an appropriate SSID.

Informative note: Advertising the SSID plays a crucial security function. If the STA does not know the SSID of some AP, it either must decline communication, or it has to guess the ESS of the AP. When the AP is not authorized, then the STA might present all of its credentials in an effort to find some that allow it to authenticate. This can result in unintended identity disclosure of the STA to the unauthorized AP.

Advertising the SSID also provides an important performance optimization. Without advertisements, if the AP is indeed authorized, the STA on average must present half its credentials before locating the correct ones at initial contact.

- The STA then uses IEEE 802.11 RSN authentication. The STa’s Authenticator shall initiate IEEE 802.1X authentication. The authentication will be mutual, as the STA needs assurance that the AP belongs to the authorized network and is not a rogue.

- During authentication, cryptographic keys are shared between the IEEE 802.1X AS and the STA. The AS distributes these keys to the AP, and the AP and STA use two key confirmation handshakes, called the 4-way handshake and group key handshake, to complete security association establishment. The key confirmation handshakes indicate when the link has been secured by the keys, so is safe to associate and allow normal data traffic.

- The STA associates to the AP. Normal data traffic shall be protected using the transient keys that were exchanged during authentication.

Informative Note: Any secure network cannot support promiscuous association as in unsecured operation of IEEE 802.11. A trust relationship must exist between the STA and the target SSID prior to association and secure operation, in order for the association to be trustworthy. The reason is that an attacker can deploy a rogue access point just as easily as a legitimate network provider, so some sort of prior enrollment procedure is necessary to establish credentials between the ESS and the STA.

Informative note: The Supplicant of a STA should silently discard IEEE 802.1X messages not received from the AP.

A STA roaming within an ESS establishes a new security association by one of two schemes:

- IEEE 802.1X authenticating to a new AP followed by (Re)association to the AP.. In this case the station repeats the same actions as for an initial contact association, but it also uses the MLME-DELETEKEYS.request to remove the cryptographic key from the IEEE 802.11 MAC when it roams from the old AP. The STA also deletes the cryptographic keys when it disassociates/deauthenticates from all BSSIDs in the ESS.

- A STA already associated with the ESS can instead request its IEEE 802.1X Management Entity to authenticate with a new AP before associating to that new AP. In this case the Management Entity can request its IEEE 802.1X Supplicant to send an AuthenticationRequest to an AP with which it is not associated. The STA’s IEEE 802.11 Management Entity delays Reassociation with the new AP until IEEE 802.1X authentication completes. If IEEE 802.1X authentication completes, then cryptographic keys shared between the new AP and the STA will be installed, creating an environment where Reassociation without a preceeding  IEEE 802.1X full authentication makes sense.

The MLME-DELETKEYS.request terminates a security association on the local STA. This primitive destroys the cryptographic keys established for the security association, so that they cannot be used to protect further IEEE 802.11 traffic. A STA’s IEEE 802.11 Management Entity uses this primitive in one of two situations: when it disassociates or deauthenticates from an AP in an ESS, and when it associates to a new AP.

The life cycle of a security association is different in an IBSS. When explicit authentication is not used, a STA sets the AuthenticationRequest variable to request that its IEEE 802.1X implementation initiate the 4-way handshake of 8.5 with a Pre-Shared Key (PSK) with IBSS peer STAs it encounters. A STA should use this primitive when it encounters another STA belonging to the IBSS with which it has no security association.

Informative Note: A STA can receive 802.1X messages from a previously unknown MAC address. Membership in the IBSS is determined by the peer STA’s ability to use the correct PSK.

Informative Note: Any STA targeted from the IBSS may decline to form a security association with the joining STA. An attempt to form a security association may also fail because, e.g., the peer uses a different pre-shared key.

In an IBSS each STA defines its own group key to secure its broadcast/multicast transmissions. After establishing a security association, each STA shall use the Group Key Handshake to distribute its transmit Group Key to its new peer STA.

A security association terminates in an IBSS in the same way it does in an ESS, by the IEEE 802.11 Management Entity invoking the MLME-DELETEKEYS.request primitive.

Informative Note: A STA should remove all association state and send a deauthenticate message if it receives an MLME-DELETEKEYS.request.

Change section 8.4.2 to:

### 8.4.2 RSN selection

In an RSN or a TSN STAs (including APs) shall advertise their capabilities by asserting the Robust Security bit of the Capabilities Information Field in Beacon and Probe Response messages. In an RSN a STA may also include the RSN Information Element (RSN IE, see 7.3.2.17) in Beacons, and Probe Responses. When doing so, the included RSN IE shall specify all the authentication and cipher suites enabled by its policy. An RSN-capable STA operating as part of a TSN may omit the RSN IE from its Beacons and Probe Responses. A STA shall not advertise any authentication or cipher suite that is not enabled and that it will not agree to use.

An RSN IE used to advertise IBSS capabilities shall specify no an authentication suite—i.e., it shall omit the authentication suite from the RSN IE—“None” as the group key cipher suite, and exactly one pairwise key cipher suite.

The STA’s 802.11 Management Entity shall utilize the MLME-SCAN.request to identify neighboring STAs that assert Robust Security and advertise an SSID identifying an authorized ESS or IBSS. A STA may decline to communicate with STAs that do not assert Robust Security, or do not advertise an authorized SSID. A STA may also decline to communicate with other STAs that do not advertise authorized authentication and cipher suites with its RSN IE.

A STA shall advertise the same RNSE in both its Beacons and Probe Responses.

Informative Note: Whether or not a STA may attempt to communicate with another STA that asserts Robust Security but which does not advertise an authorized SSID is a matter of policy.

Informative Note: Whether a STA with Robust Security enabled may attempt to communicate with a STA that does not assert RSN is a policy question.

Informative Note: It should be possible to independently enable or disable the following in an RSN AP:

- RSN

- TSN

- WEP using RSN IEEE 802.1X key management

- WEP without key management.

For RSN an AP should support TKIP as well as CCMP.

Informative Note: It should be possible to independently enable or disable the following in an RSN STA:

- RSN

- WEP using RSN IEEE 802.1X key management

- WEP without key management.

Informative Note: As a practical matter, the multicast cipher suite must be the weakest unicast cipher suite enabled.

Informative Note: An AP should support pre-shared keys.

In an IBSS a STA may also identify another STA as belonging to the same IBSS by receiving a protected message with A3 asserting the BSSID of the IBSS. If a STA does not already have a security association with the message source, the receiver will not have cryptographic keys to decapsulate messages it receives from that STA. On receiving a protected message from such a STA, the receiver should attempt to initiate a security association, as described in 8.4.1.

Informative Note: Typically this sort of message will be broadcast/multicast. It is also possible to receive a protected unicast message after a STA has reset in a way that is undetectable to the message source.

Similarly, if a STA in an IBSS receives the first message of a 4-way handshake from an unknown STA asserting the IBSS BSSID as A3, the STA’s IEEE 802.1X implementation should respond, in an attempt to establish a security association.

Change text introducing section 8.4.5 to:

### 8.4.5 MSDU filtering

After association, when data traffic is permited to flow, A STA or AP invoking RSN protections shall filter received unprotected unicast traffic. Is shall also filter received unprotected broadcast and multicast traffic  if the privacy subfield of the capability field is set to zero.

Figure 2 depicts a time-sequence diagram of the events related to filtering.

Change text introducing section 8.4.6 to:

### 8.4.6 RSN authentication in an ESS

RSN authentication is initiated by any one of the following mechanisms:

- If a STA negotiates to use RSN authentication prioe to  (re)association, the STA’s management entity can respond to the  MLME-AUTHENTICATE.request . Thus, in this case, authentication is driven by the STA’s decision to associate and the AP’s decision to access the association.

- If a STA’s MLME-SCAN.indication finds another AP within the current ESS, a STA may signal its Supplicant to use IEEE 802.1X to pre-authenticate with that AP.

- If a STA receives an IEEE 802.1X message, it delivers this to its Supplicant or Authenticator, which may initiate a new IEEE 802.1X authentication.

Informative Note: The IEEE 802.1X Authenticator of an AP initiate authentication by sending an EAP-Request/Identity message to the Supplicant of a STA.

Informative Note: When a STA (re)associates with an AP without a (recent enough) pre-authentication, the AP has no cryptographic keys configured for the STA. In this case, the AP’s Authenticator will force a full IEEE 802.1X authentication. In the case where the STA has recently pre-authenticated with the AP, the AP will retain the STA’s IEEE 802.1X identity and cryptographic keys from the pre-authentication. In this case, the AP’s Authenticator may proceed directly to key management in response to the STA’s Supplicant’s EAP-Response/Identity.

Informative Note: Pre-authentication completes when the AP’s IEEE 802.1X Authenticator sends the first message of the 4-way handshake to the STA’s IEEE 802.1X Supplicant.

Informative Note: If IEEE 802.1X authentication completes successfully, the AP’s Authenticator forwards an EAP-Success message to the STA’s Supplicant and then initiates the 4-way handshake, to complete key management. If IEEE 802.1X authentication fails, the AP’s Authenticator uses the MLME-DEAUTHENTICATE.request primitive to inform IEEE 802.11 of the problem.

The AP shall respond to an IEEE 802.1X authentication failure by sending the STA a Disassociation message with reason code one (1).

A STA (including an AP) shall pass IEEE 802.1X management authentication frames. Being management frames, they shall be sent in the clear.

Informative Note: There is a potential race condition with the final IEEE 802.1X message when an association begins, in that it may be sent unencrypted. Accordingly the MAC is required to pass all IEEE 802.1X messages even if keys have been configured. This sort of race condition is inherent in all key management schemes, and cannot be removed by “clever” design.

Informative Note: IEEE 802.1X uses the MLME-DEAUTHENTICATE.request primitive to inform the 802.11 MAC when authentication failed.

Informative Note: There is no requirement to disassociate with the associated AP if pre-authentication with a different AP fails.

Change text introducing section 8.4.6.1 to:

#### 8.4.6.1  Pre-authentication and key management

A STA shall not use pre-authentication except when pairwise keys are employed.

When pre-authentication is used, then

- Authentication is independent of roaming.

- the STA’s Supplicant may be authenticated with multiple APs at a time.

- the STA’s Supplicant shall manage Group keys per AP and configure the Group keys into the STA on association or re-association.

Informative Note. Pre-authentication can be useful as a performance enhancement, as Reassociation will not include the cost of a full reauthentication when it is used.

If pre-authentication is not used, the STA must make a roaming decision prior to authentication. Data transfer will halt during IEEE 802.1X authentication, key management and IEEE 802.11 association.

When pre-authentication is used, the STA’s IEEE 802.1X Supplicant must cache the PMK for some period, in case the STA associates with the AP with which the STA’s Supplicant has pre-authenticated.

Similarly, the AP’s IEEE 802.1X Authenticator must cache the PMK key for some period in case the pre-authenticated STA associates with the AP. If during authentication the AP’s Authenticator finds it has cached the PMK for the associated STA, it may respond with an immediate EAP-Success message and then initiate the 4-way handshake.

Both the Supplication and the Authenticator may delete a cached PMK if the pre-authenticated STA does not associate with the selected AP after some time interval.

Informative Note: Even if a STA has pre-authenticated, it is still possible that it may have to undergo a full IEEE 802.1X authentication, as the AP’s Authenticator may have purged its PMK due to, e.g., unavailability of resources, or slowness of the STA to authenticate, etc.

Pre-authentication can fail, and an AP’s Authenticator or STA’s Supplicant can destroy keys established by pre-authentication prior to association. If the AP’s Authenticator loses pre-authentication keys in this manner, it shall send an IEEE 802.11 Deauthentication message on receiving any encrypted packets from the station.

Pre-authentication introduces new opportunities for denial-of-service attack. To limit the efficacy of these attacks, STAs (including APs) shall rate-limit IEEE 802.1X messages. STAs shall ignore IEEE 802.1X from APs with which it is neither associated nor pre-authenticating.

Change text introducing section 8.4.7 to:

### 8.4.7  RSN authentication in an IBSS

No authentication is used with pre-shared key authentication in IBSS.

When authentication is used in an IBSS, it is driven by the STA wishing to establish communications. The Management Entity of this STA chooses a set of STAs with which it may want to authenticate, and then causes the MAC to send an IEEE 802.11 RSN Authentication messages to each targeted STA. Targeted STAs that wish to respond will proceed with the RSN authentication.and key management

The EAPOL-Key message is used to exchange information between the Supplicant and the Authenticator to negotiate a fresh pairwise temporal key. There is a single Pairwise key between the Supplicant and Authenticator produced by the 4-way handshake. The Pairwise key is used to transfer Group key updates and may be used as a Pairwise transient key.

Change text introducing  section 8.4.10.2 to:

#### 8.4.10.2 Illegal data transfer

In an RSN a STA and an AP protect all data frames transferred between them.. If the STA and AP key state gets out of synchronization the following rules apply:

- If an AP receives a unicast protected packet when it does not have keys to decapsulate, it shall send a Disassociate message to the STA and discard the data packet.

- If a non-AP STA receives a unicast protected packet when it does not have keys to decapsulate the packet, it shall discard the data packet and send a Disassociate message to the AP; if the STA wants communications to continue, it should follow the Disassociate message with an immediate associate request to the AP.

- On receiving a Disassociate or Deauthentication message, a STA shall delete the Pairwise key and, if it wants to continue communications, Reassociate to an AP of the same ESS.

Renumber 8.5.2 to 8.5.3 and add text introducing section 8.5.2:

Insert the clause:

### 8.5.2 EAPOL frame format for 802.11

IEEE 802.1X specifies encapsulations for IEEE 802.1X EAPOL frames to be carried over 802.3, 802.4 and 802.5 networks.

The following text describes the equivalent encapsulation that shall be used when carrying IEEE 802.1X EAPOL frames over 802.11. The format is derived from the IEEE 802.1X Token Ring/FFDI EAPOL frame format defined in IEEE 802.1X section 7.3. It differs from the Token Ring.FDDI format in that the SNAP header is not present, and is replaced with the management frame authentication header from which the content type can be determined.

The EAPOL frame shall be encapsulated in a management frame of type Management and subtype Authentication.

A summary of the 802.11 format of an EAPOL frame is shown below, the final 4 fields shown in the diagram are defined in IEEE 802.1X. The octets shown transferred within the PDU payload body of management frames of subtype authenticate.

|  | Octet Number |
| --- | --- |
| Authentication Algorithm Number (8.5.2.1) | 0-2 |
| Authentication transaction sequence number (8.5.2.2) | 3-4 |
| Status code (Table 14) | 5-6 |
| Protocol Version (8.5.2.3) | 7 |
| Packet Type (8.5.2.4) | 8 |
| Packet Body Length (8.5.2.5) | 9-10 |
| Packet Body (8.5.2.6) | 11-N |

#### 8.5.2.1 Authentication Algorithm Number

The authentication algorithm number is defined in 7.3.1.1. The authentication algorithm number shall be set to 2: RSN.

#### 8.5.2.2 Authentication transaction sequence number

The authentication transaction sequence number is defined in 7.3.1.2.

#### 8.5.2.3 Status Code

The status code is defined in table 14.

#### 8.5.2.1 Protocol Version

The protocol version is defined in IEEE 802.1X section 7.5.3 .

#### 8.5.2.2 Packet Type

The packet type field is defined in IEEE 802.1X section 7.5.4 .

#### 8.5.2.3 Packet Body Length

The packet type field is defined in IEEE 802.1X section 7.5.5 .

#### 8.5.2.4 Packet Body

The packet type field is defined in IEEE 802.1X section 7.5.6 .

Change text modifying sections 8.7, 8.7.1 and 8.7.2 to:

## 8.7 Tx and Rx Pseudo Code

When WEP, TSN or RSN protections are in place, received MSDUs are filtered based on the state of the aExcludeUnencrypted variable.

When RSN or TSN protections are in place, the passage of both the transmission and reception of MSDUs is enabled or disabled based upon the state of the 802.1X portSecure variable.

The following conditions apply in an RSN.

- dot11PrivacyInvoked shall be true in order for a STA to apply RSN protections.

- STAs protect all transmit MSDUs when when temporal keys are configured, and send and receives all MSDUs in the clear when temporal keys not configured.

- An AP should disassociate and/or deauthenticate a station on receiving an IEEE 802.1X authFail event for the STA.

### 8.7.1 Per MSDU Tx pseudo-code

if dot11PrivacyInvoked = FALSE then

transmit the MSDU without protections

else

if (portSecure == FALSE) then

discard the MSDU

// If we find a suitable unicast or group key for the mode we are in…

else if (MSDU has an individual RA and dot11WEPKeyMappings has an entry for that RA and dot11WEPKeyMappingsKeyBroadcast is false) or (the MSDU has a multicast RA and the network type is IBSS and the network is RSN and there is an entry in dot11KeyMappings for the TA and dot11WEPKeyMappingsKeyBroadcast is true) then

if entry has WEPOn = FALSE then

transmit the MSDU without protections

else

if that entry contains a null key then

discard the entire MSDU and generate an

MA-UNITDATA-STATUS.indication primitive to

notify the LLC that the MSDU was undeliverable due to a null key

else

// Note that it is assumed that no entry will be in the key

// mapping table of a cipher type that is unsupported.

// Key ID is set to zero here since we found a shared key

Set the KeyID subfield of the IV field to zero.

if cipher type of entry is AES-CCM

Transmit the MSDU, to be protected after fragmentation using AES-CCM and the entry’s key

else if cipher type of entry is AES-OCB.

Protect MSDU with AES-OCB cipher and the entry’s key

Transmit the protected MSDU

else if cipher type of entry is TKIP

Compute MIC using Michael algorithm and entry’s Tx MIC key.

Append MIC to MSDU

Transmit the MSDU, to be protected with TKIP and the entry’s key

else if cipher type of entry is WEP and dot11RSNEnabled = “false”.

Transmit the MSDU, to be protected with WEP and the entry’s key

end if

endif

endif

else 	// Else we didn’t find a key but we are protected, so handle the default key case or

// discard if the default key is null.

// But first, the following clause covers the case of an AP in a BSS with encryption, that accepts

// non protected STAs into the BSS and so must transmit broadcasts as plaintext.

if MSDU has a group RA and the Privacy subfield of the Capability Information field in this BSS is set to 0 then

Transmit the MSDU is without protections

else	// No key found so try default key

if  dot11WEPDefaultKeys[dot11WEPDefaultKeyID] = null then

discard the MSDU and generate an

MA-UNITDATA-STATUS.indication primitive to

notify LLC that the entire MSDU was undeliverable

due to a null key

else if  dot11WEPDefaultKeys[dot11WEPDefaultKeyID] is not null

Set the KeyID subfield of the IV field to dot11WEPDefaultKeyID.

if cipher type of dot11WEPDefaultKeys[dot11WEPDefaultKeyID]is AES-CCM

Transmit the MSDU, to be protected after fragmentation using AES-CCM and dot11WEPDefaultKeys[dot11WEPDefaultKeyID]

else if cipher type of entry is AES-OCB.

Protect MSDU with AES-OCB cipher and dot11WEPDefaultKeys[dot11WEPDefaultKeyID]

Transmit the protected MSDU

else if cipher type of entry is TKIP

Compute MIC using Michael algorithm and entry’s Tx MIC key.

Append MIC to MSDU

Transmit the MSDU, to be protected with TKIP and dot11WEPDefaultKeys[dot11WEPDefaultKeyID]

else if cipher type of entry is WEP and dot11RSNEnabled = “false”.

Transmit the MSDU, to be protected with WEP and dot11WEPDefaultKeys[dot11WEPDefaultKeyID]

end if

end if

end if

end if

### 8.7.2 Per MPDU Tx pseudo-code

if  MPDU is member of an MSDU that is to be transmitted without protections

transmit the MPDU without protections

else if MSDU that MPDU is a member of was protected using AES-OCB

Transmit the MPDU unaltered

else if MSDU that MPDU is a member of is to be protected using AES-CCM

Protect the MPDU using entry’s key and AES-CCM

Transmit the MPDU

else if MSDU that MPDU is a member of is to be protected using TKIP

Protect the MPDU using TKIP encryption

Transmit the MPDU

else if MSDU that MPDU is a member of is to be protected using WEP

Encrypt the MPDU using entry’s key and WEP

Transmit the MPDU

else

// should not arrive here

endif

### 8.7.3 Per MPDU Rx pseudo-code

if the Protected Frame subfield of the Frame Control Field is zero then // Received unencrypted frame

if MPDU has a group RA and aHaveGTK =”false” and dot11RSNEnabled = “true” then

Receive the frame unencrypted

//Unicast

else if aHavePTK = “false” and dot11RSNEnabled = “true”  then

Receive frame unencrypted

else if dot11RSNEnabled = “false” and aExcludeUnencrypted = “false”

Receive the frame without applying protections

else

discard the frame body without indication to LLC and

increment dot11WEPExcludedCount

endif

else						// Received Protected Frame

if (dot11PrivacyOptionImplemented = “true” and the MPDU has individual RA and there is an entry in dot11WEPKeyMappings matching the MPDU’s TA and dot11WEPKeyMappingsKeyBroadcast is false) or (the MPDU has a multicast RA and the network type is IBSS and the network is RSN and there is an entry in dot11KeyMappings for the TA and dot11WEPKeyMappingsKeyBroadcast is true) then

if entry has an AES-OCB key

receive the frame unaltered

else if entry has an AES-CCM key

decrypt frame using AES-CCM key

discard the frame if the integrity check fails

else if entry has a TKIP key

prepare a temporal key from the TA, TKIP key and PN

decrypt the frame using RC4

discard the frame if the ICV fails

else if entry has a WEP key

decrypt the frame using WEP decryption

discard the frame if the ICV fails and increment

dot11WEPUndecryptableCount

endif

else if dot11WEPDefaultKeys[KeyID] is null then

discard the frame body and increment dot11WEPUndecryptableCount

else if dot11WEPDefaultKeys[KeyID] is a CCM key

decrypt and authenticte MPDU using CCMP

else if dot11WEPDefaultKeys[KeyID] is a WRAP key

Receive the MPDU, since decryption will take place at MSDU level

else if dot11WEPDefaultKeys[KeyID] is a TKIP key

Decrypt the MPDU using TKIP

else if dot11WEPDefaultKeys[KeyID] is a WEP key

attempt to decrypt with dot11WEPDefaultKeys[KeyID],

incrementing dot11WEPICVErrorCount if the ICV check fails

end if

end if

### 8.7.4 Per MSDU Rx pseudo-code

if the frame was not protected

receive the frame without applying protections

else		// Have a protected MSDU

if dot11PrivacyOptionImplemented = TRUE then

if dot11WEPKeyMappings  entry has WEPOn set to FALSE then

discard the frame body and increment

dot11WEPUndecryptableCount

else if dot11WEPKeyMappings  entry contains a key that is null then

discard the frame body and increment

dot11WEPUndecryptableCount

else if dot11WEPKeyMappings  has an AES-OCB key then

Decrypt the frame using AES-OCB

Discard the frame if authentication fails

Accept the MSDU of the OCB decryption and authentication succeeds

else if dot11WEPKeyMappings  has an AES-CCM key then

Accept the MSDU since the decryption took place at the MPDU

else if dot11WEPKeyMappings  has a TKIP key then

Compute the MIC using the Michael algorithm

Compare the received MIC against the computed MIC

discard the frame if the MIC fails and invoke countermeasures if appropriate,

otherwise accept the MSDU

else if dot11WEPKeyMappings  has a WEP key then

Accept the MSDU since the decryption took place at the MPDU

endif

else // Cannot decrypt payload, so discard it.

discard the frame body

endif

endif

Add text modifying section 10.3.4 and its subsections:

Change the text of clause  “10.3.4 Authenticate” to:

### 10.3.4 Authenticate

This mechanism supports the process of establishing an authentication relationship with a peer MAC entity.

#### 10.3.4.1 MLME-AUTHENTICATE.request

##### 10.3.4.1.1 Function

This primitive requests authentication with a specified peer MAC entity.

##### 10.3.4.1.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-AUTHENTICATE.request 	(

PeerSTAAddress,

AuthenticationType,

AuthenticateFailureTimeout,

RSNAuthenticationMessage

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| PeerSTAAddress | MACAddress | Any vaid individual MAC address | Specifies the address of the peer MAC entity with which to perform the authentication process |
| Authentication Type | Enumeration | OPEN_SYSTEM, SHARED_KEY, RSN | Specifies the type of authentication algorithm to use during the authentication process |
| AuthenticationFailure-Timout | Integer |  1 | Specifies a time limit (in TU) after which the authentication procedure will be terminated |
| RSNAuthenticationMessage | As defined in frame format | As defined in frame format | Contains the message generated by the 802.1X entity of the requester of the authentication service |

##### 10.3.4.1.3 When generated

This primitive is generated by the SME for a STA to establish authentication with a specified peer MAC entity in order to permit Class 2 frames to be exchanged between the two STAs. During the authentication procedure, the SME may generate additional MLME-AUTHENTICATE.request primitives.

##### 10.3.4.1.4 Effect of receipt

This primitive initiates an authentication procedure. The MLME subsequently issues a MLME-AUTHENTI-CATE.confirm that reflects the results.

#### 10.3.4.2 MLME-AUTHENTICATE.confirm

##### 10.3.4.2.1 Function

This primitive reports the results of an authentication attempt with a specified peer MAC entity.

10.3.4.2.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-AUTHENTICATE.confirm	(

PeerSTAAddress,

AuthenticationType,

ResultCode,

RSNAuthenticationMessage

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| PeerSTAAddress | MACAddress | Any valid individual MAC address | Specifies the address of the peer MAC entity with which the authentication process was attempted. This value must match the peer-STAAddress parameter specified in the corresponding MLME-AUTHENTICATE.request |
| Authentication Type | Enumeration | OPEN_SYSTEM, SHARED_KEY, RSN | Specifies the type of authentication algorithm to use during the authentication process |
| ResultCode | Enumeration | SUCCESS, INVALID_ PARAMETERS, TIMEOUT, TOO_MANY_ SIMULTANEOUS_ REQUESTS, REFUSED | Specifies a time limit (in TU) after which the authentication procedure will be terminated |
| RSNAuthenticationMessage | As defined in frame format | As defined in frame format | Contains the message generated by the 802.1X entity of the responder of the authentication service |

##### 10.3.4.2.3 When generated

This primitive is generated by the MLME as a result of an MLME-AUTHENTICATE.request to authenticate with a specified peer MAC entity.

##### 10.3.4.2.4 Effect of receipt

The SME is notified of the results of the authentication procedure.

#### 10.3.4.3 MLME-AUTHENTICATE.indication

##### 10.3.4.3.1 Function

This primitive reports the establishment of an authentication relationship with a specific peer MAC entity. With RSN authentication, this primitive reports the request of an authentication relationship with a specific peer MAC entity."

##### 10.3.4.3.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-AUTHENTICATE.indication	(

PeerSTAAddress,

AuthenticationType,

RSNAuthenticationMessage,

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| PeerSTAAddress | MACAddress | Any valid individual MAC address | Specifies the address of the peer MAC entity with which the authentication relationship was established |
| Authentication Type | Enumeration | OPEN_SYSTEM, SHARED_KEY, RSN | Specifies the type of authentication algorithm to use during the authentication process |
| RSNAuthenticationMessage | As defined in frame format | As defined in frame format | Contains the message generated by the 802.1X entity of the requester of the authentication service |

##### 10.3.4.3.3 When generated

This primitive is generated by the MLME as a result of the establishment of an authentication relationship with a specific peer MAC entity RSN authentication, this primitive is generated by the MLME as a result of the receipt of a request of an authentication relationship with a specific peer MAC entity that initiated the authentication procedure.

##### 10.3.4.3.4 Effect of receipt

The SME is notified of the establishment of the authentication relationship. With RSN authentication, the SME is notified of the request of an authentication relationship with a specific peer MAC entity.

#### 10.3.4.4  MLME-AUTHENTICATE.response

##### 10.3.4.4.1 Function

This primitive responds to the request of an authentication relationship with a specific peer MAC entity.

##### 10.3.4.3.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-AUTHENTICATE.indication	(

PeerSTAAddress,

AuthenticationType,

ResultCode,

RSNAuthenticationMessage

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| PeerSTAAddress | MACAddress | Any valid individual MAC address | Specifies the address of the peer MAC entity with which the authentication process was attempted. This value must match the peer-STAAddress parameter specified in the corresponding MLME-AUTHENTICATE.request |
| Authentication Type | Enumeration | OPEN_SYSTEM, SHARED_KEY, RSN | Specifies the type of authentication algorithm to use during the authentication process |
| ResultCode | Enumeration | SUCCESS, INVALID_ PARAMETERS, TIMEOUT, TOO_MANY_ SIMULTANEOUS_ REQUESTS, REFUSED | Specifies a time limit (in TU) after which the authentication procedure will be terminated |
| RSNAuthenticationMessage | As defined in frame format | As defined in frame format | Contains the message generated by the 802.1X entity of the responder of the authentication service |

##### 10.3.4.4.3 When generated

This primitive is generated by the SME on behalf of the 802.1X of the responder of the authentication service as a result of an MLME-AUTHENTICATE.request to authenticate with a specified peer MAC entity.

##### 10.3.4.4.4 Effect of receipt

The MLME is notified of the results of the authentication procedure.

Delete the text modifying section 11.3.1, 11.3.2 and 11.3.4: