IEEE P802.11
Wireless LANs

- HCF Mandatory for AP

Date:	November 12, 2002

Authors:		Keith Amann
    SPECTRALINK

Mathilde Benveniste
AVAYA

Peter Johansson
CONGRUENT SOFTWARE

Matthew Sherman
AT&T

- Abstract

This submission contains a proposal for PICS. In this proposal for PICS, HCF is mandatory in the AP.

# Annex A – PICS

### A.4.3 IUT configuration

| Item | IUT configuration | References | Status | Support |
| --- | --- | --- | --- | --- |
| CF12 | QoS supported | 9.10, 9.11 | O | Yes, No |
| CF13 | Enhanced Distributed Coordination Function (EDCF) | 9.10.1 | CF12:O.1 | Yes, No |
| CF14 | Hybrid Coordination Function (HCF) | 9.10.2 | (CF1 AND CF12):M CF12:O.1 | Yes, No |

### A.4.x QoS Base Functionality

| Item | Protocol Capability | References | Status | Support |
| --- | --- | --- | --- | --- |
| QB1 | QoS frame format decode | 7.1.3.1, 7.1.3.5 | CF12:M | Yes, No |
| QB2 | Per-TID duplicate detection | 7.1.3.5, 9.10.1.6 | CF12:M | Yes, No |
| QB3 | Decode of no-ack policy in QoS data frames | 7.1.3.5.2 | CF12:M | Yes, No |
| QB4 | Group acknowledgements | 9.11 | CF12:O | Yes, No |
| QB5 | Automatic Power Save Delivery | 7.3.2.18, 7.4.1.6, 11.2.3 | (CF1 AND CF12):M (CF2 AND CF12):O | Yes, No |
| QB6 | Direct Link Protocol | 7.3.2.20, 7.4.2 | (CF1 AND CF12):M (CF2 AND CF12):O | Yes, No |

### A.4.y QoS EDCF

| Item | Protocol Capability | References | Status | Support |
| --- | --- | --- | --- | --- |
| QD1 | Support for four transmit queues with a separate channel access entity associated with each | 9.10.1.1 | CF13:M | Yes, No |
| QD2 | Per-channel access function differentiated channel access | 9.10.1.2, 9.10.1.3, 9.10.1.5 | CF13:M | Yes, No |
| QD3 | Continuation TXOP support | 9.10.1.4 | CF13:M | Yes, No |
| QD4 | Maintenance of within-queue ordering, exhaustive retransmission when sending non-QoS data frames | 9.10.1.6 | CF13:M | Yes, No |
| QD5 | Distributed admission control | 9.10.1.7 | CF13:M | Yes, No |

### A.4.z QoS HCF

| Item | Protocol Capability | References | Status | Support |
| --- | --- | --- | --- | --- |
| QC1 | Support for TSPEC | 7.3.2.13, 7.3.2.15, 7.3.2.19 7.4.1, 7.5, | CF14:M | Yes, No |
| QC2 | Support for HCF access rules | 9.1.3.2, 9.10.2 | CF14:M | Yes, No |
| QC3 | Support for HCF Frame Exchange Sequences | 9.10.3 | CF14:M | Yes, No |