IEEE P802.11
Wireless LANs

Replacement Text for Minimum TSPEC Parameters

Date:	November 13, 2002

Author:	Keith Amann, SpectraLink
	Javier del Prado, Philips

Abstract

This document presents a more editorially correct version of the normative text from document 02/705r0.

Instruct the editor to replace the normative text originally adopted from document 02/705r0, and inserted into clause 9.10.2.4 with the following:

A minimum set of TSPEC parameters shall be specified during the TSPEC negotiation. The specification of a minimum set of parameters is required so that the scheduler can determine a schedule for an admitted stream. These parameters are Mean Data Rate, Nominal MSDU Size and at least one of Maximum Service Interval and Delay Bound. All these parameters, shall be non-zero in the AddTS QoS Action Response frame when a stream is admitted. If any of these parameters have a value of ‘0’ in the AddTS QoS Action Request frame, the HC may reject the stream or admit it with an alternative  minimum set of parameters. If the HC admits the stream with the alternative set of TSPEC parameters they will be indicated to the WSTA through the AddTS QoS action response frame. If both Maximum Service Interval and Delay Bound are specified the HC may use only the Maximum Service Interval. If any other parameter is specified in the TSPEC element, the scheduler may use it when calculating the schedule for the stream. The HC may also use the User Priority value in the TS Info field for admission control or scheduling purposes, however this decision is outside the scope of the standard. The mandatory set of parameters can be set by any higher layer entity or may be generated autonomously by the MAC.