IEEE P802.11
Wireless LANs

Proposed Normative Text for Simplified APSD

Date:	January 14, 2003

Authors:	Mathilde Benveniste
	Avaya Labs - Research
	233 Mt. Airy Road, Basking Ridge, NJ USA
	Phone: 973-761-6105
	e-Mail: benveniste@ieee.org

Keith Amann
	SpectraLink Corporation
	5755 Central Ave., Boulder, CO USA
	Phone: 303-440-5330
	e-Mail: kamann@spectralink.com

Bob Meier
	Cisco Systems, Inc.
	320 Springfield Drive, Ste. 350, Akron, OH USA
	Phone: 330-664-7301
	e-Mail: rmeier@cisco.com

Abstract

This document contains the material proposed to GTE for inclusion in the draft in the form of insertions into and replacements for material in IEEE Std 802.11-1999, as updated by IEEE Std 802.11a-1999, IEEE Std 802.11b-1999 (already present in TGE Draft 4.0).

Editorial notes appear in bold italic Times New Roman font, informative notes appear in normal Arial font, and normative text appears in normal Times New Roman font.  Open issues are highlighted using red text in normal Arial font, and begin with "OPEN ISSUE:".  Added text appears in blue with a dotted underline.  Deleted text appears in red with a strikethrough line.

Modify text in section 7.3.2.18 as follows:

The element information field is defined in Figure 42.13.

| Octets: 1 | 1 | 1 | 1 | 8 | 4 |
| --- | --- | --- | --- | --- | --- |
| Element ID (TBD) | Length (2) | Wakeup Beacon Period | Beacon Offset | Wakeup Time Period | Start Time |

Figure 42.13 – Automatic Power-Save Delivery element format

…

The wakeup time period is the time interval, expressed as units representing 32 TSF clock ticks, during which the non-AP QSTA is requesting the QAP to buffer MSDU and management frames before releasing the frames for delivery using a prioritized, or parameterized, delivery mechanism.

The start time indicates the time, expressed as units representing 32 TSF clock ticks, when a non-AP QSTA in APSD mode may first wake up to receive downlink frames buffered at the AP.  The station may wake up at a subsequent time when

(TSF- Start Time)  mod Wakeup Time Period = 0

A non-AP QSTA may select only one of the two pairs of fields, either Wakeup Beacon Period and Beacon Offset (i.e., beacon-based parameters), or Wakeup Time Period and Start Time (i.e., time-based parameters), to specify its intended wakeup schedule.  The fields not selected must be set to 0.

Update section number 7.3.2.19 to read 7.3.2.20

Insert new section after 7.3.2.18 as follows:

#### Automatic Power-Save Delivery Schedule

The APSD schedule element is transmitted by the HC to a non-AP QSTA in response to a (re)association request or action frame containing the APSD element.  It is used to confirm or modify the wake up schedule for the non-AP QSTA in APSD mode by specifying a modified Beacon Offset or Start time.   The element information field is shown in Figure 42.15.

| Octets: 1 | 1 | 1 | 8 |
| --- | --- | --- | --- |
| Element ID (TBD) | Length (2) | Beacon Offset | Start Time |

Figure 42.15 – Automatic Power-Save Delivery Schedule element format

Modify section 11.2.3 as follows:

Non-AP QSTAs operating in a QBSS wishing to utilize the automatic power-save delivery mechanism shall inform the AP of this fact by using an automatic power-save delivery information element, signaled through a (re)association or action management frame. The QAP shall not arbitrarily transmit MSDUs to non-AP QSTAs operating in an automatic power-save delivery (APSD) mode, but shall buffer MSDUs and only transmit them at beacon intervals that correspond to the specified wakeup period.  The QAP has the option to schedule the wakeup time by indicating a beacon offset or a start time in the APSD schedule element that is different than the beacon offset in the APSD element.

A non-AP QSTA can signal its desire to utilize APSD as the power-save mode delivery method through the use of a Automatic Power-Save Delivery element contained in a (re)association request or action frame.  non-AP QSTAs use the power-save mode bit in the frame control field of a frame to indicate whether it is in active or power-save mode. The QAP shall acknowledge receipt of the automatic power-save delivery information element by returning the automatic power-save delivery schedule element. The QAP uses the power-save delivery mechanism currently in effect for a non-AP QSTA to deliver frames to the non-AP QSTA when it is operating in power-save mode.

Modify section 11.2.3.1 as follows:

QAPs shall maintain an automatic power-save delivery (APSD) status for each currently associated non-AP QSTA that indicates whether the non-AP QSTA is presently in APSD mode and the wakeup period for the non-AP QSTA.  A QAP shall, based on the APSD mode of the non-AP QSTA, temporarily buffer the MSDU or management frames destined to the non-AP QSTA.  MSDUs and management frames received for non-AP QSTAs not operating in APSD mode shall follow the appropriate frame delivery rules as related to their respective power-saving mode.

a)	MSDUs, or management frames, destined for APSD capable non-AP QSTAs shall be temporarily buffered in the QAP when requested by the non-AP QSTA.  The algorithm to manage this buffering is beyond the scope of this standard, with the exception that the QAP must preserve the order of arrival of frames on a per priority basis, including during transitions between active and APSD modes.

b)	MSDUs, or management frames, destined for APSD capable non-AP QSTAs not operating in APSD shall be transmitted according to the rules in this standard that correspond to the power-save state of the non-AP QSTA.

- If a non-AP QSTA has selected the beacon-based parameters for its wakeup schedule, at every beacon interval, the QAP shall assemble the partial virtual bitmap containing the buffer status per destination for non-AP QSTAs in the APSD mode, and shall send this out in the TIM field of the beacon.  At every beacon interval the QAP shall determine, based on the wakeup period specified by the non-AP QSTA, during (re)association or through a management action frame, whether this beacon corresponds to an APSD beacon for each non-AP QSTA.  If this is determined to be an APSD beacon, the QAP shall transmit all frames destined for the non-AP QSTA. If a non-AP QSTA has selected the time-based parameters for its wakeup schedule, a QAP shall transmit frames destined for the non-AP QSTAs in APSD mode according to their wakeup schedule.

- The More Data field of each directed data or management frame, except the last frame, shall be set to indicate the presence of multiple frames destined for the non-AP QSTA.  If necessary the QAP can generate an extra (QoS)-Null frame with the more data field cleared. When the QAP has transmitted a directed frame with “more data” zero during the APSD beacon interval wakeup period, it shall not transmit any more frames using this mechanism until the next APSD beacon wakeup period.

e)	A QAP shall have an aging function to delete pending traffic when it is buffered for an excessive time period.  Aging may be based on the listen interval specified by the non-AP QSTA in the (re)association request.

- Whenever a QAP is informed that a APSD capable non-AP QSTA changes it’s delivery mode to not be APSD mode, then the QAP shall send buffered MSDUs and management frames (if any exist) to that non-AP QSTA according to the rules corresponding to the current power-save state of the non-AP QSTA.

#### 11.2.3.2 Receive operation for non-AP QSTAs in the APSD mode

A non-AP QSTA operating in APSD mode shall operate as follows to receive an MSDU or management frame from the QAP if a non-AP QSTA has selected the beacon-based parameters for its wakeup schedule:

a)	A non-AP QSTA shall wake up early enough to receive the next beacon that corresponds to the scheduled wakeup period specified in the APSD schedule element, which is sent in response to a (re)association or action request frame the non-AP QSTA used to initiate APSD mode with the QAP.

b)	When a non-AP QSTA detects that the bit corresponding to its AID is set in the TIM, the non-AP QSTA shall remain awake until it receives a directed MSDU or management frame with the More Data field cleared, or it receives a beacon with its TIM bit cleared.

c)	A non-AP QSTA that misses it scheduled wakeup beacon shall remain awake until it receives a beacon with its TIM bit cleared, or a data frame with the More Data field set off.

d)	A non-AP QSTA operating in APSD mode may send a PS-Poll frame at any time to retrieve a single frame buffered at the QAP.

- When a non-AP QSTA transitions from active mode to APSD mode it shall remain awake until it receives a beacon with its TIM bit cleared.

A non-AP QSTA operating in APSD mode shall operate as follows to receive an MSDU or management frame from the QAP if a non-AP QSTA has selected the time-based parameters for its wakeup schedule:

- A non-AP QSTA shall wake up at the start of a scheduled period as specified in the APSD schedule element, which is sent in response to a (re)association or action request frame the non-AP QSTA used to initiate APSD mode with the QAP.

- A non-AP QSTA in APSD mode shall send a frame to the QAP, if there is a frame buffered, or send a QoS null otherwise.  When a QAP receives a frame from a non-AP QSTA in APSD mode for which it has no data buffered, it shall send an Ack with a More Data bit set to 0.