IEEE P802.11
Wireless LANs

Alternate Text for TGi CCMP Clause

Date:	March 12, 2003

Author:	Paul A. Lambert
	Airgo Networks, Inc.
	900 Arastradero Road, Palo Alto, CA, 94304
	Phone: 650-475-2006
	e-Mail: Paul@AirgoNetworks.Com

Abstract

This document provides replacement text for the CCM processing in clause 8.3.3 of the TGi specification draft 3.1.  The changes to the document embodied in this submittal resolve numerous ballot comments.

Many thanks to the many contributors to this submission that include: Russ Housley, Onno Letanche, Dorothy Stanley and Greg Chesson.

Instruct the editor to replace the normative text of TGi D30 clause 8.3.4 with the following:

Add the following text to clause 2:

Whiting, et al, “Counter with CBC-MAC (CCM)”, RFC tbdSoon, 2003 Soon, currently draft-ietf-ipsec-ciph-aes-ccm-01.txt

Replace clause 8.3.3 with the following text:

### 8.3.3  The Counter-Mode/CBC-MAC protocol (CCMP)

This clause specifies the Counter-Mode/CBC-MAC Protocol (CCMP), which provides confidentiality, authentication, integrity, and replay protection. CCMP is mandatory for RSN compliance.

#### 8.3.3.1 CCMP overview

CCMP employs the AES encryption algorithm using the CCM mode of operation. The CCM mode combines Counter Mode (CTR) for confidentiality and Cipher Block Chaining Message Authentication Code (CBC-MAC) for authentication and integrity.  CCM protects the integrity of both the MPDU data field and selected portions of the IEEE 802.11 MAC header.

The Advanced Encryption Algorithm (AES) algorithm is defined in FIPS PUB 197.  All AES processing used within CCMP uses AES with a 128 bit key and a 128 bit block size.

The CCM encryption is defined in ‘draft-housley-ccm-mode-02.txt’. <note soon to be RFC xxxx>   .  CCM is a generic mode that can be used with any block oriented encryption algorithm.  CCMP must use the AES algorithm with with a 128 bit key and 128 bit block size.  CCM provides other parameters (K, M and L) that must have the values: K=16, M=8 and L=2.

CCM requires a fresh temporal key (TK) for every session. CCM also requires a unique nonce value for each frame protected by a given TK, and CCMP uses a 48-bit packet number (PN) for this purpose. Reuse of a packet number (PN) with the same TK voids all security guarantees.

Annex F provides a reference implementation and test vectors for CCM mode.

#### 8.3.3.2  CCMP MPDU format

Figure 1 depicts the MPDU when using CCMP.

Figure 1—Expanded CCMP MPDU

CCMP processing expands the original MPDU size by 16 octets, 8 octets for the CCMP Header and 8 octets for the Message Integrity Code (MIC).  The CCMP Header is constructed from the PN, ExtIV and KeyID fields.

Bit 5 of the KeyID octet signals that the security processing the CCMP Header extends the MPDU by 8 octets (rather than 4 octets used by WEP). The Extended IV bit (bit 5) is always set for CCMP.

The reserved bits shall be set to zero (0) and shall be ignored on reception.

#### 8.3.3.3  CCMP encapsulation

Figure 2 depicts the CCMP encapsulation process. CCMP encrypts the payload of a plaintext MPDU and encapsulates the resulting ciphertext using the following steps:

Figure 2—CCMP encapsulation block diagram

- Increment the Packet Number (PN), to obtain a fresh PN for each MPDU.  The Packet Number shall never repeat for the same temporal key (TK).

- The fields in the MAC header are used to construct the Additional Authentication Data (AAD) for the CCM mode. The CCM algorithm provides integrity protection for the fields included in the AAD. MAC Header fields that may change when retransmitted are muted by either being set to zero or being excluded from the AAD.

- Construct the CCM Nonce block from the PN, A2 and the Priority of the MPDU.

- Encode the new PN and the KeyId into the 8 octet CCMP Header.

- CCM originator processing uses the temporal key (TK), AAD, Nonce and MPDU data to form the ciphertext and MIC.

- The Encrypted MPDU is formed by concatenating the original MAC Header, the CCMP header, the Encrypted Data and the MIC, as described in clause 8.3.3.2.

The CCM reference describes the processing of the Key, None, AAD and Data to produce the encrypted output. The following clauses describe the details of the creation of the AAD and Nonce from the MPDU and the associated MPDU specific processing.

##### 8.3.3.3.1  PN Processing

The PN is incremented for each MPDU.  The PN shall never repeat for a series of encrypted MPDUs using the same temporal key (TK).

##### 8.3.3.3.2  Construct AAD

The AAD is constructed from the MAC Header. The AAD does not include the header Duration field, because the Duration field value can change due to normal IEEE 802.11 operation. For similar reasons, the computation masks bits in the Frame Control field to zero.  AAD construction is performed as follows:

- FC – MPDU Frame Control field, with:

  - Subtype b4 b5 b6 bits masked to zero;

  - Retry bit masked to zero;

  - PwrMgt bit masked to zero;

  - MoreData bit masked to zero; and

  - The Protected bit shall always be set to 1.

- A1 – MDPU Address 1.

- A2 – MPDU Address 2.

- A3 – MPDU Address 3.

- A4 – MPDU Address, if present.

- SC – MPDU Sequence Control., with the sequence number field masked to zero.  The Fragment Number bits are not modified

- QC – The Quality of Service Control, if present (see sectin 8.3.3.6).

The format of the AAD is shown in Figure 3.  The length of the AAD is 22 octets in length when there is no A4 field and is 26 octets long  when the MPDU includes A4.

Figure 3—AAD Construction

##### 8.3.3.3.3  Construct CCM Nonce

The Nonce field occupies is 13 octets, and its structure is shown in Figure 4.

The Nonce has an internal structure QoS-TC || A2 || PN, where

- This Priority octet is reserved for the QoS traffic class and shall be set to the fixed value 0 (0x00 hex) when there is no QoS traffic class field.  When there is a QoS traffic class, it is encode in bits 4, 5, 6, and 7 for the Priority octet.  Bits 0, 1, 2, and 3 are reserved, and they are always set to zero.

- MPDU address A2 occupies octets 2 through 7. This shall be encoded with the octets ordered with A2 octet 0 at octet index 2 and A2 octet 5 at octet index 7.

- PN occupies octets 9 through 13. The octets of PN shall be ordered such that PN0 is at octet index 13 and PN5 is at octet index 8.

Figure 4—Nonce Construction

##### 8.3.3.3.4  Construct CCMP header

The format of the 8 octet CCMP header is given in clause 8.3.3.2.  The header encodes the PN and KeyId values used to encrypt the MPDU.

##### 8.3.3.3.5  CCM originator processing

CCM is a generic authenticate-and-encrypt block cipher mode, and in this specification, CCM is used with the AES block cipher. CCM has two parameters, and CCMP uses the following values for the CCM parameters:

- M = 8; indicating that the MIC is 8 octets.

- L = 2; indicating that the length field is 2 octets, which is sufficient to hold the length of the largest possible 802.11 MAC frame in octets.

There are four inputs to CCM originator processing:

- Key;  the key used for CCM is the TK.

- Nonce; the nonce is 13 octets, and it is constructed as described in clause 8.3.3.3.3.

- Frame body; the frame body of the MPDU.

- AAD; additional authenticated data (AAD) that is constructed from the MAC header as described in clause 8.3.3.3.2.

The CCM originator processing provides authentication and integrity of the frame body and the AAD as well as confidentiality of the frame body.  The output from the CCM originator processing consists of the encrypted data and 8 additional octets of encrypted MIC (see Figure 2).

##### 8.3.3.3.6 CCMP Encrypted MPDU

The CCMP encrypted MPDU is formed by concatenating the MAC Header, CCMP Header, Encrypted Frame Body and the MIC as described in section 8.3.3.2.

#### 8.3.3.4  CCMP decapsulation

Figure 5 depicts the CCMP decapsulation process. CCMP decrypts the payload of a ciphertext MPDU and decapsulates a plaintext MPDU using the following steps:

Figure 5—CCMP decapsulation block diagram

- The Encrypted MPDU is parsed to construct the AAD and Nonce values.

- The AAD is formed from the MAC Header of the Encrypted MPDU.

- The Nonce value is constructed from A2, the PN, and Priority (when QC field is available, see section 8.3.3.6).

- The MIC is extracted for use in the CCM integrity checking.

- The CCM recipient processing uses the temporal key (TK), AAD, Nonce, MIC and MPDU ciphertext data to recover the MPDU plaintext data as well as check the integrity of the AAD and MPDU plaintext data.

- The received MAC Header and the MPDU plaintext data from the CCM recipient processing may be concatenated to form a Plaintext MPDU.

- The decryption processing prevents replay of MPDUs by validating that the PN in the MPDU is greater than the received PN maintained for the session.

The following clauses describe the details of this processing.

##### 8.3.3.4.1  Construct AAD

The CCM Additional Authenticated Data (AAD) field is constructed from the MAC Header as described in clause 8.3.3.3.2.

##### 8.3.3.4.2  Construct CCM Nonce

The CCM Nonce is constructed as described in clause 8.3.3.3.3.

##### 8.3.3.4.3  CCM recipient processing

CCM recipient processing must use the same parameters as CCM originator processing.  That is, AES is used as the block cipher, M = 8, and L = 2.

There are four inputs to CCM recipient processing:

- Key;  the key used for CCM is the TK.

- Nonce; the nonce is 13 octets, and it is constructed as described in clause 8.3.3.4.3.

- Encrypted frame body; the encrypted frame body from the received MPDU. The Encrypted frame body includes an 8 octet encrypted MIC.

- AAD; additional authenticated data (AAD) is the canonical MAC header as described in clause 8.3.3.4.1.

The CCM recipient processing checks the authentication and integrity of the frame body and the AAD as well as decrypting the frame body.  The plaintext is returned only if the MIC check is successful.

There is one output from error free CCM recipient processing:

- Frame body; the plaintext frame body, which is exactly the same size as the encrypted frame body.

When CCM recipient processing detects an error, the frame body is not returned.  Errors in processing generate an error indication.

##### 8.3.3.4.4  Plaintext MPDU

The CCM Decryption process succeeds if the calculated MIC matches the MIC value received in the Encrypted MPDU.  The original MAC Header is then concatenated with the plaintext data resulting from the successful CCM recipient processing to create the Plaintext MPDU.

##### 8.3.3.4.5  Replay Check

This section describes the usage of the PN for replay detection. The PN is extracted from the CCMP Header as described in clause 8.3.3.2. Then, the following processing rules are used to detect replay:

- The Packet Number (PN) values shall correspond to MPDUs.

- The PN (a 48-bit counter) shall be selected from a single pool by each transmitter for each temporal key (TK). Each transmitter a separate counter for each TK.

- The PN shall be implemented as a 48-bit monotonically incrementing non-negative integer, initialized to zero when the corresponding TK is initialized or refreshed.

- A receiver shall maintain a separate set of PN replay counters for each MAC address from which it receives CCMP traffic. The receiver initializes the replay counter whenever it resets the TK for a peer.

The receiver shall discard MSDUs whose constituent MPDU PN values are not sequential. A receiver shall discard any MPDU that is received out of order and shall increment the value of dot11RSNStatsCCMPReplays for this key.

#### 8.3.3.5  CCMP state

CCMP privacy uses a MIB array called the dot11CcmpKeyMappings. This supports zero, one, or two entries for each MAC address pair with which the STA maintains secure associations. The size of the dot11CcmpKeyMappings array is implementation-specific. A global MIB variable dot11CcmpKeyMappingLength indicates the number of entries in the array.

Each entry of the dot11CcmpKeyMappings groups together the following state:

- A dot11CcmpReceiveAddress and a dot11CcmpTransmitAddress, indicating that this entry applies to all MPDUs being sent between this pair of addresses.

- A dot11CcmpKeyID, indicating the KeyID into which this entry maps.

- A 128-bit key called the dot11CcmpTemporalKey, referred to informally as the temporal key. This is the TK1 subfield portion of the Pairwise Transient Key as defined in 8.5.1.2, or the TK1 subfield of the Group Transient Key as defined in 8.5.1.3. This key is often called the temporal key.

- A set of 48-bit counters called the dot11CcmpTrafficClassNPacketNumber, for constructing the next initial block. N ranges from 0 to 15, with one traffic class defined for each QoS service class. When Priority is not used, only dot11CcmpTrafficClass0PacketNumber is used.

- A set of 48-bit replay windows called the dott11CcmpTrafficClassNReplayWindow, for detecting replays. N ranges from 0 to15. When Priority is not used, only dot11CcmpTrafficClasse0ReplayWindow is used.

- A boolean flag called dot11CcmpEnableTransmit, to indicate when the temporal key can be used for transmitting MPDUs.

- A boolean flag called dot11CcmpEnableReceive, to indicate when the temporal key can be used for receiving MPDUs.

- A 32-bit counter dot11CcmpFormatErrors, to indicate the number of MPDUs received with an invalid format, initialized to zero.

- A 32-bit counter dot11CcmpReplays, to indicate the number of received unicast MPDUs discarded by the replay mechanism, initialized to zero.

- A 32-bit counter dot11CcmpDecryptErrors, to indicate the number of received MPDUs discarded by the CCMP decryption mechanism, initialized to zero.

- A 48-bit counter dot11CcmpRecvdMPDU, to track the total number of protected MPDUs received.

Informative Note: As an optimization, implementations may compute and maintain the AES-CCM key schedule rather than maintain the temporal key.

#### 8.3.3.6  CCMP processing with QoS Control (Informative)

The CCMP processing protects fields in the MPDU header.  This section describes the additional processing requirements for CCMP when the MPDU contains the QoS Control (QC) field)

##### 8.3.3.6.1  CCM Nonce with QC-TID

The QoS-TCID occupies bits 0 to bits 3 of the first octet of the Nonce. This field is reserved for the QoS traffic class and shall be set to the fixed value 0 (0x00 hex) when there is no QC field.

##### 8.3.3.6.2  CCM AAD with QC field

The CCM AAD is constructed directly from the MAC Header.  When MPDUs contain the QC field the QC field shall be included in the AAD with all bits of the QC except the QC-TID set to zero.

Figure 6—AAD Construction

Note that both A4 and QC are optional fields.

##### 8.3.3.6.3 Replay detection

The recipient shall maintain a separate replay counter for each IEEE 802.11 Traffic Class, and shall use the PN recovered from a received frame to detect replayed frames. A replayed frame occurs when the PN extracted from a received frame is repeated or not greater than the current Traffic Class replay counter value for the frame’s traffic class. The replay counter accommodates frames that may be delayed due to traffic class priority values.