IEEE P802.11
Wireless LANs

Title

Date:	January 16, 2003

Author:	Paul A. Lambert
	Airgo Networks, Inc.
	900 Arastradero Road, Palo Alto, CA, 94304
	Phone: 650-475-2006
	e-Mail: paul@airgonetworks.com

Abstract

This document provides test vectors for Tgi.

Instruct the editor to replace test vectors in the Annex TGi D30 clause with the following:

++++++++++++++++++++ WEP Test Vectors ++++++++++++++++++++

==== WEP test mpdu   1 ====

-- MPDU Fields

Version  = 0

Type     = 2   SubType  = 0  Data

ToDS     = 0   FromDS   = 0

MoreFrag = 0   Retry    = 1

PwrMgt   = 0   moreData = 0

Encrypt  = 1

Order    = 0

Duration = 11459

A1 = 0f-d2-e1-28-a5-7c    DA

A2 = 50-30-f1-84-44-08    SA

A3 = ab-ae-a5-b8-fc-ba    BSSID

seqNum = 824  fraqNum = 0

Algorithm = WEP

KeyId = 0

Key =

c9 7c 1f 67 ce 37 11 85  51 4a 8a 19 f2

PN    = 0x66e70c  (decimal=6743820)

Header =

08 48 c3 2c 0f d2 e1 28  a5 7c 50 30 f1 84 44 08

ab ae a5 b8 fc ba 80 33

Enc Header =

0c e7 66 00

Data  =

f8 ba 1a 55 d0 2f 85 ae  96 7b b6 2f b6 cd a8 eb

7e 78 a0 50

CRC =

f6 69 9c 43

-- Encrypted MPDU with FCS

08 48 c3 2c 0f d2 e1 28  a5 7c 50 30 f1 84 44 08

ab ae a5 b8 fc ba 80 33  0c e7 66 00 a8 07 d1 97

3b 91 a2 07 53 73 2b e0  d4 91 e4 77 f5 b8 15 5e

fc 70 e0 03 10 40 d8 8a

==== WEP test mpdu   2 ====

-- MPDU Fields

Version  = 0

Type     = 2   SubType  = 3  Data+CF-Ack+CF-Poll

ToDS     = 0   FromDS   = 0

MoreFrag = 0   Retry    = 0

PwrMgt   = 0   moreData = 0

Encrypt  = 1

Order    = 1

Duration = 20842

A1 = ea-10-0c-84-68-50    DA

A2 = ee-c1-76-2c-88-de    SA

A3 = af-2e-e9-f4-6a-07    BSSID

seqNum = 3278  fraqNum = 0

Algorithm = WEP

KeyId = 2

Key =

8f 7a 05 3f a5 77 a5 59  75 29 27 20 97

PN    = 0x2a97ea  (decimal=2791402)

Header =

38 c0 6a 51 ea 10 0c 84  68 50 ee c1 76 2c 88 de

af 2e e9 f4 6a 07 e0 cc

Enc Header =

ea 97 2a 80

Data  =

83 a0 63 4b 5e d7 62 7e  b9 df 22 5e 05 74 03 42

de 19 41 17

CRC =

13 5a 89 c3

-- Encrypted MPDU with FCS

38 c0 6a 51 ea 10 0c 84  68 50 ee c1 76 2c 88 de

af 2e e9 f4 6a 07 e0 cc  ea 97 2a 80 ba 26 c1 ba

7f 0b 5c de 45 61 8e b8  12 59 e3 1e da 8d cd 64

19 68 a6 04 17 f0 a8 fe

==== WEP test mpdu   3 ====

-- MPDU Fields

Version  = 0

Type     = 2   SubType  = 11  QC&Data+CF-Ack+CF-Poll

ToDS     = 0   FromDS   = 0

MoreFrag = 0   Retry    = 1

PwrMgt   = 0   moreData = 0

Encrypt  = 1

Order    = 1

Duration = 25052

A1 = d9-57-7d-f7-63-c8    DA

A2 = b6-a8-8a-df-36-91    SA

A3 = dc-4a-8b-ca-94-dd    BSSID

seqNum = 2086  fraqNum = 0

QC = 0x8520

QoS-TID  = 0  QoS-FEC = 0

QoS-AckP = 1  QoS-TXOP/QL = 0x85

Algorithm = WEP

KeyId = 2

Key =

40 cf b7 a6 2e 88 01 3b  d6 d3 af fc c1

PN    = 0x63a5ae  (decimal=6530478)

Header =

b8 c8 dc 61 d9 57 7d f7  63 c8 b6 a8 8a df 36 91

dc 4a 8b ca 94 dd 60 82  20 85

Enc Header =

ae a5 63 80

Data  =

2c 1b d0 36 83 1c 95 49  6c 5f 4d bf 3d 55 9e 72

de 80 2a 18

CRC =

73 87 94 5e

-- Encrypted MPDU with FCS

b8 c8 dc 61 d9 57 7d f7  63 c8 b6 a8 8a df 36 91

dc 4a 8b ca 94 dd 60 82  20 85 ae a5 63 80 c6 30

4c 69 66 0d 9b 38 60 c3  77 d0 47 19 07 c6 ed f6

ca fe 1d f7 8b e7 ca c4  e4 32

==== WEP test mpdu   4 ====

-- MPDU Fields

Version  = 0

Type     = 2   SubType  = 10  QC&Data+CF-Poll

ToDS     = 0   FromDS   = 1

MoreFrag = 0   Retry    = 1

PwrMgt   = 0   moreData = 0

Encrypt  = 1

Order    = 1

Duration = 4410

A1 = 71-2a-9d-df-11-db    DA

A2 = 8e-f8-22-73-47-01    BSSID

A3 = 59-14-0d-d6-46-a2    SA

seqNum = 764  fraqNum = 0

QC = 0xa567

QoS-TID  = 7  QoS-FEC = 0

QoS-AckP = 3  QoS-TXOP/QL = 0xa5

Algorithm = WEP

KeyId = 0

Key =

8c 89 a2 eb c9 6c 76 02  70 7f cf 24 b3

PN    = 0x4a0fe3  (decimal=4853731)

Header =

a8 ca 3a 11 71 2a 9d df  11 db 8e f8 22 73 47 01

59 14 0d d6 46 a2 c0 2f  67 a5

Enc Header =

e3 0f 4a 00

Data  =

4f ad 2b 1c 29 0f a5 eb  d8 72 fb c3 f3 a0 74 89

8f 8b 2f bb

CRC =

65 fd 1b 6b

-- Encrypted MPDU with FCS

a8 ca 3a 11 71 2a 9d df  11 db 8e f8 22 73 47 01

59 14 0d d6 46 a2 c0 2f  67 a5 e3 0f 4a 00 67 36

9a 86 3c cb 0a ce 8e 41  3c 83 0b 6c f6 ab b5 22

ec 4c 3e 86 ec c1 12 e9  30 60

==== WEP test mpdu   5 ====

-- MPDU Fields

Version  = 0

Type     = 2   SubType  = 8  QC&Data

ToDS     = 0   FromDS   = 1

MoreFrag = 0   Retry    = 1

PwrMgt   = 1   moreData = 0

Encrypt  = 1

Order    = 1

Duration = 16664

A1 = 45-de-c6-9a-74-80    DA

A2 = f3-51-94-6b-c9-6b    BSSID

A3 = e2-76-fb-e6-c1-27    SA

seqNum = 3880  fraqNum = 0

QC = 0x194b

QoS-TID  = 11  QoS-FEC = 0

QoS-AckP = 2  QoS-TXOP/QL = 0x19

Algorithm = WEP

KeyId = 2

Key =

a5 74 d5 14 3b b2 5e fd  de ff 30 12 2f

PN    = 0x2c0e37  (decimal=2887223)

Header =

88 da 18 41 45 de c6 9a  74 80 f3 51 94 6b c9 6b

e2 76 fb e6 c1 27 80 f2  4b 19

Enc Header =

37 0e 2c 80

Data  =

28 96 9b 95 4f 26 3a 80  18 a9 ef 70 a8 b0 51 46

24 81 92 2e

CRC =

4a a1 e5 c4

-- Encrypted MPDU with FCS

88 da 18 41 45 de c6 9a  74 80 f3 51 94 6b c9 6b

e2 76 fb e6 c1 27 80 f2  4b 19 37 0e 2c 80 75 25

1b 00 71 bc 0f 9c 70 76  e2 80 23 15 f2 ad 2b 8d

aa f5 72 bb 66 b8 54 3f  47 8a

==== WEP test mpdu   6 ====

-- MPDU Fields

Version  = 0

Type     = 2   SubType  = 8  QC&Data

ToDS     = 0   FromDS   = 1

MoreFrag = 0   Retry    = 0

PwrMgt   = 1   moreData = 0

Encrypt  = 1

Order    = 0

Duration = 8161

A1 = 5a-f2-84-30-fd-ab    DA

A2 = bf-f9-43-b9-f9-a6    BSSID

A3 = ab-1d-98-c7-fe-73    SA

seqNum = 1813  fraqNum = 0

QC = 0x6a3d

QoS-TID  = 13  QoS-FEC = 1

QoS-AckP = 1  QoS-TXOP/QL = 0x6a

Algorithm = WEP

KeyId = 1

Key =

f7 1e ea 4e 1f 58 80 4b  97 17 23 0a d0

PN    = 0x248989  (decimal=2394505)

Header =

88 52 e1 1f 5a f2 84 30  fd ab bf f9 43 b9 f9 a6

ab 1d 98 c7 fe 73 50 71  3d 6a

Enc Header =

89 89 24 40

Data  =

ab fd a2 2d 3a 0b fc 9c  c1 fc 07 93 63 c2 fc a1

43 e6 eb 1d

CRC =

0b ac 74 cc

-- Encrypted MPDU with FCS

88 52 e1 1f 5a f2 84 30  fd ab bf f9 43 b9 f9 a6

ab 1d 98 c7 fe 73 50 71  3d 6a 89 89 24 40 4e 1d

73 e2 0c 56 76 00 fd 08  37 ce 31 cb 2c c8 91 1c

d6 bc 4d 34 0b 04 2f b5  94 75

==== WEP test mpdu   7 ====

-- MPDU Fields

Version  = 0

Type     = 2   SubType  = 1  Data+CF-Ack

ToDS     = 1   FromDS   = 0

MoreFrag = 0   Retry    = 1

PwrMgt   = 1   moreData = 1

Encrypt  = 1

Order    = 0

Duration = 18049

A1 = 9b-50-f4-fd-56-f6    BSSID

A2 = ef-ec-95-20-16-91    SA

A3 = 83-57-0c-4c-cd-ee    DA

seqNum = 2562  fraqNum = 0

Algorithm = WEP

KeyId = 3

Key =

1b db 34 98 0e 03 81 24  a1 db 1a 89 2b

PN    = 0x63e723  (decimal=6547235)

Header =

18 79 81 46 9b 50 f4 fd  56 f6 ef ec 95 20 16 91

83 57 0c 4c cd ee 20 a0

Enc Header =

23 e7 63 c0

Data  =

98 be ca 86 f4 b3 8d a2  0c fd f2 47 24 c5 8e b8

35 66 53 39

CRC =

ea e1 41 f1

-- Encrypted MPDU with FCS

18 79 81 46 9b 50 f4 fd  56 f6 ef ec 95 20 16 91

83 57 0c 4c cd ee 20 a0  23 e7 63 c0 14 d0 ae f8

41 e0 b4 f5 56 22 a8 a2  2b 69 ef 1f f3 64 4a 2a

37 5f c4 d8 b8 14 a6 c7

==== WEP test mpdu   8 ====

-- MPDU Fields

Version  = 0

Type     = 2   SubType  = 11  QC&Data+CF-Ack+CF-Poll

ToDS     = 1   FromDS   = 0

MoreFrag = 0   Retry    = 1

PwrMgt   = 1   moreData = 0

Encrypt  = 1

Order    = 1

Duration = 29260

A1 = 55-2d-5f-72-bb-70    BSSID

A2 = ca-3f-3a-ae-60-c4    SA

A3 = 8b-a9-b5-f8-2c-2f    DA

seqNum = 3765  fraqNum = 0

QC = 0x552a

QoS-TID  = 10  QoS-FEC = 0

QoS-AckP = 1  QoS-TXOP/QL = 0x55

Algorithm = WEP

KeyId = 2

Key =

6e ac 1b f5 4b d5 4e db  23 21 75 43 03

PN    = 0x6eccdd  (decimal=7261405)

Header =

b8 d9 4c 72 55 2d 5f 72  bb 70 ca 3f 3a ae 60 c4

8b a9 b5 f8 2c 2f 50 eb  2a 55

Enc Header =

dd cc 6e 80

Data  =

57 cb 5c 0e 5f cd 88 5e  9a 42 39 e9 b9 ca d6 0d

64 37 59 79

CRC =

03 76 69 b2

-- Encrypted MPDU with FCS

b8 d9 4c 72 55 2d 5f 72  bb 70 ca 3f 3a ae 60 c4

8b a9 b5 f8 2c 2f 50 eb  2a 55 dd cc 6e 80 da 24

9d b2 a9 f8 fa f5 c3 4a  0e 8c cb 0e fa cc bd e4

32 dc 9a df 40 7d 12 03  d0 69

==== WEP test mpdu   9 ====

-- MPDU Fields

Version  = 0

Type     = 2   SubType  = 10  QC&Data+CF-Poll

ToDS     = 1   FromDS   = 0

MoreFrag = 0   Retry    = 1

PwrMgt   = 1   moreData = 1

Encrypt  = 1

Order    = 1

Duration = 17096

A1 = 9a-fb-79-8b-8a-4b    BSSID

A2 = a4-ad-6d-31-99-85    SA

A3 = bc-42-9e-8f-0a-fa    DA

seqNum = 2015  fraqNum = 0

QC = 0x804a

QoS-TID  = 10  QoS-FEC = 0

QoS-AckP = 2  QoS-TXOP/QL = 0x80

Algorithm = WEP

KeyId = 3

Key =

49 4b 50 1e 19 46 75 97  1a 48 d0 8c 5b

PN    = 0x2437b3  (decimal=2373555)

Header =

a8 f9 c8 42 9a fb 79 8b  8a 4b a4 ad 6d 31 99 85

bc 42 9e 8f 0a fa f0 7d  4a 80

Enc Header =

b3 37 24 c0

Data  =

25 a9 8f 9c 1b d9 c9 3c  f3 83 ab 9d 98 15 2d 76

cb 4a 32 c6

CRC =

15 83 0c 37

-- Encrypted MPDU with FCS

a8 f9 c8 42 9a fb 79 8b  8a 4b a4 ad 6d 31 99 85

bc 42 9e 8f 0a fa f0 7d  4a 80 b3 37 24 c0 be cf

b2 fc c5 de 0e c8 6b c5  69 16 c7 d3 7f 86 ad 1f

0c a0 b2 d7 27 bc 70 76  cc 97

==== WEP test mpdu  10 ====

-- MPDU Fields

Version  = 0

Type     = 2   SubType  = 1  Data+CF-Ack

ToDS     = 1   FromDS   = 1

MoreFrag = 0   Retry    = 1

PwrMgt   = 0   moreData = 0

Encrypt  = 1

Order    = 0

Duration = 25568

A1 = 40-ec-29-fa-75-9b    RA

A2 = 53-f8-69-fe-27-9a    TA

A3 = f0-f9-f8-a6-54-16    DA

A4 = 52-bf-d2-70-3d-24    SA

seqNum = 3640  fraqNum = 0

Algorithm = WEP

KeyId = 0

Key =

48 9e 49 bc 3c fe 3f ce  38 95 82 0e 87

PN    = 0x65fea9  (decimal=6684329)

Header =

18 4b e0 63 40 ec 29 fa  75 9b 53 f8 69 fe 27 9a

f0 f9 f8 a6 54 16 80 e3  52 bf d2 70 3d 24

Enc Header =

a9 fe 65 00

Data  =

7f 91 f2 47 2d 7a 12 1c  9c dd 4b 6c 90 80 67 5a

10 20 aa 00

CRC =

4c bd 16 a6

-- Encrypted MPDU with FCS

18 4b e0 63 40 ec 29 fa  75 9b 53 f8 69 fe 27 9a

f0 f9 f8 a6 54 16 80 e3  52 bf d2 70 3d 24 a9 fe

65 00 b2 17 a5 91 45 4e  0e ec 44 c8 39 3a 07 21

6b 4c cf c6 63 6e fb 43  cd 2d 48 dd 4a d0

==== WEP test mpdu  11 ====

-- MPDU Fields

Version  = 0

Type     = 2   SubType  = 11  QC&Data+CF-Ack+CF-Poll

ToDS     = 1   FromDS   = 1

MoreFrag = 0   Retry    = 1

PwrMgt   = 1   moreData = 0

Encrypt  = 1

Order    = 1

Duration = 21610

A1 = 29-8c-0b-aa-91-90    RA

A2 = 34-7c-eb-9a-ab-ff    TA

A3 = d8-3d-48-86-e5-c2    DA

A4 = e2-9d-52-4a-e1-96    SA

seqNum = 1342  fraqNum = 0

QC = 0x623d

QoS-TID  = 13  QoS-FEC = 1

QoS-AckP = 1  QoS-TXOP/QL = 0x62

Algorithm = WEP

KeyId = 3

Key =

02 be 5c 45 45 67 2a 07  e4 e3 14 d7 0f

PN    = 0x6e55d4  (decimal=7230932)

Header =

b8 db 6a 54 29 8c 0b aa  91 90 34 7c eb 9a ab ff

d8 3d 48 86 e5 c2 e0 53  e2 9d 52 4a e1 96 3d 62

Enc Header =

d4 55 6e c0

Data  =

f9 a8 12 e4 a2 8a f7 f3  71 4d 4b f6 62 2e 59 32

f2 18 45 09

CRC =

89 08 36 2f

-- Encrypted MPDU with FCS

b8 db 6a 54 29 8c 0b aa  91 90 34 7c eb 9a ab ff

d8 3d 48 86 e5 c2 e0 53  e2 9d 52 4a e1 96 3d 62

d4 55 6e c0 d7 7d ec 78  90 63 08 1e f4 cd 8b ac

16 7c 2a 33 8a fc 48 f1  d7 53 b0 3e 33 97 17 18

==== WEP test mpdu  12 ====

-- MPDU Fields

Version  = 0

Type     = 2   SubType  = 1  Data+CF-Ack

ToDS     = 1   FromDS   = 1

MoreFrag = 0   Retry    = 1

PwrMgt   = 0   moreData = 1

Encrypt  = 1

Order    = 0

Duration = 10192

A1 = aa-28-8b-84-35-bc    RA

A2 = d8-ac-5a-7e-c4-44    TA

A3 = e8-b4-62-50-53-8b    DA

A4 = e8-14-02-c2-ee-11    SA

seqNum = 1576  fraqNum = 0

Algorithm = WEP

KeyId = 3

Key =

77 07 7e d7 94 53 e4 a1  8d 60 43 8c c6

PN    = 0x67fd8e  (decimal=6815118)

Header =

18 6b d0 27 aa 28 8b 84  35 bc d8 ac 5a 7e c4 44

e8 b4 62 50 53 8b 80 62  e8 14 02 c2 ee 11

Enc Header =

8e fd 67 c0

Data  =

43 19 81 a2 33 6d 02 f8  cb 84 48 d5 42 89 16 be

95 29 35 37

CRC =

bd 2e 8f 00

-- Encrypted MPDU with FCS

18 6b d0 27 aa 28 8b 84  35 bc d8 ac 5a 7e c4 44

e8 b4 62 50 53 8b 80 62  e8 14 02 c2 ee 11 8e fd

67 c0 e9 66 db 8e e6 09  57 9c b0 fb e4 79 ba 79

e6 61 2d 63 96 43 70 21  0d 3b 64 c5 11 fe

++++++++++++++++++++ TKIP Test Vectors ++++++++++++++++++++

==== TKIP test mpdu   1 ====

-- MPDU Fields

Version  = 0

Type     = 2   SubType  = 0  Data

ToDS     = 0   FromDS   = 0

MoreFrag = 0   Retry    = 1

PwrMgt   = 0   moreData = 0

Encrypt  = 1

Order    = 0

Duration = 11459

A1 = 0f-d2-e1-28-a5-7c    DA

A2 = 50-30-f1-84-44-08    SA

A3 = ab-ae-a5-b8-fc-ba    BSSID

seqNum = 824  fraqNum = 0

Algorithm = TKIP

KeyId = 0

Key =

a9 90 6d c8 3e 78 92 3f  86 04 e9 9e f6 cd ba bb

micKey =

53 50 45 33 6f fb 85 0f

PN    = 0xb5039776e70c  (decimal=199027030681356)

Header =

08 48 c3 2c 0f d2 e1 28  a5 7c 50 30 f1 84 44 08

ab ae a5 b8 fc ba 80 33

Enc Header =

0c 2c e7 20 76 97 03 b5

Data  =

f8 ba 1a 55 d0 2f 85 ae  96 7b b6 2f b6 cd a8 eb

7e 78 a0 50

MIC =

b9 7f e8 3d 2a 8a 08 da

CRC =

6b d7 32 ca

-- Encrypted MPDU with FCS

08 48 c3 2c 0f d2 e1 28  a5 7c 50 30 f1 84 44 08

ab ae a5 b8 fc ba 80 33  0c 2c e7 20 76 97 03 b5

4c 0a 77 0b c1 94 b6 9e  de 6c dc e3 63 0f 7b 83

1a 25 42 6a 20 9f bb 63  5a 4b b5 b9 f1 ff 77 ad

9e e2 d9 fe

==== TKIP test mpdu   2 ====

-- MPDU Fields

Version  = 0

Type     = 2   SubType  = 3  Data+CF-Ack+CF-Poll

ToDS     = 0   FromDS   = 0

MoreFrag = 0   Retry    = 0

PwrMgt   = 0   moreData = 0

Encrypt  = 1

Order    = 1

Duration = 20842

A1 = ea-10-0c-84-68-50    DA

A2 = ee-c1-76-2c-88-de    SA

A3 = af-2e-e9-f4-6a-07    BSSID

seqNum = 3278  fraqNum = 0

Algorithm = TKIP

KeyId = 2

Key =

05 09 43 ba fd fd 5f a8  b1 12 fe 57 09 cd 5f e9

micKey =

61 62 d9 13 0e b6 9a a0

PN    = 0x31f3cbba97ea  (decimal=54923164817386)

Header =

38 c0 6a 51 ea 10 0c 84  68 50 ee c1 76 2c 88 de

af 2e e9 f4 6a 07 e0 cc

Enc Header =

ea 6a 97 20 ba cb f3 31

Data  =

83 a0 63 4b 5e d7 62 7e  b9 df 22 5e 05 74 03 42

de 19 41 17

MIC =

8e 32 70 32 e9 bb a6 7a

CRC =

f9 d1 c6 04

-- Encrypted MPDU with FCS

38 c0 6a 51 ea 10 0c 84  68 50 ee c1 76 2c 88 de

af 2e e9 f4 6a 07 e0 cc  ea 6a 97 20 ba cb f3 31

99 40 62 b4 5d b3 29 a1  d1 88 d2 f8 df 55 5c 0a

6a ee 0a ba 95 89 09 b5  cb 08 b7 2c df 8b 53 93

4c 0d 96 64

==== TKIP test mpdu   3 ====

-- MPDU Fields

Version  = 0

Type     = 2   SubType  = 11  QC&Data+CF-Ack+CF-Poll

ToDS     = 0   FromDS   = 0

MoreFrag = 0   Retry    = 1

PwrMgt   = 0   moreData = 0

Encrypt  = 1

Order    = 1

Duration = 25052

A1 = d9-57-7d-f7-63-c8    DA

A2 = b6-a8-8a-df-36-91    SA

A3 = dc-4a-8b-ca-94-dd    BSSID

seqNum = 2086  fraqNum = 0

QC = 0x8520

QoS-TID  = 0  QoS-FEC = 0

QoS-AckP = 1  QoS-TXOP/QL = 0x85

Algorithm = TKIP

KeyId = 2

Key =

7c 8f 8c 02 b5 f6 ee d8  2c d9 cf f2 a8 3a f7 b0

micKey =

25 73 6a f6 93 71 bb ba

PN    = 0x2fdca0f3a5ae  (decimal=52624639632814)

Header =

b8 c8 dc 61 d9 57 7d f7  63 c8 b6 a8 8a df 36 91

dc 4a 8b ca 94 dd 60 82  20 85

Enc Header =

ae 2e a5 20 f3 a0 dc 2f

Data  =

2c 1b d0 36 83 1c 95 49  6c 5f 4d bf 3d 55 9e 72

de 80 2a 18

MIC =

9e 02 6a 55 bd 1f e6 98

CRC =

e2 5e 03 f1

-- Encrypted MPDU with FCS

b8 c8 dc 61 d9 57 7d f7  63 c8 b6 a8 8a df 36 91

dc 4a 8b ca 94 dd 60 82  20 85 ae 2e a5 20 f3 a0

dc 2f 08 4c 0d eb fa b5  b8 b4 20 61 0f 26 8e 39

2c c9 79 67 67 73 91 bb  90 54 f3 23 d1 f3 bb 7f

df 92 2e d5 a8 96

==== TKIP test mpdu   4 ====

-- MPDU Fields

Version  = 0

Type     = 2   SubType  = 10  QC&Data+CF-Poll

ToDS     = 0   FromDS   = 1

MoreFrag = 0   Retry    = 1

PwrMgt   = 0   moreData = 0

Encrypt  = 1

Order    = 1

Duration = 4410

A1 = 71-2a-9d-df-11-db    DA

A2 = 8e-f8-22-73-47-01    BSSID

A3 = 59-14-0d-d6-46-a2    SA

seqNum = 764  fraqNum = 0

QC = 0xa567

QoS-TID  = 7  QoS-FEC = 0

QoS-AckP = 3  QoS-TXOP/QL = 0xa5

Algorithm = TKIP

KeyId = 0

Key =

04 94 7a 1a 76 78 31 0f  f2 4e 06 d7 de 7a 6a cf

micKey =

8f 07 9f ce 30 12 2e 92

PN    = 0xf670a55a0fe3  (decimal=270963670912995)

Header =

a8 ca 3a 11 71 2a 9d df  11 db 8e f8 22 73 47 01

59 14 0d d6 46 a2 c0 2f  67 a5

Enc Header =

e3 63 0f 20 5a a5 70 f6

Data  =

4f ad 2b 1c 29 0f a5 eb  d8 72 fb c3 f3 a0 74 89

8f 8b 2f bb

MIC =

0e fd ec dd 2d 36 34 78

CRC =

a9 53 f9 e8

-- Encrypted MPDU with FCS

a8 ca 3a 11 71 2a 9d df  11 db 8e f8 22 73 47 01

59 14 0d d6 46 a2 c0 2f  67 a5 e3 63 0f 20 5a a5

70 f6 af 24 70 29 9f d2  83 68 aa 07 72 75 e9 9d

7b 2d ae ec 40 69 9c d6  ce f8 6b 33 f2 c5 dd 7f

fc fa 80 9f bf 2d

==== TKIP test mpdu   5 ====

-- MPDU Fields

Version  = 0

Type     = 2   SubType  = 8  QC&Data

ToDS     = 0   FromDS   = 1

MoreFrag = 0   Retry    = 1

PwrMgt   = 1   moreData = 0

Encrypt  = 1

Order    = 1

Duration = 16664

A1 = 45-de-c6-9a-74-80    DA

A2 = f3-51-94-6b-c9-6b    BSSID

A3 = e2-76-fb-e6-c1-27    SA

seqNum = 3880  fraqNum = 0

QC = 0x194b

QoS-TID  = 11  QoS-FEC = 0

QoS-AckP = 2  QoS-TXOP/QL = 0x19

Algorithm = TKIP

KeyId = 2

Key =

95 a7 05 69 b4 a8 4d 86  bb 79 e4 2c 91 48 54 57

micKey =

73 b5 89 6e 0a 44 9d 90

PN    = 0xa7ffe03c0e37  (decimal=184717420531255)

Header =

88 da 18 41 45 de c6 9a  74 80 f3 51 94 6b c9 6b

e2 76 fb e6 c1 27 80 f2  4b 19

Enc Header =

37 37 0e 20 3c e0 ff a7

Data  =

28 96 9b 95 4f 26 3a 80  18 a9 ef 70 a8 b0 51 46

24 81 92 2e

MIC =

5d 21 87 44 43 eb 73 ca

CRC =

63 da 25 3c

-- Encrypted MPDU with FCS

88 da 18 41 45 de c6 9a  74 80 f3 51 94 6b c9 6b

e2 76 fb e6 c1 27 80 f2  4b 19 37 37 0e 20 3c e0

ff a7 12 2f 49 95 76 38  53 86 fe 8e 83 ef 36 8f

f4 19 c1 de 3c 15 19 de  34 28 45 f8 22 d3 34 52

6b f8 a5 73 4f 2f

==== TKIP test mpdu   6 ====

-- MPDU Fields

Version  = 0

Type     = 2   SubType  = 8  QC&Data

ToDS     = 0   FromDS   = 1

MoreFrag = 0   Retry    = 0

PwrMgt   = 1   moreData = 0

Encrypt  = 1

Order    = 0

Duration = 8161

A1 = 5a-f2-84-30-fd-ab    DA

A2 = bf-f9-43-b9-f9-a6    BSSID

A3 = ab-1d-98-c7-fe-73    SA

seqNum = 1813  fraqNum = 0

QC = 0x6a3d

QoS-TID  = 13  QoS-FEC = 1

QoS-AckP = 1  QoS-TXOP/QL = 0x6a

Algorithm = TKIP

KeyId = 1

Key =

63 16 4a 48 59 32 cc 15  8e a6 f8 4c 3c 20 3b af

micKey =

95 e5 ce c5 98 22 a9 c7

PN    = 0x6b81eca48989  (decimal=118205765159305)

Header =

88 52 e1 1f 5a f2 84 30  fd ab bf f9 43 b9 f9 a6

ab 1d 98 c7 fe 73 50 71  3d 6a

Enc Header =

89 29 89 20 a4 ec 81 6b

Data  =

ab fd a2 2d 3a 0b fc 9c  c1 fc 07 93 63 c2 fc a1

43 e6 eb 1d

MIC =

77 df 76 bd 4d 4d 6c 3c

CRC =

2b 3f 3c 3d

-- Encrypted MPDU with FCS

88 52 e1 1f 5a f2 84 30  fd ab bf f9 43 b9 f9 a6

ab 1d 98 c7 fe 73 50 71  3d 6a 89 29 89 20 a4 ec

81 6b 4a dc 5d 34 5a 1b  08 c8 51 86 1d 01 2b 66

c1 79 9c 02 58 50 ff 6e  4c 6f d0 2b 37 1b 4c 15

97 f4 37 05 9e 3f

==== TKIP test mpdu   7 ====

-- MPDU Fields

Version  = 0

Type     = 2   SubType  = 1  Data+CF-Ack

ToDS     = 1   FromDS   = 0

MoreFrag = 0   Retry    = 1

PwrMgt   = 1   moreData = 1

Encrypt  = 1

Order    = 0

Duration = 18049

A1 = 9b-50-f4-fd-56-f6    BSSID

A2 = ef-ec-95-20-16-91    SA

A3 = 83-57-0c-4c-cd-ee    DA

seqNum = 2562  fraqNum = 0

Algorithm = TKIP

KeyId = 3

Key =

c8 68 a7 3c fc 28 28 39  dc fa 13 0f f9 90 2d 15

micKey =

31 5c c9 f7 6a bc 27 0e

PN    = 0x5eec4073e723  (decimal=104368786630435)

Header =

18 79 81 46 9b 50 f4 fd  56 f6 ef ec 95 20 16 91

83 57 0c 4c cd ee 20 a0

Enc Header =

23 23 e7 20 73 40 ec 5e

Data  =

98 be ca 86 f4 b3 8d a2  0c fd f2 47 24 c5 8e b8

35 66 53 39

MIC =

d3 a9 6e d0 51 52 6f d8

CRC =

e1 a2 fb f4

-- Encrypted MPDU with FCS

18 79 81 46 9b 50 f4 fd  56 f6 ef ec 95 20 16 91

83 57 0c 4c cd ee 20 a0  23 23 e7 20 73 40 ec 5e

6c 1a 9a de c1 76 ce cc  10 38 55 3f 99 76 8e 90

86 bb 63 c3 18 ee 7d 4c  f8 ac 0e df ec f0 16 06

78 bf 83 07

==== TKIP test mpdu   8 ====

-- MPDU Fields

Version  = 0

Type     = 2   SubType  = 11  QC&Data+CF-Ack+CF-Poll

ToDS     = 1   FromDS   = 0

MoreFrag = 0   Retry    = 1

PwrMgt   = 1   moreData = 0

Encrypt  = 1

Order    = 1

Duration = 29260

A1 = 55-2d-5f-72-bb-70    BSSID

A2 = ca-3f-3a-ae-60-c4    SA

A3 = 8b-a9-b5-f8-2c-2f    DA

seqNum = 3765  fraqNum = 0

QC = 0x552a

QoS-TID  = 10  QoS-FEC = 0

QoS-AckP = 1  QoS-TXOP/QL = 0x55

Algorithm = TKIP

KeyId = 2

Key =

49 ca d9 e0 59 b4 43 d2  c6 04 c6 da c7 26 49 61

micKey =

8c 4a 01 4f c2 05 07 de

PN    = 0xcefd996eccdd  (decimal=227588596223197)

Header =

b8 d9 4c 72 55 2d 5f 72  bb 70 ca 3f 3a ae 60 c4

8b a9 b5 f8 2c 2f 50 eb  2a 55

Enc Header =

dd 7d cc 20 6e 99 fd ce

Data  =

57 cb 5c 0e 5f cd 88 5e  9a 42 39 e9 b9 ca d6 0d

64 37 59 79

MIC =

30 59 77 8b 89 28 b7 a2

CRC =

cc 6f 2b f7

-- Encrypted MPDU with FCS

b8 d9 4c 72 55 2d 5f 72  bb 70 ca 3f 3a ae 60 c4

8b a9 b5 f8 2c 2f 50 eb  2a 55 dd 7d cc 20 6e 99

fd ce ab 92 74 19 bd 01  13 88 fb 6d 69 04 87 e8

54 32 c7 b2 36 ff 12 77  99 e1 bb b3 2c a6 82 67

d3 cd 3b db fc ab

==== TKIP test mpdu   9 ====

-- MPDU Fields

Version  = 0

Type     = 2   SubType  = 10  QC&Data+CF-Poll

ToDS     = 1   FromDS   = 0

MoreFrag = 0   Retry    = 1

PwrMgt   = 1   moreData = 1

Encrypt  = 1

Order    = 1

Duration = 17096

A1 = 9a-fb-79-8b-8a-4b    BSSID

A2 = a4-ad-6d-31-99-85    SA

A3 = bc-42-9e-8f-0a-fa    DA

seqNum = 2015  fraqNum = 0

QC = 0x804a

QoS-TID  = 10  QoS-FEC = 0

QoS-AckP = 2  QoS-TXOP/QL = 0x80

Algorithm = TKIP

KeyId = 3

Key =

5a 13 db ca e7 d0 8b e0  a9 9e 96 84 00 58 a8 f4

micKey =

d1 02 e1 db cc be 86 2b

PN    = 0xba82e93437b3  (decimal=205071421028275)

Header =

a8 f9 c8 42 9a fb 79 8b  8a 4b a4 ad 6d 31 99 85

bc 42 9e 8f 0a fa f0 7d  4a 80

Enc Header =

b3 33 37 20 34 e9 82 ba

Data  =

25 a9 8f 9c 1b d9 c9 3c  f3 83 ab 9d 98 15 2d 76

cb 4a 32 c6

MIC =

74 01 6b d7 61 f6 6c 56

CRC =

c4 33 5c 22

-- Encrypted MPDU with FCS

a8 f9 c8 42 9a fb 79 8b  8a 4b a4 ad 6d 31 99 85

bc 42 9e 8f 0a fa f0 7d  4a 80 b3 33 37 20 34 e9

82 ba c2 85 5d 50 4e 63  aa 95 d3 59 a5 60 f6 ff

1e fe d7 7d c4 18 14 6d  10 87 52 fd 9c 77 d3 61

18 64 af 63 39 38

==== TKIP test mpdu  10 ====

-- MPDU Fields

Version  = 0

Type     = 2   SubType  = 1  Data+CF-Ack

ToDS     = 1   FromDS   = 1

MoreFrag = 0   Retry    = 1

PwrMgt   = 0   moreData = 0

Encrypt  = 1

Order    = 0

Duration = 25568

A1 = 40-ec-29-fa-75-9b    RA

A2 = 53-f8-69-fe-27-9a    TA

A3 = f0-f9-f8-a6-54-16    DA

A4 = 52-bf-d2-70-3d-24    SA

seqNum = 3640  fraqNum = 0

Algorithm = TKIP

KeyId = 0

Key =

c7 8e 92 cf 74 65 ac bb  f5 cb 77 c0 0e 8b b1 9f

micKey =

e9 94 c0 82 63 fb 82 2c

PN    = 0xcf1d3e75fea9  (decimal=227724508921513)

Header =

18 4b e0 63 40 ec 29 fa  75 9b 53 f8 69 fe 27 9a

f0 f9 f8 a6 54 16 80 e3  52 bf d2 70 3d 24

Enc Header =

a9 29 fe 20 75 3e 1d cf

Data  =

7f 91 f2 47 2d 7a 12 1c  9c dd 4b 6c 90 80 67 5a

10 20 aa 00

MIC =

4e be c5 3f 1b 7d a3 e4

CRC =

bf 0b 2f a7

-- Encrypted MPDU with FCS

18 4b e0 63 40 ec 29 fa  75 9b 53 f8 69 fe 27 9a

f0 f9 f8 a6 54 16 80 e3  52 bf d2 70 3d 24 a9 29

fe 20 75 3e 1d cf c8 43  44 af 67 30 d9 68 a1 32

99 f1 c6 40 ac ed 15 be  d1 d7 9d 1b 22 40 c5 43

29 b4 5f 00 79 37 62 14  00 f2

==== TKIP test mpdu  11 ====

-- MPDU Fields

Version  = 0

Type     = 2   SubType  = 11  QC&Data+CF-Ack+CF-Poll

ToDS     = 1   FromDS   = 1

MoreFrag = 0   Retry    = 1

PwrMgt   = 1   moreData = 0

Encrypt  = 1

Order    = 1

Duration = 21610

A1 = 29-8c-0b-aa-91-90    RA

A2 = 34-7c-eb-9a-ab-ff    TA

A3 = d8-3d-48-86-e5-c2    DA

A4 = e2-9d-52-4a-e1-96    SA

seqNum = 1342  fraqNum = 0

QC = 0x623d

QoS-TID  = 13  QoS-FEC = 1

QoS-AckP = 1  QoS-TXOP/QL = 0x62

Algorithm = TKIP

KeyId = 3

Key =

70 af e9 fe 69 46 34 83  a2 f4 b1 30 71 04 ed 9a

micKey =

dd b8 de 81 4f 8b 2a c1

PN    = 0xd2d6596e55d4  (decimal=231818065237460)

Header =

b8 db 6a 54 29 8c 0b aa  91 90 34 7c eb 9a ab ff

d8 3d 48 86 e5 c2 e0 53  e2 9d 52 4a e1 96 3d 62

Enc Header =

d4 74 55 20 6e 59 d6 d2

Data  =

f9 a8 12 e4 a2 8a f7 f3  71 4d 4b f6 62 2e 59 32

f2 18 45 09

MIC =

15 9c a4 a5 61 d2 b6 f6

CRC =

ac 1c a4 46

-- Encrypted MPDU with FCS

b8 db 6a 54 29 8c 0b aa  91 90 34 7c eb 9a ab ff

d8 3d 48 86 e5 c2 e0 53  e2 9d 52 4a e1 96 3d 62

d4 74 55 20 6e 59 d6 d2  8b e0 6e 7c e5 63 75 49

c7 d9 02 76 61 8c 42 4a  23 92 ff 23 59 85 23 c2

6c e8 53 7e 08 22 77 77  9a b0 64 66

==== TKIP test mpdu  12 ====

-- MPDU Fields

Version  = 0

Type     = 2   SubType  = 1  Data+CF-Ack

ToDS     = 1   FromDS   = 1

MoreFrag = 0   Retry    = 1

PwrMgt   = 0   moreData = 1

Encrypt  = 1

Order    = 0

Duration = 10192

A1 = aa-28-8b-84-35-bc    RA

A2 = d8-ac-5a-7e-c4-44    TA

A3 = e8-b4-62-50-53-8b    DA

A4 = e8-14-02-c2-ee-11    SA

seqNum = 1576  fraqNum = 0

Algorithm = TKIP

KeyId = 3

Key =

00 0f 48 7f 6e 7a 46 8d  b0 cb 33 6d 18 1a 37 44

micKey =

12 d0 a4 1d 0d 30 1c 2a

PN    = 0x50b01e77fd8e  (decimal=88717355646350)

Header =

18 6b d0 27 aa 28 8b 84  35 bc d8 ac 5a 7e c4 44

e8 b4 62 50 53 8b 80 62  e8 14 02 c2 ee 11

Enc Header =

8e 2e fd 20 77 1e b0 50

Data  =

43 19 81 a2 33 6d 02 f8  cb 84 48 d5 42 89 16 be

95 29 35 37

MIC =

11 4c 8d f2 bf 0a 30 fd

CRC =

18 b1 19 af

-- Encrypted MPDU with FCS

18 6b d0 27 aa 28 8b 84  35 bc d8 ac 5a 7e c4 44

e8 b4 62 50 53 8b 80 62  e8 14 02 c2 ee 11 8e 2e

fd 20 77 1e b0 50 9b 30  5e 47 7b 7b ab 40 96 7e

f8 e1 17 1c 45 e8 a1 c2  05 40 8c 27 59 79 1f e1

f4 55 84 9d e7 e1 14 fd  54 09

++++++++++++++++++++ CCMP Test Vectors ++++++++++++++++++++

==== CCMP test mpdu   1 ====

-- MPDU Fields

Version  = 0

Type     = 2   SubType  = 0  Data

ToDS     = 0   FromDS   = 0

MoreFrag = 0   Retry    = 1

PwrMgt   = 0   moreData = 0

Encrypt  = 1

Order    = 0

Duration = 11459

A1 = 0f-d2-e1-28-a5-7c    DA

A2 = 50-30-f1-84-44-08    SA

A3 = ab-ae-a5-b8-fc-ba    BSSID

seqNum = 824  fraqNum = 0

Algorithm = AES_CCM

KeyId = 0

Key =

c9 7c 1f 67 ce 37 11 85  51 4a 8a 19 f2 bd d5 2f

PN    = 0xb5039776e70c  (decimal=199027030681356)

Header =

08 48 c3 2c 0f d2 e1 28  a5 7c 50 30 f1 84 44 08

ab ae a5 b8 fc ba 80 33

Enc Header =

0c e7 00 20 76 97 03 b5

Muted MAC Header =

08 40 0f d2 e1 28 a5 7c  50 30 f1 84 44 08 ab ae

a5 b8 fc ba 80 33

Data  =

f8 ba 1a 55 d0 2f 85 ae  96 7b b6 2f b6 cd a8 eb

7e 78 a0 50

MIC =

a7 3d 87 db 1b dd 45 20

-- Encrypted MPDU with FCS

08 48 c3 2c 0f d2 e1 28  a5 7c 50 30 f1 84 44 08

ab ae a5 b8 fc ba 80 33  0c e7 00 20 76 97 03 b5

f3 d0 a2 fe 9a 3d bf 23  42 a6 43 e4 32 46 e8 0c

3c 04 d0 19 a7 3d 87 db  1b dd 45 20 77 45 a4 67

==== CCMP test mpdu   2 ====

-- MPDU Fields

Version  = 0

Type     = 2   SubType  = 3  Data+CF-Ack+CF-Poll

ToDS     = 0   FromDS   = 0

MoreFrag = 0   Retry    = 0

PwrMgt   = 0   moreData = 0

Encrypt  = 1

Order    = 1

Duration = 20842

A1 = ea-10-0c-84-68-50    DA

A2 = ee-c1-76-2c-88-de    SA

A3 = af-2e-e9-f4-6a-07    BSSID

seqNum = 3278  fraqNum = 0

Algorithm = AES_CCM

KeyId = 2

Key =

8f 7a 05 3f a5 77 a5 59  75 29 27 20 97 a6 03 d5

PN    = 0x31f3cbba97ea  (decimal=54923164817386)

Header =

38 c0 6a 51 ea 10 0c 84  68 50 ee c1 76 2c 88 de

af 2e e9 f4 6a 07 e0 cc

Enc Header =

ea 97 00 a0 ba cb f3 31

Muted MAC Header =

08 c0 ea 10 0c 84 68 50  ee c1 76 2c 88 de af 2e

e9 f4 6a 07 e0 cc

Data  =

83 a0 63 4b 5e d7 62 7e  b9 df 22 5e 05 74 03 42

de 19 41 17

MIC =

b6 d8 e4 11 10 21 54 a7

-- Encrypted MPDU with FCS

38 c0 6a 51 ea 10 0c 84  68 50 ee c1 76 2c 88 de

af 2e e9 f4 6a 07 e0 cc  ea 97 00 a0 ba cb f3 31

81 4b 69 65 d0 5b f2 b2  ed 38 d4 be b0 69 fe 82

71 4a 61 0b b6 d8 e4 11  10 21 54 a7 e9 5d 09 26

==== CCMP test mpdu   3 ====

-- MPDU Fields

Version  = 0

Type     = 2   SubType  = 11  QC&Data+CF-Ack+CF-Poll

ToDS     = 0   FromDS   = 0

MoreFrag = 0   Retry    = 1

PwrMgt   = 0   moreData = 0

Encrypt  = 1

Order    = 1

Duration = 25052

A1 = d9-57-7d-f7-63-c8    DA

A2 = b6-a8-8a-df-36-91    SA

A3 = dc-4a-8b-ca-94-dd    BSSID

seqNum = 2086  fraqNum = 0

QC = 0x8520

QoS-TID  = 0  QoS-FEC = 0

QoS-AckP = 1  QoS-TXOP/QL = 0x85

Algorithm = AES_CCM

KeyId = 2

Key =

40 cf b7 a6 2e 88 01 3b  d6 d3 af fc c1 91 04 1e

PN    = 0x2fdca0f3a5ae  (decimal=52624639632814)

Header =

b8 c8 dc 61 d9 57 7d f7  63 c8 b6 a8 8a df 36 91

dc 4a 8b ca 94 dd 60 82  20 85

Enc Header =

ae a5 00 a0 f3 a0 dc 2f

Muted MAC Header =

88 c0 d9 57 7d f7 63 c8  b6 a8 8a df 36 91 dc 4a

8b ca 94 dd 60 82 00 00

Data  =

2c 1b d0 36 83 1c 95 49  6c 5f 4d bf 3d 55 9e 72

de 80 2a 18

MIC =

d1 58 8a 93 f2 fe 1d e5

-- Encrypted MPDU with FCS

b8 c8 dc 61 d9 57 7d f7  63 c8 b6 a8 8a df 36 91

dc 4a 8b ca 94 dd 60 82  20 85 ae a5 00 a0 f3 a0

dc 2f 89 d8 58 03 40 b6  26 a0 b6 d4 d0 13 bf 18

f2 91 b8 96 46 c8 d1 58  8a 93 f2 fe 1d e5 18 c3

5e 89

==== CCMP test mpdu   4 ====

-- MPDU Fields

Version  = 0

Type     = 2   SubType  = 10  QC&Data+CF-Poll

ToDS     = 0   FromDS   = 1

MoreFrag = 0   Retry    = 1

PwrMgt   = 0   moreData = 0

Encrypt  = 1

Order    = 1

Duration = 4410

A1 = 71-2a-9d-df-11-db    DA

A2 = 8e-f8-22-73-47-01    BSSID

A3 = 59-14-0d-d6-46-a2    SA

seqNum = 764  fraqNum = 0

QC = 0xa567

QoS-TID  = 7  QoS-FEC = 0

QoS-AckP = 3  QoS-TXOP/QL = 0xa5

Algorithm = AES_CCM

KeyId = 0

Key =

8c 89 a2 eb c9 6c 76 02  70 7f cf 24 b3 2d 38 33

PN    = 0xf670a55a0fe3  (decimal=270963670912995)

Header =

a8 ca 3a 11 71 2a 9d df  11 db 8e f8 22 73 47 01

59 14 0d d6 46 a2 c0 2f  67 a5

Enc Header =

e3 0f 00 20 5a a5 70 f6

Muted MAC Header =

88 c2 71 2a 9d df 11 db  8e f8 22 73 47 01 59 14

0d d6 46 a2 c0 2f 07 00

Data  =

4f ad 2b 1c 29 0f a5 eb  d8 72 fb c3 f3 a0 74 89

8f 8b 2f bb

MIC =

b0 2a 0a 66 4b 58 a4 d5

-- Encrypted MPDU with FCS

a8 ca 3a 11 71 2a 9d df  11 db 8e f8 22 73 47 01

59 14 0d d6 46 a2 c0 2f  67 a5 e3 0f 00 20 5a a5

70 f6 9d 59 b1 5f 37 14  48 c2 30 f4 d7 39 05 2e

13 ab 3b 1a 7b 10 b0 2a  0a 66 4b 58 a4 d5 a1 21

e8 49

==== CCMP test mpdu   5 ====

-- MPDU Fields

Version  = 0

Type     = 2   SubType  = 8  QC&Data

ToDS     = 0   FromDS   = 1

MoreFrag = 0   Retry    = 1

PwrMgt   = 1   moreData = 0

Encrypt  = 1

Order    = 1

Duration = 16664

A1 = 45-de-c6-9a-74-80    DA

A2 = f3-51-94-6b-c9-6b    BSSID

A3 = e2-76-fb-e6-c1-27    SA

seqNum = 3880  fraqNum = 0

QC = 0x194b

QoS-TID  = 11  QoS-FEC = 0

QoS-AckP = 2  QoS-TXOP/QL = 0x19

Algorithm = AES_CCM

KeyId = 2

Key =

a5 74 d5 14 3b b2 5e fd  de ff 30 12 2f df d0 66

PN    = 0xa7ffe03c0e37  (decimal=184717420531255)

Header =

88 da 18 41 45 de c6 9a  74 80 f3 51 94 6b c9 6b

e2 76 fb e6 c1 27 80 f2  4b 19

Enc Header =

37 0e 00 a0 3c e0 ff a7

Muted MAC Header =

88 c2 45 de c6 9a 74 80  f3 51 94 6b c9 6b e2 76

fb e6 c1 27 80 f2 0b 00

Data  =

28 96 9b 95 4f 26 3a 80  18 a9 ef 70 a8 b0 51 46

24 81 92 2e

MIC =

67 77 5c 87 75 90 cc 50

-- Encrypted MPDU with FCS

88 da 18 41 45 de c6 9a  74 80 f3 51 94 6b c9 6b

e2 76 fb e6 c1 27 80 f2  4b 19 37 0e 00 a0 3c e0

ff a7 eb 4a e4 95 6a 80  1d a9 62 4b 7e 0c 18 b2

3e 61 5e c0 3a f6 67 77  5c 87 75 90 cc 50 98 e7

6e a5

==== CCMP test mpdu   6 ====

-- MPDU Fields

Version  = 0

Type     = 2   SubType  = 8  QC&Data

ToDS     = 0   FromDS   = 1

MoreFrag = 0   Retry    = 0

PwrMgt   = 1   moreData = 0

Encrypt  = 1

Order    = 0

Duration = 8161

A1 = 5a-f2-84-30-fd-ab    DA

A2 = bf-f9-43-b9-f9-a6    BSSID

A3 = ab-1d-98-c7-fe-73    SA

seqNum = 1813  fraqNum = 0

QC = 0x6a3d

QoS-TID  = 13  QoS-FEC = 1

QoS-AckP = 1  QoS-TXOP/QL = 0x6a

Algorithm = AES_CCM

KeyId = 1

Key =

f7 1e ea 4e 1f 58 80 4b  97 17 23 0a d0 61 46 41

PN    = 0x6b81eca48989  (decimal=118205765159305)

Header =

88 52 e1 1f 5a f2 84 30  fd ab bf f9 43 b9 f9 a6

ab 1d 98 c7 fe 73 50 71  3d 6a

Enc Header =

89 89 00 60 a4 ec 81 6b

Muted MAC Header =

88 42 5a f2 84 30 fd ab  bf f9 43 b9 f9 a6 ab 1d

98 c7 fe 73 50 71 0d 00

Data  =

ab fd a2 2d 3a 0b fc 9c  c1 fc 07 93 63 c2 fc a1

43 e6 eb 1d

MIC =

69 ad e6 f8 5d 47 e4 d1

-- Encrypted MPDU with FCS

88 52 e1 1f 5a f2 84 30  fd ab bf f9 43 b9 f9 a6

ab 1d 98 c7 fe 73 50 71  3d 6a 89 89 00 60 a4 ec

81 6b 9a 70 9b 60 a3 9d  40 b1 df b6 12 e1 8b 5f

11 4b ad b6 cc 86 69 ad  e6 f8 5d 47 e4 d1 11 f8

cd 0a

==== CCMP test mpdu   7 ====

-- MPDU Fields

Version  = 0

Type     = 2   SubType  = 1  Data+CF-Ack

ToDS     = 1   FromDS   = 0

MoreFrag = 0   Retry    = 1

PwrMgt   = 1   moreData = 1

Encrypt  = 1

Order    = 0

Duration = 18049

A1 = 9b-50-f4-fd-56-f6    BSSID

A2 = ef-ec-95-20-16-91    SA

A3 = 83-57-0c-4c-cd-ee    DA

seqNum = 2562  fraqNum = 0

Algorithm = AES_CCM

KeyId = 3

Key =

1b db 34 98 0e 03 81 24  a1 db 1a 89 2b ec 36 6a

PN    = 0x5eec4073e723  (decimal=104368786630435)

Header =

18 79 81 46 9b 50 f4 fd  56 f6 ef ec 95 20 16 91

83 57 0c 4c cd ee 20 a0

Enc Header =

23 e7 00 e0 73 40 ec 5e

Muted MAC Header =

08 41 9b 50 f4 fd 56 f6  ef ec 95 20 16 91 83 57

0c 4c cd ee 20 a0

Data  =

98 be ca 86 f4 b3 8d a2  0c fd f2 47 24 c5 8e b8

35 66 53 39

MIC =

df 67 be 41 df 3e 37 12

-- Encrypted MPDU with FCS

18 79 81 46 9b 50 f4 fd  56 f6 ef ec 95 20 16 91

83 57 0c 4c cd ee 20 a0  23 e7 00 e0 73 40 ec 5e

12 c5 37 eb f3 ab 58 4e  f1 fe f9 a1 f3 54 7a 8c

13 b3 22 5a df 67 be 41  df 3e 37 12 cf 2f db bc

==== CCMP test mpdu   8 ====

-- MPDU Fields

Version  = 0

Type     = 2   SubType  = 11  QC&Data+CF-Ack+CF-Poll

ToDS     = 1   FromDS   = 0

MoreFrag = 0   Retry    = 1

PwrMgt   = 1   moreData = 0

Encrypt  = 1

Order    = 1

Duration = 29260

A1 = 55-2d-5f-72-bb-70    BSSID

A2 = ca-3f-3a-ae-60-c4    SA

A3 = 8b-a9-b5-f8-2c-2f    DA

seqNum = 3765  fraqNum = 0

QC = 0x552a

QoS-TID  = 10  QoS-FEC = 0

QoS-AckP = 1  QoS-TXOP/QL = 0x55

Algorithm = AES_CCM

KeyId = 2

Key =

6e ac 1b f5 4b d5 4e db  23 21 75 43 03 02 4c 71

PN    = 0xcefd996eccdd  (decimal=227588596223197)

Header =

b8 d9 4c 72 55 2d 5f 72  bb 70 ca 3f 3a ae 60 c4

8b a9 b5 f8 2c 2f 50 eb  2a 55

Enc Header =

dd cc 00 a0 6e 99 fd ce

Muted MAC Header =

88 c1 55 2d 5f 72 bb 70  ca 3f 3a ae 60 c4 8b a9

b5 f8 2c 2f 50 eb 0a 00

Data  =

57 cb 5c 0e 5f cd 88 5e  9a 42 39 e9 b9 ca d6 0d

64 37 59 79

MIC =

a8 6e 1a 9c be 26 97 20

-- Encrypted MPDU with FCS

b8 d9 4c 72 55 2d 5f 72  bb 70 ca 3f 3a ae 60 c4

8b a9 b5 f8 2c 2f 50 eb  2a 55 dd cc 00 a0 6e 99

fd ce 4b f2 81 ef 8e c7  73 9f 91 59 1b 97 a8 7d

c1 4b 3f a1 74 62 a8 6e  1a 9c be 26 97 20 2c 8b

5b 15

==== CCMP test mpdu   9 ====

-- MPDU Fields

Version  = 0

Type     = 2   SubType  = 10  QC&Data+CF-Poll

ToDS     = 1   FromDS   = 0

MoreFrag = 0   Retry    = 1

PwrMgt   = 1   moreData = 1

Encrypt  = 1

Order    = 1

Duration = 17096

A1 = 9a-fb-79-8b-8a-4b    BSSID

A2 = a4-ad-6d-31-99-85    SA

A3 = bc-42-9e-8f-0a-fa    DA

seqNum = 2015  fraqNum = 0

QC = 0x804a

QoS-TID  = 10  QoS-FEC = 0

QoS-AckP = 2  QoS-TXOP/QL = 0x80

Algorithm = AES_CCM

KeyId = 3

Key =

49 4b 50 1e 19 46 75 97  1a 48 d0 8c 5b c3 53 cb

PN    = 0xba82e93437b3  (decimal=205071421028275)

Header =

a8 f9 c8 42 9a fb 79 8b  8a 4b a4 ad 6d 31 99 85

bc 42 9e 8f 0a fa f0 7d  4a 80

Enc Header =

b3 37 00 e0 34 e9 82 ba

Muted MAC Header =

88 c1 9a fb 79 8b 8a 4b  a4 ad 6d 31 99 85 bc 42

9e 8f 0a fa f0 7d 0a 00

Data  =

25 a9 8f 9c 1b d9 c9 3c  f3 83 ab 9d 98 15 2d 76

cb 4a 32 c6

MIC =

21 29 49 42 27 90 7b 5a

-- Encrypted MPDU with FCS

a8 f9 c8 42 9a fb 79 8b  8a 4b a4 ad 6d 31 99 85

bc 42 9e 8f 0a fa f0 7d  4a 80 b3 37 00 e0 34 e9

82 ba 56 1a 0d 06 8e ac  2e ad b0 c5 7f e2 d0 a6

cc 73 98 b6 dd bf 21 29  49 42 27 90 7b 5a 64 c7

16 bc

==== CCMP test mpdu  10 ====

-- MPDU Fields

Version  = 0

Type     = 2   SubType  = 1  Data+CF-Ack

ToDS     = 1   FromDS   = 1

MoreFrag = 0   Retry    = 1

PwrMgt   = 0   moreData = 0

Encrypt  = 1

Order    = 0

Duration = 25568

A1 = 40-ec-29-fa-75-9b    RA

A2 = 53-f8-69-fe-27-9a    TA

A3 = f0-f9-f8-a6-54-16    DA

A4 = 52-bf-d2-70-3d-24    SA

seqNum = 3640  fraqNum = 0

Algorithm = AES_CCM

KeyId = 0

Key =

48 9e 49 bc 3c fe 3f ce  38 95 82 0e 87 2e e1 a3

PN    = 0xcf1d3e75fea9  (decimal=227724508921513)

Header =

18 4b e0 63 40 ec 29 fa  75 9b 53 f8 69 fe 27 9a

f0 f9 f8 a6 54 16 80 e3  52 bf d2 70 3d 24

Enc Header =

a9 fe 00 20 75 3e 1d cf

Muted MAC Header =

08 43 40 ec 29 fa 75 9b  53 f8 69 fe 27 9a f0 f9

f8 a6 54 16 80 e3 52 bf  d2 70 3d 24

Data  =

7f 91 f2 47 2d 7a 12 1c  9c dd 4b 6c 90 80 67 5a

10 20 aa 00

MIC =

41 b3 7c 81 6f 32 6b 22

-- Encrypted MPDU with FCS

18 4b e0 63 40 ec 29 fa  75 9b 53 f8 69 fe 27 9a

f0 f9 f8 a6 54 16 80 e3  52 bf d2 70 3d 24 a9 fe

00 20 75 3e 1d cf 25 df  51 73 83 5e 4f ba 23 bc

05 a2 53 88 5e be d3 ac  48 71 41 b3 7c 81 6f 32

6b 22 d7 85 3e 56

==== CCMP test mpdu  11 ====

-- MPDU Fields

Version  = 0

Type     = 2   SubType  = 11  QC&Data+CF-Ack+CF-Poll

ToDS     = 1   FromDS   = 1

MoreFrag = 0   Retry    = 1

PwrMgt   = 1   moreData = 0

Encrypt  = 1

Order    = 1

Duration = 21610

A1 = 29-8c-0b-aa-91-90    RA

A2 = 34-7c-eb-9a-ab-ff    TA

A3 = d8-3d-48-86-e5-c2    DA

A4 = e2-9d-52-4a-e1-96    SA

seqNum = 1342  fraqNum = 0

QC = 0x623d

QoS-TID  = 13  QoS-FEC = 1

QoS-AckP = 1  QoS-TXOP/QL = 0x62

Algorithm = AES_CCM

KeyId = 3

Key =

02 be 5c 45 45 67 2a 07  e4 e3 14 d7 0f 1f 9e 85

PN    = 0xd2d6596e55d4  (decimal=231818065237460)

Header =

b8 db 6a 54 29 8c 0b aa  91 90 34 7c eb 9a ab ff

d8 3d 48 86 e5 c2 e0 53  e2 9d 52 4a e1 96 3d 62

Enc Header =

d4 55 00 e0 6e 59 d6 d2

Muted MAC Header =

88 c3 29 8c 0b aa 91 90  34 7c eb 9a ab ff d8 3d

48 86 e5 c2 e0 53 e2 9d  52 4a e1 96 0d 00

Data  =

f9 a8 12 e4 a2 8a f7 f3  71 4d 4b f6 62 2e 59 32

f2 18 45 09

MIC =

26 b4 c7 74 10 a9 d2 29

-- Encrypted MPDU with FCS

b8 db 6a 54 29 8c 0b aa  91 90 34 7c eb 9a ab ff

d8 3d 48 86 e5 c2 e0 53  e2 9d 52 4a e1 96 3d 62

d4 55 00 e0 6e 59 d6 d2  63 15 50 0f 92 42 95 cd

3e af bd c3 e1 51 b1 df  46 46 5b 71 26 b4 c7 74

10 a9 d2 29 fc 1d 3c 21

==== CCMP test mpdu  12 ====

-- MPDU Fields

Version  = 0

Type     = 2   SubType  = 1  Data+CF-Ack

ToDS     = 1   FromDS   = 1

MoreFrag = 0   Retry    = 1

PwrMgt   = 0   moreData = 1

Encrypt  = 1

Order    = 0

Duration = 10192

A1 = aa-28-8b-84-35-bc    RA

A2 = d8-ac-5a-7e-c4-44    TA

A3 = e8-b4-62-50-53-8b    DA

A4 = e8-14-02-c2-ee-11    SA

seqNum = 1576  fraqNum = 0

Algorithm = AES_CCM

KeyId = 3

Key =

77 07 7e d7 94 53 e4 a1  8d 60 43 8c c6 48 4d 6e

PN    = 0x50b01e77fd8e  (decimal=88717355646350)

Header =

18 6b d0 27 aa 28 8b 84  35 bc d8 ac 5a 7e c4 44

e8 b4 62 50 53 8b 80 62  e8 14 02 c2 ee 11

Enc Header =

8e fd 00 e0 77 1e b0 50

Muted MAC Header =

08 43 aa 28 8b 84 35 bc  d8 ac 5a 7e c4 44 e8 b4

62 50 53 8b 80 62 e8 14  02 c2 ee 11

Data  =

43 19 81 a2 33 6d 02 f8  cb 84 48 d5 42 89 16 be

95 29 35 37

MIC =

4c f2 55 e3 98 29 a1 98

-- Encrypted MPDU with FCS

18 6b d0 27 aa 28 8b 84  35 bc d8 ac 5a 7e c4 44

e8 b4 62 50 53 8b 80 62  e8 14 02 c2 ee 11 8e fd

00 e0 77 1e b0 50 cf 71  b2 cc bd 59 0b 20 80 07

92 f3 59 ed 1c fd 74 d8  00 b4 4c f2 55 e3 98 29

a1 98 08 43 d3 6f