IEEE P802.11
Wireless LANs

# Radio Resource Measurement Vision and Architecture

Date:                                                     December 2002

Author:

Richard H. Paine
Boeing
Box 3707, MS 7L-49
Phone: 425-865-4921: 
Fax: 425-865-2965
e-Mail: richard.h.paine@boeing.com

IEEE 802.11

Radio Resource Measurement Vision and Architecture

IEEE 802.11

Radio Resource Measurement

Vision and Architecture

A reference document for 802.11

UNAPPROVED DRAFT

Editorial Committee:

Richard Paine, Boeing - Chair

Harry Worstell, AT&T - Secretary

Simon Barber, Instant802 - Editor

Radio Resource Measurement

Vision and Architecture

Jan 16, 2003

Richard H. Paine
Network Technology
Boeing, Phantom Works
Mail stop 7L-49
(425) 865-4921
richard.h.paine@boeing.com

# Team Page

# Table of Contents

# 1. Introduction

# 2. Normative references

# 3. Definitions

# 4. Abbreviations and acronyms

# 5. Requirements

## Data, Voice, and Video

## Rogue Access Points

## Quantify WLAN radio topology for AFS and TPC

## Measure BSS overlap to feed mitigation (TGe) and help balance coverage, capacity, and QoS

## Quantify each station’s local performance to assist admissions control (TGe) and to facilitate roaming and load balancing

## Detect non-802.11 interference and quantify noise to facilitate adjustments in WLAN configuration

# 6. MIB Approaches

## 6.1 Peer Measure STA MIBs

## 6.2 Self Measured STA MIBs

## 6.3 AP MIBs

# 7. Sources of Measurement

## 7.1 AP Distribution of Service Information on Setup

### Probe Reponse

### Beacon

### Special Management Frames

## 7.2 Passively Observed Parameters

### 7.2.1 MIB Variables

#### CCA

#### RSSI

#### Noise Power

#### Frame Counts

#### States

#### MAC addresses

#### Beacons

### 7.2.2 Extend/replace with dot11macStatsTable for per Interface stats

### 7.2.3 Extend doc11phy/dot11phyDSSSTable and dot11phyOFDMTable stats

### 7.2.4 Tables of all known STAs, APs, regardless of association

#### Station Tables

##### DotremSTAstatsTable

##### Dot11remSTAconfig

##### Dot11remSTAstatusTable

##### Dot11remSTAactions

#### AP Tables

## 7.3 Transfer of Parameters between STA and AP: Measurements that require specific action (11h-like)

### 7.3.1  Requests

#### Set through measurement action table in MIB

#### Include any required measurement parameters

#### May pertain to AP radio and/or client radios

### 7.3.2  Measurements

#### Serving channel is measure without disrupting traffic

#### Off-channel measurements temporarily disrupt traffic

#### Station’s traffic is buffered if it measures off-channel

#### Station may decline if measurement poses undue burden

### 7.3.3  Reports

#### Communicated through new MIB tables

#### 7.3.3.1  Beacon Report

##### Purpose

###### Detects possible rogue APs

###### Quantifies WLAN radio topology

###### Measures BSS Overlap

##### How it Works

###### Each request specifies channels or asks for current beacon table

###### Measure at AP radio and/or client radios

###### Measure on serving channel or off-channel

###### For each detected beacon and probe response

Always report channel, BSSID, RSSI, and time of detection

Optionally report other fixed fields and information elements

#### 7.3.3.2  Extended CCA Report

##### Purpose

###### Quantifies each station’s current local performance

##### How it Works

###### Measure at AP radio and/or client radios

###### Measure on serving channel only

###### Measured parameters

CCA busy fraction indicates degree of local contention at STA

Histogram of recent backoff translates to radio-related jitter at STA

#### 7.3.3.3  Non-802.11 RPI Histogram Report

##### Purpose

###### Detects non-802.11 interference and quantifies noise

##### How it Works

###### Measure at AP radio and/or client radios

###### Measure on serving channel only

###### Measured parameters

Received power indicator (RPI) is a quatized measure of received power

TGh defines eight RPI levels with 5dB ranges for inner levels

RPI Histogram measures occurrences of each RPI level

RPI Histogram in TGh does not distinguish non-802.11 energy

#### 7.3.3.4  Frame Report

##### Purpose

###### Identifies a station’s local contending stations

Information needed if AP detects acceptable CCA, but station detects unacceptably high CCA

##### How it Works

###### Finds the station causing the contention

###### Measure at AP radio or client radios, but typically client

###### Measure on serving channels only

###### Report one sub-element for each detected contending station

Transmitter MAC address

BSSID to which contending STA belongs

Number of frames decoded from STA

Average signal strength of frames from STA

###### Store the reported measurement table in MIB under SMT

### 7.3.4  Transfer Mechanisms

#### Request/Response Over Air

#### Passively Measured Parameters

#### Actively Measured Parameters

### 7.3.5  Reported parameters from other STAs should go under dot11smt, indexed via integer containing reporters’ address and observed address

# 8. Logical service interfaces

## 8.1  BSS/ESS Setup (Zhong paper 798r0)

## 8.2  MIB Variables

## 8.3  TGh-like Request/Response

# 9 Overview of the RRM services

## 9.1  Initial AP Setup Services

## 9.2  Relationships Between RRM Services

## 9.3  Relationships Among RRM Services

## 9.4  Differences between ESS and IBSS LANs

# 10. Coexistence

# 11. Interaction with other IEEE 802 layers

# 12. Interaction with non-802 Protocols

# 13 Reference model