IEEE P802.11
Wireless LANs

# Radio Resource Measurement Vision and Architecture

Date:                                                     January 2003

Author:

Richard H. Paine
Boeing
Box 3707, MS 7L-49
Phone: 425-865-4921: 
Fax: 425-865-2965
e-Mail: richard.h.paine@boeing.com

IEEE 802.11

Radio Resource Measurement Vision and Architecture

IEEE 802.11

Radio Resource Measurement

Vision and Architecture

A reference document for 802.11

UNAPPROVED DRAFT

Editorial Committee:

Richard Paine, Boeing - Chair

Harry Worstell, AT&T - Secretary

Simon Barber, Instant802 - Editor

Radio Resource Measurement

Vision and Architecture

Jan 16, 2003

Richard H. Paine
Network Technology
Boeing, Phantom Works
Mail stop 7L-49
(425) 865-4921
richard.h.paine@boeing.com

# Team Page

# Table of Contents

# 1. Introduction

# 2. Normative references

# 3. Definitions

# 4. Abbreviations and acronyms

# 5. Requirements

## Data, Voice, and Video

## Rogue Access Points

## Quantify WLAN radio topology for AFS and TPC

## Measure BSS overlap to feed mitigation (TGe) and help balance coverage, capacity, and QoS

## Quantify each station’s local performance to assist admissions control (TGe) and to facilitate roaming and load balancing

## Detect non-802.11 interference and quantify noise to facilitate adjustments in WLAN configuration (radar)

# 6. Scope of RRM

•In practice the protocol elements that are specified will be MAC and PHY extensions

–most likely MLME and PLME services

•The standard should deal with protocol and not decision-making  or algorithms

–for example a set of measurements may be defined but not any specific rule as to when these measurements should be made, or how the results should be used

•Effectively the standard should provide a tool-kit for RRM

# 7. Specific RRM Requirements

## 7.1 Capabilities, Measurements, and Statistics

•It should be possible to identify an RRM capable AP, or STA

•It should be possible for a management entity to request that an Access Point make appropriate measurements to determine the radio environment

–prior to MLME-START.request being issued

–while operating

•STAs should have the ability to make and report measurements on the radio environment

–autonomously

–when requested via the AP

•It should be possible for a Network Management Entity to obtain configuration and statistics information

–extend AP MIB for per-STA statistics and not global MAC statistics as already discussed in RRM SG/TGk

–it should be possible to generate management alerts for certain conditions

## 7.2 Improved BSS Information

•Information should be available to STAs to assist in making association decisions that result in

–good service characteristics for the individual STA without affecting those experienced by other STAs in the ESS

–efficient usage of the available resources.

–likely to be particularly important if applications with Quality-of-Service (QoS) requirements are being supported

–e.g. the provision of BSS load information

## 7.3 Information to Streamline Roaming

•Information should be available to STAs that assists in streamlining the roaming process

–Overall objective to improve the user experience at handover through the provision of additional radio resource and system information

–Transmitted by the AP

–Information regarding other APs in the ESS (neighbourhood or co-located BSSs) and possibly the boundary of an ESS

–This is just provision of information – the decision of when to scan for and join a given BSS is unchanged as an STA decision

# 8. Objectives

## 8.1 RRM is about obtaining information and delivering it to the appropriate place

## 8.2 Improved observation of AP and client performance and environment to facilitate

•Improved WLAN client performance though better network management

•Optimization of the overall system capacity – particularly in dense BSS environments

•Refinement of system installation/deployment – are new APs required? Are existing APs badly sited?

•Improved diagnosis of network management problems – audit trails, system alerts

•Managed link quality for QoS provision

•A flexible framework that can be adapted for new services

## 8.3 Enhancing informationprovided to WLAN clients to:

•Improve the AP selection process both when joining and when roaming

•Provide assisted handover support

## 8.4 Information for assisted, or possibly autonomous network configuration during installation and expansion

# 9. Summary of Actions Addressed by 802.11k

## 9.1 What do you hear? (802.11 and non-802.11)

### Extended CCA and RPI Histogram reports

### Radar

### 802.15

### 802.16

## 9.2 Who do you hear? (802.11)

### Beacon and frame reports

# 10. Standardizing Values for Measurements (timestamp all)

## 10.1 RSSI

## 10.2 Signal Quality

## 10.3 RPI Histogram

## 10.4 PSNI

## 10.5 DR

## 10.6 Loading (requested vs served)

# 11. RRM Services

## 11.1 Reports from the SME

## 11.2 Requests to the SME

## 11.3 Requests from SME to MLME

## 11.4 Requests from MLME to SME

## 11.5 Passing Requests and Receiving Responses Across the PLME SAP

## 11.6 Passing Requests and Receiving Responses Across the PMD

## 11.7 Information to Higher Layers within a given station (to and from the SME which are action frames and action frame responses)

### NDIS

### Objects for Linux

### Wind River

# 12. Framework – Six RRM Functional Scenarios

## 12.1 AP MIB Related Configuration, Control, and Statistics

## 12.2 AP Measurements

## 12.3 Distribution of RRM Information to STAs

## 12.4 STA Statistics Gathering

## 12.5 AP Commanded STA Actions

## 12.6 STA Autonomous Action

# 13. MIB Approaches

## 13.1 Per Client MIBs

## 13.2 AP MIBs

# 14. RRM Protocol Elements

•Add a bit to the Capability Information fixed field to indicate RRM capability

•Use the Action management frame type as defined in the IEEE802.11h draft

–define a new Action category value defined for RRM

•Extension of the AP MAC MIB

–enhance for per-STA link statistics

•Use the measurement request and measurement report elements and protocol as defined in the IEEE802.11h draft

–Define measurements to meet the requirements

•RSSI, retry statistics, CCA busy fraction, noise floor, observed BSSs, non-IEEE802.11 signals

–Consider cross-band measurements

•Define a BSS load element

–Review the QBSS load element in the IEEE802.11e draft as a starting point

•Define an ESS Information element

–potential neighbour BSSs

–indicate the boundary of the ESS

# 15. Sources of Measurement

## 15.1 AP Distribution of Service Information on Setup

### Probe Reponse

### Beacon

### Special Management Frames

## 15.2 Passively Observed Parameters

### 7.2.1 MIB Variables

#### CCA

#### RSSI

#### Noise Power

#### Frame Counts

#### States

#### MAC addresses

#### Beacons

### 15.2.2 Extend/replace with dot11macStatsTable for per Interface stats

### 15.2.3 Extend doc11phy/dot11phyDSSSTable and dot11phyOFDMTable stats

### 15.2.4 Tables of all known STAs, APs, regardless of association

#### Station Tables

##### DotremSTAstatsTable

##### Dot11remSTAconfig

##### Dot11remSTAstatusTable

##### Dot11remSTAactions

#### AP Tables

## 15.3 Transfer of Parameters between STA and AP: Measurements that require specific action (11h-like)

### 15.3.1  Requests

#### Set through measurement action table in MIB

#### Include any required measurement parameters

#### May pertain to AP radio and/or client radios

### 15.3.2  Measurements

#### Serving channel is measure without disrupting traffic

#### Off-channel measurements temporarily disrupt traffic

#### Station’s traffic is buffered if it measures off-channel

#### Station may decline if measurement poses undue burden

### 15.3.3  Reports

#### Communicated through new MIB tables

#### 15.3.3.1  Beacon Report

##### Purpose

###### Detects possible rogue APs

###### Quantifies WLAN radio topology

###### Measures BSS Overlap

##### How it Works

###### Each request specifies channels or asks for current beacon table

###### Measure at AP radio and/or client radios

###### Measure on serving channel or off-channel

###### For each detected beacon and probe response

Always report channel, BSSID, RSSI, and time of detection

Optionally report other fixed fields and information elements

#### 15.3.3.2  Extended CCA Report

##### Purpose

###### Quantifies each station’s current local performance

##### How it Works

###### Measure at AP radio and/or client radios

###### Measure on serving channel only

###### Measured parameters

CCA busy fraction indicates degree of local contention at STA

Histogram of recent backoff translates to radio-related jitter at STA

#### 15.3.3.3  Non-802.11 RPI Histogram Report

##### Purpose

###### Detects non-802.11 interference and quantifies noise

##### How it Works

###### Measure at AP radio and/or client radios

###### Measure on serving channel only

###### Measured parameters

Received power indicator (RPI) is a quatized measure of received power

TGh defines eight RPI levels with 5dB ranges for inner levels

RPI Histogram measures occurrences of each RPI level

RPI Histogram in TGh does not distinguish non-802.11 energy

#### 15.3.3.4  Frame Report

##### Purpose

###### Identifies a station’s local contending stations

Information needed if AP detects acceptable CCA, but station detects unacceptably high CCA

##### How it Works

###### Finds the station causing the contention

###### Measure at AP radio or client radios, but typically client

###### Measure on serving channels only

###### Report one sub-element for each detected contending station

Transmitter MAC address

BSSID to which contending STA belongs

Number of frames decoded from STA

Average signal strength of frames from STA

###### Store the reported measurement table in MIB under SMT

### 15.3.4  Transfer Mechanisms

#### Request/Response Over Air

#### Passively Measured Parameters

#### Actively Measured Parameters

### 15.3.5  Reported parameters from other STAs should go under dot11smt, indexed via integer containing reporters’ address and observed address

## 15.4  Actively Measured Parameters

### Quiet the Medium

# 16. Logical service interfaces

## 16.1  BSS/ESS Setup (Zhong paper 798r0)

## 16.2  MIB Variables

## 16.3  TGh-like Request/Response

# 17 Overview of the RRM services

## 17.1  Initial AP Setup Services

## 17.2  Relationships Between RRM Services

## 17.3  Relationships Among RRM Services

## 17.4  Differences between ESS and IBSS LANs

# 18. Coexistence

# 19. Interaction with other IEEE 802 layers

# 20. Interaction with non-802 Protocols

# 21. Reference model