IEEE P802.11
Wireless LANs

## Indoor MIMO WLAN Channel Models

(Draft Document in Progress)

Date:	March 10, 2003

Authors:

Vinko Erceg

Company: Zyray Wireless
	Address: 11455 El Camino Real, Suite 350
	Phone: (858) 704-0430
	Fax: (858) 362-0236

e-Mail: verceg@zyraywireless.com

Laurent Schumacher

Namur University

e-Mail: laurent.schumacher@ieee.org

Persefoni Kyritsi

Aalborg University

e-Mail: persa@cpk.auc.dk.

Daniel S. Baum

ETH University

e-Mail: dsbaum@nari.ee.ethz.ch

Andreas Molisch

Mitsubishi Electric

e-Mail: andreas.molisch@ieee.org

Alexei Y. Gorokhov

Philips Research

List of Participants

| Vinko Erceg (Zyray Wireless) Laurent Schumacher (Namur University) Persefoni Kyritsi (Aalborg University) Daniel Baum (ETH University) Andreas Molisch (Mitsubishi Electric) Alexei Gorokhov (Philips Research) Srinath Hosur (Texas Instruments) Srikanth Gummadi (Texas Instruments) Eilts Henry (Texas Instruments) Eric Jacobsen (Intel) Sumeet Sandhu (Intel) David Cheung (Intel) Qinghua Li (Intel) Clifford Prettie (Intel) Heejung Yu (ETRI) Yeong-Chang Maa (InProComm) Richard van Nee (Airgo) Jonas Medbo (Erricsson) Eldad Perahia (Cisco Systems) Helmut Boelcskei (ETH Univ.) | Hemanth Sampath (Marvell) H. Lou (Marvell) Pieter van Rooyen (Zyray Wireless) Pieter Roux (Zyray Wireless) Majid Malek (HP) Timothy Wakeley (HP) Dongjun Lee (Samsung) Tomer Bentzion (Metalink) Nir Tal (Metalink) Amir Leshem (Metalink, Bar IIan University) Guy Shochet (Metalink) Patric Kelly (Bandspeed) Vafa Ghazi (Cadence) Mehul Mehta (Synad Technologies) Bobby Jose (Mabuhay Networks) Charles Farlow (California Amplifier) Claude Oestges (Louvain University) Robert W. Heath (University of Texas at Austin) |
| --- | --- |

1. Introduction

Multiple antenna technologies are being considered as a viable solution for the next generation of mobile and wireless local area networks (WLAN). The use of multiple antennas offers extended range, improved reliability and higher throughputs than conventional single antenna communication systems. Multiple antenna systems can be generally separated into two main groups: smart antennas-based systems and spatial multiplexing-based multiple-input multiple-output (MIMO) systems.

Smart antennas-based systems exploit multiple transmit and/or receive antennas to provide diversity gain in a fading environment, antenna gain and interference suppression. These gains translate into improvement of the spectrum efficiency, range and reliability of wireless networks. These systems may have an array of multiple antennas only at one end of the communication link (e.g., at the transmit side, such as multiple-input single-output (MISO) systems; or at the receive side, such as single-input multiple-output (SIMO) systems) or at both ends (MIMO systems). When multiple antennas are used at the transmitter, each can broadcast at the same time and in the same bandwidth an independent signal substream. This corresponds to the second category of multi-antennas systems, referred to as spatial multiplexing-based multiple-input multiple-output (MIMO) systems. Using this technology with n transmit and n receive antennas, for example, n-fold increase in data rate can be achieved over a single antenna system [1]. This breakthrough technology appears promising in fulfilling the growing demand for future high data rate PAN, WLAN, WAN, and 4G systems.

In this document we propose a set of channel models applicable to indoor MIMO WLAN systems. The channel models are an extension of the single-input single-output (SISO) WLAN channel models proposed by Medbo et. al. [2,3]. The newly developed multiple antenna models are based on the cluster model developed by Saleh and Valenzuela [4], and further elaborated upon by Spencer et al. [5], Cramer et al. [6], and Poon and Ho [7]. Indoor SISO and MIMO indoor wireless channels were further analyzed in [8- 18].

A step-wise development of the new models follows: In each of the five models (A-E) in [2], distinct clusters were identified first. The number of clusters found was from 3 to 6, depending on the model. This finding is consistent with numerous experimentally determined results reported in the literature [4-7,9,10] and also using ray-tracing methods [8]. Next, the power of each tap in a particular cluster was determined so that the sum of the powers of overlapping taps corresponding to different clusters corresponds to the powers of the original A-E power delay profiles. Next, azimuth spread (AS), angle-of-arrival (AoA), and angle of departure (AoD) were assigned to each tap using statistical methods so that the resulting cluster AS and mean AoA (AoD) agree with experimentally determined AS and mean AoA (AoD) results reported in the literature. Cluster AS was experimentally found to be in the 20o to 40o range [5-10], and the mean AoA was found to be random with a uniform distribution. With knowledge of each tap power, AS, and AoA, for a given antenna configuration, the channel matrix H can be determined. The channel matrix H fully describes the propagation channel between all transmit and receive antennas. If the number of receive antennas is n and transmit antennas is m, the channel matrix H has a dimension of n x m. To arrive at channel matrix H, we use a method that employs correlation matrix and i.i.d. matrix (zero-mean unit variance independent complex Gaussian random variables). The correlation matrix for each tap is based on the power azimuth spectrum (PAS) with AS being the second moment of PAS [19,20]. To verify the newly developed model, we have calculated the antenna correlation coefficients and channel capacity assuming the narrowband case. The correlation coefficients agree with those reported in the literature for similar environments.

The paper is organized as follows. In Sec. 2 we describe existing SISO WLAN models. Section 3 formulates the MIMO channel matrix. Section 4 describes the clustering approach and the method for model parameters calculation. In Sec. 5 we summarize the model parameters. In Sec. 6 we briefly describe the Matlab program. Section 7 presents the antenna correlation and channel capacity results using the models, and with Sec. 8 we conclude.

2. Existing SISO WLAN Models

A set of WLAN channel models was developed by Medbo et. al. [2,3]. We use the multipath delay profiles specified in these models as a basis of our MIMO WLAN models.

In [2], five delay profile models were proposed for different environments (Models A-E):

- Model A for a typical office environment, non-line-of-sight (NLOS) conditions, and 50 ns rms delay spread.

- Model B for a typical large open space and office environments, NLOS conditions, and 100 ns rms delay spread.

- Model C for a large open space (indoor and outdoor), NLOS conditions, and 150 ns rms delay spread.

- Model D, same as model C, line-of-sight (LOS) conditions, and 140 ns rms delay spread (10 dB Ricean K-factor at the first delay).

- Model E for a typical large open space (indoor and outdoor), NLOS conditions, and 250 ns rms delay spread.

The tables with channel coefficients (tap delays and corresponding powers) can be found in [2]. The path loss model was presented in [3]. The model consists of the free space loss LFS and a linear loss per meter

### L(d) = LFS(d) + 0.5 d                                                     (1)

An alternative path loss model can be found in [21]

L(d) = LFS(d)                             d <= 8 m

(2)

L(d) = LFS(d)  + 33 log10(d/8)             d > 8 m

where d is the transmit-receive separation distance.

3. MIMO Matrix Formulation

We follow the MIMO modeling approach presented in [11,20] that utilizes receive and transmit correlation matrices. The MIMO channel matrix H for each tap, at one instance of time, in the A-E delay profile models can be separated into a fixed (constant, LOS) matrix and a Rayleigh matrix [22] (4 transmit and 4 receive antennas example)

H=PKK+1HF+1K+1HvPKK+1ejφ11ejφ12ejφ13ejφ14ejφ21ejφ22ejφ23ejφ24ejφ31ejφ32ejφ33ejφ34ejφ41ejφ42ejφ43ejφ44+1K+1X11X12X13X14X21X22X23X24X31X32X33X34X41X42X43X44             (3)

where Xij (i-th receiving and j-th transmitting antenna) are correlated zero-mean, unit variance, complex Gaussian random variables as coefficients of the Rayleigh matrix HV, exp(jij) are the elements of the fixed matrix HF, K is the Ricean K-factor, and P is the power of each tap. Note that only model D has a non-zero K-factor on the first tap. We assume that each tap consists of a number of individual rays so that the complex Gaussian assumption is valid.

To correlate the Xij elements of the matrix X, the following method can be used

X=Rrx12HiidRtx12                                                (4)

where Rtx and Rrx are the receive and transmit correlation matrices, respectively, and Hiid is a matrix of independent zero mean, unit variance, complex Gaussian random variables, and

Rtx=ρtxijRrx=ρrxij                                                                (5)

where txij are the complex correlation coefficients between i-th and j-th transmitting antennas, and rxij are the complex correlation coefficients between i-th and j-th receiving antennas. An alternative approach uses the Kronecker product of the receive and transmit correlation matrices (Hiid is an array in this case instead of matrix)

X=Rrx⊗RtxHiid                                                   (6)

Following is an example of 4x4 MIMO channel transmit and receive correlation matrices

Rtx=1ρtx12∗ρtx13∗ρtx14∗ρtx211ρtx23∗ρtx24∗ρtx31ρtx321ρtx34∗ρtx41ρtx42ρtx431

(7)

Rrx=1ρrx12∗ρrx13∗ρrx14∗ρrx211ρrx23∗ρrx24∗ρrx31ρrx321ρrx34∗ρrx41ρrx42ρrx431

The complex correlation coefficient values calculation for each tap is based on the power azimuth spectrum (PAS) with azimuth spread (AS) being the second moment of PAS [19,20]. Using the PAS shape, AS, mean angle-of-arrival (AoA), and individual tap powers, correlation matrices of each tap can be determined as described in [20]. For the uniform linear array (ULA) the complex correlation coefficient at the antenna array is expressed as

ρ=RXX(D)+jRXY(D)                                                   (8)

where D=2πdλ, and RXX and RXY are the cross-correlation functions between the real parts (equal to the cross-correlation function between the imaginary parts) and between the real part and imaginary part, respectively, with

RXX(D)=−ππcos(Dsinφ)PAS(φ)dφ                                      (9)

and

RXY(D)=−ππsin(Dsinφ)PAS(φ)dφ                                    (10)

Expressions for correlation coefficients assuming uniform, truncated Gaussian and truncated Laplacian PAS shapes can be found in [20]. To calculate the numerical values of correlation matrices we use a Matlab program developed and distributed by Schumacher [23] (see Sec. 6).

Next we briefly describe the various steps in our cluster modeling approach. We

- Start with five delay profiles A-E defined in [2].

- Manually identify clusters in each of the five models.

- Extend clusters so that they overlap, determine tap powers.

- Assume PAS shape of each cluster (Laplacian).

- Assign AS to each cluster.

- Assign mean AoA (AoD) to each cluster.

- Assume AS of each tap within each cluster (5o for all).

- Determine AoA (AoD) of each tap so that the cluster AS and cluster mean AoA (AoD) requirements are met.

- Assume antenna configuration.

- Calculate correlation matrices for each tap.

In the next section we elaborate on the above steps.

4. Cluster Modeling Approach

The cluster model was introduced first by Saleh and Valenzuela [4] and later verified, extended, and elaborated upon by other researchers in [5-10]. The received signal amplitude kl is a Rayleigh-distributed random variable with a mean-square value that obeys a double exponential decay law

βkl2=β2(0,0)e−TlΓe−τklγ                                              (11)

where β2(0,0) represents the average power of the first arrival of the first cluster, Tl represents the arrival time of the lth cluster, and kl is the arrival time of the kth arrival within the lth cluster, relative to Tl. The parameters  and  determine the inter-cluster signal level rate of decay and the intra-cluster rate of decay, respectively. The rates of the cluster and ray arrivals can be determined using exponential rate laws

p(Tl|Tl−1)=Λe−Λ(Tl−Tl−1)                                                   (12)

p(τkl|τk−1,l)=λe−λ(Tl−Tl−1)                                                (13)

where  is the cluster arrival rate and  is the ray arrival rate.

4.1 Number of clusters

The number of clusters found in different indoor environments varies between 2 and 7. In [5], the average number of clusters was found to be 3 for one building, and 7 for another building. In [7] the number of clusters reported was found to be 2 for line-of-sight (LOS) and 5 for non-LOS (NLOS) conditions. We find these reported values consistent, since they depend on the type of indoor environment.

By carefully analyzing A-E delay profiles, we manually identified 3-6 clusters in each one. Figure 1 shows Model A delay profile with clusters outlined by exponential decay (straight line on a log-scale).

Figure 1. Model A delay profile with cluster extension (overlapping clusters).

Clearly, three clusters can be easily identified. The number of clusters found agrees well with the results reported in the literature. For Models A, B, C, D, and E we identified 3, 4, 6, 6 (plus spike), and 5 clusters, respectively.

We extended each cluster in A-E models so that they overlap (see Fig.1). We use a straight line extrapolation function (in dB) on the first few visible taps of each cluster. The powers of overlapping taps were calculated so that the total sum of the powers of overlapping taps corresponding to different clusters equals to the powers of the original A-E power delay profiles. The procedure is described in detail in Appendix A.

##### 4.2 PAS Shape

The angle of arrival statistics within a cluster were found to closely match the Laplacian distribution [5,6,10]

p(θ)=12σe−2θσ                                                  (14)

where   is the standard deviation of the PAS (which corresponds to the numerical value of AS). The Laplacian distribution is shown in Fig. 2 (a typical simulated AoA (AoD) distribution within a cluster, with AS = 30o).

Figure 2. Example of Laplacian AoA (AoD) distribution, AS = 30o.

##### 4.3 Mean AoA (AoD) of Each Cluster

It was found in [5,6] that the relative cluster mean AoAs have a random uniform distribution over all angles. We use the symbol ϑ0 for the mean AoA in later sections.

4.4 Tap Time and Angle Dependence

We assume that the channel impulse response as a function of time and azimuth is a separable function, h, [5,6]

h(t,θ)=h(t)h(θ)                                                     (15)

where t is time and  is an azimuth angle.

4.5 AS of Each Cluster

In [5] the mean cluster AS values were found to be 21o and 25o for two buildings measured. In [6] the mean AS value was found to be 37o. To be consistent with these findings, we select the mean cluster AS values for models A-E in the 20o to 40o range. To assign an AS value to each cluster within a particular model, we use observations from outdoor channels. For outdoor environments, it was found that the cluster rms delay spread (DS) is highly correlated (0.7 correlation coefficient) with the AS [24]. It was also found that the cluster rms delay spread and AS can be modeled as correlated log-normal random variables. We apply this intuitive finding to our model using the following procedure (we note that the DS are calculated from the experimental data and AS values are determined following the procedure described below)

- Calculate rms DS of each cluster, convert to dB values.

- For each model, calculate the mean rms DS and its corresponding standard deviation, d (dB).

- Determine the mean AS proportionally (linear dependence) to the mean rms DS values using the following formula (the resulting mean AS is in the 20o to 40o range)

AS=0.57DS+3.65  (dB)                                          (16)

The model-dependent cluster DS and AS can be represented as log-normal random variables in the following form

DS=DS+σdx                                                   (17)

AS=AS+σay                                                    (18)

where x and y are zero-mean, unit-variance Gaussian random variables and d and a are standard deviations, respectively.

- Assume that AS standard deviation is the same as the rms DS standard deviation (a = d ), and that the correlation coefficient between the Gaussian random variables x and y is 0.7.

- Using the following formula we can determine y, with x known

y=ρx+1−ρ2z                                                     (19)

where  is the correlation coefficient and z is an independent zero-mean unit-variance Gaussian random variable.

The above procedure results in a lower AS for models with lower rms delay spread and larger AS for models with larger rms delay spread. The resulting AS and DS at the receive side are shown in Fig. 3 for all models and clusters. For the transmit side, an independent set of AS was generated and it looks similar to the results in Fig. 3 (cluster AS at receiver does not have to necessarily match the cluster AS at the transmitter).

Figure 3. Cluster AS (at the receiver) and DS for all five models (A-E).

4.6 AS and AoA of Each Tap Within Each Cluster

The question that is addressed here is how to set the tap angular properties (Angle of Arrival – AoA - ϑ0,iand Azimuth Spread - AS - σθ,i2) so that the cumulative effect of all the taps in a given cluster satisfies the cluster angular properties (mean AoA ϑ0 and composite AS σtot2 (modeled in equation (18)).

Let the angular distribution of tap i be

pΘ,i(ϑ)=f(ϑ,ϑ0,i,σθ,i2)                                                   (20)

where f is a probability density function in the azimuth domain, i.e. it is non negative and integrates to 1 over [0, 2], and its mean and variance are respectivelyϑ0,iandσθ,i2.

In setting the angular properties, the following constraints should be satisfied

- Each tap should exhibit a Laplacian PAS, and the cluster it is part of, through the summation of its contributing taps, should also exhibit a Laplacian PAS.

- The AS of the clusters determined in Sec. 4.5 should be used.

- All the taps of a given cluster should have the same AS σθ,i2.

- The AoAs of spatially adjacent taps should be smaller than the tap AS σθ,i2 so as to prevent their resolution in the azimuth domain.

Graphically, these constraints can be represented by Fig. 4.

Figure 4. Distribution of taps within a cluster.

Based on these constraints, it is shown in Appendix B that the conditions to fulfill are

ϑ0=i=1Nβiϑ0,i                                                            (21)

σtot2=i=1Nβiϑ0,i2+σϑ,i2−ϑ02                                                  (22)

where βi is a scaling parameter related to the power roll-off coefficient of the cluster (see appendix B) and N is the number of taps of the cluster.

Without loss of generality, (21) and (22) can be rewritten as

ϑ0=i=1Nβi(ϑ0−liΔ)                                                (23)

and

σtot2=i=1Nβi(ϑ0−liΔ)2+σϑ,i2−ϑ02                                  (24)

where li is a zero-mean, unit-variance Laplacian random variable (introduced to satisfy the first angular constraint presented in the beginning of this section, i.e. each cluster should have a Laplacian PAS) and  is a parameter that we determine using numerical global search to satisfy the required AS and mean AoA of each cluster.

5. Tables of Model Parameters

Tables for Model E have to be updated

In Appendix C, tables show a summary of all model parameters required for the complete channel characterization, determined using methods described in Sec. 4. Each of the five tables representing each of the models A-E is clearly separated into distinct clusters. For each tap within a given cluster, the following parameters are listed

- Power

- AoA

- AS (of arrival, i.e. at the receiver)

- AoD

- AS (of departure, i.e. at the transmitter)

The cluster-related information is also included (although it is not required as an input parameter to the Matlab program that generates the channel matrix): mean AoA and corresponding AS (as seen from the receive side), mean AoD and corresponding AS (as seen from the transmit side).

In model D, the first tap contains also a fixed signal component, with the Ricean K-factor equal to 10 dB (see (3)). We assume that the fixed signal component impinges the antenna array at the angle of 45o.

6. Matlab Program

The Matlab program that generates multiple snapshots (in time) of the channel matrix H was written by Schumacher [23] where the program description and downloading information can be found.

7. Simulated MIMO Channel Properties Using Matlab Program

In this section we determine some of the important properties of the simulated channel matrices H, specifically antenna correlation and channel capacity. To arrive at the results, we use the narrowband assumption which implies that the signal seen at the receiver is a summation of all taps. This assumption is valid, for example, for systems based on the orthogonal frequency division multiplexing (OFDM) modulation. For the simulation, we use the following antenna configuration system

- 4 transmit and 4 receive antennas (4 x 4 MIMO system)

- Uniform linear array (ULA)

- /2 adjacent antenna spacing

- Isotropic antennas

- No antenna coupling effect

- All antennas with same polarization (vertical)

7.1 Antenna Correlation Results

Results missing (to be included)

The correlation coefficient between any two variables, y1 and y2, with zero mean and standard deviations of y1 and y2, respectively, is given by

ρ=y1y2−y1y2σy1σy2					(25)

In (25) if the variables y1 and y2 represent complex signals, the correlation coefficient  denotes the complex envelope correlation coefficient. Following are the correlation matrices for all five models, for both transmit and receive side.

Model A correlation matrices (Tx and Rx)

ρrx=; ρ=

Model B correlation matrices (Tx and Rx)

ρtx=;ρrx=

Model C correlation matrices (Tx and Rx)

ρtx=;ρrx=

Model D correlation matrices (Tx and Rx)

ρtx=;ρrx=

Model E correlation matrices (Tx and Rx)

ρtx=;ρrx=

7.2 Channel Capacity Results

##### Results missing (to be included)

Channel capacity is defined as the highest transfer rate of information that can be sent with arbitrary low probability of error. We assume that the channel is known only at the receiver and that equal power is radiated from each transmitting antenna. For the nt transmit antennas with equal transmit power and nr receiving antennas the generalized formula for the theoretical capacity, C, can be derived [1]

C = log2 det [ I  + (r/nt) HH† ]    bps/Hz                                    (26)

where I is an nr x nr identity matrix, H is an nr x nt matrix, H† is its transpose conjugate, and r

is the average signal-to-noise ratio (SNR).

Figure 5 shows cumulative distribution functions (CDF) for all five models assuming r = 15 dB and 1000 channel realizations.

Figure Missing

Figure 5. CDFs of channel capacity for all five models A-E

8. Conclusion

In this document we proposed a set of channel models applicable to indoor MIMO WLAN systems. The channel models are an extension of the SISO WLAN channel models proposed by Medbo et. al. [2,3]. The newly developed models are based on the cluster modeling approach, where tap-dependent and cluster-dependent angular and power properties are characterized. Based on these parameters, an accurate time-domain MIMO channel matrix can be obtained from the Matlab program in [23], with proper antenna correlation properties.

# Appendix A – Power roll-off parameter extraction

The objective of this Appendix is to describe a method to extend the Single-Input Single-Output (SISO) indoor Power Delay Profiles (PDP) proposed in [2] so as to encompass Multiple-Input Multiple-Output (MIMO) scenarios as well. This description will successively address the identification of clusters in the above mentioned clusters and the extraction of their power roll-off coefficient, the definition of the statistical angular properties of both taps and clusters, the way to compute the correlation of a tap resulting from several overlapping clusters and the way to perform Monte Carlo simulations with the proposed model. In the appendix, one will find the proposed spatial extension of the fives models of [2].

Power roll-off parameter extraction is the iterative process that finds the power roll-off coefficients for each cluster.

It starts with one of the five original models [2] that gives the channel gain Ai at each delay i, i=1, …, Nmax. For the Medbo models, Nmax = 18. The clusters are separated such that

- Rule#1: the tap separation (in time) is constant within each cluster. It is interesting to observe that in the Medbo models the power roll-off within each cluster follows an exponential power decay law.

- Rule#2: the clusters which are the outcome of the iterative process aimed at computing the power roll-off parameters should also obey the exponential power decay law. It has been observed that, in Medbo Model B, a blind application of Rule#1 could lead to the definition of clusters whose power roll-off was actually increasing. Rule#2 is aimed at avoiding this situation. Its fulfilment requires in Medbo Model B to gather the four last taps, whose separation in time is not exactly constant, into the same cluster.

- Rule#3: the Line-of-Sight component of a Rician profile (Medbo Model D) is handled as being a cluster with a single tap.

The power azimuth within each cluster is assumed to follow a Laplacian distribution and is associated with a different mean angle of arrival (AoA) and a different composite angular spread (AS), i.e. different mean and variance of the Laplacian distribution (the Laplacian distribution agrees with results reported in the literature [5,6,10]).

This process has been implemented in Matlab, and we illustrate it with figures as an example. In this example the original taps are given as solid arrows with height proportional to the tap gain, the horizontal axis is excess delay, different clusters are assigned different colors to illustrate the fact that they correspond to different directional characteristics.

Next we describe the Matlab program functions:

Let's affect the working variable C with the original gains of the Medbo model C←A.

k=1

Step 1: Do a least squares fit for cluster k.

We approximate the power roll-off law with an exponential power delay profile, i.e. with a function of the form Ci,k=C0,k+γk(τi,k−τ0,k), where Ci,kis the power of the i-th tap in the current cluster k, C0,k, γk are the curve fitting parameters. Ci,k, C0,k are given in dB, the time delays are given in nanoseconds (nsec) and the units of γk are dB/nsec.

If this was not the last cluster proceed. Else, break.

Step 2: Extend the cluster

Following the power roll-off law found in Step 1, extend the power roll-off at taps spaced with the same separation as the one that the original cluster had. We stop introducing these extended taps when their power would be lower than the lowest allowable tap power for that profile. The original taps remain unaltered and the extensions are shown with thin shaded arrows.

Step 3: Integrate energy

Since the taps of the original model are given at specific excess delays and since we are willing to preserve this spacing in the delay domain, we need to find how much energy of the extended taps corresponds to the each of the original taps of the following clusters. These are shown as ‘fat’ shaded arrows because they result from the integration of one (or possibly more) thin shaded arrows. Let the fat arrows be denoted as Bl,k for all delays τlbeyond max(τi,k).

Step 4: Subtract energy

If the energy of the accumulated taps is lower than the energy of the original model taps, then it is subtracted (here the term ‘energy’ indicates that the tap powers have been appropriately integrated to account for the varying tap delay spacing). In the angular domain, this corresponds to having energy arriving both from the direction of the cluster to which the current tap belongs, and from the direction of the previous cluster(s). SoCj,l←Cj,l−Bj,k.

Because we have could have 3 or more clusters overlapping and energy might have to be subtracted more than once, we operate on the working variable C, instead of the initial channel gains A. This process is repeated for all clusters in the profile. Note that the subtraction is done in linear scale, not in dB.

The subtracted energy is denoted with arrows with dashed borders, while the result of the subtraction (remaining energy) is shown with patterned arrows.

k=k+1

Repeat steps 1-4 until break in step 1 (because last cluster reached).

Let us see how this procedure can be illustrated graphically using the following model as an example.

AoA2

AoA3

AoA1

Figure 1: Original model

AoA1

Figure 2: Step 1 (least squares fit) for cluster 1

Not considered because lower than minimum tap power

Minimum tap power

I

Figure 3: Step 2 (tap extension) for cluster 1

Figure 4: Step 3 (energy integration) for cluster 1

Figure 5: Step 4 (energy subtraction) for cluster 2

Figure 6: Step 1 (least squares fit) for cluster 2

Minimum tap power

Not considered because lower than minimum tap power

I

II

III

Figure 7: Step 2 (tap extension) for cluster 2

I

II

III

Not subtracted because higher than current tap power

Figure 8: Step 3 (energy integration) for cluster 2

Figure 9: Step 4 (energy subtraction) for cluster 3

Figure 10: Step 1 (least squares fit) for cluster 3

Appendix B – Tap and cluster angular properties

We are going to assume that all the taps within a cluster are characterized by probability functions of the same form f, and we constrain them to have the same variance σθ,i2, however we allow them to have different means ϑ0,i.

The angular distribution of the cluster is then

pΘ,tot(ϑ)=1PtotaliPipΘ,i(ϑ)=1PtotaliPif(ϑ,ϑ0,i,σθ,i2)

Pi is the power assigned to tap i, and Ptotal is the total power of the cluster:Ptotal=iPi.

If we assume exponential power roll off over regularly spaced taps, then

Pi=P0αi−1,Ptotal=P01−αN1−α where N is the number of taps in the cluster.

In general let us define the scaling parameters βi=PiPtotal. Specifically in the case of exponential power roll-off βiexp=(1−α)1−αNαi−1

Assume that the goal is to match the first and second order moments, i.e. that we want to impose a certain mean angle of arrival and angular spread on ptot,Θ(ϑ).

Calculation of the mean

ϑ=02πϑpΘ,tot(ϑ)dϑ=02πϑi=1NβipΘ,i(ϑ)dϑ=i=1Nβi02πϑpΘ,i(ϑ)dϑ=i=1Nβiϑ0,i

The constraint ϑ=i=1Nβiϑ0,i=ϑ0 defines a hyper-plane in the N-dimensional space ϑ0,1,…,ϑ0,N⊂0,2πN.

Calculation of the variance

To derive the last form of the expression we have used the fact that all the taps are assumed to have the same angular spread. The variance constraint can be written as

σtot2+ϑ02−σϑ,i2=i=1Nβiϑ0,i2

and therefore we have a second constraint on the mean angles of arrival of the taps.

It is interesting to point out that the solution σϑ,i2=σtot2,ϑ0,i=ϑ0 (same statistical characteristics per tap as per cluster) always satisfies the constraints, however it is not unique.

Appendix C – Extended Medbo Model A

|  | Tap index | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Excess delay [ns] | 0 | 10 | 20 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 110 | 140 | 170 | 200 | 240 | 290 | 340 | 390 |
| Cluster 1 | Power [dB] | 0 | -0.9 | -1.7 | -2.6 | -3.5 | -4.3 | -5.2 | -6.1 | -6.9 | -7.8 | -9.0712 | -11.1991 | -13.7954 | -16.3918 | -19.3710 | -23.2017 |  |  |
| Mean AoA = 154.4757° | AoA [°] | 166.646 | 172.6334 | 156.1558 | 150.0347 | 148.4607 | 109.7505 | 121.8078 | 152.7446 | 171.0199 | 155.2886 | 156.0954 | 159.6128 | 154.2316 | 149.2625 | 154.386 | 153.8697 |  |  |
| Composite AS = 18.4380° | AS [°] | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 |  |  |
| Mean AoD = 333.7349° | AoD [°] | 346.8507 | 320.9888 | 345.5986 | 292.1785 | 342.3962 | 350.6262 | 311.292 | 349.5338 | 336.1867 | 326.1379 | 350.5996 | 353.77 | 349.6664 | 19.2542 | 359.4043 | 325.516 |  |  |
| Composite AS = 19.7471 | AS [°] | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 |  |  |
| Cluster 2 | Power [dB] |  |  |  |  |  |  |  |  |  |  | -6.6756 | -9.5729 | -12.1754 | -14.7779 | -17.4358 | -21.9928 | -25.5807 |  |
| Mean AoA = 320.5062° | AoA [°] |  |  |  |  |  |  |  |  |  |  | 315.8 | 315.8042 | 2.6764 | 273.5867 | 347.9928 | 323.2924 | 328.8476 |  |
| Composite AS = 21.8956° | AS [°] |  |  |  |  |  |  |  |  |  |  | 5 | 5 | 5 | 5 | 5 | 5 | 5 |  |
| Mean AoD = 50.3512° | AoD [°] |  |  |  |  |  |  |  |  |  |  | 68.6751 | 27.4382 | 47.1456 | 15.6474 | 50.4085 | 39.5665 | 52.1188 |  |
| Composite AS = 20.4451 | AS [°] |  |  |  |  |  |  |  |  |  |  | 5 | 5 | 5 | 5 | 5 | 5 | 5 |  |
| Cluster 3 | Power [dB] |  |  |  |  |  |  |  |  |  |  |  |  |  |  | -18.8433 | -23.2381 | -25.2463 | -26.7 |
| Mean AoA = 248.58° | AoA [°] |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 239.5305 | 238.1292 | 251.7416 | 322.5986 |
| Composite AS = 24.6891° | AS [°] |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5 | 5 | 5 | 5 |
| Mean AoD = 266.6654° | AoD [°] |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 249.4722 | 264.9037 | 318.4249 | 303.1991 |
| Composite AS = 25.8612 | AS [°] |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5 | 5 | 5 | 5 |

Appendix C - Extended Medbo Model B (1/2)

|  | Tap index | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Excess delay [ns] | 0 | 10 | 20 | 30 | 50 | 80 | 110 | 140 | 180 | 230 | 280 | 330 | 380 | 430 | 490 | 560 | 640 | 730 |
| Cluster 1 | Power [dB] | -2.6 | -3.0 | -3.5 | -3.9 | -4.5644 | -5.6552 | -6.9752 | -8.2952 | -9.8222 | -11.7855 | -13.9855 | -16.1855 | -18.3855 | -20.5855 | -22.9852 |  |  |  |
| Mean AoA = 162.9601° | AoA [°] | 127.5818 | 158.5207 | 168.2946 | 219.1601 | 148.2392 | 174.5993 | 164.8998 | 156.7871 | 146.8169 | 174.6295 | 165.4558 | 160.8233 | 151.3440 | 162.7868 | 148.0195 |  |  |  |
| Composite AS = 26.7406° | AS [°] | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 |  |  |  |
| Mean AoD = 109.1325° | AoD [°] | 109.0127 | 155.0036 | 86.8503 | 121.2069 | 118.5146 | 55.0095 | 101.8253 | 102.9032 | 92.9890 | 124.7515 | 61.8749 | 77.3237 | 87.2695 | 74.6816 | 80.3405 |  |  |  |
| Composite AS = 27.9015 | AS [°] | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 |  |  |  |
| Cluster 2 | Power [dB] |  |  |  |  | -1.8681 | -3.2849 | -4.5734 | -5.8619 | -7.1920 | -9.9304 | -10.3438 | -14.3537 | -14.7671 | -18.7770 | -19.9822 | -22.4464 |  |  |
| Mean AoA = 243.7232° | AoA [°] |  |  |  |  | 215.4219 | 298.6114 | 242.7346 | 228.0415 | 232.6356 | 248.199 | 251.7176 | 216.9336 | 229.4277 | 235.1307 | 239.516 | 172.6898 |  |  |
| Composite AS = 31.1942° | AS [°] |  |  |  |  | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 |  |  |
| Mean AoD = 291.6602° | AoD [°] |  |  |  |  | 320.6981 | 315.7981 | 264.9743 | 240.0597 | 256.9901 | 317.0863 | 250.6818 | 337.0123 | 265.3312 | 284.1269 | 264.5758 | 270.7322 |  |  |
| Composite AS = 32.7862 | AS [°] |  |  |  |  | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 |  |  |

Appendix C - Extended Medbo Model B (2/2)

| Cluster 3 | Power [dB] |  |  |  |  |  |  |  |  | -7.9045 | -9.6852 | -14.2606 | -13.8128 | -18.6038 | -18.1924 | -22.8346 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mean AoA = 73.6586° | AoA [°] |  |  |  |  |  |  |  |  | 100.9991 | 63.7968 | 37.0299 | 33.7379 | 42.3082 | 65.5237 | 115.7368 |  |  |  |
| Composite AS = 27.1777° | AS [°] |  |  |  |  |  |  |  |  | 5 | 5 | 5 | 5 | 5 | 5 | 5 |  |  |  |
| Mean AoD = 50.8536° | AoD [°] |  |  |  |  |  |  |  |  | 72.2767 | 8.2047 | 62.233 | 58.1658 | 96.8573 | 38.7527 | 38.016 |  |  |  |
| Composite AS = 28.9514 | AS [°] |  |  |  |  |  |  |  |  | 5 | 5 | 5 | 5 | 5 | 5 | 5 |  |  |  |
| Cluster 4 | Power [dB] |  |  |  |  |  |  |  |  |  |  |  |  |  |  | -20.6734 | -20.5744 | -20.7000 | -24.6000 |
| Mean AoA = 172.6548° | AoA [°] |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 213.3961 | 148.3145 | 169.6745 | 140.8493 |
| Composite AS = 28.5462° | AS [°] |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5 | 5 | 5 | 5 |
| Mean AoD = 270.1223° | AoD [°] |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 280.6038 | 274.819 | 231.8822 | 326.2361 |
| Composite AS = 29.4686 | AS [°] |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5 | 5 | 5 | 5 |

Appendix C – Extended Medbo Model C (1/2)

|  | Tap index | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Excess delay [ns] | 0 | 10 | 20 | 30 | 50 | 80 | 110 | 140 | 180 | 230 | 280 | 330 | 400 | 490 | 600 | 730 | 880 | 1050 |
| Cluster 1 | Power [dB] | -3.3 | -3.6 | -3.9 | -4.2 | -4.6474 | -5.3931 | -6.2931 | -7.1931 | -8.2317 | -9.5793 | -11.0793 | -12.5793 | -14.3586 | -16.7311 | -19.9788 |  |  |  |
| Mean AoA = 304.4244° | AoA [°] | 252.3473 | 329.0832 | 318.4754 | 341.8683 | 257.0568 | 301.1511 | 321.2222 | 308.4064 | 356.0741 | 291.0914 | 354.6881 | 302.7826 | 315.5956 | 276.0021 | 317.5995 |  |  |  |
| Composite AS = 33.2826° | AS [°] | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 |  |  |  |
| Mean AoD = 56.4767° | AoD [°] | 25.5904 | 71.8642 | 36.2501 | 41.4398 | 49.9058 | 51.8931 | 160.9314 | 22.7117 | 61.5897 | 149.9837 | 35.9822 | 31.6412 | 63.0848 | 354.4689 | 72.139 |  |  |  |
| Composite AS = 38.7578° | AS [°] | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 |  |  |  |
| Cluster 2 | Power [dB] |  |  |  |  | -1.8242 | -2.8069 | -3.5528 | -4.4528 | -5.3076 | -7.4276 | -7.0894 | -10.3048 | -10.4441 | -13.8374 | -15.7621 | -19.9403 |  |  |
| Mean AoA = 167.1633° | AoA [°] |  |  |  |  | 213.1614 | 196.2415 | 148.7596 | 80.5620 | 151.613 | 215.5459 | 130.3201 | 160.7522 | 139.8534 | 76.6022 | 301.5468 | 167.2596 |  |  |
| Composite AS = 46.9604° | AS [°] |  |  |  |  | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 |  |  |
| Mean AoD = 183.5001° | AoD [°] |  |  |  |  | 208.3534 | 95.0112 | 195.5478 | 212.7959 | 214.9096 | 201.2451 | 168.0821 | 185.7938 | 204.7017 | 243.4922 | 183.7118 | 198.2884 |  |  |
| Composite AS = 43.7531° | AS [°] |  |  |  |  | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 |  |  |
| Cluster 3 | Power [dB] |  |  |  |  |  |  |  |  | -5.7960 | -6.7737 | -10.4758 | -9.6417 | -14.1072 | -12.7523 | -18.5033 |  |  |  |
| Mean AoA = 84.9886° | AoA [°] |  |  |  |  |  |  |  |  | 33.6647 | 124.3742 | 115.7943 | 84.1441 | 101.3577 | 107.7504 | 136.0386 |  |  |  |
| Composite AS = 39.4883° | AS [°] |  |  |  |  |  |  |  |  | 5 | 5 | 5 | 5 | 5 | 5 | 5 |  |  |  |
| Mean AoD = 141.9504° | AoD [°] |  |  |  |  |  |  |  |  | 95.9055 | 167.3442 | 176.0734 | 164.2723 | 142.105 | 157.3876 | 175.7435 |  |  |  |
| Composite AS = 34.0966° | AS [°] |  |  |  |  |  |  |  |  | 5 | 5 | 5 | 5 | 5 | 5 | 5 |  |  |  |

Appendix C – Extended Medbo Model C (2/2)

| Cluster 4 | Power [dB] |  |  |  |  |  |  |  |  |  |  |  |  | -8.8825 | -13.3195 | -18.7334 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mean AoA = 221.9758° | AoA [°] |  |  |  |  |  |  |  |  |  |  |  |  | 206.9324 | 260.1162 | 234.6622 |  |  |  |
| Composite AS = 23.4629° | AS [°] |  |  |  |  |  |  |  |  |  |  |  |  | 5 | 5 | 5 |  |  |  |
| Mean AoD = 136.5628° | AoD [°] |  |  |  |  |  |  |  |  |  |  |  |  | 121.8181 | 176.0053 | 141.8338 |  |  |  |
| Composite AS = 23.7804° | AS [°] |  |  |  |  |  |  |  |  |  |  |  |  | 5 | 5 | 5 |  |  |  |
| Cluster 5 | Power [dB] |  |  |  |  |  |  |  |  |  |  |  |  |  |  | -12.9472 | -14.2338 |  |  |
| Mean AoA = 63.665° | AoA [°] |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 40.5049 | 94.8107 |  |  |
| Composite AS = 27.3867° | AS [°] |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5 | 5 |  |  |
| Mean AoD = 296.1429° | AoD [°] |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 275.9985 | 323.2332 |  |  |
| Composite AS = 24.0115° | AS [°] |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5 | 5 |  |  |
| Cluster 6 | Power [dB] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | -16.3 | -21.2 |
| Mean AoA = 269.2786° | AoA [°] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 253.9274 | 316.7185 |
| Composite AS = 27.482° | AS [°] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5 | 5 |
| Mean AoD = 84.8383° | AoD [°] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 99.6363 | 39.108 |
| Composite AS = 26.6085° | AS [°] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5 | 5 |

Appendix C – Extended Medbo Model D (1/3)

|  | Tap index | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Excess delay [ns] | 0 | 10 | 20 | 30 | 50 | 80 | 110 | 140 | 180 | 230 | 280 | 330 | 400 | 490 | 600 | 730 | 880 | 1050 |
| Cluster 1 | Power [dB] | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Mean AoA = 45° | AoA [°] | 45 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Composite AS = 35° | AS [°] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Mean AoD = 45 ° | AoD [°] | 45 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Composite AS = | AS [°] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Cluster 2 | Power [dB] |  | -10.0000 | -10.3000 | -10.6000 | -11.0474 | -11.7931 | -12.6931 | -13.5931 | -14.6371 | -15.9793 | -17.4793 | -18.9793 | -20.7586 | -23.1311 | -26.3788 |  |  |  |
| Mean AoA = 69.8125° | AoA [°] |  | 42.6958 | 76.8408 | 142.7898 | 51.1934 | 82.5427 | 48.9202 | 58.9212 | 40.4445 | 70.2553 | 3.4779 | 69.4643 | 89.9798 | 64.8432 | 53.7695 |  |  |  |
| Composite AS = 34.4239° | AS [°] |  | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 |  |  |  |
| Mean AoD = 139.638° | AoD [°] |  | 208.5864 | 129.9123 | 95.8493 | 130.1839 | 156.9079 | 136.712 | 138.0347 | 45.5993 | 138.8145 | 172.7297 | 162.107 | 148.3352 | 140.3911 | 120.373 |  |  |  |
| Composite AS = 40.6025° | AS [°] |  | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 |  |  |  |
| Cluster 3 | Power [dB] |  |  |  |  | -8.2242 | -9.0528 | -9.9528 | -10.8528 | -11.7072 | -13.8478 | -13.5348 | -16.7764 | -16.9452 | -20.3795 | -22.3534 | -27.1281 |  |  |
| Mean AoA = 233.5275° | AoA [°] |  |  |  |  | 230.8113 | 212.0278 | 240.1067 | 268.3928 | 171.0819 | 346.1935 | 199.6969 | 282.978 | 214.4647 | 209.0362 | 276.3747 | 204.0853 |  |  |
| Composite AS = 40.5022° | AS [°] |  |  |  |  | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 |  |  |
| Mean AoD = 240.6583° | AoD [°] |  |  |  |  | 280.2179 | 187.8347 | 283.9894 | 186.6995 | 190.4158 | 281.8153 | 253.4861 | 265.676 | 261.3655 | 304.0844 | 276.3525 | 272.576 |  |  |
| Composite AS = 44.9177° | AS [°] |  |  |  |  | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 |  |  |

Appendix C – Extended Medbo Model D (2/3)

| Cluster 4 | Power [dB] |  |  |  |  |  |  |  |  | -12.1964 | -13.1564 | -16.7786 | -15.9811 | -20.4031 | -19.0101 | -24.7060 | -26.5976 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mean AoA = 282.2856° | AoA [°] |  |  |  |  |  |  |  |  | 318.9201 | 267.5696 | 285.034 | 203.8138 | 340.9729 | 283.6523 | 256.3295 | 264.0007 |  |  |
| Composite AS = 39.7143° | AS [°] |  |  |  |  |  |  |  |  | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 |  |  |
| Mean AoD = 119.3561° | AoD [°] |  |  |  |  |  |  |  |  | 107.2554 | 178.5724 | 53.2223 | 78.0038 | 120.9579 | 108.4593 | 201.765 | 184.2837 |  |  |
| Composite AS = 44.0073° | AS [°] |  |  |  |  |  |  |  |  | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 |  |  |
| Cluster 5 | Power [dB] |  |  |  |  |  |  |  |  |  |  |  |  | -15.2445 | -19.7603 | -25.2355 |  |  |  |
| Mean AoA = 263.8942° | AoA [°] |  |  |  |  |  |  |  |  |  |  |  |  | 263.6709 | 284.45 | 193.6027 |  |  |  |
| Composite AS = 21.6545° | AS [°] |  |  |  |  |  |  |  |  |  |  |  |  | 5 | 5 | 5 |  |  |  |
| Mean AoD = 348.0849° | AoD [°] |  |  |  |  |  |  |  |  |  |  |  |  | 331.9687 | 24.9298 | 18.927 |  |  |  |
| Composite AS = 24.6078° | AS [°] |  |  |  |  |  |  |  |  |  |  |  |  | 5 | 5 | 5 |  |  |  |
| Cluster 6 | Power [dB] |  |  |  |  |  |  |  |  |  |  |  |  |  |  | -19.2793 | -21.6504 | -25.0036 |  |
| Mean AoA = 115.3453° | AoA [°] |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 85.9242 | 127.16 | 199.6978 |  |
| Composite AS = 39.6946° | AS [°] |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5 | 5 | 5 |  |
| Mean AoD = 50.0383° | AoD [°] |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 59.388 | 3.8574 | 115.0574 |  |
| Composite AS = 36.8788° | AS [°] |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5 | 5 | 5 |  |

Appendix C – Extended Medbo Model D (3/3)

| Clu ster 7 | Power [dB] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | -26.5548 | -27.6000 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mean AoA = 143.431° | AoA [°] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 116.0443 | 178.2697 |
| Composite AS = 31.3348° | AS [°] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5 | 5 |
| Mean AoD = 314.9165° | AoD [°] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 340.5604 | 282.2948 |
| Composite AS = 29.4897° | AS [°] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5 | 5 |

Appendix C – Extended Medbo Model E (1/2)

|  | Tap index | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Excess delay [ns] | 0 | 10 | 20 | 40 | 70 | 100 | 140 | 190 | 240 | 320 | 430 | 560 | 710 | 880 | 1070 | 1280 | 1510 | 1760 |
| Cluster 1 | Power [dB] | -4.9 | -5.1 | -5.2 | -5.4410 | -5.8149 | -6.2649 | -6.7884 | -7.4615 | -8.2115 | -9.1781 | -10.5908 | -12.3805 | -14.4684 | -16.8547 | -19.07 |  |  |  |
| Mean AoA = 278.8470° | AoA [°] | 286.6401 | 359.4242 | 9.0857 | 142.8135 | 276.773 | 285.4516 | 248.2183 | 292.8891 | 312.5784 | 208.5826 | 222.4923 | 251.1836 | 272.2969 | 168.851 | 231.0657 |  |  |  |
| Composite AS = 66.5703° | AS [°] | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 |  |  |  |
| Mean AoD = 261.4363° | AoD [°] | 300.0575 | 280.8147 | 218.0454 | 218.1081 | 2.3426 | 236.5473 | 277.1004 | 288.1454 | 296.1742 | 109.9208 | 211.7947 | 255.8834 | 171.1455 | 299.9609 | 121.7108 |  |  |  |
| Composite AS = 57.3116 | AS [°] | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 |  |  |  |
| Cluster 2 | Power [dB] |  |  |  | -2.6275 | -3.1950 | -3.8793 | -4.4806 | -6.2842 | -5.5581 | -7.7986 | -10.0752 | -11.8031 | -15.3453 | -18.0825 |  |  |  |  |
| Mean AoA = 138.6095° | AoA [°] |  |  |  | 87.0957 | 157.807 | 193.597 | 146.7827 | 103.4799 | 119.776 | 190.3872 | 255.2108 | 297.1455 | 124.3463 | 182.0015 |  |  |  |  |
| Composite AS = 54.1153° | AS [°] |  |  |  | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 |  |  |  |  |
| Mean AoD = 124.8345° | AoD [°] |  |  |  | 158.8441 | 122.368 | 196.4408 | 99.8697 | 50.6864 | 54.413 | 125.9409 | 97.6148 | 144.864 | 159.7329 | 84.7008 |  |  |  |  |
| Composite AS = 46.937 | AS [°] |  |  |  | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 |  |  |  |  |
| Cluster 3 | Power [dB] |  |  |  |  |  |  | -4.3490 | -4.6358 | -7.2689 | -8.5907 | -12.3097 | -13.3736 | -19.4226 |  |  |  |  |  |
| Mean AoA = 298.134° | AoA [°] |  |  |  |  |  |  | 305.231 | 329.2409 | 214.1191 | 323.139 | 282.1011 | 343.8194 | 108.5057 |  |  |  |  |  |
| Composite AS = 44.8669° | AS [°] |  |  |  |  |  |  | 5 | 5 | 5 | 5 | 5 | 5 | 5 |  |  |  |  |  |
| Mean AoD = 356.9121° | AoD [°] |  |  |  |  |  |  | 312.8445 | 42.0589 | 0.5141 | 344.0166 | 33.7395 | 0.9365 | 306.4040 |  |  |  |  |  |
| Composite AS = 37.1407 | AS [°] |  |  |  |  |  |  | 5 | 5 | 5 | 5 | 5 | 5 | 5 |  |  |  |  |  |

Appendix C – Extended Medbo Model E (1/2)

| Cluster 4 | Power [dB] |  |  |  |  |  |  |  |  |  | -2.4047 | -3.9649 | -7.2082 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mean AoA = 90.4661° | AoA [°] |  |  |  |  |  |  |  |  |  | 106.255 | 53.2052 | 121.374 |  |  |  |  |  |  |
| Composite AS = 27.9905° | AS [°] |  |  |  |  |  |  |  |  |  | 5 | 5 | 5 |  |  |  |  |  |  |
| Mean AoD = 174.5461° | AoD [°] |  |  |  |  |  |  |  |  |  | 171.8069 | 152.8647 | 228.5776 |  |  |  |  |  |  |
| Composite AS = 25.8627 | AS [°] |  |  |  |  |  |  |  |  |  | 5 | 5 | 5 |  |  |  |  |  |  |
| Cluster 5 | Power [dB] |  |  |  |  |  |  |  |  |  |  |  | -7.8303 | -6.7369 | -8.2383 | -11.1692 | -13.4 | -17.4 | -20.9 |
| Mean AoA = 320.18° | AoA [°] |  |  |  |  |  |  |  |  |  |  |  | 312.8882 | 316.5662 | 64.6901 | 326.1956 | 139.0558 | 73.1171 | 148.4194 |
| Composite AS = 81.8273° | AS [°] |  |  |  |  |  |  |  |  |  |  |  | 5 | 5 | 5 | 5 | 5 | 5 | 5 |
| Mean AoD = 321.7817° | AoD [°] |  |  |  |  |  |  |  |  |  |  |  | 303.7648 | 69.1201 | 253.5686 | 250.2892 | 254.93 | 279.5197 | 288.5935 |
| Composite AS = 75.6771 | AS [°] |  |  |  |  |  |  |  |  |  |  |  | 5 | 5 | 5 | 5 | 5 | 5 | 5 |

References

[1]  G.J. Foschini and M.J. Gans, “On the limits of wireless communications in a fading environment when using multiple antennas,” Wireless Personal Communications, vol. 6, March 1998, pp. 311-335.

[2] J. Medbo and P. Schramm, “Channel models for HIPERLAN/2,” ETSI/BRAN document no. 3ERI085B.

[3] J. Medbo and J-E. Berg, “Measured radiowave propagation characteristics at 5 GHz for typical HIPERLAN/2 scenarios,” ETSI/BRAN document no. 3ERI084A.

[4] A.A.M. Saleh and R.A. Valenzuela, “A statistical model for indoor multipath propagation,” IEEE J. Select. Areas Commun., vol. 5, 1987, pp. 128-137.

[5] Q.H. Spencer, et. al., “Modeling the statistical time and angle of arrival characteristics of an indoor environment,” IEEE J. Select. Areas Commun., vol. 18, no. 3, March 2000, pp. 347-360.

[6] R.J-M. Cramer, R.A. Scholtz, and M.Z. Win, “Evaluation of an ultra-wide-band propagation channel,” IEEE Trans. Antennas Propagat., vol. 50, no.5, May 2002, pp. 561-570.

[7] A.S.Y. Poon and M. Ho, “Indoor multiple-antenna channel characterization from 2 to 8 GHz,” submitted to ICC 2003 Conference.

[8] G. German, Q. Spencer, L. Swindlehurst, and R. Valenzuela, “Wireless indoor channel modeling: Statistical agreement of ray tracing simulations and channel sounding measurements,” in proc.  IEEE Acoustics, Speech, and Signal Proc. Conf., vol. 4, 2001, pp. 2501-2504.

[9] J-G. Wang, A.S. Mohan, and T.A. Aubrey,” Angles-of-arrival of multipath signals in indoor environments,” in proc. IEEE Veh. Technol. Conf., 1996, pp. 155-159.

[10] Chia-Chin Chong, David I. Laurenson and Stephen McLaughlin, “Statistical Characterization of the 5.2 GHz Wideband Directional Indoor Propagation Channels with Clustering and Correlation Properties,” in proc. IEEE Veh. Technol. Conf., vol. 1, Sept. 2002, pp. 629-633.

[11] J.P. Kermoal, L. Schumacher, P.E. Mogensen and K.I. Pedersen, “Experimental investigation of correlation properties of MIMO radio channels for indoor picocell scenario,“ in Proc. IEEE Veh. Technol. Conf., Boston, USA, vol. 1, Sept. 2000, pp. 14-21.

[12] P. Kyritsi and D.C. Cox, “Correlation properties of MIMO radio channels for indoor scenarios,” in proc. Signal Systems and Computers 35th Asilomar Conf., vol. 2, 2001.

[13] C. Prettie, D. Cheung, L. Rusch, and M. Ho, “Spatial correlation of UWB signals in a home environment,” in Digest of Papers, Ultra Wideband Systems and Technologies Conf., 2002, pp. 65-69.

[14] J. Kivinen, X. Zhao, and P. Vainikainen, “Empirical characterization of wideband indoor radio channel at 5.3 GHz,” IEEE Trans. Antennas and Propagation, vol. 49, no. 8, Aug. 2001, pp. 1192-1203.

[15] H. Hashemi, “The indoor radio propagation channel,” Proceedings of the IEEE, vol. 81, no. 7, July 1993, pp. 943-968.

[16] J.B. Andersen, T.S. Rappaport, and S. Yoshida, “Propagation measurements and models for wireless communication channels,” IEEE Commun. Mag., Jan. 1995, pp. 42-49.

[17] T. Zwick, C. Fisher, D. Didascalou, and W. Wiesbeck, “A stochastic spatial channel model based on wave-propagation modeling,” IEEE J. Select. Areas Commun., vol. 18, no. 1, Jan. 2000, pp. 6-15.

[18] T. Zwick, C. Fisher, and W. Wiesbeck, “A stochastic multipath channel model including path directions for indoor environments,” IEEE J. Select. Areas Commun., vol. 20, no. 6, Aug. 2002, pp. 1178-1192.

[19] J. Salz and J.H. Winters, “Effect of fading correlation on adaptive arrays in digital mobile radio,” IEEE Trans.  Veh. Technol., vol. 43, Nov. 1994, pp. 1049-1057.

[20] L. Schumacher, K. I. Pedersen, and P.E. Mogensen, “From antenna spacings to theoretical capacities – guidelines for simulating spatial correlation in MIMO systems,” submitted for publication in IEEE Trans. Antennas Propagat., Dec. 2001.

[21] A. Kamerman, “Coexistence between Bluetooth and IEEE 802.11 CCK solutions to avoid mutual interference,” Lucent Technologies Bell Laboratories, Jan. 1999.

[22] P. Soma, D.S. Baum, V. Erceg, R. Krishnamoorthy, and A.J. Paulraj, “ Analysis and modeling of multiple-input multiple-output (MIMO) radio channel based on outdoor measurements conducted at 2.5 GHz for fixed BWA applications,” in Proc. IEEE ICC Conf., New York, April 2002.

[23] L. Schumacher “Matlab program,” – to be finalized.

[24] K.I. Pedersen, P.E. Mogensen, and B.H. Fleury, “A stochastic model of the temporal and azimuthal dispersion seen at the base station in outdoor propagation environments,” IEEE Trans. Veh. Technol., vol. 49, no. 2, March 2000, pp. 437-447.