IEEE P802.11
Wireless LANs

MIB enhancement for Radio Resource Measurement

Date:	March 11, 2003

Author:	Byoung-Jo “J” Kim
	AT&T Labs-Research
	200 Laurel Ave., A5 1C32, Middletown, NJ 07748
	Phone: 732-420-9028
	
	e-Mail: macsbug@research.att.com

Abstract

This submission describes a proposed updates to the 802.11 MIB structure to include radio resource measurement variables that are being defined by TGk.

The following is the part of the MIB section referred in the motion contained in 03-171ar1.

1. Insert the marked change in the MAC part of the “Major Sections” of 802.11MIB.

dot11mac OBJECT IDENTIFIER ::= { ieee802dot11 2 }

--  MAC GROUPS

--  reference IEEE Std 802.1f-1993

--  dot11OperationTable   	    ::= { dot11mac 1 }

--  dot11CountersTable    	    ::= { dot11mac 2 }

--  dot11GroupAddressesTable 	::= { dot11mac 3 }

-- >> J Kim: TGk addition begins

--  dot11staStatsTable	   ::=    {dot11mac4}

-- >> J Kim: TGk addition ends

2. Insert the marked attributes in the SMT Station Config table entry definition

dot11staStatsTableAgingTime	     Unsigned32

dot11staStatsTableAgingTime OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute holds the minimum time in <<unit here>> to maintain entries in the dot11staStatsTable."

::= { dot11StationConfigEntry <<proper sequence #>> }

3. Insert the following MIB table after Group Address Table of the 802.11 MIB.

(including Any other editorial changed required by the insertion of these tables)

-- **********************************************************************

-- *    dot11staStatsTable TABLE

-- **********************************************************************

dot11staStatsTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11staStatsEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"The table containing statistics for each STA with

which this STA is directly communicating or not directly communicating

but observed any traffic originated from."

::= {  dot11mac 4 }

dot11staStatsEntry OBJECT-TYPE

SYNTAX Dot11staStatsEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11staStatsEntry Table.

Indexed by dot11staAddress"

INDEX {dot11staAddress}

::= {  dot11staStatsTable 1 }

Dot11staStatsEntry ::= SEQUENCE {

dot11staAddress			MacAddress,

ifIndex				Integer32,

dot11staBSSID			MacAddress,

dot11staSSID			OCTET STRING,

-- >> J Kim: The below is basically a copy of dot11CountersTable except

--	      dot11FCSErrorCounter, dot11MulticastTransmittedFrameCounter

dot11staTransmittedFragmentCount       		Counter32,

dot11staFailedCount                    Counter32,

dot11staRetryCount                     Counter32,

dot11staMultipleRetryCount             Counter32,

dot11staFrameDuplicateCount            Counter32,

dot11staRTSSuccessCount                Counter32,

dot11staRTSFailureCount                Counter32,

dot11staACKFailureCount                Counter32,

dot11staReceivedFragmentCount          Counter32,

dot11staMulticastReceivedFrameCount    Counter32,

dot11staTransmittedFrameCount          Counter32,

dot11staWEPUndecryptableCount          Counter32,

-- >> J Kim: Similarity to CountersTable ends here

dot11staReceivedByteCount              Counter32,

dot11staTransmittedByteCount           Counter32,

dot11staRSSI				Integer32,

dot11staRxSignalQuality		Integer32,

dot11staType			INTEGER,

dot11staState			INTEGER,

dot11staAID				Unsigned32,

dot11staPHYType			INTEGER,

dot11staCurrentChannel			INTEGER,

dot11staSecurityStatus			INTEGER,

-- >> J Kim: the RSSI histogram in this table is using the same format as that of 11h

-- 	      However, this is RSSI from dot11staAddress measured by this STA on this ifIndex,

--	      if measured during routine operation or requested by the dot11staAddress.

--	      RSSI histograms measured not by this STA and reported back to this STA from other STA

--	      is recorded in the dot11ReportsTable under SMT branch.

dot11staRSSIhistogram			OCTET STRING,

dot11staRSSIhistogramDuration		Unsigned32,

dot11staRSSIhistogramTime		TimeTicks,

dot11staTxRate			Unsigned32,

dot11staRxRate			Unsigned32,

dot11staRoamed			INTEGER,

dot11staLastSeen			TimeTicks}

-- >> J Kim: 11h TPC report results are kept under SMT branch

dot11staAddress OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The MACAddress of the STA/AP whose statistics are recorded

in this entry of the dot11staStatsTable."

::= { dot11staStatsEntry 1 }

dot11staBSSID OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The BSSID of the STA of this entry, if known.

If unknown, the value should be null."

::= { dot11staStatsEntry 2 }

dot11staSSID OBJECT-TYPE

SYNTAX OCTET STRING

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The SSID of the STA of this entry, if known.

If unknown, the value should be null."

::= { dot11staStatsEntry 3 }

dot11staTransmittedFragmentCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall be incremented for an MPDU

transmitted to the dot11staAddress in the current entry for

all types of fragments"

::= { dot11staStatsEntry 4 }

dot11RetryCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall increment when an MSDU is successfully

transmitted to the dot11staAddress in the current entry

after one or more retransmissions."

::= { dot11staStatsEntry 5 }

dot11staMultipleRetryCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall increment when an MSDU is successfully

transmitted after more than one retransmission to the dot11staAddress

in the current entry."

::= { dot11staStatsEntry 6 }

dot11staFrameDuplicateCount  OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall increment when a frame is received from

the dot11staAddress in the current entry

that the Sequence Control field indicates is a

duplicate."

::= { dot11staStatsEntry 7 }

dot11staRTSSuccessCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall increment when a CTS is received in

response to an RTS to the dot11staAddress in the current entry."

::= {  dot11staStatsEntry 8 }

dot11staRTSFailureCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall increment when a CTS is not received in

response to an RTS to the dot11staAddress in the current entry."

::= { dot11staStatsEntry 9 }

dot11staACKFailureCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall increment when an ACK is not received

when expected from the dot11staAddress in the current entry."

::= {  dot11staStatsEntry 10 }

dot11staReceivedFragmentCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall be incremented for each successfully

received MPDU of type Data or Management from the dot11staAddress

in the current entry."

::= { dot11staStatsEntry 11 }

dot11staMulticastReceivedFrameCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall increment when a MSDU is received

with the multicast bit set in the destination

MAC address from the dot11staAddress in the current entry."

::= { dot11staStatsEntry 12 }

dot11staTransmittedFrameCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall increment for each successfully transmitted MSDU

to the dot11staAddress in the current entry."

::= { dot11staStatsEntry 13 }

dot11staWEPUndecryptableCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall increment when a frame is received from the

dot11staAddress in the current entry

with the WEP subfield of the Frame Control field set to one and the

WEPOn value for the key mapped to the TA's MAC address

indicates that the frame should not have been encrypted or

that frame is discarded due to the receiving STA not

implementing the privacy option."

::= { dot11staStatsEntry 14 }

dot11staReceivedByteCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter indicates the total number of octets in MAC frame

bodies successfully received from the dot11staAddress in the current entry"

::= { dot11staStatsEntry 15 }

dot11staTransmittedByteCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter indicates the total number of octets in MAC frame

body successfully transmitted to the dot11staAddress in the current entry.

This excludes multicast and broadcast."

::= { dot11staStatsEntry 16 }

dot11staRSSI  OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This contains the RSSI value of the most recent fragment from

the dot11staAddress in the current entry."

-- >> J Kim: The exact definition in absolute scale for RSSI is currently pending in TGk

::= { dot11staStatsEntry 17 }

dot11staRxSignalQuality OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This contains the Signal Quality value of the most recent fragment from

the dot11staAddress in the current entry."

-- >> J Kim: The exact definition in absolute scale for Signal Quality is currently pending in TGk

::= { dot11staStatsEntry 18 }

dot11staType OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This contains the type of the dot11staAddress. The values are

0 : unknown

1 : AP

2 : STA, not AP

3 : ####

....more types

"

-- >> J Kim: The exact definition of types and additional types currently pending in TGk

::= { dot11staStatsEntry 19 }

dot11staState OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The state of the dot11staAddress relative to this STA and this ifIndex as

specified in 802.11 standard. If this STA is not an AP, the state should be

set to null. If this STA is in IBSS, ????????"

::= { dot11staStatsEntry 20 }

dot11staAID OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"If this STA is an AP on this ifIndex, this attribute contains the AID of

the dot11staAddress."

::= { dot11staStatsEntry 21 }

dot11staPHYType OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the phy type of the dot11staAddress in the current

entry. The definitions are

0: unknown

1: FH

2: DSSS US

3: DSSS Japan, etc. etc..

4: OFDM US, etc., etc....

"

-- >> J Kim: The exact definition of types and additional types currently pending in TGk or

-- >> 		do they already exist somewhere or a different variable similar to this?

::= { dot11staStatsEntry 22 }

dot11staCurrentChannel OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the channel number of the dot11staAddress in the

current entry. If in the same BSS as in this STA, the Current channel is the

same as this STA. If not in the same BSS for DSSS PHY, since DSSS packets are

decodable sometimes even not on the exact same channel, the attribute shall be

determined only when packets with channel number explicitly included

is decoded from the dot11staAddress, beacon, probe, probe response, association, etc.

OFDM does not have this issue. Thus, this can be determined during scans or other

activities away from the current channel."

::= { dot11staStatsEntry 23 }

dot11staSecurityStatus OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates whether unicast packets from the dot11staAddress in

the current entry has WEP bit set or other combinations of WEP usage monitored

by this STA

0: unknown

1: no WEP

2: WEP unicast, no WEP broadcast

3: full WEP no RSN

4: RSN, etc, etc.

"

-- >> J Kim: The exact definition of types and additional types currently pending in TGk or TGi?

-- >> J Kim: 1x MIB may cover similar information

-- >> J Kim: 11i MIB may cover similar information

::= { dot11staStatsEntry 24 }

dot11staRSSIhistogram OBJECT-TYPE

SYNTAX OCTET STRING

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Direct mapping of the TGh RSSI histogram report format. Each octet

corresponds to the RSSI bin values."

-- >> J Kim: TGk intends to create new defintion for RSSI and as a result there may be changes to this mapping.

-- 	     Also, for human readability, each bin may need to be assigned to a separate attribute.

::= { dot11staStatsEntry 25 }

dot11staRSSIhistogramDuration OBJECT-TYPE

SYNTAX 	Unsigned32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The duration in TU as defined in 11h for the above RSSI histogram

from the received packets from the dot11staAddress."

::= { dot11staStatsEntry 26 }

dot11staRSSIhistogramTime OBJECT-TYPE

SYNTAX TimeTicks

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The time at which the latest RSSI histogram was generated for the

received packets from the dot11staAddress"

-- >> J Kim: Like many other places, the issue of how to express time instance information needs to be settled.

::= { dot11staStatsEntry 27 }

dot11staTxRate OBJECT-TYPE

-- >> J Kim: This is based on Mike Moreton's suggestion in 02-652r1-RRM

SYNTAX Unsigned32 (0..127)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Provides a weighted average of the rates at which

recent frames were sucessfully transmitted.  Every time

a frame is sent with an address1 field equal to the value of

dot11staAddress for this row, this value is

modified as follows.  If R is the variable used to store

this value, and T was the rate at which the acknowleged

frame was sent, then

R = R * 0.7 + T * 0.3

R should be initialised to the maximum supported rate at

which frames can be sent to the station in question, and

should be held in a format of sufficient accuracy to allow

convergence.  R should be rounded to the nearest integer

before being returned as the value of this object.

The units for T are the same as those for rates as

described in the 'Supported Rates' Information Element."

::= { dot11staStatsEntry 28 }

dot11staRxRate OBJECT-TYPE

-- >> J Kim: This is based on Mike Moreton's suggestion in 02-652r1-RRM

SYNTAX Unsigned32 (0..127)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Provides a weighted average of the rates at which

recent frames were sucessfully received.  Every time

a frame is received with an address2 field equal to the value

of dot11staAddress for this row, this value is

modified as follows.  If R is the variable used to store

this value, and T was the rate at which the frame was

received, then

R = R * 0.7 + T * 0.3

R should be initialised to the maximum supported rate at

which frames can be received from the station in question, and

should be held in a format of sufficient accuracy to allow

convergence.  R should be rounded to the nearest integer

before being returned as the value of this object.

The units for T are the same as those for rates as

described in the 'Supported Rates' Information Element.

"

::= { dot11staStatsEntry 29 }

dot11staRoamed OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates whether the dot11staAddress is roamed or not

0: the dot11staAddress is still part of the BSS of this AP

1: the dot11staAddress has roamed

2: N/A for cases like the dot11staAddress was not part of BSS to begin with, this STA is not AP, etc.

"

-- >> J Kim: The exact definition of types and additional types currently pending in TGk or

-- >> J Kim: The value of AID can also reflect this information so maybe the above attribute is not needed

::= { dot11staStatsEntry 30 }

dot11staLastSeen OBJECT-TYPE

SYNTAX TimeTicks

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The time of the last activity of the dot11staAddress. This value and dot11staStatsTableAgingTime

is used	to determine whether this table entry can be purged to save memory."

-- >> J Kim: Like many other places, the issue of how to express time instance information needs to be settled.

::= { dot11staStatsEntry 31 }

-- **********************************************************************

-- *    end of dot11staStatsTable

-- **********************************************************************

Parts related to the Motion 171ra1 ends here.

- For Reference, the entire 802.11 MIB with the changed noted in this document as well as other parts of the MIB being worked on by TGk, is included below.

Draft Modifications for TGk MIB additions are marked with

-- >> J Kim: TGk addition begins

<< TGk Additions >>

-- >> J Kim: TGk addition ends

-- **********************************************************************

-- * IEEE 802.11 Management Information Base

-- **********************************************************************

IEEE802dot11-MIB DEFINITIONS ::= BEGIN

IMPORTS

MODULE-IDENTITY, OBJECT-TYPE,

NOTIFICATION-TYPE,Integer32, Counter32,

Unsigned32 				FROM SNMPv2-SMI

DisplayString , MacAddress, RowStatus,

TruthValue                              FROM SNMPv2-TC

MODULE-COMPLIANCE, OBJECT-GROUP,

NOTIFICATION-GROUP                      FROM SNMPv2-CONF

ifIndex                                 FROM RFC1213-MIB;

-- **********************************************************************

-- *  Tree Definition

-- *  Added by Daniel R. Borges on 02.14.2002 (compilation error)

-- **********************************************************************

member-body     OBJECT IDENTIFIER ::= { iso 2 }

us              OBJECT IDENTIFIER ::= { member-body 840 }

-- **********************************************************************

-- *  MODULE IDENTITY

-- **********************************************************************

ieee802dot11 MODULE-IDENTITY

LAST-UPDATED "0202210000Z"

ORGANIZATION "IEEE 802.11"

CONTACT-INFO

"WG E-mail: stds-802-11@ieee.org

Chair: Stuart J. Kerry

Postal: Philips Semiconductors, Inc.

1109 McKay Drive

M/S 48 SJ

San Jose, CA 95130-1706  USA

Tel: +1 408 474 7356

Fax: +1 408 474 7247

E-mail: stuart.kerry@philips.com

Editor: Bob O'Hara

Postal: Informed Technology, Inc.

1750 Nantucket Circle, Suite 138

Santa Clara, CA 95054 USA

Tel: +1 408 986 9596

Fax: +1 408 727 2654

E-mail: bob@informed-technology.com"

DESCRIPTION

"The MIB module for IEEE 802.11 entities.

iso(1).member-body(2).us(840).ieee802dot11(10036)"

::= { us 10036 }

--  Modified by Daniel R. Borges on 02.14.2002 (compilation error)

-- **********************************************************************

-- *  Major sections

-- **********************************************************************

--  Station ManagemenT (SMT) Attributes

--  DEFINED AS "The SMT object class provides the necessary support

--  at the station to manage the processes in the station such that

--  the station may work cooperatively as a part of an IEEE 802.11

--  network."

dot11smt OBJECT IDENTIFIER ::= { ieee802dot11 1 }

--  dot11smt GROUPS

--  dot11StationConfigTable            ::= { dot11smt 1 }

--  dot11AuthenticationAlgorithmsTable ::= { dot11smt 2 }

--  dot11WEPDefaultKeysTable           ::= { dot11smt 3 }

--  dot11WEPKeyMappingsTable           ::= { dot11smt 4 }

--  dot11PrivacyTable                  ::= { dot11smt 5 }

--  dot11SMTnotification               ::= { dot11smt 6 }

--  dot11MultiDomainCapabilityTable    ::= { dot11smt 7 }

-- >> J Kim: TGk addition begins

--  dot11ReportsTable		       ::= { dot11smt 8 }

--  dot11Actions		       ::= { dot11smt 9 }

-- >> J Kim: TGk addition ends

--  MAC Attributes

--  DEFINED AS "The MAC object class provides the necessary support

--  for the access control, generation, and verification of frame

--  check sequences, and proper delivery of valid data to upper

--  layers."

dot11mac OBJECT IDENTIFIER ::= { ieee802dot11 2 }

--  MAC GROUPS

--  reference IEEE Std 802.1f-1993

--  dot11OperationTable   	    ::= { dot11mac 1 }

--  dot11CountersTable    	    ::= { dot11mac 2 }

--  dot11GroupAddressesTable 	::= { dot11mac 3 }

-- >> J Kim: TGk addition begins

--  dot11staStatsTable	   ::=    {dot11mac4}

-- >> J Kim: TGk addition ends

--  Resource Type ID

dot11res 			OBJECT IDENTIFIER 	::= { ieee802dot11 3 }

dot11resAttribute 	OBJECT IDENTIFIER 	::= { dot11res 1 }

--  PHY Attributes

--  DEFINED AS "The PHY object class provides the necessary support

--  for required PHY operational information that may vary from PHY

--  to PHY and from STA to STA to be communicated to upper layers."

dot11phy OBJECT IDENTIFIER ::= { ieee802dot11 4 }

--  PHY GROUPS

--  dot11PhyOperationTable    	    ::= { dot11phy 1 }

--  dot11PhyAntennaTable   		    ::= { dot11phy 2 }

--  dot11PhyTxPowerTable      	    ::= { dot11phy 3 }

--  dot11PhyFHSSTable         	    ::= { dot11phy 4 }

--  dot11PhyDSSSTable         	    ::= { dot11phy 5 }

--  dot11PhyIRTable           	    ::= { dot11phy 6 }

--  dot11RegDomainsSupportedTable   ::= { dot11phy 7 }

--  dot11AntennasListTable          ::= { dot11phy 8 }

--  dot11SupportedDataRatesTxTable  ::= { dot11phy 9 }

--  dot11SupportedDataRatesRxTable  ::= { dot11phy 10 }

--  dot11PhyOFDMTable               ::= { dot11phy 11 }

-- Added by Daniel R. Borges on 02.14.2002 (802.11a)

--  dot11PhyHRDSSSTable 	    	::= { dot11phy 12 }

--  dot11EHCCHoppingPatternTable	::= { dot11phy 13 }

-- **********************************************************************

-- *  Textual conventions from 802 definitions

-- **********************************************************************

WEPKeytype ::= OCTET STRING (SIZE (5))

-- **********************************************************************

-- *  MIB attribute OBJECT-TYPE definitions follow

-- **********************************************************************

-- **********************************************************************

-- *  SMT Station Config  Table

-- **********************************************************************

dot11StationConfigTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11StationConfigEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Station Configuration attributes.  In tablular form to

allow for multiple instances on an agent."

::= { dot11smt 1 }

dot11StationConfigEntry OBJECT-TYPE

SYNTAX Dot11StationConfigEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11StationConfigTable.  It is

possible for there to be multiple IEEE 802.11 interfaces

on one agent, each with its unique MAC address. The

relationship between an IEEE 802.11 interface and an

interface in the context of the Internet-standard MIB is

one-to-one.  As such, the value of an ifIndex object

instance can be directly used to identify corresponding

instances of the objects defined herein.

ifIndex - Each 802.11 interface is represented by an

ifEntry.  Interface tables in this MIB module are indexed

by ifIndex."

INDEX { ifIndex }

::= { dot11StationConfigTable 1 }

Dot11StationConfigEntry ::=

SEQUENCE {	dot11StationID                         	MacAddress,

dot11MediumOccupancyLimit          		INTEGER,

dot11CFPollable                    		TruthValue,

dot11CFPPeriod                     		INTEGER,

dot11CFPMaxDuration                		INTEGER,

dot11AuthenticationResponseTimeOut 		Unsigned32,

dot11PrivacyOptionImplemented      		TruthValue,

dot11PowerManagementMode	   			INTEGER,

dot11DesiredSSID		   				OCTET STRING,

dot11DesiredBSSType		   				INTEGER,

dot11OperationalRateSet				    OCTET STRING,

dot11BeaconPeriod		   				INTEGER,

dot11DTIMPeriod			   				INTEGER,

dot11AssociationResponseTimeOut	        Unsigned32,

dot11DisassociateReason                 INTEGER,

dot11DisassociateStation                MacAddress,

dot11DeauthenticateReason               INTEGER,

dot11DeauthenticateStation              MacAddress,

dot11AuthenticateFailStatus             INTEGER,

dot11AuthenticateFailStation            MacAddress,

dot11MultiDomainCapabilityImplemented	TruthValue,

dot11MultiDomainCapabilityEnabled		TruthValue,

dot11CountryString		            	OCTET STRING,

-- >> J Kim: TGk addition begins

dot11AssociateStation                MacAddress,

dot11LowSignalQualityThreshold	     Integer32,

dot11ReportsTableAgingTime	     Unsigned32,

dot11staStatsTableAgingTime	     Unsigned32}

-- >> J Kim: TGk addition ends

dot11StationID OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-write

STATUS deprecated

DESCRIPTION

"The purpose of dot11StationID is to allow a manager to identify

a station for its own purposes.  This attribute provides

for that eventuality while keeping the true MAC address

independent.  Its syntax is MAC address and default value

is the station's assigned, unique MAC address."

::= { dot11StationConfigEntry 1 }

dot11MediumOccupancyLimit OBJECT-TYPE

SYNTAX INTEGER (0..1000)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall indicate the maximum amount of time,

in TU, that a point coordinator may control the usage of

the wireless medium without relinquishing control for long

enough to allow at least one instance of DCF access to the

medium.  The default value of this attribute shall be 100,

and the maximum value shall be 1000."

::= { dot11StationConfigEntry 2 }

dot11CFPollable OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"When this attribute is true, it shall indicate that the STA

is able to respond to a CF-Poll with a data frame within a

SIFS time. This attribute shall be false if the STA is not

able to respond to a CF-Poll with a data frame within a SIFS

time."

::= { dot11StationConfigEntry 3 }

dot11CFPPeriod OBJECT-TYPE

SYNTAX INTEGER (0..255)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The attribute shall describe the number of DTIM intervals

between the start of CFPs.  It is modified by

MLME-START.request primitive."

::= { dot11StationConfigEntry 4 }

dot11CFPMaxDuration OBJECT-TYPE

SYNTAX INTEGER (0..65535)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The attribute shall describe the maximum duration of the CFP

in TU that may be generated by the PCF. It is modified by

MLME-START.request primitive."

::= { dot11StationConfigEntry 5 }

dot11AuthenticationResponseTimeOut OBJECT-TYPE

SYNTAX Unsigned32 (1..4294967295)

-- Modified by Daniel R. Borges on 02.19.2002 (compilation errors)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall specify the number of TU that a

responding STA should wait for the next frame in the

authentication sequence."

::= { dot11StationConfigEntry 6 }

dot11PrivacyOptionImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute, when true, shall indicate that the IEEE

802.11 WEP option is implemented.  The default value of

this attribute shall be false."

::= { dot11StationConfigEntry 7 }

dot11PowerManagementMode OBJECT-TYPE

SYNTAX INTEGER { active(1), powersave(2) }

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall specify the power management

mode of the STA. When set to active, it shall indicate

that the station is not in power-save mode. When set

to powersave, it shall indicate that the station is

in power-save mode. The power management mode is

transmitted in all frames according to the rules

in  7.1.3.1.7."

::= { dot11StationConfigEntry 8 }

dot11DesiredSSID OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(0..32))

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute reflects the Service Set ID used

in the DesiredSSID parameter of the most recent

MLME_Scan.request.  This value may be modified

by an external management entity and used by the

local SME to make decisions about the Scanning process."

::= { dot11StationConfigEntry 9 }

dot11DesiredBSSType OBJECT-TYPE

SYNTAX INTEGER { infrastructure(1), independent(2), any(3) }

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall specify the type of BSS the

station shall use when scanning for a BSS with which

to synchronize. This value is used to filter Probe

Response frames and Beacons. When set to infrastructure,

the station shall only synchronize with a BSS whose

Capability Information field has the ESS subfield set

to 1. When set to independent, the station shall only

synchronize with a BSS whose Capability Information

field has the IBSS subfield set to 1. When set to

any, the station may synchronize to either type of

BSS."

::= { dot11StationConfigEntry 10 }

dot11OperationalRateSet OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(1..126))

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall specify the set of data rates

at which the station may transmit data.  Each octet

contains a value representing a rate.  Each rate

shall be within the range from 2 to 127,

corresponding to data rates in increments of

500 kb/s from 1 Mb/s to 63.5 Mb/s, and shall be

supported (as indicated in the supported rates

table) for receiving data. This value is reported in

transmitted Beacon, Probe Request, Probe Response,

Association Request, Association Response,

Reassociation Request, and Reassociation Response

frames, and is used to determine whether a BSS

with which the station desires to synchronize is

suitable. It is also used when starting a BSS,

as specified in  10.3."

::= { dot11StationConfigEntry 11 }

dot11BeaconPeriod OBJECT-TYPE

SYNTAX INTEGER (1..65535)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall specify the number of TU that a

station shall use for scheduling Beacon transmissions.

This value is transmitted in Beacon and Probe Response

frames."

::= { dot11StationConfigEntry 12 }

dot11DTIMPeriod OBJECT-TYPE

SYNTAX INTEGER(1..255)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall specify the number of beacon

intervals that shall elapse between transmission of

Beacons frames containing a TIM element whose DTIM

Count field is 0. This value is transmitted in

the DTIM Period field of Beacon frames."

::= { dot11StationConfigEntry 13 }

dot11AssociationResponseTimeOut OBJECT-TYPE

SYNTAX Unsigned32 (1..4294967295)

-- Modified by Daniel R. Borges on 02.19.2002 (compilation errors)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall specify the number of TU that a

requesting STA should wait for a response to a

transmitted association-request MMPDU."

::= { dot11StationConfigEntry 14 }

dot11DisassociateReason OBJECT-TYPE

SYNTAX INTEGER(0..65535)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute holds the most recently

transmitted Reason Code in a Disassociation

frame.  If no Disassociation frame has been

transmitted, the value of this attribute shall

be 0."

REFERENCE "IEEE Std 802.11-1997, 7.3.1.7"

::= { dot11StationConfigEntry 15 }

dot11DisassociateStation OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute holds the MAC address from the

Address 1 field of the most recently transmitted

Disassociation frame.  If no Disassociation

frame has been transmitted, the value of this

attribute shall be 0."

::= { dot11StationConfigEntry 16 }

dot11DeauthenticateReason OBJECT-TYPE

SYNTAX INTEGER(0..65535)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute holds the most recently

transmitted Reason Code in a Deauthentication

frame.  If no Deauthentication frame has been

transmitted, the value of this attribute shall

be 0."

REFERENCE "IEEE Std 802.11-1997, 7.3.1.7"

::= { dot11StationConfigEntry 17 }

dot11DeauthenticateStation OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute holds the MAC address from the

Address 1 field of the most recently transmitted

Deauthentication frame.  If no Deauthentication

frame has been transmitted, the value of this

attribute shall be 0."

::= { dot11StationConfigEntry 18 }

dot11AuthenticateFailStatus OBJECT-TYPE

SYNTAX INTEGER(0..65535)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute holds the most recently

transmitted Status Code in a failed

Authentication frame.  If no failed

Authentication frame has been transmitted, the

value of this attribute shall be 0."

REFERENCE "IEEE Std 802.11-1997, 7.3.1.9"

::= { dot11StationConfigEntry 19 }

dot11AuthenticateFailStation OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute holds the MAC address from the

Address 1 field of the most recently transmitted

failed Authentication frame.  If no failed

Authentication frame has been transmitted, the

value of this attribute shall be 0."

::= { dot11StationConfigEntry 20 }

dot11MultiDomainCapabilityImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when TRUE, indicates that the

station implementation is capable of supporting

multiple regulatory domains.  The capability is

disabled, otherwise.  The default value of this

attribute is FALSE."

::= { dot11StationConfigEntry 21 }

dot11MultiDomainCapabilityEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when TRUE, indicates that the

capability of the station to operate in multiple

regulatory domains is enabled.  The capability is

disabled, otherwise.  The default value of this

attribute is FALSE."

::= { dot11StationConfigEntry 22 }

dot11CountryString OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(3))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute identifies the country in which the

station is operating. The first two octets of this

string is the two character country code as described

in document ISO/IEC 3166-1.  The third octet shall

be one of the following:

1. an ASCII space character, if the regulations under

which the station is operating encompass all

environments in the country,

2. an ASCII 'O' character, if the regulations under

which the station is operating are for an Outdoor

environment only, or

3. an ASCII 'I' character, if the regulations under

which the station is operating are for an Indoor

environment only."

::= { dot11StationConfigEntry 23 }

-- >> J Kim: TGk addition begins

dot11AssociateStation OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute holds the MAC address from the

Address 1 field of the most recently transmitted

Association response frame.  If no Aassociation

response frame has been transmitted, the value of this

attribute shall be 0."

::= { dot11StationConfigEntry 24 }

dot11LowSignalQualityThreshold OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute holds the threshold value to trigger

dot11LowSignalQuality Notification."

::= { dot11StationConfigEntry 25 }

dot11ReportsTableAgingTime OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute holds the minimum time in <<unit here>>

to maintain entries in the dot11ReportsTable."

::= { dot11StationConfigEntry 26 }

dot11staStatsTableAgingTime OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute holds the minimum time in <<unit here>>

to maintain entries in the dot11staStatsTable."

::= { dot11StationConfigEntry 27 }

-- >> J Kim: TGk addition ends

-- **********************************************************************

-- *    End of dot11StationConfig  TABLE

-- **********************************************************************

-- **********************************************************************

-- *    AuthenticationAlgorithms  TABLE

-- **********************************************************************

dot11AuthenticationAlgorithmsTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11AuthenticationAlgorithmsEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"This (conceptual) table of attributes shall be a set of

all the authentication algorithms supported by the

stations.  The following are the default values and the

associated algorithm:

Value = 1: Open System

Value = 2: Shared Key"

REFERENCE "IEEE Std 802.11-1997, 7.3.1.1"

::= { dot11smt 2 }

dot11AuthenticationAlgorithmsEntry OBJECT-TYPE

SYNTAX Dot11AuthenticationAlgorithmsEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An Entry (conceptual row) in the Authentication

Algorithms Table.

ifIndex - Each 802.11 interface is represented by an

ifEntry.  Interface tables in this MIB module are indexed

by ifIndex."

INDEX { ifIndex,

dot11AuthenticationAlgorithmsIndex }

::= { dot11AuthenticationAlgorithmsTable  1 }

Dot11AuthenticationAlgorithmsEntry ::=

SEQUENCE {	dot11AuthenticationAlgorithmsIndex	Integer32,

dot11AuthenticationAlgorithm      	INTEGER,

dot11AuthenticationAlgorithmsEnable	TruthValue }

dot11AuthenticationAlgorithmsIndex OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"The auxiliary variable used to identify instances

of the columnar objects in the Authentication Algorithms Table."

::= { dot11AuthenticationAlgorithmsEntry 1 }

dot11AuthenticationAlgorithm OBJECT-TYPE

SYNTAX INTEGER { openSystem(1), sharedKey(2) }

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute shall be a set of all the authentication

algorithms supported by the STAs. The following are the

default values and the associated algorithm.

Value = 1: Open System

Value = 2: Shared Key"

::= { dot11AuthenticationAlgorithmsEntry 2 }

dot11AuthenticationAlgorithmsEnable  OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when true at a station, shall enable the acceptance

of the authentication algorithm described in the corresponding table

entry in authentication frames received by the station that have odd

authentication sequence numbers.  The default value of this attribute

shall be 1 for the Open System table entry and 2 for all other table

entries."

::= { dot11AuthenticationAlgorithmsEntry 3 }

-- **********************************************************************

-- *    End of AuthenticationAlgorithms  TABLE

-- **********************************************************************

-- **********************************************************************

-- *    WEPDefaultKeys  TABLE

-- **********************************************************************

dot11WEPDefaultKeysTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11WEPDefaultKeysEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Conceptual table for WEP default keys.  This table shall

contain the four WEP default secret key values

corresponding to the four possible KeyID values.  The WEP

default secret keys are logically WRITE-ONLY.  Attempts to

read the entries in this table shall return unsuccessful

status and values of null or zero.  The default value of

each WEP default key shall be null."

REFERENCE "IEEE Std 802.11-1997, 8.3.2"

::= { dot11smt 3 }

dot11WEPDefaultKeysEntry OBJECT-TYPE

SYNTAX Dot11WEPDefaultKeysEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An Entry (conceptual row) in the WEP Default Keys Table.

ifIndex - Each 802.11 interface is represented by an

ifEntry.  Interface tables in this MIB module are indexed

by ifIndex."

INDEX { ifIndex,

dot11WEPDefaultKeyIndex}

::= { dot11WEPDefaultKeysTable  1 }

Dot11WEPDefaultKeysEntry ::=

SEQUENCE {	dot11WEPDefaultKeyIndex     INTEGER,

dot11WEPDefaultKeyValue     WEPKeytype }

dot11WEPDefaultKeyIndex OBJECT-TYPE

SYNTAX INTEGER (1..4)

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"The auxiliary variable used to identify instances

of the columnar objects in the WEP Default Keys Table.

The value of this variable is equal to the WEPDefaultKeyID + 1"

::= { dot11WEPDefaultKeysEntry 1 }

dot11WEPDefaultKeyValue OBJECT-TYPE

SYNTAX WEPKeytype

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"A WEP default secret key value."

::= { dot11WEPDefaultKeysEntry 2 }

-- **********************************************************************

-- *    End of WEPDefaultKeys  TABLE

-- **********************************************************************

-- **********************************************************************

-- *    WEPKeyMappings  TABLE

-- **********************************************************************

dot11WEPKeyMappingsTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11WEPKeyMappingsEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Conceptual table for WEP Key Mappings.  The MIB supports

the ability to share a separate WEP key for each RA/TA

pair.  The Key Mappings Table contains zero or one entry

for each MAC address and contains two fields for each

entry: WEPOn and the corresponding WEP key.  The WEP key

mappings are logically WRITE-ONLY.  Attempts to read the

entries in this table shall return unsuccessful status and

values of null or zero.  The default value for all WEPOn

fields is false."

REFERENCE "IEEE Std 802.11-1997, 8.3.2"

::= { dot11smt 4 }

dot11WEPKeyMappingsEntry OBJECT-TYPE

SYNTAX Dot11WEPKeyMappingsEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An Entry (conceptual row) in the WEP Key Mappings Table.

ifIndex - Each 802.11 interface is represented by an

ifEntry.  Interface tables in this MIB module are indexed

by ifIndex."

INDEX { ifIndex,

dot11WEPKeyMappingIndex }

::= { dot11WEPKeyMappingsTable  1 }

Dot11WEPKeyMappingsEntry ::=

SEQUENCE {	dot11WEPKeyMappingIndex	    Integer32,

dot11WEPKeyMappingAddress   MacAddress,

dot11WEPKeyMappingWEPOn 	TruthValue,

dot11WEPKeyMappingValue     WEPKeytype,

dot11WEPKeyMappingStatus	RowStatus }

dot11WEPKeyMappingIndex OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"The auxiliary variable used to identify instances

of the columnar objects in the WEP Key Mappings Table."

::= { dot11WEPKeyMappingsEntry 1 }

dot11WEPKeyMappingAddress OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The MAC address of the STA for which the values from this

key mapping entry are to be used."

::= { dot11WEPKeyMappingsEntry 2 }

dot11WEPKeyMappingWEPOn OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"Boolean as to whether WEP is to be used when communicating

with the dot11WEPKeyMappingAddress STA."

::= { dot11WEPKeyMappingsEntry 3 }

dot11WEPKeyMappingValue OBJECT-TYPE

SYNTAX WEPKeytype

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"A WEP secret key value."

::= { dot11WEPKeyMappingsEntry 4 }

dot11WEPKeyMappingStatus OBJECT-TYPE

SYNTAX RowStatus

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The status column used for creating, modifying, and

deleting instances of the columnar objects in the WEP key

mapping Table."

DEFVAL { active }

::= { dot11WEPKeyMappingsEntry 5 }

-- **********************************************************************

-- *    End of WEPKeyMappings  TABLE

-- **********************************************************************

-- **********************************************************************

-- *    dot11PrivacyTable  TABLE

-- **********************************************************************

dot11PrivacyTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11PrivacyEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Group containing attributes concerned with IEEE 802.11

Privacy.  Created as a table to allow multiple

instantiations on an agent."

::= { dot11smt 5 }

dot11PrivacyEntry OBJECT-TYPE

SYNTAX Dot11PrivacyEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11PrivacyTable Table.

ifIndex - Each 802.11 interface is represented by an

ifEntry.  Interface tables in this MIB module are indexed

by ifIndex."

INDEX { ifIndex }

::= { dot11PrivacyTable 1 }

Dot11PrivacyEntry ::=

SEQUENCE {	dot11PrivacyInvoked        	TruthValue,

dot11WEPDefaultKeyID       	INTEGER,

dot11WEPKeyMappingLength	Unsigned32,

dot11ExcludeUnencrypted 	TruthValue,

dot11WEPICVErrorCount      	Counter32,

dot11WEPExcludedCount      	Counter32 }

dot11PrivacyInvoked OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"When this attribute is true, it shall indicate that the IEEE

802.11 WEP mechanism is used for transmitting frames of type

Data. The default value of this attribute shall be false."

::= { dot11PrivacyEntry 1 }

dot11WEPDefaultKeyID  OBJECT-TYPE

SYNTAX INTEGER (0..3)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall indicate the use of the first,

second, third, or fourth element of the WEPDefaultKeys

array when set to values of zero, one, two, or three.  The

default value of this attribute shall be 0."

REFERENCE "IEEE Std 802.11-1997, 8.3.2"

::= { dot11PrivacyEntry 2 }

dot11WEPKeyMappingLength  OBJECT-TYPE

SYNTAX Unsigned32 (10..4294967295)

-- Modified by Daniel R. Borges on 02.19.2002 (compilation errors)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The maximum number of tuples that dot11WEPKeyMappings can hold."

REFERENCE "IEEE Std 802.11-1997, 8.3.2"

::= { dot11PrivacyEntry 3 }

dot11ExcludeUnencrypted  OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"When this attribute is true, the STA shall not indicate at

the MAC service interface received MSDUs that have the WEP

subfield of the Frame Control field equal to zero. When this

attribute is false, the STA may accept MSDUs that have the WEP

subfield of the Frame Control field equal to zero. The default

value of this attribute shall be false."

::= { dot11PrivacyEntry 4 }

dot11WEPICVErrorCount  OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall increment when a frame is received with the

WEP subfield of the Frame Control field set to one and the value

of the ICV as received in the frame does not match the ICV value

that is calculated for the contents of the received frame."

::= { dot11PrivacyEntry 5 }

dot11WEPExcludedCount  OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall increment when a frame is received with the

WEP subfield of the Frame Control field set to zero and the value

of dot11ExcludeUnencrypted causes that frame to be discarded."

::= { dot11PrivacyEntry 6 }

-- **********************************************************************

-- *    End of dot11Privacy  TABLE

-- **********************************************************************

-- **********************************************************************

-- *    SMT notification Objects

-- **********************************************************************

dot11SMTnotification OBJECT IDENTIFIER ::= { dot11smt 6 }

dot11Disassociate NOTIFICATION-TYPE

OBJECTS { ifIndex, dot11DisassociateReason, dot11DisassociateStation }

STATUS current

DESCRIPTION

"The disassociate notification shall be sent when the STA

sends a Disassociation frame. The value of the notification

shall include the MAC address of the MAC to which the Disassociation

frame was sent and the reason for the disassociation.

ifIndex - Each 802.11 interface is represented by an

ifEntry.  Interface tables in this MIB module are indexed

by ifIndex."

::= { dot11SMTnotification 0 1 }

dot11Deauthenticate NOTIFICATION-TYPE

OBJECTS { ifIndex, dot11DeauthenticateReason, dot11DeauthenticateStation }

STATUS current

DESCRIPTION

"The deauthenticate notification shall be sent when the STA

sends a Deauthentication frame. The value of the notification

shall include the MAC address of the MAC to which the Deauthentication

frame was sent and the reason for the deauthentication.

ifIndex - Each 802.11 interface is represented by an

ifEntry.  Interface tables in this MIB module are indexed

by ifIndex."

::= { dot11SMTnotification 0 2 }

dot11AuthenticateFail NOTIFICATION-TYPE

OBJECTS { ifIndex, dot11AuthenticateFailStatus, dot11AuthenticateFailStation }

STATUS current

DESCRIPTION

"The authenticate failure notification shall be sent when the STA

sends an Authentication frame with a status code other than

'successful'. The value of the notification

shall include the MAC address of the MAC to which the Authentication

frame was sent and the reason for the authentication failure.

ifIndex - Each 802.11 interface is represented by an

ifEntry.  Interface tables in this MIB module are indexed

by ifIndex."

::= { dot11SMTnotification 0 3 }

-- >> J Kim: TGk addition begins

dot11Associate NOTIFICATION-TYPE

OBJECTS { ifIndex, dot11AssociateStation }

STATUS current

DESCRIPTION

"The associate notification shall be sent when the STA

receives an assiciation frame and is accepted. The value of the notification

shall include the MAC address of the MAC to which the Association

response frame was sent."

::= { dot11SMTnotification 0 4 }

dot11LowSignalQuality NOTIFICATION-TYPE

OBJECTS { ifIndex, dot11staAddress}

STATUS current

DESCRIPTION

"The low signal quality notification shall be sent when the STA

measures the signal quality for associated STAs fall below

dot11LowSignalQualityThreshold.

The value of the notification shall include the MAC address of

the STA having low quality."

::= { dot11SMTnotification 0 5 }

-- >> J Kim: TGk addition ends

-- **********************************************************************

-- *    End of SMT notification Objects

-- **********************************************************************

-- ********************************************************************

-- * dot11MultiDomainCapability TABLE

-- ********************************************************************

dot11MultiDomainCapabilityTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11MultiDomainCapabilityEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"This (conceptual) table of attributes for cross-domain mobility."

::= { dot11smt 7 }

dot11MultiDomainCapabilityEntry OBJECT-TYPE

SYNTAX Dot11MultiDomainCapabilityEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry (conceptual row) in the Multiple Domain Capability Table.

IfIndex - Each 802.11 interface is represented by an ifEntry.

Interface tables in this MIB are indexed by ifIndex."

INDEX { ifIndex,

dot11MultiDomainCapabilityIndex }

::= { dot11MultiDomainCapabilityTable 1 }

Dot11MultiDomainCapabilityEntry ::=

SEQUENCE {	dot11MultiDomainCapabilityIndex		Integer32,

dot11FirstChannelNumber				Integer32,

dot11NumberofChannels				Integer32,

dot11MaximumTransmitPowerLevel		Integer32 }

dot11MultiDomainCapabilityIndex OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"The auxiliary variable used to identify instances of the columnar

objects in the Multi Domain Capability Table."

::= { dot11MultiDomainCapabilityEntry 1 }

dot11FirstChannelNumber OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall indicate the value of the lowest channel number

in the subband for the associated domain country string. The default

value of this attribute shall be zero."

::= { dot11MultiDomainCapabilityEntry 2 }

dot11NumberofChannels OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall indicate the value of the total number of channels

allowed in the subband for the associated domain country string. The

default value of this attribute shall be zero."

::= { dot11MultiDomainCapabilityEntry 3 }

dot11MaximumTransmitPowerLevel OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall indicate the maximum transmit power, in dBm,

allowed in the subband for the associated domain country string.

The default value of this attribute shall be zero."

::= { dot11MultiDomainCapabilityEntry 4 }

-- ********************************************************************

-- * End of dot11MultiDomainCapability TABLE

-- ********************************************************************

-- >> J Kim: TGk addition begins

-- **********************************************************************

-- *    dot11ReportsTable TABLE

-- **********************************************************************

dot11ReportsTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11ReportsEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Group containing attributes that are Mac/PHY Statistics per STA

reported by other STAs (not locally present at any interface)

via report frames.

Implemented as a table to allow for multiple

instantiations."

::= {  dot11smt 8 }

dot11ReportsEntry OBJECT-TYPE

SYNTAX Dot11ReportsEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11ReportsTable.

Indexed by dot11ReportsIndex"

INDEX {dot11ReportsIndex}

::= {  dot11ReportsTable 1 }

Dot11ReportsEntry ::= SEQUENCE {

dot11ReportsIndex			Unsigned32,

dot11ReporterAddress		MacAddress,

dot11TargetAddress			MacAddress,

dot11ActionIndex			Unsigned32,

dot11ActionToken			OCTET STRING,

dot11staBSSID			MacAddress,

dot11staSSID			OCTET STRING,

dot11staMulticastTxFrameCount 	Counter32,

dot11staTxFragmentCount       	Counter32,

dot11MulticastTransmittedFrameCount Counter32,

dot11FailedCount                    Counter32,

dot11RetryCount                     Counter32,

dot11MultipleRetryCount             Counter32,

dot11FrameDuplicateCount            Counter32,

dot11RTSSuccessCount                Counter32,

dot11RTSFailureCount                Counter32,

dot11ACKFailureCount                Counter32,

dot11ReceivedFragmentCount          Counter32,

dot11MulticastReceivedFrameCount    Counter32,

dot11FCSErrorCount                  Counter32,

dot11TransmittedFrameCount          Counter32,

dot11WEPUndecryptableCount          Counter32,

dot11WEPDecryptionFailureCount	Counter32,

dot11ReceivedByteCount              Counter32,

dot11TransmittedByteCount           Counter32,

dot11RSSI				Integer32,

dot11RxSignalQuality		Integer32,

dot11ReportedTxPower		INTEGER,

dot11ReportedLinkMargin		INTEGER,

dot11staType			INTEGER,

dot11staState			INTEGER,

dot11staAID				Unsigned32,

dot11BeaconCount			Counter32,

dot11currentChannel			INTEGER,

dot11staWEPEnabled			INTEGER,

dot11staTxRate			Unsigned32,

dot11staRxRate			Unsigned32,

dot11staRoamed			INTEGER,

dot11RSSIhistogram			OCTET STRING,

dot11RSSIhistogramDuration		Unsigned32,

dot11RSSIhistogramTime		TimeTicks,

dot11staLastSeen			TimeTicks}

-- variable definitions for added variables need to be put here

-- **********************************************************************

-- *    dot11ReportTable  TABLE ends

-- **********************************************************************

-- **********************************************************************

--  dot11Actions begins

-- **********************************************************************

--	This part of the MIB provides a table in which a manager can

--	create rows which specify an action (for example an 11h

--	measurement) to be carried out by the STA.  The result

--	of the action can be determined by re-reading the row at a

--	later point.

--	Management stations should poll a row no more frequently

--	than once per second, and rows may only be discarded at least

--	60 seconds after completion of the action.

dot11Actions	OBJECT IDENTIFIER ::= { dot11smt 9 }

dot11ActionsNextInxdex OBJECT-TYPE

SYNTAX		Unsigned32(0..65535)

MAX-ACCESS	read-only

STATUS		current

DESCRIPTION

"Identifies a hint for the next value of

dot11ActionsIndex to be used in a row creation attempt

for dot11ActionsTable.  If no new rows can be created

this object will have a value of 0."

::= { dot11Actions 1 }

dot11ActionsTable OBJECT-TYPE

SYNTAX		SEQUENCE OF Dot11ActionsEntry

MAX-ACCESS	not-accessible

STATUS		current

DESCRIPTION

"The table containing rows that describe an

action to be carried out, and the results of

that action."

::= { dot11Actions 2 }

dot11ActionsEntry OBJECT-TYPE

SYNTAX		Dot11ActionsEntry

MAX-ACCESS	not-accessible

STATUS		current

DESCRIPTION

"An entry in the dot11ActionsTable."

INDEX { dot11ActionIndex }

::= { dot11ActionsTable 1 }

Dot11ActionsEntry::=

SEQUENCE {

dot11ActionIndex			Unsigned32,

dot11ActionType				INTEGER,

dot11ActionAddress			MacAddress,

dot11ActionParam1			Integer32,

dot11ActionParam2			Integer32,

dot11ActionParam3			Integer32,

dot11ActionParam4			Integer32,

dot11ActionNotificationReq	TruthValue,

dot11ActionStatus			INTEGER,

dot11ActionRowStatus		RowStatus

}

dot11ActionIndex OBJECT-TYPE

SYNTAX Unsigned32 (0..4294967295)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The auxiliary variable used to identify instances

of the columnar objects in the Remote Stations

Action Table."

::= { dot11ActionEntry 1 }

dot11ActionType OBJECT-TYPE

SYNTAX		INTEGER { tpcMeasurement(1) }

MAX-ACCESS		read-create

STATUS		current

DESCRIPTION

"This object indicates the type of action to be carried out.

The value of this object also determines the

meaning of other objects in the row.

tpcMeasurement means carrying out the TPC measurement

procedure defined in 11.5.4"

::= { dot11ActionEntry 2 }

dot11ActionAddress OBJECT-TYPE

SYNTAX		MacAddress

MAX-ACCESS		read-create

STATUS		current

DESCRIPTION

"The MAC address of the STA the objects in this conceptual row

apply to.  For dot11AactionType == tcpMeasurement

indicates the remote station the TPC Request should be

sent to."

::= { dot11ActionEntry 3 }

dot11ActionParam1 OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The meaning of this object varies depending on the

value of dot11ActionType, and whether the object

is being written or read.

if dot11ActionType == tcpMeasurement

on Write:

not used

on Read:

local transmit power used to transmit TPC

Request with the units and tolerance of the

Transmit Power field defined in 7.3.2.12.

"

::= { dot11ActionEntry 4 }

dot11ActionParam2 OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The meaning of this object varies depending on the

value of dot11ActionType, and whether the object

is being written or read.

if dot11ActionType == tcpMeasurement

on Write:

not used

on Read:

The remote link margin field from the received

TPC report frame.

"

::= { dot11ActionEntry 5 }

dot11ActionParam3 OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The meaning of this object varies depending on the

value of dot11ActionType, and whether the object

is being written or read.

if dot11ActionType == tcpMeasurement

on Write:

not used

on Read:

The remote transmit power used field from the

received TPC report frame.

"

::= { dot11ActionEntry 6 }

dot11ActionParam4 OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The meaning of this object varies depending on the

value of dot11ActionType, and whether the object

is being written or read.

if dot11ActionType == tcpMeasurement

on Write:

not used

on Read:

The local link margin measured when receiving

the TPC Response with the units and tolerance

of the Link Margin field defined in 7.3.2.12.

"

::= { dot11ActionEntry 7 }

dot11ActionNotificationReq OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"If set to true when the row is created, a notification

will be sent on completion of the action."

::= { dot11ActionEntry 8 }

dot11ActionStatus OBJECT-TYPE

SYNTAX INTEGER { inProgress(1), failed(2), succeeded(3) }

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Contains the status of the requested action."

::= { dot11ActionEntry 9 }

dot11ActionRowStatus OBJECT-TYPE

SYNTAX RowStatus

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The status column used for creating, modifying, and

deleting instances of the columnar objects in the Remote

Stations Action Table.  When the row is placed in service

the action will be carried out."

::= { dot11ActionEntry 10 }

dot11ActionComplete NOTIFICATION-TYPE

OBJECTS	{

dot11ActionIndex,

dot11ActionType,

dot11ActionAddress,

dot11ActionParam1,

dot11ActionParam2,

dot11ActionParam3,

dot11ActionParam4,

dot11ActionStatus }

STATUS		current

DESCRIPTION

"Contains notification of the completion of an

action requested via the creation of a row in the

Action Table."

::= { dot11Actions 3 }

-- **********************************************************************

-- * End of dot11Actions

-- **********************************************************************

-- >> J Kim: TGk addition ends

-- **********************************************************************

-- *    MAC Attribute Templates

-- **********************************************************************

-- **********************************************************************

-- *    dot11OperationTable  TABLE

-- **********************************************************************

dot11OperationTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11OperationEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Group contains MAC attributes pertaining to the operation

of the MAC.  This has been implemented as a table in order

to allow for multiple instantiations on an agent."

::= { dot11mac 1 }

dot11OperationEntry OBJECT-TYPE

SYNTAX Dot11OperationEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11OperationEntry Table.

ifIndex - Each 802.11 interface is represented by an

ifEntry.  Interface tables in this MIB module are indexed

by ifIndex."

INDEX { ifIndex }

::= { dot11OperationTable 1 }

Dot11OperationEntry ::=

SEQUENCE {	dot11MACAddress                 MacAddress,

dot11RTSThreshold               INTEGER,

dot11ShortRetryLimit            INTEGER,

dot11LongRetryLimit             INTEGER,

dot11FragmentationThreshold     INTEGER,

dot11MaxTransmitMSDULifetime    Unsigned32,

dot11MaxReceiveLifetime         Unsigned32,

dot11ManufacturerID             DisplayString,

dot11ProductID                  DisplayString }

dot11MACAddress OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Unique MAC Address assigned to the STA."

::= { dot11OperationEntry 1 }

dot11RTSThreshold OBJECT-TYPE

SYNTAX INTEGER (0..2347)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall indicate the number of octets in an MPDU,

below which an RTS/CTS handshake shall not be performed. An

RTS/CTS handshake shall be performed at the beginning of any

frame exchange sequence where the MPDU is of type Data or

Management, the MPDU has an individual address in the Address1

field, and the length of the MPDU is greater than

this threshold. (For additional details, refer to Table 21 in

9.7.) Setting this attribute to be larger than the maximum

MSDU size shall have the effect of turning off the RTS/CTS

handshake for frames of Data or Management type transmitted by

this STA. Setting this attribute to zero shall have the effect

of turning on the RTS/CTS handshake for all frames of Data or

Management type transmitted by this STA. The default value of

this attribute shall be 2347."

::= { dot11OperationEntry 2 }

dot11ShortRetryLimit OBJECT-TYPE

SYNTAX INTEGER (1..255)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall indicate the maximum number of

transmission attempts of a frame, the length of which is less

than or equal to dot11RTSThreshold, that shall be made before a

failure condition is indicated. The default value of this

attribute shall be 7."

::= { dot11OperationEntry 3 }

dot11LongRetryLimit OBJECT-TYPE

SYNTAX INTEGER (1..255)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall indicate the maximum number of

transmission attempts of a frame, the length of which is

greater than dot11RTSThreshold, that shall be made before a

failure condition is indicated. The default value of this

attribute shall be 4."

::= { dot11OperationEntry 4 }

dot11FragmentationThreshold OBJECT-TYPE

SYNTAX INTEGER (256..2346)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall specify the current maximum size, in

octets, of the MPDU that may be delivered to the PHY. An MSDU

shall be broken into fragments if its size exceeds the value

of this attribute after adding MAC headers and trailers. An MSDU

or MMPDU shall be fragmented when the resulting frame has an

individual address in the Address1 field, and the length of the

frame is larger than this threshold. The default value for this

attribute shall be the lesser of 2346 or the aMPDUMaxLength of

the attached PHY and shall never exceed the lesser of 2346 or

the  aMPDUMaxLength of the attached PHY. The value of this

attribute shall never be less than 256. "

::= { dot11OperationEntry 5 }

dot11MaxTransmitMSDULifetime OBJECT-TYPE

SYNTAX Unsigned32 (1..4294967295)

-- Modified by Daniel R. Borges on 02.19.2002 (compilation errors)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The MaxTransmitMSDULifetime shall be the elapsed time in TU,

after the initial transmission of an MSDU, after which further

attempts to transmit the MSDU shall be terminated. The default

value of this attribute shall be 512."

::= { dot11OperationEntry 6 }

dot11MaxReceiveLifetime OBJECT-TYPE

SYNTAX Unsigned32 (1..4294967295)

-- Modified by Daniel R. Borges on 02.19.2002 (compilation errors)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The MaxReceiveLifetime shall be the elapsed time in TU,

after the initial reception of a fragmented MMPDU or MSDU,

after which further attempts to reassemble the MMPDU or

MSDU shall be terminated. The default value shall be

512."

::= { dot11OperationEntry 7 }

dot11ManufacturerID OBJECT-TYPE

SYNTAX DisplayString (SIZE(0..128))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The ManufacturerID shall include, at a minimum, the name

of the manufacturer.  It may include additional

information at the manufacturer's discretion.  The default

value of this attribute shall be null."

::= { dot11OperationEntry 8 }

dot11ProductID OBJECT-TYPE

SYNTAX DisplayString (SIZE(0..128))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The ProductID shall include, at a minimum, an identifier

that is unique to the manufacturer.  It may include

additional information at the manufacturer's discretion.

The default value of this attribute shall be null."

::= { dot11OperationEntry 9 }

-- **********************************************************************

-- *    End of dot11OperationEntry  TABLE

-- **********************************************************************

-- **********************************************************************

-- *    dot11CountersTable TABLE

-- **********************************************************************

dot11CountersTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11CountersEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Group containing attributes that are MAC counters per interface.

Implemented as a table to allow for multiple

instantiations on an agent."

::= { dot11mac 2 }

dot11CountersEntry OBJECT-TYPE

SYNTAX Dot11CountersEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11CountersEntry Table.

ifIndex - Each 802.11 interface is represented by an

ifEntry.  Interface tables in this MIB module are indexed

by ifIndex."

INDEX { ifIndex }

::= { dot11CountersTable 1 }

Dot11CountersEntry ::=

SEQUENCE {	dot11TransmittedFragmentCount       Counter32,

dot11MulticastTransmittedFrameCount Counter32,

dot11FailedCount                    Counter32,

dot11RetryCount                     Counter32,

dot11MultipleRetryCount             Counter32,

dot11FrameDuplicateCount            Counter32,

dot11RTSSuccessCount                Counter32,

dot11RTSFailureCount                Counter32,

dot11ACKFailureCount                Counter32,

dot11ReceivedFragmentCount          Counter32,

dot11MulticastReceivedFrameCount    Counter32,

dot11FCSErrorCount                  Counter32,

dot11TransmittedFrameCount          Counter32,

dot11WEPUndecryptableCount          Counter32,

-- >> J Kim: TGk addition starts

dot11ReceivedFrameCount              	Counter32,

dot11ReceivedBeaconCount	    	Counter32,

dot11TransmittedBeaconCount	    	Counter32,

dot11TransmittedManagementFrameCount	Counter32,

dot11ReceivedManagemetFrameCount	Counter32,

dot11AssociatedStaCount			Counter32,

dot11ObservedStaCount			Counter32,

dot11ObservedAPCount			Counter32}

-- >> J Kim: TGk addition ends

dot11TransmittedFragmentCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall be incremented for an acknowledged MPDU

with an individual address in the address 1 field or an MPDU

with a multicast address in the address 1 field of type Data

or Management."

::= { dot11CountersEntry 1 }

dot11MulticastTransmittedFrameCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall increment only when the multicast bit

is set in the destination MAC address of a successfully

transmitted MSDU.  When operating as a STA in an ESS, where

these frames are directed to the AP, this implies having

received an acknowledgment to all associated MPDUs."

::= { dot11CountersEntry 2 }

dot11FailedCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall increment when an MSDU is not transmitted

successfully due to the number of transmit attempts exceeding

either the  dot11ShortRetryLimit or dot11LongRetryLimit."

::= { dot11CountersEntry 3 }

dot11RetryCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall increment when an MSDU is successfully

transmitted after one or more retransmissions."

::= { dot11CountersEntry 4 }

dot11MultipleRetryCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall increment when an MSDU is successfully

transmitted after more than one retransmission."

::= { dot11CountersEntry 5 }

dot11FrameDuplicateCount  OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall increment when a frame is received

that the Sequence Control field indicates is a

duplicate."

::= { dot11CountersEntry 6 }

dot11RTSSuccessCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall increment when a CTS is received in

response to an RTS."

::= {  dot11CountersEntry 7 }

dot11RTSFailureCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall increment when a CTS is not received in

response to an RTS."

::= { dot11CountersEntry 8 }

dot11ACKFailureCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall increment when an ACK is not received

when expected."

::= {  dot11CountersEntry 9 }

dot11ReceivedFragmentCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall be incremented for each successfully

received MPDU of type Data or Management."

::= { dot11CountersEntry 10 }

dot11MulticastReceivedFrameCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall increment when a MSDU is received

with the multicast bit set in the destination

MAC address."

::= { dot11CountersEntry 11 }

dot11FCSErrorCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall increment when an FCS error is

detected in a received MPDU."

::= { dot11CountersEntry 12 }

dot11TransmittedFrameCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall increment for each successfully transmitted MSDU."

::= { dot11CountersEntry 13 }

dot11WEPUndecryptableCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall increment when a frame is received with

the WEP subfield of the Frame Control field set to one and the

WEPOn value for the key mapped to the TA's MAC address

indicates that the frame should not have been encrypted or

that frame is discarded due to the receiving STA not

implementing the privacy option."

::= { dot11CountersEntry 14 }

-- >> J Kim: TGk addition begins

-- variable definitions for added variables need to be put here

-- >> J Kim: TGk addition ends

-- **********************************************************************

-- *    End of dot11CountersEntry  TABLE

-- **********************************************************************

-- **********************************************************************

-- *    GroupAddresses  TABLE

-- **********************************************************************

dot11GroupAddressesTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11GroupAddressesEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"A conceptual table containing a set of MAC addresses

identifying the multicast addresses for which this STA

will receive frames.  The default value of this attribute

shall be null."

::= { dot11mac 3 }

dot11GroupAddressesEntry OBJECT-TYPE

SYNTAX Dot11GroupAddressesEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An Entry (conceptual row) in the Group Addresses Table.

ifIndex - Each 802.11 interface is represented by an

ifEntry.  Interface tables in this MIB module are indexed

by ifIndex."

INDEX { ifIndex,

dot11GroupAddressesIndex}

::= { dot11GroupAddressesTable  1 }

Dot11GroupAddressesEntry ::=

SEQUENCE {	dot11GroupAddressesIndex    Integer32,

dot11Address                MacAddress,

dot11GroupAddressesStatus   RowStatus }

dot11GroupAddressesIndex OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"The auxiliary variable used to identify instances

of the columnar objects in the Group Addresses Table."

::= { dot11GroupAddressesEntry 1 }

dot11Address OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"MAC address identifying a multicast addresses

from which this STA will receive frames."

::= { dot11GroupAddressesEntry 2 }

dot11GroupAddressesStatus OBJECT-TYPE

SYNTAX RowStatus

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The status column used for creating, modifying, and

deleting instances of the columnar objects in the Group

Addresses Table."

DEFVAL { active }

::= { dot11GroupAddressesEntry 3 }

-- **********************************************************************

-- *    End of GroupAddress  TABLE

-- **********************************************************************

-- >> J Kim: TGk addition begins

-- **********************************************************************

-- *    dot11staStatsTable TABLE

-- **********************************************************************

dot11staStatsTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11staStatsEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"The table containing statistics for each STA with

which this STA is directly communicating or not directly communicating

but observed any traffic originated from."

::= {  dot11mac 4 }

dot11staStatsEntry OBJECT-TYPE

SYNTAX Dot11staStatsEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11staStatsEntry Table.

Indexed by dot11staAddress"

INDEX {dot11staAddress}

::= {  dot11staStatsTable 1 }

Dot11staStatsEntry ::= SEQUENCE {

dot11staAddress			MacAddress,

ifIndex				Integer32,

dot11staBSSID			MacAddress,

dot11staSSID			OCTET STRING,

-- >> J Kim: The below is basically a copy of dot11CountersTable except

--	      dot11FCSErrorCounter, dot11MulticastTransmittedFrameCounter

dot11staTransmittedFragmentCount       		Counter32,

dot11staFailedCount                    Counter32,

dot11staRetryCount                     Counter32,

dot11staMultipleRetryCount             Counter32,

dot11staFrameDuplicateCount            Counter32,

dot11staRTSSuccessCount                Counter32,

dot11staRTSFailureCount                Counter32,

dot11staACKFailureCount                Counter32,

dot11staReceivedFragmentCount          Counter32,

dot11staMulticastReceivedFrameCount    Counter32,

dot11staTransmittedFrameCount          Counter32,

dot11staWEPUndecryptableCount          Counter32,

-- >> J Kim: Similarity to CountersTable ends here

dot11staReceivedByteCount              Counter32,

dot11staTransmittedByteCount           Counter32,

dot11staRSSI				Integer32,

dot11staRxSignalQuality		Integer32,

dot11staType			INTEGER,

dot11staState			INTEGER,

dot11staAID				Unsigned32,

dot11staPHYType			INTEGER,

dot11staCurrentChannel			INTEGER,

dot11staSecurityStatus			INTEGER,

-- >> J Kim: the RSSI histogram in this table is using the same format as that of 11h

-- 	      However, this is RSSI from dot11staAddress measured by this STA on this ifIndex,

--	      if measured during routine operation or requested by the dot11staAddress.

--	      RSSI histograms measured not by this STA and reported back to this STA from other STA

--	      is recorded in the dot11ReportsTable under SMT branch.

dot11staRSSIhistogram			OCTET STRING,

dot11staRSSIhistogramDuration		Unsigned32,

dot11staRSSIhistogramTime		TimeTicks,

dot11staTxRate			Unsigned32,

dot11staRxRate			Unsigned32,

dot11staRoamed			INTEGER,

dot11staLastSeen			TimeTicks}

-- >> J Kim: 11h TPC report results are kept under SMT branch

dot11staAddress OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The MACAddress of the STA/AP whose statistics are recorded

in this entry of the dot11staStatsTable."

::= { dot11staStatsEntry 1 }

dot11staBSSID OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The BSSID of the STA of this entry, if known.

If unknown, the value should be null."

::= { dot11staStatsEntry 2 }

dot11staSSID OBJECT-TYPE

SYNTAX OCTET STRING

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The SSID of the STA of this entry, if known.

If unknown, the value should be null."

::= { dot11staStatsEntry 3 }

dot11staTransmittedFragmentCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall be incremented for an MPDU

transmitted to the dot11staAddress in the current entry for

all types of fragments"

::= { dot11staStatsEntry 4 }

dot11RetryCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall increment when an MSDU is successfully

transmitted to the dot11staAddress in the current entry

after one or more retransmissions."

::= { dot11staStatsEntry 5 }

dot11staMultipleRetryCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall increment when an MSDU is successfully

transmitted after more than one retransmission to the dot11staAddress

in the current entry."

::= { dot11staStatsEntry 6 }

dot11staFrameDuplicateCount  OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall increment when a frame is received from

the dot11staAddress in the current entry

that the Sequence Control field indicates is a

duplicate."

::= { dot11staStatsEntry 7 }

dot11staRTSSuccessCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall increment when a CTS is received in

response to an RTS to the dot11staAddress in the current entry."

::= {  dot11staStatsEntry 8 }

dot11staRTSFailureCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall increment when a CTS is not received in

response to an RTS to the dot11staAddress in the current entry."

::= { dot11staStatsEntry 9 }

dot11staACKFailureCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall increment when an ACK is not received

when expected from the dot11staAddress in the current entry."

::= {  dot11staStatsEntry 10 }

dot11staReceivedFragmentCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall be incremented for each successfully

received MPDU of type Data or Management from the dot11staAddress

in the current entry."

::= { dot11staStatsEntry 11 }

dot11staMulticastReceivedFrameCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall increment when a MSDU is received

with the multicast bit set in the destination

MAC address from the dot11staAddress in the current entry."

::= { dot11staStatsEntry 12 }

dot11staTransmittedFrameCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall increment for each successfully transmitted MSDU

to the dot11staAddress in the current entry."

::= { dot11staStatsEntry 13 }

dot11staWEPUndecryptableCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall increment when a frame is received from the

dot11staAddress in the current entry

with the WEP subfield of the Frame Control field set to one and the

WEPOn value for the key mapped to the TA's MAC address

indicates that the frame should not have been encrypted or

that frame is discarded due to the receiving STA not

implementing the privacy option."

::= { dot11staStatsEntry 14 }

dot11staReceivedByteCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter indicates the total number of octets in MAC frame

bodies successfully received from the dot11staAddress in the current entry"

::= { dot11staStatsEntry 15 }

dot11staTransmittedByteCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter indicates the total number of octets in MAC frame

body successfully transmitted to the dot11staAddress in the current entry.

This excludes multicast and broadcast."

::= { dot11staStatsEntry 16 }

dot11staRSSI  OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This contains the RSSI value of the most recent fragment from

the dot11staAddress in the current entry."

-- >> J Kim: The exact definition in absolute scale for RSSI is currently pending in TGk

::= { dot11staStatsEntry 17 }

dot11staRxSignalQuality OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This contains the Signal Quality value of the most recent fragment from

the dot11staAddress in the current entry."

-- >> J Kim: The exact definition in absolute scale for Signal Quality is currently pending in TGk

::= { dot11staStatsEntry 18 }

dot11staType OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This contains the type of the dot11staAddress. The values are

0 : unknown

1 : AP

2 : STA, not AP

3 : ####

....more types

"

-- >> J Kim: The exact definition of types and additional types currently pending in TGk

::= { dot11staStatsEntry 19 }

dot11staState OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The state of the dot11staAddress relative to this STA and this ifIndex as

specified in 802.11 standard. If this STA is not an AP, the state should be

set to null. If this STA is in IBSS, ????????"

::= { dot11staStatsEntry 20 }

dot11staAID OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"If this STA is an AP on this ifIndex, this attribute contains the AID of

the dot11staAddress."

::= { dot11staStatsEntry 21 }

dot11staPHYType OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the phy type of the dot11staAddress in the current

entry. The definitions are

0: unknown

1: FH

2: DSSS US

3: DSSS Japan, etc. etc..

4: OFDM US, etc., etc....

"

-- >> J Kim: The exact definition of types and additional types currently pending in TGk or

-- >> 		do they already exist somewhere or a different variable similar to this?

::= { dot11staStatsEntry 22 }

dot11staCurrentChannel OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the channel number of the dot11staAddress in the

current entry. If in the same BSS as in this STA, the Current channel is the

same as this STA. If not in the same BSS for DSSS PHY, since DSSS packets are

decodable sometimes even not on the exact same channel, the attribute shall be

determined only when packets with channel number explicitly included

is decoded from the dot11staAddress, beacon, probe, probe response, association, etc.

OFDM does not have this issue. Thus, this can be determined during scans or other

activities away from the current channel."

::= { dot11staStatsEntry 23 }

dot11staSecurityStatus OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates whether unicast packets from the dot11staAddress in

the current entry has WEP bit set or other combinations of WEP usage monitored

by this STA

0: unknown

1: no WEP

2: WEP unicast, no WEP broadcast

3: full WEP no RSN

4: RSN, etc, etc.

"

-- >> J Kim: The exact definition of types and additional types currently pending in TGk or TGi?

-- >> J Kim: 1x MIB may cover similar information

-- >> J Kim: 11i MIB may cover similar information

::= { dot11staStatsEntry 24 }

dot11staRSSIhistogram OBJECT-TYPE

SYNTAX OCTET STRING

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Direct mapping of the TGh RSSI histogram report format. Each octet

corresponds to the RSSI bin values."

-- >> J Kim: TGk intends to create new defintion for RSSI and as a result there may be changes to this mapping.

-- 	     Also, for human readability, each bin may need to be assigned to a separate attribute.

::= { dot11staStatsEntry 25 }

dot11staRSSIhistogramDuration OBJECT-TYPE

SYNTAX 	Unsigned32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The duration in TU as defined in 11h for the above RSSI histogram

from the received packets from the dot11staAddress."

::= { dot11staStatsEntry 26 }

dot11staRSSIhistogramTime OBJECT-TYPE

SYNTAX TimeTicks

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The time at which the latest RSSI histogram was generated for the

received packets from the dot11staAddress"

-- >> J Kim: Like many other places, the issue of how to express time instance information needs to be settled.

::= { dot11staStatsEntry 27 }

dot11staTxRate OBJECT-TYPE

-- >> J Kim: This is based on Mike Moreton's suggestion in 02-652r1-RRM

SYNTAX Unsigned32 (0..127)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Provides a weighted average of the rates at which

recent frames were sucessfully transmitted.  Every time

a frame is sent with an address1 field equal to the value of

dot11staAddress for this row, this value is

modified as follows.  If R is the variable used to store

this value, and T was the rate at which the acknowleged

frame was sent, then

R = R * 0.7 + T * 0.3

R should be initialised to the maximum supported rate at

which frames can be sent to the station in question, and

should be held in a format of sufficient accuracy to allow

convergence.  R should be rounded to the nearest integer

before being returned as the value of this object.

The units for T are the same as those for rates as

described in the 'Supported Rates' Information Element."

::= { dot11staStatsEntry 28 }

dot11staRxRate OBJECT-TYPE

-- >> J Kim: This is based on Mike Moreton's suggestion in 02-652r1-RRM

SYNTAX Unsigned32 (0..127)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Provides a weighted average of the rates at which

recent frames were sucessfully received.  Every time

a frame is received with an address2 field equal to the value

of dot11staAddress for this row, this value is

modified as follows.  If R is the variable used to store

this value, and T was the rate at which the frame was

received, then

R = R * 0.7 + T * 0.3

R should be initialised to the maximum supported rate at

which frames can be received from the station in question, and

should be held in a format of sufficient accuracy to allow

convergence.  R should be rounded to the nearest integer

before being returned as the value of this object.

The units for T are the same as those for rates as

described in the 'Supported Rates' Information Element.

"

::= { dot11staStatsEntry 29 }

dot11staRoamed OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates whether the dot11staAddress is roamed or not

0: the dot11staAddress is still part of the BSS of this AP

1: the dot11staAddress has roamed

2: N/A for cases like the dot11staAddress was not part of BSS to begin with, this STA is not AP, etc.

"

-- >> J Kim: The exact definition of types and additional types currently pending in TGk or

-- >> J Kim: The value of AID can also reflect this information so maybe the above attribute is not needed

::= { dot11staStatsEntry 30 }

dot11staLastSeen OBJECT-TYPE

SYNTAX TimeTicks

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The time of the last activity of the dot11staAddress. This value and dot11staStatsTableAgingTime

is used	to determine whether this table entry can be purged to save memory."

-- >> J Kim: Like many other places, the issue of how to express time instance information needs to be settled.

::= { dot11staStatsEntry 31 }

-- **********************************************************************

-- *    end of dot11staStatsTable

-- **********************************************************************

-- >> J Kim: TGk addition ends

-- **********************************************************************

-- *    Resource Type Attribute Templates

-- **********************************************************************

dot11ResourceTypeIDName OBJECT-TYPE

SYNTAX DisplayString (SIZE(4))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Contains the name of the Resource Type ID managed object.

The attribute is read-only and always contains the value

RTID.  This attribute value shall not be used as a naming

attribute for any other managed object class."

REFERENCE "IEEE Std 802.1F-1993,  A.7"

DEFVAL { "RTID" }

::= { dot11resAttribute 1 }

-- **********************************************************************

-- *    dot11ResourceInfo  TABLE

-- **********************************************************************

dot11ResourceInfoTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11ResourceInfoEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Provides a means of indicating, in data readable from a

managed object, information that identifies the source of

the implementation."

REFERENCE "IEEE Std 802.1F-1993,  A.7"

::= { dot11resAttribute 2 }

dot11ResourceInfoEntry OBJECT-TYPE

SYNTAX Dot11ResourceInfoEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11ResourceInfo Table.

ifIndex - Each 802.11 interface is represented by an

ifEntry.  Interface tables in this MIB module are indexed

by ifIndex."

INDEX { ifIndex }

::= { dot11ResourceInfoTable 1 }

Dot11ResourceInfoEntry ::=

SEQUENCE {	dot11manufacturerOUI             OCTET STRING,

dot11manufacturerName            DisplayString,

dot11manufacturerProductName     DisplayString,

dot11manufacturerProductVersion  DisplayString }

dot11manufacturerOUI OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(3))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Takes the value of an organizationally unique identifier."

::= { dot11ResourceInfoEntry 1 }

dot11manufacturerName OBJECT-TYPE

SYNTAX DisplayString (SIZE(0..128))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"A printable string used to identify the manufacturer of the

resource.  Maximum string length is 128 octets."

::= { dot11ResourceInfoEntry 2 }

dot11manufacturerProductName OBJECT-TYPE

SYNTAX DisplayString (SIZE(0..128))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"A printable string used to identify the manufacturer's product

name of the resource.  Maximum string length is 128 octets."

::= { dot11ResourceInfoEntry 3 }

dot11manufacturerProductVersion OBJECT-TYPE

SYNTAX DisplayString (SIZE(0..128))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Printable string used to identify the manufacturer's product

version of the resource.  Maximum string length is 128 octets."

::= { dot11ResourceInfoEntry 4 }

-- **********************************************************************

-- *    End of dot11ResourceInfo  TABLE

-- **********************************************************************

-- **********************************************************************

-- *   PHY Attribute Templates

-- **********************************************************************

-- **********************************************************************

-- *    dot11PhyOperation  TABLE

-- **********************************************************************

dot11PhyOperationTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11PhyOperationEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"PHY level attributes concerned with

operation.  Implemented as a table indexed on

ifIndex to allow for multiple instantiations on an

Agent."

::= { dot11phy 1 }

dot11PhyOperationEntry OBJECT-TYPE

SYNTAX Dot11PhyOperationEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11PhyOperation Table.

ifIndex - Each 802.11 interface is represented by an

ifEntry.  Interface tables in this MIB module are indexed

by ifIndex."

INDEX { ifIndex }

::= { dot11PhyOperationTable 1 }

Dot11PhyOperationEntry ::=

SEQUENCE {	dot11PHYType            INTEGER,

dot11CurrentRegDomain   Integer32,

dot11TempType           INTEGER }

dot11PHYType OBJECT-TYPE

SYNTAX INTEGER { fhss(1), dsss(2), irbaseband(3), ofdm(4),

hrdsss(5) }

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is an 8-bit integer value that identifies the PHY type

supported by the attached PLCP and PMD. Currently defined

values and their corresponding PHY types are:

FHSS 2.4 GHz = 01 , DSSS 2.4 GHz = 02, IR Baseband = 03,

OFDM 5GHz = 04, HRDSSS = 05"

::= { dot11PhyOperationEntry 1 }

dot11CurrentRegDomain OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The current regulatory domain this instance of the PMD is

supporting.  This object corresponds to one of the

RegDomains listed in dot11RegDomainsSupported."

::= { dot11PhyOperationEntry 2 }

dot11TempType OBJECT-TYPE

SYNTAX INTEGER { tempType1(1), tempType2(2) }

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"There are different operating temperature requirements

dependent on the anticipated environmental conditions. This

attribute describes the current PHY's operating temperature

range capability. Currently defined values and their

corresponding temperature ranges are:

Type 1 = X'01'-Commercial range of 0 to 40 degrees C,

Type 2 = X'02'-Industrial range of -30 to 70 degrees C."

::= { dot11PhyOperationEntry 3 }

-- **********************************************************************

-- *    End of dot11PhyOperation  TABLE

-- **********************************************************************

-- **********************************************************************

-- *    dot11PhyAntenna  TABLE

-- **********************************************************************

dot11PhyAntennaTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11PhyAntennaEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Group of attributes for PhyAntenna.  Implemented as a

table indexed on ifIndex to allow for multiple instances on

an agent."

::= { dot11phy 2}

dot11PhyAntennaEntry OBJECT-TYPE

SYNTAX Dot11PhyAntennaEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11PhyAntenna Table.

ifIndex - Each 802.11 interface is represented by an

ifEntry.  Interface tables in this MIB module are indexed

by ifIndex."

INDEX { ifIndex }

::= { dot11PhyAntennaTable 1 }

Dot11PhyAntennaEntry ::=

SEQUENCE {	dot11CurrentTxAntenna  Integer32,

dot11DiversitySupport  INTEGER,

dot11CurrentRxAntenna  Integer32 }

dot11CurrentTxAntenna OBJECT-TYPE

SYNTAX Integer32 (1..255)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The current antenna being used to transmit.  This value

is one of the values appearing in dot11SupportedTxAntenna. This

may be used by a management agent to control which antenna is

used for transmission. "

::= { dot11PhyAntennaEntry 1 }

dot11DiversitySupport OBJECT-TYPE

SYNTAX INTEGER { fixedlist(1), notsupported(2), dynamic(3) }

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This implementation's support for diversity, encoded as:

X'01'-diversity is available and is performed over the fixed

list of antennas defined in dot11DiversitySelectionRx.

X'02'-diversity is not supported.

X'03'-diversity is supported and control of diversity is also

available, in which case the attribute

dot11DiversitySelectionRx can be dynamically modified by the

LME."

::= { dot11PhyAntennaEntry 2 }

dot11CurrentRxAntenna OBJECT-TYPE

SYNTAX Integer32 (1..255)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The current antenna being used to receive, if the dot11

DiversitySupport indicates that diversity is not supported.

The selected antenna shall be one of the antennae marked

for receive in the dot11AntennasListTable."

::= { dot11PhyAntennaEntry 3 }

-- **********************************************************************

-- *    End of dot11PhyAntenna  TABLE

-- **********************************************************************

-- **********************************************************************

-- *    dot11PhyTxPower  TABLE

-- **********************************************************************

dot11PhyTxPowerTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11PhyTxPowerEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Group of attributes for dot11PhyTxPowerTable.  Implemented

as a table indexed on STA ID to allow for multiple

instances on an Agent."

::= { dot11phy 3}

dot11PhyTxPowerEntry OBJECT-TYPE

SYNTAX Dot11PhyTxPowerEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11PhyTxPower Table.

ifIndex - Each 802.11 interface is represented by an

ifEntry.  Interface tables in this MIB module are indexed

by ifIndex."

INDEX { ifIndex }

::= { dot11PhyTxPowerTable 1 }

Dot11PhyTxPowerEntry ::=

SEQUENCE {	dot11NumberSupportedPowerLevels  INTEGER,

dot11TxPowerLevel1               INTEGER,

dot11TxPowerLevel2               INTEGER,

dot11TxPowerLevel3               INTEGER,

dot11TxPowerLevel4               INTEGER,

dot11TxPowerLevel5               INTEGER,

dot11TxPowerLevel6               INTEGER,

dot11TxPowerLevel7               INTEGER,

dot11TxPowerLevel8               INTEGER,

dot11CurrentTxPowerLevel         INTEGER,

-- >> J Kim: TGk addition begins'

dot11PreviousTxPowerLevel	 	INTEGER,

dot11AverageTxPower		 		INTEGER,

dot11AverageTXPowerDuration	 	Unsigned32,}

-- >> J Kim: TGk addition ends

dot11NumberSupportedPowerLevels OBJECT-TYPE

SYNTAX INTEGER (1..8)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The number of power levels supported by the PMD.

This attribute can have a value of 1 to 8."

::= { dot11PhyTxPowerEntry 1 }

dot11TxPowerLevel1 OBJECT-TYPE

SYNTAX INTEGER (0..10000)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The transmit output power for LEVEL1 in mW.

This is also the default power level."

::= { dot11PhyTxPowerEntry 2 }

dot11TxPowerLevel2 OBJECT-TYPE

SYNTAX INTEGER (0..10000)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The transmit output power for LEVEL2 in mW."

::= { dot11PhyTxPowerEntry 3 }

dot11TxPowerLevel3 OBJECT-TYPE

SYNTAX INTEGER (0..10000)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The transmit output power for LEVEL3 in mW."

::= { dot11PhyTxPowerEntry 4 }

dot11TxPowerLevel4 OBJECT-TYPE

SYNTAX INTEGER (0..10000)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The transmit output power for LEVEL4 in mW."

::= { dot11PhyTxPowerEntry 5 }

dot11TxPowerLevel5 OBJECT-TYPE

SYNTAX INTEGER (0..10000)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The transmit output power for LEVEL5 in mW."

::= { dot11PhyTxPowerEntry 6 }

dot11TxPowerLevel6 OBJECT-TYPE

SYNTAX INTEGER (0..10000)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The transmit output power for LEVEL6 in mW."

::= { dot11PhyTxPowerEntry 7 }

dot11TxPowerLevel7 OBJECT-TYPE

SYNTAX INTEGER (0..10000)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The transmit output power for LEVEL7 in mW."

::= { dot11PhyTxPowerEntry 8 }

dot11TxPowerLevel8 OBJECT-TYPE

SYNTAX INTEGER (0..10000)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The transmit output power for LEVEL8 in mW."

::= { dot11PhyTxPowerEntry 9 }

dot11CurrentTxPowerLevel OBJECT-TYPE

SYNTAX INTEGER (1..8)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The TxPowerLevel N currently being used to transmit data.

Some PHYs also use this value to determine the receiver

sensitivity requirements for CCA."

::= { dot11PhyTxPowerEntry 10 }

-- >> J Kim: TGk addition begins

-- variable definitions for added variables need to be put here

-- >> J Kim: TGk addition ends

-- **********************************************************************

-- *    End of dot11PhyTxPower  TABLE

-- **********************************************************************

-- **********************************************************************

-- *    dot11PhyFHSS  TABLE

-- **********************************************************************

dot11PhyFHSSTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11PhyFHSSEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Group of attributes for dot11PhyFHSSTable.  Implemented as a

table indexed on STA ID to allow for multiple instances on

an Agent."

::= { dot11phy 4 }

dot11PhyFHSSEntry OBJECT-TYPE

SYNTAX Dot11PhyFHSSEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11PhyFHSS Table.

ifIndex - Each 802.11 interface is represented by an

ifEntry.  Interface tables in this MIB module are indexed

by ifIndex."

INDEX { ifIndex }

::= { dot11PhyFHSSTable 1 }

Dot11PhyFHSSEntry ::=

SEQUENCE {	dot11HopTime   							INTEGER,

dot11CurrentChannelNumber  				INTEGER,

dot11MaxDwellTime  						INTEGER,

dot11CurrentDwellTime  					INTEGER,

dot11CurrentSet    						INTEGER,

dot11CurrentPattern    					INTEGER,

dot11CurrentIndex  						INTEGER,

dot11EHCCPrimeRadix						Integer32,

dot11EHCCNumberofChannelsFamilyIndex	Integer32,

dot11EHCCCapabilityImplemented			TruthValue,

dot11EHCCCapabilityEnabled				TruthValue,

dot11HopAlgorithmAdopted				INTEGER,

dot11RandomTableFlag					TruthValue,

dot11NumberofHoppingSets				Integer32,

dot11HopModulus							Integer32,

dot11HopOffset							Integer32,

-- >> J Kim: TGk addition starts

dot11FHCCABusyFraction					INTEGER,

dot11FHCCAMeasurementDuration			INTEGER,

dot11FHAverageNoisePower					Integer32,

dot11FHAverageCCAPositivePower 			Integer32,

dot11FHPLDUCRCError						Counter32}

-- >> J Kin: TGk addition ends

dot11HopTime OBJECT-TYPE

SYNTAX INTEGER (224)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The time in microseconds for the PMD to change from

channel 2 to channel 80."

::= { dot11PhyFHSSEntry 1 }

dot11CurrentChannelNumber OBJECT-TYPE

SYNTAX INTEGER (0..99)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The current channel number of the frequency output by the RF

synthesizer."

::= { dot11PhyFHSSEntry 2 }

dot11MaxDwellTime OBJECT-TYPE

SYNTAX INTEGER (1..65535)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The maximum time in TU that the transmitter

is permitted to operate on a single channel."

::= { dot11PhyFHSSEntry 3 }

dot11CurrentDwellTime OBJECT-TYPE

SYNTAX INTEGER (1..65535)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The current time in TU that the transmitter shall operate

on a single channel, as set by the MAC.  Default is 19 TU."

::= { dot11PhyFHSSEntry 4 }

dot11CurrentSet OBJECT-TYPE

SYNTAX INTEGER (1..255)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The current set of patterns the PHY

LME is using to determine the hopping sequence. "

::= { dot11PhyFHSSEntry 5 }

dot11CurrentPattern OBJECT-TYPE

SYNTAX INTEGER (0..255)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The current pattern the PHY LME is

using to determine the hop sequence."

::= { dot11PhyFHSSEntry 6 }

dot11CurrentIndex OBJECT-TYPE

SYNTAX INTEGER (1..255)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The current index value the PHY LME is using to determine

the CurrentChannelNumber."

::= { dot11PhyFHSSEntry 7 }

dot11EHCCPrimeRadix OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute indicates the value to be used as the prime radix

(N) in the HCC and EHCC algorithms."

::= { dot11PhyFHSSEntry 8 }

dot11EHCCNumberofChannelsFamilyIndex OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute indicates the value to be used as the maximum for the

family index (a) in the HCC and EHCC algorithms. The value of this field

shall not be less than the prime radix minus 3 (N - 3). The valid range

of allowed values is (N - 1), (N - 2), and (N - 3)."

::= { dot11PhyFHSSEntry 9 }

dot11EHCCCapabilityImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when TRUE, indicates that the station implementation

is capable of generating the HCC or EHCC algorithms for determining

Hopping patterns.  The capability is disabled, otherwise.  The default

value of this attribute is FALSE."

::= { dot11PhyFHSSEntry 10 }

dot11EHCCCapabilityEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when TRUE, indicates that the capability of the

station to operate using the HCC or EHCC algorithms for determining

Hopping Patterns is enabled.  The capability is disabled, otherwise.

The default value of this attribute is FALSE."

::= { dot11PhyFHSSEntry 11 }

dot11HopAlgorithmAdopted OBJECT-TYPE

SYNTAX INTEGER { crnt(1), hopindex(2), hcc(3) }

-- Modified by Daniel R. Borges on 02.14.2002 (compilation errors)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, indicates which of the algorithms will be used to generate

the Hopping Patterns. Valid Values are:

0 - hopping patterns as defined in clause 14 of the current version

of IEEE Std 802.11

1 - hop index method (with or without table)

2 - HCC/EHCC method"

::= { dot11PhyFHSSEntry 12 }

dot11RandomTableFlag OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, indicates that a Random Table is present when the value

is True. When the value is False it indicates that a Random Table is not

present and that the hop index method is to be used to determine the

hopping sequence. The default value of this attribute is True."

::= { dot11PhyFHSSEntry 13 }

dot11NumberofHoppingSets OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The Number of Sets field indicates the total number of sets within

the hopping patterns."

::= { dot11PhyFHSSEntry 14 }

dot11HopModulus OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The number of allowed channels for the hopping set. This is defined

by the governing regulatory agency for the country code of the country

in which this device is operating."

::= { dot11PhyFHSSEntry 15 }

dot11HopOffset OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The next position in the hopping set."

::= { dot11PhyFHSSEntry 16 }

-- >> J Kim: TGk addition begins

dot11FHCCABusyFraction OBJECT-TYPE

SYNTAX INTEGER (0..255)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"CCA Busy Fraction shall contain the fractional duration over which

CCA indicated the channel was busy during the measurement duration.

It is defined as Ceiling (255 * [Duration CCA indicated channel was busy] /

[Measurement duration])."

::= { dot11PhyFHSSEntry 17 }

dot11FHCCAMeasurementDuration OBJECT-TYPE

SYNTAX INTEGER (1..65535)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"Measurement Duration shall be set equal to the duration over which

the CCA was measured, expressed in TUs."

::= { dot11PhyFHSSEntry 18 }

dot11FHAverageNoisePower  OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The average Noise power level measured during CCA Negative in dBm averaged over

the last ?? seconds."

-- >> J Kim: More precise definition and the size of average window to be determined by TGk

::= { dot11PhyFHSSEntry 20 }

dot11FHAverageCCAPositivePower   OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The average Noise power level in dBm measured during CCA Positive but unsuccessful

PLDU header decoding, averaged over the last ?? seconds."

-- >> J Kim: More precise definition and the size of average window to be determined by TGk

::= { dot11PhyFHSSEntry 21 }

dot11FHPLDUCRCError   OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This counter shall increment each time CCA Positive and the PLDUCRC fails."

::= { dot11PhyFHSSEntry 22 }

-- >> J Kim: TGk addition ends

-- **********************************************************************

-- *    End of dot11PhyFHSS  TABLE

-- **********************************************************************

-- **********************************************************************

-- *    dot11PhyDSSSEntry  TABLE

-- **********************************************************************

dot11PhyDSSSTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11PhyDSSSEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Entry of attributes for dot11PhyDSSSEntry.  Implemented as a

table indexed on ifIndex allow for multiple instances on

an Agent."

::= { dot11phy 5 }

dot11PhyDSSSEntry OBJECT-TYPE

SYNTAX Dot11PhyDSSSEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11PhyDSSSEntry Table.

ifIndex - Each 802.11 interface is represented by an

ifEntry.  Interface tables in this MIB module are indexed

by ifIndex."

INDEX { ifIndex }

::= { dot11PhyDSSSTable 1 }

Dot11PhyDSSSEntry ::=

SEQUENCE {	dot11CurrentChannel    INTEGER,

dot11CCAModeSupported  INTEGER,

dot11CurrentCCAMode    INTEGER,

dot11EDThreshold       Integer32,

-- >> J Kim: TGk addition starts

dot11DSSSCCABusyFraction			INTEGER,

dot11DSSSCCAMeasurementDuration		INTEGER,

dot11DSSSAverageRxPower					INTEGER,

dot11DSSSAveragePxPowerDuration			Unsigned32,

dot11DSSSAverageNoisePower				Integer32,

dot11DSSSAverageCCAPositivePower 		Integer32,

dot11PLDUCRCError					Counter32}

-- >> J Kim: TGK addition ends

dot11CurrentChannel OBJECT-TYPE

SYNTAX INTEGER (1..14)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The current operating frequency channel of the DSSS

PHY. Valid channel numbers are as defined in 15.4.6.2"

::= { dot11PhyDSSSEntry 1 }

dot11CCAModeSupported OBJECT-TYPE

SYNTAX INTEGER (1..7)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"dot11CCAModeSupported is a bit-significant value,

representing all of the CCA modes supported by the PHY.

Valid values are:

energy detect only (ED_ONLY) = 01,

carrier sense only (CS_ONLY) = 02,

carrier sense and energy detect (ED_and_CS)= 04

or the logical sum of any of these values.  This

attribute shall not be used to indicate the CCA modes

supported by a higher rate extension PHY.  Rather, the

dot11HRCCAModeSupported attribute shall be used to

indicate the CCA modes of the higher rate extension PHY."

::= { dot11PhyDSSSEntry 2 }

dot11CurrentCCAMode OBJECT-TYPE

SYNTAX INTEGER { edonly(1), csonly(2), edandcs(4), cswithtimer(8),

hrcsanded(16) }

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The current CCA method in operation.   Valid values are:

energy detect only (edonly) = 01,

carrier sense only (csonly) = 02,

carrier sense and energy detect (edandcs)= 04

carrier sense with timer (cswithtimer)= 08

high rate carrier sense and energy detect (hrcsanded)=16."

::= { dot11PhyDSSSEntry 3 }

dot11EDThreshold OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The current Energy Detect Threshold being used by the DSSS PHY."

::= { dot11PhyDSSSEntry 4 }

-- >> J Kim: TGk addition begins

dot11DSSSCCABusyFraction OBJECT-TYPE

SYNTAX INTEGER (0..255)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"CCA Busy Fraction shall contain the fractional duration over which

CCA indicated the channel was busy during the measurement duration.

It is defined as Ceiling (255 * [Duration CCA indicated channel was busy] /

[Measurement duration])."

::= { dot11PhyDSSSEntry 5 }

dot11DSSSCCAMeasurementDuration OBJECT-TYPE

SYNTAX INTEGER (1..65535)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"Measurement Duration shall be set equal to the duration over which

the CCA was measured, expressed in TUs."

::= { dot11PhyDSSSEntry 6 }

dot11DSSSAverageNoisePower  OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The average Noise power level measured during CCA Negative in dBm averaged over

the last ?? seconds."

-- >> J Kim: More precise definition and the size of average window to be determined by TGk

::= { dot11PhyFHSSEntry 7 }

dot11DSSSAverageCCAPositivePower   OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The average Noise power level in dBm measured during CCA Positive but unsuccessful

PLDU header decoding, averaged over the last ?? seconds."

-- >> J Kim: More precise definition and the size of average window to be determined by TGk

::= { dot11PhyFHSSEntry 8 }

dot11DSSSPLDUCRCError   OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This counter shall increment each time CCA Positive and the PLDUCRC fails."

::= { dot11PhyFHSSEntry 9 }

-- >> J Kim: TGk addition ends

-- **********************************************************************

-- *    End of dot11PhyDSSSEntry  TABLE

-- **********************************************************************

-- **********************************************************************

-- *    dot11PhyIR  TABLE

-- **********************************************************************

dot11PhyIRTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11PhyIREntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Group of attributes for dot11PhyIRTable.  Implemented as a

table indexed on ifIndex to allow for multiple instances on

an Agent."

::= { dot11phy 6 }

dot11PhyIREntry OBJECT-TYPE

SYNTAX Dot11PhyIREntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11PhyIR Table.

ifIndex - Each 802.11 interface is represented by an

ifEntry.  Interface tables in this MIB module are indexed

by ifIndex."

INDEX { ifIndex }

::= { dot11PhyIRTable 1 }

Dot11PhyIREntry ::=

SEQUENCE {	dot11CCAWatchdogTimerMax       Integer32,

dot11CCAWatchdogCountMax       Integer32,

dot11CCAWatchdogTimerMin       Integer32,

dot11CCAWatchdogCountMin       Integer32 }

dot11CCAWatchdogTimerMax OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This parameter, together with CCAWatchdogCountMax,

determines when energy detected in the channel can be

ignored."

::= { dot11PhyIREntry 1 }

dot11CCAWatchdogCountMax OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This parameter, together with CCAWatchdogTimerMax,

determines when energy detected in the channel can be

ignored."

::= { dot11PhyIREntry 2 }

dot11CCAWatchdogTimerMin OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The minimum value to which CCAWatchdogTimerMax can be

set."

::= { dot11PhyIREntry 3 }

dot11CCAWatchdogCountMin OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The minimum value to which CCAWatchdogCount can be set."

::= { dot11PhyIREntry 4 }

-- **********************************************************************

-- *    End of dot11PhyIR  TABLE

-- **********************************************************************

-- **********************************************************************

-- *    dot11RegDomainsSupported  TABLE

-- **********************************************************************

dot11RegDomainsSupportedTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11RegDomainsSupportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"There are different operational requirements dependent on

the regulatory domain.  This attribute list describes the

regulatory domains the PLCP and PMD support in this

implementation.  Currently defined values and their

corresponding Regulatory Domains are:

FCC (USA) = X'10', DOC (Canada) = X'20', ETSI (most of

Europe) = X'30', Spain = X'31', France = X'32', MKK

(Japan) = X'40' "

::= { dot11phy 7}

dot11RegDomainsSupportEntry OBJECT-TYPE

SYNTAX Dot11RegDomainsSupportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11RegDomainsSupport Table.

ifIndex - Each 802.11 interface is represented by an

ifEntry.  Interface tables in this MIB module are indexed

by ifIndex."

INDEX { ifIndex,

dot11RegDomainsSupportIndex }

::= { dot11RegDomainsSupportedTable 1 }

Dot11RegDomainsSupportEntry ::=

SEQUENCE {	dot11RegDomainsSupportIndex    Integer32,

dot11RegDomainsSupportValue    INTEGER }

dot11RegDomainsSupportIndex OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"The auxiliary variable used to identify instances

of the columnar objects in the RegDomainsSupport Table."

::= { dot11RegDomainsSupportEntry 1 }

dot11RegDomainsSupportValue OBJECT-TYPE

SYNTAX INTEGER { fcc(16), doc(32), etsi(48), spain (49), france(50),

mkk (64) }

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"There are different operational requirements dependent on

the regulatory domain.  This attribute list describes the

regulatory domains the PLCP and PMD support in this

implementation.  Currently de- fined values and their

corresponding Regulatory Domains are:

FCC (USA) = X'10', DOC (Canada) = X'20', ETSI (most of

Europe) = X'30', Spain = X'31', France = X'32', MKK

(Japan) = X'40' "

::= { dot11RegDomainsSupportEntry 2 }

-- **********************************************************************

-- *    End of dot11RegDomainsSupported  TABLE

-- **********************************************************************

-- **********************************************************************

-- *    dot11AntennasList  TABLE

-- **********************************************************************

dot11AntennasListTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11AntennasListEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"This table represents the list of antennae.  An antenna can be

marked to be capable of transmitting, receiving, and/or for

participation in receive diversity.  Each entry in this table

represents a single antenna with its properties.  The maximum

number of antennae that can be contained in this table is 255."

::= { dot11phy 8 }

dot11AntennasListEntry OBJECT-TYPE

SYNTAX Dot11AntennasListEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11AntennasListTable, representing the properties

of a single antenna.

ifIndex - Each 802.11 interface is represented by an

ifEntry.  Interface tables in this MIB module are indexed

by ifIndex."

INDEX { ifIndex,

dot11AntennaListIndex }

::= { dot11AntennasListTable 1 }

Dot11AntennasListEntry ::=

SEQUENCE {	dot11AntennaListIndex     Integer32,

dot11SupportedTxAntenna   TruthValue,

dot11SupportedRxAntenna   TruthValue,

dot11DiversitySelectionRx TruthValue }

dot11AntennaListIndex OBJECT-TYPE

SYNTAX Integer32 (1..255)

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"The unique index of an antenna which is used to identify the columnar

objects in the dot11AntennasList Table."

::= { dot11AntennasListEntry 1 }

dot11SupportedTxAntenna OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"When true, this object indicates that the antenna represented by

dot11AntennaIndex can be used as a transmit antenna."

::= { dot11AntennasListEntry 2 }

dot11SupportedRxAntenna OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"When true, this object indocates that the antenna represented by the

dot11AntennaIndex xan be used as a receive antenna."

::= { dot11AntennasListEntry 3 }

dot11DiversitySelectionRx OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"When true, this object indicates that the antenna represented by

dot11AntennaIndex can be used for receive diversity.  This object

may only be true if the antenna can be used as a receive antenna,

as indicated by dot11SupportedRxAntenna."

::= { dot11AntennasListEntry 4 }

-- **********************************************************************

-- *    End of dot11AntennasList  TABLE

-- **********************************************************************

-- **********************************************************************

-- *    SupportedDataRatesTx  TABLE

-- **********************************************************************

dot11SupportedDataRatesTxTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11SupportedDataRatesTxEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"The Transmit bit rates supported by the PLCP and PMD,

represented by a count from X'02-X'7f, corresponding to data

rates in increments of 500Kb/s from 1 Mb/s to 63.5 Mb/s subject

to limitations of each individual PHY."

::= { dot11phy 9 }

dot11SupportedDataRatesTxEntry OBJECT-TYPE

SYNTAX Dot11SupportedDataRatesTxEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An Entry (conceptual row) in the dot11SupportedDataRatesTx

Table.

ifIndex - Each 802.11 interface is represented by an

ifEntry.  Interface tables in this MIB module are indexed

by ifIndex."

INDEX { ifIndex,

dot11SupportedDataRatesTxIndex }

::= { dot11SupportedDataRatesTxTable  1 }

Dot11SupportedDataRatesTxEntry ::=

SEQUENCE {	dot11SupportedDataRatesTxIndex  Integer32,

dot11SupportedDataRatesTxValue  Integer32 }

dot11SupportedDataRatesTxIndex OBJECT-TYPE

SYNTAX Integer32 (1..8)

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Index object which identifies which data rate to access.

Range is 1..8."

::= { dot11SupportedDataRatesTxEntry 1 }

dot11SupportedDataRatesTxValue OBJECT-TYPE

SYNTAX Integer32 (2..127)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The Transmit bit rates supported by the PLCP and PMD,

represented by a count from X'02-X'7f, corresponding to data

rates in increments of 500Kb/s from 1 Mb/s to 63.5 Mb/s subject

to limitations of each individual PHY."

::= { dot11SupportedDataRatesTxEntry 2 }

-- **********************************************************************

-- *    End of dot11SupportedDataRatesTx  TABLE

-- **********************************************************************

-- **********************************************************************

-- *    SupportedDataRatesRx  TABLE

-- **********************************************************************

dot11SupportedDataRatesRxTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11SupportedDataRatesRxEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"The receive bit rates supported by the PLCP and PMD,

represented by a count from X'002-X'7f, corresponding to data

rates in increments of 500Kb/s from 1 Mb/s to 63.5 Mb/s."

::= { dot11phy 10 }

dot11SupportedDataRatesRxEntry OBJECT-TYPE

SYNTAX Dot11SupportedDataRatesRxEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An Entry (conceptual row) in the dot11SupportedDataRatesRx Table.

ifIndex - Each 802.11 interface is represented by an

ifEntry.  Interface tables in this MIB module are indexed

by ifIndex."

INDEX { ifIndex,

dot11SupportedDataRatesRxIndex }

::= { dot11SupportedDataRatesRxTable  1 }

Dot11SupportedDataRatesRxEntry ::=

SEQUENCE {	dot11SupportedDataRatesRxIndex  Integer32,

dot11SupportedDataRatesRxValue  Integer32 }

dot11SupportedDataRatesRxIndex OBJECT-TYPE

SYNTAX Integer32 (1..8)

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Index object which identifies which data rate to access.

Range is 1..8."

::= { dot11SupportedDataRatesRxEntry 1 }

dot11SupportedDataRatesRxValue OBJECT-TYPE

SYNTAX Integer32 (2..127)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The receive bit rates supported by the PLCP and PMD,

represented by a count from X'02-X'7f, corresponding to data

rates in increments of 500Kb/s from 1 Mb/s to 63.5 Mb/s."

::= { dot11SupportedDataRatesRxEntry 2 }

-- **********************************************************************

-- *    End of dot11SupportedDataRatesRx  TABLE

-- **********************************************************************

--**********************************************************************

-- * dot11PhyOFDM TABLE

-- * Added by Daniel R. Borges on 02.14.2002 (802.11a)

--**********************************************************************

dot11PhyOFDMTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11PhyOFDMEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Group of attributes for dot11PhyOFDMTable. Implemented as a

table indexed on ifindex to allow for multiple instances on

an Agent."

::= { dot11phy 11 }

dot11PhyOFDMEntry OBJECT-TYPE

SYNTAX Dot11PhyOFDMEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11PhyOFDM Table.

ifIndex - Each IEEE 802.11 interface is represented by an

ifEntry. Interface tables in this MIB module are indexed

by ifIndex."

INDEX { ifIndex }

::= { dot11PhyOFDMTable 1 }

Dot11PhyOFDMEntry ::=

SEQUENCE {	dot11CurrentFrequency        INTEGER,

dot11TIThreshold             Integer32,

dot11FrequencyBandsSupported INTEGER,

-- >> J Kim: TGk addition Begins

dot11OFDMCCABusyFraction     INTEGER,

dot11OFDMCCAMeasurementDuration INTEGER,

dot11AverageRxPower	     INTEGER,

dot11AveragePxPowerDuration  Unsigned32,

dot11AverageNoisePower	     Integer32,

dot11AverageCCAPositivePower Integer32,

dot11PLDUCRCError	     Counter32}

-- >> J Kim: TGk addition ends

dot11CurrentFrequency OBJECT-TYPE

SYNTAX INTEGER (0..99)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The number of the current operating frequency channel of the OFDM PHY."

::= { dot11PhyOFDMEntry 1 }

dot11TIThreshold OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The Threshold being used to detect a busy medium (frequency).

CCA shall report a busy medium upon detecting the RSSI above this threshold."

::= { dot11PhyOFDMEntry 2 }

dot11FrequencyBandsSupported OBJECT-TYPE

SYNTAX INTEGER (1..7)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The capability of the OFDM PHY implementation to operate in the three U-NII

bands. Coded as an integer value of a three bit field as follows:

bit 0 .. capable of operating in the lower (5.15-5.25 GHz) U-NII band

bit 1 .. capable of operating in the middle (5.25-5.35 GHz) U-NII band

bit 2 .. capable of operating in the upper (5.725-5.825 GHz) U-NII band

For example, for an implementation capable of operating in the lower and mid

bands this attribute would take the value 3."

::= { dot11PhyOFDMEntry 3 }

-- >> J Kim: TGk addition begins

dot11OFDMCCABusyFraction OBJECT-TYPE

SYNTAX INTEGER (0..255)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"CCA Busy Fraction shall contain the fractional duration over which CCA indicated

the channel was busy during the measurement duration. It is defined as Ceiling

(255 * [Duration CCA indicated channel was busy] / [Measurement duration])."

::= { dot11PhyHRDSSSEntry 4 }

dot11OFDMCCAMeasurementDuration OBJECT-TYPE

SYNTAX INTEGER (1..65535)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"Measurement Duration shall be set equal to the duration over which

the CCA was measured, expressed in TUs."

::= { dot11PhyHRDSSSEntry 5 }

dot11OFDMAverageNoisePower  OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The average Noise power level measured during CCA Negative in dBm averaged over

the last ?? seconds."

-- >> J Kim: More precise definition and the size of average window to be determined by TGk

::= { dot11PhyFHSSEntry 6 }

dot11OFDMAverageCCAPositivePower   OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The average Noise power level in dBm measured during CCA Positive but unsuccessful

PLDU header decoding, averaged over the last ?? seconds."

-- >> J Kim: More precise definition and the size of average window to be determined by TGk

::= { dot11PhyFHSSEntry 7 }

dot11OFDMPLDUCRCError   OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This counter shall increment each time CCA Positive and the PLDUCRC fails."

::= { dot11PhyFHSSEntry 8 }

-- >> J Kim: TGk addition ends

-- **********************************************************************

-- * End of dot11PhyOFDM TABLE

-- **********************************************************************

-- **********************************************************************

-- *	dot11PhyHRDSSSEntry TABLE

-- **********************************************************************

dot11PhyHRDSSSTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11PhyHRDSSSEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Entry of attributes for dot11PhyHRDSSSEntry.

Implemented as a table indexed on ifIndex to allow for

multiple instances on an Agent."

::= { dot11phy 12 }

dot11PhyHRDSSSEntry OBJECT-TYPE

SYNTAX Dot11PhyHRDSSSEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11PhyHRDSSSEntry Table.

ifIndex - Each 802.11 interface is represented by an

ifEntry. Interface tables in this MIB module are indexed

by ifIndex."

INDEX { ifIndex }

::= { dot11PhyHRDSSSTable 1 }

Dot11PhyHRDSSSEntry ::=

SEQUENCE {	dot11ShortPreambleOptionImplemented	TruthValue,

dot11PBCCOptionImplemented		TruthValue,

dot11ChannelAgilityPresent 		TruthValue,

dot11ChannelAgilityEnabled 		TruthValue,

dot11HRCCAModeSupported			INTEGER,

-- >> J Kim: TGk addition starts

dot11HRCCABusyFraction			INTEGER,

dot11HRCCAMeasurementDuration		INTEGER,

dot11AverageRxPower			INTEGER,

dot11AveragePxPowerDuration		Unsigned32,

dot11AverageNoisePower			Integer32,

dot11AverageCCAPositivePower 		Integer32,

dot11PLDUCRCError			Counter32}

-- >> J Kim: TGK addition ends}

dot11ShortPreambleOptionImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute, when true, shall indicate that the

short preamble option as defined in subclause 18.2.2.2

is implemented.  The default value of this attribute

shall be false."

::= {dot11PhyHRDSSSEntry 1 }

dot11PBCCOptionImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute, when true, shall indicate that the PBCC

modulation option as defined in subclause 18.4.6.6 is

implemented.  The default value of this attribute shall

be false."

::= {dot11PhyHRDSSSEntry 2 }

dot11ChannelAgilityPresent OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates that the PHY is capable of

channel agility."

::= { dot11PhyHRDSSSEntry 3 }

dot11ChannelAgilityEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates that the PHY channel agility

functionality is enabled."

::= { dot11PhyHRDSSSEntry 4 }

dot11HRCCAModeSupported OBJECT-TYPE

SYNTAX INTEGER (1..31)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"dot11HRCCAModeSupported is a bit-significant value,

representing all of the CCA modes supported by the PHY.

Valid values are:

energy detect only (ED_ONLY) = 01,

carrier sense only (CS_ONLY) = 02,

carrier sense and energy detect (ED_and_CS)= 04,

carrier sense with timer (CS_and_Timer)= 08,

high rate carrier sense and energy detect (HRCS_and_ED)= 16

or the logical sum of any of these values.  In the high

rate extension PHY, this attribute shall be used in

preference to the dot11CCAModeSupported attribute."

::= { dot11PhyHRDSSSEntry 5 }

-- >> J Kim: TGk addition begins

dot11HRCCABusyFraction OBJECT-TYPE

SYNTAX INTEGER (0..255)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"CCA Busy Fraction shall contain the fractional duration over which CCA indicated

the channel was busy during the measurement duration. It is defined as Ceiling

(255 * [Duration CCA indicated channel was busy] / [Measurement duration])."

::= { dot11PhyHRDSSSEntry 6 }

dot11HRCCAMeasurementDuration OBJECT-TYPE

SYNTAX INTEGER (1..65535)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"Measurement Duration shall be set equal to the duration over which

the CCA was measured, expressed in TUs."

::= { dot11PhyHRDSSSEntry 7 }

dot11HRAverageNoisePower  OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The average Noise power level measured during CCA Negative in dBm averaged over

the last ?? seconds."

-- >> J Kim: More precise definition and the size of average window to be determined by TGk

::= { dot11PhyFHSSEntry 8 }

dot11HRAverageCCAPositivePower   OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The average Noise power level in dBm measured during CCA Positive but unsuccessful

PLDU header decoding, averaged over the last ?? seconds."

-- >> J Kim: More precise definition and the size of average window to be determined by TGk

::= { dot11PhyFHSSEntry 9 }

dot11HRPLDUCRCError   OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This counter shall increment each time CCA Positive and the PLDUCRC fails."

::= { dot11PhyFHSSEntry 10 }

-- >> J Kim: TGk addition ends

-- **********************************************************************

-- * End of dot11PhyHRDSSSEntry TABLE

-- **********************************************************************

-- ********************************************************************

-- * dot11 Hopping Pattern TABLE

-- ********************************************************************

dot11HoppingPatternTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11HoppingPatternEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"The (conceptual) table of attributes necessary for a frequency

hopping implementation to be able to create the hopping sequences

necessary to operate in the subband for the associated domain country

string."

::= { dot11phy 13 }

dot11HoppingPatternEntry OBJECT-TYPE

SYNTAX Dot11HoppingPatternEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry (conceptual row) in the Hopping Pattern Table that indicates the

random hopping sequence to be followed.

IfIndex - Each 802.11 interface is represented by an ifEntry. Interface

tables in this MIB are indexed by ifIndex."

INDEX { ifIndex,

dot11HoppingPatternIndex }

::= { dot11HoppingPatternTable 1 }

Dot11HoppingPatternEntry ::=

SEQUENCE {	dot11HoppingPatternIndex		Integer32,

dot11RandomTableFieldNumber		Integer32 }

dot11HoppingPatternIndex OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"The auxiliary variable used to identify instances of the columnar

objects in the Hopping Pattern Table."

::= { dot11HoppingPatternEntry 1}

dot11RandomTableFieldNumber OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall indicate the value of the starting channel

number in the hopping sequence of the subband for the associated

domain country string. The default value of this attribute shall

be zero."

::= { dot11HoppingPatternEntry 2}

-- **********************************************************************

-- * End of dot11 Hopping Pattern TABLE

--**********************************************************************

-- **********************************************************************

-- * Conformance Information

-- **********************************************************************

dot11Conformance  	OBJECT IDENTIFIER ::= { ieee802dot11 5 }

dot11Groups  		OBJECT IDENTIFIER ::= { dot11Conformance 1 }

dot11Compliances  	OBJECT IDENTIFIER ::= { dot11Conformance 2 }

-- **********************************************************************

-- * Compliance Statements

-- * Modified by Daniel R. Borges on 02.14.2002 (802.11a)

-- **********************************************************************

dot11Compliance MODULE-COMPLIANCE

STATUS  current

DESCRIPTION

"The compliance statement for SNMPv2 entities

that implement the IEEE 802.11 MIB."

MODULE  -- this module

MANDATORY-GROUPS {	dot11SMTbase2,

dot11MACbase, dot11CountersGroup,

dot11SmtAuthenticationAlgorithms,

dot11ResourceTypeID, dot11PhyOperationComplianceGroup }

GROUP dot11PhyDSSSComplianceGroup

DESCRIPTION

"Implementation of this group is required when object

dot11PHYType has the value of dsss.  This group is

mutually exclusive with the groups dot11PhyIRComplianceGroup,

dot11PhyFHSSComplianceGroup and dot11PhyOFDMComplianceGroup."

-- Modified by Daniel R. Borges on 02.14.2002 (802.11a)

GROUP dot11PhyIRComplianceGroup

DESCRIPTION

"Implementation of this group is required when object

dot11PHYType has the value of irbaseband.  This group is

mutually exclusive with the groups dot11PhyDSSSComplianceGroup,

dot11PhyFHSSComplianceGroup and dot11PhyOFDMComplianceGroup."

-- Modified by Daniel R. Borges on 02.14.2002 (802.11a)

GROUP dot11PhyFHSSComplianceGroup

DESCRIPTION

"Implementation of this group is required when object

dot11PHYType has the value of fhss.  This group is

mutually exclusive with the groups dot11PhyDSSSComplianceGroup,

dot11PhyIRComplianceGroup and dot11PhyOFDMComplianceGroup."

-- Modified by Daniel R. Borges on 02.14.2002 (802.11a)

GROUP dot11PhyOFDMComplianceGroup

DESCRIPTION

"Implementation of this group is required when object

dot11PHYType has the value of ofdm. This group is

mutually exclusive with the groups dot11PhyDSSSComplianceGroup,

dot11PhyIRComplianceGroup and dot11PhyFHSSComplianceGroup."

-- Modified by Daniel R. Borges on 02.14.2002 (802.11a)

GROUP dot11PhyHRDSSSComplianceGroup

DESCRIPTION

"Implementation of this group is required when object

dot11PHYType has the value of hrdsss. This group is

mutually exclusive with the groups

dot11PhyDSSSComplianceGroup, dot11PhyIRComplianceGroup

and dot11PhyFHSSComplianceGroup, and dot11PhyOFDMComplianceGroup."

-- Modified by Daniel R. Borges on 02.14.2002 (802.11a)

-- OPTIONAL-GROUPS { dot11SMTprivacy, dot11MACStatistics,

--    dot11PhyAntennaComplianceGroup, dot11PhyTxPowerComplianceGroup,

--    dot11PhyRegDomainsSupportGroup,

--    dot11PhyAntennasListGroup, dot11PhyRateGroup }

::= { dot11Compliances 1 }

-- >> J Kim: TGk addition begins

-- TGk conformance statement

-- >> J Kim: TGk addition ends

-- **********************************************************************

-- *   Groups - units of conformance

-- **********************************************************************

dot11SMTbase OBJECT-GROUP

OBJECTS {	dot11StationID, dot11MediumOccupancyLimit,

dot11CFPollable,

dot11CFPPeriod,

dot11CFPMaxDuration,

dot11AuthenticationResponseTimeOut,

dot11PrivacyOptionImplemented,

dot11PowerManagementMode,

dot11DesiredSSID, dot11DesiredBSSType,

dot11OperationalRateSet,

dot11BeaconPeriod, dot11DTIMPeriod,

dot11AssociationResponseTimeOut }

STATUS deprecated

DESCRIPTION

"The SMT object class provides the necessary support at the

STA to manage the processes in the STA such that the STA may

work cooperatively as a part of an IEEE 802.11 network."

::= { dot11Groups 1 }

dot11SMTprivacy OBJECT-GROUP

OBJECTS { 	dot11PrivacyInvoked,

dot11WEPKeyMappingLength, dot11ExcludeUnencrypted,

dot11WEPICVErrorCount , dot11WEPExcludedCount ,

dot11WEPDefaultKeyID,

dot11WEPDefaultKeyValue,

dot11WEPKeyMappingWEPOn,

dot11WEPKeyMappingValue , dot11WEPKeyMappingAddress,

dot11WEPKeyMappingStatus }

STATUS current

DESCRIPTION

"The SMTPrivacy package is a set of attributes that shall be

present if WEP is implemented in the STA."

::= { dot11Groups 2 }

dot11MACbase OBJECT-GROUP

OBJECTS {	dot11MACAddress, dot11Address,

dot11GroupAddressesStatus,

dot11RTSThreshold, dot11ShortRetryLimit,

dot11LongRetryLimit, dot11FragmentationThreshold,

dot11MaxTransmitMSDULifetime,

dot11MaxReceiveLifetime, dot11ManufacturerID,

dot11ProductID }

STATUS current

DESCRIPTION

"The MAC object class provides the necessary support for the

access control, generation, and verification of frame check

sequences, and proper delivery of valid data to upper

layers."

::= { dot11Groups 3 }

dot11MACStatistics OBJECT-GROUP

OBJECTS { 	dot11RetryCount, dot11MultipleRetryCount,

dot11RTSSuccessCount, dot11RTSFailureCount,

dot11ACKFailureCount, dot11FrameDuplicateCount }

STATUS current

DESCRIPTION

"The MACStatistics package provides extended statistical

information on the operation of the MAC.  This

package is completely optional."

::= { dot11Groups 4 }

dot11ResourceTypeID OBJECT-GROUP

OBJECTS {	dot11ResourceTypeIDName, dot11manufacturerOUI,

dot11manufacturerName, dot11manufacturerProductName,

dot11manufacturerProductVersion }

STATUS current

DESCRIPTION

"Attributes used to identify a STA, its manufacturer,

and various product names and versions."

::= { dot11Groups 5 }

dot11SmtAuthenticationAlgorithms OBJECT-GROUP

OBJECTS {	dot11AuthenticationAlgorithm,

dot11AuthenticationAlgorithmsEnable }

STATUS current

DESCRIPTION

"Authentication Algorithm Table."

::= { dot11Groups 6 }

dot11PhyOperationComplianceGroup OBJECT-GROUP

OBJECTS { 	dot11PHYType, dot11CurrentRegDomain, dot11TempType }

STATUS current

DESCRIPTION

"PHY layer operations attributes."

::= { dot11Groups 7 }

dot11PhyAntennaComplianceGroup OBJECT-GROUP

OBJECTS {	dot11CurrentTxAntenna, dot11DiversitySupport,

dot11CurrentRxAntenna }

STATUS current

DESCRIPTION

"Attributes for Data Rates for IEEE 802.11."

::= { dot11Groups 8 }

dot11PhyTxPowerComplianceGroup OBJECT-GROUP

OBJECTS {	dot11NumberSupportedPowerLevels, dot11TxPowerLevel1,

dot11TxPowerLevel2, dot11TxPowerLevel3, dot11TxPowerLevel4,

dot11TxPowerLevel5, dot11TxPowerLevel6, dot11TxPowerLevel7,

dot11TxPowerLevel8, dot11CurrentTxPowerLevel }

STATUS current

DESCRIPTION

"Attributes for Control and Management of transmit power."

::= { dot11Groups 9 }

dot11PhyFHSSComplianceGroup OBJECT-GROUP

OBJECTS {	dot11HopTime, dot11CurrentChannelNumber, dot11MaxDwellTime,

dot11CurrentDwellTime, dot11CurrentSet, dot11CurrentPattern,

dot11CurrentIndex}

STATUS current

DESCRIPTION

"Attributes that configure the Frequency Hopping for IEEE

802.11."

::= { dot11Groups 10 }

dot11PhyDSSSComplianceGroup OBJECT-GROUP

OBJECTS {	dot11CurrentChannel, dot11CCAModeSupported,

dot11CurrentCCAMode, dot11EDThreshold}

STATUS current

DESCRIPTION

"Attributes that configure the DSSS for IEEE 802.11."

::= { dot11Groups 11 }

dot11PhyIRComplianceGroup OBJECT-GROUP

OBJECTS {	dot11CCAWatchdogTimerMax, dot11CCAWatchdogCountMax,

dot11CCAWatchdogTimerMin, dot11CCAWatchdogCountMin}

STATUS current

DESCRIPTION

"Attributes that configure the baseband IR for IEEE 802.11."

::= { dot11Groups 12 }

dot11PhyRegDomainsSupportGroup OBJECT-GROUP

OBJECTS { 	dot11RegDomainsSupportValue }

STATUS current

DESCRIPTION

"Attributes that specify the supported Regulation Domains."

::= { dot11Groups 13}

dot11PhyAntennasListGroup OBJECT-GROUP

OBJECTS { 	dot11SupportedTxAntenna,

dot11SupportedRxAntenna, dot11DiversitySelectionRx }

STATUS current

DESCRIPTION

"Attributes that specify the supported Regulation Domains."

::= { dot11Groups 14 }

dot11PhyRateGroup OBJECT-GROUP

OBJECTS {	dot11SupportedDataRatesTxValue,

dot11SupportedDataRatesRxValue }

STATUS current

DESCRIPTION

"Attributes for Data Rates for IEEE 802.11."

::= { dot11Groups 15 }

dot11CountersGroup OBJECT-GROUP

OBJECTS { 	dot11TransmittedFragmentCount,

dot11MulticastTransmittedFrameCount,

dot11FailedCount, dot11ReceivedFragmentCount,

dot11MulticastReceivedFrameCount,

dot11FCSErrorCount,

dot11WEPUndecryptableCount,

dot11TransmittedFrameCount }

STATUS current

DESCRIPTION

"Attributes from the dot11CountersGroup that are not described

in the dot11MACStatistics group.  These objects are

mandatory."

::= { dot11Groups 16 }

dot11PhyOFDMComplianceGroup OBJECT-GROUP

OBJECTS {	dot11CurrentFrequency,

dot11TIThreshold,

dot11FrequencyBandsSupported }

STATUS current

DESCRIPTION

"Attributes that configure the OFDM for IEEE 802.11."

::= { dot11Groups 17 }

-- Added by Daniel R. Borges on 02.14.2002 (802.11a)

dot11NotificationGroup NOTIFICATION-GROUP

NOTIFICATIONS {	dot11Disassociate,

dot11Deauthenticate,

dot11AuthenticateFail }

STATUS current

DESCRIPTION

"IEEE 802.11 notifications"

::= { dot11Groups 18 }

-- Modified by Daniel R. Borges on 12.04.2002 (802.11a)

dot11SMTbase2 OBJECT-GROUP

OBJECTS { 	dot11MediumOccupancyLimit,

dot11CFPollable,

dot11CFPPeriod,

dot11CFPMaxDuration,

dot11AuthenticationResponseTimeOut,

dot11PrivacyOptionImplemented,

dot11PowerManagementMode,

dot11DesiredSSID, dot11DesiredBSSType,

dot11OperationalRateSet,

dot11BeaconPeriod, dot11DTIMPeriod,

dot11AssociationResponseTimeOut,

dot11DisassociateReason,

dot11DisassociateStation,

dot11DeauthenticateReason,

dot11DeauthenticateStation,

dot11AuthenticateFailStatus,

dot11AuthenticateFailStation }

STATUS current

DESCRIPTION

"The SMTbase2 object class provides the necessary support at the

STA to manage the processes in the STA such that the STA may

work cooperatively as a part of an IEEE 802.11 network."

::= { dot11Groups 19 }

-- Modified by Daniel R. Borges on 02.14.2002 (802.11a)

dot11PhyHRDSSSComplianceGroup OBJECT-GROUP

OBJECTS {	dot11CurrentChannel, dot11CCAModeSupported,

dot11CurrentCCAMode, dot11EDThreshold,

dot11ShortPreambleOptionImplemented,

dot11PBCCOptionImplemented, dot11ChannelAgilityPresent,

dot11ChannelAgilityEnabled, dot11HRCCAModeSupported }

STATUS current

DESCRIPTION

"Attributes that configure the HRDSSS for IEEE 802.11."

::= { dot11Groups 20 }

-- Modified by Daniel R. Borges on 02.14.2002 (802.11a)

dot11SMTbase3 OBJECT-GROUP

OBJECTS { 	dot11MediumOccupancyLimit,

dot11CFPollable,

dot11CFPPeriod,

dot11CFPMaxDuration,

dot11AuthenticationResponseTimeOut,

dot11PrivacyOptionImplemented,

dot11PowerManagementMode,

dot11DesiredSSID, dot11DesiredBSSType,

dot11OperationalRateSet,

dot11BeaconPeriod, dot11DTIMPeriod,

dot11AssociationResponseTimeOut,

dot11DisassociateReason,

dot11DisassociateStation,

dot11DeauthenticateReason,

dot11DeauthenticateStation,

dot11AuthenticateFailStatus,

dot11AuthenticateFailStation,

dot11MultiDomainCapabilityImplemented,

dot11MultiDomainCapabilityEnabled,

dot11CountryString }

STATUS current

DESCRIPTION

"The SMTbase2 object class provides the necessary support at the

STA to manage the processes in the STA such that the STA may

work cooperatively as a part of an IEEE 802.11 network."

::= { dot11Groups 21 }

-- Modified by Daniel R. Borges on 02.14.2002 (802.11a)

dot11MultiDomainCapabilityGroup OBJECT-GROUP

OBJECTS {	dot11FirstChannelNumber,

dot11NumberofChannels,

dot11MaximumTransmitPowerLevel }

STATUS current

DESCRIPTION

""

::= { dot11Groups 22 }

-- Modified by Daniel R. Borges on 02.14.2002 (802.11a)

dot11PhyFHSSComplianceGroup2 OBJECT-GROUP

OBJECTS {	dot11HopTime, dot11CurrentChannelNumber, dot11MaxDwellTime,

dot11CurrentDwellTime, dot11CurrentSet, dot11CurrentPattern,

dot11CurrentIndex, dot11EHCCPrimeRadix,

dot11EHCCNumberofChannelsFamilyIndex,

dot11EHCCCapabilityImplemented, dot11EHCCCapabilityEnabled,

dot11HopAlgorithmAdopted, dot11RandomTableFlag,

dot11NumberofHoppingSets, dot11HopModulus,

dot11HopOffset, dot11RandomTableFieldNumber }

STATUS current

DESCRIPTION

"Attributes that configure the Frequency Hopping for IEEE

802.11."

::= { dot11Groups 23 }

-- Modified by Daniel R. Borges on 02.14.2002 (802.11a)

-- >> J Kim: TGk addition begins

-- TGk conformance group

-- >> J Kim: TGk addition ends

-- **********************************************************************

-- *   End of 802.11 MIB

-- **********************************************************************

END