IEEE P802.11
Wireless LANs

Clause 8.5.1 Pairwise Key Subfield Explanation

Date:	March 13, 2003

Author:	Clint Chaplin
	Symbol Technologies, Inc.
	6480 Via Del Oro, San Jose California 95119-1208
	Phone: +1-408-528-2766
	e-Mail: cchaplin@sj.symbol.com

Abstract

This submission proposes changes to the Pairwise Key Subfield explanation in Clause 8.5.1.

# Motivation

The current text about the “Pairwise Key Subfield” in the draft is quite possibly the most confusing and least understandable part of the current IEEE 802.11i Draft 3.0.  This text has caused the most number of clarification followup qusetions from engineers attempting to implement the draft (at least three attempts from one group of engineers alone).

The proposed text attempts to clarify the conditions under which this bit should be used, and the consequences of using this bit.  This proposed text is a result of a clarification dialog between the original author of the clause and a team of engineers attempting to implement the clause.

# Proposed Text

Add the following text to the end of 8.5.1 “Key hierarchy”:

Informative Note: A STA shall support a single pairwise key for any TA/RA pair. The TA/RA identifies the pairwise key, which does not correspond to any WEP key id. WEP key ID 0 shall only be used in a TSN and shall only be used only when the “Pairwise key subfield” capability bit is 1. Instead, a group key is identified by WEP key id 1 or 2 and the TA/RA pair.

In a TSN, an AP may be talking to STAs that are using preshared WEP keys and to TKIP STAs at the same time.  The preshared WEP STAs will have preshared WEP keys on key indexes 0 through 3; the important point here is that there may be a preshared WEP key on key index 0.  The AP can be configured to use the WEP key in key index 0 for WEP transmissions; if the AP is configured in this way, there will be problems with TKIP STAs that do not implement key-mapping keys (note that if the AP is configured to use the WEP key in key indexes 1 through 3 for WEP transmissions, the problems will not exist).

TKIP STAs in a TSN are expected to support a single Pairwise key either with a key mapping key or by mapping to default key 0 - depending on whether the implementation supports key mapping keys or not. The key lookup algorithm for unicast messages uses the standard key lookup: if a key mapping key is available the ra/ta finds the key, if there is no key mapping key then the default key 0 is used because the key index in the message will be 0.

As TKIP STAs associate and authenticate with the AP, the AP will want to give the TKIP STA a pairwise key to be used for unicast traffic between the AP and the STA, and this pairwise key has to be kept on the STA as a key-mapping key.  Some TKIP STAs are not capable of handling key-mapping keys.  For these STAs, the STA advertises its lack of this capability by setting the “Pairwise Key Subfield” flag in the element sent in the association request to the AP.  If the AP wants to use key index 0 for WEP transmissions, the AP will, in response to the STA having the “Pairwise Key Subfield” flag set, clear the install bit in the third message of the four-way handshake with this STA to notify the STA not to install the pairwise key.  The AP will also send the preshared key index 0 WEP key to this STA to be plumbed as the key index 0 as a Group key IEEE 802.1X message; this key will then be used with WEP encryption to send and receive unicast traffic between the AP and the STA.

The TKIP STA that has this limitation may not know that it will be forced to use WEP for all transmissions until it has associated with the AP and been given the keys to use (the STA cannot know that the AP has been configured to use WEP key index 0 for WEP communication).  If the STA thinks that this situation is too insecure, the only recourse the STA has is to disassociate and try to associate with a different AP.

Because unicast traffic with this STA uses the preshared key index 0 WEP key, the security limitations are great.  The TKIP STA will actually be using WEP for all communication.  All other STAs that have this preshared WEP key can listen in on the unicast traffic (which is a limitation that exists with legacy preshared WEP systems).  In addition, the extra traffic using the preshared WEP key makes the breaking of the WEP key easier.