IEEE P802.11
Input to IETF EAP Working Group

Date:	March 12th, 2003

Author:	Dorothy Stanley
	Agere Systems
	2000 North Naperville Rd, Naperville, IL 60566
	Phone: 1-630-979-1572
	Fax: 
	e-Mail: dstanley@agere.com

Abstract

The IETF EAP working group has requested additional input on EAP method/credentials, and key strength requirements. This document contains the 802.11i response to the WG request.

Motion: Move to authorize Stuart Kerry, Chair of 802.11 to send the following letter to Harald Alvestrand, Chairman IETF, IESG.

Harald Alvestrand, Chairman

IETF, IESG

Dear Harald,

The purpose of the IEEE 802.11i task group is to enhance the security of IEEE 802.11 systems. The purpose of this letter is to provide the EAP WG with additional input on (a) the EAP methods and credentials that are important to 802.11 wireless LAN deployments, and (b) 802.11i EAP Key Strength requirements.

###### EAP Methods and Credentials

Deployments of 802.11 WLANs today use several EAP methods, including EAP-TLS, EAP-TTLS and EAP-PEAP and EAP-SIM. These methods support authentication credentials that include digital certificates, usernames and passwords, secure tokens, and SIM secrets.

The 802.11i requirement is that one or more published, reviewed EAP methods are available which

- Support the following credentials: digital certificates, user-names and passwords, existing secure tokens, and mobile network credentials (GSM and UMTS secrets).

- Generate keying material

- Support Mutual Authentication

- Are resistant to dictionary attacks

- Support Identity hiding?

- Fast Resume – Mentioned in the original letter?

- Support for public/private key (without necessarily requiring certs)?

- Asymmetric support (password on one side, public/private key on the other)?

The current mandatory-to-implement EAP method is EAP-MD5. EAP-MD5 does not meet 802.11’s requirements. We request that the mandatory to implement EAP method be changed to one of the methods that IEEE 802.11 is able to use.

###### Key Strength Requirements

802.11i RSN networks use 802.1X and EAP methods to implement end user authentication, and require that these EAP methods provide keying material. The 802.11i requirement is that

- The EAP method must be capable of generating keying material with 128-bits of effective key strength. Key material must be at least 256 bits in length.

We thank the IETF and the EAP WG for its ongoing work supporting the specification of EAP methods, EAP keying, and RADIUS keying attributes.

Please contact Dorothy Stanley, IEEE-IETF Liaison dstanley@agere.com and David Halasz, Chair of 802.11i dhala@cisco.com with any questions, and to discuss IETF follow-up.

Stuart Kerry