IEEE P802.11
Wireless LANs

Proposed Normative Text for Transmission of Traffic Streams using Contention Based Channel Access

Date:	April 30, 2003

Authors:

Matthew Sherman
	AT&T Labs – Research

180 Park Avenue, Florham Park, NJ  07932
	(973) 236-6925
	mjsherman@att.com

Mathilde Benveniste
	Avaya Labs - Research
	233 Mt. Airy Road, Basking Ridge, NJ USA
	Phone: 973-761-6105
	e-Mail: benveniste@ieee.org

Keith Amann

SpectraLink Corporation

5755 Central Ave., Boulder, CO USA

Phone: 303-440-5330

e-Mail: kamann@spectralink.com

Abstract

This document contains the material proposed to AT&T and Avaya for inclusion in the draft in the form of insertions into and replacements for material in TGe Draft 4.0.

Editorial notes appear in bold italic Times New Roman font, informative notes appear in normal Arial font, and normative text appears in normal Times New Roman font.  Open issues are highlighted using red text in normal Arial font, and begin with "OPEN ISSUE:".  Added text appears in blue with a solid underline.  Deleted text appears in blue with a strikethrough line.

Modify text in section 7.3.2.15 as follows:

#### 7.3.2.15 Traffic Specification (TSPEC) element

The Traffic Specification (TSPEC) element contains the set of parameters that define the characteristics and QoS expectations of a unidirectional traffic stream, in the context of a particular non-AP QSTA, for use by the HC and non-AP QSTA(s) in support of parameterized QoS traffic transfer using the procedures defined in 11.5.  The element information field comprises the items as defined below and the structure is defined in Figure 42.7.

| Octets: 1 | 1 | 2 | 2 | 2 | 4 | 4 | 4 | 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Element ID (13) | Length (44) | TS Info | Nominal MSDU Size | Maximum MSDU Size | Minimum Service Interval | Maximum Service Interval | Inactivity Interval | Minimum Data Rate |

| 4 | 4 | 4 | 4 | 4 | 2 |
| --- | --- | --- | --- | --- | --- |
| Mean Data Rate | Maximum Burst Size | Minimum PHY Rate | Peak Data Rate | Delay Bound | Surplus Bandwidth Allowance |

Figure 42.7 – Traffic Specification element format

The Traffic Specification allows a set of parameters more extensive than may be needed, or may be available, for any particular instance of parameterized QoS traffic.  The fields are set to zero for any unspecified parameter values.

The structure of the TS Info field is defined in Figure 42.8.

| B0 | B1 | B2 | B3 | B4 | B5 |  | B7 | B8 | B9 | B10 |  | B12 | B13 |  | B15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Traffic Type | Access Policy | TSInfo Ack Policy |  | Reserved | Priority |  |  | Direction |  | TSID |  |  | Reserved |  |  |
| Bits: 1 | 1 | 2 |  | 1 | 3 |  |  | 2 |  | 3 |  |  | 3 |  |  |

Figure 42.8 – TS Info field

The Traffic Type subfield is a single bit which is set to 1 for a continuous or periodic traffic pattern (e.g. isochronous traffic stream  of MSDUs, with constant or variable sizes, that are originated at fixed rate), or is set to 0 for a non-continuous, aperiodic, or unspecified traffic pattern (e.g. asynchronous traffic stream of low-duty cycles).

The access policy subfield is a single bit that shall be set to 1 by a non-AP QSTA to request permission to transmit traffic associated with the requested stream over EDCF as well as using polled access under certain conditions, as specified in Clause 9.10.2.3. This subfield is set to 1 in the ADDTS response QoS action frame by the QAP to grant permission to the non-AP QSTA to transmit traffic associated with the granted stream over EDCF as well as polled access under such conditions.  It is set to 0 by non-AP QSTA and QAP to indicate that the traffic associated with the requested or granted stream shall not be transmitted over EDCF.

Update the last paragraph of section number 9.10.2.3 as follows:

HCF contention-based channel access shall not be used to transmit MSDUs belonging to traffic streams  unless the granted TSPEC indicates it is permitted to do so by the Access Policy bit of the TS Info field, and one of the following two conditions holds:

- The MSDU has been sent previously but an acknowledgement has not been received

- The poll for the MSDU is missing; i.e., a poll has not been received by the non-AP QSTA within a time interval since the time of the last -received poll that is equal to the maximum service interval.

When frames associated with a TSPEC are transmitted over contention-based channel access, they shall use the AC associated with the UP specified in the TSPEC.

Add the following text to the end of section 11.4.1:

The ability to transmit traffic for admitted TSPECs over the EDCF is controlled by the Access Policy bit of the TS Info field.  If the non-AP QSTA associated with the TSPEC requests it, and the HC grants it, then it is permitted for frames associated with that stream to be transmitted over the EDCF, within the constraints specified in Clause 9.10.2.3.