IEEE P802.11
Wireless LANs

Proposed Text for PSNI Measurement

Date:	May13, 2003

Author:	Joe Kwak
	InterDigital Communications
	482 Degas
 	Bolingbrook, IL 60440
	Phone: 630-739-4327
	e-mail: joekwak@mindspring.com

Abstract

This document proposes text for measurement used for Perceived Signal-to-Noise-plus-Interference indicator (PSNI). This is modelled on existing specification text for RSSI.

# 4. Abbreviations and acronyms

Insert the following new definitions in alphabetical order, renumbering as necessary:

AWGN	 Additive white gaussian noise

12.3.4.3 PHY-SAP service primitives parameters

Change Table 29 as follows:

Table 29—PHY-SAP service primitive parameters

| Parameter | Associated primitive | Value |
| --- | --- | --- |
| DATA | PHY-DATA.request PHY-DATA.indication | Octet value X'00'–X'FF' |
| TXVECTOR | PHY-TXSTART.request | A set of parameters |
| STATUS | PHY-CCA.indication | BUSY, IDLE |
| RXVECTOR | PHY-RXSTART.indication PHY-RXEND.indication | A set of parameters |
| RXERROR | PHY-RXEND.indication | NoError, FormatViolation, Carrier- Lost, UnsupportedRate |

12.3.5.11 PHY-RXSTART.indication

Change 12.3.5.11.2 as follows

12.3.5.11.1 Function

This primitive is an indication by the PHY to the local MAC entity that the PLCP has received a valid start

frame delimiter (SFD) and PLCP Header.

12.3.5.11.2 Semantics of the service primitive

The primitive provides the following parameter:

PHY-RXSTART.indication (RXVECTOR)

The RXVECTOR represents a list of parameters that the PHY provides the local MAC entity upon receipt of

a valid PLCP Header or upon receipt of the last PSDU data bit in the received frame. This vector may contain both MAC and MAC management parameters. The required parameters are listed in 12.3.4.4.

12.3.5.11.3 When generated

This primitive is generated by the local PHY entity to the MAC sublayer when the PHY has successfully

validated the PLCP header error check (HEC) CRC at the start of a new PPDU.

12.3.5.11.4 Effect of receipt

The effect of receipt of this primitive by the MAC is unspecified.

12.3.5.12 PHY-RXEND.indication

Change 12.3.5.12.2  to add new parameter as follows

12.3.5.12.1 Function

This primitive is an indication by the PHY to the local MAC entity that the MPDU currently being received

is complete.

12.3.5.12.2 Semantics of the service primitive

The primitive provides the following parameters:

PHY-RXEND.indication (RXERROR, RXVECTOR)

The RXERROR parameter can convey one or more of the following values: NoError, FormatViolation, CarrierLost,

or UnsupportedRate. A number of error conditions may occur after the PLCP’s receive state

machine has detected what appears to be a valid preamble and SFD. The following describes the parameter

returned for each of those error conditions.

— NoError. This value is used to indicate that no error occurred during the receive process in the PLCP.

— FormatViolation. This value is used to indicate that the format of the received PPDU was in error.

— CarrierLost. This value is used to indicate that during the reception of the incoming MPDU, the

carrier was lost and no further processing of the MPDU can be accomplished.

— UnsupportedRate. This value is used to indicate that during the reception of the incoming PPDU, a

nonsupported date rate was detected.

The RXVECTOR represents a list of parameters that the PHY provides the local MAC entity upon receipt of

a valid PLCP Header or upon receipt of the last PSDU data bit in the received frame. This vector may contain both MAC and MAC management parameters. The required

parameters are listed in 12.3.4.4.

12.3.5.12.3 When generated

This primitive is generated by the PHY for the local MAC entity to indicate that the receive state machine

has completed a reception with or without errors.

12.3.5.12.4 Effect of receipt

The effect of receipt of this primitive by the MAC is unspecified.

##### 15.2.7 Receive PLCP

Change the second and fifth paragraph as follows:

The receive PLCP is shown in Figure 97.

In order to receive data, PHY-TXSTART.request shall be disabled so that the PHY entity is in the receive

state. Further, through station management via the PLME, the PHY is set to the appropriate channel and the

CCA method is chosen. Other receive parameters such as RSSI, PSNI, signal quality (SQ), and indicated

DATARATE may be accessed via the PHY-SAP.

Upon receiving the transmitted energy, according to the selected CCA mode, the PMD_ED shall be enabled

(according to 15.4.8.4) as the RSSI reaches the ED_THRESHOLD and/or PMD_CS shall be enabled after

code lock is established. These conditions are used to indicate activity to the MAC via PHY-CCA.indicate

according to 15.4.8.4. PHY-CCA.indicate(BUSY) shall be issued for energy detection (ED) and/or code

lock prior to correct reception of the PLCP frame. The PMD primitives PMD_SQ and PMD_RSSI are issued

to update the RSSI and SQ parameters reported to the MAC.

After PHY-CCA.indicate is issued, the PHY entity shall begin searching for the SFD field. Once the SFD

field is detected, CRC-16 processing shall be initiated and the PLCP IEEE 802.11® SIGNAL, IEEE 802.11®

SERVICE and LENGTH fields are received. The CRC-16 FCS shall be processed. If the CRC-16 FCS

check fails, the PHY receiver shall return to the RX IDLE state as depicted in Figure 98. Should the status of

CCA return to the IDLE state during reception prior to completion of the full PLCP processing, the PHY

receiver shall return to the RX IDLE state.

If the PLCP Header reception is successful (and the SIGNAL field is completely recognizable and

supported), a PHY-RXSTART.indicate(RXVECTOR) shall be issued. The RXVECTOR associated with

this primitive includes the SIGNAL field, the SERVICE field, the MPDU length in bytes (calculated from

the LENGTH field in microseconds), the antenna used for receive (RX_ANTENNA), RSSI, PSNI, and SQ.

Change Figure 97 to add newparameter as follows:

Figure 97—Receive PLCP

##### 15.4.4.2 PMD_SAP peer-to-peer service primitive parameters

Change Table 66 to add new row as follows:

Table 66—DSSS PMD_SAP peer-to-peer service primitives

| Parameter | Associated primitive | Value |
| --- | --- | --- |
| LENGTH | RXVECTOR, TXVECTOR | 0 to 2E13-1 |
| DATARATE | RXVECTOR, TXVECTOR | 1,2 Mbit/s |
| SERVICE | RXVECTOR, TXVECTOR | 1,2 Mbit/s |
| TXPWR_LEVEL | TXVECTOR | 1,2 Mbit/s |
| TX_ANTENNA | TXVECTOR | 1,2 Mbit/s |
| RSSI | RXVECTOR | 1,2 Mbit/s |
| PSNI | RXVECTOR | 1,2 Mbit/s |
| SQ | RXVECTOR | 1,2 Mbit/s |
| RX_ANTENNA | RXVECTOR | 1,2 Mbit/s |

##### 15.4.4.3 PMD_SAP sublayer-to-sublayer service primitives

Change Table 67 to add new row as follows:

Table 67—PMD_SAP sublayer-to-sublayer service primitives

| Primitive |  | Request | Indicate | Confirm | Response |
| --- | --- | --- | --- | --- | --- |
| PMD_TXSTART | X |  |  | -- | -- |
| PMD_TXEND | X |  | X | -- | -- |
| PMD_ANTSEL | X |  |  | -- | -- |
| PMD_TXPWRLEVL | X |  |  | -- | -- |
| PMD_RATE | X |  | X | -- | -- |
| PMD_RSSI |  |  | X | -- | -- |
| PMD_PSNI |  |  | X | -- | -- |
| PMD_SQ |  |  | X | -- | -- |
| PMD_CS |  |  | X | -- | -- |
| PMD_ED | X |  | X | -- | -- |

##### 15.4.4.4 PMD_SAP service primitive parameters

Change Table 68 to add new row as follows

:

Table 68—List of parameters for the PMD primitives

| Parameter | Associated primitive | Value |
| --- | --- | --- |
| DATA | PHY-DATA.request PHY-DATA.indicate | Octet value: X’00’-X’FF’ |
| TXVECTOR | PHY-DATA.request | A set of parameters |
| RXVECTOR | PHY-DATA.indicate | A set of parameters |
| TX_UNIT | PMD-DATA.request | 1,0: DBPSK dibit combinations 00,01,11,10: DQPSK |
| RX_UNIT | PMD-DATA.indicate | 1,0: DBPSK dibit combinations 00,01,11,10: DQPSK |
| RF_STATE | PMD_TXE.request | Receive, Transmit |
| ANT_STATE | PMD-ANTSEL.iindicate PMD-ANTSEL.request | 1 to 256 |
| TXPWR_LEVEL | PHY-TXSTART | 0,1,2,3 (max of 4 levels) |
| RATE | PMD-RATE.indicate PMD-RATE.request | X’0A’ for 1 Mbit/s DBPSK X’14’ for 2 Mbit/s DQPSK |
| RSSI | PMD-RSSI.indicate | 0-8 bits of RSSI |
| PSNI | PMD-PSNI.indicate | 8 bits of PSNI |
| SQ | PMD-SQ.indicate | 0-8 bits of SQ |

Insert the following new section after Section 15.4.5.15:

15.4.5.X PMD_PSNI.indicate

15.4.5.X.1 Function

This optional primitive, which is generated by the PMD sublayer, provides to the PLCP and MAC entity the

perceived signal-to-noise-plus-interference ratio.

15.4.5.X.2 Semantics of the service primitive

The primitive shall provide the following parameter:

PMD_PSNI.indicate(PSNI).

The PSNI shall be a measure of the signal-to-noise-plus-interference ratio perceived by the DSSS PHY. PSNI indications of 8 bits (256 levels) are supported. as defined in 15.4.8.X

15.4.5.X.3 When generated

This primitive shall be generated by the PMD when the DSSS PHY is in the receive state. It shall be continuously

available to the PLCP, which, in turn, provides the parameter to the MAC entity.

15.4.5.X.4 Effect of receipt

This parameter shall be provided to the PLCP layer for information only. The PSNI may be used in conjunction

with RCPI to measure input signal quality.

Insert the following new section after Section 15.4.8.4:

15.4.8.X Perceived Signal-to-noise-plus-interference Ratio (PSNI) Measurement

The PSNI indicator is a measure of the perceived, post-processing signal-to-noise-plus-interference (S/(N+I)) ratio in the demodulator.  The allowed values for the Perceived Signal to Noise Indicator (PSNI) parameter shall be an 8 bit value in the range from 0 through 255. This parameter shall be a measure by the PHY sublayer of the perceived signal quality observed after RF downconversion and is derived from internal digital signal processing metrics of the demodulator used to receive the current frame. PSNI shall be measured over the PLCP preamble and over the entire received frame.  PSNI is intended to be used in a relative manner, and it shall be a monotonically increasing, logarithmic function of the observed S/(N+I).  PSNI accuracy and range shall be specified in AWGN at given FERs for each data rate as indicated in Table XX.

Table XX—PSNI Frame Measurement Performance Specification

| Data Rate (Mbit/s) | PPDU Length (bytes) | FER | PSNI |
| --- | --- | --- | --- |
| 1 | 125 | 10% +/-0.2% | 32 +/- 8 |
| 2 | 1250 | 10% +/-0.2% | 66 +/- 8 |

When PSNI exceeds the high end of  measurable range for a given data rate, the maximum PSNI meaureable value for that rate shall be reported.  The PSNI value of 0 shall indicate an inability to measure PSNI in this frame

17.2.3 RXVECTOR parameters

Change Table 83 to add new row as follows

Table 83—RXVECTOR parameters

| Parameter | Associated primitive | Value |
| --- | --- | --- |
| LENGTH | PHY-RXSTART.indicate | 0 to 4095 |
| RSSI | PHY-RXSTART.indicate (RXVECTOR) | 0 to RSSI maximum |
| PSNI | PHY-RXSTART.indicate (RXVECTOR) PHY-RXEND.indicate (RXVECTOR) | 0 to 255 |
| DATARATE | PHY-RXSTART.request (RXVECTOR) | 6, 9, 12, 18, 24, 36, 48, 54 |
| SERVICE | PHY-RXSTART.request (RXVECTOR) | Null |

Insert the following new section after Section 17.2.3.4:

17.2.3.X RXVECTOR PSNI

The allowed values for the PSNI parameter are in the range from 0 through 256. This parameter is a measure of the signal-to-noise-plus-interference ratio perceived by the PHY. PSNI indications of 8 bits (256 levels) are supported. as defined in 17.3.10.X.  PSNI shall be measured during the reception of the PLCP preamble.

Insert the following new section after Section 17.3.10.5:

17.3.10.X Perceived Signal-to-noise-plus-interference Ratio (PSNI) Measurement

The PSNI indicator is a measure of the perceived, post-processing signal-to-noise-plus-interference (S/(N+I)) ratio in the demodulator.  The allowed values for the Perceived Signal to Noise Indicator (PSNI) parameter shall be an 8 bit value in the range from 0 through 255. This parameter shall be a measure by the PHY sublayer of the perceived signal quality observed after RF downconversion and is derived from internal digital signal processing metrics of the demodulator used to receive the current frame. PSNI shall be measured over the PLCP preamble and over the entire received frame.  PSNI is intended to be used in a relative manner, and it shall be a monotonically increasing, logarithmic function of the observed S/(N+I).  PSNI accuracy and range shall be specified in AWGN at given FERs for each data rate as indicated in Table XX.

Table XX—PSNI Frame Measurement Performance Specification

| Data Rate (Mbit/s) | FEC Rate | PPDU Length (bytes) | FER | PSNI |
| --- | --- | --- | --- | --- |
| 6 | 1/2 | 1250 | 10% +/-0.2% | 61 +/- 8 |
| 9 | 3/4 | 1250 | 10% +/-0.2% | 83 +/- 8 |
| 12 | 1/2 | 1250 | 10% +/-0.2% | 85 +/- 8 |
| 18 | 3/4 | 1250 | 10% +/-0.2% | 107 +/- 8 |
| 24 | 1/2 | 1250 | 10% +/-0.2% | 141 +/- 8 |
| 36 | 3/4 | 1250 | 10% +/-0.2% | 163 +/- 8 |
| 48 | 2/3 | 1250 | 10% +/-0.2% | 206 +/- 8 |
| 54 | 3/4 | 1250 | 10% +/-0.2% | 213 +/- 8 |

When PSNI exceeds the high end of  measurable range for a given data rate, the maximum PSNI meaureable value for that rate shall be reported.  The PSNI value of 0 shall indicate an inability to measure PSNI in this frame

17.3.12 Receive PLCP

Change the first paragraph as follows:

The receive PLCP is shown in Figure 128. In order to receive data, PHY-TXSTART.request shall be disabled

so that the PHY entity is in the receive state. Further, through station management (via the PLME) the

PHY is set to the appropriate frequency. Other receive parameters, such as RSSI, PSNI, and indicated DATARATE,

may be accessed via the PHY-SAP.

Change Figure 128 to add newparameter as follows:

Figure 128—Receive PLCP

##### 17.5.4.2 PMD_SAP sublayer-to-sublayer service primitives

Change Table 101 to add new row as follows:

Table 101—PMD_SAP sublayer-to-sublayer service primitives

| Primitive |  | Request | Indicate | Confirm | Response |
| --- | --- | --- | --- | --- | --- |
| PMD_TXSTART | X |  | -- | -- | -- |
| PMD_TXEND | X |  | -- | -- | -- |
| PMD_TXPWRLEVL | X |  | -- | -- | -- |
| PMD_RATE | X |  | -- | -- | -- |
| PMD_RSSI | -- |  | X | -- | -- |
| PMD_PSNI | -- |  | X | -- | -- |

##### 17.5.4.3 PMD_SAP service primitive parameters

Change Table 102 to add new row as follows

:

Table 102—List of parameters for the PMD primitives

| Parameter | Associated primitive | Value |
| --- | --- | --- |
| TXD_UNIT | PMD-DATA.request | One(1), Zero(0): one OFDM symbol value |
| RXD_UNIT | PMD-DATA.indicate | One(1), Zero(0): one OFDM symbol value |
| TXPWR_LEVEL | PHY-TXSTART | 1-8 (max of 8 levels) |
| RATE | PMD-RATE.request | 12 Mbit/s (for BPSK) 24 Mbit/s (for QPSK) 48 Mbit/s (for 16-QAM) 72 Mbit/s (for 64-QAM) |
| RSSI | PMD-RSSI.indicate | 0-8 bits of RSSI |
| PSNI | PMD-PSNI.indicate | 8 bits of PSNI |

Insert the following new section after Section 17.5.5.7:

17.5.5.X PMD_PSNI.indicate

17.5.5.X.1 Function

This primitive, generated by the PMD sublayer, provides the perceived signal-to-noise-plus-interference ratio to the PLCP and MAC entity.

17.5.5.X.2 Semantics of the service primitive

The primitive shall provide the following parameter:

PMD_PSNI.indicate(PSNI).

The PSNI shall be a measure of the signal-to-noise-plus-interference ratio perceived by the DSSS PHY. PSNI indications of 8 bits (256 levels) are supported. as defined in 17.3.10.X

17.5.5.X.3 When generated

This primitive shall be generated by the PMD when the DSSS PHY is in the receive state. It shall be continuously

available to the PLCP, which, in turn, provides the parameter to the MAC entity.

17.5.5.X.4 Effect of receipt

This parameter shall be provided to the PLCP layer for information only. The PSNI may be used in conjunction

with RCPI to measure input signal quality.

18.2.6 Receive PLCP

Change the third, fourth, and sixth paragraphs as follows:

The receive procedures for receivers configured to receive the mandatory and optional PLCPs, rates, and

modulations are described in this subclause. A receiver that supports this High Rate extension of the

standard is capable of receiving 5.5 Mbit/s and 11 Mbit/s, in addition to 1 Mbit/s and 2 Mbit/s. If the PHY

implements the Short Preamble option, it shall detect both short and Long Preamble formats and indicate

which type of preamble was received in the RXVECTOR. If the PHY implements the PBCC Modulation

option, it shall detect either CCK or PBCC Modulations, as indicated in the SIGNAL field, and shall report

the type of modulation used in the RXVECTOR.

The receiver shall implement the CCA procedure as defined in 18.4.8.4. Upon receiving a PPDU, the

receiver shall distinguish between a long and short header format by the value of the SFD, as specified in

18.2.2. The receiver shall demodulate a long PLCP header using BPSK at 1 Mbit/s. The receiver shall

demodulate a short PLCP header using QPSK at 2 Mbit/s. The receiver shall use the SIGNAL and SERVICE

fields of the PLCP header to determine the data rate and modulation of the PSDU.

The receive PLCP is shown in Figure 139 In order to receive data, the PHY-TXSTART.request shall be disabled

so that the PHY entity is in the receive state. Further, through station management via the PLME, the

PHY shall be set to the appropriate channel and the CCA method chosen. Other receive parameters, such as

RSSI, PSNI, SQ, and indicated DATARATE, may be accessed via the PHY-SAP.

Upon receiving the transmitted energy, according to the selected CCA mode, the PMD_ED shall be enabled

(according to 18.4.8.4) as the RSSI reaches the ED_THRESHOLD, and/or PMD_CS shall be enabled after

code lock is established. These conditions are used to indicate activity to the MAC via PHY-CCA.indicate,

according to 18.4.8.4. PHY-CCA.indicate(BUSY) shall be issued for ED and/or code lock prior to correct

reception of the PLCP header. The PMD primitives, PMD_SQ, PMD_RSSI, PMD_PSNI, are issued to update the

RSSI, PSNI, and SQ parameters reported to the MAC.

After PHY-CCA.indicate is issued, the PHY entity shall begin searching for the SFD field. Once the SFD

field is detected, CRC-16 processing shall be initiated and the PLCP SIGNAL, SERVICE, and LENGTH

fields shall be received. The CRC-16 FCS shall be processed. If the CRC-16 FCS check fails, the PHY

receiver shall return to the RX IDLE state, as depicted in Figure 140. Should the status of CCA return to the

IDLE state during reception prior to completion of the full PLCP processing, the PHY receiver shall return

to the RX IDLE state.

If the PLCP header reception is successful (and the SIGNAL field is completely recognizable and supported),

a PHY-RXSTART.indicate(RXVECTOR) shall be issued. The RXVECTOR associated with this

primitive includes

a) The SIGNAL field;

b) The SERVICE field;

c) The PSDU length in octets (calculated from the LENGTH field in microseconds and the DATARATE

in Mbit/s, in accordance with the formula in 18.2.3.5);

d) RXPREAMBLE_TYPE (which is an enumerated type taking on values SHORTPREAMBLE or

LONGPREAMBLE);

e) The antenna used for receive (RX_ANTENNA), RSSI, PSNI, and SQ.

Change Figure 139 to add newparameter as follows:

Figure 139—Receive PLCP

18.3.5 Vector descriptions

Change Table 108 to add new rows as follows:

Table 108—Parameter vectors

| Parameter | Associated vector | Value |
| --- | --- | --- |
| DATARATE | RXVECTOR, TXVECTOR | The rate used to transmit the PSDU in Mbit/s. |
| LENGTH | RXVECTOR, TXVECTOR | The length of the PSDU in octets. |
| PREAMBLE_TYPE | RXVECTOR, TXVECTOR | The preamble used for the transmission of this PPDU. This is an enumerated type that can take the value SHORTPREAMBLE or LONGPREAMBLE. |
| MODULATION | RXVECTOR, TXVECTOR | The modulation used for the transmission of this PSDU. This is an integer where 0 means CCK and 1 means PBCC. |
| RX_ANTENNA | RXVECTOR | The antenna used for receive. |
| RSSI | RXVECTOR | 0-8 bits of RSSI |
| PSNI | RXVECTOR | 8 bits of PSNI |
| SQ | RXVECTOR | 0-8 bits of SQ |

##### 18.4.4.2 PMD_SAP sublayer-to-sublayer service primitives

Change Table 110 to add new row as follows:

Table 110—PMD_SAP sublayer-to-sublayer service primitives

| Primitive | Request |  | Indicate | Confirm | Response |
| --- | --- | --- | --- | --- | --- |
| PMD_TXSTART |  | X | -- | -- | -- |
| PMD_TXEND |  | X | -- | -- | -- |
| PMD_PSNI |  | -- | X | -- | -- |
| PMD_TXPWRLVL |  | X | -- | -- | -- |
| PMD_MODULATION |  | X | X | -- | -- |
| PMD_PREAMBLE |  | X | X | -- | -- |
| PMD_RATE |  | X | X | -- | -- |
| PMD_RSSI |  | -- | X | -- | -- |
| PMD_PSNI |  | -- | X | -- | -- |
| PMD_SQ |  |  | X | -- | -- |
| PMD_CS |  |  | X | -- | -- |
| PMD_ED |  | X | X | -- | -- |

Insert the following new section after Section 18.4.5.15:

18.4.5.X PMD_PSNI.indicate

18.4.5.X.1 Function

This optional primitive may be generated by the PMD to provide the perceived signal-to-noise-plus-interference ratio to the PLCP.

18.4.5.X.2 Semantics of the service primitive

The primitive shall provide the following parameter:

| Parameter | Associated primitive | Value | Description |
| --- | --- | --- | --- |
| PSNI | PMD_PSNI.indicate | 8 bits of PSNI | The PSNI is a measure of the signal-to-noise-plus-interference ratio perceived by the High Rate PHY, as defined in 18.4.8.X. |

18.4.5.X.3 When generated

This primitive shall be generated by the PMD when the High Rate PHY is in the receive state. It is continuously

available to the PLCP, which, in turn, provides the parameter to the MAC entity.

18.4.5.X.4 Effect of receipt

This parameter shall be provided to the PLCP layer for information only. The PSNI may be used in conjunction

with RCPI to measure input signal quality.

Insert the following new section after Section 18.4.8.4:

18.4.8.X Perceived Signal-to-noise-plus-interference Ratio (PSNI) Measurement

The PSNI indicator is a measure of the perceived, post-processing signal-to-noise-plus-interference (S/(N+I)) ratio in the demodulator.  The allowed values for the Perceived Signal to Noise Indicator (PSNI) parameter shall be an 8 bit value in the range from 0 through 255. This parameter shall be a measure by the PHY sublayer of the perceived signal quality observed after RF downconversion and is derived from internal digital signal processing metrics of the demodulator used to receive the current frame. PSNI shall be measured over the PLCP preamble and over the entire received frame.  PSNI is intended to be used in a relative manner, and it shall be a monotonically increasing, logarithmic function of the observed S/(N+I).  PSNI accuracy and range shall be specified in AWGN at given FERs for each data rate as indicated in Table XX.

Table XX—PSNI Frame Measurement Performance Specification

| Data Rate (Mbit/s) | FEC Rate | PPDU Length (bytes) | FER | PSNI |
| --- | --- | --- | --- | --- |
| 5.5 | No FEC | 1250 | 10% +/-0.2% | 101 +/- 8 |
| 5.5 | 1/2 | 1250 | 10% +/-0.2% | 58 +/- 8 |
| 11 | No FEC | 1250 | 10% +/-0.2% | 125 +/- 8 |
| 11 | 1/2 | 1250 | 10% +/-0.2% | 82 +/- 8 |

When PSNI exceeds the high end of  measurable range for a given data rate, the maximum PSNI meaureable value for that rate shall be reported.  The PSNI value of 0 shall indicate an inability to measure PSNI in this frame

A.4.6 Direct sequence PHY functions

Change Table to add new row after DS29.1 as follows:

| Item | Feature | References | Status | Support |
| --- | --- | --- | --- | --- |
| DS28 | Receiver adjacent channel rejection | 15.4.8.3 | M | Yes  No  |
| DS29 DS29.1 | MIB dot11PhyDSSSComplianceGroup, dot11PhyRegDomainsSupportGroup, and dot11PhyOperationComplianceGroup | MIB 13.1, 15.3.2, Annex D 13.1, 15.3.2 | M | Yes  No  Yes  No  |
| DSXX | PSNI measurement | 15.4.8.X | M | Yes  No  |

A.4.8 OFDM PHY functions

Change Table to add new row after OF1.6  as follows:

| Item | Feature | References | Status | Support |
| --- | --- | --- | --- | --- |
| OF1.6 | RXVECTOR parameter: RSSI | 17.2.3.2 | M | Yes  No  |
| OF1.X | RXVECTOR parameter:PSNI | 17.2.3.X | M | Yes  No  |

Change Table to add new row after OF5.5  as follows:

| Item | Feature | References | Status | Support |
| --- | --- | --- | --- | --- |
| OF5.5 | CCA sensitivity | 17.3.10.5 | M | Yes  No  |
| OF5.X | PSNI measurement | 17.3.10.X | M | Yes  No  |

Change Table to add new row after OF9.3.8  as follows:

| Item | Feature | References | Status | Support |
| --- | --- | --- | --- | --- |
| OF9.3.8 | Parameter: RSSI | 17.5.4.3 | M | Yes  No  |
| OF9.3.X | Parameter: PSNI | 17.5.4.3 | M | Yes  No  |

A. 4.9 High Rate, direct sequence PHY functions

Change Table to add new row after HRDS32.1  as follows:

| Item | Feature | References | Status | Support |
| --- | --- | --- | --- | --- |
| HRDS32.1 | PHY object class | 13.1, 18.3.3 | M | Yes  No  |
| HRDSXX | PSNI measurement | 18.4.8.X | M | Yes  No  |