IEEE P802.11
Wireless LANs

Proposed Text to Address Comments 57 and 626

Date:	12 May 2003

Author:	Jesse Walker
	Intel Corporation
	2211 N.E. 25th Avenue

Hillsboro, OR  97124
Phone: +1-503 712 1849
e-Mail: jesse.walker@intel.com

Nancy Cam-Winget

Cisco Systems, Inc.

3625 Cisco Way

San Jose, Ca. 95134
Phone: 408-853-0532
e-mail: ncamwing@cisco.com

Thomas Maufer

nVIDIA

2701 San Tomas Expressway

Santa Clara, CA  95050

Phone: +1-408-486-2637

e-Mail: tmaufer@nvidia.com

Bob O’Hara

Airespace, Inc.

110 Nortech Parkway

San Jose, CA  95134

Phone +1-408-635-2025

bob@airspace.com

Fred Stivers

Texas Instruments Incorporated

3000 Aerial Center Parkway, Suite 160

Morrisville, North Carolina 27560

Phone: +1-919-463-1054

fstiver@ti.com

Sandy Turner

Los Almos National Laboratory

Los Alamos, NM  87545

Phone: +1-505-665-6820

slt@lanl.gov

Abstract

This sumission proposes new text of Clause 8.4.1 from 802.11i D3.2.

# Suggested Changes to address comment 57 and 626

Motion: Instruct the editor to replace list item 3 of the first list in Clause 8.4.1 with the following:

The AP’s Authenticator shall initiate IEEE 802.1X authentication. The authentication will be mutual, as the STA needs assurance that the AP belongs to the authorized network and is not a rogue.

And, add the following informative note after list item 3 of the first list in Clause 8.4.1:

Informative note:  Prior to the completion of IEEE 802.1X authentication and the installation of keys, the IEEE 802.1X controlled port in the AP will filter all non-IEEE 802.1X data frames.  Upon completion of a MLME-DELETEKEYS.request, the IEEE 802.1X controlled port returns to the unauthenticated state and filters all non-IEEE 802.1X data frames.