IEEE P802.11
Wireless LANs

Usage Models

Date:	July 24, 2003

Authors/Contributors:

| Name | Company | Address | Phone | Fax | Email |
| --- | --- | --- | --- | --- | --- |
| Tomer Bentzion | Metalink | Yakum Business Park, 60972 Yakum Israel | +972 9 960 5365 | +972 9 960 5399 | tomerb@metalink.co.il |
| Bjorn Bjerke | Qualcomm | 9 Damonmill Sq., Suite 2A, Concord, MA 01742, USA | +1 781-276-0912 | +1 781-276-0901 | bbjerke@qualcomm.com |
| Mary Cramer | Agere Systems | 1110 American Parkway NE, Allentown, PA 18109-9138, USA | +1 610-712-6112 | +1 610 712 1182 | mecramer@agere.com |
| Javier del Prado | Philips | 345 Scarborough Rd, Briarcliff Manor, NY, 10510, USA | +1 914 945 6000 | +1 914 945 6580 | javier.delprado@philips.com |
| Paul Feinberg | Sony | 1 Sony Drive MD TA1-5 Park Ridge, NJ 07656 | +1 201 930-6316 | +1 201 930-6397 | Paul.Feinberg@am.sony.com |
| Rahul Malik | Panasonic | Blk 1022 Tai Seng Ave. #06-3530 Tai Seng Industrial Estate, Singapore 534415 | +65 6550-5482 | +65 6550-5459 | rahul@psl.com.sg |
| Sanjeev Sharma | Samsung | 75 W. Plumeria Dr. , San Jose, CA, 95134 | +1 408- 544 5978 |  | ssharma@sisa.samsung.com |
| Adrian P. Stephens | Intel Corporation | 15 JJ Thompson Avenue, Cambridge CB3 0FD, United Kingdom | +44 1223 763457 |  | adrian.p.stephens@intel.com |
| Timothy P Wakeley | Hewlett Packard Corporation | 8000 Foothills Blvd, Roseville, CA 95747 | 916-785-1619 |  | Tim.Wakeley@hpcom |

Abstract

This document covers the process of determining usage models for the High Throughput Study Group.  The process includes establishing target use cases, determining the applications and environments used for the cases, narrowing down and grouping the applications and environments, and developing usage models to be targeted by simulations for verifying the performance of various HTSG technical proposals.

# Revision History

| ### Revision | ### Comments | ### Date | ### Commenter |
| --- | --- | --- | --- |
| R0 Draft 0 | This document is in its beginning phases. The initial target is to generate a concept that can be reviewed and commented on. The first draft has definitions of some terms along with an initial stab at a few use cases. This is intended to start discussion and review. | July 8, 2003 | Mary Cramer |
| R0 Draft 1 | APS additions | July 9, 2003 | Adrian Stephens |
| R0 | Merged in comments and changes from the group of authors and made public via the .11 reflector. | July 11, 2003 | Adrian Stephens |
| R1 Draft 1 | Javier's contribution merged in Lalit's Contribution merged in Changes made to implement telecon discussion on 14 July 2003 Application table added Usage Models adjusted to reference applications named in the application table | July 14, 2003 | Adrian Stephens |
| R1 Draft 2 | Paul Feinberg's contribution merged in | July 18, 2003 | Adrian Stephens |
| R2 Draft 3 | Chiu Ngo's comments merged in | 21 July 2003 | Adrian Stephens |
| R2 | Results of Use Case voting added and table of use cases sorted by score Printing application and use cases added (Tim Wakeley) | 22 July 2003 | Adrian Stephens |
| R3 | Updates made at face-to-face session of the special committee, (8:00 am, 24 July 2003). Validation of usage models against use cases partially considered. | 24 July 2003 | Adrian Stephens |

# Introduction

To support the definition of a higher data rate standard within the IEEE (to be published in the 802.11n amendment), this document attempts to define usage models based on various market based use cases.  The usage models are intended to support the definition of simulations that will be performed by various companies on their proposed 802.11n solutions to allow 802.11 TGn to compare the performance of the proposed high throughput solutions.

Note - These usage models that the usage model committee develops here need to satisfy the following constraints (remove note eventually):

C1:	They are relevant to the expected uses of the technology

C2:	They require higher throughput than can be achieved with existing 802.11 technology

C3:	They are capable of being turned into an unambiguous simulation scenario

# Process going forward

Document [3] described an initial process.  This section describes our future process and will be replaced with a description of what (if anything) needs to be completed in the task group.

We (the usage model committee) need to:

- Verify the usage models adequately cover the use cases (started)

- Characterise the application traffic descriptions (started)

- Create simulation scenarios from the usage models (not yet started)

Items 1-2 are in scope of this committee, and will be completed by the September 2003 meeting.

Items 3-4 will be started by this committee, although it is not sure if all the necessary application traffic profiles can be completed by the September 2003 meeting. If not completed, the work will need to continue into the task group subject to authorization by 802.11 TGn, possibly as part of work on metrics and evaluation criteria.

# Definitions

This section defines some of the terms used in this document.

Application – a source or sink of wireless data that relates to a particular type of user activity.

Examples: Streaming video. VOIP.

Environment – The type of place a WLAN system is deployed in.  Initial examples:  home, large office

### Use case – A use case is a description of how an end user uses a system that exercises that system’s deployment of WLAN.  A use case includes an application in a deployment environment with details regarding the user activity and both sides of the link.

### Examples: Watching television remote from the cable or set-top box within the home.  Talking on the telephone remote from one’s desk at work.

Usage Model – A specification of one or more applications and environments from which a simulation scenario can be created once the traffic patterns of the applications are known.  Usage models are created to "cover" use cases.

### Simulation Scenario – A simulation scenario is a description of a use case that supports simulation.  A simulation scenario includes details needed for simulation.  Types of details to be included are descriptions that link the use case to the simulation scenario: environment linked to a channel model, position of the AP (console or ceiling mounted), position of STAs w.r.t. AP, uplink and downlink traffic (# packets, size of packets, interference (number and types of users on the same WLAN channel – adjacent cells, the same cell, number and types of users on alternate channels, BT, baby monitors, GPRS or other systems).  A simulation scenario is created from a Usage Model by characterising the traffic profile of the applications and possibly merging multiple applications together to reduce simulation time.

# References

[1]	11-03-364r0,  Javier del Prado et al.

[2]	802.11-03/489r0, Chiu Ngo

[3]	11-03-327r0-HTSG A_proposed_usage_model_methology_for_the_High_Throughput_Task_Group.ppt, Adrian Stephens, Intel

[4]	11-03-354r6, Usage Model Special Committee Cumulative Minutes, Adrian Stephens, Intel

# Mappings between Application, Environment, channel-model, Use case, Usage Model and Simulation Scenario

Understanding and defining the application, environment, channel-model, use case, usage model and simulation scenario are all necessary to create comparative results from 802.11 TGn proposals.

Channel-models are currently being defined by the 802.11 High Throughput Channel Model Special Committee in document ??.   They currently have 5 channel models.  Each environment will map to a single channel model.

Each use case involves the use of one or more applications and is defined for one or more environments.  It represents a single type of use of a system using the technology.

Each application reflects a source or sink of data.   They will eventually be characterised in terms of a traffic profile that allows a simulation of the application to be created.

Each Usage Model contains a representative mixture of applications and channel models designed to adequately cover the important use cases.  There is a many to many mapping between use cases and usage models (i.e. the same use case may contribute to multiple usage models and the same usage model may include applications from multiple use cases).

There will be a one-to-one mapping between Usage Models and Simulation Scenarios.   The usage model is a marketing-oriented description of a "reasonable mixture" covering the important use cases.  The simulation scenario fills in any technical details necessary to fully define the simulation inputs that not present in the usage model.

# Environments

The list of environments we are considering is shown in the following table.  This list is here to allow us to relate an environment to a channel model.  We do not necessarily have to identify use cases for all environments.

| Environment | Includes | Applicable Channel Models |
| --- | --- | --- |
| Residential, Domestic or Home | Intra-room. Room to room. Indoor to outdoor Large multi-family Dwelling | TBD |
| Small Enterprise | Enclosed offices Meeting room / Conference Room Classroom |  |
| Large Enterprise | Enclosed offices Meeting room / Conference Room Classroom Sea of cubes Multi-story office environment Campus |  |
| Hotspot | Airport Library Convention Center Hotel Shopping Mall Train Station / bus terminal Drive-in window |  |
| Outdoor | Outdoor sport event Campus City Square Public Park |  |
| Industrial | Indoor Large factory floor Hospital Warehouse Concert Hall / Auditorium Movie Theatre |  |
| Other custom environments | Wireless Backhaul Fixed Wireless Access |  |
| Mobile | Train Bus Plane |  |

# Application

| Application | Throughput (Mbps) | MSDU Size (B) | Application PER | Delay (ms) | Jitter (ms) | Source [ref] |
| --- | --- | --- | --- | --- | --- | --- |
| DV Audio/video | 25 |  |  |  |  | ?? |
| SDTV | 4-5 | Variable ~188 | 10^-3 | 200 | 100 | 1 |
| HDTV | 19.2 | ~188 | 10^-4 | 200 | 100 | 1 |
| DVD | 9.8 peak | ~188 | 10^-4 | 200 | 100 | 1 |
| Video Conf | 0.128 - 2 | ~512 | 10^-2 | 100 | 50 | 1 |
| Internet Streaming video/audio | 0.1 – 4 | ~512 | 10^-2 | 200 | 100 | 1 |
| Internet Streaming audio | ?? | ?? |  | 200 | 100 | ?? |
| VoIP | 0.02 – 0.15 | 200 | 5% | 30 | 15 | 1 |
| PCM 5.1 Audio | 7.45 | ~200 | 10^-4 | 200 | 100 | 1 |
| Futuristic Audio (Dave Richkas) | ?? | ?? | ?? | ?? | ?? | ?? |
| MP3 Audio | 0.064 – 0.32 | ~418 | 10^-4 | 200 | 100 | 1 |
| Content download (photo camera) | 30 | ~1500 |  |  |  | 1 |
| Internet File transfer (email, web, chat) | 1 | ~300 |  |  |  | 1 |
| Local File transfer | ?? | ?? |  |  |  | ?? |
| Interactive Gaming | 0.2 | ~150 |  | 100 |  | 1 |
| Netmeeting application/desktop sharing | ?? | ?? |  |  |  | ?? |
| Backhaul traffic | ?? | ?? |  |  |  | ?? |
| FWA traffic | ?? | ?? |  |  |  | ?? |
| Printing | 50 | ~1500 | ?? | ?? | ?? | ?? |

Other characteristics to be added:

- UDP/TCP

- Up/Down

- Packet Size Distribution

- Packet inter-arrival distribution

# Use cases

Eventually, the entries in the ”Application” column should be drawn from a well-defined set of applications listed in the above mentioned “Applications” table.

TBD – should we represent interferers (e.g. 2.4GHz cordless phone, Bluetooth) as a “use case”?

TBD – should we represent adjacent channel and out-of-band effects within our “use case”s?

(e.g. DECT, 900 MHz DSSS)?

TBD – should we require that simulations use realistic analogue filters, or ideal filters?

The score relates to the results reported in [4] from the vote on 21 July 2003.  This scores 3 for high, 2 for medium and 1 for low priority.  The "Devn" column shows the weighted absolute deviation in the votes (0 shows complete agreement and 1 shows complete disagreement).

| Number | Use case | Application | Environment | Score | Devn. |
| --- | --- | --- | --- | --- | --- |
| 25 | Mixed mode AP has legacy and HT STA | Legacy: file transfer HT: File transfer + SDTV | Small Enterprise | 2.89 | 0.20 |
| 3 | Multiple TVs running throughout the home getting their content from a single remotely located AV-server/AP/set top box. Local control of the content (changing channels, etc). | HDTV, SDTV, VoD control channel (look at the DVB contribution from Andy Gowans) | Residential | 2.87 | 0.23 |
| 8 | Music real time on multiple receivers throughout the home from a remotely located AV-server/AP/set top box receiver. | PCM Audio, MP3 Audio | Residential | 2.65 | 0.49 |
| 4 | Link the home digital camera/video to the TV/display for display of pictures and movies taken. | DV Audio/Video | Residential | 2.59 | 0.55 |
| 9 | Net meeting in a conference/class room to share someone’s display. 30 participants/students | Netmeeting application/desktop sharing | Conference room/class room |  | 0.53 |
| 36 | Enterprise conference room – 20 to 30 users | Local file transfer, internet file transfer, printing | Enterprise |  |  |
| 10 | Reconfigurable/temporary office space, Ethernet cable replacement (similar throughput to wired cable). Back up files, email, web surfing, printing, etc. | Local File Transfer, printing | Enterprise – sea of cubes, home, hotel | 2.50 | 0.70 |
| 23 | Point-multipoint link for wireless backhaul | Backhaul traffic. | Arena? Hotspot? | 2.46 | 0.61 |
| 31 | Enterprise high stress. Surfing the web, e-mail, printing, file transfers within the intranet. | Internet File Transfer. Printing. Local File Transfer | Small/Large Enterprise | 2.45 | 0.5 |
| Done up to here in session. |  |  |  |  |  |
| 13 | Synchronize your local device with the server – email, calendar, etc. Hot spot/airport/airplane | Internet File Transfer | Large open area – hot spot, airport, train station, bus terminal. Airplane, Train | 2.40 | 0.69 |
| 14 | Download digital pictures and home movies to a PC/AV-server | Content download | Residential – same room | 2.34 | 0.71 |
| 24 | Point-multipoint link for Fixed Wireless Access | FWA traffic | Outdoor | 2.32 | 0.65 |
| 33 | AV Communication Video Phone: Peer to peer AV communication. Video Conferencing: AV conference between multiple devices | Internet Streaming Video/Audio (multicast/broadcast) | Residential, business. | 2.31 | 0.64 |
| 26 | Co-channel legacy BSS interference | File transfer + SDTV | Small Enterprise | 2.23 | 0.65 |
| 1 | One personal phone everywhere – home, office, car. Each person has a phone that works everywhere, home, office, car – same number. An extension of the cell phone into the office building. This includes cordless phone over VoIP. | VOIP integrated with other wireless WAN technologies | Residential, Enterprise – large and small, Car, etc. | 2.12 | 0.84 |
| 12 | Backup/transfer files between PCs located throughout the home, printing. Access point router. | Local File Transfer, printing | Residential | 2.06 | 0.63 |
| 32 | Portable /Internet AV Devices. MP3 or other player playing music directly from an internet through a residential gateway. | Internet Streaming Audio | Residential | 2.06 | 0.33 |
| 22 | Point-point link for wireless backhaul | Backhaul traffic | Arena? Hotspot? | 2.00 | 0.73 |
| 11 | Download video, music and other data files to a device in an automobile in the home garage or driveway.Broadband file transfer – HT rates. | Internet File Transfer | Residential / Outdoor | 1.78 | 0.68 |
| 18 | Access of networked software from the classroom. 30 participants, simultaneously signing on. | Local File Transfer | Conference room/class room | 1.76 | 0.73 |
| 29 | Online distance learning/broadcasting locally | Internet File Transfer, Internet Streaming Audio/Video | Residential, small/large enterprises | 1.75 | 0.63 |
| 28 | Real-time streaming of ultrasound video and real-time viewing of x-ray/MRI/CT images as well as medical diagnostics signal streams / patient monitoring data | Internet Streaming video | Hospital | 1.74 | 0.56 |
| 34 | Ad-hoc mode example |  |  | 1.74 | 0.72 |
| 2 | Multiplayer Internet gaming anywhere within the home / Internet Café. | Internet Streaming Video and Audio? | Residential/small enterprise (internet cafes) | 1.69 | 0.77 |
| 15 | Exchange files between PCs or between CE devices – ad hoc (no access point). Using IBSS mode of operation. | Local File Transfer | Residential, Enterprise | 1.60 | 0.65 |
| 7 | Remotely located security cameras transmitting video signal to a monitoring location. | Internet Streaming Video | Outside/Inside Residential, Small office building | 1.55 | 0.55 |
| 19 | Update/view medical records from patient rooms. | Local File Transfer | Large building, multiple walled rooms. | 1.53 | 0.66 |
| 6 | Watch a clear replay of an event from your seat in a sporting arena. | Internet Streaming Video | Arena | 1.51 | 0.50 |
| 21 | Interactive multi-person gaming – ad hoc. | Interactive gaming | Car, home, train station | 1.39 | 0.62 |
| 16 | Update inventory from the warehouse and the retail floor. | Local File Transfer | Large open building. | 1.24 | 0.38 |
| 5 | Watch a movie of your choice, when you want it, it your hotel room. | Internet streaming audio/video, SDTV, HDTV | Hotel – TBD needs to be mapped into an existing environment | 1.11 | 0.19 |
| 30 | Video conferencing with headset | Internet Streaming video/audio + headset interference | Small Enterprise | 1.08 | 0.15 |
| 20 | Obtain real time interactive player and game stats from your seat at a sporting event. | Content Downloading | Arena | 1.07 | 0.12 |
| 27 | Legacy mode operation in legacy BSS | File transfer | Small Enterprise | Not voted |  |

# Usage Models

The purpose of these models is to merge representative use cases to create a small number of credible mixtures of applications.  The usage models have to be realistic (in terms that they are covered by the use cases listed above), different from each other and cover some subset of the use cases that are identified to be priorities.

The number of usage models needs to be limited because each usage model adds simulation time to the preparation of results for submission against the TBD 802.11 TGn criteria.

Note, the usage models specified here define the mandatory results that will be submitted.  There is nothing to stop a submission demonstrating capabilities beyond these models – for example by increasing the numbers of stations included.

Eventually, there will be enough information here to unambiguously simulate against.

(note,  green colour shows parts of the usage model validated in the top use cases up to the "done up to here" division).

| Usage Model | Application mix | Comments |
| --- | --- | --- |
| Residential | 3 x HDTV, 1 x SDTV, 2 x VoIP [t1], 3 x internet file transfer, 2 x PCM Audio, Futuristic Audio, 1 x MP3 Audio, 1~2x Video gaming, 1 x local file transfer 1-3 printing | This scenario should be room to room or indoor/outdoor. The exact spatial distribution and mobility as well as the desired number of simultaneous connections can be referred to [2]. |
| Residential IBSS | 1 x peer-peer DV Audio/Video (rate, range combination > current technology) 1 x local file transfer peer-peer synchronisation |  |
| Small Enterprise | 1 AP 6 STA: Mix of internet file transfer, internet streaming video/audio and local file transfer, printing (10-15Mbps offered load). 4 STA: internet file transfer. 2 STA: VoIP audio. Offered load should exceed 100 Mbps. |  |
| Large Enterprise | 1 AP, cellular frequency re-use. 6 STA: internet file transfer, internet streaming video/audio, printing. 4 STA: local file transfer nn STA: VoIP Aggregate offered load should exceed 100Mbps. | Cellular re-use implies a frequency plan and a re-use topology. This topology will depend on the number of channels available given the band and channel width used in the submission, and is likely to differ between existing 2.4 GHz and 5 GHz bands. Cellular re-use implies that results will be presented for a BSS surrounded by "nearest neighbor" co-channel BSSs of the same specification. The number and distance of these co-channel BSS is determined from the cellular topology. |
| Conference room | 1 AP 30 STA netmeeting, desktop sharing | LOS small enterprise |
| Public park/Outdoor space | 1 AP, cellular frequency reuse. 25 STA: internet file transfer, printing 10 STA: internet streaming audio. 15 STA: internet streaming video/audio (Note, offered load must exceed existing .11a rates to make this a valid scenario) |  |
| Outdoor Backhaul | 1 AP: ?? STA: point-multipoint wireless backhaul |  |
| Mixed-mode BSS | 1 HT AP 3 legacy STA: internet file transfer 1 HT STA: SDTV 2 HT STA: internet file transfer |  |
| Co-channel legacy BSS | 1 legacy AP with 6 legacy STA. 1 HT AP and 6 STA. Cochannel with legacy AP. All STA doing internet file transfer. 2 HT STA: internet streaming audio. |  |

Additional coexistence usage models (802.19):

| Coex 1 | Placeholder for non 802.11 coexistence usage model #1 | To be determined by 802.19 during task-group phase |
| --- | --- | --- |
| Coex 2 | Placeholder for non 802.11 coexistence usage model #2 | To be determined by 802.19 during task-group phase |
| Coex 3 | Placeholder for non 802.11 coexistence usage model #3 | To be determined by 802.19 during task-group phase |

# Comments to carry forward

Comments from the committee members to carry forward in definition of the simulation scenarios:

(Javier) I believe it is important if we can differentiate HT scenarios from other technologies, such as 802.15.3a. For example, the residential or small enterprise scenarios are similar to the ones proposed in the 15.3a requirements document. I suggest considering this differentiation when defining the topology))

(Paul) The Usage models must also include some of the assumed mobility and ad hoc requirements identified.

We didn't get to vote on this,  so I'm putting it next to the netmeeting conference case with which it is most similar.

7File Transfer at HT at the minimum figure in the PAR should be a requirement for these applications. We may want to emphasize the differentiation between regular FTP and “Broadband FTP” (e.g. very short download time for audio files, “large” email message instantaneous sync, enterprise-class FTP etc)