IEEE P802.11
Wireless LANs

Usage Models

Date:	September 15, 2003

Authors/Contributors:

| Name | Company | Address | Phone | Fax | Email |
| --- | --- | --- | --- | --- | --- |
| Adrian P. Stephens | Intel Corporation | 15 JJ Thompson Avenue, Cambridge CB3 0FD, United Kingdom | +44 1223 763457 |  | adrian.p.stephens@intel.com |
| Bjorn Bjerke | Qualcomm | 9 Damonmill Sq., Suite 2A, Concord, MA 01742, USA | +1 781-276-0912 | +1 781-276-0901 | bbjerke@qualcomm.com |
| Eldad Perahia | Cisco |  |  |  | eperahia@cisco.com |
| Javier del Prado | Philips | 345 Scarborough Rd, Briarcliff Manor, NY, 10510, USA | +1 914 945 6000 | +1 914 945 6580 | javier.delprado@philips.com |
| Mary Cramer | Agere Systems | 1110 American Parkway NE, Allentown, PA 18109-9138, USA | +1 610-712-6112 | +1 610 712 1182 | mecramer@agere.com |
| Paul Feinberg | Sony | 1 Sony Drive MD TA1-5 Park Ridge, NJ 07656 | +1 201 930-6316 | +1 201 930-6397 | Paul.Feinberg@am.sony.com |
| Rahul Malik | Panasonic | Blk 1022 Tai Seng Ave. #06-3530 Tai Seng Industrial Estate, Singapore 534415 | +65 6550-5482 | +65 6550-5459 | rahul@psl.com.sg |
| Sanjeev Sharma | Samsung | 75 W. Plumeria Dr. , San Jose, CA, 95134 | +1 408- 544 5978 |  | ssharma@sisa.samsung.com |
| Timothy P Wakeley | Hewlett Packard Corporation | 8000 Foothills Blvd, Roseville, CA 95747 | 916-785-1619 |  | Tim.Wakeley@hp.com |
| Tomer Bentzion | Metalink | Yakum Business Park, 60972 Yakum Israel | +972 9 960 5365 | +972 9 960 5399 | tomerb@metalink.co.il |
| Youngsoo Kim | Samsung | Mt. 14-1 Nongseo-Ri, Giheung-Eup, Yongin-Si, Gyeonggi-Do, Korea 449-712 | +82-31-280-9614 | +82-31-280-9555 | kimyoungsoo@samsung.com |
| Vinko Erceg | Zyray Wireless |  |  |  | Verceg@zyraywireless.com |
| Hervé Bonneville | Mitsubishi Electric | 1 allée de Beaulieu, CS 10806, 35708 Rennes cedex 7, France | +33 (0)2 23 45 58 58 |  | bonneville@tcl.ite.mee.com |
| Bruno Jechoux | Mitsubishi Electric | 1 allée de Beaulieu, CS 10806, 35708 Rennes cedex 7, France | +33 (0)2 23 45 58 58 |  | jechoux@tcl.ite.mee.com |

Abstract

This document defines usage models for the High Throughput Study Group, intended to be used by 802.11 TGn as part of its selection process to generate simulation results for specified well-defined simulation scenarios.

# Revision History

| ### Revision | ### Comments | ### Date | ### Commenter |
| --- | --- | --- | --- |
| R0 Draft 0 | This document is in its beginning phases. The initial target is to generate a concept that can be reviewed and commented on. The first draft has definitions of some terms along with an initial stab at a few use cases. This is intended to start discussion and review. | July 8, 2003 | Mary Cramer |
| R0 Draft 1 | APS additions | July 9, 2003 | Adrian Stephens |
| R0 | Merged in comments and changes from the group of authors and made public via the .11 reflector. | July 11, 2003 | Adrian Stephens |
| R1 Draft 1 | Javier's contribution merged in Lalit's Contribution merged in Changes made to implement telecon discussion on 14 July 2003 Application table added Usage Models adjusted to reference applications named in the application table | July 14, 2003 | Adrian Stephens |
| R1 Draft 2 | Paul Feinberg's contribution merged in | July 18, 2003 | Adrian Stephens |
| R2 Draft 3 | Chiu Ngo's comments merged in | 21 July 2003 | Adrian Stephens |
| R2 | Results of Use Case voting added and table of use cases sorted by score Printing application and use cases added (Tim Wakeley) | 22 July 2003 | Adrian Stephens |
| R3 | Updates made at face-to-face session of the special committee, (8:00 am, 24 July 2003). Validation of usage models against use cases partially considered. | 24 July 2003 | Adrian Stephens |
| R4 | Merged in comments from Tiger Team (Rahul Malik, Bjorn Bjerke, Eldad Perahia, Paul Feinberg, Youngsoo Kim) to complete use case coverage in usage models. | 1 Aug 2003 | Adrian Stephens |
| R5 draft 1 | Merged in comments by Vinko Erceg on appropriate channel models to use. Merged in comment by Pratik Mehta. Merged in changes by Tim Wakely (addition of printing) | 22 August 2003 | Adrian Stephens |
| R5 draft 2 | Added comments to coexistence section to action changes requested in previous telecon. | 22 August 2003 | Adrian Stephens |
| R5 | Added new section – a sample simulation scenario for discussion | 22 August 2003 | Adrian Stephens |
| R6 | Actioned comments from: - August 26th Telecon - George Vlantis email of 29/08/2003 - Eldad Perahia email of 29/08/2003 (Scenario 4) - Hervé Bonneville email of 28/08/2003 (submitted as 11-03-0696r0) - Rahul Malik email of 3/9/2003 | 5 September 2003 | Adrian Stephens |
| R7 | Some additional scenarios added (George & Adrian) | 8 September 2003 | Adrian Stephens |
| R8 | Clarified meaning of Mean Rate / UDP/TCP using text supplied by John Ketchum Scenarios 6 and 7 contributed by Rahul Malik. | 13 September 2003 | Adrian Stephens |
| R9 | Edited at meeting held during September 2003 interim – Tuesday pm. | 16 September 2003 | Adrian Stephens |
| R9 | Edited at meeting held during September 2003 interim – Wednesday am | 17 September 2003 | Adrian Stephens |

# Introduction

To support the definition of a higher throughput WLAN standard (which will incorporate changes to both the MAC and the PHY) within the IEEE (to be published eventually as the 802.11n amendment), this document attempts to define usage models based on various market-based use-cases. The usage models are intended to support the definitions of network simulations that will allow 802.11 TGn to evaluate the performance of various proposals in terms of, for example,  network throughput, delay, packet loss and other metrics. It is anticipated that the outputs of this document will aid in the subsequent development of the evaluation and selection criteria used by TGn.

Note - These usage models that the usage model committee develops here are subject to the following constraints :

C1:	They are relevant to the expected uses of the technology

C2:	They require higher throughput than can be achieved with existing 802.11 technology

C3:	They are capable of being turned into an unambiguous simulation scenario

# Process going forward

Document [3] described an initial process.  This section describes our future process and will be replaced with a description of what (if anything) needs to be completed in the task group.

We (the usage model committee) need to:

- Characterise the application traffic descriptions (started, but not complete)

- Review and freeze the usage models

- Create simulation scenarios from the usage models (started by example)

# Definitions

This section defines some of the terms used in this document.

Application – a source or sink of wireless data that relates to a particular type of user activity.

Examples: Streaming video. VOIP.

Environment – The type of place a WLAN system is deployed in.  Initial examples:  home, large office.

### Use case – A use case is a description of how an end user uses a system that exercises that system’s deployment of WLAN.  A use case includes an application in a deployment environment with details regarding the user activity and both sides of the link.

### Examples: Watching television remote from the cable or set-top box within the home.  Talking on the telephone remote from one’s desk at work.

Usage Model – A specification of one or more applications and environments from which a simulation scenario can be created once the traffic patterns of the applications are known.  Usage models are created to "cover" use cases.

### Simulation Scenario – A simulation scenario is a description of a usage model that supports simulation.  A simulation scenario includes details needed for simulation.  Types of details to be included are descriptions that link the usage model to the simulation scenario: environment linked to a channel model, position of the AP (console or ceiling mounted), position of STAs w.r.t. AP, uplink and downlink traffic (# packets, size of packets, interference (number and types of users on the same WLAN channel – adjacent cells, the same cell, number and types of users on alternate channels, BT, baby monitors, GPRS or other systems).  A simulation scenario is created from a Usage Model by characterising the traffic profile of the applications and possibly merging multiple applications together to reduce simulation time.

# Mappings between Application, Environment, Channel Model, Use case, Usage Model and Simulation Scenario

Understanding and defining the application, environment, channel model, use case, usage model and simulation scenario are all necessary to create comparative results from 802.11 TGn proposals.

Channel models are currently being defined by the 802.11 High Throughput Channel Model Special Committee in [5].   They currently have 6 channel models.  Each environment will map to a single channel model.

Each use case involves the use of one or more applications and is defined for one or more environments.  It represents a single type of use of a system using the technology.

Each application reflects a source or sink of data.   They will eventually be characterised in terms of a traffic profile that allows a simulation of the application to be created.

Each usage model contains a representative mixture of applications and channel models designed to adequately cover the important use cases.  There is a many to many mapping between use cases and usage models (i.e., the same use case may contribute to multiple usage models and the same usage model may include applications from multiple use cases).

There will be a one-to-one mapping between usage models and simulation scenarios.   The usage model is a marketing-oriented description of a "reasonable mixture" covering the important use cases.  The simulation scenario fills in any technical details necessary to fully define the simulation inputs not present in the usage model.

# Environments

The channel models identified in [5] are described as follows:

| Model | Environment | LOS/NLOS |
| --- | --- | --- |
| A | Flat fading (no multipath) | NLOS |
| B | Residential | LOS |
| C | Residential / Small Office | LOS/NLOS |
| D | Typical Office | NLOS |
| E | Large Office | NLOS |
| F | Large Space (indoors / outdoors) | NLOS |

Note, outdoor usage is TBD.

The list of environments we are considering is shown in the following table.  This list is here to allow us to relate an environment to a channel model.  We do not necessarily have to identify use cases for all environments.

| Environment | Includes | Applicable Channel Models |
| --- | --- | --- |
| Residential, Domestic or Home | Intra-room Room to room Indoor to outdoor Large multi-family dwelling. Note: one or more PCs in the home may be notebooks or other portable devices that come home with the user. these wireless devices may have more than one wireless technology included. | B, C |
| House to house | One main house has AP with uplink connection, Another house holds single or multiple STA(s), Guest house, garage or studio. In garage model, STA may be embed inside a car. | TBD |
| Small Enterprise | Enclosed offices Meeting room / conference room Classroom | C |
| Medium/Large Enterprise | Enclosed offices Meeting room / conference room Classroom Sea of cubes Multi-story office environment Campus | D |
| Hotspot | Airport Library Convention center Hotel Shopping mall Arcade Train station / bus terminal Drive-in window | D, E |
| Outdoor | Outdoor sport event Campus City Square Public park Amusement park | TBD (outdoors model, max distance needed, maybe model F) |
| Industrial | Indoor Large factory floor Hospital Warehouse Concert hall / auditorium Movie theatre | E, F |
| Other custom environments | Wireless backhaul Fixed wireless access: outside to multiple STA inside outside to multiple STA outside | TBD (outdoors model, max distance needed, maybe model F) |
| Mobile | Train Bus Plane Roadside APs for data-service in-car (fast roaming) | Assuming AP on the bus, train, then the following models apply: B, C Otherwise Outdoor |

# Applications

This table lists the applications that are referred to from the usage models, together with relevant traffic parameters.

The parameters are defined as follows:

- MSDU size:	Packet size at the top of the MAC

- Maximum PLR:	Maximum packet loss rate at the top of the MAC.  This is defined by the loss rate that can be tolerated by the application.

- Maximum Delay: 	Maximum transport delay at the top of the MAC – i.e. between matching MA-UNITDATA.request and .indication.

- Protocol:	Indicates the network-layer protocol running between the data source and the MAC.  It takes one of two values: TCP or UDP.  These are intended to represent a generic acknowledged and a generic unacknowledged network-layer protocol.

Note on the meaning of "Offered load" and "Protocol".  Applications identified as being carried by UDP are assumed to generate MSDUs at a fixed rate, as identified in the "Offered load" column.  Inability to carry the traffic generated by a UDP application, due to insufficient throughput  capability, results in lost MSDUs, which is reported in simulation results as a packet loss rate, or an outage, associated with the application.  It is likely that the comparison criteria will include a measure of whether this packet loss rate exceeds the maximum specified for the application in this table.

Traffic carried by TCP is assumed to be served on a best-effort basis, and applications using TCP are assumed to generate MSDUs at rates up to the

value given in the "Offered load" column.  Being an acknowledged protocol with a constrained window size, TCP responds to congestion in the BSS by reducing application throughput without losing MSDUs.  This effect is reflected in simulation results by reporting achieved throughput for applications using TCP.

| Number | Application | Offered Load (Mbps) | Protocol | MSDU Size (B) | Maximum PLR | Maximum Delay (ms) | Source [ref] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | DV Audio/video | 28.8 | UDP | 1024 | 10^-7 (corresponds to a rate of 0.5 loss/hour) | 200 | SD Specifications of Consumer-Use Digital VCRs Max PLR: 15-03-276r0 |
| 2 | VoD control channel | 0.06 | UDP | 64 | 10^-2 | 100 | Guess |
| 3 | SDTV | 4-5 | UDP | 1500 | 5*10^-7 | 200 | 1 |
| 4 | HDTV (Video/Audio) | 19.2-24 | UDP | 1500 | 10^-7 | 200 | 1 |
| 5 | DVD | 9.8 peak | UDP | 1500 | 10^-7 | 200 | 1 |
| 6 | Video Conf | 0.128 - 2 | UDP | 512 | 10^-2 | 100 | 1 |
| 7 | Internet Streaming video/audio | 0.1 – 4 | UDP | 512 | 10^-2 | 200 | 1 |
| 8 | Internet Streaming audio | 0.064~0.256 | UDP | 418 | 10^-4 | 200 | ?? |
| 9 | VoIP | 0.02 – 0.15 | UDP | 100 | 5% | 30 | ITU-T G.114 300ms round-trip delay |
| 10 | Reserved |  |  |  |  |  |  |
| 11 | Reserved |  |  |  |  |  |  |
| 12 | MP3 Audio Other formats are taking over (AAC/MPEG-4, OggVorbis, etc) | 0.064 – 0.32 | UDP | 418 | 10^-4 | 200 | 1 |
| 12.5 | Reserved |  |  |  |  |  |  |
| 13 | Content download (photo camera) | 11 | TCP | 1500 | n/a |  | Corresponds to USB and flash speed |
| 14 | Internet File transfer (email, web, chat) | 1 | TCP | 300 | n/a |  |  |
| 15 | Local File transfer, printing | 30 | TCP | 1500 | n/a |  | Aps guess |
| 16 | Interactive Gaming [Controller to Console x 1] | 0.5 | UDP | 50 | 10^-4 | 16 | 2 |
| 17 | Interactive Gaming [Console to Display] | 100+ | UDP | 1500 | 10^-2 | 10 | 2 video image: 320x240x15 @ 60Hz |
| 18 | Interactive Gaming [Console to Internet Access] *NOTE : Depends on Game Type | 1 | UDP | 512 | 10^-4 |  | 2 |
| 19 | Netmeeting application/desktop sharing | 0.5 | TCP | 512 | n/a |  | Group guess |
| 20 | Reserved |  |  |  |  |  |  |
| 21 | Point-multipoint backhaul traffic | 5 | UDP | 512 | 100 |  | Bjorn Bjerke |
| 22 | Reserved |  |  |  |  |  |  |
| 23 | Video phone | 0.5 | UDP | 512 | 10^-2 | 100 | Aps guess |
| 24 | Remote user interface (X11, Terminal Server Client) (remote display/keyboard/mouse) | 0.5-1.5 (peak) | UDP | 700 | n/a | 100 | 11-03-0696r0 |

Use Cases

Eventually, the entries in the "Application" column should be drawn from a well-defined set of applications listed in the above mentioned “Applications” table.

The score relates to the results reported in [4] from the vote on 21 July 2003.  This scores 3 for high, 2 for medium and 1 for low priority.  The "Devn" column shows the weighted absolute deviation in the votes (0 shows complete agreement and 1 shows complete disagreement).

| Number | Covered by model # | Use case | Application | Environment | Score | Devn. |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 1, 3, 4, 5, 6, 7 | One personal phone everywhere – home, office. Each person has a phone that works everywhere, home, office – same number. An extension of the cell phone into the office building. This includes cordless phone over VoIP. | VOIP integrated with other wireless WAN technologies | Residential, Enterprise – large and small | 2.12 | 0.84 |
| 2 | 1 | Multiplayer Internet gaming anywhere within the home / Internet Café. | Interactive gaming (console to internet), internet gaming (controller to console) | Residential/small enterprise (internet cafes) | 1.69 | 0.77 |
| 3 | 1 | Multiple TVs running throughout the home getting their content from a single remotely located AV-server/AP/set top box. Local control of the content (changing channels, etc). | HDTV, SDTV, VoD control channel | Residential | 2.87 | 0.23 |
| 4 | 2 | Link the home digital camera/video to the TV/display for display of pictures and movies taken. | DV Audio/Video | Residential | 2.59 | 0.55 |
| 5 | 6 (HDTV not covered) | Watch a movie of your choice, when you want it, it your hotel room. | Internet streaming audio/video, SDTV, HDTV | Hotspot | 1.11 | 0.19 |
| 6 | 7 | Watch a clear replay of an event from your seat in a sporting arena. | Internet Streaming Video | Outdoor | 1.51 | 0.50 |
| 7 | 1 | Remotely located security cameras transmitting video signal to a monitoring location. | SDTV | Outside/Inside Residential, Small office building (not covered) | 1.55 | 0.55 |
| 8 | 1 | Music real time on multiple receivers throughout the home from a remotely located AV-server/AP/set top box receiver. | PCM Audio, MP3 Audio | Residential | 2.65 | 0.49 |
| 9 | 5 | Net meeting in a conference/class room to share someone’s display. 30 participants/students | Netmeeting application/desktop sharing | Conference room/class room |  | 0.53 |
| 10 | 1, 4 (hotel not covered) | Reconfigurable / temporary office space, Ethernet cable replacement (similar throughput to wired cable). Back up files, email, web surfing, printing, etc. | Local File Transfer, printing | Enterprise – sea of cubes, home, hotel | 2.50 | 0.70 |
| 11 | 1, 2 | Download video, music and other data files to a device in an automobile in the home garage or driveway. Broadband file transfer – at HT rates. | Internet File Transfer | Residential / Outdoor | 1.78 | 0.68 |
| 12 | 1, 2 | Backup/transfer files between PCs located throughout the home, printing. Access point router. | Local File Transfer | Residential, Residential IBSS. | 2.06 | 0.63 |
| 13 | 6, 7 (Airplane & train not covered) | Synchronize your local device with the server – email, calendar, etc. Hot spot/airport/airplane | Internet File Transfer | Large open area – hot spot, airport, train station, bus terminal. Airplane, Train | 2.40 | 0.69 |
| 14 | 2 | Download digital pictures and home movies to a PC/AV-server | Content download | Residential – same room, Residential IBSS. | 2.34 | 0.71 |
| 15 | 2 | Exchange files between PCs or between CE devices – ad hoc (no access point). Using IBSS mode of operation. | Local File Transfer | Residential IBSS | 1.60 | 0.65 |
| 16 | Not covered | Update inventory from the warehouse and the retail floor. | Local File Transfer | Industrial | 1.24 | 0.38 |
| 18 | 5 | Access of networked software from the classroom. 30 participants, simultaneously signing on. | Local File Transfer | Conference room/class room | 1.76 | 0.73 |
| 19 | Not covered | Update/view medical records from patient rooms. | Local File Transfer | Industrial. | 1.53 | 0.66 |
| 20 | 7 | Obtain real time interactive player and game stats from your seat at a sporting event. | Internet File Transfer | Outdoor | 1.07 | 0.12 |
| 20.5 | 15 | View broadcast SDTV video/audio at a sporting event | SDTV | Outdoor / Arena | 2 | 0 |
| 20.6 | Not covered | View Video on demand at a sporting event / concert-hall /hotspot | SDTV | Outdoor / Arena | 2 | 0.36 |
| 21 | 1, 2 (train station not specifically covered) | Interactive multi-person gaming – ad hoc. | Interactive gaming | Home, train station | 1.39 | 0.62 |
| 22 | 14 | Point-point link for wireless backhaul | Backhaul traffic | Outdoor. | 2.00 | 0.73 |
| 23 | 8 (outdoor only covered) | Point-multipoint link for wireless backhaul | Backhaul traffic. | Arena? Hotspot? Could be both outdoor and indoor | 2.46 | 0.61 |
| 24 | 8 | Point-multipoint link for Fixed Wireless Access | FWA traffic | Outdoor | 2.32 | 0.65 |
| 25 | 9 | Mixed mode AP has legacy and HT STA | Legacy: file transfer HT: File transfer + SDTV | Large Enterprise | 2.89 | 0.20 |
| 26 | 11 | Co-channel legacy BSS interference | File transfer + SDTV | Small Enterprise | 2.23 | 0.65 |
| 27 | Does not need a usage model | Legacy mode operation in legacy BSS | Internet/Local File transfer | Small Enterprise | Not voted |  |
| 28 | Not covered | Real-time streaming of ultrasound video and real-time viewing of x-ray/MRI/CT images as well as medical diagnostics signal streams / patient monitoring data | SDTV, local file transfer | Hospital (Industrial) similar to Large Enterprise (?) | 1.74 | 0.56 |
| 29 | 1, 3, 4 | Online distance learning/broadcasting locally | Internet File Transfer, Internet Streaming Audio/Video | Residential, small/large enterprises | 1.75 | 0.63 |
| 30 | 5 | Video conferencing with headset | Internet Streaming video/audio + headset interference | Small Enterprise | 1.08 | 0.15 |
| 31 | 3, 4 | Enterprise high stress. Surfing the web, e-mail, printing, file transfers within the intranet. | Internet File Transfer. Printing. Local File Transfer | Small/Large Enterprise | 2.45 | 0.5 |
| 32 | 1 | Portable /Internet AV Devices. MP3 or other player playing music directly from an internet through a residential gateway. | Internet Streaming Audio | Residential | 2.06 | 0.33 |
| 33 | 1, 2, 4 | AV Communication Video Phone: Peer to peer AV communication. Video Conferencing: AV conference between multiple devices | Internet Streaming Video/Audio (multicast/broadcast) | Residential, Small/large Enterprise | 2.31 | 0.64 |
| 34 | 2 | Ad-hoc mode example | Local File Transfer | Residential IBSS | 1.74 | 0.72 |
| 36 | 5 | Enterprise conference room – 20 to 30 users | Local file transfer, internet file transfer, printing | Enterprise | 2 | 0.29 |
| 37 | Not covered | Lightweight terminal wirelessly connected to a remote computer | Remote user interface | Residential, Industrial, Enterprise | 1.8 | 0.43 |

# Usage Models

The purpose of these models is to merge representative use cases to create a small number of credible mixtures of applications.  The usage models have to be realistic (in terms that they are covered by the use cases listed above), different from each other and cover some subset of the use cases that are identified to be priorities.

The number of usage models needs to be limited because each usage model adds simulation time to the preparation of results for submission against the TBD 802.11 TGn criteria.

Note, the usage models specified here define the mandatory results that will be submitted.  There is nothing to stop a submission demonstrating capabilities beyond these models – for example by increasing the numbers of stations included.

Eventually, there will be enough information here to unambiguously simulate against.

| Model # | Usage Model | Covers Use Cases | Score | Application mix | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | Residential | 3, 8, 10, 33, 1, 32, 11, 29, 7, 21 |  | 1 x AP STA 1: 19.2 Mbps HDTV (A4) , VoD control channel (A2) STA 3: 24 Mbps HDTV (A4) , VoD control channel (A2) STA 4: SDTV (A3), internet file transfer (A9), local file transfer A(15) STA 5 & 6: Video Phone (A23) STA 7,8 & 9: VoIP (A9) STA 10: Internet streaming video (7), MP3 Audio (A12), Video gaming, controller to internet (A18), local file transfer A(15) STA 11: Video gaming, controller to console (A16) | This scenario should be room to room or indoor/outdoor. The exact spatial distribution and mobility as well as the desired number of simultaneous connections can be referred to [2]. 20m range. |
| 2 | Residential IBSS | 4, 33, 11, 34, 15, 21 |  | 0 AP STA 1: peer-peer DV Audio/Video (A1) sink STA 2,3: local file transfer (A15) sink, local file transfer (A15) source STA 4,5,6,7: Video Phone (A23) sink, Video Phone (A23) source STA 8: 4 x Video gaming, controller to console (A16), console STA 9: content download (A13) sink STA 10: peer-peer DV Audio/Video (A1) (rate, range combination > current technology) source STA 11,12,13,14: Video gaming, controller to console (A16) controller STA 15: content download (A13) source | Note, all these devices are operating on the same channel. |
|  | Reserved |  |  | local file transfer (A15) |  |
| 4 | Large Enterprise | 10, 31, 33, 1, 29 |  | 1 AP, cellular frequency re-use. 10 STA: internet file transfer (A14), internet streaming video/audio (A7), local file transfer (A15) Video Conferencing (A6) 14 STA: local file transfer (A15), local file transfer (A15) 6 STA: VoIP (A9) Aggregate offered load should exceed 100Mbps. | Minimum 15 – 20 meter range Cellular re-use implies a frequency plan and a re-use topology. This topology will depend on the number of channels available given the band and channel width used in the submission, and is likely to differ between existing 2.4 GHz and 5 GHz bands. Cellular re-use implies that results will be presented for a BSS surrounded by "nearest neighbor" co-channel BSSs of the same specification. The number and distance of these co-channel BSS is determined from the cellular topology. |
| 5 | Conference room | 9, 36, 1, 18, 30 |  | 1 AP 30 STA (1-30): netmeeting, desktop sharing (A19), Local File Transfer (A15), local file transfer (A15) 10 STA (31-40): VoIP (A9) 1 STA: SDTV (A3) | LOS small enterprise 15m |
| 6 | Hot spot | 1, 13, 5 |  | 1 AP 30 STA: Internet file transfer (A14), internet streaming video/audio (A7), 2 STA: SDTV (A3) 20 STA: VoIP | Indoors: Large Enterprise channel model In hot spot area, most of traffic goes through internet, and the duration of the session is limited (less than 2 hours). |
| 7 | Public park / Outdoor space | 1, 13, 6, 19, 20, 20.6 |  | 1 AP, cellular frequency reuse. 25 STA: internet file transfer (A14) 10 STA: internet streaming audio (A8) 20 STA: internet streaming video/audio (A7) (at 1Mbps) 5 STA: interactive gaming (console to internet) (A18) 20 STA: VoIP (Note, offered load must exceed existing .11a rates to make this a valid scenario) | Outdoor channel model |
| 8 | Point-Multipoint Backhaul | 23, 24 |  | 1 AP: 20 STA: point-multipoint wireless backhaul (A21) (these STA provide internet access to their client networks, which may be wired or wireless) | Outdoor + outdoor-to-indoor channel models |
| 10 | Reserved |  |  |  |  |
| 9 | Mixed-mode (containing Legacy + HT STAs) BSS | 1 |  | 1 HT AP legacy STA 1-3: internet file transfer (A14) 1 HT STA (STA4): SDTV (A3) 2 HT STA (STA5,6): internet file transfer (A14) | Will apply to large enterprise 20m range |
| 11 | Co-channel legacy BSS | 26 |  | 1 legacy AP with 6 legacy STA. 1 HT AP and 6 STA. Co-channel with legacy AP. All STA doing internet file transfer (A14). 2 HT STA: internet streaming audio (A8). 1 HT STA: SDTV (A3) | Will apply to large enterprise 20m range |
| 13 | Reserved |  |  |  |  |
| 14 | (Reserved) |  |  |  |  |
| 15 | (Reserved) |  |  |  |  |

## Coexistence

Apart from legacy 802.11 coexistence that is evaluated in model 14, there may be additional requirements for coexistence simulations placed on us by the 802.19 TAG.  It is unlikely that these can be formulated in time for the selection process, as details of the methodology need to be worked on.   This section lists some placeholders just as a reminder that we do need to work on these during the life of the 802.11n task group.

Possible coexistence technologies may include:

- 802.15.1 (Bluetooth ™)

- 802.15.3

- 802.15.3a

- 802.15.4

- 802.16a

- 2.4 GHz Cordless Phone

- 5.0 GHz Cordless Phone

- 2.4 GHz Video Transmitter

- 5.0 GHz non-OFDM Video Transmitter

- 2.4 GHz Microwave Oven (victim only)

- 802.11b

- "De-facto standard" Proprietary extensions of 802.11a or 802.11g

Additional coexistence usage models (802.19):

| Coex 1 | Placeholder for non 802.11 coexistence usage model #1 | To be determined by 802.19 during task-group phase |
| --- | --- | --- |
| Coex 2 | Placeholder for non 802.11 coexistence usage model #2 | To be determined by 802.19 during task-group phase |
| Coex 3 | Placeholder for non 802.11 coexistence usage model #3 | To be determined by 802.19 during task-group phase |

# Simulation Scenarios

add note that udp is not best effort and tcp is best effort

## Common Conditions

These conditions apply to all simulation scenarios.

| Condition | Description | Value |
| --- | --- | --- |
| Tx Power | Total transmit power of a system delivered to its antenna(s). | 17dBm |
| Rx Noise Figure |  | 10 dB |
| Simulation Duration | TBD whether or how we specify this. Options: - Metric variance based on repeated runs with different random seeds (e.g. 3% accuracy throughput) - Simulation time (e.g. 20minutes simulated time) - Real time (e.g. 5 hours on a P4-3GHz machine) |  |

## Scenario 1

| STA Name: AP |  | Role: Access Point |  |  | Location: 0, 0 |  | Channel Model: |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | Delay / Jitter limit |  |  |
| STA1 | 19.2 Mbps |  | Constant, UDP | 188 |  | 200 / 100ms |  | HDTV + PCM 5.1 Audio |
| STA3 | 24 Mbps |  | Constant, UDP | 188 |  | 200 / 100ms |  | HDTV + futuristic audio |
| STA4 | 4 Mbps |  | Constant, UDP | 188 |  | 200 / 100ms |  | SDTV |
| STA4 | 1 Mbps |  | TCP | 300 |  |  |  | Internet file |
| STA5 | 1 Mbps |  | TCP | 300 |  |  |  | Internet file |
| STA7 | 0.15 Mbps |  | Constant, UDP | 200 |  | 30 / 15 ms |  | VoIP |
| STA8 | 0.15 Mbps |  | Constant, UDP | 200 |  | 30 / 15 ms |  | VoIP |
| STA9 | 0.15 Mbps |  | Constant, UDP | 200 |  | 30 / 15 ms |  | VoIP |
| STA10 | 2 Mbps |  | TCP | 512 |  | 200 / 100ms |  | Internet Streaming video + MP3 audio |

| STA Name: STA1 |  | Role: HTDV Display |  |  | Location: 0, 5 |  | Channel Model: B |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | Delay / Jitter limit |  | Notes |
| AP1 | 60kbps |  | Constant, UDP | 256 |  | 150ms |  | VoD control channel |

| STA Name: STA2 |  | Role: HTDV Display |  |  | Location: 10, 0 |  | Channel Model: C |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | Delay / Jitter limit |  |  |
| AP1 | 60kbps |  | Constant, UDP | 256 |  | 150ms |  | VoD control channel |

| STA Name: STA3 |  | Role: HDTV Display |  |  | Location: 5, 0 |  | Channel Model: B |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | Delay / Jitter limit |  |  |
| AP1 | 60kbps |  | Constant, UDP | 256 |  | 150ms |  | VoD control channel |

| STA Name: STA4 |  | Role: SDTV Display, Gaming & Printing |  |  | Location: -7, -7 |  | Channel Model: C |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | Delay / Jitter limit |  |  |
| AP1 | 1 Mbps |  | Constant, TCP | 1500 |  | 100 / 50ms |  | Controller to internet |
| STA10 | 30 Mbps |  | Constant, TCP | 1500 |  |  |  | Local file transfer |
| AP1 | 50 Mbps |  | Constant, TCP | 1500 |  |  |  | Printing |

| STA Name: STA5 |  | Role: Video Phone |  |  | Location: -15,0 |  | Channel Model: C |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | Delay / Jitter limit |  |  |
| STA6 | 0.5 |  | Constant, UDP | 512 |  | 100 / 50 |  | Video Phone |

| STA Name: STA6 |  | Role: Video Phone + internet upload |  |  | Location: 0,-15 |  | Channel Model: C |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | Delay / Jitter limit |  |  |
| STA5 | 0.5 |  | Constant, UDP | 512 |  | 100 / 50 |  | Video Phone |
| AP1 | 1 |  | Constant, TCP | 1500 |  |  |  | Internet file |

| STA Name: STA7 |  | Role: VoIP Phone |  |  | Location: 20, 0 |  | Channel Model: C |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | Delay / Jitter limit |  |  |
| AP1 | 0.15 |  | Constant, UDP | 200 |  | 30 / 15 |  | VoIP Phone |

| STA Name: STA8 |  | Role: VoIP Phone |  |  | Location: 0, 20 |  | Channel Model: C |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | Delay / Jitter limit |  |  |
| AP1 | 0.15 |  | Constant, UDP | 200 |  | 30 / 15 |  | VoIP Phone |

| STA Name: STA9 |  | Role: VoIP Phone |  |  | Location: 0, -20 |  | Channel Model: C |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | Delay / Jitter limit |  |  |
| AP1 | 0.15 |  | Constant, UDP | 200 |  | 30 / 15 |  | VoIP Phone |

| STA Name: STA10 |  | Role: Video Console + Internet Entertainment |  |  | Location: 10,10 |  | Channel Model: C |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | Delay / Jitter limit |  |  |
| AP1 | 1 |  | Constant, TCP | 1500 |  | 200 / 100 |  | Console to Internet |

| STA Name: STA11 |  | Role: Video Gaming Controller |  |  | Location: 10, 5 |  | Channel Model: B |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | Delay / Jitter limit |  |  |
| STA10 | 0.5 |  | Constant, UDP | 20 |  | 4 / 2 |  | Controller to Console |

## Scenario 2 (Residential IBSS)

Summary:  UDP offered load: ~37 Mbps, TCP offered load:  90 Mbps.

| STA Name: STA1 |  | Role: DV Audio/Video source |  |  | Location: 4, -4 |  | Channel Model: C |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | Delay / Jitter limit |  | Notes |
| STA2 | 28.8 Mbps |  | Constant, UDP | 2000 |  | 200 /100 ms |  | Peer-2-peer DV audio/video |

| STA Name: STA2 |  | Role: DV Audio/Video sink |  |  | Location: 5, 5 |  | Channel Model: |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | Delay / Jitter limit |  |  |

| STA Name: STA3 |  | Role: Local file transfer |  |  | Location: -5, 1 |  | Channel Model: C |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | Delay / Jitter limit |  |  |
| STA4 | 30Mbps |  | TCP | 1500 |  |  |  | Local file transfer |

| STA Name: STA4 |  | Role: Local file transfer |  |  | Location: 3, 1 |  | Channel Model: C |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | Delay / Jitter limit |  |  |
| STA3 | 30Mbps |  | TCP | 1500 |  |  |  | Local file transfer |

| STA Name: STA5 |  | Role: Video Phone |  |  | Location: 0, 5 |  | Channel Model: C |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | Delay / Jitter limit |  |  |
| STA6 | 0.5 |  | Constant, UDP | 512 |  | 100 / 50 |  | Video Phone |
| STA7 | 0.5 |  | Constant, UDP | 512 |  | 100 / 50 |  | Video Phone |
| STA8 | 0.5 |  | Constant, UDP | 512 |  | 100 / 50 |  | Video Phone |

| STA Name: STA6 |  | Role: Video Phone |  |  | Location: -2, 0 |  | Channel Model: C |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | Delay / Jitter limit |  |  |
| STA5 | 0.5 |  | Constant, UDP | 512 |  | 100 / 50 |  | Video Phone |
| STA7 | 0.5 |  | Constant, UDP | 512 |  | 100 / 50 |  | Video Phone |
| STA8 | 0.5 |  | Constant, UDP | 512 |  | 100 / 50 |  | Video Phone |

| STA Name: STA7 |  | Role: Video Phone |  |  | Location: -5, 5 |  | Channel Model: C |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | Delay / Jitter limit |  |  |
| STA5 | 0.5 |  | Constant, UDP | 512 |  | 100 / 50 |  | Video Phone |
| STA6 | 0.5 |  | Constant, UDP | 512 |  | 100 / 50 |  | Video Phone |
| STA7 | 0.5 |  | Constant, UDP | 512 |  | 100 / 50 |  | Video Phone |

| STA Name: STA8 |  | Role: Video Phone |  |  | Location: 1, 5 |  | Channel Model: C |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | Delay / Jitter limit |  |  |
| STA5 | 0.5 |  | Constant, UDP | 512 |  | 100 / 50 |  | Video Phone |
| STA6 | 0.5 |  | Constant, UDP | 512 |  | 100 / 50 |  | Video Phone |
| STA7 | 0.5 |  | Constant, UDP | 512 |  | 100 / 50 |  | Video Phone |

| STA Name: STA9 |  | Role: Content Download - source |  |  | Location: 2, -5 |  | Channel Model: |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | Delay / Jitter limit |  |  |
| STA10 | 30 Mbps |  | TCP | 1500 |  |  |  | Digital Content |

| STA Name: STA10 |  | Role: Content Download - sink |  |  | Location: -2, -5 |  | Channel Model: |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | Delay / Jitter limit |  |  |

| STA Name: STA11 |  | Role: Video Gaming Console |  |  | Location: 0, -3 |  | Channel Model: |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | Delay / Jitter limit |  |  |

| STA Name: STA12 |  | Role: Video Gaming Controller |  |  | Location: 1, -2 |  | Channel Model: B |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | Delay / Jitter limit |  |  |
| STA11 | 0.5 |  | Constant, UDP | 20 |  | 4 / 2 |  | Controller to Console |

| STA Name: STA13 |  | Role: Video Gaming Controller |  |  | Location: 1, -3 |  | Channel Model: B |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | Delay / Jitter limit |  |  |
| STA11 | 0.5 |  | Constant, UDP | 20 |  | 4 / 2 |  | Controller to Console |

| STA Name: STA14 |  | Role: Video Gaming Controller |  |  | Location: 0, -2 |  | Channel Model: B |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | Delay / Jitter limit |  |  |
| STA11 | 0.5 |  | Constant, UDP | 20 |  | 4 / 2 |  | Controller to Console |

| STA Name: STA15 |  | Role: Video Gaming Controller |  |  | Location: 0, -4 |  | Channel Model: B |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | Delay / Jitter limit |  |  |
| STA11 | 0.5 |  | Constant, UDP | 20 |  | 4 / 2 |  | Controller to Console |

## Scenario 4 (Large Enterprise)

Summary:  UDP offered load: 6 Mbps, TCP offered load:  457 Mbps.

Still need to add some printing.  Perhaps some of the Local File Transfers could be converted to Printing.

Based on Cisco measured data, wired Ethernet traffic has roughly the following packet size distribution:

64	60%

256	10%

1024	25%

1500	5%

The traffic mix should result in a comparable distribution.

The stations are contained in at +/-10 by +/-10 grid.  This if half the range as specified in the usage model, but still seems rather large for a cell.

Stations closer to AP were assigned channel model D, the ones farther away were assigned E.  We may want to make them all D?

| STA Name: AP |  | Role: Access Point |  |  | Location: 0, 0 |  | Channel Model: |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | Delay / Jitter limit |  |  |
| STA1 | 1 Mbps |  | TCP | 300 |  |  |  | Internet file |
| STA2 | 1 Mbps |  | TCP | 300 |  |  |  | Internet file |
| STA3 | 1 Mbps |  | TCP | 300 |  |  |  | Internet file |
| STA4 | 1 Mbps |  | TCP | 300 |  |  |  | Internet file |
| STA5 | 1 Mbps |  | TCP | 300 |  |  |  | Internet file |
| STA6 | 10 Mbps |  | TCP | 1500 |  |  |  | Internet file, downloading large email attachments |
| STA7 | 1 Mbps |  | Constant, UDP | 512 |  | 100 / 50ms |  | Video conferencing |
| STA8 | 1 Mbps |  | Constant, UDP | 512 |  | 100 / 50ms |  | Video conferencing |
| STA9 | 2 Mbps |  | TCP | 512 |  | 200 / 100ms |  | Internet Streaming video + MP3 audio |
| STA10 | 2 Mbps |  | TCP | 512 |  | 200 / 100ms |  | Internet Streaming video + MP3 audio |
| STA11 | 30 Mbps |  | TCP | 1500 |  |  |  | Local File transfer |
| STA12 | 30 Mbps |  | TCP | 1500 |  |  |  | Local File transfer |
| STA13 | 30 Mbps |  | TCP | 1500 |  |  |  | Local File transfer |
| STA14 | 30 Mbps |  | TCP | 1500 |  |  |  | Local File transfer |
| STA15 | 30 Mbps |  | TCP | 1500 |  |  |  | Local File transfer |
| STA16 | 30 Mbps |  | TCP | 1500 |  |  |  | Local File transfer |
| STA17 | 30 Mbps |  | TCP | 1500 |  |  |  | Local File transfer |
| STA18 | 30 Mbps |  | TCP | 1500 |  |  |  | Local File transfer |
| STA19 | 30 Mbps |  | TCP | 1500 |  |  |  | Local File transfer |
| STA20 | 30 Mbps |  | TCP | 1500 |  |  |  | Local File transfer |
| STA21 |  |  |  |  |  |  |  | Local File transfer |
| STA22 |  |  |  |  |  |  |  | Local File transfer |
| STA23 |  |  |  |  |  |  |  | Local File transfer |
| STA24 |  |  |  |  |  |  |  | Local File transfer |
| STA25 | 0.15 Mbps |  | Constant, UDP | 200 |  | 30 / 15 ms |  | VoIP |
| STA26 | 0.15 Mbps |  | Constant, UDP | 200 |  | 30 / 15 ms |  | VoIP |
| STA27 | 0.15 Mbps |  | Constant, UDP | 200 |  | 30 / 15 ms |  | VoIP |
| STA28 | 0.15 Mbps |  | Constant, UDP | 200 |  | 30 / 15 ms |  | VoIP |
| STA29 | 0.15 Mbps |  | Constant, UDP | 200 |  | 30 / 15 ms |  | VoIP |
| STA30 | 0.15 Mbps |  | Constant, UDP | 200 |  | 30 / 15 ms |  | VoIP |

| STA Name: STA1 |  | Role: |  |  | Location: 5, -9.5 |  | Channel Model: E |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | Delay / Jitter limit |  | Notes |
| AP1 | .256 Mbps |  | TCP | 64 |  |  |  | Clicking on web link? |

| STA Name: STA2 |  | Role: |  |  | Location: 3.5, 7.5 |  | Channel Model: D |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | Delay / Jitter limit |  | Notes |
| AP1 | .256 Mbps |  | TCP | 64 |  |  |  | Clicking on web link? |

| STA Name: STA3 |  | Role: |  |  | Location: 7.5, -95 |  | Channel Model: E |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | Delay / Jitter limit |  | Notes |
| AP1 | .256 Mbps |  | TCP | 64 |  |  |  | Clicking on web link? |

| STA Name: STA4 |  | Role: Internet file upload |  |  | Location: -4.5, 0.5 |  | Channel Model: D |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | Delay / Jitter limit |  | Notes |
| AP1 | 5Mbps |  | TCP | 1000 |  |  |  |  |

| STA Name: STA5 |  | Role: Internet file upload |  |  | Location: -1.5, 6 |  | Channel Model: D |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | Delay / Jitter limit |  | Notes |
| AP1 | 10Mbps |  | TCP | 1500 |  |  |  | Uploading large email attachments |

| STA Name: STA6 |  | Role: |  |  | Location: -5.5, 4.5 |  | Channel Model: D |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | Delay / Jitter limit |  | Notes |
| AP1 | 256 Mbps |  | TCP | 64 |  |  |  | Clicking on web link? |

| STA Name: STA7 |  | Role: Video Conferencing |  |  | Location: -9, -5 |  | Channel Model: E |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | Delay / Jitter limit |  | Notes |
| AP1 | 1Mbps |  | Constant, UDP | 512 |  | 100 / 50ms |  |  |

| STA Name: STA8 |  | Role: Video Conferencing |  |  | Location: -8.5, 8.5 |  | Channel Model: E |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | Delay / Jitter limit |  | Notes |
| AP1 | 1Mbps |  | Constant, UDP | 512 |  | 100 / 50ms |  |  |

| STA Name: STA9 |  | Role: |  |  | Location: 7, -7.5 |  | Channel Model: E |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | Delay / Jitter limit |  | Notes |
| AP1 |  |  |  |  |  |  |  | Is there a return channel for Enterprise IPTV or VoD? |

| STA Name: STA10 |  | Role: |  |  | Location: -3, 0.5 |  | Channel Model: D |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | Delay / Jitter limit |  |  |
| AP1 |  |  |  |  |  |  |  | Is there a return channel for Enterprise IPTV or VoD? |

| STA Name: STA11 |  | Role: |  |  | Location: -0.5,8 |  | Channel Model: D |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | Delay / Jitter limit |  | Notes |
| AP1 |  |  |  |  |  |  |  |  |

| STA Name: STA12 |  | Role: |  |  | Location: 7,7 |  | Channel Model: D |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | Delay / Jitter limit |  | Notes |
| AP1 |  |  |  |  |  |  |  |  |

| STA Name: STA13 |  | Role: |  |  | Location: -4,-4 |  | Channel Model: D |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | Delay / Jitter limit |  | Notes |
| AP1 |  |  |  |  |  |  |  |  |

| STA Name: STA14 |  | Role: |  |  | Location: 7.5,-1 |  | Channel Model: D |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | Delay / Jitter limit |  | Notes |
| AP1 |  |  |  |  |  |  |  |  |

| STA Name: STA15 |  | Role: |  |  | Location: 3,-0.5 |  | Channel Model: D |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | Delay / Jitter limit |  | Notes |
| AP1 |  |  |  |  |  |  |  |  |

| STA Name: STA16 |  | Role: |  |  | Location: 8,-6 |  | Channel Model: E |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | Delay / Jitter limit |  | Notes |
| AP1 |  |  |  |  |  |  |  |  |

| STA Name: STA17 |  | Role: |  |  | Location: 0,-7.5 |  | Channel Model: D |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | Delay / Jitter limit |  | Notes |
| AP1 |  |  |  |  |  |  |  |  |

| STA Name: STA18 |  | Role: |  |  | Location: 10,0.5 |  | Channel Model: E |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | Delay / Jitter limit |  | Notes |
| AP1 |  |  |  |  |  |  |  |  |

| STA Name: STA19 |  | Role: |  |  | Location: -2.5,-4.5 |  | Channel Model: D |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | Delay / Jitter limit |  | Notes |
| AP1 |  |  |  |  |  |  |  |  |

| STA Name: STA20 |  | Role: |  |  | Location: 0.5,-2 |  | Channel Model: D |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | Delay / Jitter limit |  | Notes |
| AP1 |  |  |  |  |  |  |  |  |

| STA Name: STA21 |  | Role: |  |  | Location: -6.5, -3 |  | Channel Model: D |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | Delay / Jitter limit |  | Notes |
| AP1 | 30Mbps |  | TCP | 1500 |  |  |  |  |

| STA Name: STA22 |  | Role: |  |  | Location: 0, -4.5 |  | Channel Model: D |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | Delay / Jitter limit |  | Notes |
| AP1 | 30Mbps |  | TCP | 1500 |  |  |  |  |

| STA Name: STA23 |  | Role: |  |  | Location: -1.5, 7 |  | Channel Model: D |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | Delay / Jitter limit |  | Notes |
| AP1 | 30Mbps |  | TCP | 1500 |  |  |  |  |

| STA Name: STA24 |  | Role: |  |  | Location: 3,2.5 |  | Channel Model: D |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | Delay / Jitter limit |  | Notes |
| AP1 | 30Mbps |  | TCP | 1500 |  |  |  |  |

| STA Name: STA25 |  | Role: VoIP Phone |  |  | Location: 3.5, -5 |  | Channel Model: D |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | Delay / Jitter limit |  |  |
| AP1 | 0.15 |  | Constant, UDP | 200 |  | 30 / 15 |  | VoIP Phone |

| STA Name: STA26 |  | Role: VoIP Phone |  |  | Location: 9,9.5 |  | Channel Model: E |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | Delay / Jitter limit |  |  |
| AP1 | 0.15 |  | Constant, UDP | 200 |  | 30 / 15 |  | VoIP Phone |

| STA Name: STA27 |  | Role: VoIP Phone |  |  | Location: -6,2.5 |  | Channel Model: D |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | Delay / Jitter limit |  |  |
| AP1 | 0.15 |  | Constant, UDP | 200 |  | 30 / 15 |  | VoIP Phone |

| STA Name: STA28 |  | Role: VoIP Phone |  |  | Location: -8,-5.5 |  | Channel Model: E |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | Delay / Jitter limit |  |  |
| AP1 | 0.15 |  | Constant, UDP | 200 |  | 30 / 15 |  | VoIP Phone |

| STA Name: STA29 |  | Role: VoIP Phone |  |  | Location: 1.5,3.5 |  | Channel Model: D |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | Delay / Jitter limit |  |  |
| AP1 | 0.15 |  | Constant, UDP | 200 |  | 30 / 15 |  | VoIP Phone |

| STA Name: STA30 |  | Role: VoIP Phone |  |  | Location: 9.5,3.5 |  | Channel Model: E |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | Delay / Jitter limit |  |  |
| AP1 | 0.15 |  | Constant, UDP | 200 |  | 30 / 15 |  | VoIP Phone |

## Scenario 5 (Conference Room)

| STA Name: AP1 |  | Role: AP |  |  | Location: 0,0 |  | Channel Model: |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | Delay / Jitter limit |  |  |
| STA 1-30 (replicated) | ?? |  |  |  |  |  |  | Netmeeting |
| STA 1-30 | 30Mbps |  | TCP | 1500 |  |  |  | Local File Transfer |
| STA 31-40 (replicated) | 0.15 |  | UDP | 200 |  | 30 |  | VoIP Phone |
| STA 41 | 5Mbps |  | UDP | 188 |  | 200 |  | STDV |

| STA Name: STA1 – STA30 |  | Role: laptop |  |  | Location: See table below |  | Channel Model: C |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | Delay / Jitter limit |  |  |
| AP1 | 50Mbps |  | TCP | 1500 |  |  |  | Printing |

| STA Name: STA31 - STA40 |  | Role: VoIP phone |  |  | Location: See table below |  | Channel Model: C |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | Delay / Jitter limit |  |  |
| VoIP Phone | 0.15 |  | UDP | 200 |  | 30 |  | VoIP Phone |

| STA Name: STA41 |  | Role: SDTV display |  |  | Location: See table below |  | Channel Model: C |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | Delay / Jitter limit |  |  |

Locations of the STA (m)

| STA | X | Y |
| --- | --- | --- |
| 1 | -3 | -3 |
| 2 | 5 | 10 |
| 3 | -4 | -2 |
| 4 | 3 | 2 |
| 5 | 6 | 0 |
| 6 | 8 | 0 |
| 7 | -9 | 6 |
| 8 | 14 | 9 |
| 9 | 6 | 0 |
| 10 | -12 | 5 |
| 11 | -4 | -11 |
| 12 | 2 | 10 |
| 13 | 5 | 15 |
| 14 | 14 | -13 |
| 15 | -4 | 1 |
| 16 | -7 | 3 |
| 17 | -14 | 2 |
| 18 | 6 | 14 |
| 19 | 8 | 7 |
| 20 | -2 | 4 |
| 21 | 9 | -12 |
| 22 | 13 | 12 |
| 23 | 3 | -7 |
| 24 | 11 | 0 |
| 25 | 7 | -2 |
| 26 | 14 | -13 |
| 27 | 2 | -6 |
| 28 | 11 | -5 |
| 29 | 5 | -13 |
| 30 | -4 | 0 |
| 31 | -2 | 2 |
| 32 | 3 | -12 |
| 33 | 12 | 8 |
| 34 | 9 | 9 |
| 35 | 5 | -9 |
| 36 | -7 | 4 |
| 37 | 1 | -13 |
| 38 | -12 | -7 |
| 39 | -3 | -1 |
| 40 | 12 | 3 |
| 41 | -5 | -1 |

TEMPLATE:

| STA Name: |  | Role: |  |  | Location: |  | Channel Model: D |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | Delay / Jitter limit |  |  |

## Scenario 6: Hot spot

NOTES

- Note that under channel model, we mark it as E/F. Channel model E can be used for STAs with coordinates in the range (X,Y) where X,Y < 10 in this scenario. Channel model F may be used for all other STAs

- Channel models E & F need to be revised to include high LOS conditions, as AP will generally be mounted on an elevation in such scenarios, providing high LOS

  - Channel E to be made LOS/NLOS

  - Channel F to be made LOS only – as

- This scenario may be used as a basis to construct the simulation model for Scenario 7 – Public Park/Outdoor Space and Arena by using Channel Model F (revised to be LOS – see 2a, above) and multiplying the location coordinates by a factor of 2.

| STA Name: AP1 |  | Role: AP |  |  | Location: 0,0 |  | Channel Model: E/F |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | Delay / Jitter limit |  |  |
| STA1-20 (replicated) | 1 |  | TCP | 300B |  |  |  | Internet File Transfer |
| STA21-24 (replicated) | 2 |  | UDP | 512B |  | 200ms |  | High quality streaming audio/video |
| STA25-30 (replicated) | 4 |  | UDP | 512B |  | 200ms |  | Mid quality streaming audio/video |
| STA31-32 (replicated) | 5 |  | UDP | 188B |  | 200ms |  | SDTV broadcast |
| STA33-52 (replicated) | 0.15 Mbps |  | Constant, UDP | 200B |  | 30 / 15 ms |  | VoIP |

Locations (m)

| STA | X | Y |
| --- | --- | --- |
| 1 | 21 | 23 |
| 2 | 21 | 20 |
| 3 | 21 | -16 |
| 4 | 15 | -5 |
| 5 | 13 | 1 |
| 6 | 9 | 17 |
| 7 | 7 | 6 |
| 8 | 5 | -21 |
| 9 | 2 | 9 |
| 10 | 0 | 20 |
| 11 | -1 | -5 |
| 12 | -5 | -15 |
| 13 | -7 | 3 |
| 14 | -9 | 12 |
| 15 | -11 | -5 |
| 16 | -12 | 22 |
| 17 | -15 | 6 |
| 18 | -20 | 13 |
| 19 | -23 | -2 |
| 20 | -23 | -21 |
| 21 | 13 | 19 |
| 22 | 13 | -19 |
| 23 | -16 | 16 |
| 24 | -18 | -17 |
| 25 | 16 | 2 |
| 26 | 10 | 10 |
| 27 | 3 | -10 |
| 28 | -1 | 6 |
| 29 | -5 | 15 |
| 30 | -15 | 0 |
| 31 | 4 | -1 |
| 32 | -6 | 0 |
| 33 | 21 | 22 |
| 34 | 21 | 0 |
| 35 | 19 | -11 |
| 36 | 19 | -22 |
| 37 | 15 | 10 |
| 38 | 8 | 21 |
| 39 | 9 | -5 |
| 40 | 7 | -14 |
| 41 | 4 | 12 |
| 42 | 2 | 3 |
| 43 | -2 | -22 |
| 44 | -5 | -9 |
| 45 | -8 | 18 |
| 46 | -9 | 5 |
| 47 | -14 | -18 |
| 48 | -15 | 12 |
| 49 | -17 | -8 |
| 50 | -23 | 23 |
| 51 | -23 | 10 |
| 52 | -24 | -15 |

## Scenario 7 (Public Park, Outdoor space, Warehouse)

#### See Comments preceding Scenario 6 (Hotspot) for generating this simulation scenario

## Scenario 8 (Point-Multipoint Backhaul)

TBD – do not have application traffic description

## Scenario 9 (Mixed-Mode BSS)

| STA Name: AP1 |  | Role: Mixed-mode AP |  |  | Location: 0,0 |  | Channel Model: D |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | Delay / Jitter limit |  |  |
| STA1 | 1 |  | TCP | 300B |  |  |  | Internet File Transfer |
| STA2 | 1 |  | TCP | 300B |  |  |  | Internet File Transfer |
| STA3 | 1 |  | TCP | 300B |  |  |  | Internet File Transfer |
| STA4 | 5 |  | UDP | 188 |  | 200 |  | SDTV |
| STA5 | 1 |  | TCP | 300B |  |  |  | Internet File Transfer |

| STA Name: STA1 |  | Role: Legacy STA |  |  | Location: -9,-12 |  | Channel Model: D |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | Delay / Jitter limit |  |  |

| STA Name: STA2 |  | Role: Legacy STA |  |  | Location: -19,10 |  | Channel Model: D |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | Delay / Jitter limit |  |  |

| STA Name: STA3 |  | Role: Legacy STA |  |  | Location: -2,17 |  | Channel Model: D |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | Delay / Jitter limit |  |  |

| STA Name: STA4 |  | Role: HT SDTV display |  |  | Location: -1,-3 |  | Channel Model: D |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | Delay / Jitter limit |  |  |

| STA Name: STA5 |  | Role: HT STA |  |  | Location: 14,1 |  | Channel Model: D |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | Delay / Jitter limit |  |  |

| STA Name: STA5 |  | Role: HT STA |  |  | Location: -12,7 |  | Channel Model: D |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | Delay / Jitter limit |  |  |
| AP1 | 1 Mbps |  | TCP | 300B |  |  |  | Internet File transfer |

## Scenario 11 (Co-channel legacy BSS)

| STA | X(m) | Y(m) |
| --- | --- | --- |
| 1 | -6 | 1 |
| 2 | 9 | -8 |
| 3 | 14 | 3 |
| 4 | -5 | 8 |
| 5 | 2 | -2 |
| 6 | 8 | 5 |
| 7 | 12 | 18 |
| 8 | 1 | 15 |
| 9 | -13 | 19 |
| 10 | -9 | -10 |
| 11 | 15 | 9 |
| 12 | -15 | -20 |

| STA Name: AP1 |  | Role: Legacy AP |  |  | Location: 0,0 |  | Channel Model: D |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | Delay / Jitter limit |  |  |
| STA1 – STA6 replicated | 1Mbps |  | TCP | 300B |  |  |  | Internet file transfer |

| STA Name: STA1 – STA6 |  | Role: Legacy STA |  |  | Location: See Table |  | Channel Model: D |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | Delay / Jitter limit |  |  |

| STA Name: AP2 |  | Role: HT AP |  |  | Location: 5,5 |  | Channel Model: D |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | Delay / Jitter limit |  |  |
| STA7 – STA12 | 1 Mbps |  | TCP | 300B |  |  |  | Internet file transfer |
| STA7 | 0.256Mbps |  | UDP | 512 |  | 200 |  | Internet Streaming audio |
| STA8 | 0.256Mbps |  | UDP | 512 |  | 200 |  | Internet Streaming audio |
| STA9 | 5 Mbps |  | UDP | 188 |  | 200 |  | SDTV |

| STA Name: STA7 – STA12 |  | Role: HT STA |  |  | Location: See table |  | Channel Model: D |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | Delay / Jitter limit |  |  |

## Scenario 14 (Point-point wireless backhaul)

TBD – do not have application traffic description

## Scenario 15 (Arena Video Broadcast)

| STA Name: AP |  | Role: HT AP |  |  | Location: |  | Channel Model: E |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | Delay / Jitter limit |  |  |
| STA1 – STA50 | 5Mbps |  | UDP | 512 |  | 200 |  | SDTV |

| STA | X(m) | Y(m) |
| --- | --- | --- |
| 1 | -8 | 6 |
| 2 | -9 | -1 |
| 3 | -17 | 20 |
| 4 | 3 | -3 |
| 5 | 1 | -7 |
| 6 | -3 | -11 |
| 7 | 3 | 10 |
| 8 | 1 | 6 |
| 9 | -12 | -5 |
| 10 | 11 | 7 |
| 11 | -2 | 3 |
| 12 | 12 | -18 |
| 13 | 4 | -18 |
| 14 | -3 | -8 |
| 15 | 15 | -19 |
| 16 | 11 | 19 |
| 17 | 20 | 12 |
| 18 | -2 | 0 |
| 19 | -11 | 6 |
| 20 | -7 | 18 |
| 21 | 9 | -4 |
| 22 | 10 | -9 |
| 23 | -2 | 17 |
| 24 | 7 | -11 |
| 25 | 14 | 5 |
| 26 | -15 | -12 |
| 27 | 4 | 5 |
| 28 | -5 | 3 |
| 29 | -2 | -18 |
| 30 | -19 | -7 |
| 31 | -19 | -5 |
| 32 | 7 | -16 |
| 33 | -19 | 4 |
| 34 | 4 | -19 |
| 35 | -19 | -12 |
| 36 | 3 | -18 |
| 37 | -5 | 5 |
| 38 | 9 | 8 |
| 39 | -17 | -2 |
| 40 | -2 | -6 |
| 41 | -14 | 7 |
| 42 | 8 | 9 |
| 43 | -1 | 2 |
| 44 | -15 | -2 |
| 45 | 9 | 16 |
| 46 | -9 | -10 |
| 47 | 15 | -11 |
| 48 | 12 | 16 |
| 49 | -11 | -10 |
| 50 | -18 | -17 |

| STA Name: STA1 – STA50 |  | Role: Broadcast display terminal |  |  | Location: See table |  | Channel Model: E |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | Delay / Jitter limit |  |  |

# Comments to carry forward

Comments from the committee members to carry forward in definition of the simulation scenarios:

(Javier) I believe it is important if we can differentiate HT scenarios from other technologies, such as 802.15.3a. For example, the residential or small enterprise scenarios are similar to the ones proposed in the 15.3a requirements document. I suggest considering this differentiation when defining the topology))

(Paul) The Usage models must also include some of the assumed mobility and ad hoc requirements identified.

# References

[1]	11-03-364r0, Application Characteristics for HT Usage Scenarios,  Javier del Prado et al., Philips

[2]	11-03-489r0, Home Usage Model for HT WLAN Systems, Chiu Ngo et al., Samsung

[3]	11-03-327r0 A Proposed Usage Model Methology for the High Throughput Task Group, Adrian Stephens, Intel

[4]	11-03-354, Usage Model Special Committee Cumulative Minutes, Adrian Stephens, Intel

[5]	11-03-161, Indoor MIMO WLAN Channel Models, Vinko Erceg. Zyray Wireless et al.

Note, this corresponds to a loss of a 1024B MSDU per hour.   The TS PDU PLR is higher than this.  It is not known what is the effect to the decoder of giving it burst packet losses.

Note, a PLR of 10^-7 will not be measurable in our simulation technologies.

11File Transfer at HT at the minimum figure in the PAR should be a requirement for these applications. We may want to emphasize the differentiation between regular FTP and “Broadband FTP” (e.g. very short download time for audio files, “large” email message instantaneous sync, enterprise-class FTP etc)

The usage model special committee didn't get to vote on this.