IEEE P802.11
Wireless LANs

Group Key Optimization

Date:	July 1, 2003

Author:	Tim Moore
	Microsoft
	1 Microsoft Way, Redmond, WA, 98052
	Phone: 1-425-703-9861
	Fax: 
	e-Mail: timmoore@microsoft.com

Abstract

Summary

Allow the GTK to be added to message 3 of the 4-way handshake. This has the following advantages:

- Optimizates the number of round trips needed to do the key handshake

- Removes the key synchronization problem of installing Pairwise keys and receiving Group update messages

- Removes the delay in key handshake due to the need to synchronize the sending of group key updates to the Pairwise install

- Removes the synchronization issue in IBSS of installing Pairwise keys while the other STA is still completeing its 4-way handshake.

Section 8.5.2

Replace Figure 64 with

| Descriptor Type – 1 octet |  |
| --- | --- |
| Key Information – 2 octets | Key Length – 2 octets |
| Key Replay Counter – 8 octets |  |
| Key Nonce – 32 octets |  |
| EAPOL-Key IV – 16 octets |  |
| Key RSC – 8 octets |  |
| STA MAC Address – 6 octets |  |
| GTK Length -2 octets |  |
| Key MIC – 16 octets |  |
| Key Data Length – 2 octets | Key Data – n octets |

Replace text for EAPOL Key ID with

- EAPOL Key ID (bits 4 and 5): specifies the IEEE 802.11 Key ID of the temporal key of the key derived from the message. The value of this shall be the key id of the GTK.

Group keys shall not use Key ID 0, except in a TSN. This means that key IDs 1 to 3 are available to be used to identify Group keys. This document recommends that implementations reserve key IDs 1 and 2 for Group Keys, and that key ID 3 is not used.

- Secure (bit 9): this bit is set once the initial key exchange is complete. That is, the secure bit in the EAPOL-Key frame is used to inform when the pairwise key exchange is complete and the first Group Key Handshake is complete. It shall be initialized to 0 (not secure) at the beginning of any 4-way handshake.

To

- Secure (bit 9): this bit is set once the initial key exchange is complete. That is, the secure bit in the EAPOL-Key frame is used to inform when the pairwise key exchange is complete and the first GTK has been transferred. It shall be initialized to 0 (not secure) at the beginning of any 4-way handshake.

The Supplicant and Authenticator initialize the secure bit to zero. The Authenticator sets the secure bit when it sends the first Group EAPOL-Key frame to the Supplicant and the Supplicant sets the secure bit on receiving the first Group EAPOL-Key frame. The Supplicant clears the secure bit on receiving a TKIP integrity error from the MAC or on receiving an EAPOL-Key frame with the secure bit cleared. The Authenticator clears the secure bit on receiving a TKIP integrity error from the Supplicant or from its STA.

To

The Supplicant and Authenticator initialize the secure bit to zero. The Authenticator sets the secure bit when it sends the first GTK to the Supplicant and the Supplicant sets the secure bit on receiving the first EAPOL-Key frame after receiving the GTK. The Supplicant clears the secure bit on receiving a TKIP integrity error from the MAC or on receiving an EAPOL-Key frame with the secure bit cleared. The Authenticator clears the secure bit on receiving a TKIP integrity error from the Supplicant or from its STA.

Replace Key RSC text before table 8 with

Key RSC. This field is eight (8) octets in length. It contains the receive sequence counter (RSC) for the key being installed in IEEE 802.11. It is only used in message 3 of the 4-way handshake and the first message of the Group key update, where it is used to synchronize the IEEE 802.11 replay state. The Key RSC is for the Group key. It shall contain 0 in other messages. If the Key RSC is less than eight octets in length the remaining octets shall be set to 0. The least significant octet of the TSC or PN shall be in the first octet of the Key RSC.

Informative Note: The Key RSC for TKIP is the TSC in the first 6 octets and for CCMP is the PN in the first 6 octets.

Table 1—Key RSC TSC Table

| KeyRSC 0 | KeyRSC 1 | KeyRSC 2 | KeyRSC 3 | KeyRSC 4 | KeyRSC 5 | KeyRSC 6 | KeyRSC 7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| TSC0 | TSC1 | TSC2 | TSC3 | TSC4 | TSC5 | 0 | 0 |
| PN0 | PN1 | PN2 | PN3 | PN4 | PN5 | 0 | 0 |

For WEP, the key RSC value shall be set to 0 on transmit, and shall not be used at the receiver.

Replace EAPOL Key ID with

### Key GTK Length

This field is two octets in length, taken to represent an unsigned binary number. It is 0 except for Message 3. The value defines the length of the Group key in octets to be configured into the IEEE MAC not the encrypted GTK length. For example, a value of 32 in this field indicates a 256 bit key. In Message 3 the GTK is encrypted; according to the Key Descriptor Version; and is added at the end of the EAPOL-Key Data field.

### Replace Key Data text with

Key Data.

4-way message 3: This field contains one or two RSN IEs and an encrypted GTK.

The Key Data Length is set to the length of the Data field.

The RSN IEs are from and including the RSN IE id. An AP shall insert the second RSN IE only to indicate the pairwise key cipher suite the STA must use when the STA selects an enabled pairwise key cipher suite that policy disallows this particular STA and must be one of the ciphers advertised by the Authenticator.  All other fields in the second RSN IE must be identical to the first RSN IE.

The RSN IEs shall not be not be encrypted. The Authenticator shall insert the RSN IE it sent in its Beacon or Probe Response. The Supplicant shall bit-wise compare the required RSN IE against the RSN IE received in the Beacon or Probe Response. If the second optional RSN IE is present, the STA shall either use that cipher suite with its pairwise key, or it shall disassociate. When both Beacon and Probe Response RSNIEs are received by the STA, the Probe Response RSN IE shall be used for the bit-wise compare. In either case, if the values do not match, then the receiver shall consider the RSN IE modified and shall use the MLME-DEAUTHENTICATE.request primitive to break the association. A security error should be logged at this time.

The encrypted GTK is included and the GTK Length contains the unencrypted length of the GTK. The GTK will be encrypted as specified by the Key descriptor version.

Section 8.5.5.2, Replace StaProcessEAPOL-Key with:

StaProcessEAPOL-Key (S, M, A, T, N, K, D, RSC, ANonce, GTK Length, MIC, GTK)

{

TPTK  PTK

TSNonce  0

PRSC  0

UpdatePTK  0

State  UNKNOWN

if M = 1 then

if Check MIC(PTK, EAPOL-Key message) fails then

State  FAILED

else

State  MICOK

endif

endif

if K = P && D = 0 then

if State != Failed  then

if PSK exists then – PSK is a pre-shared key

PMK  PSK

else

PMK  Master Session Key from 1X

endif

TSNonce  SNonce

if ANonce != PreANonce then

TPTK  Calc PTK(PMK, ANonce, TSNonce)

PreANonce  ANonce

endif

endif

if State = MICOK then

PTK  TPTK

UpdatePTK  T

if UpdatePTK = 1 then

if GTK Length = 0 then

PRSC  RSC

endif

if Set PTK(0, TRUE, PRSC, PTK) fails then

invoke MLME-DEAUTHENTICATE.request

endif

MLME.SetProtection.Request(TA, Rx)

endif

if GTK Length != 0 then

if (GTK[N]  Decrypt GTK) succeeds then

if Set GTK(N, 0, RSC, GTK[N]) fails then

invoke MLME-DEAUTHENTICATE.request

endif

else

State  FAILED

endif

endif

endif

else if K = G && D = 0 then

if State = MICOK  then

if (GTK[N]  Decrypt GTK) succeeds then

if Set GTK(N, T, RSC, GTK[N]) fails then

invoke MLME-DEAUTHENTICATE.request

endif

else

State  FAILED

endif

else

State  FAILED

endif

else if D = 1 then // DL key

if State = MICOK  then

if (DLK  Decrypt DLK) succeeds then

if Set DLK(0, 1, RSC, DLK) fails then

invoke MLME-DEAUTHENTICATE.request

endif

else

State  FAILED

endif

else

State  FAILED

endif

endif

if A = 1 && State != Failed then

Send EAPOL(0, 1, 0, 0, 0, K, 0, TSNonce, 0, 0, MIC(TPTK), 0)

endif

if UpdatePTK = 1 then

MLME.SetProtection.Request(TA, Tx_Rx)

endif

if State = MICOK && S = 1 then

MLME.SetProtection.Request(TA, Tx_Rx)

802.1X::portValid = TRUE

endif

}

Add after the StaProcessEAPOL-Key pseudo code the following text:

When processing message 3, the GTK is decrypted from the EAPOL-Key message and installed. The PTK shall be installed before the GTK.

Section 8.5.6 replace figure 69 with:

Add to end of PTKINITNEGOTIATING text:

When Message 3 of the 4-way handshake is sent in state PTKINITNEGOTIATING, the encrypted GTK shall be sent at the end of the data field and the GTK length is put in the GTK Length field.

Delete text from section 8.5.6.2

PInitAKeys – This variable is set to TRUE when the Authenticator is ready to send a Group key to its Supplicant after initialization.

Section 5.9.2

Add

e. Transport the GTK from Authenticator to Supplicant.

Section 5.9.3.2

For stations S2 and S3 to decrypt multicast/broadcast frames from S1, B1 must be sent to S2 and S3. This is done using the Group Key Handshake. S1 initiates the Group Key Handshake with S2 and S3.

To

For stations S2 and S3 to decrypt multicast/broadcast frames from S1, B1 must be sent to S2 and S3. This is done using the 4-way handshake or the Group Key Handshake. S1 initiates the Group Key Handshake with S2 and S3.

In a similar manner S3 needs to complete 4-way and Group Key Handshakes with S1 and S2, to deliver B3 to S1 and S2.

To

In a similar manner S3 needs to complete 4-way and optionally the Group Key Handshakes with S1 and S2, to deliver B3 to S1 and S2.

In total there are six 4-way handshakes and six group key updates, or N(N–1) exchanges among N STAs.

To

In total there are six 4-way handshakes and up to six group key updates, or N(N–1) exchanges among N STAs.

The Group Key Handshake sends the broadcast/multicast keys to the correct STAs. The 4-way handshake is used to derive the unicast key. However, since in IBSS there are two 4-way handshakes between any two STAs, the unicast key used between any two STAs is from the 4-way handshake initiated by the STA with the lowest MAC address.

To

The Group Key Handshake can be used to send the broadcast/multicast keys to the correct STAs. The 4-way handshake is used to derive the unicast key and optionally to send the broadcast/multicast key to the correct STAs. However, since in IBSS there are two 4-way handshakes between any two STAs, the unicast key used between any two STAs is from the 4-way handshake initiated by the STA with the lowest MAC address.

Section 5.9.3.3

Error: Reference source not found depicts the usage of the 4-way and Group Key Handshake when a STA joins an IBSS. An IBSS that is configured to use a pre-shared key will only use the 4-way and Group Key Handshakes. An IBSS that is configured to use IEEE 802.1X will need to use IEEE 802.1X and EAP to generate a PMK between each pair of STAs before the 4-way and Group Key Handshakes may be started.

To

Error: Reference source not found depicts the usage of the 4-way and optionally the Group Key Handshake when a STA joins an IBSS. An IBSS that is configured to use a pre-shared key will only use the 4-way and optionally the Group Key Handshakes. An IBSS that is configured to use IEEE 802.1X will need to use IEEE 802.1X and EAP to generate a PMK between each pair of STAs before the 4-way and Group Key Handshakes may be started.

Section 8.4.1

In an IBSS each STA defines its own group key to secure its broadcast/multicast transmissions. After establishing a security association, each STA shall use the Group Key Handshake to distribute its transmit Group Key to its new peer STA.

To

In an IBSS each STA defines its own group key to secure its broadcast/multicast transmissions. After establishing a security association, each STA shall use either the 4-way handshake or the Group Key Handshake to distribute its transmit Group Key to its new peer STA.

Section 8.4.8

Add

4. Optionally, transfer GTK form Authenticator to Supplicant

Section 8.4.9

Each Authenticator also uses the PTK negotiated by the exchange it initiates to distribute its own Group Transient Key. Each Authenticator generates its own Group key, and uses the Group Key handshake to transfer the GTK to other STAs with whom it has completed a 4-way handshake.

To

Each Authenticator also uses the PTK negotiated by the exchange it initiates to distribute its own Group Transient Key. Each Authenticator generates its own Group key, and uses the 4-way handshake or the Group Key handshake to transfer the GTK to other STAs with whom it has completed a 4-way handshake.

Section 8.5.3.3

Message 3 uses the following values for each of the EAPOL-Key frame fields

Descriptor Type = N – See Clause 8.5.2

Key Information.

Version = 1 (RC4 encryption with HMAC-MD5) or 2 (NIST AES key wrap with HMAC-SHA1-128) – same as Message 1

Key Type = 1 (Pairwise) – same as Message 1

Key Index = GTK Key ID

Install = 0/1 – 0 only if AP does not support key mapping keys

Key Ack = 1

Key MIC = 1

Secure = 1

Error = 0 – same as Message 1

Request = 0 – same as Message 1

Reserved = 0 – unused by this protocol version

Key Length = Cipher suite specific; see Error: Reference source not found.

Key Replay Counter = n+1

Key Nonce = ANonce – same as Message 1

Key IV = 0 – unused by the 4-way handshake

Key RSC = starting sequence number Authenticator’s STA will use in packets protected by GTK

GTK Length = Cipher suite specific

Key MIC = MIC(KCK, EAPOL) – MIC computed over the body of this EAPOL-Key frame with the Key MIC field first initialized to 0.

Key Data Length = length in octets of included RSN IEs

Key Data = the AP's Beacon/Probe RSN IE, and, optionally, a second RSN IE that is the Authenticator’s unicast cipher suite assignment and an encrypted GTK.

Section 8.5.3.5

- The Authenticator sends an EAPOL-Key frame containing ANonce, the RSN IE from its Beacon or Probe Response messages, MIC, whether to install the temporal keys and an encrypted GTK

Section 5.92

Change caption to “Establishing PTK and GTK”

Change Figure 3

Add GTK to message 3 parameters

Remove “for STA Unitcast” text

Delete Figure 4

Delete text “See Figure 4”

Change Figure 5

Delete arrows and text for group key update

Change figure 6

Delete arrows and text for group key update

Change figure 7

Delete arrows and text for group key update

Change figure 37