IEEE P802.11
Wireless LANs

Wireless Multimedia Enhancements (WME)

Date:	20th February 2004

Primary Author:

Thomas Kuehnel

Microsoft Corporation

One Microsoft Way

Redmond, WA 98052

Tel: (425) 706-4351

Fax: (425) 936-7329

Email: tkuehnel@microsoft.com

Contributions from:

Simon Barber, Instant802

Mark Bilstad, Cisco Systems

Greg Chesson, Atheros Communications

Steve Emeott, Motorola

Matthew Fischer, Broadcom

Garth Hillman, AMD

Wayne King, Microsoft

Duncan Kitchin, Intel

Thomas Kuehnel, Microsoft

Richard van Leeuwen, Agere Systems

Partho Mishra, Airgonetworks

Andrew Myles, Cisco Systems

Ivan Oaks, STMicroelectronics

Stephen Palm, Broadcom

Sid Schrum, Texas Instruments

Fred Stivers, Texas Instruments

Menzo Wentink, GlobespanVirata

Tim Wilson, Motorola

Abstract

This submission documents an effort by the authors to create an unambiguous, implementable and interoperable specification compatible with 802.11 TGe drafts.  It defines a Phase 1 WME which is viewed as a guideline for implementors in advance of the availability of a final TGe standard.

Contents

1	Overview	6

1.1	Purpose of This Document	6

1.2	Terms and Definitions	6

1.3	WME Features	6

2	WME Frame Formats	8

2.1	Data Frame Formats	8

2.1.1	Fields	8

2.1.2	Frame Control Field	8

2.1.3	Duration Field	9

2.1.4	Addresses	9

2.1.5	Sequence Control Field	9

2.1.6	QoS Control Field	9

2.2	Management Frame Formats	10

2.2.1	WME Information Element	10

2.2.2	WME Parameter Element	11

2.2.3	Beacon Frame	13

2.2.4	Probe Request Frame	13

2.2.5	Probe Response Frame	13

2.2.6	Association Request Frame	13

2.2.7	Association Response Frame	14

2.2.8	Management Notification Frame	14

2.2.9	WME TSPEC Element	15

3	WME Protocol Specification	19

3.1	Association and Capability Negotiation	19

3.1.1	Procedure at an AP	19

3.1.2	Procedure at a STA in an Infrastructure Network	19

3.1.3	Procedure at a STA in an IBSS	19

3.2	Setting of WME Parameters	20

3.2.1	Default WME parameters	20

3.2.2	WME Parameters in an Infrastructure Network	20

3.2.3	WME Parameters in an IBSS	20

3.3	Assignment of Frames to Queues	20

3.3.1	Mappings for Unicast Frames	20

3.3.2	Mappings for Received Unicast Frames at an AP	21

3.3.3	Mappings for Group Addressed and Buffered Frames at an AP	21

3.4	Channel Access Protocol	22

3.4.1	Reference Implementation	22

3.4.2	Transmit Opportunities & TXOP Limits	22

3.4.3	Obtaining an EDCA TXOP	23

3.4.4	Obtaining a Continuation of TXOP	25

3.4.5	Backoff Procedure	25

3.4.6	Retransmit Procedures	26

3.5	Admission Control Procedures	26

3.5.1	Procedures at the AP	27

3.5.2	Procedure at STAs	27

3.6	WME Power -save (PS) Procedures	28

3.6.1	AP Operation	29

3.6.2	STA Operation	29

WME (Informative) Recommended Practices Annex	30

References	34

Revision History

| Date | Clause | Comment |
| --- | --- | --- |
| 23 Jul 03 |  | Final draft |
| 24 Jul 03 | 3.4.5 | (1, CW[AC]+1) changed to (0,CW[AC]) |
| 22 Nov 03 | 2.1.2 2.2.9 3.2.1 3.3.1 3.6 App | Added Figure 2 Frame Control Field to improve readability TSpec suspension interval added, PSB bit added to TSinfo, table and text Table 12 TXOPlimit adjusted Table 13 mapping of 802.1D (EE) changed to AC_BE Power Save added Table 14 TXOP limit adjusted |
| 4 Jan 04 | 2.1.6 2.1.6 2.2.2 3.1.1, 3.1.2 3.2.2 3.3.1 3.4.3-3.4.5 3.4.7 / 3.5 3.5.2 | 802.1D replaced by TID; added that lower 2 bits correspond to AC, modified note wrt end-2-end signaling of priority Downgrading sentence removed Cleanup, moved paragraph pertaining to EDCF parameters to 3.2.2 Multiple beacon specified: all beacons within 2 or more DTIM periods Mapping of received unicast frames at an AP removed (bridging..) Updated from draft 6.0 Made Admission control heading 2 Dynamic downgrading deleted |
| 5 Jan 04 | 3.4.4 2.1.5 2.1.6 3.3.2 | Removed sentence “intention TXOP by setting Duration ID..” introduced 4/1 Added clarification of using mapping table for referencing AC Added clarification of using mapping table for referencing AC, undone change Undone removal of Mapping of received unicast frames at an AP |
| 6 Jan 04 | 1.2 2.1.6 3.3.1 2.2.9 3.5.2 Misc Figure 16 | TID added Stated TID = 802.1D Tag priority information Clarified TID = 802.1D priority TID is reference and 802.1D is included for compatibility Added TID in Response references AC using mapping of Table 13 802.1D Tag -> 802.1D priority information Added TID = 802.1D prio; added sequence number counter |
| 8 Jan 04 | 3.4.3 3.5 3.5.1 3.5.2 | Included corrections from A.Myles, M. Fischer Moved paragraphs (medium, used time) from 3.5.2, STA –>channel access Added rule for AP dropping frames for admitted traffic Made it clear that admission is per AC using TID mapping of Table 13 |
| 14 Jan 04 | Annex 2.1.6 3.4.2 3.5 2.2.2 | Added WPA WME section Added TID of each MPDU shall be the same Alignment with tge; 256 byte limit removed txop limit = 0 -> MSDU + RTS/ AC parameters not necessarily ACs ACM text to use AC Parameter as opposed to physical AC |
| 31 Jan 04 | Annex 2.1.6 2.2.9 3.3.1 3.6 | Added Downgrading section to Use of Admission Control Bit 4 in QoS control field designated as EOSP bit and added TID references flow in TSPEC, Figure 14 (TS Info field) Bit 10: PSB, Bit 16 reserved, added that PS behavior by flow and not STA Added TID field in MPDU shall be used to re-create the 802.1D priority PS Text modified, More Data, EOSP, Power-save behavior per AC Added text to define behavior for new admission over existing admission |
| 2 Feb 04 | 3.6.1 Annex | Added: AP's not required to support more than one APSD flow per STA Added Conveyance of VLAN Tags with encapsulation |
| 4 Feb 04 | 3.5 | Changed new TSPEC Request modifies previous TSPEC, AP aggregates Flow definition per AC referenced by TID |
| 5 Feb 04 | Various 3.5 | Replaced some instances of TID with UP to maintain terminology alignment with [2] Added: There are four flows per AC. Limit the number of required TSPEC supported by AP and STA |
| 6 Feb 04 | 2.2.9 3.5 3.6; 3.6.1 Annex | STA may send TSPEC even for ACs that do not mandate ACM for power save Moved behavior of powersave to 3.6 Flow definition revised Mapping clarified; APSD -> admitted power save flow Conveyance of VLAN tags cleanups |
| 9 Feb 04 | Annex 2.2.9 | Text tweaking on VLAN; AP advertises parameter according Table 12 Aligned TSPEC element ordering with [5] |
| 10 Feb 04 | 2.2.9 3.5 Annex | Replaced 802.1D in TS info field with UP and sentence stating purpose, TID from TSpec to reference flow Removed paragraph wrt. VLAN nesting rules, i.e. single VLAN Tag |
| 12 Feb 04 | 3.6 | Major rewrite of section |
| 16 Feb 04 | Annex | Added Section: TKIP Replay Counters |
| 17 Feb 04 | 3.5.1 | Added: if ACM flag is set to 0 for an AC the AP is not required to respond to a TSPEC request. |
| 20 Feb 04 | 2.2.9 3.4.3 | Media Time -> Medium Time; removed The contention based access flag is always…”; removed …for an admitted TSPEC reservation” for powersave Alignments with draft 7 ; Figure 17 updated |
| 24 Feb 04 | 3.2.2 2.2.2 | the AIFSN value for APs should be greater/equal than 1 to align with 9.9.1.3 Changed minimum value for AIFSN from 1 to 2 to align with 9.9.1.3 |

# Overview

## Purpose of This Document

This document defines the specification for WME, an 802.11 quality of service (QoS) implementation based on a subset of the draft 802.11e standard supplement [2]. It is motivated by the need to prevent market fragmentation caused by multiple, non-interoperable pre-standard subsets of the draft 802.11e standard that would otherwise occur. It is intended that WME can be implemented, subjected to interoperability testing and deployed in the market before the availability of 802.11e. This is facilitated by selecting a subset of the features of 802.11e. It in no way should be taken to detract from 802.11e itself, which is viewed as the long term endpoint of WME. Deployment of WME will deliver useful QoS functionality for streaming media and also provide key lessons which will benefit eventual deployment of 802.11e.

## Terms and Definitions

AC (Access category): A label for a set of MSDUs for which a set of EDCA parameters are used for contending the channel.  WME defines 4 ACs.

Admitted AC: Traffic transmitted using an AC based on parameters in a WME TSPEC element contained in a Setup response frame

AIFS: Arbitration Inter Frame Spacing

AIFSN: Arbitration Inter Frame Spacing Number

ECWmin: Exponent form of CWmin

ECWmax: Exponent form of CWmax

EDCA: Enhanced Distributed Channel Access

EDCF: Enhanced Distributed Coordination Function

Group traffic: Multicast and broadcast traffic

TID: Traffic Identifier; a 4-bit number that uniquely identifies a TSPEC.

TXOP (Transmission Opportunity): An interval of time when a particular WME STA has the right to initiate transmissions onto the wireless medium (WM).

Un-admitted AC: Traffic transmitted using an AC based that did not require admission.

UP: User Priority; is identical to the 3 bit priority subfield carried in the 802.1D Priority field.

## WME Features

The features supported by WME in this phase are as follows:

- Capability negotiation independent of 802.11e. That is, WME devices will not advertise 802.11e capability unless they also support those features independently. This is part of a forward compatibility strategy which is described in detail in a subsequent paragraph.

- Frame formats and over the air protocols will be based on those currently proposed for 802.11e. However, no attempt will be made to track future changes in 802.11e and reflect them back into the WME specification. Divergence between the two specifications is a necessary side effect of the need to freeze the WME specification as soon as possible.

- WME will use an EDCF mechanism only, and except where explicitly indicated otherwise in this specification other 802.11e features, including HCF polling and associated signaling, Block Acknowledgement, and side traffic, are not part of WME.

- Interfaces to the MAC which signal per-packet priority will be consistent with those used for Ethernet, both in terms of the driver API and bridging to other 802 link layers via an 802.1D bridge.

- The number of exposed queues will be fixed at four.  A fixed mapping of priority information carried in the 802.1D Priority field to those four queues will be defined, together with suggested uses for each priority consistent with the suggested uses in 802.1D.

Capability negotiation is designed to permit ultimate forward compatibility with 802.11e, taking into account the fact that the formats used for QoS data frames cannot be assumed to remain consistent. It is important feature that, on receipt of a frame, it is possible to uniquely decode it.

An AP or STA may support both WME and 802.11e. Only one may be in use for a specific association at any time, but an AP may permit both 802.11e and WME associations from different STAs at the same time.

A WME-only AP or STA does not set the “QoS” bit in the capability field of association, beacon and probe management frames. A new WME Information Element is defined in this specification and is carried in those frames. An AP may support and advertise both 802.11e and WME in probe responses and beacons, but both association requests and responses must only request or specify one of these capabilities. As a result, a given association is either WME or 802.11e, but not both, and this defines how data frames with QoS subtypes must be interpreted, should they ultimately be different.

# WME Frame Formats

## Data Frame Formats

### Fields

Data, Control and Management frames are indicated by a type subfield in the frame control field, as defined for [1]. Data frames include additional WME-specified subtypes and conditional fields.

The general frame format for data type frames is shown in Figure 1.

| 2 | 2 | 6 | 6 | 6 | 2 | 6 or 0 | 2 or 0 | n | 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frame Control | Dur | Address 1 | Address 2 | Address 3 | Seq Control | Address 4 | QoS Control | Body | FCS |
| MAC Header |  |  |  |  |  |  |  |  |  |

Figure 1 WME QoS Data Frame Format

The Address 4 and QoS control fields are conditionally present in the MAC header, determined by values in the frame control field. The Address 4 field is present if and only if both toDS and fromDS bits are set in the frame control field (see 2.1.2). The QoS control field is present if and only if the frame is of subtype QoS data or QoS null.

### Frame Control Field

There are no changes to the definition of the frame control field division into subfields. Subfield allocations are shown in Figure 2 and are identical to [1].

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 |  | 7 | 4 |  | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Order | WEP | More Data | Pwr Mgmt | Retry | More Frag | From Ds | To DS | Subtype |  |  | Type |  |  | Version |  |

Figure 2 Frame Control Field [1]

Additional data subtypes are defined for WME as shown in Table 1.

Table 1 Additional WME Data Subtype Codes

| Type (MSB-LSB) | Subtype (MSB-LSB) | Subtype Description |
| --- | --- | --- |
| 10 | 1000 | QoS data |
| 10 | 1100 | QoS null |
| 00 | 1101 | Management Action |

### Duration Field

The definition of the duration field is unchanged from [1] aside from the addition of the following provision:

When transmitting bursts of multiple MSDUs using continuation TXOPs, the duration field of a data or management frame in that burst may be selected to protect, using the NAV mechanism, either:

- only the acknowledgement frame, if present, or

- the acknowledgement frame, if present, plus the following data or management frame and its expected acknowledgement, if present or

- the entire burst of frames.

### Addresses

The definition and interpretation of address fields is unchanged from [1].

### Sequence Control Field

The sequence control field contains two subfields, the sequence number and fragment number.  Definition and use of the fragment number is unchanged from [1].  A sequence number is maintained separately for each Access Category and non-QoS traffic including management frames and group traffic.

| 15 | 4 | 3 | 0 |
| --- | --- | --- | --- |
| Sequence Number |  | Fragment Number |  |

Figure 3 Sequence Control Field

Sequence and fragment numbers shall be selected and inserted on the initial transmission attempt of each data or management frame. Any subsequent retransmissions shall use the same sequence control field as the first transmission attempt.

The sequence number is selected by the transmitter from a modulo-4096 counter, which shall be incremented by 1 for each unique MSDU or MMPDU that is queued to the associated AC for delivery.

In addition to the RA and TA of the frame, the receiver shall use the UP field that is carried in the QoS Control field in order to allow correct differentiation of the sequence number spaces among frames sent using different ACs.  The receiver shall use the mappings in Table 13 to identify the sending AC from the received UP field.

### QoS Control Field

The QoS control field consists of two octets and is shown in Figure 4.

| 15 | 7 | 6 | 5 | 4 | 3 | 2 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 |  | ack policy |  | EOSP | 0 | UP |  |

Figure 4 QoS Control Field

The three bit UP field carries the priority bits of the 802.1D Priority and is used to signal the priority for this frame.  It also implies the sending AC according to the mappings in Table 13. The UP for each MPDU of a MSDU shall be the same value.

The ack policy field specifies the expected acknowledgement response and contains one of the values shown in Table 2. All other bits are reserved and shall be set to zero on transmission and ignored on receipt.

Table 2 Ack Policy Field Values

| Ack Policy Value | Meaning |
| --- | --- |
| 00 | Acknowledge |
| 01 | Do not Acknowledge |

The End of Service Period (EOSP) field is set to 1 at the end of a triggered service period (SP), and is set to 0 otherwise.

## Management Frame Formats

### WME Information Element

The WME Information Element indicates capability or use of WME according to context and is shown in Figure 5. The fields contained in the WME Information Element are listed in Table 3.  The WME Information Element contains a version number, which is 1 for version 1.0 of the WME specification. The WME Information Element contains an QoS Info field.

| Octets:1 | 1 | 3 | 1 | 1 | 1 | 1 |
| --- | --- | --- | --- | --- | --- | --- |
| Element ID | Length | OUI | OUI Type | OUI Subtype | Version | QoS Info field |

Figure 5 WME Information Element

Table 3 WME Information Element Field Values

| Field | Value |
| --- | --- |
| Element ID | 221 |
| Length | 7 |
| OUI | 00:50:f2 (hex) |
| OUI Type | 2 |
| OUI Subtype | 0 |
| Version | 1 |
| QoS Info field | See Figure 6 |

The QoS Info field is shown in Figure 6. The QoS Info field contains the Parameter Set Count, which is initially arbitrary and is incremented each time any of the AC parameters changes.

| 7 | 4 | 3 | 0 |
| --- | --- | --- | --- |
| Reserved |  | Parameter Set Count |  |

Figure 6 QoS Info field

The reserved bits are set to 0 upon transmission and should be ignored upon reception. The Parameter Set Count is set to “0” in the association request frames and ignored upon reception.

### WME Parameter Element

The WME Parameter Element contains a set of parameters (EDCA parameters) for the EDCF channel access protocol and is shown in Figure 7. The fields contained in the WME Parameter Element are listed in Table 4.  The WME Parameter Element contains a QoS Info field. The format of the QoS Info field is shown in Figure 6. The QoS Info field contains the Parameter Set Count, which is initially arbitrary and is incremented each time any of the AC parameters changes. The reserved bits are set to 0 upon transmission and shall be ignored upon reception.

| Octets: 1 | 1 | 3 | 1 | 1 | 1 | 1 | 1 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ElementID | Length | OUI | OUI Type | OUI Subtype | Version | QoS Info field | Reserved | AC Parameters |

Figure 7 WME Parameter Element

Table 4 WME Parameter Element Field Values

| Field | Value |
| --- | --- |
| Element ID | 221 |
| Length | 24 |
| OUI | 00:50:f2 (hex) |
| OUI Type | 2 |
| OUI Subtype | 1 |
| Version | 1 |
| QoS Info field | See Figure 6 |
| Reserved | 0 |
| AC Parameters Best_Effort | AC Parameters Record AC_BE |
| AC Parameters Background | AC Parameters Record AC_BK |
| AC Parameters Video | AC Parameters Record AC_VI |
| AC Parameters Voice | AC Parameters Record AC_VO |

Each access category (AC) parameter record has the format shown in Figure 8.  The formats of the ACI/AIFSN and ECWmin/ECWmax fields are shown in Figure 9 and Figure 10, respectively.

| Octets: 1 | 1 | 2 |
| --- | --- | --- |
| ACI/AIFSN | ECWmin/ ECWmax | TXOP Limit |

Figure 8 AC Parameters Record Format

| 7 | 6 | 5 | 4 | 3 | 0 |
| --- | --- | --- | --- | --- | --- |
| Reserved | ACI |  | ACM | AIFSN |  |

Figure 9 ACI/AIFSN Field

The value of TXOP limit is specified as an unsigned integer, with the least significant octet transmitted first, in units of 32s. A TXOP limit value of 0 indicates that a single MPDU, in addition to a possible RTS/CTS exchange or CTS to itself, may be transmitted at any rate for each TXOP.  The value of the AC references the AC to which all parameters in this record correspond. The mapping between AC index (ACI) and AC is defined in Table 6. The AIFSN value indicates the number of time slots inside the Arbitration Interframe space to be used. The minimum value for AIFSN shall be 1.

Table 5 ACI to AC coding

| ACI | AC | Access Category |
| --- | --- | --- |
| 00 | AC_BE | Best Effort |
| 01 | AC_BK | Background |
| 10 | AC_VI | Video |
| 11 | AC_VO | Voice |

The ACM (Admission Control Mandatory) flag indicates that Admission Control is required for the AC.  If bit ACM is set to 0, the AC may be used without Admission Control. If bit ACM is set to 1, admission control must be performed prior to transmission with the parameters of the corresponding AC Parameter Record.

| 7 | 4 | 3 | 0 |
| --- | --- | --- | --- |
| ECWmax |  | ECWmin |  |

Figure 10 ECWmin/ECWmax field

The fields ECWmin and ECWmax encode the values of CWmin and CWmax respectively in an exponent form. The values ECWmin and ECWmax are defined such that:

CWmin=2ECWmin−1CWmax=2ECWmax−1

Hence the minimum encoded value of CWmin is 0, and the maximum value is 32767.

### Beacon Frame

Every beacon frame transmitted by a WME-enabled AP shall contain, in addition to those elements specified in [1], either a WME Information Element or a WME Parameter Element.

### Probe Request Frame

Probe request frames transmitted by a WME-enabled STA are unchanged from [1].

### Probe Response Frame

A probe response frame transmitted by a WME-enabled AP shall contain a WME Parameter Element.

### Association Request Frame

An association request frame contains a WME Information Element in addition to the information elements specified in [1].

### Association Response Frame

An association response frame shall contain a WME Parameter Element in addition to the information specified elements in [1].

### Management Notification Frame

The management notification frame is carried in a management action frame and has a format as shown in Figure 11, with fields as described in Table 6.

| 24/30 | 1 | 1 | 1 | 1 |  | 4 |
| --- | --- | --- | --- | --- | --- | --- |
| MAC Header | Category Code | Action Code | Dialog Token | Status Code | Elements | FCS |

Figure 11 Management Notification Frame Format

Table 6 Management Notification Frame Fields

| Field | Value |
| --- | --- |
| MAC Header | See [1] |
| Category code | 17 |
| Action code | See below |
| Dialog token | See below |
| Status Code | See below |
| Elements | One or more information elements |
| FCS | See [1] |

The action code field contains the values shown in Table 7.

Table 7 Management Notification Frame Action Codes

| Value | Action code |
| --- | --- |
| 0 | Setup request |
| 1 | Setup response |
| 2 | Teardown |
| 3-255 | Reserved |

The Setup request, Setup response, and Teardown management frames are used for Admission Control and shall at minimum contain a WME TSPEC element.

The Dialog Token field shall be set by the STA to a non-zero value in the Setup Request frame and shall be copied into the corresponding Setup Response notification frame. The Dialog Token shall be set to 0 in the Teardown frame.

The status code field is only filled in for the Setup response, and contains the values shown in Table 8. For other action codes, this field should be set to zero and ignored on receive.

Table 8 Setup Response Status Codes

| Value | Operation |
| --- | --- |
| 0 | Admission accepted |
| 1 | Invalid parameters |
| 2 | Reserved |
| 3 | Refused |
| 4-255 | Reserved |

### WME TSPEC Element

The format of the WME TSPEC element is shown in Figure 12.  The fields contained in the WME TSPEC element are listed in Table 9.  (Note: The fields are a snapshot of the TSPEC element in IEEE Std 802.11e/D6 plus an appended Medima Time field.)  Fields marked with a hyphen are optional, and shall be set to zero if not used. The value of zero for any field is a special reserved value, meaning "no information provided". The WME TSPEC body format is shown in Figure 13.

| Octets: 1 | 1 | 3 | 1 | 1 | 1 | 55 |
| --- | --- | --- | --- | --- | --- | --- |
| ID | Length | OUI | OUI Type | OUI Subtype | Version | WME TSPEC body |

Figure 12 WME TSPEC Element

| Octets: 3 | 2 | 2 |  | 4 | 4 | 4 |  | 4 |  | 4 |  | 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TS Info | Nominal MSDU Size | Maximum MSDU Size |  | Minimum Service Interval | Maximum Service Interval | Inactivity Interval |  | Suspension Interval |  | Service Start Time |  | Minimum Data Rate |
| 4 | 4 |  | 4 | 4 | 4 |  | 2 |  | 2 |  |  |  |
| Mean Data Rate | Peak Data Rate |  | Maximum Burst Size | Delay Bound | Minimum PHY Rate |  | Surplus Bandwidth Allowance |  | Medium Time |  |  |  |

Figure 13 WME TSPEC Body format

A WME TSPEC request shall be transmitted by a STA to an AP in order to request admission for an AC that requires admission control. The STA may transmit unadmitted traffic for those ACs for which the AP does not mandate admission control. A STA may need to transmit a WME TSPEC request for an AC that does not mandate admission control, e.g for the establishment of the triggered power save mode of operation.

Table 9 WME TSPEC Element Field Values

| Field | Value |
| --- | --- |
| Element ID | 221 |
| Length | 6 + 55 = 61 |
| OUI | 00:50:f2 (hex) |
| OUI Type | 2 |
| OUI Subtype | 2 |
| Version | 1 |
| TS Info | See below |
| Nominal MSDU Size | See below |
| Maximum MSDU Size | - |
| Minimum Service Interval | - |
| Maximum Service Interval | - |
| Inactivity Interval | - |
| Suspension Interval | - |
| Service Start Time | - |
| Minimum Data Rate | - |
| Mean Data Rate | See below |
| Peak Data Rate | - |
| Maximum Burst Size |  |
| Delay Bound | - |
| Minimum PHY Rate | See below |
| Surplus Bandwidth Allowance | See below |
| Medium Time | See below |

The TS Info field contains the subfields shown in Figure 14. The TID field, combined with the RA and the TA of the frame containing the TSPEC element, uniquely identifies the flow for which a request is being made. The UP field contains the 802.1D priority information, and is the same value used in QoS data frames associated with this flow. The direction field contains values shown in Table 10.

| 23 | 17 | 16 | 15 | 14 | 13 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reserved |  | Reserved | Reserved |  | UP |  | PSB | Reserved | 0 | 1 | Direction |  | TID |  | Reserved |

Figure 14 TS Info Field in WME TSPEC Element

Table 10 Direction Field Values

| Direction Value | Meaning |
| --- | --- |
| 00 | Uplink |
| 01 | Downlink |
| 10 | Reserved |
| 11 | Bi-directional |

The uplink and downlink characteristics are symmetric for a bidirectional reservation. The value of a TSPEC field for a bidirectional reservation represents the resources for one direction.

The Power Save Behavior (PSB) field is coded according to Table 11.

Table 11 Power Save Behavior Field Values

| PSB Value | Meaning |
| --- | --- |
| 0 | Legacy |
| 1 | Triggered |

The PSB bit determines the power save method of delivery of buffered traffic used when the station is operating in PS-mode, as described in section 3.6.

The Nominal MSDU Size field is 2 octets long and contains an unsigned integer that specifies the nominal size, in octets, of MSDUs belonging to the traffic under this traffic specification and is defined in Figure 15. If the Fixed subfield is set to 1, then the size of the MSDU is fixed and is indicated by the Size Subfield. If the Fixed subfield is set to 0, then the size of the MSDU might not be fixed and the Size indicates the nominal MSDU size.

| 15 | 14 | 0 |
| --- | --- | --- |
| Fixed | Size |  |

Figure 15 Nominal MSDU Size Field

The Mean Data Rate field is 4 octets long and contains an unsigned integer that specifies the average data rate, in units of bits per second, for transport of MSDUs belonging to this traffic within the bounds under this traffic specification.  The Mean Data Rate does not include the MAC and PHY overheads incurred in transferring the MSDUs.

The Minimum PHY Rate field is 4 octets long and contains an unsigned integer that specifies the minimum PHY rate, in units of bits per second that is required for transport of the MSDUs belonging to the traffic in this TSPEC.

The Surplus Bandwidth Allowance Factor field is 2 octets long and contains specifies the excess allocation of time (and bandwidth) over and above the stated rates required to transport an MSDU belonging to the traffic in this TSPEC.  This field is represented as an unsigned binary number with an implicit binary point after the leftmost 3 bits.  This field is included to account for retransmissions. As such, the value of this field must be greater than unity.

Medium Time is a 16-bit unsigned integer which describes the amount of time admitted to access the medium, in units of 32 microsecond periods per second.

# WME Protocol Specification

## Association and Capability Negotiation

### Procedure at an AP

An AP that supports WME shall include either a WME Information Element or a WME Parameter Element in every beacon.  In response to a probe request, a WME-enabled AP shall include a WME Parameter Element in its probe response.

On receipt of an association request and subsequent transmission of a corresponding association response: the AP shall include a WME Parameter Element in the association response if the corresponding association request contained a WME Information element and shall treat the association as WME association.

If the destination address of a data frame to be transmitted on the wireless medium corresponds to a STA with a WME association, the AP shall use WME QoS data subtype frame formats when transmitting the frame to it. If the destination address corresponds to a STA associated as a non-WME STA, the AP shall not use QoS subtype data frames.

### Procedure at a STA in an Infrastructure Network

A WME-enabled STA shall determine the WME capability of an AP with which it wishes to associate before transmitting an association request to it.  It may do this either passively, by receiving a beacon frame, or actively, by transmitting a probe request to it.

From the most recently received probe response or beacon from a specific AP, the STA shall determine whether the AP supports WME.

A STA shall include a WME Information Element in an association request if it has determined that the recipient AP supports WME and shall treat the association as WME association if the association response contained a WME Parameter element.

### Procedure at a STA in an IBSS

WME may be used in an IBSS, but since there is no negotiation of capability via association in this case, any STA wishing to use QoS data frame subtypes when transmitting to each STA must first infer the capability at that STA by other means.

A WME-capable STA operating in an IBSS shall maintain an inferred WME capability state for each destination address in the IBSS that it is aware of. It may set WME capability state to “supported” for a given destination address on receipt of a beacon frame from that STA containing a WME element, or on receipt of a probe response frame from that STA containing a WME Information Element. The WME capability state for each other STA shall be set to “not supported” until receipt of such a beacon or probe response.

WME STAs operating in an IBSS shall respond to probe frames from other STAs in the same IBSS by transmitting a probe response frame to that STA, containing a WME Information Element.

## Setting of WME Parameters

### Default WME parameters

Table 12 Default WME Parameters

| AC | CWmin | CWmax | AIFSN | TXOP Limit (802.11b) | TXOP Limit (802.11a/g) |
| --- | --- | --- | --- | --- | --- |
| AC_BK | aCWmin | aCWmax | 7 | 0 | 0 |
| AC_BE | aCWmin | aCWmax | 3 | 0 | 0 |
| AC_VI | (aCWmin + 1)/2 -1 | aCWmin | 2 | 6.016ms | 3.008ms |
| AC_VO | (aCWmin+1)/4 - 1 | (aCWmin+1)/2 -1 | 2 | 3.264ms | 1.504ms |

### WME Parameters in an Infrastructure Network

A WME-enabled AP may arbitrarily determine values for the parameters CWmin, CWmax, AIFS and TXOP limit for each of the four access categories.  An AP may change the values of these parameters at any beacon time.

An AP shall include a WME Parameter Element containing its currently determined values for all WME parameters in all beacons within two or more DTIM periods when WME parameters have changed. The AC parameter set count that is contained in the WME Parameter Element and WME Information Element is incremented following the change of one or multiple WME parameters. An AP shall include a WME Parameter element in probe response frames and in association response frames.

The STA is required to monitor the change of WME parameters conveyed in the WME Parameter Element in beacons and shall update its values accordingly. Prior to association (and used only for transmitting probe, authentication and association request frames) it shall set these parameters to the default values shown in Table 12.

A STA shall subsequently update the values of these parameters from any successfully received probe response frame, association response frame, which are addressed to the STA or beacon frame transmitted by the AP with which the STA is associated. It is the responsibility of the STA to use the current WME parameters for accessing the wireless medium.

The AP may use a different set of parameters than it advertises. A recommended set of default parameters for the AP is listed in the annex. The value of AIFSN shall be greater than or equal to 1 for APs.

### WME Parameters in an IBSS

A STA which is a member of an IBSS shall set the values of all WME parameters to the defaults listed in Table 12. In an IBSS, the WME Parameter element is not included in the beacon.

## Assignment of Frames to Queues

### Mappings for Unicast Frames

The MAC data service at a STA or AP provides for connectionless, asynchronous transport of MSDUs. Each MSDU transfer request includes an 802.1D Priority field equal to that value. The priority bits of the 802.1D field are mapped to Access Category (AC) according to Table 13 and are listed in increasing priority order. The UP field is carried in the QoS control field of an MPDU. The UP field references the AC the MPDU is transmitted at using the mapping defined in Table 13. At the receiver, the UP field carried in the MPDU shall be used to re-create the 802.1D priority information of the MSDU.

Table 13 802.1D Priority to AC mappings

| Priority | 802.1D Priority (= UP) | 802.1D Designation | Access Category | WME Designation |
| --- | --- | --- | --- | --- |
| highest lowest | 1 | BK | AC_BK | Background |
|  | 2 | - |  |  |
|  | 0 | BE | AC_BE | Best Effort |
|  | 3 | EE |  |  |
|  | 4 | CL | AC_VI | Video |
|  | 5 | VI |  |  |
|  | 6 | VO | AC_VO | Voice |
|  | 7 | NC |  |  |

Transmit frames are then placed in queues according to AC. The AP and STA may implement more queues for internal prioritization. Data frames with no priority information are treated as best effort.

Management frames have no QoS Control field, but should be transmitted with parameters of AC_VO

PS-Poll frames should be mapped to AC_BE.

### Mappings for Received Unicast Frames at an AP

Unicast frames received by an AP may be:

- Non-QoS subtypes, in which case the AP shall assign an 802.1D priority of 0 (best effort) to them. See section 2.1

- QoS subtypes, in which case the AP shall use the UP subfield of the QoS control field as the priority information.

In the event that the received frame has a destination address within the BSS, the AP shall determine the transmit queue according to the procedure described in section 3.3.1.

### Mappings for Group Addressed and Buffered Frames at an AP

The AP forwards all group addressed frames (Multicast/Broadcast) using non-QoS data frame subtypes, if not all the recipients are WME capable STAs. The AP may forward Power Save frames that are buffered with QoS Data frame subtype only to WME capable STAs. Otherwise, the frames must be sent using non-QoS data frame subtypes. The AP is not required to implement multiple AC queues for transmission of group addressed and buffered Power Save frames, and may use a single dedicated queue for this purpose. If the AP uses a single queue for pending group addressed frames, then the AP should maintain the queue in priority order if possible.

## Channel Access Protocol

### Reference Implementation

The channel access protocol is derived from the DCF procedures described in [1]. This document specifies only differences between the WME channel access protocol and the reference.

Figure 16 Reference Implementation Model

A model of the reference implementation is shown in Figure 16 and illustrates the mapping of the priority bits of the 802.1D Priority information that is contained in the MSDU to UP, access category (AC), the four transmit queues, and four independent channel access functions, one for each queue. The mapping of AC to priority is described in section 3.3. Each AC is associated with its own sequence number counter as described in 2.1.5.

### Transmit Opportunities & TXOP Limits

There are two modes of TXOP defined, EDCA TXOP and continuation TXOP.

An EDCA TXOP occurs when the EDCA rules permit access to the medium.

A continuation TXOP occurs when a channel access function retains the right to access the medium following the completion of a frame exchange sequence, such as on receipt of an Ack frame.

The TXOP limit duration values for each AC are advertised by the QAP in the WME Parameter Element in Beacons, Probe response, and Association response frames. A TXOP limit value of 0 indicates that a single MSDU or MMPDU in addition to a possible RTS/CTS or CTS to itself may be transmitted at any PHY rate for each TXOP.

STAs shall ensure that the duration of TXOPs obtained using the EDCA rules do not exceed the TXOP limit.  The duration of a TXOP is the time the TXOP holder maintains uninterrupted control of the medium, and it includes the time required to transmit frames sent as an immediate response to TXOP holder transmissions.

An STA shall fragment an MSDU so that the transmission of the first MPDU of the TXOP does not cause the TXOP limit to be exceeded at the PHY rate selected for the initial transmission attempt of that MPDU. The TXOP limit may be exceeded, when using a lower PHY rate than selected for the initial transmission attempt of the first MPDU, for a retransmission of an MPDU, or for the initial transmission of an MPDU if any previous MPDU in the current MSDU has been retransmitted. When the TXOP limit is exceeded due to the retransmission of a MPDU at a reduced PHY rate, the STA shall not transmit more than one MPDU in the TXOP.

### Obtaining an EDCA TXOP

Each channel access timer shall maintain a backoff function timer, which has a value measured in backoff slots.

The duration AIFS[AC] is a duration derived from the value AIFSN[AC] by the relation

AIFS[AC] = AIFSN[AC] × aSlotTime + aSIFSTime

An EDCA TXOP is granted to a channel access function when the channel access function determines that it shall initiate the transmission of a frame exchange sequence. Transmission initiation shall be determined according to the following rules:

On specific slot boundaries, each channel access function shall make a determination to perform one and only one of the following functions:

  - Initiate the transmission of a frame exchange sequence for that access function

  - Decrement the backoff timer for that access function

  - Invoke the backoff procedure due to an internal collision

  - Do nothing for that access function.

The specific slot boundaries at which exactly one of these operations shall be performed are defined as follows, for each channel access function:

- Following AIFSN[AC] * aSlotTime – aRxTxTurnaraoundTime of medium idle indication after SIFS (not necessarily idle time) after the last busy medium on the antenna, if the last busy medium indication was the result of a frame reception with a correct FCS ; or

- Following EIFS – DIFS + AIFSN[AC] * aSlotTime of medium idle indication after the last indicated busy medium as determined by the carrier sense mechanism if the last busy medium indication was the result of a frame reception with an FCS error or PHY-RXEND.indication (RXERROR), where the value of RXERROR is not NoError.

- When any other channel access function at this QSTA transmitted a frame requiring acknowledgement, the earlier of:

  - the end of the ACK-Timeout interval timed from the PHY_TXEND.confirm,  followed by AIFSN[AC] * aSlotTime – aTxRxTurnaraoundTime of IDLE Time

  - at the end of the first AIFSN[AC] * aSlotTime – aTxRxTurnaraoundTime of IDLE medium after the PHY-RXEND.indication when a PHY-RXSTART.indication occurs as specified in subclause 9.2.8 of [1],

- following AIFSN[AC] * aSlotTime– aTxRxTurnaraoundTime of medium idle indication after SIFS (not necessarily idle time) after the last indicated busy medium on the antenna that was the result of a transmission of a frame for any channel access function and which did not require an acknowledgement; or

- following AIFSN[AC] * aSlotTimeof medium idle time indication after the last indicated idle medium as indicated by the carrier sense mechanism that is not covered by a) through d).

- following aSlotTime of medium idle indication which occurs immediately after a decrement of the backoff counter for that channel access function.

At each of the above-described specific slot boundaries, each channel access function shall initiate a transmission sequence, if:

- there is a frame available for transmission at that channel access function, and

- the backoff timer for that channel access function has a value of zero, and

- initiation of a transmission sequence is not allowed to commence at this time for a channel access function of higher UP.

At each of the above-described specific slot boundaries, each channel access function shall decrement the backoff timer by one, if:

- The backoff timer for that channel access function has a value which is greater than zero.

At each of the above-described specific slot boundaries, each channel access function shall invoke the backoff procedure due to an internal collision, if:

- There is a frame available for transmission at that channel access function, and

- the backoff timer for that channel access function has a value of zero, and

- initiation of a transmission sequence is allowed to commence at this time for a channel access function of higher UP.

At each of the above-described specific slot boundaries, each channel access function shall do nothing, if none of the above actions is taken.

Figure 17 EDCA Mechanism Timing

An example showing the relationship between AIFS, AIFSN, DIFS and slot times immediately following a medium busy condition (and assuming that medium busy condition was not caused by a frame in error) is shown in Figure 17. In this case, with AIFSN=2, the channel access function may decrement the backoff counter for the first time at

2 * aSlotTime

following the end of the medium busy condition. Note: The end of the medium busy condition happens at the end of M1 in Figure 17. If, in this example, the backoff counter contained a value of 1 at the time the medium became idle, transmission would start as a result of an EDCA TXOP on-air at a time

aSIFSTime + 3 × aSlotTime

following the end of the medium busy condition.

### Obtaining a Continuation of TXOP

A continuation of TXOP is granted to a channel access function at a SIFS period following the successful completion of a frame exchange if there is a frame available for transmission that fits in any remaining EDCA TXOP duration. If a STA has in its transmit queue an additional frame of the same access category as the one just transmitted, and the duration of transmission of that frame plus any expected acknowledgement for that frame is less than the remaining medium occupancy timer value, then the STA may commence transmission of that frame at SIFS after the completion of the immediately preceeding frame exchange sequence.

A frame exchange may be either a multicast frame transmitted by an AP or a frame transmitted with “no acknowledgement” policy, for which there is no expected acknowledgement, or a unicast frame followed by a correctly received acknowledgement frame transmitted by either a STA or an AP.

Note that, as for an EDCA TXOP, a continuation TXOP is granted to a channel access function, not to a STA or AP, such that a continuation TXOP is only permitted for the transmission of a frame of the same access category as the access category of the frame that was granted the EDCA TXOP.

### Backoff Procedure

Each channel access function shall maintain a state variable CW[AC], which shall be initialized to the value of the parameter CWmin[AC] (see section 3.2).

If a frame is successfully transmitted for a specific channel access function, indicated by either the successful reception of a CTS in response to an RTS, or the successful reception of an Ack in response to a unicast MPDU, or by transmitting a multicast frame or a frame with “no acknowledgement” policy, CW[AC] shall be reset to CWmin[AC].

The backoff procedure shall be invoked for a channel access function when either:

  - A frame with that AC is requested to be transmitted and the medium is busy as indicated by either physical or virtual carrier sense, and the backoff timer has a value of zero for that AC.

  - The final transmission by the TXOP holder initiated during the TXOP for that AC was successful

  - The transmission of a frame of that AC fails, indicated by a failure to receive a CTS in response to an RTS, or a failure to receive an Ack that was expected in response to a unicast MPDU or MMPDU.

  - The transmission attempt collides internally with another channel access function of an AC that has higher priority, that is, two or more channel access functions in the same STA or AP are granted a TXOP at the same time.

If the backoff procedure is invoked because of reason a) above the value of CW[AC] shall be left unchanged. If the backoff procedure is invoked because of reason b) above, the value of CW[AC] shall be reset to CWmin[AC].

If the backoff procedure is invoked because of a failure event (either reason c) or d) above) the value of CW[AC] shall be updated as follows before invoking the backoff procedure:

- if the short or long retry count for the STA has reached aShortRetryLimit or aLongRetryLimit[AC] respectively, CW[AC] will be reset to CWmin[AC].

- Otherwise,

    - if CW[AC] is less than CWmax[AC], CW[AC] shall be set to the value (CW[AC]+1)*2-1

    - if CW[AC] is equal to CWmax[AC], CW[AC] shall remain unchanged for the remainder of any retries

Following the update of the value of CW[AC], the backoff timer is set to an integer value chosen randomly with a uniform distribution taking values in the range (0,CW[AC]) inclusive.

### Retransmit Procedures

If a STA or AP, in an infrastructure BSS or an IBSS, transmits frames to a destination using QoS data types, it may following a failed transmission of a frame attempt to transmit another frame with a different access category to the same or any other destination. The STA has to contend for the medium when transmitting another frame with a different access category to the same or any other destination using the rules defined in [1].

If a STA or an AP does not use QoS data types when transmitting frames to a particular receiver address, once an initial attempt, excluding internal collisions, has been made to transmit a frame, it shall not transmit other data frames to that receiver address until the first frame is either successfully transmitted or discarded. It may, however, transmit other frames to different receiver addresses.

## Admission Control Procedures

WME STA may support admission control procedures.  APs shall support admission control procedures, at least to the minimal extent of advertising that admission is not mandatory on its ACs.

The AP uses the ACM (admission control mandatory) flags advertised in the WME Parameter element to indicate whether admission control is required for each of the ACs, as a matter of policy. While the CWmin, CWmax, AIFS, TXOP limit parameters may be adjusted over time by the AP, the ACM bit shall be static for the duration of the lifetime of the BSS. A WME TSPEC request shall be transmitted by a STA to an AP in order to request admission of a flow in any direction (uplink, downlink, or bidirectional) using the AC parameters of those ACs that require admission control. Each admitted flow is uniquely identified by the combination of TID from the TSPEC, RA and TA from the frame that contained the accepted TSPEC. Within a WME STA in an infrastructure BSS, the RA and TA are constant. This means that up to 16 flows are allowed per associated STA within the BSS. The 16 possible flows may be distributed among the various AC in any manner. E.g. All 16 flows might correspond to AC_VO, or there may be four flows admitted per AC. The UP value in the TSPEC for each flow, combined with the static mapping of UP to AC will determine the distribution of admitted flows among the four AC. A WME STA may choose to aggregate flows locally by combining the parameters of multiple flows into a single TSPEC, thereby allowing more than 16 flows to exist. The STA may transmit unadmitted traffic using AC parameters of those ACs which the AP does not require admission control.

Each channel access function shall maintain two variables. The first of these is the admitted time, and the second is the used time. The admitted time and used time shall be set at association time to zero.

The channel access function shall update the value of used time:

a) at one second intervals

used_time = max((used_time - admitted_time), 0)

b) after each frame exchange

used_time = used_time + FrameExchangeTime.

The FrameExchangeTime equals the time required to transmit the frame plus one ACK frame plus one SIFS interval. If the used time reaches or exceeds the admitted time value, the corresponding channel access function shall no longer transmit using the EDCA parameters for that AC as specified in the QoS parameter set element.

If an explicit admission is torn down, the admitted time and the used time for the AC shall both be set to zero.

A Setup request for a flow (TID) that has an established explicit admission shall be regarded as a request for a change to the existing admitted flow.

If a request for a change to an admitted flow is refused, the previously accepted admission for the same flow remains valid.

Admitted time for each channel access function is the sum of the admitted times for all admitted flows which map into that AC. A WME STA may choose to police admitted time and used time on a per-TID basis, with the rules for not transmitting when admitted time is exceeded applying per TID as well.

A WME STA shall not be required to support any more than one admitted TSPEC per AC.

A WME AP shall not be required to support any more than one admitted TSPEC per AC per WME STA.

### Procedures at the AP

The AP shall respond to requests for admission conveyed in the WME TSPEC request elements for those AC with set ACM flag.  If the ACM flag is set to 0 for an AC, the AP is not required to respond to a TSPEC request. On receipt of a WME TSPEC request element conveyed in any management notification frame except a probe request from an associated STA, the AP shall make a determination as to whether to

- accept the request

- deny the request

The AP may use any algorithm in making such a determination. If the AP decides to accept the request, the AP shall also derive the Medium Time from the information conveyed in the WME TSPEC request element.  The AP may use any algorithm in deriving the Medium Time, but normally it will use the procedure described in the Annex. Having made such a determination, the AP shall transmit a WME TSPEC element to the requesting STA contained in a Setup response frame. If the AP is accepting the request, the Medium Time field shall be specified.

The AP may choose at any time to tear down an explicit admission by sending a management notification frame with the action code set to Teardown.

By observing the used time and the admitted time variables for admitted traffic destined to an STA, the AP may drop MPDUs that are in excess of the admitted traffic using an unspecified algorithm.

### Procedure at STAs

At any point, following association, the STA may decide, to explicitly request admission of traffic to be transmitted or/and received on a specific AC. The STA shall use the mappings in Table 13 to identify the sending AC from the UP field

In order to make such a request, the STA shall transmit a WME TSPEC element contained in a Setup request management frame with the following fields specified (i.e. non-zero):  Nominal MSDU Size, Mean Data Rate, Minimum PHY Rate, and Surplus Bandwidth Allowance. The Medium Time field is not used in the request frame and shall be set to zero.

On receipt of a WME TSPEC element, contained in a Setup response frame, indicating that the request has been accepted, the STA shall set the admitted time variable for the specified channel access function to the value contained in the Medium Time field of that element. The channel access function is referenced via the 802.1D priority field carried in the response using the mapping described in Table 13.

The STA or AP may choose to tear down the explicit request at any time. In order for the STA to tear down an explicit admission, the STA shall transmit a Teardown frame containing the WME TSPEC element to the AP.

If the STA receives a management notification frame with the action code set to Teardown it shall disable the use of the specified AC.

If an explicit admission is torn down, the STA shall disable the use of the AC.

By observing the used time and the admitted time variables for admitted traffic for an AC, the STA shall drop MPDUs that are in excess of the admitted traffic using an unspecified algorithm

If, for example, a STA has made and had accepted an explicit admission for an AC, and the channel conditions subsequently worsen, possibly including a change in PHY layer data rate, such that it requires more time to send the same data, the STA shall drop excess traffic and may make a request for more admitted time to the AP.

## WME Power -save (PS) Procedures

The procedures defined in this section apply to unicast QoS-Data and QoS-Null frames that are to be delivered to a WME STA when the STA is in PS-mode.

The WME power-save procedures are based on the legacy procedures defined in [1], but an option for triggered power-save delivery is added. A WME STA chooses legacy versus triggered behavior on a per-AC basis. A STA causes an AC to be “trigger-enabled” by setting PSB=1 in at least one admitted downlink TSPEC that maps to the AC. (Note: throughout this section, a bi-directional TSPEC is equivalent to an uplink and a downlink TSPEC with identical characteristics).

If, for a particular STA, an AC is not trigger-enabled, then all downlink frames destined to that STA that map to that AC are buffered and delivered using the procedures described in [1]. The buffer used to hold these frames will be referred to as the legacy PS buffer. The QAP uses the TIM and the More Data bit carried in Frame Control Field to indicate the status of the legacy buffer as specified in [1].

If, for a particular STA, an AC is trigger-enabled, then all downlink frames destined to that STA that map to that AC are temporarily held by the QAP in a “triggered PS buffer”. Conceptually, there is one triggered PS buffer per trigger-enabled AC and PS-mode STA (this conceptual model does not, of course, constrain the implementation). The QAP shall not use the TIM bits or the More Data bits to indicate the status of any triggered PS buffer.

Downlink frames are released from a triggered PS buffer by a trigger frame. A trigger frame is defined as a QoS-Data or QoS-Null frame, sent uplink by a PS-mode STA, in which the UP maps to a trigger-enabled AC. (Note: transmission of trigger frames is not implicitly allowed by admission of a downlink flow. If the trigger frame maps to an AC that has ACM=1, then the STA must establish a suitable uplink flow before sending triggers.) The combination of sending STA and UP uniquely identifies a triggered PS buffer; the trigger frame causes all frames to be released from that buffer.

The released frames are delivered during a triggered service period. The triggered service period begins after the QAP acknowledges the trigger frame transmitted by the station. The frames released from the triggered PS buffer shall be delivered using the access parameters of the corresponding AC. The QAP uses the EOSP bit that is carried in the QoS Control field to indicate the end of the triggered service period.

It should be clear that, for a particular STA, the triggered PS buffer, the trigger frame, and the triggered service period are all per-AC. Therefore, if a WME STA establishes flows that result in more than one trigger-enabled AC, the STA must initiate triggered service periods separately for each such AC in order to retrieve all the buffered frames.  The WME station must remain awake as long as there is at least one triggered service period still in progress.

### AP Operation

If a WME STA is in Active mode, all frames destined to that STA shall be transmitted immediately, without PS buffering.

If a WME STA is in PS-mode, the following shall apply:

- An AC shall be considered trigger-enabled, for a particular STA, if there exists at least one admitted downlink or bi-directional TSPEC with PSB=1 that maps to that AC. Downlink unicast QoS-Data frames associated with a trigger-enabled AC shall be temporarily held in the AC’s triggered PS buffer.

- Other downlink frames shall be temporarily held in the legacy PS buffer.

- The algorithm to manage the buffering is beyond the scope of this standard, with the exception that the AP must preserve the order of arrival of frames on a per-AC, per-station basis.

- At every beacon interval, the AP shall assemble the partial virtual bitmap containing the legacy PS buffer status and shall send this out in the TIM field of the beacon.

- The AP shall not consider the status of any triggered PS buffer when assembling the TIM.

- The AP shall deliver all frames from the legacy PS buffer using the power management procedures described in [1].

- The AP shall deliver all frames from a triggered PS buffer during a triggered service period.

- The AP shall have an aging function to delete pending traffic when it is buffered for an excessive time period.  Aging may be based on the listen interval specified by the station in its (re-) association request.

A triggered service period shall begin, for a particular trigger-enabled AC and a particular WME STA, when the QAP receives a QoS-Data or QoS-Null frame sourced by the STA and containing a UP that maps to the trigger-enabled AC.

A triggered service period shall end, for a particular AC and WME STA, when the AP sends to that station a QoS-Data or QoS-Null frame that:

- contains a UP subfield that maps to the AC and

- has its EOSP bit set to 1.

AP's shall not be required to support more than one admitted downlink TSPEC with PSB=1 per associated STA.

### STA Operation

WME stations in PS-mode shall operate as follows to receive buffered frames from the AP.

- WME STAs shall use the power management procedures defined in [1] to retrieve frames held in the legacy PS buffer.

- WME STAs shall initiate a triggered service period to retrieve frames from a triggered PS buffer.

- A WME STA initiating multiple triggered service periods shall remain in the awake state until the AP indicates the end of all triggered service periods.

WME (Informative) Recommended Practices Annex

QoS Parameter Updates

It is recommended that the mechanism to update QoS parameters by way of the WME Parameter Element in beacon frames be used infrequently. It is not the intent of the designers of the protocol to specify this as a dynamic adaptation mechanism, but rather as a means of auto-configuring policy at different locations.

However, in the event that the number of associated STAs changes or some other event occurs that significantly alters the conditions, it is expected that the AP may change the policy settings. There is no expectation of rapid update by the STAs, which may take of the order of seconds or tens of seconds if necessary to update their parameter settings.

To assure that all STAs use the proper QoS parameters following modification of AC parameters, the WME Parameter Element should be present for multiple beacon intervals as specified in 3.2.2.

Use of Admission Control and Downgrading

It is recommended not to require Admission Control for the access categories AC_BE (Best Effort) and AC_BK (Background). The Admission Control Mandatory Bit (ACM) for these categories should be cleared to "0". The AC parameters chosen by the AP should account for unadmitted traffic in these access categories.

In the case the QBSS requires admission control, i.e. ACM bit is set for an AC, and the WME STA has an MSDU to send on that AC, but has not performed successful admission control, the following options exist:

- The MSDU is dropped

- The access parameters of the access function corresponding to the AC are adjusted to match the parameters of a lower AC that does not require admission control. As a result, there will be two access functions with identical admission control parameters within the WME STA competing for medium access. The cumulative throughput of this WME STA may be higher.

- The MSDU is sent using a different UP.  The UP is changed to map to a lower AC that does not require admission control.  The UP has to be changed prior to calculating the MIC, assignment of the TSC and mixing the keys. Changing of the UP (2) is performed outside the MAC and is out of the scope of this spec.

It is the responsibility of the application invoking the SAP to assure consistency of traffic that is mapped to one AC, i.e. Admitted and non-admitted traffic mapped to one AC shall not be mixed.

A WME AP should use option (1) or (3)

Deriving Medium Time

It is recommended that the AP use the following procedure to derive Medium Time in its Setup response.

There are two requirements to consider:  1) the traffic requirements of the application, and 2) the expected error performance of the medium.  The application requirements are captured by two TSPEC parameters:  Nominal MSDU Size and Mean Data Rate.  The medium requirements are captured by two TSPEC parameters:  Surplus Bandwidth Allowance and Minimum PHY Rate. The following formula describes how Medium Time may be calculated:

Medium Time = Surplus Bandwidth Allowance * pps * medium time per frame exchange where:

pps =  ceiling( (Mean Data Rate / 8) / Nominal MSDU Size ); medium time per frame exchange =  duration(Nominal MSDU Size, Minimum PHY Rate) + SIFS + ACK duration; duration() is the PLME-TXTIME primitive defined in the standard that returns the duration of a packet based on its payload size and the PHY data rate employed.

WME AP Default Parameter

It is recommended that the AP uses the default EDCA parameter listed in Table 14 and advertises the Table 12 values in WME Parameter Elements.

Table 14 Default EDCA Parameters for the AP

| AC | CWmin | CWmax | AIFSN | TXOP Limit (802.11b) | TXOP Limit (802.11a/g) |
| --- | --- | --- | --- | --- | --- |
| AC_BK | aCWmin | aCWmax | 7 | 0 | 0 |
| AC_BE | aCWmin | 4*(aCWmin+1) - 1 | 3 | 0 | 0 |
| AC_VI | (aCWmin + 1)/2 -1 | aCWmin | 1 | 6.016ms | 3.008ms |
| AC_VO | (aCWmin+1)/4 - 1 | (aCWmin+1)/2 -1 | 1 | 3.264ms | 1.504ms |

Changes to WPA for WME

Michael MIC Generation

Per the WPA pointer document, the TKIP MIC is computed over the MSDU destination address (DA), MSDU source address is computed over the MSDU destination address (DA), MSDU source address (SA), the MSDU priority, 3 zero bytes and the entire unencrypted MSDU data (payload):

| 6 | 6 | 1 | 3 | M | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DA | SA | Priority | 0 | Data | M0 | M1 | M2 | M3 | M4 | M5 | M6 | M7 |

Figure 18 TKIP MIC Processing Format

Figure 18 is treated as a byte stream using the conventions described in Clause 7.1.1 of IEEE Std. 802.11, 1999 Edition.  M0 – M7 are each byte of the Michael MIC.

On transmit, the MSDU priority used to calculate the Michael MIC shall be the numeric value of the UP for each MPDU of the MSDU.  On receive, the MSDU priority shall be the numeric value of the UP for each MPDU of the MSDU.

TKIP Replay Counters

Each receiver shall maintain at least 4 unicast replay counters for each MAC address it receives WME traffic (at least 1 for each AC).  (Note: All WPA stations must also maintain 1 replay counter for each MAC address it receives multicast traffic.)  The number of unicast replay counters supported by a receiving station is indicated by the Number of Replay Counters sub-field in the RSN Capabilities field of the WPA IE (a value of 2 = 4 replay counters, a value of 3 = 16 replay counters).  When a receiver supports 4 replay counters, transmitters shall not ‘reorder’ MPDUs within an AC that have the same MAC address, but different TID, once the TSC has been assigned (prevents discarding out of order MPDUs at the receiver).  Receivers may support 16 unicast replay counters, as indicated by the Number of Replay Counters.  In this case, there will be 1 replay counter for each TID and transmitters may ‘reorder’ MPDUs within an AC that have the same MAC address but different TID, after the TSC has been assigned.  Note:  It is never allowed to reorder MPDUs within an AC that have the same MAC address and TID.

Conveyance of 802.1Q VLAN tags

If an 802.1Q VLAN tag is present within a WME frame, the frame should have the SNAP encoded tag format as defined in [4] (see section 9.3). The frame format is shown in Figure 19. For comparison a frame without 802.1Q VLAN tag is depicted in Figure 20. The general format of IP Datagrams over IEEE 802 networks and the SNAP header are defined in [3].

| 24-32 | 0 or 4 | 6 | 2 | 2 | 2 | n | 4 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| MAC Header | IV | AAAA03000000h | 8100h | VLAN-TAG | Type | Payload and ICV | FCS |
| MAC Header | WEP | SNAP Header | 802.1Q |  | Ether-type |  |  |

Figure 19 Frame Format with 802.1Q VLAN tag

| 24-32 | 0 or 4 | 6 | 2 | n | 4 |
| --- | --- | --- | --- | --- | --- |
| MAC Header | IV | AAAA03000000h | Type | Payload and ICV | FCS |
| MAC Header | WEP | SNAP Header | Ether-type |  |  |

Figure 20 Frame Format without 802.1Q VLAN Tag

WME STAs shall be able to receive frames with and without VLAN tag as depicted in Figure 19 and Figure 20.

When a VLAN tag is present, the CFI bit in the VLAN tag will be set to 0.

In general, VLAN tags should NOT be present within WME frames. The UP of the QOS subfield contains the information contained in the priority field of the VLAN tag, such that a priority tagged frame (i.e. a frame with a VLAN tag where VLAN_ID = 0) is not necessary.

In the case of a non-zero VLAN_ID, the use of VLAN tags is generally restricted to trunking links which comprise a larger switching network's backbone, or for the connection of VLAN aware servers. WDS frames would more naturally be employed for trunking purposes, and therefore, VLAN Tags may be deemed necessary within WDS frame types. VLAN tags may appear in non-WDS WME frames, e.g. when attaching a VLAN aware server to the network wirelessly.

Additionally, VLAN tags may be deemed necessary by some protocols in order to convey priority information across the WME link where QOS subtypes are not used. In such cases, a priority tagged frame would be employed (i.e. a VLAN tag with VLAN_ID = 0).

Best effort, non-QOS subtype frames should not include VLAN tags where those frames may have a non-WME STA as the intended receiver. A non-WME STA might not be capable of interpreting the frame format of Figure 19. WME STAs shall be able to parse non-QOS subtype frames with a VLAN tag. This ensures that if you are using a mix of legacy and WME APs a WME STA works in both situations where tags are present on the network.

Differences between Legacy DCF Formulation and WME

DCF Formulation (IEEE Std 802.11-1999)

A random backoff value is selected in the range 0 to CW inclusive. CW is always of the form 2n-1. In the case of 802.11a, in which CWmin is 15, initial backoff values are therefore in the range [0,15].

In congested conditions, decrement of the backoff counter may be interrupted by a transmission from another STA. The only STAs which will suspend decrement of their backoff counters are those which have not yet counted down to zero. Therefore, when resuming a suspended backoff, the smallest value of backoff counter is 1. As a result, the earliest time that a STA can transmit after the medium is no longer busy depends on whether the STA has selected a new backoff value (minimum value 0) or is resuming a previously suspended backoff (minimum value 1). Figure 21 illustrates the Backoff behavior.

Figure 21 Backoff Behavior

The AP may choose to adjust the backoff parameters for WME STAs to mitigate differences in the access priority with respect to Best_Effort traffic and legacy STAs.

References

- ISO/IEC 8802-11:1999(E) ANSI/IEEE Std 802.11, 1999 Reaffirmed 2003 edition

- IEEEP802.11e/D6.0 November 2003; Draft amendment to IEEE Std. 802.11, 1999 Edition (Reaff 2003).

- IETF RFC1042: A Standard for the Transmission of IP Datagrams over IEEE 802 Networks.

- IEEE Std 802.1Q-1998s

- IEEEP802.11e/D7.0 January 2004; Draft amendment to IEEE Std. 802.11, 1999 Edition (Reaff 2003).