IEEE P802.11
Wireless LANs

Proposed Text for Periodic Measurements

Date:	July 22, 2003

Author:	Joe Kwak
	InterDigital Communications
	482 Degas
 	Bolingbrook, IL 60440
	Phone: 630-739-4327
	e-mail: joekwak@mindspring.com

Abstract

This document proposes text to permit periodic measurements of the new measurements defined in the TGk draft specification. The concepts for periodic measurements are largely borrowed from similar concepts used in 3GPP.

#### Modify proposed new section of TGk specification text as follows:

##### 7.3.2.19.1 Beacon Request

The Measurement Request field corresponding to a Beacon Request is shown in Figure 0-1 and contains the Measurement Duration and Channel Number for which the request applies. A response to a Beacon Request is a Beacon Report.

|  | Channel Number | Channel Band | Measurement Duration | Scan Mode |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 2 | 1 |

|  | BSSID | Measurement Period | Reporting Condition | Threshhold /Offset | Hysteresis |
| --- | --- | --- | --- | --- | --- |
| Octets: | 6 | 2 | 1 | 1 | 1 |

Figure 0-1 – Measurement Request field format for a Beacon Request

Channel Number indicates the channel number on which the requesting STA instructs the receiving STA to report detected beacons and probe responses.

Channel Band indicates the frequency band, taken fromTable 0-1, in which the receiving STA shall conduct its measurement.

Scan Mode shall be set to the type of scan, according toTable 0-2. The scanning behavior shall be as follows:

- In Active Scan mode, the measuring STA shall transmit a probe request with the broadcast SSID. The measuring STA’s Beacon Report shall contain one information element for each STA from which it detects a beacon or probe response, regardless of whether the probe response was triggered by the measuring STA’s own probe request

- In Passive Scan mode, the measuring STA shall passively receive on the specified channel and return a Beacon Report containing one information element for each STA from which it detects a beacon or probe response. If the measuring channel is also the serving channel, the STA shall concurrently carry out its normal data traffic operation.

- In Beacon Table mode, the measuring STA shall return a Beacon Report containing the current contents of its beacon table without performing additional measurements.

The Measurement Duration field shall be set equal to the duration of the requested measurement, expressed in TUs.

Table 0-1 – Channel Band definitions for radio measurement requests

| Name | Channel Band |
| --- | --- |
| 2.4-GHz Band | 0 |
| 5-GHz Band | 1 |

Table 0-2 – Scan Mode definitions for Beacon Request element

| Name | Scan Mode |
| --- | --- |
| Passive Scan | 0 |
| Active Scan | 1 |
| Beacon Table | 2 |
| Reserved | 3-255 |

BSSID indicates the BSSID of the particular AP for which this measurement is requested.  The BSSID shall be set to the broadcast BSSID when the measurement is to be performed on any AP(s) on this channel.

The Measurement Period indicates whether this measurment is a single measurement event or is a periodic measurment which is repeated each Measurment Period.  The Measurement Period is divided into two subfields: Unit and Period.  The Unit subfield defines the time unit for the Period subfield and consists of the 2 MSBs with the following values:

0: TU

1:  msec

2: sec

3: reserved

The Period subfield consists of the 14 LSBs and is an unsigned integer number representing the repeating time interval for this periodic measurment.  A Period subfield value of 0 shall indicate that the measurment is not periodic but is a single measurement.  A period subfield value of 16383 (3FFF Hex) shall indicate that the measurement is periodic with no requested measurement period; in this case the measurment is performed on a best effort basis and as frequently as conditions will permit.

The Reporting Condition defines when the measured results are to be reported to the requesting STA.  The Reporting Condition values are defined in Table 0-3.

Table 0-3 – Reporting Condition definitions for Beacon Request element

| Condition Description | Reporting Condition |
| --- | --- |
| Report to be issued after each measurment. | 0 |
| Report to be issued when the RCPI level of the measured AP crosses above an absolute threshold with hysteresis. | 1 |
| Report to be issued when the RCPI level of the measured AP crosses below an absolute threshold with hysteresis. | 2 |
| Report to be issued when the PSNI level of the measured AP crosses above an absolute threshold with hysteresis. | 3 |
| Report to be issued when the PSNI level of the measured AP crosses below an absolute threshold with hysteresis. | 4 |
| Report to be issued when the RCPI level of the measured AP crosses above a threshold defined by an offset (with hysteresis) from the serving AP’s RCPI. | 5 |
| Report to be issued when the RCPI level of the measured AP crosses below a threshold defined by an offset (with hysteresis) from the serving AP’s RCPI. | 6 |
| Report to be issued when the PSNI level of the measured AP crosses above a threshold defined by an offset (with hysteresis) from the serving AP’s RCPI. | 7 |
| Report to be issued when the PSNI level of the measured AP crosses below a threshold defined by an offset (with hysteresis) from the serving AP’s RCPI. | 8 |
| Periodic reports (one per measurement) to begin when the RCPI level of the measured AP enters and remains in a range bound by the serving AP’s RCPI and an offset (with hysteresis) from the serving AP’s RCPI. | 9 |
| Periodic reports (one per measurement) to begin when the PSNI level of the measured AP enters and remians in a range bound by the serving AP’s PSNI and an offset (with hysteresis) from the serving AP’s PSNI. | 10 |
| Reserved | 11-255 |

Threshold/Offset provides either the threshold value or the offset value to be used for conditional reporting.  A threshold value is an unsigned 8 bit integer having units equivalent to PSNI or RCPI. An offset value is a signed 7 bit integer in the range [-127, +127].

Hysteresis provides an unsigned 8 bit integer hysteresis value having units equivalent to the units used in the Threshold/Offset field.

##### 7.3.2.19.2 Frame Request

The Measurement Request field corresponding to a Frame Request is shown in Figure 0-2 and contains the Measurement Duration and Channel Number for which the request applies. A response to a Frame Request is a Frame Report.

|  | 0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Channel Number | Channel Band | Measurement Start Time | Randomization Interval | Measurement Duration |
| Octets: | 1 | 1 | 8 | 2 | 2 |

|  | Transmitter Address | BSSID | Measurement Period | Reporting Condition | Threshhold | Hysteresis |
| --- | --- | --- | --- | --- | --- | --- |
| Octets | 6 | 6 | 2 | 1 | 1 | 1 |

Figure 0Error! No text of specified style in document.-2 – Measurement Request field format for a Frame Request

Channel Number indicates the channel number on which the requesting STA instructs the receiving STA to report detected 802.11 frames from any neighboring BSS or IBSS.

Channel Band indicates the frequency band, taken from Table 0-1, in which the receiving STA shall conduct its measurement.

Measurement Start Time shall be set equal to the TSF time the measurement should start if there is no random delay.

Randomization Interval shall be set equal to the desired maximum random delay in the start time, expressed in TUs.

The Measurement Duration field shall be set equal to the duration of the requested measurement, expressed in TUs.

Transmitter Address indicates the individual MAC address of the STA whose transmitted frames are to be measured and reported.  If frames from all STAs are to be reported, Transmitter Address shall  be set to the broadcast BSSID.

BSSID indicates the BSSID of the particular frames for which this measurement is requested.  The BSSID shall be set to the broadcast BSSID when the measurement is to be performed on frames from any BSS on this channel.

The Measurement Period indicates whether this measurment is a single measurement event or is a periodic measurment which is repeated each Measurment Period.  The Measurement Period is divided into two subfields: Unit and Period.  The Unit subfield defines the time unit for the Period subfield and consists of the 2 MSBs with the following values:

0: TU

1:  msec

2: sec

3: reserved

The Period subfield consists of the 14 LSBs and is an unsigned integer number representing the repeating time interval for this periodic measurment.  A Period subfield value of 0 shall indicate that the measurment is not periodic but is a single measurement.  A period subfield value of 16383 (3FFF Hex) shall indicate that the measurement is periodic with no requested measurement period; in this case the measurment is performed on a best effort basis and as frequently as conditions will permit.

The Reporting Condition defines when the measured results are to be reported to the requesting STA.  The Reporting Condition values are defined in Table 0-4.

Table 0-4 – Reporting Condition definitions for Frame Request element

| Condition Description | Reporting Condition |
| --- | --- |
| Report to be issued after each measurment. | 0 |
| Report to be issued when the RCPI level of the measured frame crosses above an absolute threshold with hysteresis. | 1 |
| Report to be issued when the RCPI level of the measured frame crosses below an absolute threshold with hysteresis. | 2 |
| Report to be issued when the PSNI level of the measured frame crosses above an absolute threshold with hysteresis. | 3 |
| Report to be issued when the PSNI level of the measured frame crosses below an absolute threshold with hysteresis. | 4 |
| Reserved | 5-255 |

Threshold provides  the threshold value to be used for conditional reporting.  A threshold value is an unsigned 8 bit integer having units equivalent to PSNI or RCPI.

Hysteresis provides an unsigned 8 bit integer hysteresis value having units equivalent to the units used in the Threshold field.

##### 7.3.2.19.3 Channel Load Request

The Measurement Request field corresponding to a Channel Load Request is shown in Figure 0-3 and contains the Measurement Duration and Channel Number for which the request applies. A response to a Channel Load Request is a Channel Load Report.

|  | Channel Number | Channel Band | Measurement Start Time | Randomization Interval | Measurement Duration |
| --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 8 | 2 | 2 |

|  | Measurement Period | Reporting Condition | Threshhold | Hysteresis |
| --- | --- | --- | --- | --- |
| Octets: | 2 | 1 | 1 | 1 |

Figure 0-3 – Measurement Request field format for a Channel Load Request

Channel Number indicates the channel number on which the AP instructs the receiving STA to issue a Channel Load Report.

Channel Band indicates the frequency band, taken from Table 0-1, in which the receiving STA shall conduct its measurement.

Measurement Start Time shall be set equal to the TSF time the measurement should start if there is no random delay.

Randomization Interval shall be set equal to the desired maximum random delay in the start time, expressed in TUs.

The Measurement Duration field shall be set equal to the duration of the requested measurement, expressed in TUs.

The Measurement Period indicates whether this measurment is a single measurement event or is a periodic measurment which is repeated each Measurment Period.  The Measurement Period is divided into two subfields: Unit and Period.  The Unit subfield defines the time unit for the Period subfield and consists of the 2 MSBs with the following values:

0: TU

1:  msec

2: sec

3: reserved

The Period subfield consists of the 14 LSBs and is an unsigned integer number representing the repeating time interval for this periodic measurment.  A Period subfield value of 0 shall indicate that the measurment is not periodic but is a single measurement.  A period subfield value of 16383 (3FFF Hex) shall indicate that the measurement is periodic with no requested measurement period; in this case the measurment is performed on a best effort basis and as frequently as conditions will permit.

The Reporting Condition defines when the measured results are to be reported to the requesting STA.  The Reporting Condition values are defined in Table 0-5.

Table 0-5 – Reporting Condition definitions for Channel Load Request element

| Condition Description | Reporting Condition |
| --- | --- |
| Report to be issued after each measurment. | 0 |
| Report to be issued when the CCA Busy Fraction of the measured channel crosses above an absolute threshold with hysteresis. | 1 |
| Report to be issued when the CCA Busy Fraction of the measured channel crosses below an absolute threshold with hysteresis. | 2 |
| Reserved | 3-255 |

Threshold provides  the threshold value to be used for conditional reporting.  A threshold value is an unsigned 8 bit integer having units equivalent to CCA Busy Fraction.

Hysteresis provides an unsigned 8 bit integer hysteresis value having units equivalent to the units used in the Threshold field.

##### 7.3.2.19.4 Noise Histogram Request

The Measurement Request field corresponding to an Noise Histogram Request is shown in Figure  and contains the Measurement Duration and Channel Number for which the request applies. A response to an Noise Histogram Request is an Noise Histogram Report.

|  | Channel Number | Channel Band | Measurement Start Time | Randomization Interval | Measurement Duration |
| --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 8 | 2 | 2 |

|  | Measurement Period | Reporting Condition | Selected RPI Bins | Threshhold | Hysteresis |
| --- | --- | --- | --- | --- | --- |
| Octets: | 2 | 1 | 1 | 1 | 1 |

Figure 0-4 – Measurement Request field format for an Noise Histogram Request

Channel Number indicates the channel number on which the AP instructs the receiving STA to report background RF energy and non-802.11 interference on the specified channel.

Channel Band indicates the frequency band, taken from Table 0-1, in which the receiving STA shall conduct its measurement.

Measurement Start Time shall be set equal to the TSF time the measurement should start if there is no random delay.

Randomization Interval shall be set equal to the desired maximum random delay in the start time, expressed in TUs.

The Measurement Duration field shall be set equal to the duration of the requested measurement, expressed in TUs.

The Measurement Period indicates whether this measurment is a single measurement event or is a periodic measurment which is repeated each Measurment Period.  The Measurement Period is divided into two subfields: Unit and Period.  The Unit subfield defines the time unit for the Period subfield and consists of the 2 MSBs with the following values:

0: TU

1:  msec

2: sec

3: reserved

The Period subfield consists of the 14 LSBs and is an unsigned integer number representing the repeating time interval for this periodic measurment.  A Period subfield value of 0 shall indicate that the measurment is not periodic but is a single measurement.  A period subfield value of 16383 (3FFF Hex) shall indicate that the measurement is periodic with no requested measurement period; in this case the measurment is performed on a best effort basis and as frequently as conditions will permit.

The Reporting Condition defines when the measured results are to be reported to the requesting STA.  The Reporting Condition values are defined in Table 0-6.

Table 0-6 – Reporting Condition definitions for Noise Histogram Request element

| Condition Description | Reporting Condition |
| --- | --- |
| Report to be issued after each measurment. | 0 |
| Report to be issued when the sum of the RPI densities in the set of selected RPI bins crosses above an absolute threshold with hysteresis. | 1 |
| Report to be issued when the sum of the RPI densities in the set of selected RPI bins crosses below an absolute threshold with hysteresis. | 2 |
| Reserved | 3-255 |

Selected RPI Bins indicates the subset of the RPI bins which are to be used for conditional reporting.  The RPI bins are defined in 7.3.2.20.4. Noise Histogram Report.  A bit in Selected RPI Bins represents each of the defined RPI bins, bit 0 represents RPI0, bit 1 represents RPI1, and so on.  A "one" in the Selected RPI Bins indicates that the RPI density for the corresponding measured RPI bin is to be used in the summation for the reporting condition.

Threshold provides  the threshold value to be used for conditional reporting.  A threshold value is an unsigned 8 bit integer having units equivalent to the RPI density.

Hysteresis provides an unsigned 8 bit integer hysteresis value having units equivalent to the units used in the Threshold field.

##### 7.3.2.19.5 Hidden Node Request

The Measurement Request field corresponding to a Hidden Node Request is shown Figure 0-5.

|  | Channel Number | Channel Band | Measurement Start Time | Randomization Interval | Measurement Duration | Meaurement Period |
| --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 8 | 2 | 2 |  |

Figure Error! No text of specified style in document.-5 – Measurement Request field format for a Hidden Node Request

Channel Number indicates the channel number on which the AP instructs the receiving STA to report hidden nodes.

Channel Band indicates the frequency band, taken from Table 0-1, in which the receiving STA shall conduct its measurement.

Measurement Start Time shall be set equal to the TSF time the measurement should start if there is no random delay.

Randomization Interval shall be set equal to the desired maximum random delay in the start time, expressed in TUs.

The Measurement Duration field shall be set equal to the duration of the requested measurement, expressed in TUs.

The Measurement Period indicates whether this measurment is a single measurement event or is a periodic measurment which is repeated each Measurment Period.  The Measurement Period is divided into two subfields: Unit and Period.  The Unit subfield defines the time unit for the Period subfield and consists of the 2 MSBs with the following values:

0: TU

1:  msec

2: sec

3: reserved

The Period subfield consists of the 14 LSBs and is an unsigned integer number representing the repeating time interval for this periodic measurment.  A Period subfield value of 0 shall indicate that the measurment is not periodic but is a single measurement.  A period subfield value of 16383 (3FFF Hex) shall indicate that the measurement is periodic with no requested measurement period; in this case the measurment is performed on a best effort basis and as frequently as conditions will permit.