IEEE P802.11
Wireless LANs

Proposed Normative Text for Inclusion of Optimal ChannelTime

Date:	July 21, 2003

Author: 	Arunesh Mishra, Minho Shin, William Arbaugh

U of Maryland

e-Mail: {arunesh, mhsin, waa}@cs.umd.edu

Insun Lee, Kyunghun Jang

Samsung Electronics
	
	e-Mail: {insun,khjang]@samsung.com

Abstract

This document adds the Optimal ChannelTime parameter to the Site Report and Beacon Report Frames.

##### 7.3.2.20.1 Beacon Report

In section 7.3.2.20.1 Beacon Report, change figure “ Measurement Report field format for a Beacon Report” to the following :

|  | Channel Number | Channel Band | Actual Measurement Start Time | Measurement Duration | PHY Type |  | Received Signal Power |  | BSSID |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 8 | 2 | 1 |  | 1 |  | 6 |  |
|  | Parent TSF | Target TSF | Beacon Interval | Capability Information | Optimal ChannelTime | Received Elements |  |  |  |  |
| Octets: | 4 | 8 | 2 | 2 | 1 | variable |  |  |  |  |

Insert the following after line 14 on page 13, i.e. after the description of Capability Information:

OptimalChannelTime is 1 byte value of  time(in TU) between a probe request and the last probe response on one particular channel averaged over all channels as observed by the measuring  STA.

##### 7.3.4 Site Report

Change figure to the following:

|  |  | Element ID |  | Length |  | Optimal ChannelTime |  | AP Information Element #1 |  | … | AP Information Element #N |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 |  | 1 |  | 1 |  | 10 |  | … |  | 10 |

### Site Report Information Element

Change the first paragraph under the above heading to the following:

The Length of the field shall be set to the number of neighbor APs that are being reported times the size of the AP Information Element. OptimalChannelTime is 1 byte value of  time(in TU) that the STA should wait on one channel after sending a probe request when executing an MLME-SCAN operation. The STA can set the MaxChannelTime parameter to be equal to the OptimalChannelTime