IEEE P802.11
Wireless LANs

Structure of MIB extensions in 802.11k

Date:	July 22, 2003

Author:	Joe Kwak, Marian Rudolf
	(InterDigital)

e-Mail: joekwak@mindspring.com; marian.rudolf@interdigital.com

Abstract

This submission contains a text proposal accompanying contribution 03-xx2-r0-K on the structure of future MIB extensions as by the 802.11k draft supplement.

The following editorial comments should be noted:

- This text proposal is edited on top of the latest available P802_11k-D0_2 TGk draft (July 2003).

- Parts altered or changed by this text proposal are highlighted by enabling the Track Changes feature.

- Instructions to the editor are in bold italic.

# Annex D

Update the ASN.1 encoding of the MAC and PHY MIB according to the following unified diff:

--- dot11mib+with+11a+11b+and+11d.mib.orig	2003-05-13 14:28:18.000000000 -0700

+++ dot11mib+with+11a+11b+and+11d.mib	2003-05-14 13:53:38.000000000 -0700

@@ -91,6 +91,7 @@

--  dot11OperationTable   	    ::= { dot11mac 1 }

--  dot11CountersTable    	    ::= { dot11mac 2 }

--  dot11GroupAddressesTable 	::= { dot11mac 3 }

+         --  dot11peerCountersTable	        ::= { dot11mac 4 }

--  Resource Type ID

dot11res 			OBJECT IDENTIFIER 	::= { ieee802dot11 3 }

@@ -993,7 +994,9 @@

dot11MaxTransmitMSDULifetime    Unsigned32,

dot11MaxReceiveLifetime         Unsigned32,

dot11ManufacturerID             DisplayString,

-            		dot11ProductID                  DisplayString }

-- **********************************************************************

-- *    End of dot11OperationEntry  TABLE

-- **********************************************************************

@@ -1369,6 +1402,211 @@

-- **********************************************************************

-- **********************************************************************

+-- *    dot11peerCountersTable TABLE

+-- **********************************************************************

+dot11peerCountersTable OBJECT-TYPE

+        SYNTAX SEQUENCE OF Dot11peerStatsEntry

+        MAX-ACCESS not-accessible

+        STATUS current

+        DESCRIPTION

+

+            "This table contains statistics for peer STAs. The following

+        peer STAs appear in this table:

+

+        1. Authenticated peer STAs must appear in this table. Their

+        entries must not be removed until at least

+        dot11peerStatsTableBSSaging seconds have passed after they are

+        marked as deauthenticated.

+

+        2. Peer STAs joined to the same IBSS as this STA that have

+        sent any data frame to this STA must appear in this table.

+        Entries for these peer STAs may be removed when this STA

+        is no longer joined to the same IBSS, or if

+        dot11peerStatsTableIBSSaging seconds have passed since a data

+        frame was exchanged with the peer STA.

+

+		3. Other peer STAs may be included in this table."

+

+    ::= { dot11mac 4 }

+

+dot11peerCountersEntry OBJECT-TYPE

+        SYNTAX Dot11peerCountersEntry

+        MAX-ACCESS not-accessible

+        STATUS current

+        DESCRIPTION

+            "An entry in the Dot11peerCountersEntry Table.

+            Indexed by ifIndex and the mac address of the remote STA - dot11peerStaAddress"

+        INDEX { ifIndex, dot11peerAddress }

+    ::= { dot11peerCountersTable 1 }

+

+Dot11peerStatsEntry ::= SEQUENCE {

+            dot11peerAddress						MacAddress,

+            dot11peerTransmittedFragmentCount		Counter32,

+            dot11peerMulticastTransmittedFrameCount Counter32,

+            dot11peerFailedCount                    Counter32,

+            dot11peerRetryCount                     Counter32,

+            dot11peerMultipleRetryCount             Counter32,

+            dot11peerFrameDuplicateCount            Counter32,

+            dot11peerRTSSuccessCount                Counter32,

+            dot11peerRTSFailureCount                Counter32,

+            dot11peerACKFailureCount                Counter32,

+            dot11peerReceivedFragmentCount          Counter32,

+            dot11peerMulticastReceivedFrameCount    Counter32,

+		    dot11peerTransmittedFrameCount          Counter32,

+		    dot11peerWEPUndecryptableCount          Counter32,

+	    	dot11peerWEPDecryptionFailureCount		Counter32

}

+

+dot11peerAddress OBJECT-TYPE

+        SYNTAX MacAddress

+        MAX-ACCESS read-create

+        STATUS current

+        DESCRIPTION

+            "MAC address identifying a peer STA."

+    ::= { dot11peerStatsEntry 1 }

+

+dot11peerTransmittedFragmentCount OBJECT-TYPE

+        SYNTAX Counter32

+        MAX-ACCESS read-only

+        STATUS current

+        DESCRIPTION

+            "This counter shall be incremented for an acknowledged MPDU

+            with an individual address in the address 1 field or an MPDU

+            with a multicast address in the address 1 field of type Data

+            or Management."

+    ::= { dot11peerCountersEntry 2 }

+

+dot11peerMulticastTransmittedFrameCount OBJECT-TYPE

+        SYNTAX Counter32

+        MAX-ACCESS read-only

+        STATUS current

+        DESCRIPTION

+            "This counter shall increment only when the multicast bit

+            is set in the destination MAC address of a successfully

+            transmitted MSDU.  When operating as a STA in an ESS, where

+            these frames are directed to the AP, this implies having

+            received an acknowledgment to all associated MPDUs."

+    ::= { dot11peerCountersEntry 3 }

+

+dot11peerFailedCount OBJECT-TYPE

+        SYNTAX Counter32

+        MAX-ACCESS read-only

+        STATUS current

+        DESCRIPTION

+	        "This counter shall increment when an MSDU is not transmitted

+	        successfully due to the number of transmit attempts exceeding

+	        either the  dot11ShortRetryLimit or dot11LongRetryLimit."

+    ::= { dot11peerCountersEntry 4 }

+

+dot11peerRetryCount OBJECT-TYPE

+        SYNTAX Counter32

+        MAX-ACCESS read-only

+        STATUS current

+        DESCRIPTION

+            "This counter shall increment when an MSDU is successfully

+            transmitted after one or more retransmissions."

+    ::= { dot11peerCountersEntry 5 }

+

+dot11peerMultipleRetryCount OBJECT-TYPE

+        SYNTAX Counter32

+        MAX-ACCESS read-only

+        STATUS current

+        DESCRIPTION

+            "This counter shall increment when an MSDU is successfully

+            transmitted after more than one retransmission."

+    ::= { dot11peerCountersEntry 6 }

+

+dot11peerFrameDuplicateCount  OBJECT-TYPE

+        SYNTAX Counter32

+        MAX-ACCESS read-only

+        STATUS current

+        DESCRIPTION

+            "This counter shall increment when a frame is received

+            that the Sequence Control field indicates is a

+            duplicate."

+    ::= { dot11peerCountersEntry 7 }

+

+dot11peerRTSSuccessCount OBJECT-TYPE

+        SYNTAX Counter32

+        MAX-ACCESS read-only

+        STATUS current

+        DESCRIPTION

+            "This counter shall increment when a CTS is received in

+            response to an RTS."

+    ::= {  dot11peerCountersEntry 8 }

+

+dot11peerRTSFailureCount OBJECT-TYPE

+        SYNTAX Counter32

+        MAX-ACCESS read-only

+        STATUS current

+        DESCRIPTION

+	        "This counter shall increment when a CTS is not received in

+	        response to an RTS."

+    ::= { dot11peerCountersEntry 9 }

+

+dot11peerACKFailureCount OBJECT-TYPE

+        SYNTAX Counter32

+        MAX-ACCESS read-only

+        STATUS current

+        DESCRIPTION

+            "This counter shall increment when an ACK is not received

+            when expected."

+    ::= {  dot11peerCountersEntry 10 }

+

+dot11peerReceivedFragmentCount OBJECT-TYPE

+        SYNTAX Counter32

+        MAX-ACCESS read-only

+        STATUS current

+        DESCRIPTION

+            "This counter shall be incremented for each successfully

+            received MPDU of type Data or Management."

+    ::= { dot11peerCountersEntry 11 }

+

+dot11peerMulticastReceivedFrameCount OBJECT-TYPE

+        SYNTAX Counter32

+        MAX-ACCESS read-only

+        STATUS current

+        DESCRIPTION

+            "This counter shall increment when a MSDU is received

+            with the multicast bit set in the destination

+            MAC address."

+    ::= { dot11peerCountersEntry 12 }

+

+dot11peerTransmittedFrameCount OBJECT-TYPE

+        SYNTAX Counter32

+        MAX-ACCESS read-only

+        STATUS current

+        DESCRIPTION

+			"This counter shall increment for each successfully transmitted MSDU."

+    ::= { dot11peerCountersEntry 13 }

+

+dot11peerWEPUndecryptableCount OBJECT-TYPE

+        SYNTAX Counter32

+        MAX-ACCESS read-only

+        STATUS current

+        DESCRIPTION

+	        "This counter shall increment when a frame is received with

+	        the WEP subfield of the Frame Control field set to one and the

+	        WEPOn value for the key mapped to the TA's MAC address

+	        indicates that the frame should not have been encrypted or

+	        that frame is discarded due to the receiving STA not

+	        implementing the privacy option."

+    ::= { dot11peerCountersEntry 14 }

+

+dot11peerLastReceivedRCPI OBJECT-TYPE

+        SYNTAX INTEGER (0..255)

+        MAX-ACCESS read-only

+        STATUS current

+        DESCRIPTION

+	        "The RCPI from the most recently received frame, or 0 if no

+	        frames have been received."

+	::= { dot11peerCountersEntry 15 }

+

+

+-- **********************************************************************

-- *    Resource Type Attribute Templates

-- **********************************************************************

Insert the following into the 802.11 MIB in Annex D, between the section entitled “End of dot11PhyERPEntry Table” and the section entitled “Conformance information”

+-- **********************************************************************

+-- *    dot11peerPhyMeasTable TABLE

+-- **********************************************************************

+dot11peerPhyMeasTable OBJECT-TYPE

+        SYNTAX SEQUENCE OF Dot11peerPhyMeasEntry

+        MAX-ACCESS not-accessible

+        STATUS current

+        DESCRIPTION

+

+            "This table contains statistics for peer STAs. The following

+        peer STAs appear in this table:

+

+        1. Authenticated peer STAs must appear in this table. Their

+        entries must not be removed until at least

+        dot11peerStatsTableBSSaging seconds have passed after they are

+        marked as deauthenticated.

+

+        2. Peer STAs joined to the same IBSS as this STA that have

+        sent any data frame to this STA must appear in this table.

+        Entries for these peer STAs may be removed when this STA

+        is no longer joined to the same IBSS, or if

+        dot11peerStatsTableIBSSaging seconds have passed since a data

+        frame was exchanged with the peer STA.

+

+		3. Other peer STAs may be included in this table."

+

+    ::= { dot11phy 15 }

+

+dot11peerPhyMeasEntry OBJECT-TYPE

+        SYNTAX Dot11peerPhyMeasEntry

+        MAX-ACCESS not-accessible

+        STATUS current

+        DESCRIPTION

+            "An entry in the Dot11peerPhyMeasEntry Table.

+            Indexed by ifIndex and the mac address of the remote STA –

+            dot11peerStaAddress"

+        INDEX { ifIndex, dot11peerAddress }

+    ::= { dot11peerPhyMeasTable 1 }

+

+Dot11peerPhyMeasEntry ::= SEQUENCE {

+          	dot11peerAddress				MacAddress,

+	    	dot11peerReceivedByteCount      		Counter32,

+	    	dot11peerTransmittedByteCount     		Counter32,

+	    	dot11peerReceivedBeaconCount			Counter32,

+	    	dot11peerLastReceivedRCPI			INTEGER (0..255) }

+

+dot11peerAddress OBJECT-TYPE

+        SYNTAX MacAddress

+        MAX-ACCESS read-create

+        STATUS current

+        DESCRIPTION

+            "MAC address identifying a peer STA."

+    ::= { dot11peerStatsEntry 1 }

+

+-- **********************************************************************

-- *    Resource Type Attribute Templates

-- **********************************************************************

Insert the following into the 802.11 MIB in Annex D, between the end of the section entitled “Conformance” and the section entitled “Textual conventions from 802 definitions”

+ -- RRM Attributes

+    -- DEFINED AS "The RRM object class provides the necessary support

+    -- for required RRM operational information."

+

+    dot11rrm OBJECT IDENTIFIER ::= {ieee802dot11 6}

+

+    -- rrm GROUPS

+    -- dot11BSSGlobalStatusTable ::= {dot11rrm 1}

+    -- dot11BSSGlobalStatisticsTable ::= {dot11rrm 2}

+    -- dot11BSSRrmConfigurationTable ::= {dot11rrm 3}

+    -- dot11BSSRrmActionsTable ::= {dot11rrm 4}

+    -- dot11peerCapabilityTable ::= {dot11rrm 5}

+    -- dot11peerConfigurationTable ::= {dot11rrm 6}

Insert the following into the 802.11 MIB in Annex D, between the end of the section entitled “Groups - units of conformance” and the section entitled “End of 80211 MIB”

+-- **********************************************************************

+-- *    RRM Attribute Templates

+-- **********************************************************************

+-- **********************************************************************

+-- *    dot11BSSGlobalStatusTable TABLE

+-- **********************************************************************

+dot11BSSGlobalStatusTable OBJECT-TYPE

+        SYNTAX SEQUENCE OF Dot11 BSSGlobalStatusEntry

+        MAX-ACCESS not-accessible

+        STATUS current

+        DESCRIPTION

+

+            "This table contains information pertaining to the global status

+            of STAs present in the BSS.”

+

+    ::= { dot11rrm 1 }

+

+dot11peer BSSGlobalStatusEntry OBJECT-TYPE

+        SYNTAX BSSGlobalStatusEntry

+        MAX-ACCESS not-accessible

+        STATUS current

+        DESCRIPTION

+            "An entry in the Dot11BSSGlobalStatusTable.

+            Indexed by ifIndex”

+        INDEX { ifIndex }

+    ::= { dot11 BSSGlobalStatusTable 1 }

+

+Dot11 BSSGlobalStatusEntry ::= SEQUENCE {

+          Editors note: To be completed later.

+                                        }

+

+-- **********************************************************************

-- *    Resource Type Attribute Templates

-- **********************************************************************

+-- **********************************************************************

+-- *    dot11BSSGlobalStatisticsTable TABLE

+-- **********************************************************************

+dot11BSSGlobalStatisticsTable OBJECT-TYPE

+        SYNTAX SEQUENCE OF Dot11 BSSGlobalStatisticsEntry

+        MAX-ACCESS not-accessible

+        STATUS current

+        DESCRIPTION

+

+            "This table contains statistics and measurements relevant

+             to the performance of the BSS.”

+

+    ::= { dot11rrm 2 }

+

+dot11peer BSSGlobalStatisticsEntry OBJECT-TYPE

+        SYNTAX BSSGlobalStatisticsEntry

+        MAX-ACCESS not-accessible

+        STATUS current

+        DESCRIPTION

+            "An entry in the Dot11BSSGlobalStatisticsTable.

+            Indexed by ifIndex”

+        INDEX { ifIndex }

+    ::= { dot11BSSGlobalStatisticsTable 1 }

+

+Dot11 BSSGlobalStatisticsEntry ::= SEQUENCE {

+          Editors note: To be completed later.

+                                        }

+

+-- **********************************************************************

-- *    Resource Type Attribute Templates

-- **********************************************************************

+-- **********************************************************************

+-- *    dot11BSSRrmConfigurationTable TABLE

+-- **********************************************************************

+dot11BSSRrmConfigurationTable OBJECT-TYPE

+        SYNTAX SEQUENCE OF Dot11 BSSRrmConfigurationEntry

+        MAX-ACCESS not-accessible

+        STATUS current

+        DESCRIPTION

+

+            "This table configuration and operation settings for RRM.”

+

+    ::= { dot11rrm 3 }

+

+dot11peer BSSRrmConfigurationEntry OBJECT-TYPE

+        SYNTAX BSSRrmConfigurationEntry

+        MAX-ACCESS not-accessible

+        STATUS current

+        DESCRIPTION

+            "An entry in the Dot11BSSRrmConfigurationTable.

+            Indexed by ifIndex”

+        INDEX { ifIndex }

+    ::= { dot11BSSRrmConfigurationTable 1 }

+

+Dot11 BSSRrmConfigurationEntry ::= SEQUENCE {

+            	dot11peerStatsTableBSSaging	Unsigned32,

+            	dot11peerStatsTableIBSSaging	Unsigned32 }

+dot11peerStatsTableBSSaging OBJECT-TYPE

+		SYNTAX Unsigned32

+		MAX-ACCESS read-write

+		STATUS current

+		DESCRIPTION

+			"The peerStatsTableBSSaging indicates the period of time

+			which entries for peer STAs that are in the peerStatsTable

+			because the peer STA was authenticated or associated with the

+			local STA must be kept in the table after the peer STA

+			becomes disassociated. The peer STA entry may be kept in the

+			peerStatsTable for longer than this time."

+	::= { dot11RrmConfigurationEntry 1 }

+

+dot11peerStatsTableIBSSaging OBJECT-TYPE

+		SYNTAX Unsigned32

+		MAX-ACCESS read-write

+		STATUS current

+		DESCRIPTION

+

+			"The peerStatsTableIBSSaging indicates the period of time

+			which entries for peer STAs that are in the peerStatsTable

+			because the peer STA was in an IBSS that the local STA was

+			joined to, and send a data frame to the local STA, or the

+			local STA attempted to send any frame to the peer STA

+			must be kept in the table after the peer STA

+			last communicated with the local STA. The peer STA entry

+			may be kept in the

+			peerStatsTable for longer than this time."

+	::= { dot11RRmConfigurationEntry 2 }

+

+-- **********************************************************************

-- *    Resource Type Attribute Templates

-- **********************************************************************

+-- **********************************************************************

+-- *    dot11BSSRrmActionsTable TABLE

+-- **********************************************************************

+dot11BSSRrmActionsTable OBJECT-TYPE

+        SYNTAX SEQUENCE OF Dot11 BSSRrmActionsEntry

+        MAX-ACCESS not-accessible

+        STATUS current

+        DESCRIPTION

+

+            "This table is the generic interface for RRM action settings

+             as set by an internal or external Network Management Entity.”

+

+    ::= { dot11rrm 4 }

+

+dot11peer BSSRrmActionsEntry OBJECT-TYPE

+        SYNTAX BSSRrmActionsEntry

+        MAX-ACCESS not-accessible

+        STATUS current

+        DESCRIPTION

+            "An entry in the Dot11BSSRrmActionsTable.

+            Indexed by ifIndex”

+        INDEX { ifIndex }

+    ::= { dot11BSSRrmActionsTable 1 }

+

+Dot11 BSSRrmActionsEntry ::= SEQUENCE {

+          Editors note: To be completed later.

+                                        }

+

+

+-- **********************************************************************

-- *    Resource Type Attribute Templates

-- **********************************************************************

+-- **********************************************************************

+-- *    dot11peerCapabilityTable TABLE

+-- **********************************************************************

+dot11peerCapabilityTable OBJECT-TYPE

+        SYNTAX SEQUENCE OF Dot11peerCapabilityEntry

+        MAX-ACCESS not-accessible

+        STATUS current

+        DESCRIPTION

+

+            "This table contains capability information for peer STAs. The following

+        peer STAs appear in this table:

+

+        1. Authenticated peer STAs must appear in this table. Their

+        entries must not be removed until at least

+        dot11peerStatsTableBSSaging seconds have passed after they are

+        marked as deauthenticated.

+

+        2. Peer STAs joined to the same IBSS as this STA that have

+        sent any data frame to this STA must appear in this table.

+        Entries for these peer STAs may be removed when this STA

+        is no longer joined to the same IBSS, or if

+        dot11peerStatsTableIBSSaging seconds have passed since a data

+        frame was exchanged with the peer STA.

+

+		3. Other peer STAs may be included in this table."

+

+    ::= { dot11rrm 5 }

+

+dot11peerCapabilityEntry OBJECT-TYPE

+        SYNTAX Dot11peerCapabilityEntry

+        MAX-ACCESS not-accessible

+        STATUS current

+        DESCRIPTION

+            "An entry in the Dot11peerCapabilityEntry Table.

+            Indexed by ifIndex and the mac address of the remote STA –

+            dot11peerStaAddress"

+        INDEX { ifIndex, dot11peerAddress }

+    ::= { dot11peerCapabilityTable 1 }

+

+Dot11peerCapabilityEntry ::= SEQUENCE {

+          	dot11peerAddress				MacAddress }

+

+dot11peerAddress OBJECT-TYPE

+        SYNTAX MacAddress

+        MAX-ACCESS read-create

+        STATUS current

+        DESCRIPTION

+            "MAC address identifying a peer STA."

+    ::= { dot11peerCapabilityEntry 1 }

+

+-- **********************************************************************

-- *    Resource Type Attribute Templates

-- **********************************************************************

+-- **********************************************************************

+-- *    dot11peerConfigurationTable TABLE

+-- **********************************************************************

+dot11peerConfigurationTable OBJECT-TYPE

+        SYNTAX SEQUENCE OF Dot11peerConfigurationEntry

+        MAX-ACCESS not-accessible

+        STATUS current

+        DESCRIPTION

+

+            "This table contains configuration information for peer STAs.

+             The following peer STAs appear in this table:

+

+        1. Authenticated peer STAs must appear in this table. Their

+        entries must not be removed until at least

+        dot11peerStatsTableBSSaging seconds have passed after they are

+        marked as deauthenticated.

+

+        2. Peer STAs joined to the same IBSS as this STA that have

+        sent any data frame to this STA must appear in this table.

+        Entries for these peer STAs may be removed when this STA

+        is no longer joined to the same IBSS, or if

+        dot11peerStatsTableIBSSaging seconds have passed since a data

+        frame was exchanged with the peer STA.

+

+		3. Other peer STAs may be included in this table."

+

+    ::= { dot11rrm 6 }

+

+dot11peerConfigurationEntry OBJECT-TYPE

+        SYNTAX Dot11peerConfigurationEntry

+        MAX-ACCESS not-accessible

+        STATUS current

+        DESCRIPTION

+            "An entry in the Dot11peerConfigurationEntry Table.

+            Indexed by ifIndex and the mac address of the remote STA –

+            dot11peerStaAddress"

+        INDEX { ifIndex, dot11peerAddress }

+    ::= { dot11peerConfigurationTable 1 }

+

+Dot11peerConfigurationEntry ::= SEQUENCE {

+          	dot11peerAddress				MacAddress }

+

+dot11peerAddress OBJECT-TYPE

+        SYNTAX MacAddress

+        MAX-ACCESS read-create

+        STATUS current

+        DESCRIPTION

+            "MAC address identifying a peer STA."

+    ::= { dot11peerConfigurationEntry 1 }

+

+-- **********************************************************************

-- *    Resource Type Attribute Templates

-- **********************************************************************