IEEE P802.11
Wireless LANs

Proposed Normative Text for EDCA

Date:	July 21, 2003

Author: 	Matthew Fischer

190 Mathilda Place, Sunnyvale, CA 94086
	Phone: 408 543 3370
	Fax: 
	e-Mail: mfischer@broadcom.com

Menzo Wentink

Intersil
	1A Rembrandtlaan, 3511 AB, Bilthoven, Netherlands

Phone: +31 30 225 9752
	Fax: 
	e-Mail: mwentink@intersil.com

Abstract

The changes described in this document are relative to 802.11e D4.4.

Change the sixth paragraph of 7.3.2.14 to read as follows

The AIFSN[AC] values field specifies 4 AIFSN values, for access categories 0 through 3, respectively.  Each AIFSN value is 1 octet in length and contains an unsigned integer. A QSTA shall update the dot11EDCATableAIFSN MIB values according to the AIFSN[AC] values in the most recent EDCA parameter set element received by the QSTA from the QAP of a QBSS. The minimum allowed value for AIFSN[AC] shall be 1.

Within clause 7.3.2.14, modify table 20.1Default EDCA Parameter Set by changing the AIFSN value for AC 0 from 2 to 3, the AIFSN value for AC 1 from 1 to 2, the AIFSN value for AC 2 from 1 to 2, and the AIFSN value for AC3 from 1 to 2.

#### 9.10.1.3 Obtaining an EDCA TXOP

Each channel access timer shall maintain a backoff function (timer), which has a value measured in backoff slots.

The duration AIFS[AC] is a duration derived from the value AIFSN[AC] by the relation

AIFS[AC] = AIFSN[AC] × aSlotTime + aSIFSTime

An EDCA TXOP is granted to a channel access function when the channel access function determines that it shall initiate the transmission of a frame exchange sequence. Transmission initiation shall be determined according to the following rules:

On specific slot boundaries, each channel access function shall make a determination to perform one and only one of the following functions:

- Initiate the transmission of a frame exchange sequence for that access function

- Decrement the backoff timer for that access function

- Invoke the backoff procedure due to an internal collision

- Do nothing for that access function

The specific slot boundaries at which exactly one of these operations shall be performed are defined as follows, for each channel access function:

- Following AIFS[AC] of medium idle indication after the end of the last indicated busy medium which was the result of a frame reception which not was received in error; or

- Following EIFS – DIFS + AIFS[AC] of medium idle indication after the last indicated busy medium which was the result of a frame reception which was received in error; or

- at the latter of the end of the ACK-Timeout interval and AIFS[AC] after the PHY-TXEND.indication, and the end of the first AIFS[AC] of IDLE time following the PHY-TXEND.indication, or at AIFS[AC] after the PHY-RXEND.indication when a PHY-RXSTART.indication occurs as specified in subclause 9.2.8, when  any other channel access function at this QSTA transmitted a frame requiring acknowledgement immediately previously.

- following AIFS[AC] of medium idle indication after the end of the last indicated busy medium which was the result of a transmission of a frame for any channel access function and which did not require an acknowledgement; or

- following aSlotTime of medium idle indication which occurs immediately after a decrement of the backoff counter for that channel access function

At each of the above-described specific slot boundaries, each channel access function shall initiate a transmission sequence, if:

- there is a frame available for transmission at that channel access function, and

- the backoff timer for that channel access function has a value of zero, and

- initiation of a transmission sequence is not allowed to commence at this time for a channel access function of higher UP

At each of the above-described specific slot boundaries, each channel access function shall decrement the backoff timer, if:

  - The backoff timer for that channel access function has a non-zero value

At each of the above-described specific slot boundaries, each channel access function shall invoke the backoff procedure due to an internal collision, if:

- There is a frame available for transmission at that channel access function, and

- the backoff timer for that channel access function has a value of zero, and

- initiation of a transmission sequence is allowed to commence at this time for a channel access function of higher UP

At each of the above-described specific slot boundaries, each channel access function shall do nothing, if none of the above actions is taken.

- .

Any of these conditions might be met immediately at the time a frame is requested to be transmitted in the case that the medium is already idle, or it might be necessary to wait for the expiry of the relevant backoff timer.

Figure 62.2 – EDCA Mechanism Timing Relationships

An example showing the relationship between AIFS, AIFSN, DIFS and slot times immediately following a medium busy condition (and assuming that medium busy condition was not caused by a frame in error) is shown in Figure 62.2. In this case, with AIFSN=1, the channel access function may decrement the backoff counter for the first time at

aSlotTime

following the end of the medium busy condition.

If, in this example, the backoff counter contained a value of 1 at the time the medium became idle, transmission would start as a result of an EDCA TXOP on-air at a time

aSIFSTime + 2 × aSlotTime

following the end of the medium busy condition.

#### 9.10.1.5 Backoff Procedure

Each channel access function shall maintain a state variable CW[AC], which shall be initialized to the value of the parameter CWmin[AC].

If a frame is successfully transmitted for a specific AC, indicated by either the successful reception of a CTS in response to an RTS, the successful reception of an Ack in response to a unicast MSDU or MMPDU, or the successful reception of a BlockAck or ACK in response to a BlockAckReq, or by transmitting a multicast frame or a frame with “no acknowledgement” policy, CW[AC] shall be reset to CWmin[AC].

The backoff procedure shall be invoked for a channel access function when either:

a)	A frame with that AC is requested to be transmitted and the medium is busy as indicated by either physical or virtual carrier sense, and the backoff timer has a value of zero for that AC.

b)	A frame is transmitted for that AC and a continuation of the EDCA TXOP cannot be used to send the following frame.

c)	The transmission of a frame of that AC fails, indicated by a failure to receive a CTS in response to an RTS, or a failure to receive an ACK that was expected in response to a unicast MSDU or MMPDU, or failure to receive a BlockAck or ACK in response to a BlockAckReq.

d)	The transmission attempt collides internally with another channel access function of higher AC, that is, two channel access functions in the same QSTA are granted a TXOP at the same time.

If the backoff procedure is invoked for either reason a) or b) above, the value of CW[AC] shall be left unchanged.

If the backoff procedure is invoked because of a failure event (either reason c or d above) the value of CW[AC] shall be updated before invoking the backoff procedure:

a)	if the short or long retry count for the QSTA has reached aShortRetryLimit or aLongRetryLimit respectively, CW[AC] shall be reset to CWmin[AC].

b)	Otherwise,

1)	if CW[AC] is less than CWmax[AC], CW[AC] shall be set to the value (CW[AC]+1)*2-1;

2)	if CW[AC] is equal to CWmax[AC], CW[AC] shall remain unchanged for the remainder of any retries.

Following the update of the value of CW[AC], the backoff timer is set to an integer value chosen randomly with a uniform distribution taking values in the range (0,CW[AC]) inclusive.

If an internal collision occurs between channel access functions, the QSTA shall increment either the MSDU or STA short or long retry counters, as appropriate.