IEEE P802.11
Wireless LANs

Title

Date:	July 3, 2003

Author:	Bob O’Hara
	Airespace
	110 Nortech Parkway
                                                             San Jose, CA 95134
	Phone: +1 408 635 2025
	Fax: +1 408 635 2020
	e-Mail: bob@airepsace.com

Interpretation Number: 1-07/03 (delayed CFP Beacon)

Topic: 802.11 Beacon

Relevant Clauses: 9.3.2.1

Classification: unambiguous

Interpretation Request

original line : In the case of a busy medium due to DCF traffic, the beacon
shall be delayed for the time required to  complete the current DCF frame
exchange.


I think there is no direct answer about the following case.

Q : When a PCF beacon(CFPeriod=0, DTIM COunt=0) is defered due to a busy
medium(DCF), PC shall use xxxxxxxxxx delay to start the CFP after this DCF
medium busy.

A :
<1> served as normal DCF beacon, use DIFS+random backoff delay
<2> served as normal PCF beacon while not deffered by medium, use PIFS
delay

Interpretation for IEEE STD 802.11-1999 (reaffirmed 2003)

Clause 9.3.2.1 says: “When the medium is determined to be idle for one PIFS period, the PC shall transmit a Beacon frame containing the CF Parameter Set element and a DTIM element.”  This is a clear statement that the Beacon is to be transmitted immediately after the medium has been idle for a single PIFS period.

This area of the standard is being modified by the work going on 802.11 Task Group e.  You may be interested in following that work as it progresses.