IEEE P802.11
Wireless LANs

A Proposal for STA Statistics Measurements

Date:	July 20, 2003

Author:	Simon Black, Hasse Sinivaara
	Nokia

e-Mail: simon.black@motix.demon.co.uk, hasse.sinivaara@nokia.com

Abstract

This submission contains proposed normative text for STA statistics gathering.

The following editorial comments should be noted:

1.	Italic text outlined within a frame is commentary and is not a part of the normative text proposal

2.	New tables and figures are numbered sequentially with a preceding ‘+’ starting at +1. It is left to the TGk editor to correct these on insertion of any adopted text. Edited figures and tables take their original numbering.

3.	This text assumes a baseline comprising current edition of IEEE Std 802.11 1999 Edition and the 802.11a, 802.11b, 802.11b-cor1 and 802.11d supplements.

References to this Submission

[1]	11-03-232r0-K-STA_MIB_Statistics

[2]	11-03-338r0-K-STA_Statistics_and_RRM_Information_Distribution

Insert the following new subsections under Section 7.3.2, adjusting the subsection numbers, figure numbers and table numbers as necessary:

#### 7.3.2.19 Measurement Request Element

## This submission builds upon the Measurement Request-Response mechanism in the current Draft D0.2 and assumes the presence of that text.

Table +1 – Measurement Type definitions for Measurement Request element

| Measurement Name | Measurement Type |
| --- | --- |
| Specified in current draft | 0 - 7 |
| STA Statistics Request | 8 |
| Reserved | 10 - 255 |

##### 7.3.2.19.5 STA Statistics Request

The STA Statistics Request Measurement enables a STA to query statistics and counters in a peer STA MIB. The Measurement Request field corresponding to a STA Statistics Request is shown in Figure +1.

|  | Measurement Duration | Group Identity |
| --- | --- | --- |
| Octets: | 2 | 1 |

Figure +1 – Measurement Request field format for a STA Statistics Request

The Measurement Duration field shall be set equal to the duration of the requested measurement in TUs.

Group Identity indicates the requested statistics group according to Table +2.

Table +2 – Group Identity for a STA Statistics Report

| Statistics Name | Group Identity |
| --- | --- |
| STA Counters from dot11CountersTable | 0 |
| Reserved | 1 - 255 |

#### 7.3.2.20 Measurement Report Element

Table -1 – Measurement Type definitions for Measurement Report element

| Measurement Name | Measurement Type |
| --- | --- |
| Specified in IEEE802.11k D0.2 | 0 - 7 |
| STA Statistics Report | 8 |
| Reserved | 9 - 255 |

##### 7.3.2.20.5 STA Statistics Report

The format of the Measurement Report field of an STA Statistics Report is shown in Figure +2.

|  | Measurement Duration | Statistics Group Data |
| --- | --- | --- |
| Octets: | 2 | Variable |

Figure +2 – Measurement Report field format for a STA Statistics Report

Measurement Duration shall be set equal to the duration over which the Statistics Data was measured, expressed in TUs. A Measurement duration of 0 indicates instantaneous data.

Statistics Group Data shall contain the requested statistics from MIB related to the interface on which the request was received according to Table +3.

Table +3 – Group Identity for a STA Statistics Report

| Group Identity Requested | Statistics Returned |
| --- | --- |
| 0 | Dot11CountersEntry for the Interface on which the STA Statistics Request was received with the exception of WEPUndecryptableCount: dot11TransmittedFragmentCount (Counter32), dot11MulticastTransmittedFrameCount (Counter32), dot11FailedCount (Counter32), dot11RetryCount (Counter32), dot11MultipleRetryCount (Counter32), dot11FrameDuplicateCount (Counter32), dot11RTSSuccessCount (Counter32), dot11RTSFailureCount (Counter32), dot11ACKFailureCount (Counter32) dot11ReceivedFragmentCount (Counter32), dot11MulticastReceivedFrameCount (Counter32), dot11FCSErrorCount (Counter32), dot11TransmittedFrameCount (Counter32). |
| 1 – 255 | None |

#### 11.7.7.6 STA Statistics Report

A STA receiving a STA Statistics Request shall respond with a Radio Measurement Report frame containing one or more Measurement (STA Statistics) Report elements. Each element contains a group of STA Statistics.