IEEE P802.11
Wireless LANs

Proposed Normative Text for NAV Protection

Date:	July 22, 2003

Author:	Mathilde Benveniste
	Avaya Labs - Research
	233 Mt. Airy Road, Basking Ridge, NJ USA
	Phone: 973-761-6105
	e-Mail: benveniste@ieee.org

Matthew Sherman
	AT&T Labs - Research
	180 Park Ave, Florham Park, NJ USA
	Phone: 973-236-6925
	e-Mail: mjsherman@att.com

Abstract

This document contains the material proposed to GTE for inclusion in the draft in the form of insertions into and replacements for material in IEEE Std 802.11-1999, as updated by IEEE Std 802.11a-1999, IEEE Std 802.11b-1999 (already present in TGE Draft 3.3).

Editorial notes appear in bold italic Times New Roman font, informative notes appear in normal Arial font, and normative text appears in normal Times New Roman font.  Open issues are highlighted using red text in normal Arial font, and begin with "OPEN ISSUE:".  Added text appears in red.  Deleted text appears in blue with an underline.

Insert at the beginning of 9.2.2.1

A QSTA may track multiple NAV values.  The NAV setting of the QSTA is the maximum of these NAV values for the QSTA. When a QSTA retains a new NAV value that QSTA shall also save the TXOP holder address setting the NAV value. A QSTA may retain a subset consisting of the longest NAV values. A NAV value is removed when it expires.  A NAV value is reset to 0 when such a request comes from the address stored for the NAV value, unless at least one NAV value has been discarded and none of the non-discarded NAV values has expired.

When a QSTA updates it’s sets a NAV value setting using the duration value from a QoS (+)CF-Poll containing the BSSID of this a QBSS, that QSTA shall also save the TXOP holder address, which is the MAC address from the Address1 field of the frame containing the QoS (+)CF-Poll. If an RTS frame is received and the MAC address in the TA field in the RTS frame matches the saved TXOP holder address, the QSTA shall send the CTS frame after SIFS, without regard for, and without resetting, its NAV. The saved TXOP holder address shall be cleared whenever the NAV is reset or when the NAV counts down to 0. A non-AP QSTA that receives a CF-End frame containing the BSSID of this QBSS shall reset its NAV the corresponding NAV value to 0. A non-AP QSTA that receives a QoS CF-Poll with a MAC address in the Address1 which matches the HC's MAC address and a Duration/ID value equal to zero shall reset the NAV corresponding to the address in Address 1 of the received QoS CF-Poll clear its NAV.