IEEE P802.11
Wireless LANs

Fixes for IBSS Normative Text

Date:	July 22, 2003

Author:	Patrick Worfolk
	Kiwi Networks
	1684 Dell Avenue, Campbell, CA
	Phone: 408-379-5800
	Fax: 408-871-8305
	e-Mail: patrick@kiwinetworks.com

Abstract

This document contains proposed normative text for fixing a number of places in the 802.11 TGk 0.4 draft where IBSSs were overlooked.

7.3.2.19.3

Modify the second paragraph as indicated below:

Channel Number indicates the channel number on which the  requesting STA instructs the receiving STA to issue a Channel Load Report.

7.3.2.19.4

Modify the second paragraph as indicated below:

Channel Number indicates the channel number on which the  requesting STA instructs the receiving STA to report background RF energy and non-802.11 interference on the specified channel.

7.3.2.19.5

Modify the second paragraph as indicated below:

Channel Number indicates the channel number on which the  requesting STA instructs the receiving STA to report hidden nodes.

7.3.2.20.1

Modify the tenth paragraph as indicated below:

Parent TSF contains the lower 4 bytes of the  measuring STA’s TSF value at the time the measuring STA received the beacon or probe response frame.

7.3.2.20.5

Modify the sixth paragraph as indicated below:

Hidden Station Address contains the 6-byte MAC address of the destination of the 802.11 frame  received by the measuring STA without  detecting the subsequent acknowledgement.

11.7.6

Modify the last  paragraph as indicated below:

Since measurements on non-serving channels could potentially degrade a station’s performance, non-serving channel measurements should be requested sparingly and for short durations. Since measurements on the serving channel execute concurrently with normal traffic processing, serving channel measurements may be requested more frequently and for longer durations. If desired, the  requesting STA may issue periodic concurrent measurement requests to achieve near-continuous reporting.

11.7.7.1

Modify the first paragraph as indicated below:

A STA receiving a Beacon Request shall respond with a Radio Measurement Report frame containing one Measurement (Beacon) Report element for each BSSID from which it detected a beacon or probe response. If requested on the serving channel, the measuring STA shall include a Radio Measurement (Beacon) Report element describing its  BSS beacon. In this case, the element shall contain all of the Beacon Report fixed fields and may optionally eliminate the Received Elements.

11.7.7.5

Modify the second paragraph as indicated below:

A measuring STA detects a potential hidden node when it receives a  frame to which it detects no acknowledgement. To minimize false alarms, the measuring STA shall issue a Hidden Node report only when the detected downlink frame is an initial transmission. It shall not issue a Hidden Node report when the downlink frame is a retransmission.