IEEE P802.11
Wireless LANs

Proposed Draft Text for End-of-Service-Period Bit Explanation

Date:	July 23, 2003

Author:	Sai Shankar

Amjad Soomro

Philips
	345 Scarborough Road, Briarcliff Manor, NY 10510
	Phone: +1 914 945 6310
	e-Mail: [sai.shankar, amjad.soomro@philips.com

Abstract

This document proposes resolution for comment number 1103 on letter ballot 39 on Tge draft 4.0.

1. Proposed Resolution

The dual token bucket is composed of two token buckets: 1) The first token bucket value is always set to 0; and, 2) the second token bucket value is set to the value equal to the maximum burst size. The tokens arrive at the first bucket, which has 0 bucket size, at the peak data rate and the tokens arrive at the second bucket at the mean data rate. If an incoming MSDU of size L of the stream arrives at the first token bucket, it needs to wait till L tokens arrive from the first generator. If it does not find a token at its time of arrival it needs to wait a maximum of L/P time units before the tokens are generated and it is passed to the second token bucket policer. If there are at least L tokens in the second token bucket, it is immediately sent to the MAC buffer or it needs to wait for the maximum time, L/ before it is sent to the MAC buffer.

P – Peak Data Rate

Mean Data Rate

Maximum Burst Size

First Token Bucket

Second Token Bucket

Arriving Trafffic Stream

To MAC Buffer

Figure 1 - Twin Token Bucket Policer

2. References

[RFC 3290] Bernet, Y., Blake, S., Grossman, D., and Smith, A. "An Informal Management Model for Diffserv Routers", RFC 3290, May 2002.

[RFC 2215] Shenker, and J. Wroclawski "General Characterization Parameters for

Integrated Service Network Elements", RFC 2215, September 1997.

[RFC 2212] Shenker, S., Partridge, C., and R. Guerin "Specification

of the Guaranteed Quality of Service", RFC 2212, September 1997.