IEEE P802.11
Wireless LANs

Proposed Changes to Annex F to Resolve Ballot Comments

Date:	July 23, 2003

Author:	Paul Lambert

Airgo Networks,

900 Arastradero Road, Palo Alto, CA, 94304
Phone: 650-475-2006

e-Mail: PaulLambert@AirgoNetworks.Com

Abstract

Annex F updated to include comment resolutions.  Updates made to 11-03-0485r0.

<The following is proposed replacement text for in Annex F to resolve ballot comments>

# Annex F RSNA reference implementations and test vectors

## F.1 (informative) TKIP Temporal Key Mixing Function reference implementation and test vector

This clause provides a “C” language reference implementation of the temporal key mixing function.

/***********************************************************************

Contents:    Generate 802.11 per-packet RC4 key hash test vectors

Date:        April 19, 2002

Authors:     Doug Whiting,   Hifn

Russ Housley,   RSA Labs

Niels Ferguson, MacFergus

Doug Smith,     Cisco

Notes:

This code is released to the public domain use, built solely out of

the goodness of our hearts for the benefit of all mankind. As such,

there are no warranties of any kind given on the correctness or

usefulness of this code.

This code is written for pedagogical purposes, NOT for performance.

************************************************************************/

#include <stdio.h>

#include <stdlib.h>

#include <string.h>

#include <assert.h>

#include <time.h>

typedef unsigned char  byte;   /*  8-bit byte (octet)  */

typedef unsigned short u16b;   /* 16-bit unsigned word */

typedef unsigned long  u32b;   /* 32-bit unsigned word */

/* macros for extraction/creation of byte/u16b values  */

#define RotR1(v16)   ((((v16) >> 1) & 0x7FFF) ^ (((v16) & 1) << 15))

#define   Lo8(v16)   ((byte)( (v16)       & 0x00FF))

#define   Hi8(v16)   ((byte)(((v16) >> 8) & 0x00FF))

#define  Lo16(v32)   ((u16b)( (v32)       & 0xFFFF))

#define  Hi16(v32)   ((u16b)(((v32) >>16) & 0xFFFF))

#define  Mk16(hi,lo) ((lo) ^ (((u16b)(hi)) << 8))

/* select the Nth 16-bit word of the temporal key byte array TK[]   */

#define  TK16(N)     Mk16(TK[2*(N)+1],TK[2*(N)])

/* S-box lookup: 16 bits --> 16 bits */

#define _S_(v16)     (Sbox[0][Lo8(v16)] ^ Sbox[1][Hi8(v16)])

/* fixed algorithm "parameters" */

#define PHASE1_LOOP_CNT   8    /* this needs to be "big enough"     */

#define TA_SIZE           6    /*  48-bit transmitter address       */

#define TK_SIZE          16    /* 128-bit temporal key              */

#define P1K_SIZE         10    /*  80-bit Phase1 key                */

#define RC4_KEY_SIZE     16    /* 128-bit RC4KEY (104 bits unknown) */

/* configuration settings */

#define DO_SANITY_CHECK   1    /* validate properties of S-box?     */

/* 2-byte by 2-byte subset of the full AES S-box table */

const u16b Sbox[2][256]=       /* Sbox for hash (can be in ROM)     */

{ {

0xC6A5,0xF884,0xEE99,0xF68D,0xFF0D,0xD6BD,0xDEB1,0x9154,

0x6050,0x0203,0xCEA9,0x567D,0xE719,0xB562,0x4DE6,0xEC9A,

0x8F45,0x1F9D,0x8940,0xFA87,0xEF15,0xB2EB,0x8EC9,0xFB0B,

0x41EC,0xB367,0x5FFD,0x45EA,0x23BF,0x53F7,0xE496,0x9B5B,

0x75C2,0xE11C,0x3DAE,0x4C6A,0x6C5A,0x7E41,0xF502,0x834F,

0x685C,0x51F4,0xD134,0xF908,0xE293,0xAB73,0x6253,0x2A3F,

0x080C,0x9552,0x4665,0x9D5E,0x3028,0x37A1,0x0A0F,0x2FB5,

0x0E09,0x2436,0x1B9B,0xDF3D,0xCD26,0x4E69,0x7FCD,0xEA9F,

0x121B,0x1D9E,0x5874,0x342E,0x362D,0xDCB2,0xB4EE,0x5BFB,

0xA4F6,0x764D,0xB761,0x7DCE,0x527B,0xDD3E,0x5E71,0x1397,

0xA6F5,0xB968,0x0000,0xC12C,0x4060,0xE31F,0x79C8,0xB6ED,

0xD4BE,0x8D46,0x67D9,0x724B,0x94DE,0x98D4,0xB0E8,0x854A,

0xBB6B,0xC52A,0x4FE5,0xED16,0x86C5,0x9AD7,0x6655,0x1194,

0x8ACF,0xE910,0x0406,0xFE81,0xA0F0,0x7844,0x25BA,0x4BE3,

0xA2F3,0x5DFE,0x80C0,0x058A,0x3FAD,0x21BC,0x7048,0xF104,

0x63DF,0x77C1,0xAF75,0x4263,0x2030,0xE51A,0xFD0E,0xBF6D,

0x814C,0x1814,0x2635,0xC32F,0xBEE1,0x35A2,0x88CC,0x2E39,

0x9357,0x55F2,0xFC82,0x7A47,0xC8AC,0xBAE7,0x322B,0xE695,

0xC0A0,0x1998,0x9ED1,0xA37F,0x4466,0x547E,0x3BAB,0x0B83,

0x8CCA,0xC729,0x6BD3,0x283C,0xA779,0xBCE2,0x161D,0xAD76,

0xDB3B,0x6456,0x744E,0x141E,0x92DB,0x0C0A,0x486C,0xB8E4,

0x9F5D,0xBD6E,0x43EF,0xC4A6,0x39A8,0x31A4,0xD337,0xF28B,

0xD532,0x8B43,0x6E59,0xDAB7,0x018C,0xB164,0x9CD2,0x49E0,

0xD8B4,0xACFA,0xF307,0xCF25,0xCAAF,0xF48E,0x47E9,0x1018,

0x6FD5,0xF088,0x4A6F,0x5C72,0x3824,0x57F1,0x73C7,0x9751,

0xCB23,0xA17C,0xE89C,0x3E21,0x96DD,0x61DC,0x0D86,0x0F85,

0xE090,0x7C42,0x71C4,0xCCAA,0x90D8,0x0605,0xF701,0x1C12,

0xC2A3,0x6A5F,0xAEF9,0x69D0,0x1791,0x9958,0x3A27,0x27B9,

0xD938,0xEB13,0x2BB3,0x2233,0xD2BB,0xA970,0x0789,0x33A7,

0x2DB6,0x3C22,0x1592,0xC920,0x8749,0xAAFF,0x5078,0xA57A,

0x038F,0x59F8,0x0980,0x1A17,0x65DA,0xD731,0x84C6,0xD0B8,

0x82C3,0x29B0,0x5A77,0x1E11,0x7BCB,0xA8FC,0x6DD6,0x2C3A,

},

{  /* second half of table is byte-reversed version of first! */

0xA5C6,0x84F8,0x99EE,0x8DF6,0x0DFF,0xBDD6,0xB1DE,0x5491,

0x5060,0x0302,0xA9CE,0x7D56,0x19E7,0x62B5,0xE64D,0x9AEC,

0x458F,0x9D1F,0x4089,0x87FA,0x15EF,0xEBB2,0xC98E,0x0BFB,

0xEC41,0x67B3,0xFD5F,0xEA45,0xBF23,0xF753,0x96E4,0x5B9B,

0xC275,0x1CE1,0xAE3D,0x6A4C,0x5A6C,0x417E,0x02F5,0x4F83,

0x5C68,0xF451,0x34D1,0x08F9,0x93E2,0x73AB,0x5362,0x3F2A,

0x0C08,0x5295,0x6546,0x5E9D,0x2830,0xA137,0x0F0A,0xB52F,

0x090E,0x3624,0x9B1B,0x3DDF,0x26CD,0x694E,0xCD7F,0x9FEA,

0x1B12,0x9E1D,0x7458,0x2E34,0x2D36,0xB2DC,0xEEB4,0xFB5B,

0xF6A4,0x4D76,0x61B7,0xCE7D,0x7B52,0x3EDD,0x715E,0x9713,

0xF5A6,0x68B9,0x0000,0x2CC1,0x6040,0x1FE3,0xC879,0xEDB6,

0xBED4,0x468D,0xD967,0x4B72,0xDE94,0xD498,0xE8B0,0x4A85,

0x6BBB,0x2AC5,0xE54F,0x16ED,0xC586,0xD79A,0x5566,0x9411,

0xCF8A,0x10E9,0x0604,0x81FE,0xF0A0,0x4478,0xBA25,0xE34B,

0xF3A2,0xFE5D,0xC080,0x8A05,0xAD3F,0xBC21,0x4870,0x04F1,

0xDF63,0xC177,0x75AF,0x6342,0x3020,0x1AE5,0x0EFD,0x6DBF,

0x4C81,0x1418,0x3526,0x2FC3,0xE1BE,0xA235,0xCC88,0x392E,

0x5793,0xF255,0x82FC,0x477A,0xACC8,0xE7BA,0x2B32,0x95E6,

0xA0C0,0x9819,0xD19E,0x7FA3,0x6644,0x7E54,0xAB3B,0x830B,

0xCA8C,0x29C7,0xD36B,0x3C28,0x79A7,0xE2BC,0x1D16,0x76AD,

0x3BDB,0x5664,0x4E74,0x1E14,0xDB92,0x0A0C,0x6C48,0xE4B8,

0x5D9F,0x6EBD,0xEF43,0xA6C4,0xA839,0xA431,0x37D3,0x8BF2,

0x32D5,0x438B,0x596E,0xB7DA,0x8C01,0x64B1,0xD29C,0xE049,

0xB4D8,0xFAAC,0x07F3,0x25CF,0xAFCA,0x8EF4,0xE947,0x1810,

0xD56F,0x88F0,0x6F4A,0x725C,0x2438,0xF157,0xC773,0x5197,

0x23CB,0x7CA1,0x9CE8,0x213E,0xDD96,0xDC61,0x860D,0x850F,

0x90E0,0x427C,0xC471,0xAACC,0xD890,0x0506,0x01F7,0x121C,

0xA3C2,0x5F6A,0xF9AE,0xD069,0x9117,0x5899,0x273A,0xB927,

0x38D9,0x13EB,0xB32B,0x3322,0xBBD2,0x70A9,0x8907,0xA733,

0xB62D,0x223C,0x9215,0x20C9,0x4987,0xFFAA,0x7850,0x7AA5,

0x8F03,0xF859,0x8009,0x171A,0xDA65,0x31D7,0xC684,0xB8D0,

0xC382,0xB029,0x775A,0x111E,0xCB7B,0xFCA8,0xD66D,0x3A2C,

}

};

#if DO_SANITY_CHECK

/*

**********************************************************************

* Routine: SanityCheckTable -- verify Sbox properties

*

* Inputs:  Sbox

* Output:  None, but an assertion fails if the tables are wrong

* Notes:

*   The purpose of this routine is solely to illustrate and

*   verify the following properties of the Sbox table:

*      - the Sbox is a "2x2" subset of the AES table:

*            Sbox + affine transform + MDS.

*      - the Sbox table can be easily designed to fit in a

*            512-byte table, using a byte swap

*      - the Sbox table can be easily designed to fit in a

*            256-byte table, using some shifts and a byte swap

**********************************************************************

*/

void SanityCheckTable(void)

{

const static int  M_x = 0x11B;   /* AES irreducible polynomial */

const static byte Sbox8[256] = { /* AES 8-bit Sbox */

0x63,0x7c,0x77,0x7b,0xf2,0x6b,0x6f,0xc5,

0x30,0x01,0x67,0x2b,0xfe,0xd7,0xab,0x76,

0xca,0x82,0xc9,0x7d,0xfa,0x59,0x47,0xf0,

0xad,0xd4,0xa2,0xaf,0x9c,0xa4,0x72,0xc0,

0xb7,0xfd,0x93,0x26,0x36,0x3f,0xf7,0xcc,

0x34,0xa5,0xe5,0xf1,0x71,0xd8,0x31,0x15,

0x04,0xc7,0x23,0xc3,0x18,0x96,0x05,0x9a,

0x07,0x12,0x80,0xe2,0xeb,0x27,0xb2,0x75,

0x09,0x83,0x2c,0x1a,0x1b,0x6e,0x5a,0xa0,

0x52,0x3b,0xd6,0xb3,0x29,0xe3,0x2f,0x84,

0x53,0xd1,0x00,0xed,0x20,0xfc,0xb1,0x5b,

0x6a,0xcb,0xbe,0x39,0x4a,0x4c,0x58,0xcf,

0xd0,0xef,0xaa,0xfb,0x43,0x4d,0x33,0x85,

0x45,0xf9,0x02,0x7f,0x50,0x3c,0x9f,0xa8,

0x51,0xa3,0x40,0x8f,0x92,0x9d,0x38,0xf5,

0xbc,0xb6,0xda,0x21,0x10,0xff,0xf3,0xd2,

0xcd,0x0c,0x13,0xec,0x5f,0x97,0x44,0x17,

0xc4,0xa7,0x7e,0x3d,0x64,0x5d,0x19,0x73,

0x60,0x81,0x4f,0xdc,0x22,0x2a,0x90,0x88,

0x46,0xee,0xb8,0x14,0xde,0x5e,0x0b,0xdb,

0xe0,0x32,0x3a,0x0a,0x49,0x06,0x24,0x5c,

0xc2,0xd3,0xac,0x62,0x91,0x95,0xe4,0x79,

0xe7,0xc8,0x37,0x6d,0x8d,0xd5,0x4e,0xa9,

0x6c,0x56,0xf4,0xea,0x65,0x7a,0xae,0x08,

0xba,0x78,0x25,0x2e,0x1c,0xa6,0xb4,0xc6,

0xe8,0xdd,0x74,0x1f,0x4b,0xbd,0x8b,0x8a,

0x70,0x3e,0xb5,0x66,0x48,0x03,0xf6,0x0e,

0x61,0x35,0x57,0xb9,0x86,0xc1,0x1d,0x9e,

0xe1,0xf8,0x98,0x11,0x69,0xd9,0x8e,0x94,

0x9b,0x1e,0x87,0xe9,0xce,0x55,0x28,0xdf,

0x8c,0xa1,0x89,0x0d,0xbf,0xe6,0x42,0x68,

0x41,0x99,0x2d,0x0f,0xb0,0x54,0xbb,0x16 };

int i,k,k2,k3;

byte bitmap[0x2000];

/* show that smaller tables can be used, if desired */

for (i=0;i<256;i++)

{

k  = Sbox8[i];

k2 = (k << 1) ^ ((k & 0x80) ? M_x : 0);

k3 =  k ^ k2;

assert(Sbox[0][i] == ((k2 << 8) ^ k3));

assert(Sbox[1][i] == ((k3 << 8) ^ k2));

}

/* now make sure that it's a 16-bit permutation */

memset(bitmap,0,sizeof(bitmap));

for (i=0;i<0x10000;i++)

{

k = _S_(i); /* do an S-box lookup: 16 --> 16 bits */

assert(k < (1 << 16));

assert((bitmap[k >> 3] & (1 << (k & 7))) == 0);

bitmap[k >> 3] |= 1 << (k & 7);

}

for (i=0;i<sizeof(bitmap);i++)

assert(bitmap[i] == 0xFF);

/* if we reach here, the 16-bit Sbox is ok */

printf("Table sanity check successful\n");

}

#endif

/*

**********************************************************************

* Routine: Phase 1 -- generate P1K, given TA, TK, IV32

*

* Inputs:

*     TK[]      = temporal key                         [128 bits]

*     TA[]      = transmitter's MAC address            [ 48 bits]

*     IV32      = upper 32 bits of IV                  [ 32 bits]

* Output:

*     P1K[]     = Phase 1 key                          [ 80 bits]

*

* Note:

*     This function only needs to be called every 2**16 packets,

*     although in theory it could be called every packet.

*

**********************************************************************

*/

void Phase1(u16b *P1K,const byte *TK,const byte *TA,u32b IV32)

{

int  i;

/* Initialize the 80 bits of P1K[] from IV32 and TA[0..5]     */

P1K[0]      = Lo16(IV32);

P1K[1]      = Hi16(IV32);

P1K[2]      = Mk16(TA[1],TA[0]); /* use TA[] as little-endian */

P1K[3]      = Mk16(TA[3],TA[2]);

P1K[4]      = Mk16(TA[5],TA[4]);

/* Now compute an unbalanced Feistel cipher with 80-bit block */

/* size on the 80-bit block P1K[], using the 128-bit key TK[] */

for (i=0; i < PHASE1_LOOP_CNT ;i++)

{                 /* Each add operation here is mod 2**16 */

P1K[0] += _S_(P1K[4] ^ TK16((i&1)+0));

P1K[1] += _S_(P1K[0] ^ TK16((i&1)+2));

P1K[2] += _S_(P1K[1] ^ TK16((i&1)+4));

P1K[3] += _S_(P1K[2] ^ TK16((i&1)+6));

P1K[4] += _S_(P1K[3] ^ TK16((i&1)+0));

P1K[4] +=  i;                    /* avoid "slide attacks" */

}

}

/*

**********************************************************************

* Routine: Phase 2 -- generate RC4KEY, given TK, P1K, IV16

*

* Inputs:

*     TK[]      = Temporal key                         [128 bits]

*     P1K[]     = Phase 1 output key                   [ 80 bits]

*     IV16      = low 16 bits of IV counter            [ 16 bits]

* Output:

*     RC4KEY[]  = the key used to encrypt the packet   [128 bits]

*

* Note:

*     The value {TA,IV32,IV16} for Phase1/Phase2 must be unique

*     across all packets using the same key TK value. Then, for a

*     given value of TK[], this TKIP48 construction guarantees that

*     the final RC4KEY value is unique across all packets.

*

* Suggested implementation optimization: if PPK[] is "overlaid"

*     appropriately on RC4KEY[], there is no need for the final

*     for loop below that copies the PPK[] result into RC4KEY[].

*

**********************************************************************

*/

void Phase2(byte *RC4KEY,const byte *TK,const u16b *P1K,u16b IV16)

{

int  i;

u16b PPK[6];                        /* temporary key for mixing    */

/* all adds in the PPK[] equations below are mod 2**16         */

for (i=0;i<5;i++) PPK[i]=P1K[i];    /* first, copy P1K to PPK      */

PPK[5]  =  P1K[4] + IV16;           /* next,  add in IV16          */

/* Bijective non-linear mixing of the 96 bits of PPK[0..5]         */

PPK[0] +=    _S_(PPK[5] ^ TK16(0)); /* Mix key in each "round"     */

PPK[1] +=    _S_(PPK[0] ^ TK16(1));

PPK[2] +=    _S_(PPK[1] ^ TK16(2));

PPK[3] +=    _S_(PPK[2] ^ TK16(3));

PPK[4] +=    _S_(PPK[3] ^ TK16(4));

PPK[5] +=    _S_(PPK[4] ^ TK16(5)); /* Total # S-box lookups == 6  */

/* Final sweep: bijective, linear. Rotates kill LSB correlations   */

PPK[0] +=  RotR1(PPK[5] ^ TK16(6));

PPK[1] +=  RotR1(PPK[0] ^ TK16(7)); /* Use all of TK[] in Phase2   */

PPK[2] +=  RotR1(PPK[1]);

PPK[3] +=  RotR1(PPK[2]);

PPK[4] +=  RotR1(PPK[3]);

PPK[5] +=  RotR1(PPK[4]);

/* At this point, for a given key TK[0..15], the 96-bit output */

/*     value PPK[0..5] is guaranteed to be unique, as a function   */

/*     of the 96-bit "input" value   {TA,IV32,IV16}. That is, P1K  */

/*     is now a keyed permutation of {TA,IV32,IV16}.               */

/* Set RC4KEY[0..3], which includes cleartext portion of RC4 key   */

RC4KEY[0] = Hi8(IV16);              /* RC4KEY[0..2] is the WEP IV  */

RC4KEY[1] =(Hi8(IV16) | 0x20) & 0x7F; /* Help avoid FMS weak keys  */

RC4KEY[2] = Lo8(IV16);

RC4KEY[3] = Lo8((PPK[5] ^ TK16(0)) >> 1);

/* Copy 96 bits of PPK[0..5] to RC4KEY[4..15]  (little-endian)       */

for (i=0;i<6;i++)

{

RC4KEY[4+2*i] = Lo8(PPK[i]);

RC4KEY[5+2*i] = Hi8(PPK[i]);

}

}

/*

**********************************************************************

* Routine: doTestCase -- execute a test case, and print results

**********************************************************************

*/

void DoTestCase(byte *RC4KEY,u32b IV32,u16b IV16,const byte *TA,const byte *TK)

{

int  i;

u16b P1K[P1K_SIZE/2];  /* "temp" copy of phase1 key */

printf("\nTK    =");

for (i=0;i<TK_SIZE;i++) printf(" %02X",TK[i]);

printf("\nTA    =");

for (i=0;i<TA_SIZE;i++) printf(" %02X",TA[i]);

printf("\nIV32  = %08X   [transmitted as",IV32);  /* show byte order */

for (i=0;i<4;i++) printf(" %02X",(IV32 >> (24-8*i)) & 0xFF);

printf("]");

printf("\nIV16  = %04X",IV16);

Phase1(P1K,TK,TA,IV32);

printf("\nP1K   =");

for (i=0;i<P1K_SIZE/2;i++) printf(" %04X ",P1K[i] & 0xFFFF);

Phase2(RC4KEY,TK,P1K,IV16);

printf("\nRC4KEY= ");

for (i=0;i<RC4_KEY_SIZE;i++) printf("%02X ",RC4KEY[i]);

}

/*

**********************************************************************

* Static (Repeatable) Test Cases

**********************************************************************

*/

void DoStaticTestCases(int testCnt)

{

int  i,j;

byte TA[TA_SIZE],TK[TK_SIZE],RC4KEY[RC4_KEY_SIZE];

u16b IV16=0;

u32b IV32=0;

/* set a fixed starting point */

for (i=0;i<TK_SIZE;i++) TK[i]=i;

for (i=0;i<TA_SIZE;i++) TA[i]=(i+1)*17;

TA[0] = TA[0] & 0xFC;             /* Clear I/G and U/L bits in OUI */

/* now generate tests, feeding results back into new tests */

for (i=0; i<testCnt/2; i++)

{

printf("\n\nTest vector #%d:",2*i+1);

DoTestCase(RC4KEY,IV32,IV16,TA,TK);

IV16++;                      /* emulate per-packet "increment" */

if (IV16 == 0) IV32++;

printf("\n\nTest vector #%d:",2*i+2);

DoTestCase(RC4KEY,IV32,IV16,TA,TK);

/* feed results back to seed the next test input values    */

IV16 = (i) ? Mk16(RC4KEY[15],RC4KEY[4]) : 0xFFFF;/* force wrap */

IV32 =       Mk16(RC4KEY[14],RC4KEY[5]);

IV32 =       Mk16(RC4KEY[13],RC4KEY[7]) + (IV32 << 16);

for (j=0;j<TA_SIZE;j++) TA[j]^=RC4KEY[12-j];

for (j=0;j<TK_SIZE;j++) TK[j]^=RC4KEY[(j+i+1) % RC4_KEY_SIZE] ^

RC4KEY[(j+i+7) % RC4_KEY_SIZE] ;

TA[0] = TA[0] & 0xFC;         /* Clear I/G and U/L bits in OUI */

}

/* comparing the final output is a good check of correctness   */

printf("\n");

}

/*

**********************************************************************

* Test Cases Generated at Random

**********************************************************************

*/

void DoRandomTestCases(int testCnt)

{

int  i,j;

u16b IV16;

u32b IV32;

byte TA[TA_SIZE],RC4KEY[RC4_KEY_SIZE],TK[TK_SIZE];

printf("Random tests:\n");

/* now generate tests "recursively" */

for (i=0; i<testCnt; i++)

{

IV16 = rand() & 0xFFFF;

IV32 = rand() + (rand() << 16);

for (j=0;j<TK_SIZE;j++) TK[j]=rand() & 0xFF;

for (j=0;j<TA_SIZE;j++) TA[j]=rand() & 0xFF;

TA[0] = TA[0] & 0xFC;         /* Clear I/G and U/L bits in OUI */

printf("\n\nRandom test vector #%d:",i+1);

DoTestCase(RC4KEY,IV32,IV16,TA,TK);

}

printf("\n");

}

/*

**********************************************************************

* Usage text

**********************************************************************

*/

#define NUM_TEST_CNT  8

void Usage(void)

{

printf(

"Usage:   TKIP48 [options]\n"

"Purpose: Generate test vectors for 802.11 TKIP48\n"

"Options  -?   -- output this usage text\n"

"         -r   -- generate test vectors at random\n"

"         -sN  -- init random seed to N\n"

"         -tN  -- generate N tests (default = %d)\n",

NUM_TEST_CNT

);

exit(0);

}

/*

**********************************************************************

* Main

**********************************************************************

*/

int main(int argc, char **argv)

{

char *parg;

int   i,doRand = 0;

int   testCnt  = NUM_TEST_CNT;

u32b  seed     = (u32b) time(NULL);

#if DO_SANITY_CHECK

SanityCheckTable();

#endif

for (i=1; i<argc; i++)

{

parg = argv[i];

switch (parg[0])

{

case '-':

switch (parg[1])

{

case '?':

case 'H':

case 'h':

Usage();

return 0;

case 'R':

case 'r':  /* generate some random test vectors */

doRand  = 1;

break;

case 'S':

case 's':

seed    = atoi(parg+2);

break;

case 'T':

case 't':

testCnt = atoi(parg+2);

break;

default:

break;

}

break;

case '?':

Usage();

return 0;

default:

printf("Invalid argument: \"%s\"\n", parg);

return 1;

}

}

srand(seed);

if (doRand) printf("Seed = %u\n",seed);

/* generate some test vectors */

if (doRand) DoRandomTestCases(testCnt);

else        DoStaticTestCases(testCnt);

return 0;

}

/*

**********************************************************************

* Routine: Phase 2 -- generate RC4KEY, given TK, P1K, IV16

*

* Inputs:

*     TK[]      = Temporal key                         [128 bits]

*     P1K[]     = Phase 1 output key                   [ 80 bits]

*     IV16      = low 16 bits of IV counter            [ 16 bits]

* Output:

*     RC4KEY[]  = the key used to encrypt the packet   [128 bits]

*

* Note:

*     The value {TA,IV32,IV16} for Phase1/Phase2 must be unique

*     across all packets using the same key TK value. Then, for a

*     given value of TK[], this TKIP48 construction guarantees that

*     the final RC4KEY value is unique across all packets.

*

* Suggested implementation optimization: if PPK[] is "overlaid"

*     appropriately on RC4KEY[], there is no need for the final

*     for loop below that copies the PPK[] result into RC4KEY[].

*

**********************************************************************

*/

void Phase2(byte *RC4KEY,const byte *TK,const u16b *P1K,u16b IV16)

{

int  i;

u16b PPK[6];                        /* temporary key for mixing    */

/* all adds in the PPK[] equations below are mod 2**16         */

for (i=0;i<5;i++) PPK[i]=P1K[i];    /* first, copy P1K to PPK      */

PPK[5]  =  P1K[4] + IV16;           /* next,  add in IV16          */

/* Bijective non-linear mixing of the 96 bits of PPK[0..5]         */

PPK[0] +=    _S_(PPK[5] ^ TK16(0)); /* Mix key in each "round"     */

PPK[1] +=    _S_(PPK[0] ^ TK16(1));

PPK[2] +=    _S_(PPK[1] ^ TK16(2));

PPK[3] +=    _S_(PPK[2] ^ TK16(3));

PPK[4] +=    _S_(PPK[3] ^ TK16(4));

PPK[5] +=    _S_(PPK[4] ^ TK16(5)); /* Total # S-box lookups == 6  */

/* Final sweep: bijective, linear. Rotates kill LSB correlations   */

PPK[0] +=  RotR1(PPK[5] ^ TK16(6));

PPK[1] +=  RotR1(PPK[0] ^ TK16(7)); /* Use all of TK[] in Phase2   */

PPK[2] +=  RotR1(PPK[1]);

PPK[3] +=  RotR1(PPK[2]);

PPK[4] +=  RotR1(PPK[3]);

PPK[5] +=  RotR1(PPK[4]);

/* At this point, for a given key TK[0..15], the 96-bit output */

/*     value PPK[0..5] is guaranteed to be unique, as a function   */

/*     of the 96-bit "input" value   {TA,IV32,IV16}. That is, P1K  */

/*     is now a keyed permutation of {TA,IV32,IV16}.               */

/* Set RC4KEY[0..3], which includes cleartext portion of RC4 key   */

RC4KEY[0] = Hi8(IV16);              /* RC4KEY[0..2] is the WEP IV  */

RC4KEY[1] =(Hi8(IV16) | 0x20) & 0x7F; /* Help avoid FMS weak keys  */

RC4KEY[2] = Lo8(IV16);

RC4KEY[3] = Lo8((PPK[5] ^ TK16(0)) >> 1);

/* Copy 96 bits of PPK[0..5] to RC4KEY[4..15]  (little-endian)       */

for (i=0;i<6;i++)

{

RC4KEY[4+2*i] = Lo8(PPK[i]);

RC4KEY[5+2*i] = Hi8(PPK[i]);

}

}

/*

**********************************************************************

* Routine: doTestCase -- execute a test case, and print results

**********************************************************************

*/

void DoTestCase(byte *RC4KEY,u32b IV32,u16b IV16,const byte *TA,const byte *TK)

{

int  i;

u16b P1K[P1K_SIZE/2];  /* "temp" copy of phase1 key */

printf("\nTK    =");

for (i=0;i<TK_SIZE;i++) printf(" %02X",TK[i]);

printf("\nTA    =");

for (i=0;i<TA_SIZE;i++) printf(" %02X",TA[i]);

printf("\nIV32  = %08X   [transmitted as",IV32);  /* show byte order */

for (i=0;i<4;i++) printf(" %02X",(IV32 >> (24-8*i)) & 0xFF);

printf("]");

printf("\nIV16  = %04X",IV16);

Phase1(P1K,TK,TA,IV32);

printf("\nP1K   =");

for (i=0;i<P1K_SIZE/2;i++) printf(" %04X ",P1K[i] & 0xFFFF);

Phase2(RC4KEY,TK,P1K,IV16);

printf("\nRC4KEY= ");

for (i=0;i<RC4_KEY_SIZE;i++) printf("%02X ",RC4KEY[i]);

}

/*

**********************************************************************

* Static (Repeatable) Test Cases

**********************************************************************

*/

void DoStaticTestCases(int testCnt)

{

int  i,j;

byte TA[TA_SIZE],TK[TK_SIZE],RC4KEY[RC4_KEY_SIZE];

u16b IV16=0;

u32b IV32=0;

/* set a fixed starting point */

for (i=0;i<TK_SIZE;i++) TK[i]=i;

for (i=0;i<TA_SIZE;i++) TA[i]=(i+1)*17;

TA[0] = TA[0] & 0xFC;             /* Clear I/G and U/L bits in OUI */

/* now generate tests, feeding results back into new tests */

for (i=0; i<testCnt/2; i++)

{

printf("\n\nTest vector #%d:",2*i+1);

DoTestCase(RC4KEY,IV32,IV16,TA,TK);

IV16++;                      /* emulate per-packet "increment" */

if (IV16 == 0) IV32++;

printf("\n\nTest vector #%d:",2*i+2);

DoTestCase(RC4KEY,IV32,IV16,TA,TK);

/* feed results back to seed the next test input values    */

IV16 = (i) ? Mk16(RC4KEY[15],RC4KEY[4]) : 0xFFFF;/* force wrap */

IV32 =       Mk16(RC4KEY[14],RC4KEY[5]);

IV32 =       Mk16(RC4KEY[13],RC4KEY[7]) + (IV32 << 16);

for (j=0;j<TA_SIZE;j++) TA[j]^=RC4KEY[12-j];

for (j=0;j<TK_SIZE;j++) TK[j]^=RC4KEY[(j+i+1) % RC4_KEY_SIZE] ^

RC4KEY[(j+i+7) % RC4_KEY_SIZE] ;

TA[0] = TA[0] & 0xFC;         /* Clear I/G and U/L bits in OUI */

}

/* comparing the final output is a good check of correctness   */

printf("\n");

}

/*

**********************************************************************

* Test Cases Generated at Random

**********************************************************************

*/

void DoRandomTestCases(int testCnt)

{

int  i,j;

u16b IV16;

u32b IV32;

byte TA[TA_SIZE],RC4KEY[RC4_KEY_SIZE],TK[TK_SIZE];

printf("Random tests:\n");

/* now generate tests "recursively" */

for (i=0; i<testCnt; i++)

{

IV16 = rand() & 0xFFFF;

IV32 = rand() + (rand() << 16);

for (j=0;j<TK_SIZE;j++) TK[j]=rand() & 0xFF;

for (j=0;j<TA_SIZE;j++) TA[j]=rand() & 0xFF;

TA[0] = TA[0] & 0xFC;         /* Clear I/G and U/L bits in OUI */

printf("\n\nRandom test vector #%d:",i+1);

DoTestCase(RC4KEY,IV32,IV16,TA,TK);

}

printf("\n");

}

/*

**********************************************************************

* Usage text

**********************************************************************

*/

#define NUM_TEST_CNT  8

void Usage(void)

{

printf(

"Usage:   TKIP48 [options]\n"

"Purpose: Generate test vectors for 802.11 TKIP48\n"

"Options  -?   -- output this usage text\n"

"         -r   -- generate test vectors at random\n"

"         -sN  -- init random seed to N\n"

"         -tN  -- generate N tests (default = %d)\n",

NUM_TEST_CNT

);

exit(0);

}

/*

**********************************************************************

* Main

**********************************************************************

*/

int main(int argc, char **argv)

{

char *parg;

int   i,doRand = 0;

int   testCnt  = NUM_TEST_CNT;

u32b  seed     = (u32b) time(NULL);

#if DO_SANITY_CHECK

SanityCheckTable();

#endif

for (i=1; i<argc; i++)

{

parg = argv[i];

switch (parg[0])

{

case '-':

switch (parg[1])

{

case '?':

case 'H':

case 'h':

Usage();

return 0;

case 'R':

case 'r':  /* generate some random test vectors */

doRand  = 1;

break;

case 'S':

case 's':

seed    = atoi(parg+2);

break;

case 'T':

case 't':

testCnt = atoi(parg+2);

break;

default:

break;

}

break;

case '?':

Usage();

return 0;

default:

printf("Invalid argument: \"%s\"\n", parg);

return 1;

}

}

srand(seed);

if (doRand) printf("Seed = %u\n",seed);

/* generate some test vectors */

if (doRand) DoRandomTestCases(testCnt);

else        DoStaticTestCases(testCnt);

return 0;

}

### F.1.2  Test Vectors (Normative)

The following output is provided to test implementations of the temporal key hash algorithm. All input and output values are shown in hexadecimal.

Test vector #1:

TK    = 00 01 02 03 04 05 06 07 08 09 0A 0B 0C 0D 0E 0F [LSB on left, MSB on right]

TA    = 10-22-33-44-55-66

PN    = 000000000000 [transmitted as:  00 20 00 DefKeyID 00 00 00 00]

IV32  = 00000000

IV16  = 0000

P1K   = 3DD2  016E  76F4  8697  B2E8

RC4KEY= 00 20 00 33 EA 8D 2F 60 CA 6D 13 74 23 4A 66 0B

Test vector #2:

TK    = 00 01 02 03 04 05 06 07 08 09 0A 0B 0C 0D 0E 0F [LSB on left, MSB on right]

TA    = 10-22-33-44-55-66

PN    = 000000000001 [transmitted as:  01 20 00 DefKeyID 00 00 00 00]

IV32  = 00000000

IV16  = 0001

P1K   = 3DD2  016E  76F4  8697  B2E8

RC4KEY= 00 20 01 90 FF DC 31 43 89 A9 D9 D0 74 FD 20 AA

Test vector #3:

TK    = 63 89 3B 25 08 40 B8 AE 0B D0 FA 7E 61 D2 78 3E [LSB on left, MSB on right]

TA    = 64-F2-EA-ED-DC-25

PN    = 20DCFD43FFFF [transmitted as:  FF 7F FF DefKeyID 43 FD DC 20]

IV32  = 20DCFD43

IV16  = FFFF

P1K   = 7C67  49D7  9724  B5E9  B4F1

RC4KEY= FF 7F FF 93 81 0F C6 E5 8F 5D D3 26 25 15 44 CE

Test vector #4:

TK    = 63 89 3B 25 08 40 B8 AE 0B D0 FA 7E 61 D2 78 3E [LSB on left, MSB on right]

TA    = 64-F2-EA-ED-DC-25

PN    = 20DCFD440000 [transmitted as:  00 20 00 DefKeyID 44 FD DC 20]

IV32  = 20DCFD44

IV16  = 0000

P1K   = 5A5D  73A8  A859  2EC1  DC8B

RC4KEY= 00 20 00 49 8C A4 71 FC FB FA A1 6E 36 10 F0 05

Test vector #5:

TK    = 98 3A 16 EF 4F AC B3 51 AA 9E CC 27 1D 73 09 E2 [LSB on left, MSB on right]

TA    = 50-9C-4B-17-27-D9

PN    = F0A410FC058C [transmitted as:  8C 25 05 DefKeyID FC 10 A4 F0]

IV32  = F0A410FC

IV16  = 058C

P1K   = F2DF  EBB1  88D3  5923  A07C

RC4KEY= 05 25 8C F4 D8 51 52 F4 D9 AF 1A 64 F1 D0 70 21

Test vector #6:

TK    = 98 3A 16 EF 4F AC B3 51 AA 9E CC 27 1D 73 09 E2 [LSB on left, MSB on right]

TA    = 50-9C-4B-17-27-D9

PN    = F0A410FC058D [transmitted as:  8D 25 05 DefKeyID FC 10 A4 F0]

IV32  = F0A410FC

IV16  = 058D

P1K   = F2DF  EBB1  88D3  5923  A07C

RC4KEY= 05 25 8D 09 F8 15 43 B7 6A 59 6F C2 C6 73 8B 30

Test vector #7:

TK    = C8 AD C1 6A 8B 4D DA 3B 4D D5 B6 54 38 35 9B 05 [LSB on left, MSB on right]

TA    = 94-5E-24-4E-4D-6E

PN    = 8B1573B730F8 [transmitted as:  F8 30 30 DefKeyID B7 73 15 8B]

IV32  = 8B1573B7

IV16  = 30F8

P1K   = EFF1  3F38  A364  60A9  76F3

RC4KEY= 30 30 F8 65 0D A0 73 EA 61 4E A8 F4 74 EE 03 19

Test vector #8:

TK    = C8 AD C1 6A 8B 4D DA 3B 4D D5 B6 54 38 35 9B 05 [LSB on left, MSB on right]

TA    = 94-5E-24-4E-4D-6E

PN    = 8B1573B730F9 [transmitted as:  F9 30 30 DefKeyID B7 73 15 8B]

IV32  = 8B1573B7

IV16  = 30F9

P1K   = EFF1  3F38  A364  60A9  76F3

RC4KEY= 30 30 F9 31 55 CE 29 34 37 CC 76 71 27 16 AB 8F

## F.2 Michael reference implementation and test vectors (Normative)

### F.2.1 Michael test vectors

To ensure correct implementation of Michael, here are some test vectors. These test vectors still have to be confirmed by an independent implementation.

#### F.2.1.1 Block function

Here are some test vectors for the block function.

| Input | # times | output |
| --- | --- | --- |
| (00000000, 00000000) | 1 | (00000000, 00000000) |
| (00000000, 00000001) | 1 | (c00015a8, c0000b95) |
| (00000001, 00000000) | 1 | (6b519593, 572b8b8a) |
| (01234567, 83659326) | 1 | (441492c2, 1d8427ed) |
| (00000001, 00000000) | 1000 | (9f04c4ad, 2ec6c2bf) |

The first four rows give test vectors for a single application of the block function b. The last row gives a test vector for 1000 repeated applications of the block function. Together these should provide adequate test coverage.

#### F.2.1.2 Michael

Here are some test vectors for Michael.

| Key | message | output |
| --- | --- | --- |
| 0000000000000000 | "" | 82925c1ca1d130b8 |
| 82925c1ca1d130b8 | "M” | 434721ca40639b3f |
| 434721ca40639b3f | "Mi “ | E8f9becae97e5d29 |
| e8f9becae97e5d29 | "Mic" | 90038fc6cf13c1db |
| 90038fc6cf13c1db | "Mich" | d55e100510128986 |
| d55e100510128986 | "Michael" | 0a942b124ecaa546 |

Note that each key is the result of the previous line, which makes it easy to construct a single test out of all of these test cases.

### F.2.2 Example code (Informative)

//

// Michael.h    Reference implementation for Michael

//              written by Niels Ferguson

//

// The author has put this code in the public domain.

//

//

// A Michael object implements the computation of the Michael MIC.

//

// Conceptually, the object stores the message to be authenticated.

// At construction the message is empty.

// The append() method appends bytes to the message.

// The getMic() method computes the MIC over the message and returns the result.

// As a side-effect it also resets the stored message

// to the empty message so that the object can be re-used

// for another MIC computation.

class Michael

{

public:

// Constructor requires a pointer to 8 bytes of key

Michael( Byte * key );

// Destructor

~Michael();

// Clear the internal message,

// resets the object to the state just after construction.

void clear();

// Set the key to a new value

void setKey( Byte * key );

// Append bytes to the message to be MICed

void append( Byte * src, int nBytes );

// Get the MIC result. Destination should accept 8 bytes of result.

// This also resets the message to empty.

void getMIC( Byte * dst );

// Run the test plan to verify proper operations

static void runTestPlan();

private:

// Copy constructor declared but not defined,

//avoids compiler-generated version.

Michael( const Michael & );

// Assignment operator declared but not defined,

//avoids compiler-generated version.

void operator=( const Michael & );

// A bunch of internal functions

// Get UInt32 from 4 bytes LSByte first

static UInt32 getUInt32( Byte * p );

// Put UInt32 into 4 bytes LSByte first

static void putUInt32( Byte * p, UInt32 val );

// Add a single byte to the internal message

void appendByte( Byte b );

// Conversion of hex string to binary string

static void hexToBin( char *src, Byte * dst );

// More conversion of hex string to binary string

static void hexToBin( char *src, int nChars, Byte * dst );

// Helper function for hex conversion

static Byte hexToBinNibble( char c );

// Run a single test case

static void runSingleTest( char * cKey, char * cMsg, char * cResult );

UInt32  K0, K1;         // Key

UInt32  L, R;           // Current state

UInt32  M;              // Message accumulator (single word)

int     nBytesInM;      // # bytes in M

};

//

// Michael.cpp  Reference implementation for Michael

//              written by Niels Ferguson

//

// The author has put this code in the public domain.// All rights reserved,

//

// Adapt these typedefs to your local platform

typedef unsigned long UInt32;

typedef unsigned char Byte;

#include <assert.h>

#include <stdio.h>

#include <stdlib.h>

#include <string.h>

#include "Michael.h"

// Rotation functions on 32 bit values

#define ROL32( A, n ) \

( ((A) << (n)) | ( ((A)>>(32-(n)))  & ( (1UL << (n)) - 1 ) ) )

#define ROR32( A, n ) ROL32( (A), 32-(n) )

UInt32 Michael::getUInt32( Byte * p )

// Convert from Byte[] to UInt32 in a portable way

{

UInt32 res = 0;

for( int i=0; i<4; i++ )

{

res |= (*p++) << (8*i);

}

return res;

}

void Michael::putUInt32( Byte * p, UInt32 val )

// Convert from UInt32 to Byte[] in a portable way

{

for( int i=0; i<4; i++ )

{

*p++ = (Byte) (val & 0xff);

val >>= 8;

}

}

void Michael::clear()

{

// Reset the state to the empty message.

L = K0;

R = K1;

nBytesInM = 0;

M = 0;

}

void Michael::setKey( Byte * key )

{

// Set the key

K0 = getUInt32( key );

K1 = getUInt32( key + 4 );

// and reset the message

clear();

}

Michael::Michael( Byte * key )

{

setKey( key );

}

Michael::~Michael()

{

// Wipe the key material

K0 = 0;

K1 = 0;

// And the other fields as well.

//Note that this sets (L,R) to (K0,K1) which is just fine.

clear();

}

void Michael::appendByte( Byte b )

{

// Append the byte to our word-sized buffer

M |= b << (8*nBytesInM);

nBytesInM++;

// Process the word if it is full.

if( nBytesInM >= 4 )

{

L ^= M;

R ^= ROL32( L, 17 );

L += R;

R ^= ((L & 0xff00ff00) >> 8) | ((L & 0x00ff00ff) << 8);

L += R;

R ^= ROL32( L, 3 );

L += R;

R ^= ROR32( L, 2 );

L += R;

// Clear the buffer

M = 0;

nBytesInM = 0;

}

}

void Michael::append( Byte * src, int nBytes )

{

// This is simple

while( nBytes > 0 )

{

appendByte( *src++ );

nBytes--;

}

}

void Michael::getMIC( Byte * dst )

{

// Append the minimum padding

appendByte( 0x5a );

appendByte( 0 );

appendByte( 0 );

appendByte( 0 );

appendByte( 0 );

// and then zeroes until the length is a multiple of 4

while( nBytesInM != 0 )

{

appendByte( 0 );

}

// The appendByte function has already computed the result.

putUInt32( dst, L );

putUInt32( dst+4, R );

// Reset to the empty message.

clear();

}

void Michael::hexToBin( char *src, Byte * dst )

{

// Simple wrapper

hexToBin( src, strlen( src ), dst );

}

void Michael::hexToBin( char *src, int nChars, Byte * dst )

{

assert( (nChars & 1) == 0 );

int nBytes = nChars/2;

// Straightforward conversion

for( int i=0; i<nBytes; i++ )

{

dst[i] = (Byte)((hexToBinNibble( src[0] ) << 4)

| hexToBinNibble( src[1] ));

src += 2;

}

}

Byte Michael::hexToBinNibble( char c )

{

if( '0' <= c && c <= '9' )

{

return (Byte)(c - '0');

}

// Make it upper case

c &= ~('a'-'A');

assert( 'A' <= c && c <= 'F' );

return (Byte)(c - 'A' + 10);

}

void Michael::runSingleTest( char * cKey, char * cMsg, char * cResult )

{

Byte key[ 8 ];

Byte result[ 8 ];

Byte res[ 8 ];

// Convert key and result to binary form

hexToBin( cKey, key );

hexToBin( cResult, result );

// Compute the MIC value

Michael mic( key );

mic.append( (Byte *)cMsg, strlen( cMsg) );

mic.getMIC( res );

// Check that it matches

assert( memcmp( res, result, 8 ) == 0 );

}

void Michael::runTestPlan()

// As usual, test plans can be quite tedious but this should

// ensure that the implementation runs as expected.

{

Byte key[8] ;

Byte msg[12];

int i;

// First we test the test vectors for the block function

// The case (0,0)

putUInt32( key, 0 );

putUInt32( key+4, 0 );

putUInt32( msg, 0 );

Michael mic( key );

mic.append( msg, 4 );

assert( mic.L == 0 && mic.R == 0 );

// The case (0,1)

putUInt32( key, 0 );

putUInt32( key+4, 1 );

mic.setKey( key );

mic.append( msg, 4 );

assert( mic.L == 0xc00015a8 && mic.R == 0xc0000b95 );

// The case (1,0)

putUInt32( key, 1 );

putUInt32( key+4, 0 );

mic.setKey( key );

mic.append( msg, 4 );

assert( mic.L == 0x6b519593 && mic.R == 0x572b8b8a );

// The case (01234567, 83659326)

putUInt32( key, 0x01234567 );

putUInt32( key+4, 0x83659326 );

mic.setKey( key );

mic.append( msg, 4 );

assert( mic.L == 0x441492c2 && mic.R == 0x1d8427ed );

// The repeated case

putUInt32( key, 1 );

putUInt32( key+4,0 );

mic.setKey( key );

for( i=0; i<1000; i++ )

{

mic.append( msg, 4 );

}

assert( mic.L == 0x9f04c4ad && mic.R == 0x2ec6c2bf );

// And now for the real test cases

runSingleTest( "0000000000000000", ""         , "82925c1ca1d130b8" );

runSingleTest( "82925c1ca1d130b8", "M"        , "434721ca40639b3f" );

runSingleTest( "434721ca40639b3f", "Mi"       , "e8f9becae97e5d29" );

runSingleTest( "e8f9becae97e5d29", "Mic"      , "90038fc6cf13c1db" );

runSingleTest( "90038fc6cf13c1db", "Mich"     , "d55e100510128986" );

runSingleTest( "d55e100510128986", "Michael"  , "0a942b124ecaa546" );

}

## F.3 PRF reference implementation and test vectors

### F.3.1 PRF Reference code (Informative)

/*

* PRF -- Length of output is in octets rather than bits

*     since length is always a multiple of 8 output array is

*     organized so first N octets starting from 0 contains PRF output

*

*     supported inputs are 16, 24, 32, 48, 64

*     output array must be 80 octets to allow for sha1 overflow

*/

void PRF(

unsigned char *key, int key_len,

unsigned char *prefix, int prefix_len,

unsigned char *data, int data_len,

unsigned char *output, int len)

{

int i;

unsigned char input[1024]; /* concatenated input */

int currentindex = 0;

int total_len;

memcpy(input, prefix, prefix_len);

input[prefix_len] = 0; /* single octet 0 */

memcpy(&input[prefix_len+1], data, data_len);

total_len = prefix_len + 1 + data_len;

input[total_len] = 0; /* single octet count, starts at 0 */

total_len++;

for(i = 0; i < (len+19)/20; i++) {

hmac_sha1(input, total_len, key, key_len,

&output[currentindex]);

currentindex += 20;	/* next concatenation location */

input[total_len-1]++; /* increment octet count */

}

}

### F.3.2 PRF Test vectors (Normative)

Test case 1

Key	0x0b0b0b0b0b0b0b0b0b0b0b0b0b0b0b0b0b0b0b0b

Key length	20

Prefix	“prefix”

Prefix length	6

Data	"Hi There"

Data length	8

PRF-512 	 0xbcd4c650b30b9684951829e0d75f9d54b862175ed9f00606e17d8da35402ffee75df78c3d31e0f889f012120c0862beb67753e7439ae242edb8373698356cf5a

Test case 2

Key	"Jefe"

Key length	4

Prefix	“prefix”

Prefix length	6

Data	"what do ya want for nothing?"

Data length	28

PRF-512 0x51f4de5b33f249adf81aeb713a3c20f4fe631446fabdfa58244759ae58ef9009a99abf4eac2ca5fa87e692c440eb40023e7babb206d61de7b92f41529092b8fc

Test case 3

Key	0xaaaaaaaaaaaaaaaaaaaaaaaaaaaaaaaaaaaaaaaa

Key length	20

Prefix	“prefix”

Prefix length	6

Data	0xdd repeated 50 times

Data length	50

PRF-512	 0xe1ac546ec4cb636f9976487be5c86be17a0252ca5d8d8df12cfb0473525249ce9dd8d177ead710bc9b590547239107aef7b4abd43d87f0a68f1cbd9e2b6f7607

## F.4. CCM

### F.4.1. CCM reference implementation (Informative)

/*================================================================

* Proposed AES CTR/CBC-MAC mode test vector generation

*

* 11-02-001r2-I-AES-Encryption & Authentication

* Using-CTR-Mode-with-CBC-MAC

*

* Author:  Doug Whiting, Hifn  (dwhiting@hifn.com)

*

*

*================================================================

*/

#include <stdio.h>

#include <stdlib.h>

#include <string.h>

#include <time.h>

#include <assert.h>

#include "aes_defs.h"       /* AES calling interface*/

#include "aes_vect.h"       /* NIST AES test vectors*/

typedef int BOOL;           /* boolean */

enum {

BLK_SIZE   = 16,    /* # octets in an AES block */

MAX_PACKET = 3*512, /* largest packet size */

N_RESERVED = 0,     /* reserved nonce octet value */

A_DATA     = 0x40,  /* the Adata bit in the flags */

M_SHIFT    = 3,     /* how much to shift the 3-bit M field */

L_SHIFT    = 0,     /* how much to shift the 3-bit L field */

L_SIZE     = 2     /* size of the l(m) length field (in octets) */

};

union block {             /* AES cipher block */

u32b x[BLK_SIZE/4]; /* access as 8-bit octets or 32-bit words */

u08b b[BLK_SIZE];

};

struct packet {

BOOL  encrypted; /* TRUE if encrypted */

u08b  TA[6];     /* xmit address */

int   micLength; /* # octets of MIC appended to plaintext (M) */

int   clrCount;  /* # cleartext octets covered by MIC */

u32b  pktNum[2]; /* unique packet sequence number (like WEP IV) */

block key;       /* the encryption key (K) */

int   length;    /* # octets in data[] */

u08b  data[MAX_PACKET+2*BLK_SIZE]; /* packet contents */

};

struct {

int   cnt;            /* how many words left in ct */

block ptCntr;         /* the counter input */

block ct;             /* the ciphertext (prng output) */

} prng;

/* return the 32-bit value read to be stored as a big-endian word */

u32b BigEndian(u32b x)

{

static block b = {0,0,0,0};

if (b.x[0] == 0)      /* first time, figure out endianness */

b.x[0] = 0xFF000001;

if (b.b[0] == 0xFF)   /* is this a big-endian CPU? */

return x;      /* if so, just return x */

if (b.b[0] != 0x01) /* not big-Endian; check it's little-Endian */

assert(0);     /* if not, bomb! */

/* little-endian: do the byte swapping */

return (x >> 24) + (x << 24) +

((x >>  8) & 0x00FF00) + ((x <<  8) & 0xFF0000);

}

void InitRand(u32b seed)

{

memset(prng.ptCntr.b,0,BLK_SIZE);

prng.ptCntr.x[(BLK_SIZE/4)-1] = seed*17;

prng.cnt = 0;  /* the pump is dry */

}

/* prng: does not use C rand(), so should be usable across platforms */

u32b Random32(void)

{

if (prng.cnt == 0) { /* use whatever key is currently defined */

prng.cnt = BLK_SIZE/4;

prng.ptCntr.x[0]++;

if (prng.ptCntr.x[0] == 0) /* ripple carry? */

prng.ptCntr.x[1]++;  /* stop at 64 bits */

AES_Encrypt(prng.ptCntr.x, prng.ct.x);

}

--prng.cnt;

return BigEndian(prng.ct.x[prng.cnt]);

}

/* display a block */

void ShowBlock(

const block *blk,

const char *prefix,

const char *suffix,

int a)

{

int i, blkSize = BLK_SIZE;

printf(prefix,a);

if (suffix == NULL) {

suffix = "\n";

blkSize = a;

}

for (i = 0; i < blkSize; i++)

printf("%02X%s", blk->b[i], ((i&3)==3) ? "  ":" ");

printf (suffix);

}

void ShowAddr(const packet *p)

{

int i;

printf("      TA = ");

for (i = 0; i < 6 ; i++)

printf("%02X%s",p->TA[i],(i==3)?"  ":" ");

printf("  48-bit pktNum = %04X.%08X\n",p->pktNum[1],p->pktNum[0]);

}

/* display a packet */

void ShowPacket(const packet *p, const char *pComment, int a)

{

int i;

printf("Total packet length = %4d. ", p->length);

printf(pComment, a);

if (p->encrypted)

printf("[Encrypted]");

for (i = 0; i < p->length; i++) {

if ((i & 15) == 0)

printf("\n%11s","");

printf("%02X%s", p->data[i], ((i&3)==3) ? "  ":" ");

}

printf("\n");

}

/* make sure that encrypt/decrypt work according to NIST vectors */

void Validate_NIST_AES_Vectors(int verbose)

{

int i;

block key,pt,ct,rt;

printf("AES KAT Vectors:\n");   /* known-answer tests */

/* variable text (fixed-key) tests */

memcpy(key.b,VT_key,BLK_SIZE);

AES_SetKey(key.x,BLK_SIZE*8);

for (i = 0; i < sizeof(VT_pt_ct_pairs); i += 2 * BLK_SIZE) {

memcpy(pt.b, VT_pt_ct_pairs+i, BLK_SIZE);

AES_Encrypt(pt.x, ct.x);

if (memcmp(ct.x, VT_pt_ct_pairs+i+BLK_SIZE, BLK_SIZE)) {

printf("Vector miscompare at VT test #%d", i);

exit(1);

}

AES_Decrypt(ct.x, rt.x); /* sanity check on decrypt */

if (memcmp(pt.b, rt.b, BLK_SIZE)) {

printf("Decrypt miscompare at VT test #%d", i);

exit(1);

}

if (verbose) { /* only do a little if we're "debugging" */

printf("\n");

break;

} else if (i==0) {  /* display the first vector */

ShowBlock(&key,"Key:       ","\n",0);

ShowBlock(&pt ,"PT:        ","\n",0);

ShowBlock(&ct ,"CT:        ","\n\n",0);

}

}

/* variable key (fixed-text) tests */

memcpy(pt.b, VK_pt, BLK_SIZE);

for (i = 0; i < sizeof(VK_key_ct_pairs); i += 2*BLK_SIZE) {

memcpy(key.b, VK_key_ct_pairs+i, BLK_SIZE);

AES_SetKey(key.x, BLK_SIZE*8);

AES_Encrypt(pt.x, ct.x);

if (memcmp(ct.x, VK_key_ct_pairs+i+BLK_SIZE, BLK_SIZE)) {

printf("Vector miscompare at VK test #%d", i);

exit(1);

}

AES_Decrypt(ct.x, rt.x); /* sanity check on decrypt */

if (memcmp(pt.b, rt.b, BLK_SIZE)) {

printf("Decrypt miscompare at VK test #%d",i);

exit(1);

}

if (verbose) {  /* only do a little if we're "debugging" */

printf("\n");

break;

} else if (i==0) {  /*  display the first vector */

ShowBlock(&key, "Key:       ", "\n", 0);

ShowBlock(&pt , "PT:        ", "\n", 0);

ShowBlock(&ct , "CT:        ", "\n\n", 0);

}

}

printf("NIST AES Vectors: OK\n"); /* ok if we got here */

}

/* assumes AES_SetKey is called elsewhere */

void Generate_CTR_CBC_Vector(packet *p, int verbose)

{

int i, j, len, needPad, blkNum;

block   m, x, T;

assert(p->length >= p->clrCount && p->length <= MAX_PACKET);

assert(p->micLength > 0 && p->micLength <= BLK_SIZE);

len = p->length - p->clrCount;  /* l(m) */

ShowPacket(p,"[Input (%d cleartext header octets)]", p->clrCount);

/* ---- generate the first AES block for CBC-MAC */

m.b[ 0] = (u08b) (((p->clrCount)?A_DATA:0) +

(((p->micLength-2)/2 << M_SHIFT)) +

((L_SIZE-1) << L_SHIFT));  /* flags octet */

m.b[ 1] = N_RESERVED;  /* reserved nonce octet  */

m.b[ 2] = (u08b) (p->pktNum[1] >> 8) & 0xFF; /* 48 bit pkt # */

m.b[ 3] = (u08b)  p->pktNum[1]       & 0xFF;

m.b[ 4] = (u08b) (p->pktNum[0] >>24) & 0xFF;

m.b[ 5] = (u08b) (p->pktNum[0] >>16) & 0xFF;

m.b[ 6] = (u08b) (p->pktNum[0] >> 8) & 0xFF;

m.b[ 7] = (u08b)  p->pktNum[0]       & 0xFF;

m.b[ 8] = p->TA[0];  /* 48 bit Transmit Addr */

m.b[ 9] = p->TA[1];

m.b[10] = p->TA[2];

m.b[11] = p->TA[3];

m.b[12] = p->TA[4];

m.b[13] = p->TA[5];

m.b[14] = (len >> 8) & 0xFF;  /* l(m) field */

m.b[15] = len        & 0xFF;

/*---- compute the CBC-MAC tag (MIC) */

AES_Encrypt(m.x, x.x);  /* produce the CBC IV */

ShowBlock(&m,"CBC IV in: ", "\n", 0);

if (verbose)

ShowBlock(&x, "CBC IV out:", "\n", 0);

j = 0;    /* j = octet counter inside the block */

if (p->clrCount) {  /* is there a header? */

/* if so, "insert" length field: l(a) */

assert(p->clrCount < 0xFFF0);

/* [don't handle larger cases (yet)] */

x.b[j++] ^= (p->clrCount >> 8) & 0xFF;

x.b[j++] ^=  p->clrCount       & 0xFF;

}

for (i = blkNum = 0; i < p->length; i++) { /* CBC-MAC */

x.b[j++] ^= p->data[i]; /* perform the CBC xor */

needPad = (i == p->clrCount-1) || (i == p->length-1);

if ((j == BLK_SIZE) || needPad) {

/* full block, or hit pad boundary */

if (verbose)

ShowBlock(&x, "After xor: ",

(i >= p->clrCount) ? " [msg]\n" : " [hdr]\n",blkNum);

AES_Encrypt(x.x, x.x);  /* encrypt in place */

if (verbose)

ShowBlock(&x, "After AES: ", "\n", blkNum);

blkNum++;  /* count the blocks */

j = 0;  /* the block is now empty */

}

}

memcpy(T.b,x.b,p->micLength);       // save the MIC tag

ShowBlock(&T,"MIC tag  : ",NULL,p->micLength);

/* ---- encrypt the data packet using CTR mode */

m.b[0] &= ~(A_DATA | (7<<M_SHIFT));

/* clear flag fields for counter mode */

for (i=blkNum=0;i+p->clrCount < p->length;i++) {

if ((i % BLK_SIZE) == 0) {

/* generate new keystream block */

blkNum++;  /* start data with block #1 */

m.b[14] = blkNum/256;

m.b[15] = blkNum%256;

AES_Encrypt(m.x, x.x); /* encrypt the counter */

if (verbose && i==0)

ShowBlock(&m,"CTR Start: ","\n",0);

if (verbose)

ShowBlock(&x,"CTR[%04X]: " ,"\n",blkNum);

}

/* merge in the keystream */

p->data[i+p->clrCount] ^= x.b[i % BLK_SIZE];

}

/* ---- truncate, encrypt, and append MIC to packet */

m.b[14] = m.b[15] = 0; /* use block counter value zero for tag */

AES_Encrypt(m.x, x.x); /* encrypt the counter */

if (verbose)

ShowBlock(&x,"CTR[MIC ]: " ,NULL,p->micLength);

for (i = 0; i < p->micLength; i++)

p->data[p->length+i] = T.b[i] ^ x.b[i];

p->length += p->micLength; /* adjust pkt length accordingly */

p->encrypted = 1;

ShowPacket(p,"",0); /* show the final encrypted packet */

}

int main(int argc,char *argv[])

{

int i, j, k, len, pktNum, seed;

packet p;

seed = (argc > 1) ? atoi(argv[1]) : (int) time(NULL);

InitRand(seed);

printf("%s C compiler [%s %s].\nRandom seed = %d\n",

COMPILER_ID,__DATE__,__TIME__,seed);

/* 1st, make sure that our AES code matches NIST KAT vectors */

Validate_NIST_AES_Vectors(_VERBOSE_);

/* generate CTR-CBC vectors for various parameter settings */

for (k = pktNum = 0; k < 2; k++) {

/* k==1 => random vectors.

k==0 => "visually simple" vectors */

for (i = 0; i < BLK_SIZE  ; i++)

p.key.b[i] =

k) ? (u08b) Random32() & 0xFF : i + 0xC0;

for (i = 0; i < 6; i++)

p.TA[i] = (k) ? (u08b) Random32() & 0xFF : i + 0xA0;

AES_SetKey(p.key.x, BLK_SIZE*8);

/* run key schedule */

/* now generate the vectors */

for (p.micLength  = 8;p.micLength  <12;p.micLength+=2)

for (p.clrCount   = 8;p.clrCount   <16;p.clrCount+=4)

for (len          =32;len          <64;len*=2)

for (i            =-1;i            < 2;i++) {

p.pktNum[0] = (k) ? Random32() :

pktNum*0x01010101 + 0x03020100;

p.pktNum[1] = (k) ? Random32() & 0xFFFF : 0;

/* 48-bit IV */

p.length    = len+i;  /* len+i is packet length */

p.encrypted = 0;

assert(p.length <= MAX_PACKET);

for (j = 0; j < p.length; j++) /* random pkt */

p.data[j]= (k) ? (u08b) Random32() & 0xFF : j;

pktNum++;

printf("=========== Packet Vector #%d ==============\n",pktNum);

ShowBlock(&p.key ,"AES Key:   ","\n",0);

ShowAddr (&p);

Generate_CTR_CBC_Vector(&p,1);

}

}

return 0;

}

### F.5. Suggested pass-phrase-to-preshared-key mapping (Informative)

### F.5.1 Introduction

The RSNA pre-shared key consists of 256 bits, or 64 bytes when represented in hex. It is difficult for a user to correctly enter 64 hex characters. Most users, however, are familiar with passwords and pass-phrases, and feel more comfortable entering them than entering keys. A user is more likely to be able to enter an ASCII password or pass-phrase, even though doing so limits the set of possible keys. This suggests that the best that can be done is to introduce a pass-phrase to preshared key mapping.

This clause defines a pass-phrase to preshared key mapping that is the recommended practice for use with Robust Security Network Associations. This pass-phrase mapping was introduced to encourage users unfamiliar with cryptographic concepts to enable the security features of their WLAN.

A pass-phrase typically has about 2.5 bits of security per character, so the pass-phrase mapping converts an n byte password into a key with about 2.5n + 12 bits of security. Hence, it provides a relatively low level of security, with keys generated from short passwords subject to dictionary attack. Use of the key hash is recommended only where it is impractical to make use of a stronger form of user authentication.A key generated from a pass-phrase of less than about 20 characters is unlikely to deter attacks.

The pass-phrase mapping defined here uses the PBKDF2 method from PKCS #5 v2.0: Password-based Cryptography Standard.

PSK = PBKDF2(PassPhrase, ssid, ssidLength, 4096, 256)

- PassPhrase is an ASCII string which has a minimum of 8 and a maximum of 63 characters not including the null terminator. The limit of 63 characters comes from the fact that 256 bits is represented by 64 characters in hex.

PassPhrase should consist of characters from the following groups

| Group | Examples |
| --- | --- |
| Letters (upper and lower case) | A, B, C, … (and a, b, c,…) |
| Numerals | 0, 1, 2, 3, 4, 5, 6, 7, 8, 9 |
| Whitespace | space |
| Symbols (all characters not defined as letters or numerals) | `~!@#$%^&*()_+=-{}¦[]\”:;’<>?,./ |

- ssid is the SSID of ESS or IBSS where this pass-phrase is in use, encoded as the hex string used in the Beacons and Probe Responses for the ESS or IBSS. Implementations of this pass-phrase mapping should by default define a unique SSID for each AP, e.g., the BSSID of the AP, and non-AP STA’s should learn the SSID from the AP, so the user need not enter it.

- ssidlength is the number of octets of the ssid.

- 4096 is the number of times the pass-phrase is hashed.

- 256 is the number of bits output by the pass-phrase mapping.

### F.5.2 Reference implementation

/*

* F(P, S, c, i) = U1 xor U2 xor ... Uc

* U1 = PRF(P, S || Int(i))

* U2 = PRF(P, U1)

* Uc = PRF(P, Uc-1)

*/

void F(

char *password,

unsigned char *ssid,

int ssidlength,

int iterations,

int count,

unsigned char *output)

{

unsigned char digest[36], digest1[A_SHA_DIGEST_LEN];

int i, j;

for (i = 0; i < strlen(password); i++) {

assert((password[i] >= 32) && (password[i] <= 126));

}

/* U1 = PRF(P, S || int(i)) */

memcpy(digest, ssid, ssidlength);

digest[ssidlength] = (unsigned char)((count>>24) & 0xff);

digest[ssidlength+1] = (unsigned char)((count>>16) & 0xff);

digest[ssidlength+2] = (unsigned char)((count>>8) & 0xff);

digest[ssidlength+3] = (unsigned char)(count & 0xff);

hmac_sha1(digest, ssidlength+4, (unsigned char*) password,

(int) strlen(password), digest, digest1);

/* output = U1 */

memcpy(output, digest1, A_SHA_DIGEST_LEN);

for (i = 1; i < iterations; i++) {

/* Un = PRF(P, Un-1) */

hmac_sha1(digest1, A_SHA_DIGEST_LEN, (unsigned char*) password, (int) strlen(password), digest);

memcpy(digest1, digest, A_SHA_DIGEST_LEN);

/* output = output xor Un */

for (j = 0; j < A_SHA_DIGEST_LEN; j++) {

output[j] ^= digest[j];

}

}

}

/*

* password - ascii string up to 63 characters in length

* ssid - octet string up to 32 octets

* ssidlength - length of ssid in octets

* output must be 40 octets in length and outputs 256 bits of key

*/

int PasswordHash (

char *password,

unsigned char *ssid,

int ssidlength,

unsigned char *output)

{

if ((strlen(password) > 63) || (ssidlength > 32))

return 0;

F(password, ssid, ssidlength, 4096, 1, output);

F(password, ssid, ssidlength, 4096, 2,

&output[A_SHA_DIGEST_LEN]);

return 1;

}

### F.5.3 Test vectors

Test case 1

Passphrase = “password”

SSID = { ‘I’, ‘E’, ‘E’, ‘E’ }

SSIDLength = 4

PSK  = f42c6fc52df0ebef9ebb4b90b38a5f902e83fe1b135a70e23aed762e9710a12e

Test case 2

Passphrase = “ThisIsAPassword”

SSID = { ‘T’, ‘h’, ‘i’, ‘s’, ‘I’, ‘s’, ‘A’, ‘S’, ‘S’, ‘I’, ‘D’ }

SSIDLength = 11

PSK = 0dc0d6eb90555ed6419756b9a15ec3e3209b63df707dd508d14581f8982721af

Test case 3

Password = “aaaaaaaaaaaaaaaaaaaaaaaaaaaaaaaa”

SSID = {‘Z’,‘Z’,‘Z’,‘Z’, ‘Z’,‘Z’,‘Z’,‘Z’, ‘Z’,‘Z’,‘Z’,‘Z’, ‘Z’,‘Z’,‘Z’,‘Z’, ‘Z’,‘Z’,‘Z’,‘Z’, ‘Z’,‘Z’,‘Z’,‘Z’, ‘Z’,‘Z’,‘Z’,‘Z’,‘Z’,‘Z’,‘Z’,‘Z’}

SSIDLength = 32

PSK =  becb93866bb8c3832cb777c2f559807c8c59afcb6eae734885001300a981cc62

## F.6. Suggestions for random number generation (Informative)

In order to properly implement cryptographic protocols, every platform needs the ability to generate cryptographic quality random numbers. RFC 1750 explains the notion of cryptographic quality random numbers and provides advice on ways to harvest suitable randomness. It recommends sampling multiple sources each of which contains some randomness, and by passing the complete set of samples through a pseudo-random function. By following this advice, an implementation can usually collect enough randomness to distill into a seed for a pseudo-random number generator whose output will be unpredictable.

This annex suggests two sample techniques that can be combined with the other recommendations of RFC 1750 to harvest randomness. The first method is a software solution that can be implemented on most hardware; the second is a hardware-assisted solution. These solutions are expository only, to demonstrate that it is feasible to harvest randomness on any 802.11 platform. They are not mutually exclusive, and they do not preclude the use of other sources of randomness when available; in this case, the more the merrier. As many sources of randomness as possible should be gathered into a buffer, and then hashed, to obtain a seed for the pseudo-random number generator.

### F.6.1 Software Sampling

Due to the nature of clock circuits in modern electronics, there will be some lack of correlation between two clocks in two different pieces of equipment, even when high quality crystals are used—crystal clocks are subject to jitter, noise, drift, and frequency mismatch. This randomness may be as little as the placement of the clock waveform edges. Even if one entity were to attempt to synchronize itself to another entity’s clock, the correlation cannot be perfect, due to noise and uncertainties of the synchronization.

Two clock circuits in the same piece of equipment may synchronize in frequency, but again the correlation will not be perfect due to the noise and jitter of the circuits.

The randomness between the two clocks may not be much per sample—a tenth of a bit or less—but enough samples may be collected to gather enough randomness to form a seed.

A device can use software methods to take advantage of this lack of synchronization, to collect randomness from different sources. As an example, an AP might measure the packet arrival times on an Ethernet  wireless ports. There is always some amount of traffic on modern Ethernets: ARPs, DHCP requests, NetBIOS advertisements, etc. The following example algorithm takes this traffic. In the example, an AP obtains randomness from the available traffic; if Ethernet traffic is available, the AP utilizes that for a source of randomness; otherwise it waits for the first association and creates traffic that it can obtain randomness from.

The clocks used to time the packets should be the highest resolution available, preferable 1ms resolution or better. The clock used to time packet arrival should not be related to the clock used for packet serialization.

Initialize result to empty array

LoopCounter = 0

Wait until Ethernet traffic or association

Repeat until global key counter "random enough" or 32 times {

result = PRF-256(0, "Init Counter",

Local Mac Address || Time || result || LoopCounter)

LoopCounter++

Repeat 32 times {

If Ethernet traffic available then

Take lowest byte of time when Ethernet packet is seen

Concatenate the seen time onto result

else

Start 4-way handshake, aborting after message 2

Take lowest byte of time of when message 1 is sent

Take lowest byte of time of when message 2 is received

Take lowest byte of RSSI from message 2

Take SNonce from message 2

Concatenate the sent time; receive time, RSSI and SNonce onto result

}

}

Global key counter = result = PRF-256(0, "Init Counter",

Local Mac Address || Time || result || LoopCounter)

Note: The Time may be 0 if it is not available.

### F.6.2 Hardware Assisted Solution

This example implementation uses hardware ring oscillators to generate randomness, as depicted in below.

Figure 1—Randomness generating circuit

The above circuit generates randomness. The clock input should be about the same frequency as the ring oscillator’s natural frequencies. The LFSR should be chosen to be one that is maximal length. Example LFSRs can be found in E.G. Arazi, 1988, ‘A Commonsense Approach to the Theory of Error Correcting Codes’, MIT Press

The three ring oscillators should be isolated from each other as much as possible, to avoid harmonic locking between them. In addition, the three ring oscillators should not be near any other clock circuitry within the system, to avoid these ring oscillators locking to system clocks.  The oscillators should be tested to ensure that there output is not correlated.

The output of the LFSR is read by software and concatenated until enough randomness is collected. As a rule of thumb, reading from the LFSR eight to sixteen times the number of bits as the desired number of random bits is sufficient.

Initialize result to empty array

Repeat 1024 times {

Read LFSR

result = result | LFSR

Wait a time period

}

Global key counter = PRF-256(0, "Init Counter", result)

## F.7. Additional test vectors

### F.7.1 Notation

In the examples here, frames are represented as a stream of octets, each octet in hex notation, sometimes with text annotation.  The order of transmission for octets is left to right, top to bottom.  For example, consider the following representation of a frame:

| Description #1 | 00 01 02 03 |
| --- | --- |
|  | 04 05 |
| Description #2 | 06 07 08 |

The frame consists of nine octets, represented in hex notation as “00”, “01”, ..., “08”.  The octet represented by “00” is transmitted first, and the octet represented by “08” is transmitted last.  Similar tables are used for other purposes, such as describing a cryptographic operation.

In the text discussion outside of tables, integer values are represented in either hex notation using an “0x” prefix or in decimal notation using no prefix.  For example, the hex notation 0x12345 and the decimal notation 74565 represent the same integer value.

### F.7.2 WEP Encapsulation (Normative)

The discussion here represents an RC4 encryption using a table that shows the key, plaintext input, and ciphertext output.  For reference, here is a table that describes test vector “Commerce” of  <draft-kaukonen-cipher-arcfour-03.txt>, a work-in-progress.

| Key | 61 8a 63 d2 fb |
| --- | --- |
| Plaintext | dc ee 4c f9 2c |
| Ciphertext | f1 38 29 c9 de |

The MPDU data, prior to WEP encapsulation, is as follows:

| MPDU data | aa aa 03 00 00 00 08 00 45 00 00 4e 66 1a 00 00 80 11 be 64 0a 00 01 22 0a ff ff ff 00 89 00 89 00 3a 00 00 80 a6 01 10 00 01 00 00 00 00 00 00 20 45 43 45 4a 45 48 45 43 46 43 45 50 46 45 45 49 45 46 46 43 43 41 43 41 43 41 43 41 43 41 41 41 00 00 20 00 01 |
| --- | --- |

RC4 encryption is performed as follows:

| Key | fb 02 9e 30 31 32 33 34 |
| --- | --- |
| Plaintext | aa aa 03 00 00 00 08 00 45 00 00 4e 66 1a 00 00 80 11 be 64 0a 00 01 22 0a ff ff ff 00 89 00 89 00 3a 00 00 80 a6 01 10 00 01 00 00 00 00 00 00 20 45 43 45 4a 45 48 45 43 46 43 45 50 46 45 45 49 45 46 46 43 43 41 43 41 43 41 43 41 43 41 41 41 00 00 20 00 01 1b d0 b6 04 |
| Ciphertext | f6 9c 58 06 bd 6c e8 46 26 bc be fb 94 74 65 0a ad 1f 79 09 b0 f6 4d 5f 58 a5 03 a2 58 b7 ed 22 eb 0e a6 49 30 d3 a0 56 a5 57 42 fc ce 14 1d 48 5f 8a a8 36 de a1 8d f4 2c 53 80 80 5a d0 c6 1a 5d 6f 58 f4 10 40 b2 4b 7d 1a 69 38 56 ed 0d 43 98 e7 ae e3 bf 0e 2a 2c a8 f7 |

The plaintext consists of the MPDU data, followed by a 4-octet CRC-32 calculated over the MPDU data.

The expanded MPDU, after WEP encapsulation, is as follows:

| IV | fb 02 9e 80 |
| --- | --- |
| MPDU data | f6 9c 58 06 bd 6c e8 46 26 bc be fb 94 74 65 0a ad 1f 79 09 b0 f6 4d 5f 58 a5 03 a2 58 b7 ed 22 eb 0e a6 49 30 d3 a0 56 a5 57 42 fc ce 14 1d 48 5f 8a a8 36 de a1 8d f4 2c 53 80 80 5a d0 c6 1a 5d 6f 58 f4 10 40 b2 4b 7d 1a 69 38 56 ed 0d 43 98 e7 ae e3 bf 0e |
| ICV | 2a 2c a8 f7 |

The IV consists of the first three octets of the RC4 key, followed by an octet containing the KeyID value in the upper two bits.  In this example, the KeyID value is 2.  The MPDU data consists of the ciphertext, excluding the last four octets.  The ICV consists of the last four octets of the ciphertext, which is the encrypted CRC-32 value.

### F.7.3 TKIP test vectors (Normative)

An example of a TKIP packet is provided. The key and PN are used to create the IV, Phase1 and Phase2 keys.

| Key | 12 34 56 78 90 12 34 56 78 90 12 34 56 78 90 12 34 56 78 90 12 34 56 78 90 12 34 56 78 90 12 34 |
| --- | --- |
| PN | 0x000000000001 |
| IV | 00 20 01 20 00 00 00 00 |
| Phase1 | bb 58 07 1f 9e 93 b4 38 25 4b |
| Phase2 | 00 20 01 4c fe 67 be d2 7c 86 7b 1b f8 02 8b 1c |

| Plaintext MPDU with TKIP MIC | 08 42 2c 00 02 03 04 05 06 08 02 03 04 05 06 07 02 03 04 05 06 07 d0 02 00 20 01 20 00 00 00 00 aa aa 03 00 00 00 08 00 45 00 00 54 00 00 40 00 40 01 a5 55 c0 a8 0a 02 c0 a8 0a 01 08 00 3a b0 00 00 00 00 cd 4c 05 00 00 00 00 00 08 09 0a 0b 0c 0d 0e 0f 10 11 12 13 14 15 16 17 18 19 1a 1b 1c 1d 1e 1f 20 21 22 23 24 25 26 27 28 29 2a 2b 2c 2d 2e 2f 30 31 32 33 34 35 36 37 68 81 a3 f3 d6 48 d0 3c |
| --- | --- |
| Encrypted MPDU with MIC and ICV | 08 42 2c 00 02 03 04 05 06 08 02 03 04 05 06 07 02 03 04 05 06 07 d0 02 00 20 01 20 00 00 00 00 c0 0e 14 fc e7 cf ab c7 75 47 e6 66 e5 7c 0d ac 70 4a 1e 35 8a 88 c1 1c 8e 2e 28 2e 38 01 02 7a 46 56 05 5e e9 3e 9c 25 47 02 e9 73 58 05 dd b5 76 9b a7 3f 1e bb 56 e8 44 ef 91 22 85 d3 dd 6e 54 1e 82 38 73 55 8a db a0 79 06 8a bd 7f 7f 50 95 96 75 ac c4 b4 de 9a a9 9c 05 f2 89 a7 c5 2f ee 5b fc 14 f6 f8 e5 f8 |

### F.7.4 CCMP test vectors (Normative)

Twelve examples of CCMP encrypted packets are provided.

==== CCMP test mpdu   1 ====

-- MPDU Fields

Version  = 0

Type     = 2   SubType  = 0  Data

ToDS     = 0   FromDS   = 0

MoreFrag = 0   Retry    = 1

PwrMgt   = 0   moreData = 0

Encrypt  = 1

Order    = 0

Duration = 11459

A1 = 0f-d2-e1-28-a5-7c    DA

A2 = 50-30-f1-84-44-08    SA

A3 = ab-ae-a5-b8-fc-ba    BSSID

SC = 0x3380

seqNum = 824 (0x0338)  fraqNum = 0 (0x00)

Algorithm = AES_CCM

KeyId = 0

TK =	c9 7c 1f 67 ce 37 11 85  51 4a 8a 19 f2 bd d5 2f

PN =	199027030681356  (0xB5039776E70C)

802.11 Header =	08 48 c3 2c 0f d2 e1 28 a5 7c 50 30 f1 84 44 08 ab ae a5 b8 fc ba 80 33

Muted 802.11 Header =	08 40 0f d2 e1 28 a5 7c 50 30 f1 84 44 08 ab ae a5 b8 fc ba 00 00

CCMP Header =	0c e7 00 20 76 97 03 b5

CCM Nonce =	00 50 30 f1 84 44 08 b5  03 97 76 e7 0c

Plaintext Data =	f8 ba 1a 55 d0 2f 85 ae 96 7b b6 2f b6 cd a8 eb 7e 78 a0 50

CCM MIC =	78 45 ce 0b 16 f9 76 23

-- Encrypted MPDU with FCS

08 48 c3 2c 0f d2 e1 28 a5 7c 50 30 f1 84 44 08 ab ae a5 b8 fc ba 80 33 0c e7 00 20 76 97 03 b5 f3 d0 a2 fe 9a 3d bf 23 42 a6 43 e4 32 46 e8 0c 3c 04 d0 19 78 45 ce 0b 16 f9 76 23 1d 99 f0 66

==== CCMP test mpdu   2 ====

-- MPDU Fields

Version  = 0

Type     = 2   SubType  = 3  Data+CF-Ack+CF-Poll

ToDS     = 0   FromDS   = 0

MoreFrag = 0   Retry    = 0

PwrMgt   = 0   moreData = 0

Encrypt  = 1

Order    = 1

Duration = 20842

A1 = ea-10-0c-84-68-50    DA

A2 = ee-c1-76-2c-88-de    SA

A3 = af-2e-e9-f4-6a-07    BSSID

SC = 0xCCE0

seqNum = 3278 (0x0CCE)  fraqNum = 0 (0x00)

Algorithm = AES_CCM

KeyId = 2

TK =	8f 7a 05 3f a5 77 a5 59  75 29 27 20 97 a6 03 d5

PN =	54923164817386  (0x31F3CBBA97EA)

802.11 Header =	38 c0 6a 51 ea 10 0c 84 68 50 ee c1 76 2c 88 de af 2e e9 f4 6a 07 e0 cc

Muted 802.11 Header =	08 c0 ea 10 0c 84 68 50 ee c1 76 2c 88 de af 2e e9 f4 6a 07 00 00

CCMP Header =	ea 97 00 a0 ba cb f3 31

CCM Nonce =	00 ee c1 76 2c 88 de 31  f3 cb ba 97 ea

Plaintext Data =	83 a0 63 4b 5e d7 62 7e b9 df 22 5e 05 74 03 42 de 19 41 17

CCM MIC =	54 2f bf 8d a0 6a a4 ae

-- Encrypted MPDU with FCS

38 c0 6a 51 ea 10 0c 84 68 50 ee c1 76 2c 88 de af 2e e9 f4 6a 07 e0 cc ea 97 00 a0 ba cb f3 31 81 4b 69 65 d0 5b f2 b2 ed 38 d4 be b0 69 fe 82 71 4a 61 0b 54 2f bf 8d a0 6a a4 ae 25 3c 47 38

==== CCMP test mpdu   3 ====

-- MPDU Fields

Version  = 0

Type     = 2   SubType  = 11  QC&Data+CF-Ack+CF-Poll

ToDS     = 0   FromDS   = 0

MoreFrag = 0   Retry    = 1

PwrMgt   = 0   moreData = 0

Encrypt  = 1

Order    = 1

Duration = 25052

A1 = d9-57-7d-f7-63-c8    DA

A2 = b6-a8-8a-df-36-91    SA

A3 = dc-4a-8b-ca-94-dd    BSSID

SC = 0x8260

seqNum = 2086 (0x0826)  fraqNum = 0 (0x00)

QC = 0x8520

QoS-TID  = 0 (0x0)  QC-bit-4 = 0

QoS-AckP = 1     QoS-TXOP/QL = 0x85

Algorithm = AES_CCM

KeyId = 2

TK =	40 cf b7 a6 2e 88 01 3b  d6 d3 af fc c1 91 04 1e

PN =	52624639632814  (0x2FDCA0F3A5AE)

802.11 Header =	b8 c8 dc 61 d9 57 7d f7 63 c8 b6 a8 8a df 36 91 dc 4a 8b ca 94 dd 60 82 20 85

Muted 802.11 Header =	88 c0 d9 57 7d f7 63 c8 b6 a8 8a df 36 91 dc 4a 8b ca 94 dd 00 00 00 00

CCMP Header =	ae a5 00 a0 f3 a0 dc 2f

CCM Nonce =	00 b6 a8 8a df 36 91 2f dc a0 f3 a5 ae

Plaintext Data  =	2c 1b d0 36 83 1c 95 49 6c 5f 4d bf 3d 55 9e 72 de 80 2a 18

CCM MIC =	fd 1f 1f 61 a9 fb 4b b3

-- Encrypted MPDU with FCS

b8 c8 dc 61 d9 57 7d f7 63 c8 b6 a8 8a df 36 91 dc 4a 8b ca 94 dd 60 82 20 85 ae a5 00 a0 f3 a0 dc 2f 89 d8 58 03 40 b6 26 a0 b6 d4 d0 13 bf 18 f2 91 b8 96 46 c8 fd 1f 1f 61 a9 fb 4b b3 60 3f 5a ad

==== CCMP test mpdu   4 ====

-- MPDU Fields

Version  = 0

Type     = 2   SubType  = 10  QC&Data+CF-Poll

ToDS     = 0   FromDS   = 1

MoreFrag = 0   Retry    = 1

PwrMgt   = 0   moreData = 0

Encrypt  = 1

Order    = 1

Duration = 4410

A1 = 71-2a-9d-df-11-db    DA

A2 = 8e-f8-22-73-47-01    BSSID

A3 = 59-14-0d-d6-46-a2    SA

SC = 0x2FC0

seqNum = 764 (0x02FC)  fraqNum = 0 (0x00)

QC = 0xa567

QoS-TID  = 7 (0x7)  QC-bit-4 = 0

QoS-TID  = 7 (0x7)  QoS-FEC = 0

QoS-AckP = 3     QoS-TXOP/QL = 0xa5

Algorithm = AES_CCM

KeyId = 0

TK =	8c 89 a2 eb c9 6c 76 02  70 7f cf 24 b3 2d 38 33

PN =	270963670912995  (0xF670A55A0FE3)

802.11 Header =	a8 ca 3a 11 71 2a 9d df 11 db 8e f8 22 73 47 01 59 14 0d d6 46 a2 c0 2f 67 a5

Muted 802.11 Header =	88 c2 71 2a 9d df 11 db 8e f8 22 73 47 01 59 14 0d d6 46 a2 00 00 07 00

CCMP Header =	e3 0f 00 20 5a a5 70 f6

CCM Nonce =	07 8e f8 22 73 47 01 f6  70 a5 5a 0f e3

Plaintext Data =	4f ad 2b 1c 29 0f a5 eb d8 72 fb c3 f3 a0 74 89 8f 8b 2f bb

CCM MIC =	31 fc 88 00 4f 35 ee 3d

-- Encrypted MPDU with FCS

a8 ca 3a 11 71 2a 9d df 11 db 8e f8 22 73 47 01 59 14 0d d6 46 a2 c0 2f 67 a5 e3 0f 00 20 5a a5 70 f6 9d 59 b1 5f 37 14 48 c2 30 f4 d7 39 05 2e 13 ab 3b 1a 7b 10 31 fc 88 00 4f 35 ee 3d 45 a7 4a 30

==== CCMP test mpdu   5 ====

-- MPDU Fields

Version  = 0

Type     = 2   SubType  = 8  QC&Data

ToDS     = 0   FromDS   = 1

MoreFrag = 0   Retry    = 1

PwrMgt   = 1   moreData = 0

Encrypt  = 1

Order    = 1

Duration = 16664

A1 = 45-de-c6-9a-74-80    DA

A2 = f3-51-94-6b-c9-6b    BSSID

A3 = e2-76-fb-e6-c1-27    SA

SC = 0xF280

seqNum = 3880 (0x0F28)  fraqNum = 0 (0x00)

QC = 0x194b

QoS-TID  = 11 (0xB)  QC-bit-4 = 0

QoS-AckP = 2     QoS-TXOP/QL = 0x19

Algorithm = AES_CCM

KeyId = 2

TK =	a5 74 d5 14 3b b2 5e fd  de ff 30 12 2f df d0 66

PN = 184717420531255  (0xA7FFE03C0E37)

802.11 Header =	88 da 18 41 45 de c6 9a 74 80 f3 51 94 6b c9 6b e2 76 fb e6 c1 27 80 f2 4b 19

Muted 802.11 Header =	88 c2 45 de c6 9a 74 80 f3 51 94 6b c9 6b e2 76 fb e6 c1 27 00 00 0b 00

CCMP Header =	37 0e 00 a0 3c e0 ff a7

CCM Nonce =	0b f3 51 94 6b c9 6b a7 ff e0 3c 0e 37

Plaintext Data =	28 96 9b 95 4f 26 3a 80 18 a9 ef 70 a8 b0 51 46 24 81 92 2e

CCM MIC =	ce 0c 3b e1 97 d3 05 eb

-- Encrypted MPDU with FCS

88 da 18 41 45 de c6 9a 74 80 f3 51 94 6b c9 6b e2 76 fb e6 c1 27 80 f2 4b 19 37 0e 00 a0 3c e0 ff a7 eb 4a e4 95 6a 80 1d a9 62 4b 7e 0c 18 b2 3e 61 5e c0 3a f6 ce 0c 3b e1 97 d3 05 eb c8 9e a1 b5

==== CCMP test mpdu   6 ====

-- MPDU Fields

Version  = 0

Type     = 2   SubType  = 8  QC&Data

ToDS     = 0   FromDS   = 1

MoreFrag = 0   Retry    = 0

PwrMgt   = 1   moreData = 0

Encrypt  = 1

Order    = 0

Duration = 8161

A1 = 5a-f2-84-30-fd-ab    DA

A2 = bf-f9-43-b9-f9-a6    BSSID

A3 = ab-1d-98-c7-fe-73    SA

SC = 0x7150

seqNum = 1813 (0x0715)  fraqNum = 0 (0x00)

QC = 0x6a3d

QoS-TID  = 13 (0xD)  QC-bit-4 = 1

QoS-AckP = 1     QoS-TXOP/QL = 0x6a

Algorithm = AES_CCM

KeyId = 1

TK =	f7 1e ea 4e 1f 58 80 4b 97 17 23 0a d0 61 46 41

PN    = 118205765159305  (0x6B81ECA48989)

802.11 Header =	88 52 e1 1f 5a f2 84 30 fd ab bf f9 43 b9 f9 a6 ab 1d 98 c7 fe 73 50 71  3d 6a

Muted 802.11 Header =	88 42 5a f2 84 30 fd ab bf f9 43 b9 f9 a6 ab 1d 98 c7 fe 73 00 00 0d 00

CCMP Header =	89 89 00 60 a4 ec 81 6b

CCM Nonce =	0d bf f9 43 b9 f9 a6 6b  81 ec a4 89 89

Plaintext Data =	ab fd a2 2d 3a 0b fc 9c c1 fc 07 93 63 c2 fc a1 43 e6 eb 1d

CCM MIC =	30 9a 8d 5c 46 6b bb 71

-- Encrypted MPDU with FCS

88 52 e1 1f 5a f2 84 30 fd ab bf f9 43 b9 f9 a6 ab 1d 98 c7 fe 73 50 71 3d 6a 89 89 00 60 a4 ec 81 6b 9a 70 9b 60 a3 9d 40 b1 df b6 12 e1 8b 5f 11 4b ad b6 cc 86 30 9a 8d 5c 46 6b bb 71 86 c0 4e 97

==== CCMP test mpdu   7 ====

-- MPDU Fields

Version  = 0

Type     = 2   SubType  = 1  Data+CF-Ack

ToDS     = 1   FromDS   = 0

MoreFrag = 0   Retry    = 1

PwrMgt   = 1   moreData = 1

Encrypt  = 1

Order    = 0

Duration = 18049

A1 = 9b-50-f4-fd-56-f6    BSSID

A2 = ef-ec-95-20-16-91    SA

A3 = 83-57-0c-4c-cd-ee    DA

SC = 0xA020

seqNum = 2562 (0x0A02)  fraqNum = 0 (0x00)

Algorithm = AES_CCM

KeyId = 3

TK =	1b db 34 98 0e 03 81 24 a1 db 1a 89 2b ec 36 6a

PN =	104368786630435  (0x5EEC4073E723)

Header =	18 79 81 46 9b 50 f4 fd 56 f6 ef ec 95 20 16 91 83 57 0c 4c cd ee 20 a0

Muted MAC Header =	08 41 9b 50 f4 fd 56 f6 ef ec 95 20 16 91 83 57 0c 4c cd ee 00 00

CCMP Header =	23 e7 00 e0 73 40 ec 5e

CCM Nonce =	00 ef ec 95 20 16 91 5e ec 40 73 e7 23

Plaintext Data =	98 be ca 86 f4 b3 8d a2 0c fd f2 47 24 c5 8e b8 35 66 53 39

CCM MIC =	2d 09 57 ec fa be 95 b9

-- Encrypted MPDU with FCS

18 79 81 46 9b 50 f4 fd 56 f6 ef ec 95 20 16 91 83 57 0c 4c cd ee 20 a0 23 e7 00 e0 73 40 ec 5e 2 c5 37 eb f3 ab 58 4e f1 fe f9 a1 f3 54 7a 8c 13 b3 22 5a 2d 09 57 ec fa be 95 b9 aa fa 0c c8

==== CCMP test mpdu   8 ====

-- MPDU Fields

Version  = 0

Type     = 2   SubType  = 11  QC&Data+CF-Ack+CF-Poll

ToDS     = 1   FromDS   = 0

MoreFrag = 0   Retry    = 1

PwrMgt   = 1   moreData = 0

Encrypt  = 1

Order    = 1

Duration = 29260

A1 = 55-2d-5f-72-bb-70    BSSID

A2 = ca-3f-3a-ae-60-c4    SA

A3 = 8b-a9-b5-f8-2c-2f    DA

SC = 0xEB50

seqNum = 3765 (0x0EB5)  fraqNum = 0 (0x00)

QC = 0x552a

QoS-TID  = 10 (0xA)  QC-bit-4 = 0

QoS-AckP = 1     QoS-TXOP/QL = 0x55

Algorithm = AES_CCM

KeyId = 2

TK =	6e ac 1b f5 4b d5 4e db 23 21 75 43 03 02 4c 71

PN    = 227588596223197  (0xCEFD996ECCDD)

802.11 Header =	b8 d9 4c 72 55 2d 5f 72 bb 70 ca 3f 3a ae 60 c4 8b a9 b5 f8 2c 2f 50 eb 2a 55

Muted 802.11 Header =	88 c1 55 2d 5f 72 bb 70 ca 3f 3a ae 60 c4 8b a9 b5 f8 2c 2f 00 00 0a 00

CCMP Header =	dd cc 00 a0 6e 99 fd ce

CCM Nonce =	0a ca 3f 3a ae 60 c4 ce fd 99 6e cc dd

Plaintext Data =	57 cb 5c 0e 5f cd 88 5e 9a 42 39 e9 b9 ca d6 0d 64 37 59 79

CCM MIC =	6d ba 8e f7 f0 80 87 dd

-- Encrypted MPDU with FCS

b8 d9 4c 72 55 2d 5f 72  bb 70 ca 3f 3a ae 60 c4 8b a9 b5 f8 2c 2f 50 eb 2a 55 dd cc 00 a0 6e 99 fd ce 4b f2 81 ef 8e c7 73 9f 91 59 1b 97 a8 7d c1 4b 3f a1 74 62 6d ba 8e f7 f0 80 87 dd 0c 65 74 3f

==== CCMP test mpdu   9 ====

-- MPDU Fields

Version  = 0

Type     = 2   SubType  = 10  QC&Data+CF-Poll

ToDS     = 1   FromDS   = 0

MoreFrag = 0   Retry    = 1

PwrMgt   = 1   moreData = 1

Encrypt  = 1

Order    = 1

Duration = 17096

A1 = 9a-fb-79-8b-8a-4b    BSSID

A2 = a4-ad-6d-31-99-85    SA

A3 = bc-42-9e-8f-0a-fa    DA

SC = 0x7DF0

seqNum = 2015 (0x07DF)  fraqNum = 0 (0x00)

QC = 0x804a

QoS-TID  = 10 (0xA)  QC-bit-4 = 0

QoS-AckP = 2     QoS-TXOP/QL = 0x80

Algorithm = AES_CCM

KeyId = 3

TK =	49 4b 50 1e 19 46 75 97 1a 48 d0 8c 5b c3 53 cb

PN = 205071421028275  (0xBA82E93437B3)

802.11 Header =	a8 f9 c8 42 9a fb 79 8b 8a 4b a4 ad 6d 31 99 85 bc 42 9e 8f 0a fa f0 7d 4a 80

Muted MAC Header =	88 c1 9a fb 79 8b 8a 4b a4 ad 6d 31 99 85 bc 42 9e 8f 0a fa 00 00 0a 00

CCMP Header =	b3 37 00 e0 34 e9 82 ba

CCM Nonce =	0a a4 ad 6d 31 99 85 ba  82 e9 34 37 b3

Plaintext Data  =	25 a9 8f 9c 1b d9 c9 3c f3 83 ab 9d 98 15 2d 76 cb 4a 32 c6

CCM MIC =	cf e4 38 cb ea 61 fa 9a

-- Encrypted MPDU with FCS

a8 f9 c8 42 9a fb 79 8b 8a 4b a4 ad 6d 31 99 85 bc 42 9e 8f 0a fa f0 7d 4a 80 b3 37 00 e0 34 e9 82 ba 56 1a 0d 06 8e ac 2e ad b0 c5 7f e2 d0 a6 cc 73 98 b6 dd bf cf e4 38 cb ea 61 fa 9a 7d 46 d3 72

==== CCMP test mpdu  10 ====

-- MPDU Fields

Version  = 0

Type     = 2   SubType  = 1  Data+CF-Ack

ToDS     = 1   FromDS   = 1

MoreFrag = 0   Retry    = 1

PwrMgt   = 0   moreData = 0

Encrypt  = 1

Order    = 0

Duration = 25568

A1 = 40-ec-29-fa-75-9b    RA

A2 = 53-f8-69-fe-27-9a    TA

A3 = f0-f9-f8-a6-54-16    DA

A4 = 52-bf-d2-70-3d-24    SA

SC = 0xE380

seqNum = 3640 (0x0E38)  fraqNum = 0 (0x00)

Algorithm = AES_CCM

KeyId = 0

TK =	48 9e 49 bc 3c fe 3f ce 38 95 82 0e 87 2e e1 a3

PN    = 227724508921513  (0xCF1D3E75FEA9)

802.11 Header =	18 4b e0 63 40 ec 29 fa 75 9b 53 f8 69 fe 27 9a f0 f9 f8 a6 54 16 80 e3 52 bf d2 70 3d 24

Muted 802.11 Header =	08 43 40 ec 29 fa 75 9b 53 f8 69 fe 27 9a f0 f9 f8 a6 54 16 00 00 52 bf  d2 70 3d 24

CCMP Header =	a9 fe 00 20 75 3e 1d cf

CCM Nonce =	00 53 f8 69 fe 27 9a cf 1d 3e 75 fe a9

Plaintext Data  =	7f 91 f2 47 2d 7a 12 1c 9c dd 4b 6c 90 80 67 5a 10 20 aa 00

CCM MIC =	c8 68 a7 25 55 2c 55 65

-- Encrypted MPDU with FCS

18 4b e0 63 40 ec 29 fa 75 9b 53 f8 69 fe 27 9a f0 f9 f8 a6 54 16 80 e3 52 bf d2 70 3d 24 a9 fe 00 20 75 3e 1d cf 25 df 51 73 83 5e 4f ba 23 bc 05 a2 53 88 5e be d3 ac 48 71 c8 68 a7 25 55 2c 55 65 d2 c0 83 6e

==== CCMP test mpdu  11 ====

-- MPDU Fields

Version  = 0

Type     = 2   SubType  = 11  QC&Data+CF-Ack+CF-Poll

ToDS     = 1   FromDS   = 1

MoreFrag = 0   Retry    = 1

PwrMgt   = 1   moreData = 0

Encrypt  = 1

Order    = 1

Duration = 21610

A1 = 29-8c-0b-aa-91-90    RA

A2 = 34-7c-eb-9a-ab-ff    TA

A3 = d8-3d-48-86-e5-c2    DA

A4 = e2-9d-52-4a-e1-96    SA

SC = 0x53E0

seqNum = 1342 (0x053E)  fraqNum = 0 (0x00)

QC = 0x623d

QoS-TID  = 13 (0xD)  QC-bit-4 = 1

QoS-AckP = 1     QoS-TXOP/QL = 0x62

Algorithm = AES_CCM

KeyId = 3

TK =	02 be 5c 45 45 67 2a 07  e4 e3 14 d7 0f 1f 9e 85

PN =	231818065237460  (0xD2D6596E55D4)

802.11 Header =	b8 db 6a 54 29 8c 0b aa 91 90 34 7c eb 9a ab ff d8 3d 48 86 e5 c2 e0 53 e2 9d 52 4a e1 96 3d 62

Muted 802.11 Header =	88 c3 29 8c 0b aa 91 90 34 7c eb 9a ab ff d8 3d 48 86 e5 c2 00 00 e2 9d 52 4a e1 96 0d 00

CCMP Header =	 d4 55 00 e0 6e 59 d6 d2

CCM Nonce = 	0d 34 7c eb 9a ab ff d2  d6 59 6e 55 d4

Plaintext Data =	f9 a8 12 e4 a2 8a f7 f3 71 4d 4b f6 62 2e 59 32 f2 18 45 09

CCM MIC =	68 1f de e8 51 3c 62 dc

-- Encrypted MPDU with FCS

b8 db 6a 54 29 8c 0b aa 91 90 34 7c eb 9a ab ff d8 3d 48 86 e5 c2 e0 53 e2 9d 52 4a e1 96 3d 62 d4 55 00 e0 6e 59 d6 d2 63 15 50 0f 92 42 95 cd 3e af bd c3 e1 51 b1 df 46 46 5b 71 68 1f de e8 51 3c 62 dc 9d 3e ca 99

==== CCMP test mpdu  12 ====

-- MPDU Fields

Version  = 0

Type     = 2   SubType  = 1  Data+CF-Ack

ToDS     = 1   FromDS   = 1

MoreFrag = 0   Retry    = 1

PwrMgt   = 0   moreData = 1

Encrypt  = 1

Order    = 0

Duration = 10192

A1 = aa-28-8b-84-35-bc    RA

A2 = d8-ac-5a-7e-c4-44    TA

A3 = e8-b4-62-50-53-8b    DA

A4 = e8-14-02-c2-ee-11    SA

SC = 0x6280

seqNum = 1576 (0x0628)  fraqNum = 0 (0x00)

Algorithm = AES_CCM

KeyId = 3

TK =	77 07 7e d7 94 53 e4 a1 8d 60 43 8c c6 48 4d 6e

PN =	88717355646350  (0x50B01E77FD8E)

802.11 Header =	18 6b d0 27 aa 28 8b 84 35 bc d8 ac 5a 7e c4 44 e8 b4 62 50 53 8b 80 62 e8 14 02 c2 ee 11

Muted 802.11 Header =	08 43 aa 28 8b 84 35 bc d8 ac 5a 7e c4 44 e8 b4 62 50 53 8b 00 00 e8 14  02 c2 ee 11

CCMP Header =	8e fd 00 e0 77 1e b0 50

CCM Nonce =	00 d8 ac 5a 7e c4 44 50 b0 1e 77 fd 8e

Plaintext Data  =	 43 19 81 a2 33 6d 02 f8 cb 84 48 d5 42 89 16 be 95 29 35 37

CCM MIC =	fd 0f 41 f4 26 bb 8f 30

-- Encrypted MPDU with FCS

18 6b d0 27 aa 28 8b 84 35 bc d8 ac 5a 7e c4 44 e8 b4 62 50 53 8b 80 62 e8 14 02 c2 ee 11 8e fd 00 e0 77 1e b0 50 cf 71 b2 cc bd 59 0b 20 80 07 92 f3 59 ed 1c fd 74 d8 00 b4 fd 0f 41 f4 26 bb 8f 30 f6 b2 91 ea

### F.7.5 PRF Test Vectors (Normative)

A set of test vectors are provided for each size of PRF function used in this specification.  The input to the PRF function are strings for ‘key’, ‘prefix’ and ‘data’.  The length can be any multiple of 8, but the values: 192, 256, 384, 512 and 768 are used in this specification. The test vectors were taken from RFC2202 with additional vectors added to test larger key and data sizes.

| Test_case | 1 |
| --- | --- |
| Key | 0b 0b 0b 0b 0b 0b 0b 0b 0b 0b 0b 0b 0b 0b 0b 0b 0b 0b 0b 0b |
| prefix | "prefix" |
| Data | "Hi There" |
| length | 192 |
| PRF-192 | bc d4 c6 50 b3 0b 96 84 95 18 29 e0 d7 5f 9d 54 b8 62 17 5e d9 f0 06 06 |

| Test_case | 2 |
| --- | --- |
| Key | 'Jefe' |
| prefix | "prefix-2" |
| Data | "what do ya want for nothing?" |
| length | 256 |
| PRF-256 | 47 c4 90 8e 30 c9 47 52 1a d2 0b e9 05 34 50 ec be a2 3d 3a a6 04 b7 73 26 d8 b3 82 5f f7 47 5c |

| Test_case | 3 |
| --- | --- |
| Key | aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa |
| prefix | "prefix-3" |
| Data | "Test Using Larger Than Block-Size Key - Hash Key First" |
| length | 384 |
| PRF-384 | 0a b6 c3 3c cf 70 d0 d7 36 f4 b0 4c 8a 73 73 25 55 11 ab c5 07 37 13 16 3b d0 b8 c9 ee b7 e1 95 6f a0 66 82 0a 73 dd ee 3f 6d 3b d4 07 e0 68 2a |

| Test_case | 4 |
| --- | --- |
| Key | 0b 0b 0b 0b 0b 0b 0b 0b 0b 0b 0b 0b 0b 0b 0b 0b 0b 0b 0b 0b |
| prefix | "prefix-4" |
| Data | "Hi There Again" |
| length | 512 |
| PRF-512 | 24 8c fb c5 32 ab 38 ff a4 83 c8 a2 e4 0b f1 70 eb 54 2a 2e 09 16 d7 bf 6d 97 da 2c 4c 5c a8 77 73 6c 53 a6 5b 03 fa 4b 37 45 ce 76 13 f6 ad 68 e0 e4 a7 98 b7 cf 69 1c 96 17 6f d6 34 a5 9a 49 |

| Test_case | 5 |
| --- | --- |
| Key | aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa |
| Prefix | "prefix-5" |
| Data | "Test Using Larger Than Block-Size Key and Larger Than One Block-Size Data" |
| Length | 768 |
| PRF-768 | 67 27 a3 e8 d5 2c f2 70 08 ce 4d 68 3e 45 99 25 c6 23 5b e0 0c 8c 13 03 77 26 af fc bc 02 29 17 a5 94 1c 0c 77 4b 00 25 7f 77 c6 e2 4c 81 02 87 8e 04 b7 2c f6 c7 88 a7 ba ec 4f 69 68 7b eb d6 30 15 59 ca 1f c2 6f 93 04 2e 1e 82 ba 28 9a 05 2c a8 51 ef cd 4e 15 a1 5d d0 4c bb e1 f6 94 58 |

## F.8 Key hierarchy test vectors (Normative)

The following test vectors provide an example of both pairwise and group key derivation for CCMPand TKIP.

### F.8.1 Pairwise Key Derivation

Pairwise keys are derived from the PMK, AA, SA, SNonce and ANonce. The values below are used as input to the pairwise key derivation test vectors.

| PMK | 0d c0 d6 eb 90 55 5e d6 41 97 56 b9 a1 5e c3 e3 20 9b 63 df 70 7d d5 08 d1 45 81 f8 98 27 21 af |
| --- | --- |
| AA | a0 a1 a1 a3 a4 a5 |
| SA | b0 b1 b2 b3 b4 b5 |
| SNonce | c0 c1 c2 c3 c4 c5 c6 c7 c8 c9 d0 d1 d2 d3 d4 d5 d6 d7 d8 d9 |
| ANonce | e0 e1 e2 e3 e4 e5 e6 e7 e8 e9 f0 f1 f2 f3 f4 f5 f6 f7 f8 f9 |

#### F.8.1.1 CCMP Pairwise Key Derivation

Using the values from section F.8.1 for PMK, AA, SA, SNonce and ANonce the key derivation process for CCMP generates:

| TK | 8c b7 78 33 2e 94 ac a6 d3 0b 89 cb e8 2a 9c a9 |
| --- | --- |

#### F.8.1.2 TKIP Pairwise Key Derivation

Using the values from section F.8.1 for PMK, AA, SA, SNonce and ANonce the key derivation process for TKIP generates:

| KCK | Aa 7c fc 85 60 25 1e 4b c6 87 e0 cb 8d 29 83 63 |
| --- | --- |
| KEK | Ba 53 16 3d f3 2a 86 38 f4 79 ab e3 4b fd 2b c8 |
| TK | 8c b7 78 33 2e 94 ac a6 d3 0b 89 cb e8 2a 9c a9 36 4a ff bb ce 87 5f 5d f2 dd 58 41 c0 ed 2a 41 |
| Authenticator Tx MIC_key | 36 4a ff bb ce 87 5f 5d |
| Supplicant Tx MIC_key | f2 dd 58 41 c0 ed 2a 41 |

## F.9 Michael Countermeasures Logic (Informative)

Figure 2 and Figure 3 depict an informative representation of logic implementing Michael Countermeasures. Figure 2 depicts the portions of the countermeasures algorithm implemented by the IEEE 802.11 AP, Figure 3 while  depicts the portions implemented by the AP’s IEEE 802.1X Authenticator.

Figure 2—AP Michael Countermeasures Logic (Informative)

Figure 3—Authenticator Michael Countermeasure Logic (Informative)

Figure 4, Figure 5 and Figure 6 depict an informative representation of one possible partitioning of the Michael countermeasures among the STA and its Supplicant. Figure 4 and Figure 5 depict the portions of the algorithm that might be implemented in the IEEE 802.11 STA, while Figure 6 depicts the portions that might be implemented in the STA’s IEEE 802.1X Supplicant.

Figure 4—STA Michael Countermeasure Logic, Part 1 (Informative)

Figure 5—STA Michael Countermeasures Logic, Part 2 (Informative)

Figure 6—Supplicant Michael Countermeasures Logic