IEEE P802.11
Wireless LANs

Letter Ballot 60 Clause 7 Individual Motions

Date:	August 26, 2003

Authors:	Dave Halasz
	Cisco Systems

Frank Ciotti
 	Apacheta Corp.

Abstract

Individual motions drafted to address Letter Ballot 60 Clause 7 Comments of 802.11i draft 5.0.

Motion 1

Comment 157

In Clause 7.2.2, page 19 line 4, replace

The frame body consists of the MSDU or a fragment thereof, and a security header and trailer (if and only if the Protected Frame subfield in the frame control field is set to 1). The frame body is null (0 octets in length) in data frames of Subtype Null function (no data), CF-Ack (no data), CF-Poll (no data), and CF-Ack+CF-Poll (no data). These frame types shall be checked that the frame body is null and if not discard the frame.

With:

The frame body consists of the MSDU or a fragment thereof, and a security header and trailer (if and only if the Protected Frame subfield in the frame control field is set to 1). The frame body is null (0 octets in length) in data frames of Subtype Null function (no data), CF-Ack (no data), CF-Poll (no data), and CF-Ack+CF-Poll (no data). If a received frame of these types does include a frame body, the frame body shall be ignored.

Motion 2

Comment 62

In Clause 7.3.2.9, page 22 Table 2, replace:

| 00:00:00 | 6-255 | Reserved |
| --- | --- | --- |

With:

| 00:00:00 | 6 | No Encryption |
| --- | --- | --- |
| 00:00:00 | 7-255 | Reserved |

In Clause 7.3.2.9, page 22 Table 3, insert the following row after Use Group Key:

| No Encryption | Yes | Yes | No |
| --- | --- | --- | --- |

Motion 3

Comment 159

In Clause 7.3.2.9, page 22 Table 1, replace:

| 00:00:00 | 1 | Authentication negotiated over IEEE 802.1X – RSNA default | IEEE 802.1X Key Management as defined in Clause 8.5 or using PMK caching as defined in Clause 5.9.5 – RSNA default |
| --- | --- | --- | --- |

With:

| 00:00:00 | 1 | Authentication negotiated over IEEE 802.1X or using PMK caching as defined in Clause 5.9.5 – RSNA default | IEEE 802.1X Key Management as defined in Clause 8.5 or using PMK caching as defined in Clause 5.9.5 – RSNA default |
| --- | --- | --- | --- |

Motion 4

Comment 159a

In Clause 7.3.2.9, page 22 Table 1, replace:

| 00:00:00 | 2 | None | IEEE 802.1X Key Management as defined in Clause 8.5, using pre-shared key |
| --- | --- | --- | --- |

With:

| 00:00:00 | 2 | Pre-shared key | IEEE 802.1X Key Management as defined in Clause 8.5, using pre-shared key |
| --- | --- | --- | --- |