3GPP TSG SA2 #33					S2-032727

Sofia Antipolis, France, 7-11.07.2003

Title:	LS on 802.11i / WPA and RADIUS to Diameter co-existence analysis and recommendations for WLAN interworking

Release:	Release 6

Work Item:	WLAN Interworking

Source:	SA2

To:	CN4, IEEE / WNG

Cc:	SA3

Contact Person:

Name:	Tomas Goldbeck-Löwe

Tel. Number:	+46 (0)8 7641467

E-mail Address:	tomas.goldbeck-lowe@ericsson.com

Attachments:	S2-031745 LS on the specification of 802.11i and WPA link layer security and Radius to Diameter interworking

S3-030265 Co-Existence of RADIUS and Diameter

- Introduction

SA2 have received the attached LS from SA3, as a response regarding 802.11i / WPA link layer security and RADIUS – Diameter co-existence.

No architectural impacts were seen at this stage.

It was noted that the detailed analysis is too early to be considered in the current work in SA2. Therefore, the analysis is forwarded to the CN4 group to be considered in their work about WLAN interworking.

- Actions:

CN4 is asked to consider the analysis and recommendations in the attached documents in their stage 3 work with WLAN interworking.

IEEE / WNG is asked to take the analysis and recommendations about RADIUS – Diameter co-existence into consideration in their work with WLAN 3G interworking.

- Date of Next TSG–SA WG2 Meetings:

| Meeting | Date | Location | Host |
| --- | --- | --- | --- |
| SA2#34 | 18-22. August 2003 | Brussels |  |
| SA2#35 | 27-31. October 2003 | TBD |  |

3GPP TSG SA WG3 Security — S3#28	Tdoc S3-030265

6 - 9 May 2003,

Berlin, Germany

# Source:		Ericsson

# Title:		Co-Existence of RADIUS and Diameter

# Document for:	Discussion

# Agenda Item:	T.B.D

Abstract

This paper presents the differences between Diameter and RADIUS protocols, and discusses for the use of these protocols in WLAN inter-working in 3GPP in an interoperable manner. We also discuss the security-related impacts of this, as well as the status of, e.g., EAP support in both of these protocols in IETF.

# Introduction

Diameter [DIAMETER] and RADIUS [RADIUS] protocols define a framework for carrying authentication, authorization and accounting information between the Network Access Server (NAS) and Authentication Server (AAA Server). This discussion paper presents the differences between these protocols and discusses the transition of the network from one protocol to the other. The transition mechanism is based on IETF standard-track proposals.

RADIUS is a client-server protocol, while Diameter is based on a peer-to-peer model. Therefore, it is difficult, e.g., to implement server initiated messages in RADIUS without extensions to the protocol. Also, some existing applications such as the IMS rely on specific protocol extensions, which can only run on top of Diameter.

Currently, RADIUS is the AAA protocol that is most widely used in WLAN environments and all 802.1X and 802.11i compliant access points are expected to support RADIUS. Diameter, on the other hand, has only recently been approved as a standards-track RFC in IETF, and hence there are not many access points yet supporting it. This paper discusses how both protocols can live in the same network and existing access points can be used. This has some implications for the features and security of the AAA system when using those access points. These implications are listed here as well.

Finally, the IETF status of RADIUS and Diameter drafts related to WLAN inter-working is outlined.

# Comparison

Chapter 8 compares RADIUS [RADIUS] and Diameter [DIAMETER] against following properties: failover mechanisms, transmission-level security, reliable transport, agent support, server-initiated messages, audit-ability, transition support, capability negotiation, peer discovery and configuration, roaming support. The material has been derived largely from [DIAMETER] . As a summary, the differences are as follows:

| Property: | RADIUS: | Diameter: |
| --- | --- | --- |
| Failover mechanisms | Not defined (depends on implementation) | Supported |
| Transmission-level security (authentication and integrity) | Defined only for response packets. In [RADEAP] extension IPsec and IKE support is optional. | IPsec support is mandatory and TLS support is optional for access points, both for servers and proxies. |
| Reliable transport | UDP. Reliability varies between implementations. | TCP/SCTP. Reliable. |
| Accounting support | Defined in a non-standards track extension RFC. Reliability in various network and device error situations is implementation dependent. | Supported. The base protocol defines mechanisms for reliable transport and failover as above, and the accounting behaviour in network partition situations is controlled. |
| Agent support | Not a part of the core protocol, though [DYNAUTH] extension defines server-initiated messages. Status of the definition (Internet Draft) and support in products is unclear. | Supported. |
| Audit-ability | Not supported. | Supported / optional, but the required Diameter component is still being standardized. |
| Capability negotiation | Not supported | Supported |
| Peer discovery and configuration | Manual configuration | Dynamic |
| Roaming support | Not suitable for global roaming in open environments due to lack of security. | Secure and scalable roaming support. |

# Transition support

While Diameter does not share a common protocol data unit (PDU) with RADIUS, considerable effort has been expended in enabling backward compatibility with RADIUS, so that the two protocols may be deployed in the same network. Initially, it is expected that Diameter will be deployed within new network devices, as well as within gateways enabling communication between legacy RADIUS devices and servers. This capability, described in [NASREQ], enables Diameter support to be added to legacy networks, by addition of a gateway or proxy speaking both RADIUS and Diameter.

RADIUS is currently widely used protocol in WLAN environments. At the same time RADIUS is missing several features (see above), such as server initiated messages and may not operate with the highest possible security turned on. Diameter is better protocol, but it is not very widely deployed yet. Therefore, gradual migration from RADIUS to Diameter seems to be one potential way to go further.

It seems reasonable to start from an initial model of the AAA network where most or all of the access points implement only RADIUS, and a core which uses Diameter but is capable of talking to the RADIUS-only capable access points. This would mean that leaf AAA proxies should support both RADIUS and Diameter. As Diameter-capable access points are inserted to the network, they can be taken into use immediately. An advantage of placing the RADIUS/Diameter-capable nodes on the leafs of the network is that it becomes easier to take advantage of the features found in Diameter. For instance, even accounting may be more reliable if only the first hop is run in RADIUS but the traversal of the access provider, roaming consortium, and home operator proxies is done via DIAMETER.

The actual translation gateway must be able to run both RADIUS and Diameter protocols. The [NASREQ] extension defines a framework for the protocol conversion, where the RADIUS attribute space is included into Diameter, which eliminates the need to perform many attribute translations. However, some explicit translations between RADIUS and Diameter attributes must be made, like translating vendor specific and accounting information.

Some Diameter related messages can not be translated during the communication with RADIUS client, such as messages initiated by Diameter server. Interoperability between RADIUS and DIAMETER in the presence of some of the non-standard RADIUS extensions has not been specified.

# Security in Transition

The gateway needs to add use RADIUS application layer security mechanisms towards RADIUS, and IPsec or TLS towards Diameter. Given the use of the hop-by-hop security mechanisms, this translation can be performed without the knowledge of the original sender of the message. RADIUS requires pre-shared keys, while Diameter can take advantage of either IKE or TLS.

In addition, the translation gateway must secure attribute data towards the home server using Diameter CMS techniques (when the RFC is published). That is, end-to-end security mechanisms can be employed between the translation proxy and the home server, but not between the RADIUS-only access point and the translation proxy.

# Standardization Status

RADIUS authentication is a standards-track RFC, while RADIUS accounting is an informational RFC. RADIUS has several extensions. Many of the extensions are Internet Drafts, and it is not even clear whether they will be completed as RFCs.

On the other hand, while the core parts of Diameter have been approved as standards-track RFCs (base protocol and transport have been approved, the NASREQ extension will be soon), the CMS security extension is still being worked on. Diameter deployments during 2003 cannot take advantage of a standards-based CMS security, but need to rely on either transport or IP layer security.

The support for EAP in RADIUS is being reissued as RFC 2869bis, to clarify a number of interoperability issues that have been recognised. Base RADIUS RFC requires only the use of the application level MAC for some (not all) messages, but RFC 2869bis recommends the usage of IPsec. The Internet Draft [RADEAP] has passed IETF Last Call. When this draft is approved as an RFC, the same technical solution will be used to produce the DIAMETER EAP support RFC.

However, there is currently no standardised way to transport AAA-derived session keys from the home AAA server to the access point. The Microsoft vendor-specific attributes [MSATTR] are widely used, though believed to be quite insecure by today’s standards. IETF is working on a keying framework for EAP along with standardisation of session key transport attributes.

# Recommendation

We make the following recommendations on the basis of mature IETF Internet-drafts, which are on standard-tracks:

- Consider the adoption of Diameter – RADIUS compatibility mode i.e. support of both protocols along with the necessary translation mechanisms in order to enable the use of RADIUS-only access points. Such translation should occur as near the leaves of the network as possible. As not all functions can be translated in full, some loss of functionality occurs for those devices, which use RADIUS, and this should be documented.

- Additionally, take a stand on whether IPsec is required in those cases where RADIUS is used, as currently required in RFC 2869bis. This may help to eliminate some of the vulnerabilities of RADIUS.

- Adopt the use of RFC 2869bis and corresponding Diameter counterpart as the standard for running EAP over AAA protocols.

- The participation of SA3 member companies in the standardisation of EAP keying framework and key transport is highly desired.

# References

[DIAMETER]	P. R. Calhoun, et. al., “Diameter Base Protocol”, IETF Work in Progress (approved as an RFC).

[RADIUS]	C. Rigney, A. Rubens, W. Simpson, S. Willens, "Remote Authentication Dial In User Service (RADIUS)", RFC 2865,  June 2000.

[RADEAP] 	B. Aboba, and P. Calhoun, "RADIUS Support for Extensible Authentication Protocol (EAP)", IETF work in progress.

[RADACCT] 	Rigney, C., "RADIUS Accounting", RFC 2866, June 2000.

[RFC3162] 	B. Adoba, et. al., ”RADIUS and IPv6”, RFC 3162, August 2001.

[ACCMGMT]	B. Aboba, J. Arkko, D. Harrington. "Introduction to Accounting Management", RFC 2975, October 2000.

[AAATRANS]	B. Aboba, J. Wood, "Authentication, Authorization and Accounting (AAA) Transport Profile", IETF Work in Progress (approved as an RFC).

[DYNAUTH]	Chiba, M., et al., "Dynamic Authorization Extensions to RADIUS", IETF work in progress.

[AAACMS]	P. Calhoun, W. Bulley, S. Farrell, "Diameter CMS Security application," IETF Work in Progress.

[NASREQ]	P. Calhoun, W. Bulley, A. Rubens, J. Haag, "Diameter NASREQ Application", IETF work in progress.

[ROAMREV]	B. Aboba, et. al. "Review of Roaming Implementations", RFC 2194, September 1997.

[ROAMCRIT]	B. Aboba, G. Zorn, "Criteria for Evaluating Roaming Protocols", RFC 2477, January 1999.

[PROXYCHAIN] 	B. Aboba, J. Vollbrecht, "Proxy Chaining and Policy              Implementation in Roaming", RFC 2607, June 1999.

[MSATTRS] G. Zorn, “Microsoft Vendor-specific RADIUS Attributes”, RFC 2548, March 1999.

# Appendix: RADIUS – Diameter Differences

## Failover

In the event that a transport failure is detected with a peer, it is necessary for all pending request messages to be forwarded to an alternate agent, if possible. This is commonly referred to as failover.

RADIUS

RADIUS does not define failover mechanisms, and as a result, failover behaviour differs between implementations.

Diameter

In order to provide well-defined failover behaviour, DIAMETER supports application-layer acknowledgements, and defines failover algorithms and the associated state machine.

## Transmission-level security

End-to-end security services include confidentiality and message origin authentication. These services can be provided by supporting message integrity and confidentiality between two peers, communicating through agent.

RADIUS

RADIUS defines an application-layer authentication and integrity scheme that is required only for use with Response packets. While RADIUS Extensions [RADEAP] defines an additional authentication and integrity mechanism, use is only required during Extensible Authentication Protocol (EAP) sessions. While attribute hiding is supported, RADIUS does not provide support for per-packet confidentiality. In accounting, RADIUS Accounting [RADACCT] assumes that replay protection is provided by the back-end billing server, rather than within the protocol itself.

While [RFC3162] defines the use of IPsec with RADIUS, support for IPsec is not required. Since within IKE authentication occurs only within Phase 1 prior to the establishment of IPsec SAs in Phase 2, it is typically not possible to define separate trust or authorization schemes for each application. This limits the usefulness of IPsec in inter-domain AAA applications (such as roaming) where it may be desirable to define a distinct certificate hierarchy for use in a AAA deployment than for some other use of IPsec from the same node.

Diameter

In order to provide universal support for transmission-level security, and enable both intra- and inter-domain AAA deployments, IPsec support is mandatory in Diameter clients, and TLS support is optional.

## Reliable transport

As described in [ACCMGMT], reliable transport is a major issue in accounting, where packet loss may translate directly into revenue loss.

RADIUS

RADIUS runs over UDP, and does not define retransmission behaviour; as a result, reliability varies between implementations.

Diameter

In order to provide well-defined transport behaviour, Diameter runs over reliable transport mechanisms (TCP, SCTP) as defined in [AAATRANS]. Diameter also defines an accounting mode, which can be used during network partitions and other transmission problems.

## Accounting Support

Support for accounting relates to reliable transport of accounting data and ability to perform failovers as discussed above. In addition, different applications require different accounting record contents and generation mechanisms, and the treatment of fatal transport problems may be different in different sitautions.

RADIUS

RADIUS accounting exists as an Informational RFC and is not a Standards Track protocol. As discussed above, there are some limitations in the reliability and failover mechanisms in RADIUS.

RADIUS employs just one form of accounting, an event-based mechanism. The accounting data transported over it has a limited space for new defined attributes and a limited length of data in those attributes.

Diameter

Diameter accounting is a part of the Standards Track base protocol. In addition to the reliable transport and failover support, the specification provides the following:

- Application and home server directed control of error situations, such as network partitions.

- Application and home server directed control of the accounting record generation either as an event, start-stop, or interim.

- Large attribute space and length.

## Agent support

Agent support includes Proxies, Redirects and Relays.

RADIUS

RADIUS does not provide for explicit support for agents.  Since the expected behaviour is not defined, it varies between implementations.

Diameter

Diameter defines agent behaviour explicitly.

## Server-initiated messages

Server-initiated messages contain features such as unsolicited disconnect or re-authentication / re-authorization on demand across a heterogeneous deployment

RADIUS

RADIUS does not support server-initiated messages. However, there exists an Internet Draft [DYNAUTH] which adds this capability. (We can not indicate how widely this feature is supported, but at this point at least it is not an approved standards-track RFC.)

Diameter

Support for server-initiated messages is mandatory in Diameter.

## Audit-ability

The audit-ability property allows the system to detect if untrusted proxies modify attributes or even packet headers.

RADIUS

RADIUS does not define data-object security mechanisms. Combined with lack of support for capabilities negotiation, this makes it very difficult to determine what occurred in the event of a dispute.

Diameter

While implementation of data object security is not mandatory within Diameter, these capabilities are supported, and are described in [AAACMS]. However, this feature is not only an Internet Draft and is believed to require significant additional work before being approved as a standards-track RFC.

## Capability negotiation

Capability negotiation allows the discovery of peer's capabilities like, protocol version number, supported applications, security mechanisms, etc.

RADIUS

RADIUS does not support error messages, capability negotiation, or a mandatory/non-mandatory flag for attributes. Since RADIUS clients and servers are not aware of each other's capabilities, they may not be able to successfully negotiate a mutually acceptable service, or in some cases, even be aware of what service has been implemented.

Diameter

Diameter includes support for error handling, capability negotiation, and mandatory/non-mandatory attribute-value pairs (AVPs).

## Peer discovery and configuration

Allowing for dynamic agent discovery make it possible for simpler and more robust deployment of services.

RADIUS

RADIUS implementations typically require that the name or address of servers or clients be manually configured, along with the corresponding shared secrets. This results in a n administrative burden, and creates the temptation to reuse the RADIUS shared secret, which can result in major security vulnerabilities if the Request Authenticator is not globally and temporally unique as required in RADIUS.

Diameter

Through DNS, Diameter enables dynamic discovery of peers. Derivation of dynamic session keys is enabled via transmission-level security.

## Roaming support

RADIUS

The ROAMOPS WG provided a survey of roaming implementations [ROAMREV], detailed roaming requirements [ROAMCRIT], defined the Network Access Identifier (NAI)[NAI], and documented existing implementations (and imitations) of RADIUS-based roaming [PROXYCHAIN]. In order to improve scalability, [PROXYCHAIN] introduced the concept of proxy chaining via an intermediate server, facilitating roaming between providers.  However, since RADIUS does not provide explicit support for proxies, and lacks audit-ability and transmission-level security features, RADIUS-based roaming is vulnerable to attack from external parties as well as susceptible to fraud perpetrated by the roaming partners themselves. As a result, it is not suitable for wide-scale deployment e.g. on the Internet [PROXYCHAIN].

Diameter

By providing explicit support for inter-domain roaming and message routing, audit-ability [AAACMS], and transmission-layer security features, Diameter addresses these limitations and provides for secure and scalable roaming. However, a part of the functions required for this are still being standardized in [AAACMS].

### 3GPP TSG-SA WG2 meeting #32     	Tdoc S2-031745

- San Diego, U.S.A. , 12th – 16th May 2003

__________________________________________________________________________________

3GPP TSG-SA3 Meeting #28 	Tdoc S3-030299

Berlin, Germany 6th-9th May, 2003

Title:	LS on the specification of 802.11i and WPA link layer security and Radius to Diameter interworking

Response to:	S2-031510 Security in WLAN and 3G interworking

Release:	Rel-6

Work Item:	WLAN Interworking Security

Source:	SA3

To:	SA2

Cc:

Contact Person:

Name:			Colin Blanchard

Tel. Number:		+44 1473 605353

E-mail Address:		colin.blanchard@bt.com

Attachments:	S3-030265 Co-Existence of RADIUS and Diameter

- Overall Description

SA3 would like to thank SA2 for their response regarding the specification of 802.11i and WPA link layer security. It was SA3 original working assumption that we should be able treat these aspects as a “black box” and just provide support for mutual authentication and generation of keying material in line with the requirements set out in the SA2 response.   However, SA3 note the comment by SA2 “As well as protecting the WLAN air interface, the above techniques can be used to build support for non-repudiation of WLAN accounting records”. In addition, SA3 have also been considering the implications of the trust relation between the Cellular Operator and the WLAN Access Provider and the impact of this on the WLAN Interworking Security TS 33.234. Until this work is complete, SA3 are unable to confirm that the WPA defined encryption meets the security requirements for WLAN-3GPP inter-working.

On the issue of Radius to Diameter interworking, SA3 has been studying the security implications. A contribution (S3-030265 Co-Existence of RADIUS and Diameter) which makes a number of recommendations which were endorsed by SA3, is attached, for consideration by SA2.

2. Actions

SA2 to comment on the recommendations in (S3-030265 Co-Existence of RADIUS and Diameter)

3. Dates of Next TSG SA WG3 Meetings:

| Meeting | Date | Location |
| --- | --- | --- |
| SA3#29 | 15-18 July 2003 | San Francisco, USA |
| SA3#30 | 7-10 October 2003 | Europe ( TBA) |