IEEE P802.11
Wireless LANs

PMK sharing

Date:	Sept 17, 2003

Author:	Tim Moore
	Microsoft
	1 Microsoft Way, Redmond, WA, 98052
	Phone: 1-425-703-9861
	Fax: 
	e-Mail: timmoore@microsoft.com

Author:	Clint Chaplin
	Symbol Technologies
	6480 Via Del Oro, San Jose, CA, 95119
	Phone: 1-408-528-2766
	Fax: 
	e-Mail: cchaplin@sj.symbol.com

Abstract

Summary

Powerpoint is 11-03-0769-00-000i-PMKsharing.ppt

Add definitions to Clause 3

Supplicant Group MAC Address: A MAC Address for a group of Supplicants that are a secure group and can share an MSK among themselves.

Authenticator Group MAC Address: A MAC Address for a group of Authenticators that are a secure group and can share an MSK among themselves.

Clause 5.9.5

Add

The size of the domain that an authentication key is valid for is not limited to a single authenticator or a single supplicant.  A group of authenticators that form a secure group should be allowed to share an authentication key, and by the same token a group of supplicants that form a secure group should be allowed to share an authentication key.

In order to label a group, Authenticator Group MAC Addresses and Supplicant Group MAC Addresses are defined.  These Group MAC Addresses could be set to one of the individual MAC addresses in the group.

The result of authentication is an MSK; this MSK is derived in the supplicant, and also in the authentication server; the authentication server then passes the MSK to the authenticator.  This MSK is then sharable among all supplicants in the Supplicant Group the supplicant belongs to, and among all authenticators in the Authenticator Group that the authenticator belongs to.

The MSK is then used to derive a separate PMK for each BSSID interface within the Authenticator Group, and each supplicant within the Supplicant Group can then use the MSK to derive the correct PMK for the BSSID it is associated with.

What a key cache contains can be implementation dependent.  The best implementation would cache the intermediate results of the PMK derivation up to inclusion of the BSSID.

Clause 7.3.2.9

Change Figure 7

| Element ID 1 octet | Length 1 octet | Version 2 octets | Group Key Cipher Suite 4 octets | Pairwise Key Cipher Suite Count 2 octets | Pairwise Key Cipher Suite List 4m octets | Authentication and Key Management Suite Count 2 octets | Authentication and Key Management Suite List 4n octets | RSN Capabilities 2 octets | Group Addr 6 octets | PMKID Count 2 octets | PMKID List 16s octets |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Add

7.3.2.9.4 Group Addr

Group Addr is the Supplicant Group MAC address or the Authenticator Group MAC address. It is supplied if the Supplicant or Authenticator is a member of a group.

Informational Note: The Group MAC address may be one of the individual MAC addresses in the group.

The Authenticator Group MAC address is used when the RSN IE used in a Beacon or Probe response. The Supplicant Group MAC address is used when the RSN IE is used in a (re)association request.

Renumber 7.3.2.9.4 PMKID to 7.3.2.9.5 PMKID

Clause 5.9.5

Add the following at the end of 5.9.5

Supplicant can only use PMKs that were obtained from an IEEE 802.1X authentication via Authenticators in the same Authenticator Group MAC Address.

Authenticators can only use PMKs that were obtained from an IEEE 802.1X authentication with Supplicants in the same Supplicant Group MAC Address.

8.5.1.2

The PMK is derived from the Master Session Key. The MSK is available directly to the Supplicant. The MSK is available to the Authenticator as the AAA Key.

PMK = PRF-256(MSK(0-255),

“PMK Key”|Supplicant-Group-MAC-Address|Authenticator-Group-MAC-Address

|Supplicant-MAC-Address|Authenticator-MAC-Address)

Where MSK(0-255) is the first 256bits of the MSK.