IEEE P802.11
Wireless LANs

Dangers of PSK and the lack of role identity in authentication

Date:	September 18, 2003

Author:	William Arbaugh
	University of Maryland 
	College Park, MD
	Phone: 301-405-2774
	Fax: 
	e-Mail: waa@cs.umd.edu

Insun Lee

Samsung Electronics

e-Mail: insun@samsung.com

Abstract

Proposed draft text for inclusion into the TGi draft.

Change clause 8.4.4 lines 12-14

From:

Informative Note: When an IBSS network uses pre-shared keys, STAs can negotiate a unicast cipher. However, any STA in the IBSS can derive the pairwise keys of any other that uses the same pre-shared key by capturing the first two messages of the 4-Way Handshake.

To:

Informative Note: Warning. When an IBSS network uses pre-shared keys, STAs can negotiate a unicast cipher. However, any STA in the IBSS can derive the pairwise keys of any other that uses the same pre-shared key by capturing the first two messages of the 4-Way Handshake. This provides malicious insiders with the ability to eavesdrop as well as the ability to establish a man-in-the-middle.

Add the following paragraph to section 8.4.3

Informative Note: Warning. When an IBSS network uses pre-shared keys, STAs can negotiate a unicast cipher. However, any STA in the IBSS can derive the pairwise keys of any other that uses the same pre-shared key by capturing the first two messages of the 4-Way Handshake. This provides malicious insiders with the ability to eavesdrop as well as the ability to establish a man-in-the-middle

Add the following text to the draft at a location to be determined by the editor

Informative Note: The use of EAP authentication methods where server and client credentials can not be differentiated reduces the security of the method to that of a pre-shared key due to the fact that malicious insiders can masquerade as servers and establish a man-in-the-middle.