IEEE P802.11
Wireless LANs

Remedy to the Lost Ack Problem

Date:	November 11, 2003

Author:	Mathilde Benveniste
	Avaya Labs - Research
	233 Mt. Airy Road, Basking Ridge, NJ USA
	Phone: 973-761-6105
	e-Mail: benveniste@ieee.org

Menzo Wentink 
	Globespan B.V.	
	 Rembrandtlaan 1a      
	Bilthoven, the Netherlands   
	e-Mail: mwentink@globespan.net

Abstract

This document contains the material proposed to GTE for inclusion in the draft in the form of insertions into and replacements for material in IEEE Std 802.11-1999, as updated by IEEE Std 802.11a-1999, IEEE Std 802.11b-1999 (already present in TGE Draft 5.0).

Editorial notes appear in bold italic Times New Roman font, informative notes appear in normal Arial font, and normative text appears in normal Times New Roman font.  Open issues are highlighted using red text in normal Arial font, and begin with "OPEN ISSUE:".  Added text appears in blue with a dotted underline.  Deleted text appears in red with a strikethrough line.

Insert the following in section 11.2.1.4 after subclause h):

- If the QAP does not receive an acknowledgement to a directed MPDU or management frame sent to a STA in power-save mode following receipt of a PS Poll from that STA, it may retransmit the frame fewer times than the Max Retry Limit before the next TIM, but it shall retransmit that frame at least once before the next TIM, time permitting – subject to the Max Retry Limit and the MSDULifetime limit.  If an Ack to the retransmission is not received, it may wait until the next TIM to further retransmit that frame subject to the Max Retry Limit and the MSDULifetime limit.

Insert the following in section 11.2.3.1 after subclause c):

- If the QAP does not receive an acknowledgement to a directed MPDU or management frame sent with the More Data sub-field set to 0 or the EOSP set to 1, it may retransmit the frame fewer times than the Max Retry Limit within the same service period, but it shall retransmit that frame at least once within the same service period – subject to the Max Retry Limit and the MSDULifetime limit.  If an Ack to the retransmission is not received, it may wait until the next service period to further retransmit that frame subject to the Max Retry Limit and the MSDULifetime limit.