IEEE P802.11
Wireless LANs

802.11 TGn Functional Requiremenets

Date:	October 21, 2003

Author:	Adrian P Stephens
	Intel Corporation
	15 JJ Thompson Avenue, Cambridge, CB3 0FD, United Kingdom
	Phone: +44 1223 763457
	e-Mail: adrian.p.stephens@intel.com

Abstract

This document defines functional requirements that must be addressed by any proposal claiming that it is a complete proposal in response to the IEEE 802.11 TGn call for proposals.

# Contributors

(This will grow to reflect those providing email contributions / review comments of this document)

| Name | Company | Address | Phone | Fax | Email |
| --- | --- | --- | --- | --- | --- |
| Rahul Malik | Panasonic | Blk 1022 Tai Seng Ave. #06-3530 Tai Seng Industrial Estate, Singapore 534415 | +65 6550-5482 | +65 6550-5459 | rahul@psl.com.sg |
| Hervé |  |  |  |  |  |
| Javier |  |  |  |  |  |
| Paul Feinberg |  |  |  |  |  |
| Bjorn Bjerke |  |  |  |  |  |
| Adrian |  |  |  |  |  |

# Revision History

| Revision | Comments | Date |
| --- | --- | --- |
| R0 | Skeleton document created | 20 October 2003 |
| R1 | New content partially presented to discussed at the FRCC telecon. Includes updates agreed at the telecon. | 21 October 2003 |
| R2 | Comments on R1 merged in. This involved rewording and re-organisation of the comments so that the resulting combined document made sense. | 31 October 2003 |

# Introduction

## Purpose of document

A proposal submitted for consideration under the 802.11 TGn selection process [1], and declared to be complete is required to meet the functional requirements defined in this document.

A partial proposal may meet some of the requirements stated here.

## Form of Disclosure

A proposal shall declare its compliance with these functional requirements using the template defined below.

## Relationship to Comparison Criteria and Usage Models

Functional requirements may be described in terms of metrics defined in the 802.11 TGn comparison criteria document [2], or the simulation scenarios defined in the 802.11 TGn Usage Models document [3].

## Requirements for this document

(This section may be removed at a later date.  It is really only relevant while we are writing this document)

The functional requirements document shall include a set of requirement statements.  Each of these:

- Shall be defined unambiguously

- Can be verified from a reasonable simulation environment, or verified from an examination of the proposed submission

- Are compliant to the 802.11 HT PAR [1] and 5C [2]

- Should be relevant to one or more mandatory use cases in UM doc (TBD)

Note, at the September TGn meeting, it was decided that this document should relate only to mandatory features of the proposal.  Optional features, qualities, performances are considered in [2].

# Functional Requirements

(Chair's comment on mandatory/optional:

These terms can cause a lot of confusion and debate.  The job of this document is to define what is mandatory for a Proposal.  In my opinion, there is absolutely no point defining what is optional for a proposal (and it's also out of scope for this document).

Any decisions the authors of a proposal make about the mandatory/optional status of various features or modes of operation are very unlikely to survive unchanged through the subsequence comment resolution process.  That means it makes even less sense for us to try and do that even further up the food chain.)

| Number | Name | Requirement | Status of Requirement | Notes |
| --- | --- | --- | --- | --- |
| 1 | HT Usage Models supported | Mandatory HT Usage Models supported. Note: supported => PLR of QoS streams is satisfied. Note: packet loss is a packet that doesn’t arrive or arrives late according to the specified delay bound. | Consensus of FRCC, 21 Oct 2003. | OK Yes |
| - "HT Usage Models supported": some discussion of the specified maximum PLR for DV/SDTV/HDTV. The agreed position was that the specified max PLR is acceptable if it is based on a 1% PLR rate for Over-the-Air single packet transmission and a maximum delay of (~200ms) -no consensus on the exact delay but most agreed to this as a good approximation. The group also assumed that a 200msec delay requirement allowed for ample ARQ and therefore a lower overall PLR. - SHOULD BE SPECIFIED WITH ONE OR ALL OF THE USAGE MODELS |  |  |  |  |
|  | Point-point HT rate supported | Supports a point-point throughput of 100Mbps at the top of the MAC at a range of 15m (TBD) for channel models (B, C, D - TBD) defined in [4]. | APS initial proposal | OK Yes |
| - "Point-point HT rate supported": some discussion on whether this requirement complements PAN expectations, possibly suggesting 802.15 usage model is needed to satisfy isolated PAN networks using HT LAN feature to access a content server. No recommended changes to this functional requirement. - FR# 2 (P2P rate): If this is a functional requirement, it should rather appear in a usage model scenario. In the usage model document, the simulation environment is (will be) well defined, which is not the case in the functional requirements document. Answering by just yes/no to this question without fixing the rules won't bring much helpful information on the real capacity of the system. This is why we decided to define scenarios, isn't it? - Need to clarify scenario characteristics under which 100 Mbps has to be supported. For example Frame Size, Number of TX STAs and RXs STAs. QoS need to be supported as well. |  |  |  |  |
|  | HT rate supported in 20MHz channel | Point-point throughput of 100Mbps shall be met using a 20MHz channel in at least one of the modes of operation. | Rahul proposal | The regulatory bodies of many countries do not allow operation of a single device using several channels |
|  | Multi-point to point high throughput supported | Supports a aggregated throughput of 100Mbps at the MAC SAP at a range of upto 30m (TBD) for channel models corresponding to the environments specified by the use-cases in [3] and defined in [4]. This requirement shall be met using a 20MHz channel in atleast one of the modes of operation | Rahul proposal | The regulatory bodies of many countries do not allow operation of a single device using several channels |
|  | Current Spectral flexibility | Protocol supports 2.4GHz ISM and 5GHz UNII bands Alternative: Protocol supports the 5GHz UNII bands and optionally the 2.4GHz ISM bands. | APS initial proposal | Based on the PAR, the only real requirement on backwards compatability is with 11a. As there are several FRs in the initial document which are related to compatability with b and g, I suggest these be made CCs, as they are optional. OK Yes |
|  | Japanese Bands Spectral flexibility | Protocol supports .11j 10MHz channels in an optional mode of operation. | APS initial proposal | OK NO. Support of 11j should be optional. |
|  | Cost / Complexity flexibility | Protocol provides options that allow low-cost lower performance variants and high-cost higher performance variants. Alternative: Protocol provides options that allow performance and cost tradeoff for different modes of operation . | APS initial proposal | IT IS DIFFICULT TO MAKE THIS A HARD REQUIREMENT. USE AS A COMPARISON CRITERION INSTEAD Yes |
|  | .11a/b/g BSS (backward) compatibility | A HT STA, including the AP, can communicate directly with a legacy .11b/g STA and/or .11a STA (depending on the frequency band of operation) in both BSS, managed by a HT or legacy AP, and IBSS modes. | [5] PAR, section 18 and APS initial proposal Javier proposal for combined FR | YES. (We have attempted to combine all possible cases listed below into one single Functional Requirement) |
|  | .11b BSS compatibility | A HT STA in an optional mode of operation. can communicate with a legacy .11b STA operating within a BSS managed by a legacy .11b AP | APS initial proposal | OK Replace with general statement |
|  | .11g BSS compatibility | A HT STA in an optional mode of operation.can communicate with a legacy .11g STA operating within a BSS managed by a legacy .11g AP | [5] PAR, section 18 | OK Replace with general statement |
|  | .11a BSS compatibility | A HT STA can communicate with a legacy .11a STA operating within a BSS managed by a legacy .11a AP | [5] PAR, section 18 | OK Replace with general statement |
|  | .11b STA compatibility | A HT STA in an optional mode of operation.can communicate with a legacy .11b STA operating within a BSS managed by a HT AP | APS initial proposal | OK Replace with general statement |
|  | .11g STA compatibility | A HT STA in an optional mode of operation.can communicate with a legacy .11g STA operating within a BSS managed by a HT AP | [5] PAR, section 18 | OK Replace with general statement |
|  | .11a STA compatibility | A HT STA can communicate with a legacy .11a STA operating within a BSS managed by a HT AP | [5] PAR, section 18 | OK Replace with general statement |
|  | .11b IBSS Backward compatibility | HT and legacy .11b STA in an optional mode of operation.can communicate directly within a single IBSS consisting of a mixture of legacy .11b and HT devices | APS initial proposal | OK Replace with general statement |
|  | .11g IBSS Backward compatibility | HT and legacy .11g STA in an optional mode of operation.can communicate directly within a single IBSS consisting of a mixture of legacy .11g and HT devices | APS initial proposal | OK Replace with general statement |
|  | .11a IBSS Backward compatibility | HT and legacy .11a STA can communicate directly within a single IBSS consisting of a mixture of legacy .11a and HT devices | APS initial proposal | OK Replace with general statement |
|  | .11e QoS support | All HT STAs shall support the channel access mechanisms provided by 11e. | Rahul Proposal |  |
|  | All changes pertain to higher throughput | All the proposed mechanisms have a positive effect on throughput | APS initial proposal | RM: delete this FR. NO (see below) USE PAR LANGUAGE: Proposal does not redefine mechanisms in the baseline that do not pertain to higher throughput |
| Some mechanisms can be defined to achieve higher robustness/range etc… The PAR already states that 802.11n shall not redefine mechanisms specified in the baseline, that should be enough |  |  |  |  |
|  | Spectral Efficiency | The value of metric (spectral efficiency) has a value of 3bps/Hz for at least one mode of operation of the system (that mode of operation being optional for the device) | APS initial proposal | USE PAR LANGUAGE: The spectral efficiency is at least 3 bps/Hz for the PSDU for one or more modes of operation YES. Isn’t this a PAR requirement? |
|  | Fair sharing | A HT BSS shares the medium fairly with a co-channel co-located legacy BSS. | APS initial proposal | MORE SPECIFIC: A HT BSS shares the medium fairly on an equal time basis with a co-channel co-located legacy BSS. NO |
| I have the same concerns as above, i.e. what does fairly really means? RM: I am not sure how to evaluate this criterion and hence suggest that it be deleted. Definition of fairness? |  |  |  |  |
|  | Increased Range for current rates | The protocol supports increased ranges for rates matching the legacy rates. Alternative: The protocol supports increased ranges for similar legacy rates. | APS initial proposal | THIS BELONGS IN THE COMPARISON CRITERIA YES |
|  | Power management | The protocol permits operation of the 802.11 (+e) power-saving modes | APS initial proposal | RM: There is no need to limit power-saving to current 11e modes. HT devices would have enhanced MAC features and this may facilitate better PS mechanisms. THIS BELONGS IN THE COMPARISON CRITERIA YES. Should we consider making 802.11n backward compatible to 802.11e? |
|  | Regulatory | The protocol provides signalling of any constraints on modes of operation specific to regulatory constraints due to geopolitical or regional regulations | APS initial proposal | OK NO. It should be an optional feature depending on the regulatory domain. |
|  | Baseline | The protocol builds on the baseline specification defined by 802.11 and ammendments a,b,d,e,g,h,i,j The proposed mechanisms cannot redefine mechanisms already supported in the baseline. | [5] PAR, section 18 | OK YES. |

General Review comments:

My first feeling is that there is a lot a items concerning compatibility comparing to the ones concerning performances.

# Template for Complete Submissions

The results for a complete submission shall include a statement of coverage of the functional requirements and results for simulation scenarios following the outline presented here.

## Coverage of Functional Requirements

Each requirement shall be declared to be covered or not covered.  For each requirement relating to measurements, a reference to the document/page showing the appropriate performance measurements should be added.

| Number | Name | Requirement | Coverage | Results Reference |
| --- | --- | --- | --- | --- |
| R1 | TBD | TBD |  |  |
| Rest of table will be filled in when the functional requirements are agreed… |  |  |  |  |

# References

[1] IEEE 802 11-03/665, TGn Selection Procedure

[2] IEEE 802 11-03/814, TGn Comparison Criteria

[3] IEEE 802 11-03/802, TGn Usage Models

[4] IEEE 802 11-03/161r2, TGn Channel Models

[5] IEEE 802.11-02/798r7, Draft PAR for High Throughput Study Group

[6] IEEE 802.11-02/799r6, Criteria for Standards Development (HTSG 5 Criteria Document)