IEEE P802.11
Wireless LANs

802.11 TGn Functional Requiremenets

Date:	November 12, 2003

Author:	Adrian P Stephens
	Intel Corporation
	15 JJ Thompson Avenue, Cambridge, CB3 0FD, United Kingdom
	Phone: +44 1223 763457
	e-Mail: adrian.p.stephens@intel.com

Abstract

This document defines functional requirements that must be addressed by any proposal claiming that it is a complete proposal in response to the IEEE 802.11 TGn call for proposals.

# Contributors

(This will grow to reflect those providing explicit email contributions / review comments of this document.)

| Name | Company | Address | Phone | Fax | Email |
| --- | --- | --- | --- | --- | --- |
| Adrian P. Stephens | Intel Corporation | 15 JJ Thompson Avenue, Cambridge CB3 0FD, United Kingdom | +44 1223 763457 |  | adrian.p.stephens@intel.com |
| Bjorn Bjerke | Qualcomm | 9 Damonmill Sq., Suite 2A, Concord, MA 01742, USA | +1 781-276-0912 | +1 781-276-0901 | bbjerke@qualcomm.com |
| Hervé Bonneville | Mitsubishi Electric | 1 allée de Beaulieu, CS 10806, 35708 Rennes cedex 7, France | +33 (0)2 23 45 58 58 |  | bonneville@tcl.ite.mee.com |
| Javier del Prado | Philips | 345 Scarborough Rd, Briarcliff Manor, NY, 10510, USA | +1 914 945 6000 | +1 914 945 6580 | javier.delprado@philips.com |
| Paul Feinberg | Sony | 1 Sony Drive MD TA1-5 Park Ridge, NJ 07656 | +1 201 930-6316 | +1 201 930-6397 | Paul.Feinberg@am.sony.com |
| Rahul Malik | Panasonic | Blk 1022 Tai Seng Ave. #06-3530 Tai Seng Industrial Estate, Singapore 534415 | +65 6550-5482 | +65 6550-5459 | rahul@psl.com.sg |

# Revision History

| Revision | Comments | Date |
| --- | --- | --- |
| R0 | Skeleton document created | 20 October 2003 |
| R1 | New content partially presented to discussed at the FRCC telecon. Includes updates agreed at the telecon. | 21 October 2003 |
| R2 | Comments on R1 merged in. This involved rewording and re-organisation of the comments so that the resulting combined document made sense. | 31 October 2003 |
| R3 | Changes approved at the 4 Nov 2003 FRCC telecon merged in. Approved changes are shown without change tracking. Comments made by the FRCC which have not been considered are shown as tracked changes. | 10 November 2003 |
| R4 | Comments by WFA merged in | 12 November 2003 |
| R5 | Updated during 12 Nov 2003 TGn session |  |

# Introduction

## Purpose of document (Informative)

A proposal submitted for consideration under the 802.11 TGn selection process [1], and declared to be complete is required to meet the functional requirements defined in this document.

A partial proposal may meet some of the requirements stated here.

## Form of Disclosure

A proposal shall declare its compliance with these functional requirements using the template defined below in clause ??.

## Relationship to Comparison Criteria and Usage Models

Individual functional requirements may reference terms or metrics defined in the 802.11 TGn comparison criteria document [2], or the simulation scenarios defined in the 802.11 TGn Usage Models document [3].

## Requirements for this document

(This section may be removed at a later date.  It is really only relevant while we are writing this document)

The functional requirements document shall include a set of requirement statements.  Each of these:

- Shall be defined unambiguously

- Can be verified from an examination of the proposed submission or a reasonable simulation environment.

- Are compliant to the 802.11 HT PAR [1] and 5C [2]

Note, at the September TGn meeting, it was decided that this document should relate only to mandatory features of the proposal.  Optional features, qualities, performances are considered in [2].

Functional Requirements

(FRCC Chair's comment on mandatory/optional: (to be removed before final version)

These terms can cause a lot of confusion and debate.  The job of this document is to define what is mandatory for a Proposal.  In my opinion, there is absolutely no point defining what is optional for a proposal (and it's also out of scope for this document).

Any decisions the authors of a proposal make about the mandatory/optional status of various features or modes of operation are very unlikely to survive unchanged through the subsequence comment resolution process.  That means it makes even less sense for us to try and do that even further up the food chain.)

| Number |  |  | Name |  | Requirement |  | Status of Requirement |  | Notes |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  | Single Link HT rate supported |  | Proposal supports a single link throughput of 100Mbps at the top of the MAC SAP measured in the context of the simulation scenario #??. |  | Agreed at 4 Nov 2003 telecon, but see note. 20+20/2+7 Add usage model for single-link case and reference from here. |  | OK Yes |  |  |  |  |  |
| 2 |  |  | Single link HT rate supported at specified range |  | The single link HT rate measured in FR1 is met at a range of 15m |  | Lack of consensus in TGn as to whether to keep the 15m. |  |  |  |  |  |  |  |
|  |  |  | SAP Compatibility |  | Backwards compatible with existing 802.11 architectural interfaces, such that the current MAC-SAP functionality is retained |  | WFA proposal 14+18/6+7 |  |  |  |  |  |  |  |
|  |  |  | HT rate supported in 20MHz channel |  | Point-point throughput of 100Mbps as measured by the 20MHz throughput / range comparison criterion shall be met in at all modes of operation. |  | Rahul proposal Agreed at 4 Nov 2003 telecon Agreed at 12 Nov TGn. |  | The regulatory bodies of many countries do not allow operation of a single device using several channels |  |  |  |  |  |
|  |  |  | Supports 5GHz bands |  | Protocol supports 5GHz bands (including those supported by .11a) |  | Straw poll: 92/1 on 12 Nov 2003 |  |  |  |  |  |  |  |
|  |  |  | .11a backwards compatibility |  | Some of the modes of operation defined in the proposal shall be backwards compatible and interoperable with 802.11a. |  | 34+43/1 on 12 Nov 2003 |  | Note, weak consensus on expressing .11a and .11g separately. Note: backwards compatibility means that it supports all the mandatory modes of the .11a standard. (vote 54/2) |  |  |  |  |  |
|  |  |  | .11g backwards compatibility |  | If it supports 2.4 GHz operation, some of the modes of operation defined in the proposal shall be backwards compatible and interoperable with 802.11g. |  | 25+44/1+8 |  | Note: backwards compatibility means that it supports all the mandatory modes of the .11g standard. (vote: 54/2) |  |  |  |  |  |
|  |  |  | Control of support for legacy STA from .11n AP |  | . A .11n AP can be configured to reject or accept associations from legacy STA because they are legacy STA. |  | WFA proposal 49/17 on 12 Nov 2003 |  |  |  |  |  |  |  |
|  |  |  | .11e QoS support |  | All HT STAs shall support the channel access mechanisms provided by 11e. |  | Rahul Proposal 52/30 on 12 Nov 2003 |  | Note, consensus is weak. |  |  |  |  |  |
|  |  |  | .11e QoS support |  | The proposal shall permit implementation of the 802.11e options within a .11n STA |  | John K proposal 70/6 on 12 Nov 2003. |  |  |  |  |  |  |  |
|  |  |  | Spectral Efficiency |  | The highest throughput mode of the proposal shall achieve a spectral efficiency of at least 3 bps/Hz for the PSDU |  | Vote 73/0 on 13 Nov 2003 |  | USE PAR LANGUAGE: The spectral efficiency is at least 3 bps/Hz for the PSDU for one or more modes of operation YES. Isn’t this a PAR requirement? |  |  |  |  |  |
|  |  |  |  |  |  |  | APS initial Proposal Voted no, 14/51 on 13 nov 2003 |  | MORE SPECIFIC: A HT BSS shares the medium fairly on an equal time basis with a co-channel co-located legacy BSS. NO |  |  |  |  |  |
| I have the same concerns as above, i.e. what does fairly really means? RM: I am not sure how to evaluate this criterion and hence suggest that it be deleted. Definition of fairness? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | APS initial proposal Voted no 17/39 on 13 nov 2003 |  | THIS BELONGS IN THE COMPARISON CRITERIA YES |  |  |  |  |  |
| Some mechanisms can be defined to achieve higher robustness/range etc… The PAR already states that 802.11n shall not redefine mechanisms specified in the baseline, that should be enough |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | APS initial proposal Voted no 0/42 on 13 Nov 2003 |  | RM: There is no need to limit power-saving to current 11e modes. HT devices would have enhanced MAC features and this may facilitate better PS mechanisms. THIS BELONGS IN THE COMPARISON CRITERIA YES. Should we consider making 802.11n backward compatible to 802.11e? |  |  |  |  |  |
|  |  |  | Regulatory |  | If the proposal introduces any new regulatory dependencies, it shall also define the mechanisms to comply with them. |  | APS initial proposal Passed very weakly 16/12 on 13 Nov 2003 |  | OK NO. It should be an optional feature depending on the regulatory domain. |  |  |  |  |  |
|  |  |  |  |  |  |  | WFA proposal Voted no 12/61 on 13 Nov 2003 |  |  |  |  |  |  |  |
|  |  |  | Anticipating new spectrum |  | Proposal shall not prevent use of future bands provided regulations using those bands are consistent with regulations for which .11a/.11g is currently deployed |  | Voted no 1/54 |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | Voted no, 19/61 on 13 Nov 2003 |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | [5] PAR, section 18 WFA proposal Existence of this FR fails 6/18 13 Nov 2003. |  | OK YES. |  |  |  |  |  |
|  |  |  | Compliance to PAR |  | The proposal complies with all the mandatory requirements of the PAR [5] and 5 Criteria [6] |  | Voted yes 71/0 13 Nov 2003 |  |  |  |  |  |  |  |
|  |  |  | Support of HT usage models |  | The proposal shall support the mandatory 801.1n usage models |  | Voted yes 38/26 13 Nov 2003 |  |  |  |  |  |  |  |

General Review comments:

My first feeling is that there is a lot a items concerning compatibility comparing to the ones concerning performances.

# Template for Complete Submissions

The results for a complete submission shall include a statement of coverage of the functional requirements and results for simulation scenarios following the outline presented here.

## Coverage of Functional Requirements

Each requirement shall be declared to be covered or not covered.  For each requirement relating to measurements, a reference to the document/page showing the appropriate performance measurements should be added.

| Number | Name | Requirement | Coverage | Results Reference |
| --- | --- | --- | --- | --- |
| R1 | TBD | TBD |  |  |
| Rest of table will be filled in when the functional requirements are agreed… |  |  |  |  |

# References

[1] IEEE 802 11-03/665, TGn Selection Procedure

[2] IEEE 802 11-03/814, TGn Comparison Criteria

[3] IEEE 802 11-03/802, TGn Usage Models

[4] IEEE 802 11-03/161r2, TGn Channel Models

[5] IEEE 802.11-02/798r7, Draft PAR for High Throughput Study Group

[6] IEEE 802.11-02/799r6, Criteria for Standards Development (HTSG 5 Criteria Document)